| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4704 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16, 5/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.245: DC parameter format update |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This contribution discusses the editor’s note regarding the data channel parameter format.

2.	Discussion

In H.245 there is an editor's note regarding replacing the reliability parameter from DCEP with the parameter from draft-ejzak-mmusic-data-channel-sdpneg (now draft-mmusic-data-channel-sdpneg). i.e.

DataChannelProfile ::= SEQUENCE

{

channelType                  INTEGER (0..255) OPTIONAL,

priority                     INTEGER (0..65535) OPTIONAL,

relParameter                 INTEGER (0..4294967295)OPTIONAL,

label                        IA5String ( SIZE (1..65535) ) OPTIONAL,

protocol                     IA5String ( SIZE (1..65535) ) OPTIONAL,

genericInformation           SEQUENCE OF GenericInformation OPTIONAL,

establishmentType ::= CHOICE

{

sctpStreamID             INTEGER (0..65535) OPTIONAL,

dcep                    NULL,

...

},

...

}

-- {Editor’s note: relParameter may be replaced by elements for the Max-retr,

-- Max-time and Ordered parameters from draft-ejzak-mmusic-data-channel-sdpneg.}

The current format in the ASN.1 reuses the encoding from the DCEP reliability parameter (clause 5/[draft-ietf-rtcweb-data-protocol].  The channelType is used to determine reliability and ordering and the reliability parameter is used to indicate maximum retransmission or lifetime. The DCEP reliability parameter and channel type are signalled over the data channel in an establishment message. The reuse of the parameter was proposed in H.245 as the IETF had not adopted the out-of-band data channel establishment method and the CLUE protocol still used the in-band method. This situation has now changed with the IETF adopting draft-mmusic-data-channel-sdpneg and CLUE adopting this method.

Instead of using two parameters, draft-mmusic-data-channel-sdpneg uses three parameters, “ordered”, “max-retr” and “max-time” to convey the reliability information. These parameters are used in SDP signalling before the data channel is established. This is a similar situation to H.323 where H.245 is signalling before media is established.

In order to align H.245 with draft-mmusic-data-channel-sdpneg, the Contributors propose the updates below.

3.	Proposal

It is proposed to accept the following changes. The changes are marked up against TD 223R1/WP1 from the Geneva, February Q2/16 Rapporteurs meeting.

# Recommendation ITU-T H.245

# Control protocol for multimedia communication

…

# 2	References

…

[122]	IETF draft-ietf-mmusic-data-channel-sdpneg-02 (QQQQ), SDP-based "SCTP over DTLS" data channel negotiation (work in progress).

…

Annex A

Messages: Syntax

…

# DataChannelProfile ::= SEQUENCE

{

ordered						BOOLEAN OPTIONAL,

priority						INTEGER (0..65535) OPTIONAL,

reliabilityParm ::= CHOICE

{

maxRetr					INTEGER (0.. 4294967295),

maxTime 					INTEGER (0.. 4294967295),

...

} OPTIONAL,

label						IA5String ( SIZE (1..65535) ) OPTIONAL,

protocol						IA5String ( SIZE (1..65535) ) OPTIONAL,

genericInformation				SEQUENCE OF GenericInformation OPTIONAL,

establishmentType ::= CHOICE

{

sctpStreamID                INTEGER (0..65535) OPTIONAL,

dcep						NULL,

...

},

...

}

…

#### B.2.2.7	Data application capabilities

…

dataChannel requests the use of an WebRTC data channel as defined by draft-ietf-rtcweb-data-channel [110] according to the configuration described by dataChannelProfile.  A single dataChannel is defined per SCTP association.

DataChannelProfile contains parameters associated with each of the protocols controlled by the data channel establishment protocol (draft-ietf-rtcweb-data-protocol [114]).

ordered indicates whether the datachannel is ordered or unordered as defined by clause 6.1.1.7/ draft-ietf-mmusic-data-channel-sdpneg [122]. If ordered is not included, a default of ordered=TRUE is assumed.

priority indicates the relative priority of the channel as defined by clause 5.1 / draft-ietf-rtcweb-data-protocol [114].

reliabilityParm allows the either the maxRetr (clause 6.1.1.5 / draft-ietf-mmusic-data-channel-sdpneg [122]) or the maxTime (clause 6.1.1.6 / draft-ietf-mmusic-data-channel-sdpneg [122]) parameter optionally to be sent.

label associates a tag with the channel as defined by clause 5.1 / draft-ietf-rtcweb-data-protocol [114].

protocol associates a protocol name with the channel as defined by clause 5.1 / draft-ietf-rtcweb-data-protocol [114].  The value is a protocol registered in the “WebSocket Subprotocol Name Registry” created in clause 11.5 / IETF RFC 6455 [115].

establishmentType indicates whether the data channel is established via DCEP or via out-of-band signalling. The latter is indicated by the sctpStreamID, which is the SCTP stream identity associated with the data channel.

__________________________

____________________