| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4712 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.TPS-SIG: Editor’s notes – H.245 CLUE control capability |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This contribution discusses the support of H.245 CLUE control capability in H.TPS-SIG.

2.	Discussion

H.TPS-SIG currently contains two editor’s notes related to a “H.245 CLUE control capability”, i.e.

1) Clause 7.2.1.2 “Editor’s note: this clause is to define a method/elements for carrying an indication to initiate CLUE messages.”

2) Clause 8.3.1 “Editor’s note: A similar capability exchange will likely be needed if H.245 transport is used.”

Clause 7.2.1 on CLUE capability negotiation already discusses how the choice of using either the data channel or H.245 generic message method for carrying CLUE is signalled. Basically if the support of CLUE via a data channel is not signalled in the TerminalCapability exchange then the H.245 generic message approach is used. However the downside of that approach is that an endpoint cannot signal that it supports both the data channel and H.245 generic message methods. This may cause problems with interworking.

There are several possible approaches moving forward.

Option A – Keep the current procedure whereby an endpoint can only signal support of either data channel or H.245 generic message transport for CLUE.

Option B – Update the current procedure whereby endpoint must support the H.245 generic message transport as a fallback and may offer a data channel transport for CLUE. If the receiving endpoint did not support data channel then it could simply remove that datacapability in the reply. No additional code points would be needed.

Option C – Define new procedures that allow an endpoint to indicate support for CLUE over data channel and/or H.245 generic messages. This would require an updated CLUEControlCapability to indicate which CLUE transport option is used.

3.	Proposal

It is proposed to accept one of the options below. The changes are marked up against TD 226R1/WP1 from the Geneva, February Q5/16 Rapporteurs meeting.

# Option A

# Draft new ITU-T H.TPS-SIG

Signalling for telepresence-enabled conferencing

…

…

8.3.1 CLUE protocol capability exchange

Clause 7.2.1 indicates that an endpoint may use a separate data connection or use H.245 for the transport of CLUE messages.

If the initiating H.323 telepresence endpoint decides to use a separate data connection, in the initial TerminalCapabilitySet message it should set a bidirectional DataApplicationCapability indicating the use of “CLUE” over a WebRTC Data Channel as defined by Annex U/[ITU-T H.245]. If the peer endpoint is a H.323 telepresence endpoint then it should return a TerminalCapabilitySet message also indicating the support of a data connection for the CLUE protocol. The initiating endpoint shall then establish a WebRTC Data Channel association containing the CLUE protocol as per Annex U/[ITU-T H.245].

If the initiating H.323 telepresence endpoint does not want to use a separate data connection and the peer endpoint has indicated support for CLUE as per clause 8.2 then the preceding step is not needed.

Once the initiating endpoint is ready to send CLUE messages (either via H.245 transport or data channel transport) it shall initiate messaging according to [IETF CLUE PROT]. The endpoints shall exchange CLUE Option messages and each provide an initial CLUE Advertisement. The CLUE Advertisement messages shall contain encodingIDs related to CLUE controlled media capabilities communicated via H.245 TerminalCapabilitySet messages. This may relate to already communicated capabilities or capabilities yet to be communicated.

…

# Option B

# Draft new ITU-T H.TPS-SIG

Signalling for telepresence-enabled conferencing

…

6.2 Phase B - Initial communication and capability exchange

In the knowledge that a peer supports CLUE signalling the endpoint issues a H.245 TerminalCapabilitySet message with its supported configurations. In addition, in order to link the particular configurations with particular CLUE capture encodings a CLUE Encoding Correlation containing the relevant encodingID is appended to each configuration.

In cases where the peers’s support of CLUE signalling is not known, the endpoint should include at least one configuration that is not labelled with an encodingID.

The endpoint then uses CLUE messages to perform the procedures of [IETF CLUE SIG]. The first part of the exchange is to negotiate the CLUE protocol version and any supported extensions. The second part is the Advertisement of the endpoint capabilities using the CLUE data as outlined in [IETF CLUE DM]. The encodingIDs used in the CLUE Advertisement provide a link to the H.245 capabilities with the same encodingID.

The TerminalCapabilitySet configurations need to align with the simultaneous transmission sets (STS) in the CLUE data.

On reception of the CLUE Advertisement, the peer endpoint will select which captures and encodings it requires based on the information in the Advertisement message and the capability information from the TerminalCapabilitySet message. The peer endpoint will then send a CLUE Configure message indicating which Captures and Encodings it has selected to receive.

The peer endpoint also replies with a TerminalCapabilitySetAck indicating that it has received the terminal capability set configurations.

{Contributor’s note: One of the consequences of Option B is that at least the H.245 Generic message method should be supported. The data channel method may be supported in addition.}

…

7.2.1 CLUE capability negotiation

This clause describes the methods for negotiating the use of CLUE in H.323 systems.

An endpoint should implement the H.245 Generic Message Transport (clause 7.2.2.2) method for transporting CLUE. In addition an endpoint may implement the SCTP transport (clause 7.2.2.1) method. An endpoint may determine which method to use through the capabilities contained in the TerminalCapability exchange. The presence of a bidirectional DataApplicationCapability with the application indicating “dataChannel” and the protocol field of the DataChannelProfile indicating “CLUE” indicates the use of CLUE over an SCTP connection. In the absence of these fields, the endpoint shall transport CLUE messages using H.245 generic messages.

…

7.2.2 CLUE Messaging

This clause describes the methods for the carriage of CLUE messages within a H.323 system.

…

8.3 Capability exchange

Capability exchange for telepresence endpoints is based on H.323 capability exchange procedures in clause 8.2/[ITU-T H.323] with the addition of CLUE protocol messaging to provide further information regarding media. The telepresence capability exchange procedures have two parts. The capability exchange related to the establishment of the CLUE protocol and the capability exchange related to the CLUE controlled media.

8.3.1 CLUE protocol capability exchange

Clause 7.2.1 indicates that an endpoint may use a separate SCTP based data connection or use H.245 for the transport of CLUE messages.

If the initiating H.323 telepresence endpoint decides to use a separate data connection, in the initial TerminalCapabilitySet message it should set a bidirectional DataApplicationCapability indicating the use of “CLUE” over a WebRTC Data Channel as defined by Annex U/[ITU-T H.245]. If the peer endpoint is a H.323 telepresence endpoint then it should return a TerminalCapabilitySet message also indicating the support of a data connection for the CLUE protocol. The initiating endpoint shall then establish a WebRTC Data Channel association containing the CLUE protocol as per Annex U/[ITU-T H.245].

If the initiating H.323 telepresence endpoint does not want to use a separate SCTP based data connection and the peer endpoint has indicated support for CLUE as per clause 8.2 then the preceding step is not needed.

Once the initiating endpoint is ready to send CLUE messages (either via H.245 transport or data channel transport) it shall initiate messaging according to [IETF CLUE PROT]. The endpoints shall exchange CLUE Option messages and each provide an initial CLUE Advertisement. The CLUE Advertisement messages shall contain encodingIDs related to CLUE controlled media capabilities communicated via H.245 TerminalCapabilitySet messages. This may relate to already communicated capabilities or capabilities yet to be communicated.

…

# Option C

# Draft new ITU-T H.TPS-SIG

Signalling for telepresence-enabled conferencing

…

7.2.1 CLUE capability negotiation

This clause describes the methods for negotiating the use of CLUE in H.323 systems. An endpoint uses the CLUEControlCapability to indicate and determine which method is used for transporting CLUE messages.

7.2.1.1 H.225 Generic Feature “CLUE control capability”

This clause describes a generic feature “CLUE control capability” utilising the generic extensible framework (GEF) [ITU-T H.460.1] to negotiate the use CLUE protocol messages within H.245.

Table 1 – CLUE control capability

| Feature name | CLUEControlCapability |
| --- | --- |
| Feature Description: | This features indicates the use of CLUE for providing additional information about the advertised media streams. |
| Feature identifier type: | OID |
| Feature identifier value: | {itu-t(0) recommendation(0) h(8) h.tps-sig (?) CLUEControlCapability(1)}. |

Table 2 – CLUE control capability parameters

| Parameter name | CLUE H.245 Transport |
| --- | --- |
| Parameter description: | This parameter indicates that the endpoint supports the H.245 Generic Message Transport method for transporting CLUE messages as defined by clause 7.2.2.2. |
| Parameter identifier type: | OID |
| Parameter identifier value: | 0 |
| Parameter type: | Bool |
| Parameter cardinality: | Once and once only |

| Parameter name | CLUE STCP Transport |
| --- | --- |
| Parameter description: | This parameter indicates that the endpoint supports the SCTP based method for transporting CLUE messages as defined by clause 7.2.2.1. |
| Parameter identifier type: | OID |
| Parameter identifier value: | 1 |
| Parameter type: | Bool |
| Parameter cardinality: | Once and once only |

In order to negotiate the use of CLUE within [ITU-T H.225.0] an indication needs to be transmitted to the remote endpoint. This capability is sent to the remote end via feature negotiation as indicated by clause 7.9/[ITU-T H.323]. The endpoint shall include the CLUE H.245 Transport and CLUE STCP Transport parameters indicating the transport options that are supported.

The remote endpoint upon receiving the CLUEControlCapability then decides whether to support CLUE in its capabilities announcement.

…

7.2.2 CLUE Messaging

This clause describes the methods for the carriage of CLUE messages within a H.323 system.

…

8.2 Call session setup

If two endpoints both support CLUE signalling an exemplary call session setup procedure is shown in the figure below.

Figure 6 – Exemplary CLUE call establishment procedure

Telepresence endpoint A carries the CLUEControlCapability in supportedFeatures field in order to signal that it supports CLUE signalling. It includes the CLUE H.245 Transport and/or CLUE SCTP Transport parameters to indicate which CLUE transports are supported. When telepresence endpoint B receives the setup message it knows the peer supports CLUE signalling.  Therefore, B responds with the Alerting message that also carries the supported feature CLUEControlCapability with the supported transport options.  This informs A that B also supports CLUE signalling. The CLUEControlCapability feature also can be carried in Call proceeding or Connect messages. After the Connect message, a CLUE call is established.

If the support for CLUEControlCapability is not received from B, then either B does not support CLUE, or the GEF component of the message was removed somewhere along the signalling path. In that case, a non-CLUE traditional H.323 call setup occurs as shown in Figure 7.

Figure 7 – Non-CLUE call establishment procedure

However, if the H.245 TCS messages indicate the CLUE is in fact supported by B, then the CLUE protocol can still be used.

8.3 Capability exchange

Capability exchange for telepresence endpoints is based on H.323 capability exchange procedures in clause 8.2/[ITU-T H.323] with the addition of CLUE protocol messaging to provide further information regarding media. The telepresence capability exchange procedures have two parts. The capability exchange related to the establishment of the CLUE protocol and the capability exchange related to the CLUE controlled media.

8.3.1 CLUE protocol capability exchange

Clause 7.2.1 indicates that an endpoint may use a separate SCTP based data connection or use H.245 for the transport of CLUE messages.

If as a result of the procedures in clause 8.2 the initiating H.323 telepresence endpoint decides to use a separate SCTP based data connection, in the initial TerminalCapabilitySet message it should set a bidirectional DataApplicationCapability indicating the use of “CLUE” over a WebRTC Data Channel as defined by Annex U/[ITU-T H.245]. If the peer endpoint is a H.323 telepresence endpoint then it should return a TerminalCapabilitySet message also indicating the support of a data connection for the CLUE protocol. The initiating endpoint shall then establish a WebRTC Data Channel association containing the CLUE protocol as per Annex U/[ITU-T H.245].

If as a result of the procedures the initiating H.323 telepresence endpoint decides to use a H.245 based CLUE transport method no additional SCTP transport data capability is needed.

Once the initiating endpoint is ready to send CLUE messages (either via H.245 transport or data channel transport) it shall initiate messaging according to [IETF CLUE PROT]. The endpoints shall exchange CLUE Option messages and each provide an initial CLUE Advertisement. The CLUE Advertisement messages shall contain encodingIDs related to CLUE controlled media capabilities communicated via H.245 TerminalCapabilitySet messages. This may relate to already communicated capabilities or capabilities yet to be communicated.

…

__________________________

____________________