| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4715 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.TPS-SIG: H.TPS-AV parameter signalling |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This contribution discusses the support of H.TPS-AV parameters in H.TPS-SIG.

2.	Discussion

Since the start of the work on H.TPS-SIG there haves been some questions regarding how parameters defined by H.TPS-AV will be supported by H.TPS-SIG. This has been recorded in two editor’s notes in clauses 6 and 7.1. I.e.:

{Editor’s note: The above statement is a placeholder. There needs to be further investigation how the additional elements from H.TPS-AV are supported.}

And

{Editor’s note: This section shall contain information regarding the carriage of H.TPS-AV parameters in both SIP and H.245-based systems.  This might be accomplished by registering XML parameters defined by Q5/16 with IANA}

Since the addition of the editor’s notes H.TPS-AV has gone some way in answering the question how the various parameters are signalled through the “Signalling Mechanism” field associated with each parameter. H.TPS-AV Appendix I “Parameter signalling analysis” also discusses how each of the parameters are signalled.

Part of the motivation for the inclusion of the Editor’s notes was to discuss the procedure for adding any Q.5/16 defined parameter to CLUE where they weren’t already defined by CLUE. Clause 9/[draft-ietf-clue-protocol-03] defines procedures for registering various CLUE elements that could be utilised by Q5/16 to define addition CLUE attributes if and when they are identified.

Parameters could also be added via other mechanisms e.g. codec related signalling if more appropriate there.

So rather than discussing how each H.TPS-AV parameter is handled in H.TPS-SIG the Contributors propose to reference the work in H.TPS-AV describing signalling mechanisms. In addition the Contributors propose to reference the CLUE extension procedures. As a result the Contributors believe these editor’s notes can be deleted.

3.	Proposal

It is proposed to accept the following changes. The changes are marked up against TD 226R1/WP1 from the Geneva, February Q5/16 Rapporteurs meeting.

Draft new ITU-T H.TPS-SIG

Signalling for telepresence-enabled conferencing

…

2 References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T F.734]	Recommendation ITU-T F.734 (2014), Definitions, Requirements, and use cases for Telepresence Systems.

[ITU-T H.420]	Recommendation ITU-T H.420 (2014), Telepresence System Architecture.

[ITU-T H.TPS-AV]	Recommendation ITU-T H.yyy (date), Audio/Video Parameters for Telepresence systems.

[ITU-T H.323]	Recommendation ITU-T H.323 v7 (12/2009), Packet-based multimedia communications systems.

[ITU-T H.245]	Recommendation ITU-T H.245 v16 (05/2011), Control protocol for multimedia communication.

[ITU-T H.225.0]	Recommendation ITU-T H.225.0 v7 (12/2009), Call signalling protocols and media stream packetization for packet-based multimedia communication systems.

[ITU-T H.460.1]	Recommendation ITU-T H.460.1 (03/2002), Guidelines for the use of the generic extensible framework.

[IETF CLUE DM]	IETF draft-ietf-clue-data-model-schema-09, An XML Schema for the CLUE data model.

[IETF CLUE FW]	IETF draft-ietf-clue-framework-22, Framework for Telepresence Multi-Streams.

[IETF CLUE SIG] 	IETF draft-ietf-clue-signaling-05, CLUE Signaling.

[IETF CLUE DC]		IETF draft-ietf-clue-datachannel-09, CLUE Protocol Data Channel.

[IETF CLUE PROT]		IETF draft-ietf-clue-protocol-03, CLUE protocol.

[IETF RTCWEB DC]		IETF draft-ietf-rtcweb-data-channel-12, WebRTC Data Channels.

[IETF DCEP]		IETF draft-ietf-rtcweb-data-protocol-09, WebRTC Data Channel Establishment Protocol.

…

6 Framework

A telepresence system is characterised by a set of functions, devices and network elements that are able to capture, deliver, manage and render multiple high quality interactive audio and video signals in a Telepresence conference. An appropriate number of devices (e.g. cameras, screens, loudspeakers, microphones, codecs, MCUs, PCs) and environmental characteristics are used to establish Telepresence. Such functions, elements and characteristics are described in [ITU-T H.420]. In order to support a telepresence experience, information related to the functions and characteristics must be negotiated and transmitted between the network elements.

[ITU-T H.TPS-AV] defines telepresence system parameters. These parameter may or may not be signalled. If signalled they may be signalled via various means. Many of the parameters may be signalled through the use of the CLUE framework [IETF CLUE FW]  in order that the network elements may provide a telepresence experience. This Recommendation defines how CLUE signalled is supported by H.323 systems.

Furthermore, additional information is provided by native [ITU-T H.245] signalling that provides further additional enhancements.

A H.323 telepresence session follows the same basic call signalling procedures as outlined in clause 8/[ITU-T H.323] but with some additional behaviour to negotiate the use of CLUE signalling and to link the CLUE information to H.245 capabilities. These aspects are briefly outlined in the clauses below.

…

7.1 Carriage of H.TPS-AV parameters

No additional attributes to those already specified in [IETF CLUE FW] have been identified in [ITU-T H.TPS-AV]. Any future parameters identified by H.TPS-AV that require CLUE signalling shall utilise the CLUE extension procedures defined by clause 9/[IETF CLUE PROT].

…

__________________________

____________________