| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4767 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | China Telecom |  |  |  |
| Title: | Proposal to update clause 6.1 and 6.2 in H.VSMprot |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

The work item H.VSMprot “Signalling and protocols for mobile visual surveillance” was updated in last ITU-T SG16 meeting, the current document updates the signalling and protocols of the mobile visual surveillance.

2.	Discussion

In order to complete the work item: H.VSMprot “Signalling and protocols for mobile visual surveillance”, we need to update the signalling and protocols.

3.	Proposal

It is proposed to discuss and accept the updated 6.1 and 6.2 of H.VSMprot.

[Begin]

Signalling and protocols for mobile visual surveillance

# Signalling and protocols

[NOTE – This clause will provide the identification of signalling or protocols.]

The following is some illustrative items:

- The SIP message format should comply with the definition in RFC3261;

- The RTSP message format should comply with the definition in RFC2326;

- The HTTP message format should comply with HTTP1.1 defined in RFC2616;

- RTP/RTCP signalling and message refers to RFC3550;

- TBD.

## Real-time video/audio acquisition

When a user wants to view the real-time surveillance video/audio by mobile terminal, the M_CU initiates a request to the VAU for real-time video/audio streams after MSP performing authentication and returning VAU information to M_CU.

Figure 6-1: Procedural flows for real-time media acquisition

This figure shows the procedural flows for real-time Media Acquisition for one M_CU:

1)	First, the M_CU initiates a request and creates a connection with the MSP.

2)	The MSP initiates a request and creates a connection with the CMU.

3)	The CMU returns the VAU information.

4)	The MSP gets the request and returns VAU information to M_CU.

5)	When the M_CU gets VAU information, it requests the VAU for the real-time media address with the information about the network type in which the M_CU is located.

6)	VAU receives this request and requests to SCU for the media address. At the same time, VAU queries the corresponding transcoding parameters from SCU.

7)	The SCU transfers this request to the MDU.

8)	After receiving messages, the MDU returns the real-time media address to the SCU.

9)	The SCU returns the real-time media address to the VAU.

10)	After receiving the real-time media address, the VAU requests the MDU to start transmitting media stream.

11)	The MDU sends a request to the PU for initiating a media flow.

12)	The PU creates media channel with the MDU and sends media flow to the MDU.

13)	The MDU transfers the media flow to the VAU. VAU performs transcoding according to the transcoding parameters.

14)	The transcoded media flow is transferred from the VAU to the M_CU. The M_CU decodes and plays the received media flow.

15)	When the M_CU stops the real-time media acquisition it sends a stop-media request to the VAU.

16)	VAU stops transcoding after receives the request. The VAU stops sending media stream to the MDU.

17)	The MDU sends response to the VAU for stopping media flow.

18)	The VAU requests the SCU to disconnect which then returns a response to the VAU.

19)	Then the VAU ends this real-time media acquisition and sends response to the SCU.

## Recording and playback

The playback function is used to play audio and video which has been previously recorded and stored.

#### 6.2.1  Recording and Playback in platform

When users want to playback the surveillance point using mobile phones, they must log into the MSP by the M_CU. The MSP performs authentication and returned the playback surveillance list.

After the users choose one surveillance point name, the M_CU sends a request for this surveillance video to the VAU, and then VAU returns the media stream to the M_CU to play.

Figure 6-2: Procedural flows for playback in platform

This figure shows the procedural flows for playback in platform:

1)	The M_CU sends a request to the MSP for querying recording information.

2)	The MSP sends a request to the CMU for querying recording information.

3)	The CMU sends a request to the MSU for querying recording information.

4)	The MSU returns the querying result to the CMU.

5)	When the CMU gets the result, it transfers the result to the MSP.

6)	When the MSP gets the result, it transfers the result to the M_CU.

7)	The M_CU selects one playback point and starts a request to playback.

8)	The MSP sends a request for the playback address to the CMU.

9)	The request is transferred from the CMU to the SCU.

10)	The request is transferred from the SCU to the MDU.

11)	The MDU responses to the request and sends the playback address back to the SCU.

12)	The playback address is sent from the SCU to the CMU.

13)	The CMU sends the playback address to the MSP.

14)	The playback address is sent from the MSP to the M_CU.

15)	The M_CU starts playback request for the VAU with the playback address and the network type.

16)	The VAU starts playback request for the MSU with the playback address. At the same time, VAU queries the corresponding transcoding parameters from SCU based on the network types.

17)	The MSU returns a response to the VAU and sends the media stream to the VAU.

18)	Once the VAU gets the media stream, it performs transcoding according to the transcoding parameters and then transfers the transformed stream to the M_CU.

19)	The M_CU returns a response to the VAU and sends a stop playback request. VAU stops transcoding after receiving the request.

20)	The VAU sends a stop playback request to the MSU.

21)	In response, the MSU stops sending the stream.

22)	At the same time, the MSU notifies the MDU to stop playing.

23)	The MDU returns a response to the MSU.

#### 6.2.2	Recording and Playback in PU

Figure 6-3: Procedural flows for playback in PU

This figure shows the procedural flows for playback in PU:

1)	The M_CU sends a request to the MSP for querying recording information.

2)	The MSP sends a request to the CMU for querying recording information.

3)	The CMU sends a request to the PU for querying recording information.

4)	The PU returns the querying result to the CMU.

5)	When the CMU gets the result, it transfers the result to the MSP.

6)	When the MSP gets the result, it transfers the result to the M_CU.

7)	The M_CU selects one playback point and starts a request to playback.

8)	The MSP sends a request for the playback address to the CMU.

9)	The request is transferred from the CMU to the SCU.

10)	The request is transferred from the SCU to the MDU.

11)	The MDU responses to the request and sends the playback address back to the SCU.

12)	The playback address is sent from the SCU to the CMU.

13)	The CMU sends the playback address to the MSP.

14)	The playback address is sent from the MSP to the M_CU.

15)	The M_CU starts playback request for the VAU with the playback address and network type.

16)	The VAU starts playback request for the PU with the playback address. At the same time, VAU queries the corresponding transcoding parameters from SCU based on the network types.

17)	The PU returns a response to the VAU and sends the media stream to the VAU.

18)	Once the VAU gets the media stream, it performs transcoding based on the transcoding parameters and then transfers the transformed stream to the M_CU.

19)	The M_CU returns a response to the VAU and sends a stop playback request. VAU stops transcoding after receiving the request.

20)	The VAU sends a stop playback request to the PU.

21)	In response, the PU stops sending stream.

22)	At the same time, the PU notifies the MDU to stop playing.

23)	The MDU returns a response to the PU.

Bibliography

[b-ITU-T H.Sup.1]	ITU-T H-series Recommendations – Supplement 1 (1999), Application profile – Sign language and lip-reading real-time conversation using low bit rate video communication.

[End]

__________________