| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 427 (PLEN/16) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Geneva, 12-23 October 2015 |
| TD |  |  |
| Source: | Editor H.248.94 |  |
| Title: | Consent: H.248.94 (ex H.248.WEBRTC) "Gateway control protocol: Web-based real-time communication services - H.248 protocol support and profile guidelines" (New) |  |

This document contains draft new H.248.94 (ex H.248.WEBRTC) "Gateway control protocol: Web-based real-time communication services - H.248 protocol support and profile guidelines", which is proposed for Consent at this SG16 meeting.

AAP Summary

Recommendation ITU-T H.248.94 provides guidelines on the use and configuration of ITU-T H.248 WebRTC gateways via ITU-T H.248 profiles. These guidelines may be used by other standards developing organizations (SDOs) when defining their ITU-T H.248.1 profiles in support of WebRTC gateways.

CONTENTS

Page

1 Scope 8 1.1 Applicability statements 8 1.1.1 Release 1 8 2 References 9 3 Definitions 11 3.1 Terms defined elsewhere 11 3.2 Terms defined in this Recommendation 12 4 Abbreviations and acronyms 12 5 Conventions 14 5.1 Acronym usage 14 5.2 Prescriptive language 14 5.3 ITU-T H.248 protocol element notation 14 6 WEBRTC core technology overview 15 6.1 WEBRTC gateways 15 6.2 Capabilities 15 6.3 Transport of user plane WebRTC traffic 16 6.3.1 Objective 16 6.3.2 Protocol stack for UDP-based transport 16 6.3.3 Protocol stack for TCP-based transport 17 7 H.248 WEBRTC gateways: example use cases 17 7.1 Point-to-point communication 17 7.1.1 Use case #1: WebRTC-to-WebRTC interworking 17 7.1.2 Use case #2: WebRTC-to-NGN/IMS interworking 18 7.1.3 Use case #3: WebRTC-to-PSTN/ISDN interworking 18 7.2 Multipoint communication 19 8 Functional requirements for H.248 WEBRTC gateways 19 8.1 Requirements related to NAT traversal support 19 8.1.1 Media latching based NAT traversal support 19 8.1.2 ICE based NAT traversal support 19 8.2 Requirements related to communication topologies 20 8.2.1 H.248 WEBRTC gateway for point-to-point interworking 20 8.2.2 H.248 WEBRTC gateway as WEBRTC conferencing point 20 8.3 Requirements related to bearer traffic multiplexing 20 8.4 Requirements related to RTP-to-RTP type interworking 20 8.4.1 RTCP control flow component 20 8.4.2 RTP profiles 21 8.4.3 RTP multiplexing 21 8.4.4 RTP topologies 21 8.4.5 RTCP based services, report types and packet formats 22 8.4.6 Resource allocation rules for RTP/RTCP 22 8.5 Requirements related to TCP-based media transport 22 8.5.1 Requirements related to TCP connection establishment 22 8.5.2 Requirements related to connectivity checks of TCP connection candidates 22 8.5.3 Requirements related to TCP connection release 22 8.5.4 Requirements related to application level framing protocol 23 8.5.5 Requirements related to MG-internal interworking of TCP traffic 23 8.6 Requirements related to media-aware type of interworking 24 8.6.1 Requirements related to WEBRTC audio 24 8.6.2 Requirements related to WEBRTC video 24 8.6.3 Requirements related to WEBRTC data 25 8.6.4 Requirements related to WEBRTC text 25 8.7 Requirements related to bearer security 26 8.7.1 Requirements related to transport security 26 8.7.2 Requirements related to media security 26 8.8 Requirements related to WEBRTC emergency and priority services 27 8.8.1 Emergency services within WebRTC calls 27 8.8.2 Priority services within WebRTC calls 27 8.9 Requirements related to QoS support 27 8.9.1 IP traffic control: QoS marking 27 8.9.2 IP traffic control: traffic policing – IP byte rate policing 27 8.9.3 Improved transport robustness for RTP 27 8.10 Requirements related to bearer-path coupled congestion controls 28 8.11 Requirements related to performance monitoring 28 8.11.1 Requirements related to measurement point (MP) 28 8.11.2 Requirements related to collection point (CP) 29 8.11.3 Requirements related to reporting point (RP) 29 8.11.4 Requirements related to filtering point (FP) 29 8.11.5 Requirements related to loopback point (LP) 30 8.12 Requirements related to SDP-based description, declaration and negotiation of WEBRTC media configurations 30 8.12.1 Requirements related to supported SDP elements – Overall media descriptions 30 8.12.2 Requirements related to supported SDP elements – Codec configurations 33 8.12.3 Requirements related to SDP Offer/Answer 36 8.13 Requirements related to H.248 signalling 36 8.13.1 Indication of H.248 "WebRTC" Terminations 36 9 Data Channel Establishment Protocol Support Package 36 9.1 Properties 36 9.2 Events 37 9.2.1 Detect DCEP Messages 37 9.3 Signals 40 9.3.1 Send DCEP Open 40 9.3.2 DCEP Open Response 41 9.4 Statistics 42 9.5 Error Codes 42 9.6 Procedures 42 9.6.1 Auditing 42 9.6.2 Pre-conditions 42 9.6.3 Opening a data channel based on a remote request 42 9.6.4 Local Request to open a data channel 45 9.6.5 Data channel closure 46 9.6.6 Termination of DCEP messages 46 10 Out of band WebRTC data channel negotiation 47 11 H.248 profile specification guidelines 47 11.1 Profile Identification 47 11.2 Summary 47 11.3 Gateway Control Protocol Version 47 11.4 Connection Model 47 11.5 Context Attributes 47 11.6 Terminations 48 11.7 Descriptors 48 11.7.1 TerminationState Descriptor 48 11.7.2 Stream Descriptor 48 11.7.3 Events Descriptor 48 11.7.4 EventBuffer Descriptor 49 11.7.5 Signals Descriptor 49 11.7.6 DigitMap Descriptor 51 11.7.7 Statistics Descriptor 51 11.7.8 ObservedEvents Descriptor 51 11.7.9 Topology Descriptor 51 11.7.10 Error Descriptor 51 11.8 Command API 51 11.8.1 Add 52 11.8.2 Modify 52 11.8.3 Subtract 52 11.8.4 Move 52 11.8.5 AuditValue 52 11.8.6 AuditCapabilities 52 11.8.7 Notify 52 11.8.8 ServiceChange 52 11.8.9 Manipulating and auditing context attributes 52 11.9 Generic command syntax and encoding 52 11.10 Transactions 52 11.11 Messages 52 11.12 Transport 52 11.13 Security 52 11.14 Packages 53 11.14.1 Mandatory Packages 53 11.14.2 Optional Packages 53 11.14.3 Package usage information 55 11.15 Mandatory support of SDP and ITU-T H.248.1 Annex C information elements 72 11.16 Optional support of SDP and ITU-T H.248.1 Annex C information elements 73 11.17 Procedures 75 11.17.1 Package-independent NAT-T procedures 75 11.17.2 Example procedures for communication establishment phase 75 11.17.3 Example procedures for release of data channels and call release 86 Appendix I Use case specific capability sets – Some examples 95 I.1 Overview 95 Appendix II Distributed Text-over-IP endpoints for WebRTC data 'text' 97 II.1 Purpose 97 II.2 Problem statement 97 II.3 Solution – Guidelines for the WebRTC media gateway (T.140)-IWF 102 Bibliography 102

List of Tables

Page

Table 1 – Requirements related to supported SDP elements – Part I SDP "m="-line 30 Table 2 – Requirements related to supported SDP elements – Part II SDP "a="-line 31 Table 3 – Requirements related to supported SDP elements – Part III other SDP line s 32 Table 4 – SDP parameters for IANA registered media type "audio/opus" 34 Table 5 – Remote open request example step 1 43 Table 6 – Remote open request example step 2 44 Table 7 – Remote open request example step 3 44 Table 8 – Local open request example step 1 45 Table 9 – Local open request example step 2 46 Table 10 – Use case #1 and variations 75 Table 11 – Use case #2 and variations 78 Table 12 – Use case #3 and variations 80 Table 13 – Use case #4 and variations 83 Table 14 – Use case #7 and variations 87 Table 15 – Use case #8 and variations 91 Table 16 – Use case #9 and variations 93 Table I.1 – Examples of use case specific capability sets (CS) 95

List of Figures

Page

Figure 1 – User plane WebRTC protocol stack in case of UDP- and TCP-based transport 16 Figure 2 – Use case #1: WebRTC-to-WebRTC interworking 17 Figure 3 – Use case #2: WebRTC-to-NGN/IMS interworking 18 Figure 4 – Use case #3: WebRTC-to-PSTN/ISDN interworking 18 Figure 5 – H.248 WebRTC gateway – H.248 Context model – Use case #1: audio & video only, unbundled audio & video 76 Figure 6 – Example signalling flow for use case #1.0 77 Figure 7 – H.248 WebRTC gateway – H.248 Context model – Use case #2: audio & video only, bundled audio & video 79 Figure 8 – H.248 WebRTC gateway – H.248 Context model – Use case #3.0: additional data, preparation of transport 81 Figure 9 – Example signalling flow for use case #3.0 82 Figure 10 – H.248 WebRTC gateway – H.248 Context model – Use case #4.0: first data channel in service 84 Figure 11 – Example signalling flow for use case #4.0 85 Figure 12 – H.248 WebRTC gateway – H.248 Context model – Use case #4.2: three data channels in service 86 Figure 13 – H.248 WebRTC gateway – H.248 Context model – Use case #7.0: release of a data channel without call release 89 Figure 14 – Example signalling flow for use case #7.0 90 Figure 15 – H.248 WebRTC gateway – H.248 Context model – Use case #8.0: SCTP Association shutdown without DTLS connection release 92 Figure 16 – Example signalling flow for use case #8.0 93 Figure 17 – Example signalling flow for use case #9.0 94 Figure II.1 – Starting point: legacy ToIP in all-IP networks 97 Figure II.2 – peer-to-peer WebRTC: options for WebRTC realtime text transport 98 Figure II.3 – WebRTC gateway scenario – protocol stack model in user plane 99 Figure II.4 – WebRTC gateway scenario – Network engineering aspects 100 Figure II.5 – WebRTC gateway scenario – Functional processing stages of the distributed "T.140-over-RTP endpoint" in WebRTC client to non-WebRTC UE direction 101

Draft new Recommendation ITU-T H.248.94 (ex H.248.WEBRTC)

Gateway control protocol: Web-based real-time communication services - H.248 protocol support and profile guidelines

Summary

Recommendation ITU-T H.248.94 provides guidelines on the use and configuration of ITU-T H.248 WebRTC gateways via ITU-T H.248 profiles. These guidelines may be used by other standards developing organizations (SDOs) when defining their ITU-T H.248.1 profiles in support of WebRTC gateways. WebRTC represents an extensive realtime multimedia conversational service with a specific protocol stack in order to address NAT traversal as well as maximize multiplexing support. The WebRTC gateway consequently requires the support of user plane interworking functions for connecting WebRTC clients to non-WebRTC networks. The Recommendation also defines a new package to support the Data Channel Establishment Protocol (DCEP).

# 1	Scope

Web-based real-time communication is a service standardized by the IETF and W3C (called WEBRTC or RTCWEB service) and defines a particular protocol suite for IP-based communication in web browser environments. This application is related to multimedia conversational services which cover typical service components such as telephony, conferencing, instant messaging, etc. The native IETF / W3C WEBRTC service (as defined for the public Internet by the IETF) will be embedded in other IP communication infrastructures such as NGN/IMS.

This Recommendation

- describes example use cases with H.248 WEBRTC gateway involvement;

- identifies H.248 capabilities for such gateways;

- introduces a new H.248 package for the support of WebRTC data channel protocol procedures; and

- provides guidelines for the specification of H.248 profiles for dedicated H.248 WEBRTC gateway types.

It is expected that the WebRTC communication service will evolve, be enhanced and/or extended in the future. The next sub-clause provides information about the specific scope of the current Release of this Recommendation.

## 1.1	Applicability statements

### 1.1.1	Release 1

Release 1 provides a basic WebRTC service, but has some limitations due to dependencies on other standards, which were still in progress.

Release 1 supports:

- WebRTC calls with audio and video, the two WebRTC media components are either unbundled or bundled, i.e., using H.248 stream grouping or not.

- WebRTC calls with additional data, but limited to WebRTC-embedded instant messaging (based on MSRP) only.

Release 1 does not yet support:

- WebRTC calls with data applications related to WebRTC conferencing control (based on BFCP) and WebRTC text conversation (based on T.140).

- Full support of DTLS for WebRTC data: basic establishment and release procedures are supported via H.248. However, the ability to influence the DTLS negotiation process and other DTLS capabilities is not supported.

- ICE-based NAT traversal: current H.248 WebRTC gateway capabilities support an asymmetric network access model. Additional extensions cannot be excluded for other network configurations, e.g. additional ICE aspects.

- SRTP key management schemes: the single "DTLS-SRTP"-related key management schemes, as subject of WebRTC, is implicity supported. Additional SRTP key management schemes demand the explicit support of revised [ITU-T H.248.77].

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.37]	Recommendation ITU-T H.248.37 (06/2008), Gateway control protocol: IP NAPT traversal package.

[ITU-T H.248.39]	Recommendation ITU-T H.248.39 (07/2014), Gateway control protocol: H.248 SDP parameter identification and wildcarding.

[ITU-T H.248.48]	Recommendation ITU-T H.248.48 (02/2012), Gateway control protocol: RTCP XR block reporting package.

[ITU-T H.248.50]	Recommendation ITU-T H.248.50 (09/2010), Gateway control protocol: NAT traversal toolkit packages.

[ITU-T H.248.52]	Recommendation ITU-T H.248.52 (06/2008), Gateway control protocol: QoS support packages.

[ITU-T H.248.53]	Recommendation ITU-T H.248.53 (03/2009), Gateway control protocol: Traffic management packages.

[ITU-T H.248.57]	Recommendation ITU-T H.248.57 (07/2014), Gateway control protocol: RTP control protocol package.

[ITU-T H.248.71]	Recommendation ITU-T H.248.71 (02/2010), Gateway control protocol: RTCP support packages.

[ITU-T H.248.77]	Recommendation ITU-T H.248.77 (09/2010), Gateway control protocol: Secure real-time transport protocol (SRTP) package and procedures.

[ITU-T H.248.78]	Recommendation ITU-T H.248.78 (03/2013), Gateway control protocol: Bearer-level backhauling and application level gateway.

[ITU-T H.248.80]	Recommendation ITU-T H.248.80 (01/2014), Gateway control protocol: Usage of the revised SDP offer/answer model with ITU-T H.248.

[ITU-T H.248.84]	Recommendation ITU-T H.248.84 (07/2012), Gateway control protocol: NAT traversal for peer-to-peer services.

[ITU-T H.248.87]	Recommendation ITU-T H.248.87 (01/2014), Gateway control protocol: Guidelines on the use of ITU-T H.248 capabilities for performance monitoring in RTP networks in ITU-T H.248 profiles.

[ITU-T H.248.88]	Recommendation ITU-T H.248.88 (01/2014), Gateway control protocol: RTP topology dependent RTCP handling by ITU-T H.248 media gateways with IP terminations.

[ITU-T H.248.89]	Recommendation ITU-T H.248.89 (10/2014), Gateway control protocol: TCP support packages.

[ITU-T H.248.90]	Recommendation ITU-T H.248.90 (10/2014), Gateway control protocol: ITU-T H.248 packages for control of transport security using transport layer security (TLS).

[ITU-T H.248.92]	Recommendation ITU-T H.248.92 (07/2014), Gateway control protocol: Stream endpoint interlinkage package.

[ITU-T H.248.93]	Recommendation ITU-T H.248.93 (07/2014), Gateway control protocol: H.248 support for control of transport security using DTLS.

[ITU-T H.248.96]	Recommendation ITU-T H.248.96 (10/2015), Gateway control protocol: H.248 Stream grouping and aggregation.

[ITU-T H.248.97]	Recommendation ITU-T H.248.97 (10/2015), Gateway control protocol: H.248 support for control of SCTP bearer connections.

[ITU-T H.320]	Recommendation ITU-T H.320 (03/2004), Narrow-band visual telephone systems and terminal equipment.

[ITU-T H.324]	Recommendation ITU-T H.324 (04/2009), Terminal for low bit-rate multimedia communication.

[ITU-T H.351]	Recommendation ITU-T H.351 (06/2008), Semantic web interface for multimedia terminal and system directories (SWIM-D).

[ITU-T T.140]	Recommendation ITU-T T.140 (02/1998), Protocol for multimedia application text conversation.

[IETF RFC 4103]	IETF RFC 4103 (06/2005), RTP Payload for Text Conversation.

[IETF RFC 4571]	IETF RFC 4571 (07/2006), Framing Real-time Transport Protocol (RTP) and RTP Control Protocol (RTCP) Packets over Connection-Oriented Transport.

[IETF RFC 4585]	IETF RFC 4585 (2006), Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF).

[IETF RFC 4961]	IETF RFC 4961 (2007), Symmetric RTP / RTP Control Protocol (RTCP).

[IETF RFC 5104]	IETF RFC 5104 (2008), Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF).

[IETF RFC 5109]	IETF RFC 5109 (2007), RTP Payload Format for Generic Forward Error Correction.

[IETF RFC 5124]	IETF RFC 5124 (2008), Extended Secure RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/SAVPF).

[IETF RFC 5506]	IETF RFC 5506 (2009), Support for Reduced-Size Real-Time Transport Control Protocol (RTCP): Opportunities and Consequences.

[IETF RFC 5576]	IETF RFC 5576 (2009), Source-Specific Media Attributes in the Session Description Protocol (SDP).

[IETF RFC 5764]	IETF RFC 5764 (2010), Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP).

[IETF RFC 7007]	IETF RFC 7007 (2013), Update to Remove DVI4 from the Recommended Codecs for the RTP Profile for Audio and Video Conferences with Minimal Control (RTP/AVP).

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation refers to following document related to all terms used in context of the DTLS protocol: [b-IETF tls-terms].

This Recommendation also uses the following terms defined elsewhere:

3.1.1	media path [b-IETF rtcweb-overview]: The path that media data follows from one WebRTC endpoint to another.

NOTE – Relates to the bearer plane interface (at MG) in case of a WEBRTC H.248 gateway.

3.1.2	signalling path [b-IETF rtcweb-overview]: The communication channels used between entities participating in signaling to transfer signaling. There may be more entities in the signaling path than in the media path.

NOTE – Relates to the signalling plane call control interface (at MGC) in case of a WEBRTC H.248 gateway.

3.1.3	web browser [ITU-T H.351]: A software application capable of rendering HTML and XHTML documents.

3.1.4	WebRTC browser [b-IETF rtcweb-overview]: (also called a WebRTC User Agent or WebRTC UA) is something that conforms to both the protocol specification and the Javascript API defined 'above' (NOTE).

NOTE – See [b-IETF rtcweb-overview] concerning the referred to "Javascript API specification".

3.1.5	WebRTC endpoint [b-IETF rtcweb-overview]: is either a WebRTC browser or a WebRTC non-browser.  It conforms to the protocol specification.

3.1.6	WebRTC-compatible endpoint [b-IETF rtcweb-overview]: is an endpoint (NOTE) that is able to successfully communicate with a WebRTC endpoint, but may fail to meet some requirements of a WebRTC endpoint.  This may limit where in the network such an endpoint can be attached, or may limit the security guarantees that it offers to others.  It is not constrained by this specification; when it is mentioned at all, it is to note the implications on WebRTC-compatible endpoints of the requirements placed on WebRTC endpoints.

NOTE – The self-contained notion of 'endpoint' is consistent with [b-ITU-T H.Sup.13].

3.1.7	WebRTC gateway [b-IETF rtcweb-overview]: is a WebRTC-compatible endpoint that mediates media traffic to non-WebRTC entities.

3.1.8	WebRTC non-browser [b-IETF rtcweb-overview]: is something that conforms to the protocol specification, but does not claim to implement the Javascript API. This can also be called a "WebRTC device" or "WebRTC native application".

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	WEBRTC H.248 gateway: a decomposed gateway according H.248 (i.e., a MGC-MG tandem), using an H.248 profile with support of web-based real-time communication services.

NOTE 1 – The WEBRTC H.248 gateway could operate in WebRTC gateway mode and in WebRTC endpoint mode.

NOTE 2 – The particular (H.248) gateway type may be further qualified, - based on the application specific profile -, such as WEBRTC PSTN gateway, WEBRTC border gateway, WEBRTC IMS access gateway, etc.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| ALG | Application Layer Gateway |
| --- | --- |
| AN | Access Network |
| API | Application Programming Interface |
| B2BRE | Back-to-Back RTP End System |
| B2BTE | Back-to-Back TCP Endpoint |
| B2BUA | Back-to-Back User Agent |
| BFCP | Binary Floor Control Protocol |
| CNAME | Canonical Name |
| CS | Capability Set |
| DC | Data Channel |
| DCEP | Data Channel Establishment Protocol |
| DTLS | Datagram Transport Layer Security |
| DTMF | Dual Tone Multi Frequency |
| FB | Feed Back |
| FEC | Forward Error Correction |
| FW | Fire Wall |
| GoS | Grade of Service |
| HTML | HyperText Markup Language |
| ICE | Interactive Connectivity Establishment |
| IMS | IP multimedia subsystem |
| IP | Internet protocol |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |
| Lx | Layer number x |
| MG | Media gateway |
| MGC | Media gateway controller |
| MSRP | Message Session Relay Protocol |
| MTI | Mandatory To Implement |
| NACK | (RTP/RTCP) Negative Acknowledgements |
| NAT | Network address translation |
| NAT-T | NAT Traversal |
| NGN | Next generation network |
| PCM | Pulse Code Modulated |
| PPID | (SCTP) Payload Protocol Identifier |
| PSFB | (RTCP) Payload-Specific Feedback |
| PSTN | Public Switched Telephone Network |
| QoE | Quality of Experience |
| QoS | Quality of Service |
| RMCAT | RTP Media Congestion Avoidance Techniques (IETF WG) |
| RR | (RTCP) Receiver Report |
| RTCP | RTP Control Protocol |
| RTCWEB | Real-Time Communication in WEB-browsers (as work item in IETF) |
| RTP | Real-time Transport Protocol |
| RTPFB | RTCP Transport layer Feedback (message) |
| SCTP | Stream Control Transmission Protocol |
| SDES | Source Description (RTCP Packet) |
| SDP | Session description protocol |
| SEP | (H.248) Stream endpoint |
| SIP | Session initiation protocol |
| SSRC | Synchronization Source |
| SR | (RTCP) Sender Report |
| SRTP | Secure RTP |
| STUN | Session Traversal Utilities for NAT |
| TCP | Transmission control protocol |
| TLS | Transport Layer Security |
| TURN | Traversal Using Relays around NAT |
| UA | User Agent |
| UDP | User datagram protocol |
| VP9 | (abbreviation unknown) |
| W3C | World Wide Web Consortium (http://www.w3.org/) |
| WEB | "WWW” (World Wide Web) |
| WEBRTC | Web Real-Time Communication = Real-Time Communication in WEB-browsers (as work item in IETF) |
| XHTML | Extensible HyperText Markup Language |
| XR | (RTCP) Extension Report |

# 5	Conventions

## 5.1	Acronym usage

The two acronyms RTCWEB and WEBRTC denote the works to support real-time communication in Web-browsers in the two different SDOs IETF and W3C. RTCWEB relates to a "protocol specification" (note: it is actually a suite of protocols) while WebRTC provides an API specification. Both are synonym from the perspective of H.248 entities in their role as "WebRTC-compatible endpoint" or "WebRTC endpoint". This Recommendation uses the acronym WEBRTC only for the underlying communication service.

## 5.2	Prescriptive language

This document provides a list of items, labelled as R-x/y, where x refers to the clause number and y a number within that clause. Such items use the following keywords with meanings as prescribed below:

The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

The keywords "is prohibited from" indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required.  Thus this requirement need not be present to claim conformance.

The keywords "can optionally" indicate an optional requirement which is permissible, without implying any sense of being recommended. This term is not intended to imply that the vendor’s implementation must provide the option and the feature can be optionally enabled by the network operator/service provider.  Rather, it means the vendor may optionally provide the feature and still claim conformance with the specification.

## 5.3	ITU-T H.248 protocol element notation

Elements of the ITU-T H.248 protocol model, e.g., Context, Termination, Stream, Event are represented using the first letter capitalized. Property, Event, Signal and Parameter identities are given in italics.

The suffix ".req" added to an ITU-T H.248 command name stands for a command request, while the suffix ".rep" stands for a command reply. For example "Notify.req" represents a Notify Request.

# 6	WEBRTC core technology overview

## 6.1	WEBRTC gateways

The WEBRTC communication model considers usage of WEBRTC gateways besides the native browser-to-browser scenarios. [b-IETF rtcweb-gateway] describes such gateways in general, whereas the specific instance of an H.248 WEBRTC gateway is in scope of this Recommendation.

## 6.2	Capabilities

The H.248 WEBRTC gateway is a peer to WEBRTC endpoints, given by IP host entities with "WEBRTC capable web browsers" as the application instance. WEBRTC endpoints are characterized by following mandatory (and optional) capabilities (according to the "RTCWEB protocol specification" [b-IETF rtcweb-overview]):

- WEBRTC as multimedia conversational communication service with application components audio, video and data (which covers again a bunch of "data services");

- Application data transport (see [b-IETF rtcweb-transports]):

  - audio, video: RTP/RTCP in specific capability sets (see [b-IETF rtp-usage])

  - data: SCTP/DTLS in specific capability sets

- Application data framing and securing (see [b-IETF rtcweb-security-arch], [b-IETF rtcweb-secur]):

  - audio, video: SRTP

  - data: (SCTP over) DTLS

- Application data formats:

  - audio codec types (see [b-IETF rtcweb-audio]): Opus [b-IETF RFC 6716] and G.711 PCM µ-/A-law [b-ITU-T G.711]

  - video codec types: H.264 [b-ITU-T H.264], VP8 [b-IETF RFC 6386] (and their successor technologies H.265 [b-ITU-T H.265] and VP9 probably in future)

  - real-time text conversation (based on [ITU-T T.140])

  - instant messaging (using MSRP session-mode)

  - image sharing (e.g., data transfer application with binary encoded data)

  - conference / floor control (based BFCP)

- WebRTC data channel services:

  - two data services are under consideration (see [b-IETF WebRTCDC]):

    - unreliable data  (non-critical information includes state information)

    - reliable data  (both real-time and non-realtime data)

- NAT traversal techniques for end-to-end IP media path connectivity

  - ICE for UDP-based media transport

  - ICE for alternative TCP-based media transport

  - Latching (in case of hosted NAT traversal support [b-IETF RFC 7362])

- Quality of service

- Performance monitoring [b-IETF rtcweb-xr]

NOTE – The capabilities listed without any explicit reference are given by the conception of the WebRTC service as well as underlying network architectural assumptions.

## 6.3	Transport of user plane WebRTC traffic

### 6.3.1	Objective

There is the basic assumption that a WebRTC endpoint could be located behind a NAT/FW device, and thus needs NAT/FW traversal in order to have end-to-end connectivity at IP network and transport layer. This objective results in mainly in two design aspects:

- usage of multiplexing methods in order to minimize the number of required IP transport connections; and

- usage of TCP-based transport as a last resort when the preferred option of UDP-based transport is blocked due to NAT/FW behaviour.

Thus, there are two protocol stacks of consideration from H.248 WEBRTC gateway perspective.

### 6.3.2	Protocol stack for UDP-based transport

See Figure 1:

Figure 1 – User plane WebRTC protocol stack in case of UDP- and TCP-based transport

Notably, there is

- a single L4 port used (Figure 1) in case of multiplexing;

- SCTP/DTLS is used for the transport of text conversation according to [ITU-T T.140] (instead of the native RTP-based transport according to [IETF RFC 4103]

### 6.3.3	Protocol stack for TCP-based transport

See also Figure 1:

Notably, there is

- DTLS-over-TCP used ("despite the fact of TLS-over-TCP as native transport security protocol"), and

- SRTP used for securing RTP.

- SRTP key exchange using DTLS with [IETF RFC 4571]-based framing.

Hence, there is a single DTLS connection again as in case of UDP-based transport. Whether the underlying DTLS session is of type resumable or not is not specified.

# 7	H.248 WEBRTC gateways: example use cases

Some example use cases are described in this clause.

## 7.1	Point-to-point communication

All examples here are characterized by a two-party call (between A and B) and that party A always uses a WebRTC browser environment.

### 7.1.1	Use case #1: WebRTC-to-WebRTC interworking

See Figure 2.

Figure 2 – Use case #1: WebRTC-to-WebRTC interworking

Use case #1 relates to a browser-to-browser scenario: the WEBRTC H.248 gateway may be needed e.g. due to different NAT-T requirements in the two networks where the WebRTC endpoints reside, due to codec mismatch between both WebRTC endpoints, due to different QoS architecture of the networks where the WebRTC endpoints reside, etc.

### 7.1.2	Use case #2: WebRTC-to-NGN/IMS interworking

Figure 3 depicts an interworking scenario with NGN/IMS networks.

Figure 3 – Use case #2: WebRTC-to-NGN/IMS interworking

Use case #2 is fundamentally motivated by the fact that NGN/IMS IP user equipment is supposed not to be fully identical to WebRTC endpoints (in terms of capabilities as listed in clause 8).

### 7.1.3	Use case #3: WebRTC-to-PSTN/ISDN interworking

Figure 4 outlines an interworking scenario to existing PSTN/ISDN infrastructures.

Figure 4 – Use case #3: WebRTC-to-PSTN/ISDN interworking

Use case #3 provides again many variants, such as

- the down-negotiation of an initial multimedia call request to an audio-only call,

- the transcoding between different audio codecs,

- the support of PSTN/ISDN multimedia terminals (e.g. [ITU-T H.324], [ITU-T H.320]).

## 7.2	Multipoint communication

Multiparty calls are basically in scope of WEBRTC, leading to the usual realtime conferencing support from the network side. For instance, WebRTC components audio and video may demand "RTP mixer" functionality, as e.g. typically provided by media servers. Such a network scenario may therefore relate to H.248 profile types concerning media servers (aka media resource processors).

The usual connection model of media servers for multiparty call types is given by "back-to-back endpoint" structures, i.e., there are multiple interconnected WebRTC client instances as part of such an H.248 Context in a media server.

# 8	Functional requirements for H.248 WEBRTC gateways

There are general and use case specific functional requirements for H.248 WEBRTC gateways. WEBRTC gateway requirements could be classified in subsequent functional areas.

## 8.1	Requirements related to NAT traversal support

### 8.1.1	Media latching based NAT traversal support

R-8.1.1/1: The H.248 WEBRTC gateway is required to support media latching according to the H.248 IP NAPT traversal package [ITU-T H.248.37].

NOTE – Latching as hosted NAT traversal mechanism (see [b-IETF RFC 7362]) translates to [ITU-T H.248.37] when provided by a decomposed H.248 gateway.

R-8.1.1/2: The H.248 WEBRTC gateway is recommended to support the media latching associated address reporting capability according to the H.248 address reporting package [ITU-T H.248.37].

NOTE – This is an optional capability and not mandatory for the basic WebRTC call establishment. The MGC might use this capability as complementary information, e.g. as part of an overall service concerning the identification of NAT behavioural types.

### 8.1.2	ICE based NAT traversal support

IP transport protocols:

R-8.1.2/1: The H.248 WEBRTC gateway is required to support ICE for UDP connections, according to [ITU-T H.248.50].

R-8.1.2/2: The H.248 WEBRTC gateway is required to support ICE for TCP connections, according to [ITU-T H.248.50].

The following [ITU-T H.248.50] building blocks are behind the two L4 protocol type related requirements at minimum:

- support of package-independent procedures (clause 10/[ITU-T H.248.50]); and

- STUN support profile given by the three H.248 packages "MG Act-as STUN server", "Originate STUN continuity check" and "STUN consent freshness".

ICE mode:

R-8.1.2/3: The H.248 WEBRTC gateway is required to support basically ICE-full mode (see clause 3.4/[b-IETF rtcweb-transports]), according to [ITU-T H.248.50].

R-8.1.2/4: The H.248 WEBRTC gateway is required to support ICE-lite mode only (according to [ITU-T H.248.50]), dependent on the location of the H.248 WEBRTC gateway in the end-to-end path and/or the "NAT traversal" relevant network architecture.

Trickle ICE:

R-8.1.2/5: The H.248 WEBRTC gateway is required to support the ICE extensions "trickle ICE" (see [b-IETF trickle-ice]), according to [ITU-T H.248.50].

IPv4/IPv6 dual stack fairness and multihomed extension for ICE:

R-8.1.2/5: The H.248 WEBRTC gateway can optionally support the ICE extensions "IPv4/IPv6 dual stack fairness" (see [b-IETF ice-dualstack]), according to [ITU-T H.248.50].

NOTE – The capability is optional for the basic WebRTC call establishment. It is rather a network operator policy concerning IP version preferences.

Additional support of TURN server functionality:

A WebRTC access domain could require additional TURN functionality on top of STUN support. There is then the basic assumption that the IP media path will still be routed via the H.248 WebRTC media gateway, which implies that the H.248 MG could provide an embedded TURN server function. There seems to be no need for an additional TURN server in the media path.

R-8.1.2/6: The H.248 WEBRTC gateway can optionally provide TURN support according to [ITU-T H.248.50].

NOTE – The specific TURN related capabilities are for further studies.

## 8.2	Requirements related to communication topologies

### 8.2.1	H.248 WEBRTC gateway for point-to-point interworking

R-8.2.1/1: The H.248 WEBRTC gateway is required to support an H.248 (IP, IP) or (IP, non-IP connection models (use cases #1, 2 and #3).

### 8.2.2	H.248 WEBRTC gateway as WEBRTC conferencing point

R-8.2.2/1: The H.248 WEBRTC gateway is required to support the conferencing extensions for RTP media as indicated in clause 5.1/[b-IETF rtp-usage].

## 8.3	Requirements related to bearer traffic multiplexing

R-8.3.1/1: The H.248 WEBRTC gateway is required to support at least one UDP- or TCP-based transport connection for a single, multiplexed WEBRTC multimedia call.

NOTE – This is the bearer configuration with a maximum degree of multiplexing. All audio and video traffic are aggregated by usage of RTP transport and RTP media multiplexing. All data traffic is aggregated via a single SCTP Association which is encrypted via DTLS. All three "middle stack" traffic components based on SRTP/SRTCP, DTLS and STUN share a single L4 transport connection.

## 8.4	Requirements related to RTP-to-RTP type interworking

### 8.4.1	RTCP control flow component

R-8.4.1/1: The H.248 WEBRTC gateway is required to always support an RTCP control flow in addition to the RTP media flow (see clause 4.1/[b-IETF rtp-usage]).

Support of RTCP (i.e., the handling and processing of specific RTCP packet types) is dependent on the RTP topology used for the H.248 WEBRTC gateway (see clause 8.4.4). The following requirements are applicable for the example of the RTP topology "Back-to-back RTP end system (B2BRE)":

R-8.4.1/2: Basic RTCP services: The H.248 WEBRTC gateway is required to support sending and receiving RTCP SR, RR, SDES, and BYE packet types.

R-8.4.1/3: RTP profile dependent RTCP services: The H.248 WEBRTC gateway is required to support sending and receiving RTCP RTPFB (Generic RTP Feedback) and PSFB (Payload-specific Feebdack) packet types.

R-8.4.1/4: Supplementary RTCP services: The H.248 WEBRTC gateway could optionally support sending and receiving RTCP XR (extended report) packet types with respect to performance monitoring of RTP traffic (see also [ITU-T H.248.87]).

### 8.4.2	RTP profiles

R-8.4.2/1: The H.248 WEBRTC gateway is required to support RTP profile "RTP/SAVPF" according to [IETF RFC 5124, IETF RFC 7007] (see clause 4.2/[b-IETF rtp-usage]).

### 8.4.3	RTP multiplexing

R-8.4.3/1: The H.248 WEBRTC gateway is required to support RTP media multiplexing ("SSRC multiplexing") (see clause 4.4/[b-IETF rtp-usage]).

NOTE 1 – The multiplexing mode will only be used when all participants agree. If not, the un-multiplexed mode will be used. Thus, any MGC involved in the call control level end-to-end capability negotiations could actually downgrade to un-multiplexed mode.

R-8.4.3/2: The H.248 WEBRTC gateway is required to support RTP transport multiplexing ("RTP / RTCP transport multiplexing") (see clause 4.5/[b-IETF rtp-usage]).

NOTE 2 – Same comment as in NOTE 1: backward compatibility requires the support of un-multiplexed mode as well.

### 8.4.4	RTP topologies

NOTE – All required RTP topologies are already outlined by [ITU-T H.248.88].

R-8.4.4/1: Use case independent: the H.248 WEBRTC gateway is required to comply with the "RTP topology" behaviour according to [ITU-T H.248.88].

R-8.4.4/2: Use case #1: the H.248 WEBRTC gateway is required to support topology

- "RTP transparent forwarding",

- "RTP transport translator" or

- "RTP media translator",

dependent on the call-level end-to-end negotiation of the used "RTP configuration" for WebRTC.

R-8.4.4/3: Use case #2: The H.248 WEBRTC gateway is required to support, in addition to RTP topologies of use case #1, the toplogy

- "Back-to-back RTP end system (B2BRE)"

due to the "legacy usage of RTP" in the "NGN/IMS IP domain".

NOTE – All existing H.248 profiles for border gateways are not explicit on RTP topologies in detail. Thus, the B2BRE topology represents a default topology (due to the back-to-back IP host configuration of an RTP-RTP H.248 Context).

R-8.4.4/4: Use case #3: The H.248 WEBRTC gateway is required to support the "RTP endsystem" (RTPE) topology (see clause 7.1/[ITU-T H.248.88]).

### 8.4.5	RTCP based services, report types and packet formats

R-8.4.5/1: The H.248 WEBRTC gateway is required to support reduced size RTCP according to [IETF RFC 5506] (see clause 4.6/[b-IETF rtp-usage]).

### 8.4.6	Resource allocation rules for RTP/RTCP

#### 8.4.6.1	Port allocation rules for RTP and RTCP

R-8.4.6.1/1: The H.248 WEBRTC gateway is required to support symmetric RTP/RTCP according to [IETF RFC 4961] (see clause 4.7/[b-IETF rtp-usage]), which relates to the H.248-controlled port allocation as defined by [ITU-T H.248.57].

#### 8.4.6.2	SSRC allocation rules for RTP

R-8.4.6.2/1: The H.248 WEBRTC gateway is required to support signalled RTP synchronisation source (SSRC) identifiers according to [IETF RFC 5576] (see clause 4.8/[b-IETF rtp-usage]).

#### 8.4.6.3	CNAME allocation rules for RTP/RTCP

R-8.4.6.2/1: The H.248 WEBRTC gateway is required to support unique RTCP CNAME value(s) (see clause 4.9/[b-IETF rtp-usage]).

NOTE – A single CNAME might be sufficient for a single H.248 MG entity.

R-8.4.6.2/2: The H.248 WEBRTC gateway can optionally be requested to report received RTCP CNAME value(s) from remote WebRTC clients (according to the RTCP Source Description package as defined by [ITU-T H.248.71].

## 8.5	Requirements related to TCP-based media transport

### 8.5.1	Requirements related to TCP connection establishment

The establishment of the TCP connection segment between the remote WebRTC client and H.248 WebRTC gateway is subject of the overall NAT traversal framework given by ICE/STUN procedures as executed between both WebRTC endpoints. This results in the following TCP specific requirements.

R-8.5.1/1: The H.248 WEBRTC gateway is required to support autonomous TCP connection establishment according to the implicit results of the ICE candidate selection process.

NOTE – Above requirements translates in active, passive or simultaneous TCP connection establishment procedures. Thus, the TCP-enabled H.248 SEP does not use [ITU-T H.248.84] and [ITU-T H.248.89] during the TCP establishment phase.

### 8.5.2	Requirements related to connectivity checks of TCP connection candidates

R-8.5.2/1: The H.248 WEBRTC gateway is required to support correspondent STUN-based connectivity checks for TCP (within the overall requirement R-8.1.2/2).

### 8.5.3	Requirements related to TCP connection release

#### 8.5.3.1	Requirements related to immediate TCP connection release in case of non-used ICE candidates

R-8.5.3.1/1: The H.248 WEBRTC gateway is required to support the immediate release of non-selected TCP connection candidates.

#### 8.5.3.2	Requirements related to regular TCP connection release at the end of the WebRTC call

R-8.5.3.2/1: The H.248 WEBRTC gateway is required to support the usual TCP connection release capabilities as outlined in [ITU-T H.248.89].

### 8.5.4	Requirements related to application level framing protocol

R-8.5.4/1: The H.248 WEBRTC gateway is required to support the L4+ framing scheme according to [IETF RFC 4571] in case of TCP-based WebRTC transport.

### 8.5.5	Requirements related to MG-internal interworking of TCP traffic

#### 8.5.5.1	Requirements related to TCP-to-UDP interworking

R-8.5.5.1/1: The H.248 WEBRTC gateway is required to provide a complete termination of the TCP, i.e., a (TCP)-connection-endpoint function.

#### 8.5.5.2	Requirements related to TCP-to-TCP interworking

[ITU-T H.248.84] and [ITU-T H.248.89] describe various modes of operation related to TCP-to-TCP interworking in H.248 media gateways. Such a TCP-to-TCP connection model is actually only given for the case of a WebRTC call with a single data channel only. The correspondent TCP interworking model would be an "application-aware, stateful TCP" proxy mode (clause 3.2.1/[ITU-T H.248.89]). However, the general case of multi-data-channel WebRTC calls imply a one-to-many L4 topology, i.e., the TCP Endsystem of the WebRTC client might be connected to more than one TCP Endsystem at the non-WebRTC termination.

R-8.5.5.2/1: The H.248 WEBRTC gateway is required to support basically a Back-to-Back TCP Endsystem (B2BTE) mode due to

- firstly, the "ICE-controlled" TCP connection segment towards remote WebRTC clients (NOTE 1) and

- secondly, the principle one-to-many connection endpoint relation at the TCP layer (NOTE 2) within an H.248 "WebRTC gateway" Context.

NOTE 1 – Background: L4 connectivity issues during the active call phase could lead to ICE refresh procedures, which might impact L4 connection updates.

NOTE 2 – The MG might autonomously transition to a more efficient mode of interworking than B2BTE, however, such MG behaviour is basically conditional and considered to be implementation specific. Example conditions might be the knowledge of the MG of a single-data-channel-only WebRTC call ("which might allow a tighter relation of the TCP flow controls of each connection segment"), or required application level interworking ("such as transparent forwarding or not of IP application protocol data").

#### 8.5.5.3	Requirements related to MG-internal buffering of TCP data and TCP flow control

This requirements area is implementation specific and out of scope of this Recommendation due to the variety of B2BTE topologies (see clause 8.5.5.2) and the general assumption that MG-internal TCP data forwarding behaviour is out of scope of standardization.

Some general considerations might be given:

- TCP flow control during establishment phase

  - Without early application data

Is the normal case for the H.248 WebRTC termination due to the initial ICE phase. Hence, there would be firstly a pure TCP connection establishment phase without any application data transfer (either SRTP-related media or DTLS messages for DTLS connection establishment) at this stage. There is consequently not any TCP data for MG-internal forwarding (and thus buffering) in WebRTC to non-WebRTC H.248 Termination directions.

  - With early application data

The TCP establishment processes at the TCP connection segments of the WebRTC and non-WebRTC termination are basically asynchronous, i.e., one side is ready for data transfer earlier than the other side. It is up to the MG to buffer or discard TCP data at this stage of the WebRTC call phase, but it is important that a MG discarding TCP data has to indicate such "loss" via TCP AN acknowledgement process towards the remote TCP endpoint.

- TCP flow control during active data transfer phase

It is up to the MG to minimize the amount of internal buffered TCP data by active intervention in the TCP AN acknowledgement processes of all TCP connection endpoints within the H.248 "WebRTC gateway" Context.

## 8.6	Requirements related to media-aware type of interworking

### 8.6.1	Requirements related to WEBRTC audio

#### 8.6.1.1	Requirements related to media format (audio)

R-8.6.1.1/1: The H.248 WEBRTC gateway is required to support audio transcoding between the WEBRTC MTI audio codec(s) and other audio codecs as typically used in NGNs.

#### 8.6.1.2	Requirements related to media adaptation and rate control (audio)

Requirements are subject of clause 7/[b-IETF rtp-usage], however, there are not yet any explicit capabilities identified for the first phase of WebRTC service deployment. Correspondent capabilities are currently developed by IETF working group RMCAT (RTP Media Congestion Avoidance Techniques), see e.g. requirements in [b-IETF rmcat-cc-requirements], which are planned to be integrated later on in WEBRTC.

### 8.6.2	Requirements related to WEBRTC video

#### 8.6.2.1	Requirements related to media format (video)

R-8.6.2.1/1: The H.248 WEBRTC gateway acting as IP-IP media gateway is not required to support video transcoding (because, if required at all, it is provided by centralized media servers).

R-8.6.2.1/2: The H.248 WEBRTC gateway as media server can optionally support video transcoding between the WEBRTC video codec(s) and other video codecs as typically used in NGNs.

NOTE – The optional tagging of this requirement is due to the fact that it is expected that potential scenarios for video transcoding might be fairly low and exceptional. It is rather expected that WebRTC endpoints will negotiate a common video codec, or fallback to a default codec, or even not use video.

#### 8.6.2.2	Requirements related to media adaptation and rate control (video)

See clause 8.6.1.2.

### 8.6.3	Requirements related to WEBRTC data

#### 8.6.3.1	Requirements for MSRP-based instant messaging

R-8.6.3.1/1: The H.248 WEBRTC gateway is required to support transparent forwarding of MSRP protocol data units ("MSRP messages"), independent of the underlying (WebRTC specific or legacy) transport (protocol stack). This includes interworking between WebRTC clients or a WebRTC and non-WebRTC client.

R-8.6.3.1/2: The H.248 WEBRTC gateway is required to support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to "TCP" for MSRP traffic.

R-8.6.3.1/3: The H.248 WEBRTC gateway can optionally support a bearer level application gateway (B-ALG) at MSRP layer according to [ITU-T H.248.78]. Such a capability relates to a specific MSRP B2BUA function with the purpose of a L4+ level NAT traversal support.

#### 8.6.3.2	Requirements for BFCP-based conference control

R-8.6.3.2/1: The H.248 WEBRTC gateway is required to support transparent forwarding of BFCP protocol data units ("BFCP messages"), independent of the underlying (WebRTC specific or legacy) transport (protocol stack). This includes interworking between WebRTC clients or a WebRTC and non-WebRTC client.

R-8.6.3.2/2: The H.248 WEBRTC gateway is required to support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to "TCP" for BFCP traffic.

R-8.6.3.2/1: The H.248 WEBRTC gateway can optionally support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to "UDP" for BFCP traffic (see [b- draft-ietf-bfcpbis-rfc4582bis]).

#### 8.6.3.3	Requirements for binary-encoded WEBRTC data

R-8.6.3.3/1: The H.248 WEBRTC gateway is required to support transparent forwarding of binary data units (images), independent of the underlying (WebRTC specific or legacy) transport (protocol stack). This includes interworking between WebRTC clients or a WebRTC and non-WebRTC client.

R-8.6.3.1/2: The H.248 WEBRTC gateway is required to support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to an applicable transport e.g. "TCP" for binary traffic.

### 8.6.4	Requirements related to WEBRTC text

WebRTC text telephony (text conversation) is considered as "data traffic" (despite the fact of the usual RTP-based transport in non-WebRTC environments).

R-8.6.4/1: The H.248 WEBRTC gateway is required to support transparent forwarding of T.140 protocol data units (according to [ITU-T T.140]), independent of the underlying (WebRTC specific or legacy) transport (protocol stack). This includes interworking between WebRTC clients or a WebRTC and non-WebRTC client.

R-8.6.4/2: The H.248 WEBRTC gateway is required to support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to "RTP/UDP" for T.140 traffic.

R-8.6.4/3: The H.248 WEBRTC gateway is required to emulate a "T.140/RTP" endpoint in the non-WebRTC domain, which implies an RTP source/RTP sink behaviour according to [b-IETF RFC 4103]. Hence, the H.248 WEBRTC gateway is required to be aware of the T.140 IP application protocol despite of the use of a transparent forwarding mode. The "T.140 awareness" by the H.248 WEBRTC gateway is limited to the detection of active/silence periods related to the transfer of T.140 PDUs, as well as a "packet loss concealment" method related to incoming T.140/RTP packets. Appendix II provides more background information on that interworking aspect.

## 8.7	Requirements related to bearer security

### 8.7.1	Requirements related to transport security

#### 8.7.1.1	Protocol stacks with DTLS

R-8.7.1.1/1: The H.248 WEBRTC gateway is required to support transport security for UDP using DTLS, according to [ITU-T H.248.93].

R-8.7.1.1/2: The H.248 WEBRTC gateway is required to support transport security for TCP using DTLS (according to [ITU-T H.248.93]) and [ITU-T RFC 4571] as the interim framing protocol.

R-8.7.1.1/3: The H.248 WEBRTC gateway is required to use a shared DTLS connection for SCTP-encapsulated WebRTC data traffic, according to [ITU-T H.248.93, H.248.97].

R-8.7.1.1/4: The H.248 WEBRTC gateway is required to use a shared DTLS connection for SRTP related key exchange procedures (for WebRTC audio and video traffic), according to [ITU-T H.248.93, H.248.77].

#### 8.7.1.2	DTLS interworking scenarios

Transport of data traffic in a native WEBRTC domain relates to the protocol layering of "data-over-SCTP/DTLS/UDP/IP", whereas transport of data traffic in the native Internet uses basically "data-over-TCP/IP". The H.248 WEBRTC gateway needs to address interworking between both network paradigms (see also use cases #1 and #2 in clause 7.1), leading to following requirements:

R-8.7.1.2/1: The H.248 WEBRTC gateway is required to support DTLS transparent forwarding for the interworking between two network domains with SCTP/DTLS/UDP based WEBRTC data transport. This relates to the use case according to clause 6.2.2.1/[ITU-T H.248.93].

R-8.7.1.2/2: The H.248 WEBRTC gateway is required to support DTLS-to-non-DTLS interworking for the interworking between two network domains with SCTP/DTLS/UDP and (TLS)/TCP based WEBRTC data transport. This relates to the use case #1.2 according to clause 6.2.1/[ITU-T H.248.93].

#### 8.7.1.3	TLS support

R-8.7.1.3/1: The H.248 WEBRTC gateway is required to support TLS termination in case of the DTLS interworking scenario according to R-8.7.1.2/2 and transport security usage in the TCP domain.

### 8.7.2	Requirements related to media security

#### 8.7.2.1	Encryption method

R-8.7.2.1/1: The H.248 WEBRTC gateway is required to support media security for RTP using SRTP, according to [b-IETF RFC 3711].

#### 8.7.2.2	Key exchange scheme for SRTP

R-8.7.2.2/1: The H.248 WEBRTC gateway is required to support the media-path coupled SRTP key exchange (using DTLS-SRTP [IETF RFC 5764], see [b-IETF RFC 5763]).

R-8.7.2.2/2: The H.248 WEBRTC gateway can optionally support the signalling-path coupled SRTP key exchange, i.e., the "SDES" (SDP security descriptions) related method according to [ITU-T H.248.77].

R-8.7.2.2/3: The H.248 WEBRTC gateway can optionally support multiple key exchange schemes for SRTP within a single H.248 profile. The H.248 WebRTC gateway is then required to indicate the applied SRTP key exchange scheme according to [ITU-T H.248.77].

## 8.8	Requirements related to WEBRTC emergency and priority services

### 8.8.1	Emergency services within WebRTC calls

From the IETF side, requirements for WEBRTC emergency services are so far contained in the (expired) draft [b-IETF rtcweb-ecrit]. No additional requirements for H.248 WEBRTC gateways are derived taking this initial document as baseline.

### 8.8.2	Priority services within WebRTC calls

This Release of the Recommendation does not provide any (explicit) requirements related to priority services.

NOTE – There appears to be no discussion or method to communicate priority in JSEP [b-IETF rtcweb-jsep]. Therefore it appears WebRTC does not support priority in SDP form. Priority is able to be specified as part of data channel [b-IETF DCEP]. [b-IETF rtp-usage] indicates that the WebRTC API also allows prioritisation of MediaStreamTrack but there’s no discussion in the API document.

## 8.9	Requirements related to QoS support

### 8.9.1	IP traffic control: QoS marking

R-8.9.1/1: The H.248 WEBRTC gateway is required to support DiffServ based QoS marking according to [ITU-T H.248.52] (see clause 12.1.3/[b-IETF rtp-usage]).

NOTE 1 – The H.248 WEBRTC MG provides the so-called "DS pre-marker" role (see Appendix IV/[ITU-T H.248.52]).

NOTE 2 – QoS marking takes effect at IP level, i.e., on the complete aggregate of all WebRTC traffic components. Any kind of application protocol individual QoS markings is not required for initial WebRTC service support.

### 8.9.2	IP traffic control: traffic policing – IP byte rate policing

R-8.9.2/1: The H.248 WEBRTC gateway is required to support IP byte rate policing according to [ITU-T H.248.53].

NOTE 1 – Traffic policing takes effect at IP level, i.e., on the complete aggregate of all WebRTC traffic components.

### 8.9.3	Improved transport robustness for RTP

See clause 6/[b-IETF rtp-usage].

#### 8.9.3.1	Negative acknowledgements

In general:

R-8.9.3.1/1: The H.248 WEBRTC gateway is recommended to support negative acknowledgements (NACKs) for RTP data packets [IETF RFC 4585] (see clause 6.1/[b-IETF rtp-usage]).

In particular:

The origination and termination of RTCP Feedback (FB) messages is subject of RTP sender and RTP receiver entities [IETF RFC 4585], thus tightly coupled to "RTP topologies" (see clause 8.4.4). Hence, an H.248 WEBRTC gateway, as located within an end-to-end RTP media path, is normally only partially involved in the processing of RTCP FB messages. The following high level behaviour is derived:

R-8.9.3.1/2: The H.248 WEBRTC gateway is recommended to support the forwarding of RTCP FB messages when using a RTP translator topology.

R-8.9.3.1/3: The H.248 WEBRTC gateway is recommended to originate and terminate RTCP FB messages when using a Back-to-back RTP end system topology.

#### 8.9.3.2	Forward Error Correction

In general:

R-8.9.3.2/1: The H.248 WEBRTC gateway can optionally support Forward Error Correction (FEC) for RTP [IETF RFC 5109] (see clause 6.2/[b-IETF rtp-usage]).

In particular:

Roughly, an RTP sender generates the usual RTP "media packets" plus RTP "FEC packets", which are evaluated again by the RTP receiver in case of lost RTP "media packets". And there is an underlying assumption of unmodified RTP packets along the RTP media path. Thus, FEC becomes useless in case of interim RTP entities which would modify RTP packets. Leading to following high-level behaviour:

R-8.9.3.2/2: The H.248 WEBRTC gateway is required to support the transparent forwarding of RTP traffic when using a RTP translator topology, - i.e., when FEC is required, then only RTP transport translator topology would be allowed.

R-8.9.3.2/3: The H.248 WEBRTC gateway is recommended to originate and terminate RTP "FEC packets" when using a Back-to-back RTP end system topology.

## 8.10	Requirements related to bearer-path coupled congestion controls

These requirements are tightly coupled with the capability of "media adaptation and rate control" (see clause 8.6). Specific requirements are not yet settled from IETF side for the first release of this Recommendation (see also [b-IETF rmcat-cc-requirements]).

## 8.11	Requirements related to performance monitoring

The requirements framework in this clause follows the [ITU-T H.248.87] "Guidelines on the use of ITU-T H.248 capabilities for performance monitoring in RTP networks".

Browser-based WEBRTC endpoints may likely support the local generation of measurements for performance metrics according to [b-IETF rtcweb-xr]. The bowser-embedded measurement point and the set of supported WebRTC's statistics are defined by [b-W3C webrtc-stats].

### 8.11.1	Requirements related to measurement point (MP)

Generally, the set of performance measurements typically differs between WEBRTC endpoints (such as a WEBRTC browser) and WEBRTC gateways, e.g., because the observation of application level performance metrics is primary subject of communication endpoints and normally questionable or even meaningless in case of "in-path equipment".

R-8.11.1/1: The H.248 WEBRTC gateway is recommended to execute measurements for transport level performance metrics (term definition see clause 3.2.6/[ITU-T H.248.87]).

R-8.11.1/2: The H.248 WEBRTC gateway can optionally execute measurements for application level performance metrics (definition see clause 3.2.1/[ITU-T H.248.87]), - under the condition of a valuable semantic of the concerned metric.

Example metrics:

- RTP based WEBRTC traffic:

- see Appendix III/[ITU-T H.248.87] for a general overview

- RTCP XR related metrics are recommended (e.g., RTCP XR metrics are used instead of H.248 nt / rtp package statistics)

- Non-RTP based WEBRTC traffic: e.g. transport level metrics related to

- DTLS (see [ITU-T H.248.93]),

- SCTP (see [ITU-T H.248.97]),

- TLS (see [ITU-T H.248.90]) and

- TCP (see [ITU-T H.248.84, ITU-T H.248.90]).

The actual set of performance metrics for a particular H.248 "WEBRTC gateway" profile depends, as usual, on the specific purpose, such as

- usage metering (relates typically to the traffic volume on the application level);

- reporting of user experience related metrics (i.e., QoS/QoE related metrics);

- reporting of network condition related metrics (i.e., GoS related metrics);

- recording of successful, unsuccessful or specific policing actions; or

- validation of network capacity allocations (relates typically to the traffic volume on the lowest layer of transport capacity reservation)

The primary focus is most likely (c) in case of a H.248 WEBRTC gateway due to its positioning at the border of network domains (with typically different Grade of Service conditions).

### 8.11.2	Requirements related to collection point (CP)

The set of supported performance metrics, which should be measured by WebRTC endpoints (as peered by H.248 WEBRTC gateway) is still undefined at the release of this Recommendation. It is e.g. expected that [b-IETF rtcweb-xr] might be supported for the RTP based WEBRTC traffic components. Performance metrics for non-RTP WEBRTC traffic is still unclear.

When WebRTC endpoints support the measurement of RTCP XR based metrics, then it is expected that they report their measurements via RTCP along the WebRTC media path.

R-8.11.2/1: The H.248 WEBRTC gateway is required to collect remote measurement data via incoming RTCP reports.

### 8.11.3	Requirements related to reporting point (RP)

R-8.11.3/1: The H.248 WEBRTC gateway is required to support the reporting of RTCP XR related H.248 statistics according to the RTCP XR block reporting package [ITU-T H.248.48].

### 8.11.4	Requirements related to filtering point (FP)

Measurement data carried by RTCP XR reports could be filtered by H.248 WEBRTC gateways, see [ITU-T H.248.87]. However, the particular H.248 WEBRTC gateway filter behaviour is subject of operator policies, thus out of scope of this Recommendation.

### 8.11.5	Requirements related to loopback point (LP)

None.

## 8.12	Requirements related to SDP-based description, declaration and negotiation of WEBRTC media configurations

### 8.12.1	Requirements related to supported SDP elements – Overall media descriptions

The H.248 WEBRTC gateway is required to support SDP elements (for the purpose of media gateway control) according to Tables 1, 2 and 3:

Table 1 – Requirements related to supported SDP elements – Part I SDP "m="-line

| Requirement | SDP "m=" line element | Purpose |
| --- | --- | --- |
| Media type values (in case of media type aware interworking modes): |  |  |
| R-8.12.1.1/1: | media type = audio | WEBRTC audio component |
| R-8.12.1.1/2: | media type = video | WEBRTC video component |
| R-8.12.1.1/3: | media type = application | WEBRTC data components |
| Transport protocol (stack) values: |  |  |
| R-8.12.1.1/4: | proto = RTP/SAVPF | RTP profile for all RTP-based traffic components |
| R-8.12.1.1/5: | proto = UDP/DTLS/SCTP proto = TCP/DTLS/SCTP | WEBRTC data traffic components (NOTE 1) and L4-aware NAT traversal support ("MGC knows the L4 protocol used at the start of communication phase") |
| R-8.12.1.1/6: | proto = DTLS/SCTP | WEBRTC data traffic components (NOTE 1) and L4-agnostic NAT traversal support ("MG autonomously concludes whether UDP or TCP will be used at the start of communication phase") |
| Media format values (in case of media format aware interworking modes) (NOTE 2): a) audio formats (from WebRTC endpoint perspective, see [b-IETF rtcweb-audio]): |  |  |
| R-8.12.1.1/7: | OPUS | Internet audio code [b-IETF RFC 6716] |
| R-8.12.1.1/8: | PCMA / PCMU & CN | voice G.711 µ/A-law with optional comfort noise |
| R-8.12.1.1/9: | telephone-event (event codepoints 0 to 11) | audio/telephone-event media format as specified in [b-IETF RFC 4733] for DTMF |
| b) audio formats (optional, from NGN/IMS endpoint perspective, see use case #2): |  |  |
| R-8.12.1.1/10: | AMR-WB | Adaptive Multi-Rate Wideband [b-IETF RFC 4867] |
| R-8.12.1.1/11: | EVRC | Enhanced Variable Rate Codec [b-IETF RFC 4788] |
| c) video formats (for consideration, see [b-IETF rtcweb-video]): |  |  |
| R-8.12.1.1/12: | VP8 | video codec [b-IETF RFC 6386], [b-IETF rtp-vp8] |
| R-8.12.1.1/13: | H264 | video codec [b-IETF RFC 6184] |
| R-8.12.1.1/14: | VP9 | video codec [b-IETF codec-vp9] |
| R-8.12.1.1/15: | H265 | video codec [b-IETF rtp-h265] |
| d) text formats (for consideration): |  |  |
| R-8.12.1.1/16: | t140 | conversational text [IETF RFC 4103] |
| e) WebRTC data channel formats |  |  |
| R-8.12.1.1/17: | webrtc-datachannel | generic data channel, realized as SCTP Stream over a DTLS session (NOTE 3). |
| NOTE 1 – SDP element defined by [b-IETF sdp-sctp]. There might be a subset of the protocol stack related name used in case of specific H.248 stream grouping models. NOTE 2 – Codec technologies are continuously improving, leading to a limited timeline of usage and their future replacement by evolved codecs. Thus, all listed media format specific types are not mandatory, but are listed in order to provide concrete use cases for the SDP-based parameterization of such codecs in H.248 WEBRTC gateways (see clause 8.12.2). Mandatory to implement (MTI) type of media formats are out of scope of this Recommendation, rather subject of concrete H.248 profile specifications. NOTE 3 – SDP "m="-line media format value is defined by [b-IETF sdp-sctp]. |  |  |

Table 2 – Requirements related to supported SDP elements – Part II SDP "a="-line

| Requirement | SDP element | Purpose |
| --- | --- | --- |
| NAT traversal support for media / bearer path: |  |  |
| R-8.12.1.2/1: | a=ice-ufrag | ICE credentials for media level STUN authentication procedure |
| R-8.12.1.2/2: | a=ice-pwd |  |
| R-8.12.1.2/3: | a=candidate: … TCP … | in case of last resort of TCP-based media transport (instead of UDP) |
| Security in the media / bearer path: |  |  |
| R-8.12.1.2/4: | a=fingerprint | Authentication procedure for WEBRTC DTLS sessions. |
| R-8.12.1.2/5: | a=crypto | in case of optional SDES-based SRTP |
| Indication of bearer establishment direction: |  |  |
| R-8.12.1.2/6: | a=setup | This SDP attribute is used at SIP-based WebRTC call control level, but not required for WebRTC gateway control (NOTE 1). |
| Traffic multiplexing in the media / bearer path: |  |  |
| R-8.12.1.2/7: | a=group:BUNDLE | General media grouping using "bundle" (NOTE 2). [ITU-T H.248.96] defines the usage of these SDP attributes because "SIP/SDP bundling" leads to "H.248 stream grouping", i.e. both SDP attributes are not used for WebRTC gateway control. |
| R-8.12.1.2/7: | a=mid: |  |
| R-8.12.1.2/8: | a=rtcp-mux | for transport multiplexed RTP traffic [ITU-T H.248.57] |
| R-8.12.1.2/9: | a=rtcp | optional, if transport unmultiplexed mode for RTP and explicit RTCP transport address allocation [ITU-T H.248.57] |
| R-8.12.1.2/10: | a=ssrc: | Not required at H.248 interface for basic WebRTC call services (NOTE 3). |
| WEBRTC usage of RTP / RTCP: |  |  |
| R-8.12.1.2/11: | a=rtcp-rsize | Support of reduced-size RTCP [IETF RFC 5506] |
| R-8.12.1.2/12: | a=rtcp-fb | RTP profile "RTP/SAVPF" is mandatory (see R-8.12.1.1/4). There are a number of potential RTCP-feedback based services in WebRTC. This SDP attribute is used to indicate which RTCP FB messages (from [IETF RFC 4585], [IETF RFC 5104]) are required for a specific WebRTC call. |
| WEBRTC data traffic configuration: |  |  |
| R-8.12.1.2/13: | a=sctp-port:… | The actual SCTP port value of the SCTP Association carried over DTLS/UDP, which is used for "WEBRTC data channels" (NOTE 4) |
| R-8.12.1.2/14: | a=max-message-size:… | The maximum message size (indicated in bytes) that an SCTP Stream endpoint is willing to receive on the SCTP Association, which is used for "WEBRTC data channels" (NOTE 4) |
| R-8.12.1.2/15: | a) application-agnostic: a=dcmap:<SCTP Stream ID> | Assignment of a particual SCTP Stream to a WEBRTC data traffic component (NOTE 5) without any indication of the subprotocol. |
| R-8.12.1.2/16: | a) application-aware a=dcmap:<SCTP Stream ID> subprotocol="…" | Assignment of a particual SCTP Stream to a WEBRTC data traffic component (NOTE 5) with indication of the subprotocol. |
| R-8.12.1.2/17: | a=dcsa:… | application-specific information of a particular subprotocol asWEBRTC data channel (NOTE 5, NOTE 6). |
| NOTE 1 – There are primarily two reasons for that: - The SDP attribute is semantically overloaded (because defined for multiple protocols), therefore it is mapped (by the MGC) to H.248 Signal/Event-based bearer control procedures. - Semantical differences between WebRTC clients and WebRTC gateways. Example: [b-IETF data-channel-msrp] describes the usage of this SDP attribute in context of WebRTC MSRP-based data service. The attribute clarifies, inter alia, MSRP role assignments between two WebRTC client instances. However, the MSRP will be not terminated by the H.248 WebRTC gateway because it primarily acts as a pure MSRP message forwarding instance, leading to MSRP role agnostic behaviour. NOTE 2 – SDP element defined by [b-IETF sdp-bundle]. NOTE 3 – There might be in future an overlay of well-known supplementary services (such as call transfer) for WebRTC clients, which could require explicit signalling of (RTP) source specific information. However, such WebRTC service extensions are for further studies. NOTE 4 – SDP element defined by [b-IETF sdp-sctp]. NOTE 5 – SDP element defined by [b-IETF DCSDP]. NOTE 6 – See [b-IETF data-channel-msrp] concerning the SDP profiling for the example usage of a WebRTC data channel for application protocol "MSRP". |  |  |

Table 3 – Requirements related to supported SDP elements – Part III other SDP line s

| Requirement | SDP element | Purpose |
| --- | --- | --- |
| Transport capacity reservation and allocation – H.248 Stream admission control: |  |  |
| R-8.12.1.3/1: | "b="-line | see [b-IETF rtp-usage] ; The following bandwidth modifier values might be used: RR, RS, AS, CT. The concrete usage at call control level is conditional, dependent on multiplexing, RTCP services and overall QoS architecture. The "b="-line information is typically subject of bandwidth and admission control at H.248 interfaces (see e.g., clause 5.17.1.5/[b-ETSI TS 183 018]). The MGC needs to provide sufficient "b="-line information to the MG, to be in concert with any optional traffic policing parameter values (e.g. when [ITU-T H.248.53] is applied). |

### 8.12.2	Requirements related to supported SDP elements – Codec configurations

Table 1 introduced the required media formats ("codecs"), which usually include a set of parameters concerning their configuration. These parameters are signalled via SDP from MGC to MG and affect the RTP packet encapsulation ("internal structure of the protocol data unit"), codec processing and RTCP handling, dependent on media-aware / media-agnostic modes of operation of the H.248 WEBRTC gateway.

#### 8.12.2.1	SDP elements related the parameterization of audio codec "OPUS"

See clause 2.1/[b-IETF RFC 6716]:

The Opus codec includes a number of control parameters that can be changed dynamically during regular operation of the codec, without interrupting the audio stream from the encoder to the decoder. These parameters only affect the encoder since any impact they have on the bitstream is signaled in-band such that a decoder can decode any Opus stream without any out-of-band signaling. Any Opus implementation can add or modify these control parameters without affecting interoperability. The most important encoder control parameters in the reference encoder are listed below.

The H.248 WEBRTC gateway, when enabled for media aware interworking, would provide such an encoder and decoder function, hence is required to support SDP elements (for the purpose of media gateway control) according to Table 4. [b-IETF RFC 7587] defines the formal codec parameters (in clause 6.1) and their mapping on SDP elements (in clause 7.1/[b-IETF RFC 7587]). The parameter value ranges and default values in Table 4 are defined in clause 6.1/[b-IETF RFC 7587].

Table 4 – SDP parameters for IANA registered media type "audio/opus"

| Requirement | Codec parameter | SDP parameter | M/O | Value range | Default | Comments and H.248 profile |
| --- | --- | --- | --- | --- | --- | --- |
| R-8.11.2.1/1: | rate ("sampling rate") | a=rtpmap:… opus/48000/2 | M | 8000, 12000, 16000, 24000, 48000 | 48000 | all (NOTE 2) |
| R-8.11.2.1/2: | maxplaybackrate | a=fmtp: (NOTE 1) | O |  | 48000 | all |
| R-8.11.2.1/3: | sprop-maxcapturerate | a=fmtp: (NOTE 1) | O |  | 48000 | all |
| R-8.11.2.1/4: | maxptime | a=maxptime | O | 3, 5, 10, 20, 40, 60, or an arbitrary multiple of a frame size rounded up to the next full integer value, up to a max. value of 120 | 120 (NOTE 3) | When "media transcoding" mode, then it is recommended to align these parameter values between the OPUS and non-OPUS RTP domain in order to minimize end-to-end latency and to avoid interworking complexity. |
| R-8.11.2.1/5: | ptime | a=ptime | O |  | 20 (NOTE 3) |  |
| R-8.11.2.1/6: | maxaveragebitrate | a=fmtp: (NOTE 1) | O | Any positive integer is allowed, but values outside the range 6000 to 510000 SHOULD be ignored | Dependent on OPUS mode | all |
| R-8.11.2.1/7: | stereo | a=fmtp: (NOTE 1) | O | 0 (mono), 1 (stereo) | 0 (mono) | at least 'mono' (NOTE 4) |
| R-8.11.2.1/8: | sprop-stereo | a=fmtp: (NOTE 1) | O |  | 0 (mono) | at least 'mono' (NOTE 4) |
| R-8.11.2.1/9: | cbr | a=fmtp: (NOTE 1) | O | 1 (constant bitrate), 0 (variable bitrate) | 0 (vbr) | Specific mode interacts normally with traffic policer settings ([ITU-T H.248.53]). |
| R-8.11.2.1/10: | useinbandfec | a=fmtp: (NOTE 1) | O | 0 (no FEC), 1 (FEC) | 0 (no FEC) |  |
| R-8.11.2.1/11: | usedtx | a=fmtp: (NOTE 1) | O | 0 (no DTX), 1 (DTX) | 0 (no DTX) |  |
| NOTE 1 – Media type string in the form of a semicolon-separated list of parameter=value pairs, as part of the "a=fmtp" SDP attribute. E.g., a=fmtp:101 maxplaybackrate=16000; sprop-maxcapturerate=16000; maxaveragebitrate=20000; stereo=1; useinbandfec=1; usedtx=0 NOTE 2 – The actual clock rate is signalled in the RTP payload and is not restricted by the control plane signalled values. NOTE 3 – Default values are typically subject of network operator preferences, which lead to correspondent settings in supported user equipment. NOTE 4 – This parameter is only relevant in case of media translation mode, i.e., transcoding OPUS to a different audio codec. The majority of legacy audio codecs only support 'mono'. Transcoding scenarios to other, stereo-capable audio codecs is for further studies. |  |  |  |  |  |  |  |

Table 4 provides the codec parameter value framework for the usage of OPUS in context of H.248 WebRTC gateways. This information is complemented by [b-ITU-T H.248.CodecSDPprofile], which provides general guidelines of OPUS parameter usage at H.248 Descriptor level, as well as their effect on H.248 MG resource management and detailed configuration of the media interworking function for OPUS.

### 8.12.3	Requirements related to SDP Offer/Answer

WEBRTC does not mandate a specific call control signalling protocol. Thus, there are no guidelines for SDP offer/answer from IETF side in case of SIP as WEBRTC call control protocol.

In case of SIP networks with H.248 WEBRTC gateways, handling of SDP offer/answer procedures between call control level and their representation at H.248 gateway control level is outlined by [ITU-T H.248.80], particularly when the revised SDP offer/answer model is used.

## 8.13	Requirements related to H.248 signalling

There might be additional, H.248 specific requirements beyond the pure WebRTC service consideration.

### 8.13.1	Indication of H.248 "WebRTC" Terminations

A WebRTC-enabled H.248 Termination fundamentally implies that the MG needs to provide a WebRTC client function for the particular Termination in a Context. Such information might be relevant e.g. from MG resource management perspective.

R-8.13.1/1: The H.248 WEBRTC gateway is not required to support additional H.248 signalling capabilities in order to discriminate (in MGC to MG direction) between H.248 WebRTC and non-WebRTC terminations.

NOTE – The above requirement makes the assumption that the Termination type indication could be indirectly derived, e.g., from the Termination naming scheme or Termination/Stream specific H.248 Descriptor contents.

# 9	Data Channel Establishment Protocol Support Package

|  | Package name: | Data Channel Establishment Protocol Support Package |
| --- | --- | --- |
|  | Package ID: | dcep (0x0124) |
|  | Description: | This package allows the support of the data channel establishment protocol (DCEP) [b-IETF DCEP] on a MGC/MG. The package allows the MG to detect the reception of DCEP messages (SCTP PPID=50) on WebRTC datachannels. |
|  | Version: | 1 |
|  | Extends: | None |

## 9.1	Properties

None.

## 9.2	Events

### 9.2.1	Detect DCEP Messages

|  | Event name: | Detect DCEP Messages |
| --- | --- | --- |
|  | Event ID: | detmess (0x0001) |
|  | Description: | This event detects the reception of DCEP DATA_CHANNEL_OPEN and DATA_CHANNEL_ACK messages on any of the SCTP Streams on a SCTP Association. |

#### 9.2.1.1	EventsDescriptor parameters

##### 9.2.1.1.1	DCEP data channel open response

|  | Parameter name: | DCEP data channel open response |
| --- | --- | --- |
|  | Parameter ID: | openresp (0x0001) |
|  | Description: | This parameter indicates to the MG whether it should respond autonomously to the reception of a DCEP data channel open. |
|  | Type: | Enumeration |
|  | Optional: | Yes |
|  | Possible values: | “auto”; The MG shall accept incoming DCEP DATA_CHANNEL_OPEN request message on an SCTP Stream by sending a DATA_CHANNEL_ACK message on an outgoing SCTP Stream with the same identity. If the incoming DATA_CHANNEL_OPEN results in an error the MG shall generate an SCTP Stream reset.. “deny”; The MG shall deny the incoming DCEP DATA_CHANNEL_OPEN requests by initiating an SCTP Stream reset on the applicable SCTP Stream.. “mgc”; The MGC will decide the action based on the reception of a DCEP DATA_CHANNEL_OPEN. E.g. The MG may issues a “DCEP data channel open response” (dcep/dcopenresp) Signal or initiate an SCTP reset via the sctpreset/initreset Signal.. |
|  | Default: | “auto” |

#### 9.2.1.2	ObservedEventsDescriptor parameters

##### 9.2.1.2.1	SCTP StreamID

|  | Parameter name: | SCTP StreamID |
| --- | --- | --- |
|  | Parameter ID: | sctpid (0x0001) |
|  | Description: | This parameter indicates the SCTP StreamID that the DCEP DATA_CHANNEL_OPEN message was received on. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 9.2.1.2.2	Sub-protocol ID

|  | Parameter name: | Sub-protocol ID |
| --- | --- | --- |
|  | Parameter ID: | protocol (0x0002) |
|  | Description: | This parameter contains the “Protocol” from the received DCEP DATA_CHANNEL_OPEN message. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per “protocol” in clause 5.1/[b-IETF DCEP]. |
|  | Default: | None |

##### 9.2.1.2.3	Label

|  | Parameter name: | Label |
| --- | --- | --- |
|  | Parameter ID: | label (0x0003) |
|  | Description: | This parameter contains the “Label” from the received DCEP DATA_CHANNEL_OPEN message. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per “label” in clause 5.1/[b-IETF DCEP]. |
|  | Default: | None |

##### 9.2.1.2.4	Channel Type

|  | Parameter name: | Channel Type |
| --- | --- | --- |
|  | Parameter ID: | chtype (0x0004) |
|  | Description: | This parameter contains the “Channel Type” from the received DCEP DATA_CHANNEL_OPEN message. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | As per “Channel Type” in clause 5.1/[b-IETF DCEP]. |
|  | Default: | None |

##### 9.2.1.2.5	Reliability Parameter

|  | Parameter name: | Reliability Parameter |
| --- | --- | --- |
|  | Parameter ID: | reli (0x0005) |
|  | Description: | This parameter contains the “Reliability Parameter” from the received DCEP DATA_CHANNEL_OPEN message. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | As per “reliability parameter” in clause 5.1/[b-IETF DCEP]. |
|  | Default: | None |

##### 9.2.1.2.6	Priority

|  | Parameter name: | Priority |
| --- | --- | --- |
|  | Parameter ID: | priority (0x0006) |
|  | Description: | This parameter contains the “Priority” from the received DCEP DATA_CHANNEL_OPEN message. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | As per “priority” in clause 5.1/[b-IETF DCEP]. |
|  | Default: | None |

##### 9.2.1.2.7	Error

|  | Parameter name: | Error |
| --- | --- | --- |
|  | Parameter ID: | error (0x0007) |
|  | Description: | This parameter indicates that the MG has received a DCEP DATA_CHANNEL_OPEN but has determined it to be erroneous. The action taken by the MG is dependent on the openresp parameter. |
|  | Type: | Boolean |
|  | Optional: | Yes |
|  | Possible values: | On An error has been generated. Off No error has been generated. |
|  | Default: | Off |

##### 9.2.1.2.8	Data Channel Message Type

|  | Parameter name: | Data Channel Message Type |
| --- | --- | --- |
|  | Parameter ID: | ack (0x0008) |
|  | Description: | This parameter indicates that the MG has received a DCEP DATA_CHANNEL_ACK. |
|  | Type: | Enumeration |
|  | Optional: | Yes |
|  | Possible values: | ACK A DATA_CHANNEL_ACK was received. OPEN A DATA_CHANNEL_OPEN was received. |
|  | Default: | OPEN |

## 9.3	Signals

### 9.3.1	Send DCEP Open

|  | Signal name: | Send DCEP Open |
| --- | --- | --- |
|  | Signal ID: | sndopen (0x0001) |
|  | Description: | This Signal requests the MG to send a DCEP DATA_CHANNEL_OPEN message with the indicated parameters on a particular SCTP Stream. |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 9.3.1.1	Additional parameters

##### 9.3.1.1.1	SCTP StreamID

|  | Parameter name: | SCTP StreamID |
| --- | --- | --- |
|  | Parameter ID: | sctpid (0x0001) |
|  | Description: | This parameter indicates the SCTP StreamID that the DCEP DATA_CHANNEL_OPEN message is to be sent on. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 9.3.1.1.2	Sub-protocol ID

|  | Parameter name: | Sub-protocol ID |
| --- | --- | --- |
|  | Parameter ID: | protocol (0x0002) |
|  | Description: | This parameter contains the “Protocol” to be sent in the DCEP DATA_CHANNEL_OPEN message. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per “protocol” in clause 5.1/[b-IETF DCEP]. |
|  | Default: | None |

##### 9.3.1.1.3	Label

|  | Parameter name: | Label |
| --- | --- | --- |
|  | Parameter ID: | label (0x0003) |
|  | Description: | This parameter contains the “Label” to be sent in the DCEP DATA_CHANNEL_OPEN message. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per “label” in clause 5.1/[b-IETF DCEP]. |
|  | Default: | None |

##### 9.3.1.1.4	Channel Type

|  | Parameter name: | Channel Type |
| --- | --- | --- |
|  | Parameter ID: | chtype (0x0004) |
|  | Description: | This parameter contains the “Channel Type” to be sent in the DCEP DATA_CHANNEL_OPEN message. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | As per “Channel Type” in clause 5.1/[b-IETF DCEP]. |
|  | Default: | None |

##### 9.3.1.1.5	Reliability Parameter

|  | Parameter name: | Reliability Parameter |
| --- | --- | --- |
|  | Parameter ID: | reli (0x0005) |
|  | Description: | This parameter contains the “Reliability Parameter” to be sent in the DCEP DATA_CHANNEL_OPEN message. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | As per “reliability parameter” in clause 5.1/[b-IETF DCEP]. |
|  | Default: | None |

##### 9.3.1.1.6	Priority

|  | Parameter name: | Priority |
| --- | --- | --- |
|  | Parameter ID: | priority (0x0006) |
|  | Description: | This parameter contains the “Priority” to be sent in the DCEP DATA_CHANNEL_OPEN message. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | As per “priority” in clause 5.1/[b-IETF DCEP]. |
|  | Default: | None |

### 9.3.2	DCEP Open Response

|  | Signal name: | DCEP Open Response |
| --- | --- | --- |
|  | Signal ID: | dcopenresp (0x0002) |
|  | Description: | This Signal requests the MG to send a DCEP DATA_CHANNEL_ACK message on a particular SCTP Stream. |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 9.3.2.1	Additional parameters

##### 9.3.2.1.1	SCTP StreamID

|  | Parameter name: | SCTP StreamID |
| --- | --- | --- |
|  | Parameter ID: | sctpid (0x0001) |
|  | Description: | This parameter indicates the SCTP StreamID that the DCEP DATA_CHANNEL_ACK message is to be sent on. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

## 9.4	Statistics

None.

## 9.5	Error Codes

None

## 9.6	Procedures

### 9.6.1	Auditing

A MGC may determine if a MG supports the use of DCEP by auditing packages. If the MG returns the “dcep” package then the DCEP is supported.

### 9.6.2	Pre-conditions

In order to detect DCEP messages the MGC shall first establish a UDP/DTLS/SCTP association for WebRTC data channel. See [ITU-T H.248.97] for details on the establishment of an SCTP Association. The establishment of an SCTP Association may also require the use of [ITU-T H.248.96] and a de-aggregation stream. A single H.248 component Stream is used for an incoming and outgoing SCTP Stream with the same identity. The Signals and Events in the “dcep” package are applied to the H.248 Stream representing the SCTP Association as a whole (i.e. a de-aggregation stream and not to the component streams). When setting these Signals and Events the MGC shall provide the H.248 StreamID of the de-aggregation Stream representing the SCTP Association. Notifications of ObservedEvents shall also indicate the H.248 StreamID of the de-aggregation Stream representing the SCTP Association.

NOTE: In addition, the Signals and the Observed Event specified in this package contain the identifier of the SCTP StreamID where the DCEP messages are sent or received.

### 9.6.3	Opening a data channel based on a remote request

In order to detect DCEP DATA_CHANNEL_OPEN and DATA_CHANNEL_ACK messages on the SCTP Association the MGC shall set the “Detect DCEP Data Channel Open” (dcep/detmess) Event on the H.248 Stream on the applicable Termination. Setting this Event indicates that the Termination/Stream shall support the procedures specified in clause 6/[b-IETF DCEP] . The MG shall monitor the SCTP Association for the reception of a DCEP DATA_CHANNEL_OPEN and DATA_CHANNEL_ACK messages (indicated by SCTP PPID=50) on the SCTP Streams in the SCTP Association.

On reception of a DCEP DATA_CHANNEL_OPEN message the MG shall generate a dcep/detmess ObservedEvent and notify the MGC. If the MG detects an error it shall include the “error” ObservedEvent parameter and any other relevant parameters. Further action taken by the MG in response to the detection of a DCEP DATA_CHANNEL_OPEN message is dependent on the “DCEP data channel open response” (openresp) Event parameter. If openresp is set to “mgc” then the MGC will accept the open request by sending the dcep/dcopenresp Signal or deny the open request by initiating a SCTP Stream reset via the sctpreset/initreset signal. If openresp is set to “auto” then the MG shall response autonomously as per clause 9.2.1.1.1.

As the DCEP DATA_CHANNEL_OPEN sender may immediately send data on the SCTP Stream the MG should preferably have the capability to buffer incoming SCTP user data during short periods until the MGC creates a component stream to deliver the data internal to the Context.

NOTE: SCTP offers reliable transmission, therefore it can recover from an eventual loss of user messages, but at the cost of retransmission.

The MGC in turn should react with no delay to the notification of the reception of the DCEP DATA_CHANNEL_OPEN from the peer (i.e. by creating the component Stream and setting the sctpid property to the correct value) in order not to exhaust the buffering capabilities of the MG.

On reception of a DCEP DATA_CHANNEL_ACK message the MG shall generate a dcep/detmess ObservedEvent with the “Data Channel Acknowledgement” (ack) parameter and notify the MGC. In order to instantiate the data channel and to allow data to flow into the Context, the MGC should create a new H.248 Stream and assign the SCTP StreamID received in the ObservedEvent DCEP DATA_CHANNEL_OPEN message to it via the sctp/sctpid property (see clause 8.1.2/[ITU-T H.248.97]). The MGC should add the H.248 Stream as a component stream of a de-aggregation stream via the mgroup/strdeagg property and add it to the “SCTP” semantic via the mgroup/groupse property. See [ITU-T H.248.96] for more details. Once created any buffered data should be delivery internally to the Context via the component Stream.

The following sub-clauses show an example information flow.

#### 9.6.3.1	Remote open request example step 1

The MGC sends an Add.request to the MG adding a UDP/DTLS/SCTP association and requests the MG to detect DCEP DATA_CHANNEL_OPEN messages.

Table 5 – Remote open request example step 1

| Step 1 | Commments |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10000 { Context = 1 { Add = T1 { Events = 1235 {dcep/detmess{stream=1,openresp=mgc}}, Media { Stream = 1 { Local { v=0 c= IN IP4 $ m=application $ UDP/DTLS/SCTP webrtc- datachannel a=max-message-size: 100000 a=sctp-port:5000 } }, } } } } | A H.248 Stream is added to a Termination representing the UDP/DTLS/SCTP association used for WebRTC DataChannel. The MGC sets the dcep/detmess event to detect DCEP DATA_CHANNEL_OPEN messages. The MG will notify the MGC when the open is detected and await a response from the MGC. NOTE - The RemoteDescriptor and SDP regarding the DTLS establishment e.g. a=connection, a=fingerprint is not shown. |

On reception of the Add.request, the MG starts monitoring the SCTP Streams for DCEP DATA_CHANNEL_OPEN messages. It sends an Add.reply.

#### 9.6.3.2	Remote open request example step 2

The MG detects a DCEP DATA_CHANNEL_OPEN message related to SCTP StreamID = 5. It generates a notification to the MGC.

Table 6 – Remote open request example step 2

| Step 2 | Commments |
| --- | --- |
| MEGACO/3 [125.125.125.111]:55555 Transaction = 2000 { Context = 1 { Notify = T1 {ObservedEvents =12345 { 20150115T22020002:dcep/detmess{ stream=1,sctpid=5, chtype=0,reli=0,priority=1}}} } } | The MG reports that a DCEP DATA_CHANNEL_OPEN has been received on SCTP Stream 5. It reports the SCTP Stream ID, channel type, reliability parameter and priority. |

The MGC sends a Notify.reply.

#### 9.6.3.3	Remote open request example step 3

The MGC decides to allow the DCEP channel open request and adds the SCTP Stream as a component of a de-aggregation stream.

Table 7 – Remote open request example step 3

| Step 3 | Commments |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10001 { Context = 1 { Modify = T1 { Signals {dcep/dcopenresp{Stream=1, sctpid=5}}, Events = 1235 {dcep/detmess{Stream=1}}, Media { TerminationState { mgroup/groupse= [”SCTP 2”] }, Stream = 1 { LocalControl { mgroup/strdeagg=[2] } }, Stream = 2 { LocalControl { sctpbcc/sctpid=5 } } } } } } | The MGC modifies H.248 StreamID = 1 and indicates that it is a de-aggregation stream. The group semantic is indicated as “SCTP” H.248 StreamID = 2 is added to represent the data in SCTP Stream 5. |

On reception of the Modify.request the MG will create a new component Stream for H.248 StreamID = 2 and start to pass data received in SCTP StreamID=5 internally to the Context.

### 9.6.4	Local Request to open a data channel

Once the MGC has added a H.248 Stream that establishes a UDP/DTLS/SCTP association for WebRTC Datachannel with the remote peer, it can request via the “Send DCEP Open” (dcep/sndopen) signal that the MG send a DCEP DATA_CHANNEL_OPEN on a certain SCTP Stream. The MGC shall include the SCTP StreamID, channel type, reliability parameter and priority parameters in the signal. The Signal shall be set on the Stream representing the whole SCTP Association (i.e. the de-aggregation Stream).

The MGC should also set an Event to detect the DCEP DATA_CHANNEL_ACK message for the incoming SCTP Stream with the same ID as used in the Signal. See clause 9.6.3 for the procedures for setting Events. It should also set an Event to detect SCTP resets indicating a failure of the channel open.

In order to prepare the MG to send data on the SCTP Stream and to allow the reception of data from the remote peer once it sends a DCEP DATA_CHANNEL_OPEN the MGC should:

- indicate that the H.248 Stream representing the SCTP Association is a de-aggregation stream,

- indicate that the grouping semantic for the de-aggregation stream is “SCTP”,

- add a component stream representing the SCTP StreamID that the DCEP DATA_CHANNEL_OPEN message is sent on.

The MG shall then forward any data received on the SCTP StreamID internally in the Context via the component stream.

The following sub-clauses show an example message sequence.

#### 9.6.4.1	Local open request example step 1

The MGC sends a Modify.request to the MG requesting that the MG send a DCEP DATA_CHANNEL_OPEN message on a particular stream. A de-aggregation and component Stream is also added in order to receive data from the remote peer.

Note: This example builds on the example in clause 9.6.3.

Table 8 – Local open request example step 1

| Step 4 | Commments |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 1004 { Context = 1 { Modify = T1 { Signals {dcep/sndopen{Stream=1, sctpid=7,chtype=0,reli=0,priority=1}}, Media { TerminationState { mgroup/groupse= [”SCTP 2 3”] }, Stream = 1 { LocalControl { mgroup/strdeagg=[2,3] } }, Stream = 3 { LocalControl { sctpbcc/sctpid=7 } } } } } | The MGC requests the MG to send a DCEP DATA_CHANNEL_OPEN message on SCTP StreamID= 7. It creates H.248 StreamID=3 as a component stream and assigns it to SCTP StreamID=7. It adds H.248 StreamID=3 to the SCTP grouping semantic and adds it to the de-aggregation stream. |

On reception of the Modify.request the MG sends DCEP DATA_CHANNEL_OPEN message message on SCTP StreamID=7 and monitors the SCTP Stream for data. It also sends a Modify.reply.

#### 9.6.4.2	Local open request example step 2

The MG sends a notify request to the MGC indicating that it has received a DCEP DATA_CHANNEL_ACK message on SCTP Stream 7. It indicates that the DCEP DATA_OPEN_REQUEST was successful.

Table 9 – Local open request example step 2

| Step 5 | Commments |
| --- | --- |
| MEGACO/3 [125.125.125.111]:55555 Transaction = 2010 { Context = 1 { Notify = T1 {ObservedEvents =12345 { 20150115T22020012:dcep/detmess{ stream=1,sctpid=7,ack=”ACK”}}} } } | The MG reports that a DCEP DATA_CHANNEL_ACK has been received on SCTP Stream 7. |

### 9.6.5	Data channel closure

WebRTC data channels are closed according to the procedures in clause 6.7/[b-IETF WebRTCDC]. The procedures use the mechanism in [IETF RFC 6525] to support the closure of an SCTP Stream. The “SCTP Re-configuration Stream Reset Package” (sctpreset) allows for the support of the RE-CONFIG outgoing SCTP Stream reset. This package shall be implemented if the dcep package is used.

To initiate a channel closure the MGC shall use the "Initiating an outgoing SCTP Stream reset" procedures defined in clause 9.6.2/[ITU-T H.248.97].

The MGC shall also use the “Responding to an outgoing SCTP Stream reset request” procedures in clause 9.6.3/[ITU-T H.248.97] in order to respond to remote channel closure requests.

If the MGC requires that a MG respond to a received outgoing stream reset request by autonomously sending its own outgoing reset request the MGC shall set the sctpreset/initreset Signal parameter outresp to "accint".

### 9.6.6	Termination of DCEP messages

The use of this package on an H.248.SEP implies that DCEP messages are terminated in the SEP, i.e. incoming DCEP messages are not transferred internally to the other SEP, even if the Context Topology would allow the transfer of SCTP user messages.

# 10	Out of band WebRTC data channel negotiation

The use of individual data channels may be negotiated by out-of-band procedures such as those defined by [b-IETF DCSDP]. The individual channels are assigned a protocol and SCTP Stream de-aggregation is applied via the “SCTP Grouping Semantic” procedures of clause 11.2/[ITU-T H.248.97].

In order to send and receive data on the individual data channels the MGC should use [ITU-T H.248.78] to:

- backhaul the relevant bearer-level application protocol across H.248 and on to the relevant H.248 Stream/SCTP StreamID, or

- request the MG to perform bearer level application level gateway functions (i.e. by utilizing the mgbalg package [ITU-T H.248.78].

# 11	H.248 profile specification guidelines

This clause provides guidelines for H.248 profile specifications. The structure follows the profile template according to Appendix III/[ITU-T H.248.1].

The template elements which are not applicable in this Recommendation are indicated by "Subject to profile specification".

Any profile guidelines are primarily dependent on the concerned network configuration and use case. The guidelines in this clause are therefore basically conditional. Some exemplary use cases are considered (as described in Appendix I), termed as capability sets CSA, CSB, CSC and CSD. Tag 'CS*' indicates unconditional profile elements, which are either generic because they are basic WebRTC capabilities or required for all four CS examples.

## 11.1	Profile Identification

Subject to profile specification.

## 11.2	Summary

Subject to profile specification.

## 11.3	Gateway Control Protocol Version

Subject to profile specification.

## 11.4	Connection Model

| Maximum number of contexts: | Subject to profile specification. |
| --- | --- |
| Maximum number of terminations per context: | Subject to profile specification. Examples: IF CSA OR CSB OR CSC THEN "2". IF CSD THEN "N". |
| Allowed termination type combinations in a context: | Subject to profile specification. |

## 11.5	Context Attributes

Subject to profile specification.

## 11.6	Terminations

Subject to profile specification.

## 11.7	Descriptors

### 11.7.1	TerminationState Descriptor

IF CS* THEN following table:

| TerminationState: Group Semantics (mgroup/groupse) | See clause 11.14.3.8. |
| --- | --- |
| TerminationState: … | … |

All other aspects related to TerminationState Descriptor (e.g., ServiceState, EventBuffer Control) are "Subject to profile specification".

### 11.7.2	Stream Descriptor

Subject to profile specification.

It has to be noted that an H.248 IP Termination for the WebRTC service may carry additional H.248 (de-)aggregation streams besides the legacy H.248 component streams. There are then additional, dedicated Stream Descriptors at (de-)aggregation stream level.

### 11.7.3	Events Descriptor

IF CSA OR CSB OR CSC THEN following table:

| Events settable on termination types and stream types: |  | Yes |  |  |
| --- | --- | --- | --- | --- |
| If yes | Event ID | Termination type | Stream type |
|  | See clause 11.14.3.4 stnconfres/constate stnconfres/confail | IP | IP-enabled |
|  | See clause 11.14.3.5 tlsbsc/BNCChange | IP | DTLS-enabled |
|  | See clause 11.14.3.6 sctpbcc/BNCChange | IP | SCTP-enabled |
|  | See clause 11.14.3.7 sctpreset/detreset sctpreset/result | IP | SCTP-enabled |

IF CSA THEN following table:

| Events settable on termination types and stream types: |  | Yes |  |  |
| --- | --- | --- | --- | --- |
| If yes | Event ID | Termination type | Stream type |
|  | See clause 11.14.3.3 - ostuncc/ccr - ostuncc/nprc | IP | IP-enabled |
| NOTE – These ostuncc events are only required for ICE full mode. |  |  |  |

IF CSA OR CSB THEN following table:

| Events settable on termination types and stream types: |  | Yes |  |  |
| --- | --- | --- | --- | --- |
| If yes | Event ID | Termination type | Stream type |
|  | See clause 11.14.3.13 - srtp/mke | IP | SRTP-enabled |
|  | See clause 11.14.3.16 - tlsm/mgea | IP | DTLS-enabled |
|  | See clause 11.14.3.18 - adr/rtac | IP | IP-enabled |

IF CSA OR CSD THEN following table:

| Events settable on termination types and stream types: |  | Yes |  |  |
| --- | --- | --- | --- | --- |
| If yes | Event ID | Termination type | Stream type |
|  | See clause 11.14.3.9 - dcep/detmess | IP | SCTP-enabled |
|  | See clause 11.14.3.10 - mcbalg/det | IP | SCTP-enabled |

All other aspects related to Events Descriptor (e.g., EventBuffer Control, KeepActive, Notification Behaviour) are "Subject to profile specification".

### 11.7.4	EventBuffer Descriptor

Subject to profile specification.

### 11.7.5	Signals Descriptor

IF CSA OR CSB OR CSC THEN following table:

| The setting of signals is dependent on termination or streams types: |  | Yes |  |
| --- | --- | --- | --- |
| If yes | Signal ID | Termination Type | Stream Type/ID |
|  | See clause 11.14.3.1 - ipnapt/latch | IP | IP-enabled |
|  | See clause 11.14.3.4 - stnconfres/contst | IP | IP-enabled |
|  | See clause 11.14.3.5 - tlsbsc/EstBNC - tlsbsc/RelBNC | IP | DTLS-enabled |
|  | See clause 11.14.3.6 - sctpbcc/EstBNC - sctpbcc/RelBNC | IP | SCTP-enabled |
|  | See clause 11.14.3.7 - sctpreset/initreset - sctpreset/resetresp | IP | SCTP-enabled |

IF CSA THEN following table:

| The setting of signals is dependent on termination or streams types: |  | Yes |  |
| --- | --- | --- | --- |
| If yes | Signal ID | Termination Type | Stream Type/ID |
|  | See clause 11.14.3.3 - ostuncc/scc - ostuncc/sacc | IP | IP-enabled |
| NOTE – These ostuncc signals are only required for ICE full mode. |  |  |  |

IF CSA OR CSB THEN following table:

| The setting of signals is dependent on termination or streams types: |  | Yes |  |
| --- | --- | --- | --- |
| If yes | Signal ID | Termination Type | Stream Type/ID |
|  | See clause 11.14.3.16 - tlsm/mgcea | IP | DTLS-enabled |

IF CSA OR CSD THEN following table:

| The setting of signals is dependent on termination or streams types: |  | Yes |  |
| --- | --- | --- | --- |
| If yes | Signal ID | Termination Type | Stream Type/ID |
|  | See clause 11.14.3.9 - dcep/sndopen - dcep/dcopenresp | IP | SCTP-enabled |
|  | See clause 11.14.3.10 - mcbalg/sblm | IP | SCTP-enabled |

All other aspects related to Signals Descriptor (e.g., Signal Direction, Signal List) are "Subject to profile specification".

### 11.7.6	DigitMap Descriptor

Subject to profile specification.

### 11.7.7	Statistics Descriptor

IF CSB OR CSC OR CSD THEN none.

IF CSA THEN following table entries: see clauses 11.14.3.13, 11.14.3.17 and 11.14.3.19.

### 11.7.8	ObservedEvents Descriptor

Subject to profile specification.

### 11.7.9	Topology Descriptor

Subject to profile specification.

### 11.7.10	Error Descriptor

IF CS* THEN following table:

Error Codes sent by the MGC:

| Supported ITU-T H.248.8 Error Codes: | … |
| --- | --- |
| Supported Error Codes defined in packages: | … |

Error Codes sent by the MG:

| Supported ITU-T H.248.8 Error Codes: | … |
| --- | --- |
| Supported Error Codes defined in packages: | #489 "Invalid aggregation and/or de-aggregation" |

IF CSA THEN the following table in addition:

Error Codes sent by the MGC:

| Supported ITU-T H.248.8 Error Codes: | … |
| --- | --- |
| Supported Error Codes defined in packages: | … |

Error Codes sent by the MG:

| Supported ITU-T H.248.8 Error Codes: | … |
| --- | --- |
| Supported Error Codes defined in packages: | #488 "Incorrect stream endpoint interlinkage" |

All other error codes are "Subject to profile specification".

## 11.8	Command API

NOTE – It is assumed that an Error Descriptor may be returned in any command reply.

### 11.8.1	Add

Subject to profile specification.

### 11.8.2	Modify

Subject to profile specification.

### 11.8.3	Subtract

Subject to profile specification.

### 11.8.4	Move

Subject to profile specification.

### 11.8.5	AuditValue

Subject to profile specification.

### 11.8.6	AuditCapabilities

Subject to profile specification.

### 11.8.7	Notify

Subject to profile specification.

### 11.8.8	ServiceChange

Subject to profile specification.

### 11.8.9	Manipulating and auditing context attributes

Subject to profile specification.

## 11.9	Generic command syntax and encoding

Subject to profile specification.

## 11.10	Transactions

Subject to profile specification.

NOTE – There is no impact on Transactions.

## 11.11	Messages

Subject to profile specification.

## 11.12	Transport

Subject to profile specification.

NOTE – Usage of bearer security may demand for a secured H.248 transport mode, too.

## 11.13	Security

Subject to profile specification.

## 11.14	Packages

### 11.14.1	Mandatory Packages

Mandatory: specifies the packages that shall be supported in this profile.

Examples:

IF CSA OR CSB OR CSC THEN "following table":

| Mandatory packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported (NOTE) |
| "IP NAT traversal package" [ITU-T H.248.37] | ipnapt (0x0099) | v1 | IP |
| "MG Act-as STUN server package" [ITU-T H.248.50] | mgastuns (0x00c2) | v1 | IP |
| "Originate STUN continuity check package" [ITU-T H.248.50] | ostuncc (0x00c3) | v1 | IP |
| "STUN consent freshness package" [ITU-T H.248.50] | stnconfres (0x0120) | v1 | IP |
| "TLS basic session control package" [ITU-T H.248.90] | tlsbsc (0x0117) | v1 | IP; Stream type: "DTLS-enabled UDP or TCP connection" |
| "SCTP basic connection control package" [ITU-T H.248.97] | sctpbcc (0x0121) | v1 | IP; Stream type: "DTLS-enabled L4 connection" |
| "SCTP re-configuration stream reset package" [ITU-T H.248.97] | sctpreset (0x0122) | v1 | IP; Stream type: "DTLS-enabled L4 connection" |
| "Media grouping package" [ITU-T H.248.96] | mgroup (0x011f) | v1 | IP |
| NOTE – Termination types: an H.248 profile might further differentiate IP-based H.248 terminations in "WebRTC" and "non-WebRTC" terminations (see also clause 8.12). This note applies also for the tables in following clause 11.14.2. |  |  |  |

### 11.14.2	Optional Packages

Examples:

IF CSA OR CSB THEN "following table":

| Optional packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| "Stream endpoint interlinkage package" [ITU-T H.248.92] | seplink (0x011b) | v1 | IP |
| "TLS capability negotiation package" [ITU-T H.248.90] (NOTE 1) | tlscn (0x0118) | v1 | IP |
| "TLS session maintenance package" [ITU-T H.248.90] (NOTE 2) | tlsm (0x0119) | v1 | IP |
| NOTE 1 – Applied for DTLS-enabled Stream endpoints (in context of the SCTP/DTLS/UDP protocol layering). NOTE 2 – Related to DTLS layer. |  |  |  |

Examples:

IF CSA OR CSB OR CSC THEN "following table":

| Optional packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| "Secure RTP Package" [ITU-T H.248.77] (NOTE) | srtp (0x0107) | v2 | IP |
| NOTE – Selection of key management scheme for SRTP. |  |  |  |

Examples:

IF CSA OR CSD THEN "following table":

| Optional packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| "MGC controlled bearer level ALG package" [ITU-T H.248.78] (NOTE 1, NOTE 2) | mcbalg (0x0108) | v2 | IP |
| "Data channel establishment protocol support package" [this Recommendation] (NOTE 1) | dcep (0x0124) | v1 | IP |
| NOTE 1 – There are two options related to the end-to-end control of WebRTC data channels: a) "out-of-band" method using call control signalling, or b) "inband" method. The first approach seems to be default for WebRTC support in SIP networks (such as IMS), and the baseline in this Recommendation. H.248 WebRTC gateways which would need to support the second method would require an H.248 profile with dcep v1 and mcbalg v2 packages support. NOTE 2 – In case of CLUE-based WebRTC conferencing control. |  |  |  |

Examples:

IF CSA THEN "following table":

| Optional packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| "Address reporting package" [ITU-T H.248.37] | adr (0x00ac) | v1 | IP |
| "Statistics for discarded packets due to latching package" [ITU-T H.248.37] | lstat (0x00e4) | v1 | IP |
| "Advanced SDP wildcarding package" [ITU-T H.248.39] (NOTE 1) | aswp (0x011c) | v1 | IP |
| "TLS traffic volume metrics package" [ITU-T H.248.90] (NOTE 2) | tlstv (0x011a) | v1 | IP |
| NOTE 1 – When MGC and MG agree to benefit from advanced SDP wildcarding. NOTE 2 – Related to DTLS layer. |  |  |  |

### 11.14.3	Package usage information

The following is a non-exhaustive list of package usage indications.

#### 11.14.3.1	IP NAPT traversal package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provision-ed value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| Latching (ipnapt/latch, 0x0099/0x0001) | M | ADD, MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | napt (0x0001) | M | ALL | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.2	MG act-as STUN server package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Act-as STUN Server (mgastuns/astuns, 0x00c2/0x0001) | M | ADD, MOD | ALL | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.3	Originate STUN continuity check package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Host Candidate Realm (ostuncc/hcr, 0x00c3/0x0001) | O | ADD, MOD | ALL | Yes | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| Send Connectivity Check (ostuncc/scc, 0x00c3/0x0001) | M | ADD. MOD |  | not applicable |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | cntrl (0x0001) | O | controlling, controlled | not applicable |  |
| Send Additional Connectivity Check (ostuncc/sacc, 0x00c3/0x0002) | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
|  | M | MOD |  | not applicable |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | cntrl (0x0001) | O | controlling, controlled | not applicable |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| Connectivity Check Result (ostuncc/ccr, 0x00c3/0x0001) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | ctp (0x0001) | M | ALL | not applicable |  |
| New Peer Reflexive Candidate (ostuncc/nprc, 0x00c3/0x0002) | Mandatory/ Optional | Used in command: |  |  |  |
|  | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | can (0x0001) | M | ALL | not applicable |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.4	STUN consent freshness package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| Consent Test (stnconfres/contst, 0x0120/0x0001) | M | ADD. MOD |  | not applicable |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | tstint (0x0001) | O | ALL | '5000 ms' |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| Consent State (stnconfres/constate, 0x0120/0x0001) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | reqstate (0x0001) | O | ALL | 'B' |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | state (0x0001) | M | ALL | not applicable |  |
| STUN Consent Request Failure (stnconfres/confail, 0x0120/0x0002) | Mandatory/ Optional | Used in command: |  |  |  |
|  | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.5	TLS basic session control package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Incoming security session establishment blocking (tlsbsc/bceb, 0x0117/0x0001) | O (NOTE 1) | ADD, MOD | ALL | "Un-blocked " | DTLS |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| Establish BNC (tlsbsc/EstBNC, 0x0117/0x0001) | M | ADD. MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Release BNC (tlsbsc/RelBNC, 0x0117/0x0002) | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
|  | O (NOTE 2) | ADD, MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| TLS session state change (tlsbsc/BNCChange, 0x0117/0x0001) | O (NOTE 3) | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | Type | M | Est (0x01), Rel (0x05) | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | Type | M | Est (0x01), Rel (0x05) | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |
| NOTE 1 – Required for e.g. too early incoming DTLS messages (due to security threat), or delayed DTLS session establishment (due to multiple SDP offer/answer cycles, H.248 two-stage resource reservation, to await firstly successful L4 connectivity, etc.). NOTE 2 – When the MGC wants to explicitly trigger the DTLS bearer session release procedure (instead of the implicit trigger related to the removal of the H.248 Stream (via a MODify.request or SUBtract.request command)). NOTE 3 – When the MGC wants to monitor the execution of DTLS bearer control procedures. |  |  |  |  |  |

#### 11.14.3.6	SCTP basic connection control package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Incoming bearer connection establishment blocking (sctpbcc/bceb, 0x0121/0x0001) | O (NOTE 1) | ADD, MOD | ALL | "Un-blocked " | SCTP |
| SCTP StreamID (sctpbcc/sctpid (0x0121/0x0002) | M | ADD, MOD | ALL | None. | SCTP |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| Establish BNC (sctpbcc/EstBNC, 0x0121/0x0001) | M | ADD. MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Release BNC (sctpbcc/RelBNC, 0x0121/0x0002) | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
|  | O (NOTE 2) | ADD, MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| SCTP connection state change (sctpbcc/BNCChange, 0x0121/0x0001) | O (NOTE 3) | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | Type | M | Est (0x01), Rel (0x05) | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | Type | M | Est (0x01), Rel (0x05) | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |
| NOTE 1 – Required for blocking incoming SCTP Association establishment requests. NOTE 2 – When the MGC wants to explicitly trigger the SCTP Association shutdown procedure (instead of the implicit trigger related to the removal of the H.248 Stream (via a MODify.request or SUBtract.request command)). NOTE 3 – When the MGC wants to monitor the execution of SCTP bearer control procedures. |  |  |  |  |  |

#### 11.14.3.7	SCTP Re-configuration Stream Reset package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| Initiate Outgoing SCTP Stream Reset (sctpreset/initreset, 0x0122/0x0001) | M | ADD. MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | sctpid (0x0001) | M | ALL | None. |  |
| Outgoing SCTP Stream Reset Response (sctpreset/resetresp, 0x0122/0x0002) | M | ADD. MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | sctpid (0x0001) | M | ALL | None. |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| Detect outgoing SCTP stream reset (sctpreset/detreset, 0x0122/0x0001) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | outresp (0x0001) | O | ALL | “accept” |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | sctpid (0x0001) | M | ALL | None. |  |
| Outgoing SCTP stream reset result (sctpreset/result, 0x0122/0x0002) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | None. | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | sctpid (0x0001) | M | ALL | None. |  |
|  | result (0x0002) | M | ALL | None. |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.8	Media grouping package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Group Semantics (mgroup/groupse, 0x011f/0x0001) | M | ADD, MOD | 'BUNDLE’, SCTP’ | None | ALL (NOTE 1) |
| Stream Aggregation (mgroup/stragg, 0x011f/0x0002) | M (NOTE 2) | ADD, MOD | ALL | None | ALL (NOTE 1) |
| Stream De-aggregation (mgroup/strdeagg, 0x011f/0x0003) | M (NOTE 3) | ADD, MOD | ALL | None | ALL (NOTE 1) |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| #489 | M |  |  |  |  |
| NOTE 1 – Some stream grouping semantic values are only applicable for specific protocols (stacks). NOTE 2 – Required for RTP media multiplexing ('BUNDLE'). NOTE 3 – Required for WebRTC data channel stack. |  |  |  |  |  |

#### 11.14.3.9	Data channel establishment protocol support package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| Send DCEP Open (dcep/sndopen, 0x0124/0x0001) | M | ADD. MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | sctpid (0x0001) | M | ALL | None. |  |
|  | protocol (0x0002) | M | ALL | None. |  |
|  | label (0x0003) | O | ALL | None. |  |
|  | chtype (0x0004) | M | ALL | None. |  |
|  | reli (0x0005) | M | ALL | None. |  |
|  | priority (0x0006) | M | ALL | None. |  |
| DCEP Open Response (dcep/ dcopenresp, 0x0124/0x0002) | M | ADD. MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | sctpid (0x0001) | M | ALL | None. |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| Detect DCEP Messages (dcep/detmess, 0x0124/0x0001) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | None. | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | sctpid (0x0001) | M | ALL | None. |  |
|  | protocol (0x0002) | M | ALL | None. |  |
|  | label (0x0003) | O | ALL | None. |  |
|  | chtype (0x0004) | M | ALL | None. |  |
|  | reli (0x0005) | M | ALL | None. |  |
|  | priority (0x0006) | M | ALL | None. |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.10	MGC controlled bearer level ALG package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| Send bearer level message (mcbalg/sblm, 0x0108/0x0001) | M | ADD. MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | Message content (mc) | M | ALL | - |  |
|  | Sent application protocol (sap) | O | ALL | - |  |
|  | ## Label (lbl) | O | ALL | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| Detect bearer level message (mcbalg/det, 0x0108/0x0001) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | Protocol Filter (pf) | M | ALL (NOTE) | ## - |  |
|  | Message Filter (mf) | O | ALL | "*" |  |
|  | Forwarding Flag (ff) | O | ALL | FALSE |  |
|  | Enhanced Protocol Filter (ehpf) | O | - | - |  |
|  | Label (lbl) | O | ALL | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | Message content (mc) | M | ALL | - |  |
|  | Detected protocol (dtp) | O | ALL | - |  |
|  | Label (lbl) | O | ALL | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |
| NOTE – At least codepoint '2855' (MSRP) is in scope of Release 1 of this Recommendation. |  |  |  |  |  |

#### 11.14.3.11	Stream endpoint interlinkage package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Interlinkage topology (seplink/linktopo, 0x011b/0x0001) | M | ADD, MOD | ALL | _ | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| #488 | - |  |  |  |  |

#### 11.14.3.12	MG located bearer level ALG package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Protocol type bearer level ALG (mgbalg/ptbalg, 0x011d/0x0001) | M | ADD, MOD | ALL | _"OFF" | - |
| Upper layer protocol filter (mgbalg/ulpf, 0x011d/0x0002) | O (NOTE) | ADD, MOD | ALL | WebRTC related codepoints only. | "0" |
| Upper Layer Enhanced Protocol Filter (mgbalg/ulepf, 0x011d/0x0005) | O (NOTE) | ADD, MOD | ALL | WebRTC related codepoints only. | - |
| Source of replaced source address information part (mgbalg/sosaip, 0x011d/0x0003) | O (NOTE) | ADD, MOD | ALL | "SD" | - |
| Source of replaced destination address information part (mgbalg/sodaip, 0x011d/0x0004) | O (NOTE) | ADD, MOD | ALL | "SD" | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |
| NOTE - When B-ALG service configuration is provisioned in H.248 MG. |  |  |  |  |  |

#### 11.14.3.13	Secure RTP package

Not supported in Release 1 of this Recommendation.

#### 11.14.3.14	Advanced SDP wildcarding package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Advanced Wildcarding Support (aswp/aws, 0x011c/0x0001) | M | AuditValue | ALL | - | Root only |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.15	TLS capability negotiation package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| TLS Domain Profile Identifier (tlscn/dpid, 0x0118/0x0001) | O (NOTE 1) | ADD, MOD, AUDITVALUE | ALL | _ | Root or non-Root '(D)TLS', but not both |
| TLS Versions (tlscn/tlsv, 0x0118/0x0002) | O (NOTE 1) | ADD, MOD, AUDITVALUE | ALL | _ | Root or non-Root '(D)TLS', but not both |
| Chipher Suites (tlscn/cs, 0x0118/0x0003) | O (NOTE 1) | ADD, MOD, AUDITVALUE | ALL | _ | Root or non-Root '(D)TLS', but not both |
| Compression Methods (tlscn/cm, 0x0118/0x0004) | O (NOTE 1) | ADD, MOD, AUDITVALUE | ALL | _ | Root or non-Root '(D)TLS', but not both |
| Support for Renegotiation of the Security Context (tlscn/srsc, 0x0118/0x0005) | O (NOTE 1) | ADD, MOD, AUDITVALUE | ALL | _ | Root or non-Root '(D)TLS', but not both |
| Renegotiation Period (tlscn/rp, 0x0118/0x0006) | O (NOTE 1) | ADD, MOD, AUDITVALUE | ALL | _ | Root or non-Root '(D)TLS', but not both |
| Client Authentication Required (tlscn/car, 0x0118/0x0007) | O (NOTE 1) | ADD, MOD, AUDITVALUE | ALL | _ | Root or non-Root '(D)TLS', but not both |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |
| NOTE 1 – When tagged as 'optional', then property value(s) needs to be provisioned. |  |  |  |  |  |

#### 11.14.3.16	TLS session maintenance package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| MGC triggered TLS error alert (tlsm/mgcea, 0x0119/0x0001) | M | ADD. MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | al (0x0001) | M | ALL | - |  |
|  | ad (0x0002) | M | ALL | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| TLS error alert (tlsm/mgea, 0x0119/0x0001) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | None. | - | - | ## - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | blai (0x0001) | M | ALL | - |  |
|  | eat (0x0002) | M | ALL | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.17	TLS traffic volume metrics package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| NOTE 1 (tlstv/*, 0x011a/*) | O | ADD, MOD, SUBTRACT, NOTIFY | ALL |  | (D)TLS enabled |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |
| NOTE 1 – There are twenty statistics available, where usually just a subset is often sufficient. See clause 12.6.1.3/[ITU-T H.248.90] on "Profile selection guidelines". |  |  |  |  |  |

#### 11.14.3.18	Address reporting package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Current Remote Transport Address Value (adr/crta, 0x00ac/0x0001) | M | ADD, MOD | ALL | None. | IP |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| Remote Transport Address Change (adr/rtac, 0x00ac/0x0003) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | None. | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | nrta (0x0001) | M | ALL | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.19	Latch statistics package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| Discarded Packets (lstat/dp, 0x00e4/0x0001) | M | ADD, MOD, SUBTRACT, NOTIFY | ALL |  | IP |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

## 11.15	Mandatory support of SDP and ITU-T H.248.1 Annex C information elements

At least the following:

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| "m="-line <type> | not supported (in this example) | Value(s): - application-aware: 'audio', 'video and 'application' - application-agnostic: '-' |
| "m="-line <proto> | not supported (in this example) | Value(s): 'RTP/SAVPF' Purpose: H.248 components streams at H.248 WebRTC termination for media types 'audio' and 'video'. Value(s): '-' (if "m="-line inserted in Stream Descriptor) Purpose: H.248 components streams at H.248 WebRTC termination for media type 'application'. Value(s): 'DTLS/SCTP' or '…/DTLS/SCTP' Purpose: H.248 deaggregation streams at H.248 WebRTC termination for media type 'application'. |
| "m="-line <fmt> | not supported (in this example) | Value: 'webrtc-datachannel' |
| "a=fingerprint:" | not supported (in this example) | Value(s): all Purpose: DTLS authentication procedures |
| " a=sctp-port" |  | Value(s): all Purpose: The actual SCTP port value of the SCTP Association carried over DTLS/UDP, which is used for "WEBRTC data channels" |
| "a= max-message-size:" |  | Value(s): all Purpose: The maximum message size (indicated in bytes) that an SCTP Stream endpoint is willing to receive on the SCTP Association, which is used for "WEBRTC data channels". |
| "a=dcmap:<SCTP Stream ID>" |  | Value(s): all Purpose: application-agnostic WebRTC data channels |
| "a=dcmap:<SCTP Stream ID> subprotocol="…"" |  | Value(s): MSRP (NOTE 1) Purpose: application-aware WebRTC data channels |
| "a=dcsa:…" |  | Value(s): all Purpose: application-specific information of a particular subprotocol asWEBRTC data channel |
| ## NOTE 1 – The additional potential values "t140" and "BFCP" are currently under discussion, thus out of scope of this Release. |  |  |

Rest: subject to profile specification.

## 11.16	Optional support of SDP and ITU-T H.248.1 Annex C information elements

At least the following:

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| "a=setup:" | not supported (in this example) | Not applicable for H.248 Streams of H.248 WebRTC terminations (NOTE 1). |
| "a=rtcp-rsize:" | not supported (in this example) | Value(s): no value Purpose: support of reduced size RTCP (NOTE 2) |
| "a=rtcp-fb:" | not supported (in this example) | Value(s): for further studies (NOTE 3) Purpose: RTCP feedback based services for a specific WebRTC call. |
| NOTE 1 – Call control signalling level "a=setup:" information is mapped by the MGC on H.248 Signal/Event-based bearer control procedures. NOTE 2 – Support of reduced size RTCP is mandatory for WebRTC clients, but has to be negotiated call-individually due to backward compatibility (see clause 4.6/[b-IETF rtp usage]). Thus, the MGC could require every new WebRTC call to use "compound RTCP package" mode only. More detailed H.248 profile guidelines are subject of a future release of this Recommendation. NOTE 3 – There are multiple potential RTCP feedback based services for WebRTC, see [b-IETF rtp-usage]. The identification of rtcp-fb codepoints for WebRTC deployments is a subject for a future release of this Recommendation. Furthermore, there might be a split of mandatory and optional RTCP feedback message types in future for WebRTC. |  |  |

IF CSA OR CSB OR CSC THEN "following table":

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| "a=ice-ufrag:" | not supported (in this example) | Value(s): all Purpose: |
| "a=ice-pwd:" |  | Value(s): all Purpose: |

IF CSA OR CSB THEN "following table":

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| "a=group:BUNDLE" | not supported (in this example) | Value(s): "BUNDLE" Purpose: general media grouping using "bundle" H.248 usage: SDP attribute not used, but value mapped on [ITU-T H.248.96] property mgroup/groupse in order to indicate H.248 stream grouping. |
| "a=mid:" |  | Value(s): not used Purpose: general media grouping using "bundle" H.248 usage: correspondent SDP media descriptions are mapped on individual H.248 component streams, see clause 8.6.2/[ITU-T H.248.96] |
| "a=rtcp-mux": |  | Value(s): all Purpose: for transport multiplexed RTP traffic |
| "a=rtcp:" |  | Value(s): all Purpose: optional, if transport unmultiplexed mode for RTP and explicit RTCP transport address allocation |
| "a=ssrc:" |  | Value(s): all Purpose: for RTCP supplementary service with source specific information. |

Rest: subject to profile specification.

## 11.17	Procedures

The initial release of this Recommendation focuses on the previous profile elements. Specific guidelines for the procedural section are for further studies with exception of following package-independent procedures.

### 11.17.1	Package-independent NAT-T procedures

#### 11.17.1.1	Support for MG terminated STUN-based connectivity checks

IF CSA OR CSB OR CSC THEN the procedures according clause 10.1 of [ITU-T H.248.50].

### 11.17.2	Example procedures for communication establishment phase

WebRTC as a multimedia communication service is characterized by the fact that individual media components might be dynamically added and removed during the lifetime of the overall WebRTC call. There are consequently many variations of media configurations how a WebRTC call might start and evolve over the timeline. E.g., a later added media component might somehow "reuse gateway resources" or start from scratch. Some high level use cases are outlined.

#### 11.17.2.1	Use case #1: audio & video only, unbundled

It is expected that the majority of WebRTC calls will always use at least audio due to its positioning as "conversational / telephony" service. The likelihood of video might be in the same range or lower. Thus, a WebRTC call would typically start as "audio only" or "audio & video only", - and if "audio only" then video might be added (and removed again) at a later point in time.

Table 10 indicates the main use case #1.0 (as further discussed below) as well as some example variations:

Table 10 – Use case #1 and variations

| UC: | Characteristic: | Comments |
| --- | --- | --- |
| #1.0 | - audio & video only - unbundled (i.e., no RTP media multiplexing) - no RTP transport multiplexing - media security model: "e2ae" - IWF "audio": transcoding (TC) - IWF "video": transparent forwarding (TF) - NAT-T (AN side): stable connectivity (i.e., no ICE updates) - SRTP key exchange: only once, DTLS connection remains established in order not to lose the security context - DTLS: non-resumable DTLS session | Number of L4 connections: - 4 (2 x RTP and 2 x RTCP) - ICE/STUN procedures: four times, for each L4 connection Number of DTLS connections: - 4 (because non-resumable DTLS sessions) - thus, 4 DTLS full handshakes, leading to four DTLS connections - Enhancement option: a single resumable DTLS session … Number of SRTP key exchange procedures: - 4 (SRTP master key negotiation for each RTP and RTCP stream) - Enhancement option: SRTCP keys derived from SRTP keys |
| #1.1 | as #1.0 with following change: - media security model: "e2e" | i.e., SRTP-to-SRTP interworking in a back-to-back RTP endsystem (B2BRE) topology with two different SRTP key management schemes (H.248 Terminations T1: DTLS-SRTP; T2: SDES) |
| #1.2 | as #1.0 with following change: - DTLS connection remains in "data transfer ready state" |  |
| #1.3 | as #1.0 with following change: - IWF "audio": also transparent forwarding |  |

Figure 5 outlines a correspondent H.248 Context model for use case category #1 (for the case of a IMS-embedded WebRTC):

Figure 5 – H.248 WebRTC gateway – H.248 Context model – 
Use case #1: audio & video only, unbundled audio & video

H.248 Termination T1 is connected with the "WebRTC domain", Termination T2 to the "non-WebRTC domain". There are two H.248 Streams for audio and video. Each H.248 Stream contains two UDP connections (for RTP and RTCP). ICE/STUN procedures need to be executed on each individual UDP connection. After successful STUN connectivity checks, then the DTLS connections need to be established for SRTP key exchange.

The worst case (and the assumption here) is the usage of non-resumable DTLS sessions, which lead to the execution of full DTLS handshake procedures on each UDP connection, resulting in finally four DTLS connections.

Figure 6 outlines an example signalling flow:

Figure 6 – Example signalling flow for use case #1.0

Observations/discussion of selected signalling steps:

| Step: | Comments |
| --- | --- |
| 15 | The establishment (incoming or outging) of the DTLS (non-resumable) session / DTLS connections is here explicitly triggered, dependent on successfully reported L4 connectivity on all four UDP connections between UE and MG. Thus, alternatively step 15 might be merged into step 2. |

#### 11.17.2.2	Use case #2: audio & video only, bundled

Additional bundling requires support of H.248 stream grouping based Context models. Table 11 indicates the main use case #2.0 as well as some example variations:

Table 11 – Use case #2 and variations

| UC: | Characteristic: | Comments |
| --- | --- | --- |
| #2.0 | as #1.0 with following changes: - bundled (i.e., RTP media multiplexing, also known as RTP SSRC multiplexing) - also RTP transport multiplexing | Number of L4 connections: - 1 (both RTP media flows and associated RTCP control flows multiplexed on a single UDP connection) - ICE/STUN procedures: once Number of DTLS connections: - 1 Number of SRTP key exchange procedures: - 1 (SRTP master key negotiation and then key derivation for all RTP media and RTCP control flows) |
| #2.1 | as #2.0 with following change: - media security model: "e2e" |  |

Figure 7 shows two options of a correspondent H.248 Context model for use case category #2:

Figure 7 – H.248 WebRTC gateway – H.248 Context model – 
Use case #2: audio & video only, bundled audio & video

Bundling implies the usage of H.248 stream grouping with [ITU-T H.248.96] semantic "BUNDLE" (which indicates to the MG that multiple H.248 streams share common resources). There are two H.248 streams in case of option 1. The H.248 Stream Descriptor of each stream describes the RTP media and RTCP control flow components for a particular media type. However, there is only a single UDP connection, thus single ICE/STUN procedures and a single DTLS session as well, inclusive a single DTLS connection and also only a single SRTP key exchange procedure, which are shared by audio and video. This leads to the question whether ICE, STUN, DTLS and SRTP related information should be associated to H.248 Stream T1(S1) or T1(S2) (in case of option 1). Option 1 is basically feasible, an H.248 profile specification should indicate a rule where the DTLS and lower layer procedures would be signalled via H.248. E.g., using the H.248 Stream with the lowest StreamID value.

That issue could also be addressed by a dedicated H.248 Stream T1(S3), see option 2. The [ITU-T H.248.96] "BUNDLE" stream group would then cover all three H.248 Streams due to the common usage of the same and single UDP connection.

Observations/discussion of selected signalling steps, - versus use case #1.0 (Figure 5):

| Step: | Comments |
| --- | --- |
| 2, 3 | H.248 stream group SG1 is created, containing three streams. |
| 13 | SDP offer with "media bundled" forwarded |
| 16 | Start of DTLS procedures at Stream endpoint T1(S3) |
| 25 | Active communication phase: - audio IWF in transcoding mode - video IWF in transparent forwarding mode; This semantic implies that the media format for video is not changed (despite the other SRTP-to-RTP interworking functions) |

#### 11.17.2.3	Use case #3: additional data component(s), preparation of transport

The description of a protocol stack for WebRTC data and associated H.248 Stream/Termination models requires the usage of the H.248 media grouping (mgroup) package.

The initial SDP Offer could contain a media description for WebRTC data but still miss information elements for a concrete WebRTC data application (i.e., the SDP attributes "a=dcmap" and "a=dcsa" are omitted in the SDP Offer). That's a valid scenario because the WebRTC user may want to start with audio only or audio/video telephony first before a potential later addition of a data service.

Independent of the SIP-level signalling for the establishment of a WebRTC call, the MGC could prepare the H.248 Context for a later usage of additional WebRTC data. The level of preparation of correspondent MG resources is usually dependent on MGC local policies (e.g., traffic distribution between WebRTC calls with and without data, expected WebRTC data application(s), expected NAT traversal support at application protocol level, etc.) as well as the amount of SDP information signalled in call control signalling.

Table 12 indicates the main use case #3.0 as well as some example variations:

Table 12 – Use case #3 and variations

| UC: | Characteristic: | Comments |
| --- | --- | --- |
| #3.0 | - SIP: SDP Offer indicates "…/DTLS/SCTP" transport, but still missing SDP attributes for a WebRTC data application - L4 protocol: UDP, expected successful NAT traversal - single SCTP Association per DTLS connection (default for WebRTC) - preparation of three H.248 Streams for potential future WebRTC data applications in the H.248 WebRTC gateway; the preparation includes already the local reservation and assignment of SCTP StreamIDs for each DC candidate | - the preparation and establishment of a DTLS session/DTLS connection for SCTP traffic is already feasible - the preparation and establishment of an SCTP Association after successful DTLS connectivity is also feasible - however, the preparation of local DC resources is conditional, dependent on pro-active or on-demand resource management strategies of MGW resources - the pro-active strategy is selected for this use case due to expected later WebRTC data applications |
| #3.1 | as #3.0 with following change: - no preparation of WebRTC data channels | - thus, only the H.248 deaggregation stream will be established, but no H.248 component streams for DCs are be created |

The purpose of the consideration of the two use cases #3.0 and #3.1 is to emphasize the feedback on the H.248 handling of Stream Group SG2: the size is fixed and static in #3.0, but dynamic in #3.1 (NOTE -  'dynamic' implies future H.248 modifications of the TerminationState property mgroup/groupse of T1).

Figure 8 illustrates a correspondent H.248 Context model for use case category #3.0:

Figure 8 – H.248 WebRTC gateway – H.248 Context model – 
Use case #3.0: additional data, preparation of transport

H.248 Stream endpoint T1(S4) represents the H.248 deaggregation stream. The protocol stack segment "SCTP Association over DTLS connection over L4 over IP" is allocated to this SEP. The three potential future WebRTC data channels would be provided by the three H.248 component streams S5, S6 and S7 at Termination T1. There are no SEP counterparts at T2 (in this example). The four H.248 Streams for WebRTC data are denoted as Stream Group SG2 in Figure 6. The name SG2 is only used for illustration purposes; it does not appear in any H.248 signalling syntax.

Figure 9 outlines an example signalling flow:

Figure 9 – Example signalling flow for use case #3.0

Observations/discussion of selected signalling steps:

| Step: | Comments |
| --- | --- |
| 0 | Context C1 and Termination T1 already exists as a result of the previous establishment of audio/SRTP and video/SRTP streams. There are three major types concerning the start of a WebRTC call: O1) audio & video only, without any data indication at all; O2) audio & video, plus indication of data but not yet any specific data application; and O3) audio, video & data. The example in Figure 9 provides a scenario from category "initial audio and video only without any initial data indication at all" (due to the history of Context C1). However, the option "of an initial, additional indication of an application-agnostic data indication" seems to be more likely from WebRTC client perspective, hence that option should be considered from profile specfication perspective rather than the first scenario. It has to be noted that the third option of a WebRTC call starting already with "audio, video plus data" is applicable in general,. |
| 2 | NOTE – The concrete H.248 command type depends on the signalling history (e.g. a MOD.req instead of an ADD.req). However, that level of detail is out of scope of this Recommendation. Signalled information: protocol stack segment indication, ICE/STUN info (as well as explicit notification about successful STUN connectivity check), stream group SG2. With regards to H.248 stream group SG2: - H.248 deaggregation stream T1(S4) could be already fully specified (apart from the usual H.248 wildcarding of local MG resources); - the three H.248 component streams T1(S5, S6 & S7) would be prepared by signalling at least the binding information between H.248 Stream Identifier and SCTP Stream Identifier (i.e., [ITU-T H.248.97] property sctpbcc/sctpid in the LocalControl Descriptor of each component stream; SCTP StreamID value might be wildcarded or explicitly provided by MGC) |
| 8 | The MGC is notified upon successful L4 connectivity (here: one UDP connection for SCTP-over-DTLS because the UDP connection is NOT shared with audio and video in this use case). NOTE – Step 8 dependent on 9, see next row. |
| 9 | The establishment of the DTLS session/connection is here explicitly triggered (using the [ITU-T H.248.90] tlsbsc package) by the MGC. That request could be of course already be combined with step 2. The DTLS protocol profile (actually the "DTLS domain profile", see [ITU-T H.248.90]) is out of scope here, i.e., expected to be pre-provisioned in the MG. For instance, a non-resumable DTLS session, etc. |
| 12, 13 | The establishment direction of the DTLS connection is out of scope here. |
| 14 | The MGC is notified of the successful establishment of the DTLS connection (i.e., the local (DTLS)-connection-endpoint is in state "ESTABLISHED"). |
| 15 | The establishment of the SCTP Association is here explicitly triggered (using the [ITU-T H.248.97] sctpbcc package) by the MGC. That request could be of course already combined with step 9, or the usage of [ITU-T H.248.92] Stream endpoint interlinkage might be applied. |
| 18, 19 | The establishment direction of the SCTP Association is out of scope here. |
| 20 | The MGC is notified about the successful establishment of the SCTP Association (i.e., the local (SCTP)-association-endpoint is in state "ESTABLISHED"). |

#### 11.17.2.4	Use case #4: dynamic establishment of data channels

There is already an existing end-to-end WebRTC communication with audio and video and a prepared "tunnel" (realized as SCTP Association over a DTLS connection) for data. A WebRTC endpoint or WebRTC-compatible endpoint wants to add a first data application, for instance MSRP-based instant messaging.

Table 13 indicates the main use case #4.0 as well as some example variations:

Table 13 – Use case #4 and variations

| UC: | Characteristic: | Comments |
| --- | --- | --- |
| #4.0 | - SIP reINVITE: SDP Offer provides a media description containing at least, e.g.: m=application <…> UDP/DTLS/SCTP webrtc-datachannel a=dcmap:1 label="…"; subprotocol="MSRP" … - H.248 component stream S5 (of SG2) is used for MSRP traffic | The use case highlights following aspects: - Data channel "open": no usage of the DCEP (Data Channel Establishment Protocol) due to the alternative of complete out-of-band DC signalling; - a (WebRTC)-DC-endpoint could immediately send data, which implies the allocation of buffer resources in the MG; - H.248 WebRTC gateway interworking function: "MSRP transparent forwarding" or with a B-ALG function in case of L4+ NAT-T support. |
| #4.1 | as #4.0 with following change: - #3.1 as the starting point (i.e., there wasn't yet any preparation of the "tunnel" in the user plane, thus no H.248 stream group SG2 …) |  |
| #4.2 | as #4.0 with following change: - immediate establishment of three (instead of just one) WebRTC data channels |  |

Figure 10 shows an example H.248 Context model for use case #4.0:

Figure 10 – H.248 WebRTC gateway – H.248 Context model – 
Use case #4.0: first data channel in service

H.248 Stream endpoint T1(S5) represents the H.248 component stream for "MSRP" in the WebRTC domain. Figure 11 outlines an example signalling flow:

Figure 11 – Example signalling flow for use case #4.0

Observations/discussion of selected signalling steps:

| Step: | Comments |
| --- | --- |
| 0 | Context C1 and Termination T1 already exist due to the previous establishment of audio/SRTP and video/SRTP streams as well as Stream Group SG2 for WebRTC data applications. ("same comment as in use case #3.0") |
| 1 | The SIP reINVITE / SDP Offer provides a media description containing at least, e.g.: m=application <…> UDP/DTLS/SCTP webrtc-datachannel a=dcmap:1 label="…"; subprotocol="MSRP" |
| 2 | H.248 Stream endpoint T1(S5): the associated SCTP Stream is configured. NOTE – Any originally allocated local SCTP StreamID value might be overwritten or renewed ("in case of wildcarding"). |
| 5, 11 | H.248 Stream endpoint T2(S5): the (MSRP(/TLS)/TCP)-connection-endpoint is configured |

Figure 12 illustrates the H.248 Context model in case of three established and active H.248 component streams for WebRTC data services (NOTE – DC application T.140 and BFCP are subject of a future Release of this Recommendation):

Figure 12 – H.248 WebRTC gateway – H.248 Context model – 
Use case #4.2: three data channels in service

A correspondent signaling example for use case #4.2 is not discussed further.

#### 11.17.2.5	Use case #5: data IWF and B-ALG for MSRP

MSRP-based instant messaging allows also to demonstrate: a) the additional usage of an (optional) B-ALG (Bearer-level Application Level Gateway; see [ITU-T H.248.78]) function for L4+ NAT traversal support (due to address information at MSRP message level) and b) TCP-related interworking. Concrete use cases are not detailed in this Release of the Recommendation.

#### 11.17.2.6	Use case #6: MG enhancement – additional usage of stream interlinkage

[ITU-T H.248.92] is applicable in principle whenever a protocol uses explicit bearer control procedures. That's basically the case for H.248 WebRTC gateway interworking functions, given by connection-oriented protocols DTLS, SCTP (and possibly DCEP) and TCP (if used as L4 protocol) at an "H.248 WebRTC Termination", and TCP and TLS at the "H.248 non-WebRTC Termination". Concrete use cases with stream interlinkage are not detailed in this Release of the Recommendation.

### 11.17.3	Example procedures for release of data channels and call release

There are multiple levels of release procedures due to the hierarchical protocol stack as well as the nature of the communication service (see intial comments in clause 11.17.3.1). Only a few examples are discussed in this Recommendation (clauses 11.17.3.2 to 11.17.3.4).

#### 11.17.3.1	Service cancellation for WebRTC data in H.248 WebRTC media gateways

The hierarchical protocol stack for WebRTC data ("DC/SCTP/DTLS/(UDP|TCP)/IP") contains up to four levels of user plane bearer control procedures, from top to bottom:

- DC: SCTP Stream reconfiguration procedures for resetting SCTP Streams;

- SCTP Association: shutdown procedure;

- DTLS: DTLS connection release procedure (apart from resumption and renegotiation procedures) and

- TCP (if not UDP): TCP connection release procedure.

There are basically two options of service cancellation, attributed as "forced" and "graceful" in the discussion here.

##### 11.17.3.1.1	Forced service cancellation for WebRTC data in H.248 WebRTC media gateways

A situation where the MGC removes an H.248 SEP for a particular protocol layer or protocol stack segment, a H.248 Stream Group or even the complete substraction of the Termination from the Context, without execution (by the MG) at all of the protocol specific bearer control procedures for releasing protocol endpoints. Such a forced service cancellation represents an incorrect service termination in the user plane because this will cause protocol failure indications at the remote endpoint.

##### 11.17.3.1.2	Graceful service cancellation for WebRTC data in H.248 WebRTC media gateways

This is the normal behaviour of the MG as a communication endpoint by executing bearer release procedures (by the MG) protocol correctly and in the right order. Some examples:

- the H.248 command request for releasing the DTLS connection (despite the fact of a still established SCTP Association and still open DCs) should firstly lead to the execution of a SCTP Association shutdown procedure before starting the DTLS connection release procedure;

- the H.248 command request for releasing the SCTP Association (despite the fact of still open data channels) might benefit from firstly resetting all SCTP Streams before starting the SCTP Association shutdown;

- the H.248 command request for subtracting the H.248 WebRTC termination (despite the fact of still open data channels).

#### 11.17.3.2	Use case #7: release of a data channel without call release

Data channels might be removed before the overall end of the WebRTC call. There are multiple variations possible because the "CLOSURE of a data channel" results in a reset of the SCTP Stream (see clause 6.7/[b-IETF WebRTCDC], i.e., the concerned SCTP Stream still exists. Such a resetted SCTP Stream could be basically reused again (for the same or another WebRTC data application). There are consequently two options from MGC perspective: the correspondent H.248 Stream endpoint is still kept, i.e., remains allocated and part of the H.248 Stream Group, or the H.248 Stream endpoint is completely removed from the termination.

Table 14 indicates the main use case #7.0 as well as some example variations:

Table 14 – Use case #7 and variations

| UC: | Characteristic: | Comments |
| --- | --- | --- |
| #7.0 | - existing H.248 Stream Group with three active data channels (i.e., three H.248 component streams) (see Figure 13, "Phase 0") - closing of 1-out-of-3 data channels - underlying SCTP Stream is kept after the SCTP Stream reset procedure ("Phase 1") - later, the MGC decides to remove also the unused SCTP Stream ("Phase 2") | - the level of detail concerning the directionality of the DC closure procedure is not considered here - MGC removes SEP at the non-WebRTC Termination T2, but keeps SEP at WebRTC Termination T1 (of the original H.248 SEPP) |
| #7.1 | as #7.0 with following change: - resetted SCTP is not intended to be reused, thus, the MGC removes H.248 SEP | - there are two options again: the H.248 Stream Group SG2 is either not modified or updated according the actual Stream Group size |
| #7.2 | as #7.0 with following change: - closing of 2-out-of-3 or 3-out-of-3 data channels | - the [b-IETF RFC 6525] SCTP Stream re-configuration procedure allows the reset of multiple streams in parallel … - … hence, that "SCTP bearer control procedure" is actually running at the H.248 deaggregation stream T1(S4) and not at the affected H.248 component streams |
| #7.3 | - release of all data channels and subsequent shutdown of the SCTP Association | - there are two options again: either the underyling DTLS connection is also immediately released or in future |

Figure 13 illustrates a correspondent H.248 Context model for use case #7.0 (using H.248 Stream S6 as example), highlighting the three phases of Context modification:

Figure 13 – H.248 WebRTC gateway – H.248 Context model – 
Use case #7.0: release of a data channel without call release

Figure 14 outlines an example signalling flow:

Figure 14 – Example signalling flow for use case #7.0

Observations/discussion of selected signalling steps:

| Step: | Comments |
| --- | --- |
| 1 | the SDP offer doesn't contain the SDP attributes of the orignal data channel and data application |
| 2 | the MGC maps the SDP on the [ITU-T H.248.97] sctpreset package elements, dependent on an incoming or outgoing SCTP Stream reconfiguration procedure |
| 15 | H.248 Stream Group SG2 is not modified as such, i.e., SEP T1(S6) still part of the group and allocated SCTP StreamID values (for potential future "reuse") |

#### 11.17.3.3	Use case #8: SCTP Association shutdown without DTLS connection release

There might be two reasons for keeping the DTLS connection after a shutdown of the SCTP Association: either the DTLS connection is still used for DTLS-SRTP or there again might be WebRTC DCs opened at a future point in time. Such an approach is possible however the establishment of a new DTLS connection represents a significant cost factor in terms of CPU cycles and memory, as well as introducing delay.

Table 15 indicates the main use case #8.0 as well as some example variations:

Table 15 – Use case #8 and variations

| UC: | Characteristic: | Comments |
| --- | --- | --- |
| #8.0 | - existing H.248 Stream Group with active data channels - closing of all data channels - subsequent shutdown of the SCTP Association without DTLS connection release | - the use case demonstrates the removal of all H.248 component streams of Stream Group SG2, and the modification of the H.248 deaggregation stream T1(S4) (which is kept due to the still established DTLS connection) |
| #8.1 | as #8.0 with following change: - but keeping the Stream Group SG2 definition, i.e., unused H.248 component streams) | - looks feasible from H.248 perspective ("a real world use case is dependent on "reuse debates" in context of WebRTC data discussions, hence out of scope or Release 1 of this Recommendation") |

Figure 15 discusses H.248 Context models for use case #8.0:

Figure 15 – H.248 WebRTC gateway – H.248 Context model – 
Use case #8.0: SCTP Association shutdown without DTLS connection release

Figure 16 outlines an example signalling flow:

Figure 16 – Example signalling flow for use case #8.0

Observations/discussion of selected signalling steps:

| Step: | Comments |
| --- | --- |
| 3 | the H.248 Property mgroup/groupse is removed from the TerminationState Descriptor of Termination T1 as well as property mgroup/strdeaggr is removed from the LocalControl Descriptor of SEP T1(S4) |
| 5, 6 | H.248 SEPs T1(S4, S5 & S6) are explicitly removed by the MGC |
| 17 | the former H.248 deaggreation streams becomes now a "normal" H.248 component stream T1(S4), which still contains the protocol stack segment "DTLS/IP/L2" |

#### 11.17.3.4	Use case #9: complete call release

Table 16 indicates the main use case #9.0 as well as some example variations:

Table 16 – Use case #9 and variations

| UC: | Characteristic: | Comments |
| --- | --- | --- |
| #9.0 | - all WebRTC data channels are closed and the underyling transport stack is completely released, but the H.248 Termination isn't subtracted | - see general comments in clause 11.17.3.1 |
| #9.1 | as #9.0 with following change: - subtraction of H.248 WebRTC Termination | - see general comments in clause 11.17.3.1 |

Figure 17 illustrates an example signalling flow for use case #9.0:

Figure 17 – Example signalling flow for use case #9.0

Observations/discussion of selected signalling steps:

| Step: | Comments |
| --- | --- |
| 9, 10 | NOTE - there might be a subsequent TCP connection relase in case of "L4 = TCP" |

Appendix I

Use case specific capability sets – Some examples

(This appendix does not form an integral part of this Recommendation)

## I.1	Overview

Profile content is mainly use case dependent (given be the inherent motivation of "protocol profiling"). In order to demonstrate profile specification guidelines in clause 11, this Recommendation considers some exemplary use cases (see Table I.1):

- Capability set 'A' (CSA) – a H.248 WEBRTC gateway as positioned in use case #1 (see clause 7.1.1):

Use case #1 is based on the assumption that the WebRTC endpoint could request and use all mandatory WebRTC capabilities as defined by the IETF / W3C. Thus, all mandatory and optional capabilities according to [b-IETF rtcweb-gateway] are basically in scope. The H.248 WebRTC gateway does then need to support all mandatory requirements as listed in clause 8. CSA also covers all optional features for enhanced gateway operation.

- Capability set 'B' (CSB) – a H.248 WebRTC gateway as positioned in use case #2 with scope on the NGN/IMS domain located WEBRTC endpoint (see clause 7.1.2).

- Capability set 'C' (CSC) – a H.248 WebRTC gateway again for use case #2, but a light weight capability set in comparison to CSB, which reflects the service focus at market introduction phase.

Capability set 'A' (CSA) is therefore a superset of capability set 'B' (CSB), and 'B' a superset of capability set 'C' (CSC). Hence, reflecting the usual evolution and phased introduction of new communication services.

Furthermore, besides pure H.248 WebRTC gateways with point-to-point connection models due to their network positioning at the edge, access or peering level:

- Capability set 'D' (CSD) – a H.248 WebRTC media server with primary scope on WebRTC-based conferencing topologies and support. The correspondent column in Table I.1 indicates a lot of capabilities as not applicable (n.a.), e.g. due to assumptions that any kind of NAT traversal support is already provided at the network access segment of the IP paths.

Table I.1 – Examples of use case specific capability sets (CS)

| Capability | CSA | CSB | CSC | CSD |
| --- | --- | --- | --- | --- |
| NAT-T I: ICE/STUN for UDP | X | X | X | n.a. |
| NAT-T II: ICE/STUN for TCP | X | X | X | n.a. |
| NAT-T III: ICE refreshes during active call phase | X | X | - | n.a. |
| NAT-T IV: ICE lite mode only | X | X | X | n.a. |
| NAT-T V: ICE full mode | X | - | - | n.a. |
| NAT-T VI: latching | X | X | X | n.a. |
| NAT-T VII: B-ALG for L4+ support for WebRTC data applications | X | X | X | n.a. |
| Multiplexing I: RTP transport multiplexing | X | - | - | X |
| Multiplexing II: RTP media multiplexing | X | - | - | X |
| Multiplexing III: UDP payload multiplexing | X | X | - | X |
| H.248 MG type I: WebRTC gateway | X | X | X | - |
| H.248 MG type II: WebRTC media server (media resource function) | - | - | - | X |
| WebRTC service profile I: audio & video only | X | X | X | X |
| WebRTC service profile II: audio, video and MSRP-based data only | X | X | X | X |
| WebRTC service profile III: audio & video and multiple data channels | X | X | - | X |
| WebRTC service profile IV: CLUE-based conferencing control | X | - | - | X |
| WebRTC service profile V: performance monitoring | X | - | - | - |
| WebRTC data channel control I: Out-of-band control | X | X | X | X |
| WebRTC data channel control II: Inband control | X | - | - | X |
| WebRTC data channel control III: reset of data channels | X | X | - | X |
| Multiple SRTP key management schemes (due to non-WebRTC SRTP) | X | X | X | X |
| Enhanced DTLS support (negotiation, maintenance & monitoring) | X | X | - | - |
| Enhanced H.248 control I: stream interlinkage support | X | X | - | n.a. |
| Enhanced H.248 control II: advanced wildcarding | X | X | - | n.a. |

Appendix II
Distributed Text-over-IP endpoints for WebRTC data 'text'

(This Appendix does not form an integral part of this Recommendation.)

## II.1	Purpose

The specific transport of T.140 text messages in WebRTC leads to a new interworking model required for WebRTC gateways, resulting in application-specific configurations of the "lower layer protocols" SCTP and RTP in the WebRTC and non-WebRTC domain, as well as dedicated support by the WebRTC gateway in partially emulating a virtual T.140 endpoint (i.e., protocol behaviour of [b-RFC 4103]). Such kind of MG-embedded interworking capabilities are usual implementation specific and out of scope of this Recommendation. The purpose of this Appendix is to summarize the problem statement and indicate potential solutions.

## II.2	Problem statement

Figure II.1 recalls the reference architecture for the definition of the [b-ITU-T V.151] text relay mode ("Text-over-IP", ToIP) in case of end-to-end communication between IP user equipment:

Figure II.1 – Starting point: legacy ToIP in all-IP networks

There are fundamental interactions between the application level framing protocol (i.e., RTP) and the application protocol (T.140), given by the network transport requirements for communication service "text conversation" (or realtime text).

Figure II.2 outlines the two main options which were considered for text conversation embedded in WebRTC:

Figure II.2 – peer-to-peer WebRTC: options for WebRTC realtime text transport

Option #1 represents the legacy usage of text-over-RTP (i.e., according to the baseline specification [b-ITU-T V.151]). This approach looks firstly straightforward because:

- WebRTC already uses audio-over-RTP and video-over-RTP, i.e., text-over-RTP would be just the third application component within the WebRTC "RTP suite"; and

- text conversation is inherently of type "realtime" as per the two other conversational realtime components audio and video (thus, transport via the "Realtime Transport Protocol").

The IETF selected option #2, i.e., considering text conversation just as one out of many possible WebRTC data applications. Hence, realtime text in a WebRTC environment uses the WebRTC data channel based transport with its SCTP/DTLS/L4/IP based stack.

Figure II.3 indicates the associated WebRTC gateway scenario, which is the exclusive scope in this Recommendation:

Figure II.3 – WebRTC gateway scenario – protocol stack model in user plane

The required WebRTC gateway service for interworking realtime text between WebRTC and non-WebRTC network domains appearsstraightforward:

The H.248 WebRTC media gateway must provide (see clause 8.6.4)
a) a "T.140 PDU transparent forwarding" service besides 
b) the protocol stack interworking of "SCTP/DTLS/L4" to "(S)RTP/UDP".

The "T.140 transparent forwarding" mode is basically possible, but the WebRTC gateway needs to provide a few support functions in order to address RTP packet loss and T.140 inactivity periods. [b-IETF RFC 4103] defines the required behaviour for RTP endsystems (i.e., the RTP source and RTP sink of a T.140 stream), which might need a few modifications:

More precisely (using the "RTP grouping taxonomy" terminology): there will be a distributed model in case of WebRTC gateways because (NOTE – the referred to processing stages are according to the model in Appendix II/[b-ITU-T H.248.95]) 
a) the T.140 entities "media capture / media renderer", "media source / media sink", "media encoder / media decoder" and "media packetizer / media depacketizer" are located at the remote endsystem (the WebRTC client), and 
b) the media transport level entities "SCTP Stream source / SCTP Stream sink" and "RTP source / RTP sink" represent MG local functions.

Another aspect of consideration is the correct configuration of the underlying protocol stack with respect to support of sufficient reliability for text transport. Figure II.4 provides a summary:

Figure II.4 – WebRTC gateway scenario – Network engineering aspects

Figure II.5 illustrates the complete, unidirectional media processing model in WebRTC to non-WebRTC direction. It underlines the geographical separation (and SCTP-based interruption) of the normally tighlty coupled functions of the "T.140 media endcoder" (S:3) and the "RTP media packetizer" (S:5).

Figure II.5 – WebRTC gateway scenario – Functional processing stages of the distributed "T.140-over-RTP endpoint" in WebRTC client to non-WebRTC UE direction

## II.3	Solution – Guidelines for the WebRTC media gateway (T.140)-IWF

The WebRTC media gateway needs to provide some support functions see Table II.1 in order to optimize the end-to-end service quality:

Table II.1 – Policy rules for (T.140)-IWF by WebRTC media gateways

| Rule | Condition(s): If … | Action(s): Then … | Protocol intervention: |
| --- | --- | --- | --- |
| ROut,1: | C1: "Inactivity of (T.140)-PDU traffic in WebRTC-to-non-WebRTC direction?" | A1: "MG behaviour: RTP source function according to clause 5.2 of [IETF RFC 4103], "Transmission before and after "Idle Periods"". | MG sends "empty RTP packets" under consideration of RTP redundancy or/and FEC. |
| RIn,1: | C1: "Incoming RTP packets out of order?" | A1: "MG behaviour: RTP sink function according to clause 5.4 of [IETF RFC 4103], "Compensation for Packets Out of Order"". | MG delays internal forwarding of RTP payload data (i.e., T140block) due to required buffering periods. |
| RIn,2: | C1: "Incoming RTP packets lost?" | A1: "MG behaviour: RTP sink function according to clause 5.3 of [IETF RFC 4103], "Detection of Lost Text Packets"". | MG inserts new T140blocks with "missing text marker" information. |

The indicated support functions would be located in processing stage "T.140 PDU mapping between SCTP Stream and RTP" when using a model such as illustrated in Figure II.5.

Bibliography

[b-ITU-T G.711]	Recommendation ITU-T G.711 (11/1988), Pulse code modulation (PCM) of voice frequencies.

[b-ITU-T H.248.95]	Recommendation ITU-T H.248.95 (10/2015), Gateway control protocol: H.248 support for RTP multiplexing.

[b-ITU-T H.264]	Recommendation ITU-T H.264 (02/2014), Advanced video coding for generic audiovisual services.

[b-ITU-T H.265]	Recommendation ITU-T H.265 (04/2013), High efficiency video coding.

[b-ITU-T H.Sup.13]	Supplement ITU-T H.Sup.13 (10/2015), Gateway Control Protocol: ITU-T H.248.x-series - Supplement on common H.248 terminology – Release 2.

[b-IETF RFC 3711]	IETF RFC 3711 (2004), The Secure Real-time Transport Protocol (SRTP).

[b-IETF RFC 4733]	IETF RFC 4733 (2006), RTP Payload for DTMF Digits, Telephony Tones, and Telephony Signals.

[b-IETF RFC 4788]	IETF RFC 4788 (2007), Enhancements to RTP Payload Formats for EVRC Family Codecs.

[b-IETF RFC 4867]	IETF RFC 4867 (2007), RTP Payload Format and File Storage Format for the Adaptive Multi-Rate (AMR) and Adaptive Multi-Rate Wideband (AMR-WB) Audio Codecs.

[b-IETF RFC 5763]	IETF RFC 5763 (05/2010), Framework for Establishing a Secure Real-time Transport Protocol (SRTP) Security Context Using Datagram Transport Layer Security (DTLS).

[b-IETF RFC 6184]	IETF RFC 6184 (05/2011), RTP Payload Format for H.264 Video.

[b-IETF RFC 6386]	IETF RFC 6386 (2011), VP8 Data Format and Decoding Guide.

[b-IETF RFC 6716]	IETF RFC 6716 (2012), Definition of the Opus Audio Codec.

[b-IETF RFC 7362]	IETF RFC 7362 (09/2014), Latching: Hosted NAT Traversal (HNT) for Media in Real-Time Communication.

[b-IETF RFC 7587]	IETF RFC 7587 (06/2015), RTP Payload Format for Opus Speech and Audio Codec.

[b-IETF rmcat-cc-requirements]	IETF draft-ietf-rmcat-cc-requirements (2014), Congestion Control Requirements for Interactive Real-Time Media.

[b-IETF rtcweb-gateway]	IETF draft- rtcweb-gateways (2015), WebRTC Gateways.

[b-IETF trickle-ice]	IETF draft-ietf-mmusic-trickle-ice (2015), Trickle ICE: Incremental Provisioning of Candidates for the Interactive Connectivity Establishment (ICE) Protocol.

[b-IETF ice-dualstack]	IETF draft-ietf-mmusic-ice-dualstack-fairness 2015), ICE Multihomed and IPv4/IPv6 Dual Stack Fairness.

[b-IETF rtp-vp8]	IETF draft-ietf-payload-vp8 (2015), RTP Payload Format for VP8 Video.

[b-IETF codec-vp9]	IETF draft-grange-vp9-bitstream (2013), A VP9 Bitstream Overview.

[b-IETF rtp-h265]	IETF draft-ietf-payload-rtp-h265 (2015), RTP Payload Format for High Efficiency Video Coding.

[b-IETF rtcweb-ecrit]	IETF draft-aboba-rtcweb-ecrit-01 (2013), Emergency Services Support in WebRTC.

[b-IETF rtcweb-audio]	IETF draft-ietf-rtcweb-audio (2014), WebRTC Audio Codec and Processing Requirements.

[b-IETF rtcweb-video]	IETF draft-ietf-rtcweb-video (2014), WebRTC Video Processing and Codec Requirements.

[b-IETF rtcweb-xr]	IETF draft-ietf-xrblock-rtcweb-rtcp-xr-metrics (2015), Considerations for Selecting RTCP Extended Report (XR) Metrics for the WebRTC Statistics API.

[b-IETF rtcweb-jsep]	IETF draft-ietf-rtcweb-jsep (2015), Javascript Session Establishment Protocol.

[b-IETF sdp-bundle]	IETF draft-ietf-mmusic-sdp-bundle-negotiation (2015), Negotiating Media Multiplexing Using the Session Description Protocol (SDP).

[b-IETF sdp-sctp]	IETF draft-ietf-mmusic-sctp-sdp (2015), Stream Control Transmission Protocol (SCTP)-Based Media Transport in the Session Description Protocol (SDP).

[b-IETF data-channel-msrp]	IETF draft-ietf-mmusic-msrp-usage-data-channel (2015), MSRP over SCTP/DTLS data channels.

[b-IETF data-channel-t140]	IETF draft-schwarz-mmusic-t140-usage-data-channel (2015), T.140 Text Conversation over Data Channels.

[b-IETF ietf-bfcpbis-rfc4582bis]	IETF draft-ietf-bfcpbis-rfc4582bis (2015), The Binary Floor Control Protocol (BFCP).

[b-W3C webrtc-stats]	W3C draft webrtc-stats (2015), Identifiers for WebRTC's Statistics API. < http://www.w3.org/TR/webrtc-stats/ >

[b-IETF tls-terms]	IETF draft-guballa-tls-terminology (2015), Terminology related to TLS and DTLS. 
<https://tools.ietf.org/html/draft-guballa-tls-terminology-02>

[b-IETF rtcweb-overview]	IETF draft-ietf-rtcweb-overview (2015), Overview: Real Time Protocols for Brower-based Applications.

[b-IETF rtp-usage]	IETF draft-ietf-rtcweb-rtp-usage (2015), Web Real-Time Communication (WebRTC): Media Transport and Use of RTP.

[b-IETF rtcweb-secur]	IETF draft-ietf-rtcweb-security (2015), Security Considerations for WebRTC.

[b-IETF rtcweb- security-arch]	IETF draft-ietf-rtcweb-security-arch (2015), WebRTC Security Architecture.

[b-IETF rtcweb-transports]	IETF draft-ietf-rtcweb-transports (2015), Transports for RTCWEB.

[b-IETF DCEP]	IETF draft-ietf-rtcweb-data-protocol (2015), WebRTC Data Channel Establishment Protocol.

[b-IETF WebRTCDC] 	IETF draft-ietf-rtcweb-data-channel (2015), WebRTC Data Channels.

[b-IETF DCSDP]	IETF draft-ietf-mmusic-data-channel-sdpneg (2015), SDP-based "SCTP over DTLS" data channel negotiation.

______________________

There were already similar discussions for non-WebRTC related TCP media, e.g. TLS-based transport security [ITU-T H.248.90] and the interworking of TLS-to-TLS, TLS-to-non-TLS in a particular TCP interworking mode.

The decision to transport realtime text over a data channel in WebRTC (instead of RTP based transport) is constituted by use case "U-C 5: Realtime text chat during an audio and/or video call with an individual or with multiple people in a conference", see clause 3.2/[b-IETF-rtcweb-data-channel].