| Rapporteur Meeting of Question 21/16 |  |  |  | AVD-4785 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | China Telecom |  |  |  |
| Title: | H.VSMprot: Proposal to add PTZ control to clause 6.3 and resource query to clause 6.4 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposed to add PTZ control to clause 6.3 and resource query to clause 6.4 to draft H.VSMprot: "Signalling and Protocols for mobile visual surveillance".

# Introduction

The Recommendation H.VSMprot focus on the Signalling and Protocols of a visual surveillance system with mobile units, such as mobile customer unit or premises unit, as well as on the services flow related to mobile units, such as a mobile customer unit accessing real-time video stream from a visual surveillance system.

# Discussion

In order to push forward the progress of the work item: H.VSMprot “Signalling and Protocols for mobile visual surveillance”, The clause 6.3 and 6.4 are about PTZ control and resource query.

# Conclusion

It is proposed to add clause 6.3 and clause 6.4 to draft H.VSMprot: "Signalling and Protocols for mobile visual surveillance".

Attachment
Proposal text

[Begin proposed text]

Signalling and protocols for mobile visual surveillance

# 6	Signalling and protocols

The following is some illustrative items:

- The SIP message format should comply with the definition in RFC3261;

- The RTSP message format should comply with the definition in RFC2326;

- The HTTP message format should comply with HTTP1.1 defined in RFC2616;

- RTP/RTCP signaling and message refers to RFC3550.

## 6.3	PTZ control

The PTZ Control function is used to pan, tilt and zoom cameras, switch auxiliary peripherals (such as flashlight, fan and rain brush), and to preset the position of related physical devices.

Figure 6-3 PTZ control procedural flows

- The M_CU sends PTZ control request to the VAU .The message body carries UserID and DestID (DestID  is PUID-ChannelNo).

- The VAU sends PTZ code to the CMU. The code is that the VAU matches routing table

according to DestID.

- The CMU sends PTZ control response to the VAU.

- Within configuration time, if the VAU receives the stop request from the M_CU, the VAU sends PTZ stop request to the CMU, otherwise, the VAU sends PTZ stop request to the CMU automatically.

- The CMU sends PTZ stop response to the VAU.

- The M_CU sends RTSP TEARDOWN to the VAU in order to disconnect the real-time session.

- The VAU sends the disconnect request to the CMU to stop session.

- The CMU sends the disconnect response to the VAU to stop session.

- The VAU sends the disconnect response to the M_CU to stop session.

## 6.4 Resource query

Figure 6-4 USER resource query procedural flows

- USER log in the MSP using MSISDN or USERID and password.

- The MSP gets MSISDN-No  from MSISDN, then recognizes user type from MSISDN-No. At last the MSP translates user type and MSISDN-No  into UserID.

- The MSP queries UserID in the CMU.

- The CMU queries user resource list according to UserID.

- The CMU returns the list and user info  to the MSP including PUID-ChannelNo, PUName, PuProperty.

- The MSP builds web page according to uesr order mobile service.

- The MSP returns the PSS-URL and mobile service order to user.

_________________