| Rapporteur Meeting of Question21/16 |  |  |  | TD-10 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Changzhou, 26 – 29 September 2016 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Beijing University of Posts and Telecommunications, China Telecom |  |  |  |
| Title: | H.CSVSArch: "Architecture for cloud storage in visual surveillance" |  |  |  |
| Purpose: | Proposal |  |  |  |

This document contains the new version of draft Recommendation ITU-T H.CSVSArch: "Architecture for cloud storage in visual surveillance" according to result of discussion in SG16 meeting (Changzhou, 26–29 September 2016).

NOTE – This text is based on the Contribution AVD-4777.

Draft Recommendation ITU-T H.CSVSArch

Architecture for cloud storage in visual surveillance

# 1	Scope

The visual surveillance service is a telecommunication service focusing on visual (audio included also) application technology, which is used to capture remotely multimedia (such as audio, visual, image and various alarm signal) and present them to the end users with friendly manner, based on managed broadband network with ensured quality, security and reliability.

Cloud storage can realize scalable, flexible and reliable data storage for large scale video surveillance. It enables service users to have ubiquitous, convenient and on-demand network access to a shared pool of configurable storage resources, which can be rapidly provisioned and released with minimal management effort or service-provider interaction.

This Recommendation describes the architectural requirements of cloud storage in video surveillance, which satisfies the needs described in cloud storage requirements in visual surveillance [ITU-T F.743.2].This document defines the functional requirements, model, architecture, entities, reference points and service-control flow of cloud storage in visual surveillance.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T F.743]		Recommendation ITU-T F.743 (2009), Requirements and service description for visual surveillance.

[ITU-T H.626]	Recommendation ITU-T H.626 (2011), Architectural requirements for visual surveillance.

[ITU-T H.262.1]		Recommendation ITU-T H.262.1 (2013), Architecture for mobile visual surveillance.

[ITU-T F.743.2]		Recommendation ITU-T F.743.2 (2016), Requirements for cloud storage in visual surveillance.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

[TBD]

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

[TBD]

# 4	Abbreviations and acronyms

[TBD]

# 5	Conventions

[TBD]

# 6	Overview of cloud storage in visual surveillance

The large-scale deployment of visual surveillance system has contributed to the explosive growth of surveillance video data. It induces the stringentneeds for the efficient storage of massive video data and the fast access of information of interest to users.

The cloud storage is a data storage model for enabling service users to have ubiquitous, convenient and on-demand network access to a shared pool of configurable storage resources, which can be rapidly provisioned and released with minimal management effort or service-provider interaction. In the cloud storage system, the physical and virtual resources can be dynamically assigned and reassigned according to user demands.

# 7	Functional architecture for cloud storage in visual surveillance

## 7.1	Functional model

Figure 1 – Architecture model for cloud storage in visual surveillance

The basic model for cloud storage in visual surveillance is mainly divided into four functional layers, which are respectively the storage layer, the management layer, the interface layer and the application layer. Each functional layer provides services and functions for the above functional layer to make the whole cloud storage system operate effectively.

- Storage layer: The storage layer is the most basic part in the cloud storage. It mainly consists of storage devices and foundation softwares. The distributed storage devices in the cloud storage system are usually deployed in the different places, and they can communicate with each other through networks.

- Management layer: The management layer is the core parts in the cloud storage which provides the resource management, the data management, the user management, the service management, the security management and the log management. The management layer is used to achieve collaboration among the different storage devices and guarantee the better data access performance.

- Interface layer: The interface layer is a relatively flexible layer. Different cloud storage departments can design the different interfaces according to their demands. The main interfaces in this layer are the web service interfaces, the APIs and the Mibs interface.

- Application layer: Any of the authorized users can log in the cloud storage system through the application layer, and the application layer presents all functions that the cloud storage system provides to the users.

## 7.2	Functional architecture framework

Figure 2 – Components for cloud storage in visual surveillance

The cloud storage based visual surveillance platform consists of two parts. One part is the traditional visual surveillance system, and the other part is the cloud storage system.

The traditional visual surveillance service is a telecommunication service focusing on video (and audio) application technology, which is used to remotely capture multimedia (such as audio, video, image and various alarm signals), and present them to end users in a friendly manner (including accessibility aspects). The traditional system mainly includes six functional entities which are respectively CMU, SCU, MDU, MSU, PU, and CU.

The cloud storage is a data storage model to provide a shared pool of configurable storage resources to service users. In cloud storage system, the physical and virtual resources can be dynamically assigned and reassigned according to user demands. The cloud storage mainly includes three functional entities: CSMU, CSDU, CSAU.

- CSMU: Cloud storage management unit. The CSMU is used to provide management services for the whole cloud storage, including the transmission of management signals with CMU and other components in cloud storage.

- CSDU: Cloud storage data unit. The CSDU is data node in a cloud storage system which is organized in a distributed way. The CSDU is used to storage massive data and information produced by video surveillance system.

- CSAU: Cloud storage access unit. The CSAU is the key unit for implementing communication between the CSDU and the units defined in video surveillance system [ITU-T H.626]. The main functions of the CSAU include: receiving and responding the control information of the SCU, distributing the media, receiving the media from MDU and writing the media in CSDU, reading the media in CSDU and distributing the media to the requesting units of the video surveillance system.

## 7.3	Functional entities

### 7.3.1	Cloud storage management unit

The cloud storage management unit (CSMU) is used to manage the whole cloud storage          system. The main functions of the CSMU include supporting the storage space management          and data management, providing the scalability of the storage resource.

### 7.3.2	Cloud storage data unit

The cloud storage data unit(CSDU) is the basic functional entities in a cloud storage  system. The CSDU is used to store data and information as a data node which is organized in a distributed way. The CSDU can be expanded according to the users’ demands.

### 7.3.3	Cloud storage access unit

The cloud storage access unit (CSAU) is the key unit to implement control between cloud storage and other units defined in [ITU-T H.626]. The main functions of CSAU are illustrated in Figure 3 and include the following functions: service control function, media control function, media processing function and configuration management function.

Figure 3 – Function components of CSAU

Service control function:

- initiate, maintain and release the connection with the SCU

- acquire the media stream from the MDU and/or the PU

- process service requests from the CSMU, forward them to the SCU and respond to the CSMU.

Media control function:

- receive the media stream from the MDU and/or the PU.

- transmit or forward the media stream to the CSDU.

Media processing function:

- transcode the media streams before transferring the streams from the MDU and/or the PU tothe CSDU according to the ability of the CSDU.

Configuration management function:

- manage the overall configuration in the CSAU, including transmission, decoding, transcoding and other control of the stream.

## 7.4	Reference points

### 7.4.1	Reference point Pc: PU-CSAU

The reference point Pc is located between the PU and the CSAU. It is used to deliver interactive signals, such as RTSP request or response, directly between the CSAU and the PU. It is also used to deliver the media stream directly from the PU to the CSAU.

### 7.4.2	Reference point Mdc: MDU-CSAU

The reference point Mdc is located between the MDU and the CSAU. It is used by the MDU to distribute, store and deliver media to the CSAU using streaming methods.

### 7.4.3	Reference point Ss: CSAU-SCU

The reference point Ssis located between the CSAU and the SCU. It is used for the following functions:

- control CSAU registration and access authentication, authorization and accounting by the SCU.

- deliver the control signals between the CSAU and the SCU.

- query the transcoding parameters from the SCU according to the network type in order to store the media.

### 7.4.4	Reference point Cms: CMU-CSMU

The reference point Cms is located between the CMU and the CSMU, it is used to deliver management signals between the CMU and the CSMU.

### 7.4.5	Reference point Csm: CSMU-CSAU

The reference point Csm is located between the CSMU and the CSAU, it is used by the CSMU to manage the service control function, media control function, media processing function and configuration management function of the CSAU.

### 7.4.6	Reference point Csd: CSMU-CSDU

The reference point Csd is located between the CSMU and the CSDU, it is used to deliver media requests, media storage or receive responses sent by the CSMU and the CSDU.

### 7.4.7	Reference point Csd’: CSMU-CSDU

The reference point Csd’ is located between the CSMU and the CSDU, it is used to support the storage space management. The main functions are as follows.

- increase or decrease the storage space flexibly according to the users’ demands.

- support data management based in the CSDU, including adding, deleting, browsing, indexing,searching,etc.

### 7.4.8 	Reference point Csc: CSDU-CSAU

The reference point Csc is located between the CSDU and the CSAU. It is used to deliver media request by the CSAU and storage request to the CSDU.

### 7.4.9	Reference point Cc: CSDU-CSAU

The reference point Cc is located between the CSAU and the CSDU. It is used as a media stream to transport media to CSDU.

### 7.4.10	Reference point Csa: CSMU-CSAU

The reference point Csa is located between the CSAU and the CSMU. It is used to deliver access AAA signal between the CSMU and the CSAU.

## 7.5	Service control flow

### 7.5.1	Real-time media storage

When a user wants to store real-time media obtained by the PU in the cloud, the CU will set a storage plan and send it to the SCU to start the storage.

Figure 4 – High-level procedural flows for media storage

Figure 4 shows the procedural flows of media storage in cloud storage system:

- First, the CU sets a storage plan in the service platform of the SCU. The SCU transforms this storage plan to the CMU, which saves this plan, and the CMU transfers this plan to the CSMU.

- The CSMU returns a response to the CMU for this storage plan and the CMU transfers the response to the SCU. At last, the response is sent to the CU.

- When the SCU gets a storage event, it sends a storage request to the MDU which transfers this request to the PU.

- The media stream is sent from the PU to the MDU which then distributes media stream to the CSAU.

- The CSAU transfers media stream to the CSDU to store it in the cloud.

- When the storage plan is completed, the CU will send a stop-media request to the SCU and the SCU makes a check and returns a response.

- The SCU sends the stop-media request to the SDU.

- The SDU stops media transform to the CSAU.

- After that, the CSAU returns a response to the MDU, and the MDU sends this response to the SCU. Then the whole real-time storage is finished.

### 7.5.2	Video file uploading from mobile devices

When a user wants to upload video files from mobile devices, the MCU will initiates a request to the SCU to start file uploading.

Figure 5 – High-level procedural flows for video file uploading from mobile devices

Figure 5 shows the procedural flows of video file uploading from mobile devices:

- First, the MCU initiates a request and creates a connection with the MSP.

- The MSP then initiates a request and creates a connection with the CMU.

- The CMU then returns the VAU information

- The MSP then receives the request and returns the VAU information to the MCU

- Then the CMU initiates a video file upload request to the CSMU, and the CSMU transfers this request to the CSAU.

- The CSAU returns a response to the CSMU and the CSMU transfers this response to the SCU.

- After that, the MCU starts to upload video file stream to the VAU.

- The VAU transmits this video file stream to the MDU.

- The MDU distributes this stream to the CSAU which transfers stream to the CSDU to store it.

- When the MCU stops to upload video file stream, it initiates a stop-media request to the VAU.

- The VAU transfers this request to the SCU.

- The SCU requests the CSAU to stop video file upload and the CSAU returns a response to it.

- The SCU transfers this response to the VAU. The video file upload from mobile device is finished.

### 7.5.3	Video file uploading from NVR

A NVR is usually used as the local storage device for continuously recording surveillance video from network cameras. When a user wants to upload video file from NVR, the CU will set a storage plan to the SCU.

Figure 6 – High-level procedural flows for video file uploading from NVR

Figure 6 shows the procedural flows of video file uploading from NVR:

- First, the CU sets a storage plan in the service platform of the SCU. The SCU transforms this storage plan to the CMU, which saves this plan, and the CMU transfers this plan to the CSMU.

- Then the CSMU returns a response to the CMU for this storage plan and the CMU transfers the response to the SCU. At last, the response is sent to the CU.

- When the SCU gets a storage event, it sends a storage request to the MDU which transfers this request to the NVR.

- Then the file stream is sent from the NVR to the MDU which then distributes file stream to the CSAU.

- The CSAU transfers file stream to the CSDU to store it in the cloud.

- When the storage plan is completed, the CU will send a stop-media request to the SCU and the SCU makes a check and returns a response.

- Then the SCU sends the stop-media request to the SDU.

- The SDU stops media transform to the CSAU.

- After that, the CSAU returns a response to the MDU, and the MDU sends this response to the SCU. Then the whole video file upload is finished.

### 7.5.4	Picture upload from surveillance camera

Network cameras continuously capture pictures, and send these pictures directly to the cloud. When pictures are captured by cameras, the CU will initiate a request to the SCU to upload pictures.

Figure 7 – High-level procedural flows for picture uploading from surveillance camera

Figure 7 shows the procedural flows of picture uploading from surveillance camera:

- First, the CU sets a storage plan in the service platform of the SCU. The SCU transforms this storage plan to the CMU, which saves this plan, and the CMU transfers this plan to the CSMU.

- The CSMU returns a response to the CMU for this storage plan and the CMU transfers the response to the SCU. At last, the response is sent to the CU.

- When the SCU gets a storage event, it sends a storage request to the MDU which transfers this request to the PU.

- The picture stream is sent from the PU to the MDU which then distributes picture stream to the CSAU.

- The CSAU transfers picture stream to the CSDU to store it in the cloud.

- When the storage plan is completed, the CU will send a stop-media request to the SCU and the SCU makes a check and returns a response.

- The SCU sends the stop-media request to the SDU.

- The SDU stops media transform to the CSAU.

- After that, the CSAU returns a response to the MDU, and the MDU sends this response to the SCU. Then whole picture upload is finished.

### 7.5.5	Picture upload from mobile device

When a user wants to upload picture from mobile device, the MCU initiates a request to the CMU to start uploading.

Figure 8 – High-level procedural flows for picture upload from mobile device

Figure 8 shows the procedural flows of picture upload from mobile device:

- First, the MCU initiates a request and creates a connection with the MSP.

- The MSP then initiates a request and creates a connection with the CMU.

- The CMU then returns the VAU information

- The MSP then receives the request and returns the VAU information to the MCU

- The CMU initiates a picture upload request to the CSMU, and the CSMU transfers this request to the CSAU.

- The CSAU returns a response to the CSMU and the CSMU transfers this response to the SCU.

- After that, the MCU starts to upload picture stream to the VAU.

- The VAU transmits this picture stream to the MDU.

- The MDU distributes this stream to the CSAU which transfers stream to the CSDU to store it.

- When the MCU stops to upload picture stream, it initiates a stop-media request to the VAU.

- The VAU transfers this request to the SCU.

- The SCU requests the CSAU to stop picture upload and the CSAU returns a response.

- The SCU transfers this response to the VAU. The picture upload from mobile device is finished.

### 7.5.6	Video metadata storage

The VS system can provide various intelligent services by using video/image analysis technologies. When a user wants to store metadata in the cloud, the CU initiates a request to the SCU to transmit metadata.

Figure 9 – High-level procedural flows for metadata storage

Figure 9 shows the procedural flows of metadata storage:

- First, the CU sets a storage request to the SCU. Then the SCU transforms this storage request to the CMU, which saves this request plan.

- Then the CMU sends this storage request to the CSMU, and the CSMU returns a response to the CMU for storage request.

- The CMU transfers this response to the SCU. At last, the response is sent to the CU.

- When the CU receives the response from the SCU, it will start metadata stream to the MDU.

- The MDU distributes metadata stream to the CSAU.

- The CSAU transfers metadata stream to the CSDU to store it in the cloud.

- When the storage plan is completed, the CU will send a stop-media request to the SCU, then the SCU makes a check and returns a response.

- The SCU sends the stop-media request to the SDU.

- The SDU stops metadata transform to the CSAU.

- After that, the CSAU returns a response to the MDU, and the MDU sends this response to the SCU. Then whole metadata storage is finished.

### 7.5.7	Media acquisition from cloud to CU

When a user wants to view media which stores in the cloud storage system, the CU initiates a request to the SCU in order to acquire media.

Figure 10 – High-level procedural flows for media acquisition to the CU

Figure 10 shows the procedural flows of media acquisition to the CU when the media is stored in the cloud.

- First, the CU initiates a request to the SCU in order to acquire media.

- The SCU then transmits the request to the CMU and then the CMU transmits this request to the CSMU and creates a connection with the CSMU.

- Then the CSMU returns a response to the CMU, and the CMU transmits this response to the SCU and the CU.

- The CSMU sends a request to the CSAU to start media.

- The CSAU sends a signal to the CSDU to acquire media which is stored in the CSDU.

- The CSDU transfers the media stream to the CSAU and the CSAU transfers the media stream to the MDU.

- The MDU distributes the media stream to the CU.

- When the CU stops the media acquisition, it sends a stop-media request to the SCU.

- The SCU transfers the stop-request to the CSAU.

- The CSAU transfers the stop-request to the CSDU.

- The CSDU sends a response to the CSAU and stops the media stream.

- The CSAU transfers this response to the MDU and the MDU transfers this response to the SCU. The media acquisition is finished.

### 7.5.8	Media acquisition from cloud to MCU

When a user wants to view media on the mobile device, the MCU will initiate a request to the SCU to acquire media from cloud to mobile device.

Figure 11 – High-level procedural flows for media acquisition to mobile device

Figure 11 shows the procedural flows of media acquisition to mobile device when the media is stored in the cloud.

- First, the MCU initiates a request and creates a connection with the MSP.

- The MSP then initiates a request and creates a connection with the CMU.

- The CMU then returns the VAU information

- The MSP then receives the request and returns the VAU information to the MCU

- When the MCU receives the VAU information, it requests the VAU to start media.

- The VAU transmits the media request to the SCU, the CMU and the CSMU.

- After receiving the request, the CSMU returns a response to the CMU and the CMU transfers this response to the SCU.

- The CSMU requests the CSAU to start media and the CSAU requests the CSDU to start media.

- The CSDU transfers the media stream to the CSAU and the CSAU transfers the media stream to the MDU.

- The MDU distributes the media stream to the VAU and the VAU transfers this stream to the MCU.

- When the MCU requests to stop the media, the MCU will form a stop-media to the VAU, and the VAU transfers this media to the SCU.

- The SCU requests the CSAU to stop media, and the CSAU returns a response to the SCU.

- The SCU transfers this response to the VAU. The media acquisition to mobile device is finished.

_____________________