- 2 -

IPTV-GSI-C-498

	INTERNATIONAL TELECOMMUNICATION UNION
	IPTV-GSI

	TELECOMMUNICATION
STANDARDIZATION SECTOR

STUDY PERIOD 2009-2012
	IPTV-GSI-C-498

	
	English only

Original: English

	Question(s):
	13/16
	Pune, India, 13-17 December 2010

	CONTRIBUTION

	Source:
	Huawei Technologies

	Title:
	H.IPTV-AM.0: AM user permissions managed by IPTV service provider


Background

In the previous meeting, discussion started on user permission management and federation of user permissions managed by the aggregation function for all terminals used by the same user.
Summary

The solution for user permission management introduced in the current H.IPTV-AM.0 draft Recommendation relies on the aggregation function which is not the only possible.

This contribution proposes to add another solution relying on the IPTV service provider for the management of the user permissions, as it would be easier to include the AM permissions with the other user preferences in the user profile stored by the IPTV service provider.

Proposal 1

It is proposed to add a new section as follows to introduce the possibility for the IPTV service provider to host and manage the user permissions:
6.5.2.3. Audience measurement function with user permission management by IPTV SP
In this method, AM user permissions are parts of the user profile stored by the IPTV service provider. The IPTV SP provides the tool to allow the user to set its AM user permissions for the different services it offers.
The TD-AMF supports the ability to access these AM user permissions:

· To get them from the IPTV service provider

· To send them or part of them to the aggregation function(s) if needed

· To enable or disable audience measurement as requested by the AM service provider depending upon these permissions.
· To report audience measurement with all the elements or part of the elements requested by the AM service provider depending upon these permissions.

The following figure illustrates the different steps of the IPTV system operation with a focus on the AM aspects where the AM user permissions are managed by the IPTV SP.

[image: image1]
Note 1: The "userId" is an Id generated by the IPTV SP, independent of any service login identifier.

Note 2: User permission setting is optional before the first connection to the IPTV service provider. The default "AM user permission level" will be set to "no AM permitted at all".
Note 3: After connection to the IPTV service provider, the user may modify its AM user permissions at any time, like the other elements of its user profile stored by the IPTV service provider. In there is a significant change that may have an impact on the configuration package it received, then a new AM configuration package will be requested by the terminal device.
Proposal 2.
Existing text in section "8.1 Audience measurement functional requirements"

1. The audience measurement functions are required to support the ability to ask for the end-user’s permission when use of end-user privacy related information is involved. 
It is proposed to replace this text by:

1. The audience measurement functions are required to support the ability to check the end-user’s permission when use of end-user privacy related information is involved. 
Rationale: The word "check" includes both "ask for something not yet available to the AMF and check with it" and "check with something already available to the AMF" and both cases should be covered here.
___________




































	Contact:
	Christian Bertin
Huawei Technologies
China
	Tel: 
+33 2 99 31 61 20
Fax: 
+33 6 74 88 89 82
Email: 
xian.bertin@huawei.com 

	Contact:
	Aaron Zhang
Huawei Technologies
China
	Tel:
+86-25-82276772
Fax:
+86-25-82276760
Email:
zhangchuxiong@huawei.com 

	Attention: This is not a publication made available to the public, but an internal ITU-T Document intended only for use by the Member States of ITU, by ITU-T Sector Members and Associates, and their respective staff and collaborators in their ITU related work. It shall not be made available to, and used by, any other persons or entities without the prior written consent of ITU-T.



[image: image2.png]user

™

(Default terminal permissions)

1P’

User permission setting

»f
SP connection

oo — — —
User login

oo —— =

User permission & userld delivery
fe

TV SP AM SP

(user profile) |

Senvice discovery

Optional userld info declaration
e

AM config package request (IPTVId, userld, permission level, etc.)

AM con
le

Gpfional info request {userid)

userld Info
[ we@mo 000000000

fig package

AMF configuation

IPTV service consumption

& AM o)

IPTV service operation

peration

AM reporting

User disconnection

e — - -




