- 22 -

IPTV-GSI-C-513

	INTERNATIONAL TELECOMMUNICATION UNION
	IPTV-GSI

	TELECOMMUNICATION
STANDARDIZATION SECTOR

STUDY PERIOD 2009-2012
	IPTV-GSI-C-513

	
	English only

Original: English

	Question(s):
	13/16
	Pune, India, 13-17 December 2010

	CONTRIBUTION

	Source:
	ETRI, Huawei Technologies

	Title:
	Review Proposal on Appendix I within H .IPTV-SBSD


1.
Introduction

During Q.13/16 meeting in Geneva, 19~30 July 2010, an initial draft Recommendation H.IPTV-SBSD, “SIP-based Service Provider Discovery and Service Discovery for IPTV services”, had been developed. 
2.
Discussion

This contribution is based on TD-411 H.IPTV-SBSD “SIP-based Service Provider Discovery and Service Discovery for IPTV services” (ex HSTP.IPTV-SBSD): Initial draft, an output document of July 2010 IPTV-GSI Q.13/SG16 meeting.
During the Q13/16 meeting held 19 - 23 July 2010 during the IPTV-GSI event, at Geneva. The group discuss on HSTP.IPTV-SBSD like as follows;

 “During the Q.13/16meeting of IPTV-GSI event in July 2010, it was agreed to make a new draft Recommendation based on Technical Paper H.HSTP-SBSD. Because the text of H.HSTP-SBSD has not been accepted as normative in the Q.13/16 meeting, current text of H.HSTP.SBSD is adopted as an Appendix of New initial draft Recommendation H.IPTV-SBSD. The Appendix of draft Recommendation H.IPTV-SBSD will be reviewed to be a normative part of H.IPTV-SBSD based on meeting agreement.”
We reviewed briefly related reference standards such as ITU-T H.770, ETSI TS 183 063, ATIS-0800017.v002 etc. 

Current Appendix I. within H.IPTV-SBSD describes the service provider discovery process as well as service discovery process using SIP. It provides not only procedural framework for discover of service provider and service but also reference to detail protocol specifications. During the development of Technical Paper of SBCD, the proposed text has been reviewed well within Q.13/16 meeting.
Therefore, we sure that “Appendix I: Proposed text of Draft Recommendation H.IPTV-SBSD” should be handled as a normative part of Draft Recommendation H.IPTV-SBSD. 

3.
Proposal

Based on above discussions, we propose the current Appendix of H.IPTV-SBSD to move into the main body of this draft recommendation as a normative part.
The proposed document which reflects the above proposals is as follows:
ITU-T Draft Recommendation H.IPTV-SBSD
SIP-based Service Provider Discovery and Service Discovery for IPTV services
Summary
This document describes the process for IPTV service provider discovery and service discovery based on SIP. The processes enable IPTV terminal device to acquire information about available IPTV service providers and IPTV services.
Keywords

IPTV services, service provider, service provider discovery, service provider information, service discovery, service discovery information
ITU-T Draft Recommendation H.IPTV-SBSD
SIP-based Service Provider Discovery and Service Discovery for IPTV
1
Scope

This document describes the process for IPTV service provider discovery and service discovery based on SIP. The processes enable IPTV terminal device to acquire information about available IPTV service providers and IPTV services.

2
References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.
[ITU-T Y.1910]
ITU-T Recommendation Y.1910 (2008), IPTV functional architecture.

[ITU-T H.770]
ITU-T Recommendation H.770, Mechanisms for service discovery and selection for IPTV services
[IETF RFC 3261]
IETF RFC 3261 (2002), SIP: Session Initiation Protocol
[IETF RFC 3265]
IETF RFC 3265 (2002), Session Initiation Protocol (SIP)-Specific Event Notification
[ETSI TS 183 063]
ETSI TS 183 063 v2.4.2 (2009-06), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); IMS-based IPTV stage 3 specification
[ATIS-0800017.v002]
ATIS standard ATIS-0800017.v002 (2009), Network Attachment and Initialization of Devices and Client Discovery of IPTV Services

3
Definitions

3.1
Terms defined elsewhere
3.2.1 Application [ITU-T Y.101]: A functional implementation realized as software running in one or spread over several interplaying hardware entities.
3.2.2 End-user [ITU-T Y.1910]: The actual user of the products or services.
3.2.3 IPTV [ITU-T Y.1901]: Multimedia services such as television, video, audio, text, graphics, data delivered over IP-based networks managed to support the required level of QoS/QoE, security, interactivity and reliability.
3.2.4 IPTV terminal device [ITU-T Y.1901]: A terminal device which has ITF functionality, e.g. a STB.
3.2.5 Service [ITU-T Y.101]: A structure set of capabilities intended to support applications.
3.2.6 Service provider [ITU-T M.1400]: A general reference to an operator that provides telecommunication services to customers and other users either on a tariff or contract basis. A service provider may or may not operate a network. A service provider may or may not be a customer of another service provider.

3.2.7 Terminal device (TD) [ITU-T Y.1901]: An end-user device which typically presents and/or processes the content, such as a personal computer, a computer peripheral, a mobile device, a TV set, a monitor, a VoIP Terminal or an audio-visual media player.

4
Abbreviations and acronyms

This document uses the following abbreviations and acronyms:
	SPD
	Service provider description

	SD
	Service description



5
Introduction

[H.770] defines Figure 1 and several steps for service discovery in order to acquire IPTV contents in the IPTV service.

[image: image1.emf]SP 

discovery

Content 

selection & 

acquisition

SP service 

discovery

SP content 

discovery

Among a number 

of SP

Among a number 

of contents on live 

or VoD

Among a number 

of services (live, 

VoD, etc.)


Figure 1 - From Service Provider discovery to content acquisition

Notes:

· SP discovery: Discovery of the service providers available on the network

· SP service discovery: Discovery of the services offered by a specific or all service providers.

· SP content discovery: Discovery of the individual services or contents from a specific service provider.
· Content selection & acquisition: At the end of the navigation through the offered services and contents, the end-user selects a service or content and acquires it.
· End-users and/or devices may be authenticated before service provider attachment or service attachment as part of the security provisions.
ITU-T H.770 Recommendation includes service provider description information for IPTV service provider discovery process and service information for IPTV service discovery process, and its delivery protocols covering both unicast and multicast transport mechanisms. This document describes the detailed service provider discovery process and service discovery process using SIP as an alternative delivery protocol. This document also describes the message flow for three architectures stated in ITU-T Y.1910.
Figure 2 is derived from Appendix I of ITU-T Y.1910 Recommendation shows the high level procedural flows for initialization of IPTV application access. Figure 2 has added IPTV SPD provider and IPTV SD provider in the SADS functional block to define its position in the IPTV architecture.

[image: image2]
Figure 2 - High level procedure for initialization of IPTV application access
Terminal Device may receive service and application discovery information which is performed in push mode or pull mode.

· Push mode[ITU-T Y.1910]: the SADS functional block actively sends the service and application discovery information to the Terminal Device.
· Pull mode[ITU-T Y.1910]: the Terminal Device actively requests the service and application discovery information from the SADS functional block.

6
Service Provider Discovery process using SIP Messages
This clause describes process for end-user device to get information about available IPTV service providers.
6.1
SUBSCRIBE/NOTIFY message

Figure 3 shows flows for retrieving the description of the available IPTV service providers for the Non-NGN and NGN non-IMS IPTV architecture.

[image: image3.emf]Terminal 

Device

IPTV

Service 

Description 

provider

IPTV Service 

Provider 

Description 

provider

Request for IPTV Service Provider Description

(SUBSCRIBE)

Description of Service providers 

(NOTIFY)


Figure 3 - Flows for retrieving the description of the available IPTV service providers
Service provider discovery process is necessary for end-user device to discover the IPTV service providers when several service providers are available on the network. This step is also necessary for the case in which end-user device does not have any preconfigured service provider. This step may be omitted in the case where there is only one IPTV service provider or in case where the user is directly registered (or pre-configured) with one particular service provider.

In order to acquire the initial service provider description information (and subsequent notifications of updates to that information), IPTV terminal device connects to IPTV SPD (Service Provider Description) provider by establishing a standard subscription session based on the SIP Notification framework specified in IETF RFC 3265. The SIP notification framework provides a pull mechanism for initial data retrieval; when the data changes, it uses a push mechanism for notification of changes through the use of a NOTIFY message.
Therefore, IPTV terminal device sends SUBSCRIBE message to IPTV SPD provider in order to acquire service provider description information. And then, IPTV SPD provider periodically sends NOTIFY message including service provider description information to IPTV terminal device. Service provider description information is included in the body part of NOTIFY message which expresses information defined in clause 7.1 of ITU-T H.770 Recommendation.

IPTV SPD provider and IPTV SD (Service Description) provider can reside in a single service provider or in a separate service provider.

It is possible to define “IPTV-SP-Description” as an example of an event name of an event header for the initial SUBSCRIBE which is delivered to IPTV SPD provider. Detail description of SUBSCRIBE and NOTIFY message is beyond the scope of this document.

Meanwhile, in the NGN IMS based IPTV architecture, Terminal Device may receive service and application discovery information via the core IMS function [ITU-T Y.1910]. Therefore, IPTV terminal device sends SUBSCRIBE message through Core IMS to IPTV SPD provider in order to acquire IPTV service provider description information. And then, IPTV SPD provider periodically sends NOTIFY message including IPTV service provider description information via Core IMS to IPTV terminal device.
Figure 4 shows flows for retrieving the description of the available IPTV service providers according to the NGN IMS IPTV architecture.
Detailed description of the service provider discovery process defined in the following standards.
- Clause 5.1 and 5.2 of [ETSI TS 183 063];

- Clause 8.5 of [ATIS-0800017.v002].


[image: image4.emf]Terminal 

Device

IPTV

Service 

Description 

provider

Core

IMS

IPTV Service 

Provider 

Description 

provider

Request for IPTV Service 

Provider Description

(SUBSCRIBE)

Description of Service 

providers 

(NOTIFY)

Request for IPTV Service 

Provider Description

(SUBSCRIBE)

Description of Service 

providers 

(NOTIFY)


Figure 4 - Flows for retrieving the description of the available IPTV service providers via Core IMS
6.2
MESSAGE message
This SUBSCRIBE and NOTIFY mechanism is also necessary in the case that end-user device does not have any preconfigured service provider. Different mechanism can be used in the case where there is only one IPTV service provider or where the user is directly registered with one particular service provider.

Figure 5 shows flows for retrieving the description of the particular IPTV service provider for the Non-NGN and NGN non-IMS IPTV architecture.

[image: image5.emf]Terminal 

Device

IPTV

Service 

Description 

provider

IPTV Service 

Provider 

Description 

provider

Description of Service providers 

(MESSAGE)


Figure 5 - Flows for retrieving the description of the particular IPTV service provider
When IPTV terminal device is registered in the IPTV service system through REGISTER message after network attachment, its IPTV SPD provider immediately sends IPTV service provider description information with MESSAGE message to IPTV terminal device in push mode. During this process, IPTV SPD provider acts as a third-party registrar during the IMS registration phase. Service provider description information is included in the body part of MESSAGE message which expresses information defined in clause 7.1 of ITU-T H.770 Recommendation.
Figure 6 shows flows for retrieving the description of the particular IPTV service provider according to the NGN IMS IPTV architecture.
Detailed description of the service provider discovery process defined in the following standards.
- Clause 5.1 and 5.2 of [ETSI TS 183 063].

[image: image6.emf]Terminal 

Device

IPTV

Service 

Description 

provider

Core

IMS

IPTV Service 

Provider 

Description 

provider

Description of Service 

providers 

(MESSAGE)

Description of Service 

providers 

(MESSAGE)


Figure 6 - Flows for retrieving the description of the particular IPTV service provider via Core IMS

7
Service Discovery process using SIP messages
This clause describes process for end-user device for retrieving information about available IPTV services.
In the SP discovery procedure defined in H.770, one SPD Provider may relate to multiple SD providers. For the case that SD provider is also a SIP server, the service discovery procedure is described in 7.1; for the case that SD provider is not a SIP server, the service discovery procedure is described in 7.2.

7.1
 Procedure in case SD Provider is SIP Server

Figure 7 shows flows for retrieving the description of the available IPTV services for the Non-NGN and NGN non-IMS IPTV architecture.

[image: image7.emf]Terminal 

Device

IPTV

Service 

Description 

provider

IPTV Service 

Provider 

Description 

provider

Description of Services 

(NOTIFY)

Request for IPTV Service Description

(SUBSCRIBE)


Figure 7 - Flows for retrieving the description of the available IPTV services

An end-user can access the IPTV services from different service providers depending on the services to which the end-user is subscribed. The end-user can select applications from the same or a different service provider. This can include the discovery and selection of services from multiple service providers [ITU-T Y.1910].
In order to acquire the initial service description information (and subsequent notifications of updates to that information), IPTV terminal device connects to IPTV SD (Service Description) provider by establishing a standard subscription session based on the SIP Notification framework specified in IETF RFC 3265. The SIP notification framework provides a pull mechanism for initial data retrieval; when the data changes, it uses a push mechanism for notification of those changes through the use of a NOTIFY message.
So, IPTV terminal device sends SUBSCRIBE message to IPTV SD provider in order to acquire service description information. And then, IPTV SD provider periodically sends NOTIFY message including service description information to IPTV terminal device. Service description information is included in the body part of NOTIFY message which expresses information defined in clause 10 of ITU-T H.770 Recommendation.
During this process, service description information can be from one IPTV service description provider or multiple IPTV service description providers. 

It is possible to define ”IPTV-Service-Description” as an event name example of an event package for the initial SUBSCRIBE, which is delivered to IPTV SD provider. Detail description of SUBSCRIBE and NOTIFY message is beyond the scope of this specification. Meanwhile, in the NGN IMS based IPTV architecture, ITF may receive service and application discovery information via the Core IMS function [ITU-T Y.1910]. Therefore, IPTV terminal device sends SUBSCRIBE message through Core IMS to IPTV SD provider in order to request IPTV service description information. And then, IPTV SD provider periodically replies NOTIFY message including IPTV service description information via Core IMS to IPTV terminal device.
Figure 8 shows flows for retrieving the description of the available IPTV services according to the NGN IMS IPTV architecture.
Relevant standards of the service discovery process are specified in the following standard.:
- Clause 9.4 of [ATIS-0800017.v002].

[image: image8.emf]Terminal 

Device

IPTV

Service 

Description 

provider

Core

IMS

IPTV Service 

Provider 

Description 

provider

Request for IPTV Service 

Description

(SUBSCRIBE)

Description of Services 

(NOTIFY)

Request for IPTV Service Description

(SUBSCRIBE)

Description of Services 

(NOTIFY)


Figure 8 - Flows for retrieving the description of the available IPTV services via Core IMS

7.2 Procedure in case SD Provider is not a SIP Server
This clause is the case that IPTV SD provider is not a SIP server. 

In order to retrieve IPTV service provider description information from IPTV SPD provider, IPTV terminal device uses the SIP SUBSCRIBE/NOTIFY framework based on clause 6.1 is used.

While powering on, IPTV TD sends a SIP SUBSCRIBE message to IPTV SPD Provider, and waits for NOTIFY message, which triggers IPTV TD to send e.g. a HTTP request to IPTV SD Provider to update the service description.   

When updates happened, IPTV SD Provider will send an update request to IPTV SPD Provider, who will then generate and send a SIP NOTIFY message to IPTV TD. The protocol between IPTV SD Provider and IPTV SPD Provider is out of the scope

After receiving the SIP NOTIFY message from IPTV SPD Provider, IPTV TD can use HTTP which is one of the delivery protocols specified in H.770, to update the service description information from the appropriate SD provider.

[image: image9.emf]Service 

Provider 

Discovery 

Process

Service  

Discovery 

Process

Terminal 

Device

IPTV

Service 

Description 

provider

IPTV Service 

Provider 

Description 

provider

Request for IPTV Service Provider Description

(SUBSCRIBE)

Description of Service providers 

(NOTIFY)

Request for IPTV Service Description

(HTTP Request)

Description of Services 

(HTTP Response)

sending the updates request

Records Changed

Generating Notify Message

Figure 9: Flows for retrieving the description of the available IPTV services through HTTP
Figure 10 shows the detailed flows for retrieving the available IPTV service description through HTTP where Core IMS involved.


[image: image10.emf]Terminal 

Device

IPTV

Service 

Description 

provider

Core

IMS

IPTV Service 

Provider 

Description 

provider

Request for IPTV Service 

Provider Description

(SUBSCRIBE)

Description of Service 

providers 

(NOTIFY)

Request for IPTV Service 

Provider Description

(SUBSCRIBE)

Description of Service 

providers 

(NOTIFY)

Request for IPTV Service Description

(HTTP Request)

Description of Services 

(HTTP Response)

sending the updates request

Records Changed

Generating Notify Message

Service 

Provider 

Discovery 

Process

Service  

Discovery 

Process

Figure 10: Flows for retrieving the description of the available IPTV services through HTTP














3.2.8 
3.2.9 
3.2.10 
3.2.11 
3.2.12 
3.2.13 
3.2.14 


	
	

	
	








· 
· 
· 
· 
· 





· 
· 































































_________________
IPTV SP Description provider / IPTV Service Description provider 








SADS Functional Block





Service Control Functions








Network Functions





IPTV Applications (LTV, VOD, etc.)





IPTV Service Consumption





IPTV Service Discovery








IPTV Service Provider Discovery 


Network Attachment





Terminal Device





Application Function






































IPTV SP Description provider / IPTV Service Description provider 








SADS Functional Block





Service Control Functions








Network Functions





IPTV Applications (LTV, VOD, etc.)





IPTV Service Consumption





IPTV Service Discovery








IPTV Service Provider Discovery 


Network Attachment





Terminal Device





Application Function











	Contact:
	Miyoung Huh 

ETRI

Korea (Republic of)
	Tel:
+82 42 860 6118

Fax:
+82 42 861 5404

Email:
myhuh@etri.re.kr

	Contact:
	Shingak Kang

ETRI

Korea (Republic of)
	Tel:       +82 42 860 6117

Fax:      +82 42 861 5404

Email:
sgkang@etri.re.kr

	Contact:
	Aaron Zhang
Huawei Technologies
China
	Tel:
+86-25-82276772
Fax:
+86-25-82276760
Email:
zhangchuxiong@huawei.com

	Attention: This is not a publication made available to the public, but an internal ITU-T Document intended only for use by the Member States of ITU, by ITU-T Sector Members and Associates, and their respective staff and collaborators in their ITU related work. It shall not be made available to, and used by, any other persons or entities without the prior written consent of ITU-T.



_1317035835.vsd
�

�

SP 
discovery�

Content selection & acquisition


SP service discovery�

SP content discovery�

Among a number of SP


Among a number of contents on live or VoD�

Among a number of services (live, VoD, etc.)



_1341257373.vsd
�

�

텍스트�

Terminal Device�

IPTV
Service Description provider�

IPTV Service Provider Description provider�

Request for IPTV Service Provider Description
(SUBSCRIBE)


Description of Service providers 
(NOTIFY)�

Request for IPTV Service Description
(HTTP Request)


Description of Services 
(HTTP Response)�

sending the updates request


Records Changed�

Generating Notify Message�

�

�

Service 
Provider 
Discovery 
Process�

Service  
Discovery 
Process�


_1341258776.vsd
�

�

텍스트�

Terminal Device�

IPTV
Service Description provider�

Core
IMS�

IPTV Service Provider Description provider�

Request for IPTV Service Provider Description
(SUBSCRIBE)


Description of Service providers 
(NOTIFY)�

�

Description of Service providers 
(NOTIFY)�

Request for IPTV Service Provider Description
(SUBSCRIBE)


Request for IPTV Service Description
(HTTP Request)


Description of Services 
(HTTP Response)�

sending the updates request


Records Changed�

Generating Notify Message�

�

�

텍스트�

Service 
Provider 
Discovery 
Process�

Service  
Discovery 
Process�


_1312003715.vsd
�

�

Terminal Device�

IPTV
Service Description provider�

IPTV Service Provider Description provider�

Description of Service providers 
(MESSAGE)�


_1312005625.vsd
�

�

Terminal Device�

IPTV
Service Description provider�

Core
IMS�

IPTV Service Provider Description provider�

Request for IPTV Service Description
(SUBSCRIBE)


�

Description of Services 
(NOTIFY)�

Request for IPTV Service Description
(SUBSCRIBE)


Description of Services 
(NOTIFY)�


_1310999486.vsd
�

�

Terminal Device�

IPTV
Service Description provider�

Core
IMS�

IPTV Service Provider Description provider�

Description of Service providers 
(MESSAGE)�

�

Description of Service providers 
(MESSAGE)�


_1311166704.vsd
�

�

Terminal Device�

IPTV
Service Description provider�

IPTV Service Provider Description provider�

Request for IPTV Service Provider Description
(SUBSCRIBE)


Description of Service providers 
(NOTIFY)�


_1311166965.vsd
�

�

Terminal Device�

IPTV
Service Description provider�

IPTV Service Provider Description provider�

�

Description of Services 
(NOTIFY)�

Request for IPTV Service Description
(SUBSCRIBE)



_1306383204.vsd
�

�

Terminal Device�

IPTV
Service Description provider�

Core
IMS�

IPTV Service Provider Description provider�

Request for IPTV Service Provider Description
(SUBSCRIBE)


Description of Service providers 
(NOTIFY)�

�

Description of Service providers 
(NOTIFY)�

Request for IPTV Service Provider Description
(SUBSCRIBE)



