| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-F0062 |
| --- | --- |

| Title: | New aerial photography sequences for video coding standard development |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng, Weiran Li, Zisheng Cao, Wenyi Su, Chunting Zhao, Yujing Li, Zacharia Lorenz, Huy Wu, Ze Du, Dang Anh Hoang | Tel: Email: | xiaozhen.zheng@dji.com |
| Source: | DJI |  |  |

_____________________________

- Abstract

This document proposes three clips video test sequences for aerial photography. The sequence content description, RD performance and copyright are also included at the document.

Introduction

Unmanned aerial vehicle (UAV) becomes a popular photography and video capture device recently. Aerial photography shows wide applications in film-making, professional photography, outdoor travelling, industry device inspection, agricultural liquid fertilizers and herbicides and etc. Compared to the traditional scenarios, aerial photography scenario shows some different characterises. For examples, aerial photography sequences contain larger scenes with dramatically zoom-in, zoom-out, pan, rotation, pull-up and pull-down actions.

At Geneva meeting January 2017, some experts express the interests on drone sequences, and encourage people to submit new drone sequences [1]. Based on the previous discussion, we select three new test sequences, which cover typical aerial photography characterises, captured by DJI’s UAV device. Detailed information is show in the next three sections.

Sequences descriptions

All three test sequences are captured by DJI’s inspire 1 RAW UAV with 12bit cinemaDNG linear raw format. In order to get 4:20 YUV file from cinemaDNG file, the conversion processes are performed by the following two steps:

- DNG to TIF conversion: cinemaDNG files are converted to TIFF files with sRGB color domain and 16bits for every color channel. The format conversion between cinemaDNG and TIFF is performed by Adobe LightRoom Photoshop CC software.

- TIF to YUV conversion: TIFF files are converted to 4:2:0 10bit files by ffmpeg. The command line for this conversion is shown as below:

ffmpeg.exe -i input.tif -vcodec rawvideo -s 3840x2160 -r 30 -pix_fmt yuv420p10le output.yuv

All sequences after the above conversion have the same video parameters which are described in Table 1. The thumbnail of every sequence and its description are shown in Table2.

Table 1: Specification of aerial photography test sequences

| Resolution | Frame count | Frame rate(fps) | Bit Depth | Color Space |
| --- | --- | --- | --- | --- |
| 3840x2160 | 300 | 30 | 10 | 4:2:0 |

Table 2: Description of aerial photography test sequences

| Scenario | Description | Thumbnail |
| --- | --- | --- |
| BeachMountain_ 3840x2160_30fps_ 420_10bit | The scenario contains the left-down to right-up pull up motion. The beach, mountain and sky are shown in the scenario which has plentiful details. |  |
| MountainBay_ 3840x2160_30fps_ 420_10bit | The scenario contains slow clockwise rotation. The sea, mountain, sky and cloud are shown in the scenario which has plentiful details. |  |
| DroneTakeOff_ 3840x2160_30fps_ 420_10bit | A typical drone take-off scenario which contains fast pull up and horizontal spin motions. |  |

RD performance

RD performance of those test sequences are evaluated by HM16.14 and JEM5.0.1 platform. According to the content features and bitrates selection at the preliminary CfE document [2], each aerial sequence provided at this document has the specified bitrate ranges. The coding conditions are followed the description in the section “UHD 4:2:0 SDR” at the preliminary CfE document. The bitrate information for the three sequences evaluated at HM16.14 and JEM5.0.1 are provided at Table 3. The coding performance between HM16.14 and JEM5.0.1 for each sequence is summarized at Table 4.

Table 3.1 Bitrate, QP and coding results for HM16.14

| BeachMountain_3840x2160_30fps_420_10bit |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| target rate [Mbps] | bitrate [Mbps] | QP | PSNR Y [dB] | PSNR U [dB] | PSNR V [dB] |
| 10 | 10.008764 | 31.15 | 35.1105 | 46.995 | 44.0399 |
| 6 | 5.9676232 | 33.9433 | 33.7806 | 46.4994 | 43.5343 |
| 3.3 | 3.2775096 | 37 | 32.4784 | 45.5895 | 42.6594 |
| 2 | 1.9715968 | 39.83 | 31.4282 | 45.2967 | 42.3209 |
| MountainBay_3840x2160_30fps_420_10bit |  |  |  |  |  |
| target rate [Mbps] | bitrate [Mbps] | QP | PSNR Y [dB] | PSNR U [dB] | PSNR V [dB] |
| 4 | 4.0970872 | 29.94333 | 38.5756 | 47.1599 | 49.5532 |
| 2.4 | 2.4454752 | 32.98 | 37.2189 | 46.1333 | 48.6957 |
| 1.5 | 1.4823376 | 35.88667 | 35.9002 | 44.7768 | 47.587 |
| 1 | 0.9954896 | 38.37667 | 34.889 | 44.1017 | 46.9659 |
| DroneTakeOff_3840x2160_30fps_420_10bit |  |  |  |  |  |
| target rate [Mbps] | bitrate [Mbps] | QP | PSNR Y [dB] | PSNR U [dB] | PSNR V [dB] |
| 12 | 11.9946568 | 36 | 30.6664 | 37.5434 | 42.3772 |
| 10 | 9.9807344 | 37.32 | 29.9336 | 37.1663 | 42.1659 |
| 7 | 7.0090232 | 39.60333 | 28.70568 | 36.61664 | 41.89998 |
| 3 | 2.9963288 | 47.49 | 26.0896 | 34.9939 | 40.8189 |

Table 3.2 Bitrate, QP and coding results for JEM5.0.1

|  | BeachMountain_3840x2160_30fps_420_10bit |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| target rate [Mbps] |  | bitrate [Mbps] | RAS# | QP | PSNR Y [dB] | PSNR U [dB] | PSNR V [dB] |
| 10 |  | 9.8101992 | 0 | 30.48485 | 35.492 | 48.2086 | 44.8552 |
| 6 |  | 6.0307072 | 2 | 33.484845 | 34.185 | 47.7738 | 44.3155 |
| 3.3 |  | 3.314812 | 1 | 36.484845 | 32.8599 | 47.296 | 43.744 |
| 2 |  | 1.9791064 | 3 | 39.48485 | 31.8309 | 46.7991 | 43.2324 |
|  | MountainBay_3840x2160_30fps_420_10bit |  |  |  |  |  |  |
| target rate [Mbps] |  | bitrate [Mbps] | RAS# | QP | PSNR Y [dB] | PSNR U [dB] | PSNR V [dB] |
| 4 |  | 3.927868 | 3 | 29.48485 | 39.0233 | 49.1374 | 50.3115 |
| 2.4 |  | 2.013612 | 3 | 33.48485 | 37.3463 | 47.9862 | 49.4064 |
| 1.5 |  | 1.4937672 | 6 | 35.48485 | 36.612 | 47.3536 | 48.9035 |
| 1 |  | 1.00852 | 2 | 37.48485 | 35.6548 | 46.7204 | 48.3882 |
|  | DroneTakeOff_3840x2160_30fps_420_10bit |  |  |  |  |  |  |
| target rate [Mbps] |  | bitrate [Mbps] | RAS# | QP | PSNR Y [dB] | PSNR U [dB] | PSNR V [dB] |
| 12 |  | 11.8098728 | 5 | 35.48485 | 31.4597 | 38.1428 | 42.7585 |
| 10 |  | 9.8860048 | 3 | 36.48485 | 30.775 | 37.8004 | 42.5892 |
| 7 |  | 7.0155216 | 1 | 38.48485 | 29.5624 | 37.2688 | 42.322 |
| 3 |  | 2.9481456 | 1 | 45.48485 | 26.8123 | 35.7136 | 41.3885 |

Table 4: BD-RATE results between HM16.14 and JEM5.0.1

| BeachMountain_3840x2160_30fps_420_10bit |  |  |  |  |  |  |  | BDRATE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HM16.14 |  |  |  |  |  |  |  |  |
| 10.008764 | 35.1105 | 46.995 | 44.0399 | 9.8101992 | 35.492 | 48.2086 | 44.8552 | -15.3% |
| 5.9676232 | 33.7806 | 46.4994 | 43.5343 | 6.0307072 | 34.185 | 47.7738 | 44.3155 |  |
| 3.2775096 | 32.4784 | 45.5895 | 42.6594 | 3.314812 | 32.8599 | 47.296 | 43.744 |  |
| 1.9715968 | 31.4282 | 45.2967 | 42.3209 | 1.9791064 | 31.8309 | 46.7991 | 43.2324 |  |
| MountainBay_3840x2160_30fps_420_10bit |  |  |  |  |  |  |  | BDRATE |
| HM16.14 |  |  |  | JEM5.0.1 |  |  |  |  |
| bitrate | PSNR Y | PSNR U | PSNR V | bitrate | PSNR Y | PSNR U | PSNR V | -22.2% |
| 4.0970872 | 38.5756 | 47.1599 | 49.5532 | 3.927868 | 39.0233 | 49.1374 | 50.3115 |  |
| 2.4454752 | 37.2189 | 46.1333 | 48.6957 | 2.013612 | 37.3463 | 47.9862 | 49.4064 |  |
| 1.4823376 | 35.9002 | 44.7768 | 47.587 | 1.4937672 | 36.612 | 47.3536 | 48.9035 |  |
| 0.9954896 | 34.889 | 44.1017 | 46.9659 | 1.00852 | 35.6548 | 46.7204 | 48.3882 |  |
| DroneTakeOff_3840x2160_30fps_420_10bit |  |  |  |  |  |  |  | BDRATE |
| HM16.14 |  |  |  | JEM5.0.1 |  |  |  |  |
| bitrate | PSNR Y | PSNR U | PSNR V | bitrate | PSNR Y | PSNR U | PSNR V | -22.4% |
| 11.9946568 | 30.6664 | 37.5434 | 42.3772 | 11.8098728 | 31.4597 | 38.1428 | 42.7585 |  |
| 9.9807344 | 29.9336 | 37.1663 | 42.1659 | 9.8860048 | 30.775 | 37.8004 | 42.5892 |  |
| 7.0090232 | 28.70568 | 36.61664 | 41.89998 | 7.0155216 | 29.5624 | 37.2688 | 42.322 |  |
| 2.9963288 | 26.0896 | 34.9939 | 40.8189 | 2.9481456 | 26.8123 | 35.7136 | 41.3885 |  |

Copyright information

The proposed test sequences in this document and all intellectual property rights therein remain the property of SZ DJI Technology Co., Ltd.

The following uses are allowed for the proposed sequences:
·         Can be published in technical papers, played at technology research and development events.
·         Can be used by standards committees.

The following uses are not allowed for the proposed sequences:
·         Do not publish snapshots in product brochures.
·         Do not redistribute video with a commercial product.
·         Do not use in television shows, commercials, or movies.

References

[1] T. Suzuki, BoG report on test material, 5th Meeting: Geneva, CH, 12–20 January 2017

[1] M. Wien, V. Baroncini, J. Boyce, A. Segall, T. Suzuki, Preliminary Joint Call for Evidence on Video Compression with Capability beyond HEVC, 5th Meeting: Geneva, CH, 12–20 January 2017

Patent rights declaration(s)

SZ DJI Technology Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).