| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0014-v4 |
| --- | --- |

| Title: | Description of SDR, HDR, and 360° video coding technology proposal by Fraunhofer HHI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | M. Albrecht, C. Bartnik, S. Bosse, J. Brandenburg, B. Bross, J. Erfurt, V. George, P. Haase, P. Helle, C. Helmrich, A. Henkel, T. Hinz, S. de Luxan Hernandez, S. Kaltenstadler, P. Keydel, H. Kirchhoffer, C. Lehmann, W.-Q. Lim, J. Ma, D. Maniry, D. Marpe, P. Merkle, T. Nguyen, J. Pfaff, J. Rasch, R. Rischke, C. Rudat, M. Schaefer, T. Schierl, H. Schwarz, M. Siekmann, R. Skupin, B. Stallenberger, J. Stegemann, K. Suehring, G. Tech, G. Venugopal, S. Walter, A. Wieckowski, T. Wiegand, M. Winken | Email: | heiko.schwarz@hhi.fraunhofer.de detlev.marpe@hhi.fraunhofer.de thomas.wiegand@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

Version 4 provides a link to software (in sec. 8)

# Abstract

This document describes Fraunhofer HHI’s response to the Call for Proposals. The proposal is based on Fraunhofer HHI’s NextSoftware, which has been presented in JVET-I0034 and represents an alternative implementation of JEM-7.0. The contribution proposes the following additional coding tools: Generalized binary block partitioning, line-based intra coding mode, intra prediction mode with neural networks, intra region-based template matching, bilateral filter for intra reference sample smoothing, multi-reference line intra prediction, multi-hypothesis inter prediction, restricted merge mode, signal-dependent boundary padding for motion compensation, diffusion filter and DCT thresholding for prediction signal filtering, modified adaptive transforms for intra blocks, dependent scalar quantization, modified QP prediction, modified arithmetic coding engine, modified coding of transform coefficient levels, and a modified adaptive loop filter. The proposal does not include any HDR or 360° video specific coding tools.

Relative to the JEM-7.0 anchors, the following BD rates are reported: -7.5%, -6.9%, -6.0% (Y, U, V) for SDR constraint set 1; -7.2%, -7.6%, -5.7% (Y, U, V) for SDR constraint set 2, -8.0%, -17.3%, -12.7% (Y, U, V PSNR) for HDR; -15.7%, -16.3%, -14.5% (Y, U, V E2E WS-PSNR) for 360°. In comparison to the HM 16.16 anchors, the following BD rates are reported: -38.1%, -46.9%, -46.5% (Y, U, V) for SDR constraint set 1; -29.7%, -46.5%, -45.5% (Y, U, V) for SDR constraint set 2, -32.7%, -62.3%, -58.1% (Y, U, V PSNR) for HDR; -35.7%, -52.7%, -53.7% (Y, U, V E2E WS-PSNR) for 360°.

If only proposed coding tools are enabled, the following luma BDR rates relative to the HM anchor are reported: -24% for SDR constraint set 1, -20% for SDR constraint set 2, -22% for HDR (PSNR), and -29% for 360° (E2E WS-PSNR).

It is further reported that by changing the relationship between luma Qp and chroma QP, the following BD rates are obtained relative to the JEM-7.0 anchors: -13.0%, +23.4%, +22.6% (Y, U, V) for SDR constraint set 1; -10.5%, +8.8% +22.3% (Y, U, V) for HDR. This corresponds to the following BD rates relative to the HM anchors: -42.3%, -25.5%, -26.3% (Y, U, V) for SDR constraint set 1; -35.1% -48.1%, -29.0% (Y, U, V) for HDR.

# Contents

Abstract	i

Contents	ii

1	Introduction	1

2	Decoder algorithm description	2

2.1	General algorithm description	2

2.1.1	Bitstream structure	2

2.1.2	Block partitioning	13

2.1.3	Entropy decoding	18

2.1.4	Conversion process from entropy decoded data to symbols	20

2.1.5	Decoder-side motion refinement	30

2.1.6	Inverse quantization	31

2.1.7	Inverse transforms	34

2.1.8	Motion compensation	36

2.1.9	Intra prediction	39

2.1.10	In-loop filtering	55

2.1.11	Additional tools	56

2.1.12	Additional algorithmic description topic(s)	63

2.2	360° Video	63

2.2.1	Projection format (including padding)	63

2.2.2	Quantization handling	65

2.2.3	360° video specific decoding tools	65

2.3	HDR	66

2.3.1	Post-processing	66

2.3.2	Quantization handling	66

2.3.3	HDR specific decoding tools	66

3	Encoder algorithm description	66

3.1	General algorithm description	66

3.1.1	Rate-distortion optimization	66

3.1.2	Partitioning structure selection	67

3.1.3	Intra prediction mode estimation and type selection	68

3.1.4	Motion estimation and motion segment selection	70

3.1.5	Mode decisions	71

3.1.6	In-loop filtering type selection and parameter estimation	71

3.1.7	Transforms and transform type selection	72

3.1.8	Quantization and quantization type selection	72

3.1.9	Entropy coding	74

3.1.10	Additional tools	75

3.1.11	Additional encoder optimization	75

3.1.12	Additional algorithmic description topic(s)	78

3.2	360°	79

3.2.1	Usage of projection format adaptation	79

3.2.2	360° Video specific encoding tools	79

3.2.3	Additional encoder optimization	79

3.3	HDR	79

3.3.1	Pre-processing	79

3.3.2	HDR specific encoding tools	79

3.3.3	Additional encoder optimization	79

4	Compression performance	80

4.1	SDR: Constraint set 1 configuration relative to HM anchor	80

4.1.1	Class SDR-A (RD-curves)	80

4.1.2	Class SDR-B (RD-curves)	81

4.1.3	Overall (BD-rate summary tables)	82

4.2	SDR: Constraint set 2 configuration relative to HM anchor	82

4.2.1	Class SDR-B	82

4.2.2	Overall	83

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	83

4.3.1	Class SDR-A	83

4.3.2	Class SDR-B	84

4.3.3	Overall	86

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	86

4.4.1	Class SDR-B	86

4.4.2	Overall	87

4.5	HDR: Constraint set 1 configuration relative to HM anchor	87

4.5.1	Class HDR-A	87

4.5.2	Class HDR-B	88

4.5.3	Overall	89

4.6	HDR: Constraint set 1 configuration relative to JEM anchor	90

4.6.1	Class HDR-A	90

4.6.2	Class HDR-B	90

4.6.3	Overall	92

4.7	360° Video: Constraint set 1 configuration relative to HM anchor	92

4.7.1	Class 360	92

4.7.2	Overall	93

4.8	360° Video: Constraint set 1 configuration relative to JEM anchor	93

4.8.1	Class 360	93

4.8.2	Overall	95

5	Decoder complexity analysis	95

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	95

5.2	Description of computing platform used to determine decoding times reported in section 5.1	97

5.3	Memory usage of decoder	97

5.4	Complexity characteristics of decoder entropy decoding operation	97

5.5	Complexity characteristics of decoder inverse quantization	98

5.6	Complexity characteristics of decoder inverse transform operation	98

5.7	Complexity characteristics of decoder-side motion refinement	98

5.8	Complexity characteristics of decoder motion compensation	98

5.9	Complexity characteristics of decoder intra-frame prediction operation	98

5.10	Complexity characteristics of decoder in-loop filtering operation	99

5.11	Complexity characteristics of additional decoder tools	99

5.12	Complexity characteristics of 360° video specific decoding tools	100

5.13	Complexity characteristics of HDR specific decoding tools	100

5.14	Degree of capability for decoder parallel processing	100

6	Encoder complexity analysis	100

6.1	Encoding time and measurement methodology	100

6.2	Description of computing platform used to determine encoding times reported in section 6.1	102

6.3	Memory usage of encoder	102

6.4	Complexity characteristics of encoder partitioning	102

6.5	Complexity characteristics of encoder intra-frame prediction type selection	103

6.6	Complexity characteristics of encoder motion estimation and motion segmentation selection	103

6.6.1	General	103

6.6.2	Complexity characteristics of usage of decoder-side motion refinement	103

6.6.3	Complexity characteristics of multi-hypotheses motion estimation	103

6.7	Complexity characteristics of encoder transforms and transform type selection	103

6.8	Complexity characteristics of encoder quantization and quantization type selection	104

6.9	Complexity characteristics of encoder in-loop filtering type selection and parameter estimation	104

6.10	Complexity characteristics of encoder entropy coding type selection	104

6.11	Complexity characteristics of additional encoder tools	104

6.12	Complexity characteristics of 360° video specific encoding tools	104

6.13	Complexity characteristics of HDR specific encoding tools	104

6.14	Degree of capability for encoder parallel processing	104

7	Algorithmic characteristics	105

7.1	Random access characteristics	105

7.2	Delay characteristics	105

8	Software implementation description	105

9	Highlighted aspects discussion	105

10	Closing remarks	108

11	References	108

12	Patent rights declaration(s)	109

13	Annex: Questionnaire	110

13.1	Codec level questions	110

13.2	Tool level questions	111

# Introduction

This documents describes video coding technology proposed by Fraunhofer HHI in response to the call for proposals (CfP) [1] issued jointly by ISO/IEC JTC 1/SC 29/WG 11 (MPEG) and ITU-T SG 16 Q6 (VCEG).  The video codec follows the concept of block-based hybrid video coding, where blocks of samples are first predicted using intra- or inter-picture prediction and then the resulting prediction error blocks are transmitted using transform coding.

The proposal is based on the NextSoftware [3], which represents an alternative implementation of the Joint Exploration Model (JEM).  The following coding tools are proposed:

- Block partitioning:

  - Generalized binary block partitioning (sec. 2.1.2)

- Intra-picture prediction and coding:

  - Line-based intra coding (sec 2.1.9.1)

  - Intra prediction with neural networks (sec. 2.1.9.2)

  - Intra region-based template matching (sec. 2.1.9.3)

  - Bilateral filter for intra reference sample smoothing (sec. 2.1.9.4)

  - Multi-reference line intra prediction (sec. 2.1.9.5)

- Inter-picture prediction:

  - Multi-hypothesis inter prediction (sec 2.1.8.1)

  - Restricted merge mode (sec. 2.1.8.2)

  - Signal-dependent boundary padding (sec. 2.1.8.3)

- Filters for improving intra- and inter prediction signals:

  - Diffusion filter (sec. 2.1.11.1)

  - DCT thresholding filter (sec. 2.1.11.2)

- Transforms and Quantization:

  - Improvement of adaptive transforms [2] (sec. 2.1.7)

  - Dependent scalar quantization (sec. 2.1.6)

  - Modified QP prediction (sec. 2.1.4.3)

- Entropy Coding:

  - Arithmetic coding engine (sec. 2.1.3.1)

  - CTU-line based context initialization from previous frames (sec. 2.1.3.2)

  - Binarization, coding order, and context modeling for transform coefficients (sec. 2.1.4.4)

- In-loop filter:

  - Improvement of adaptive loop filter [2] (sec. 2.1.10)

When only these tools, with exception of the adaptive loop filter, are enabled in addition to HEVC, BDR rate savings of about 22% for SDR constraint set 1, 18% for SDR constraint set 2, 20% for HDR (PSNR), and 27% for 360° (E2E WS-PSNR) are obtained. In that configuration, the decoder run time is about 130%-140% of the HM decoder run time (20% JEM decoder run time).

Additionally enabling the adaptive loop filter yields BDR rate savings of about 24% for SDR constraint set 1, 20% for SDR constraint set 2, 22% for HDR (PSNR), and 29% for 360° (E2E WS-PSNR). The decoder run time is then increased to about 200% of the HM decoder run time (about 27% of the JEM decoder run time).

In addition, the proposed codec includes the following JEM tools [2] without modifications:

- Intra-picture prediction and coding:

  - 67 intra prediction modes

  - 4-tap interpolation filter for intra prediction

  - Boundary filter for intra prediction signals

  - Cross-component linear model prediction

  - Position-dependent intra prediction combination in planar mode

  - Multiple cross-component intra prediction modes

- Inter-picture prediction:

  - Sub-PU level motion vector derivation

  - Locally adaptive motion vector resolution

  - 1/16-th luma sample accurate motion vectors

  - Overlapped block motion compensation

  - Local illumination compensation

  - Affine motion prediction

  - Pattern matched motion vector derivation

  - Decoder-side motion vector refinement

  - Bi-directional optical flow

- In-loop filter:

  - Bilateral in-loop filter

  - Content-adaptive clipping

By additionally enabling these tools, the coding efficiency improvements are increased to about 38% for SDR constraint set 1, 30% for SDR constraint set 2, 33% for HDR (PSNR), and 36% for 360° (E2E WS-PSNR) are obtained. These results are achieved at decoder run time of about 700-800% of the HM decoder run time (100-120% JEM decoder run time).

Fraunhofer HHI submitted two sets of bitstreams. Both sets of bitstreams use the same bitstream syntax and decoding process. The only difference is that for one set of bitstreams, the perceptually optimized QP adaptation (QPA) described in sec. 3.1.11.1 was used. For the other set of bitstreams, the same QP setting as for the anchors was used (fixed QP for SDR and 360°, Sharp delta QP for HDR).

# Decoder algorithm description

In the following, the newly added coding tools are described. For a description of the JEM coding tools used (see sec. 1), the reader is referred to the algorithmic description of JEM coding tools in [2].

## General algorithm description

### Bitstream structure

The proposed video codec uses the NAL unit syntax of HEVC.  Moreover, the same high-level syntax as in HEVC is used, with the following exception. The sequence parameter set (SPS) syntax structure is extended. The added SPS syntax elements are used for enabling/disabling particular coding tools or selecting particular operating points for coding tools.

The low-level syntax of HEVC was modified in a way that additional syntax elements for the added coding tools are included. An overview of the low-level syntax is given in the following subsections. Syntax elements that are added for JEM coding tools are marked blue, while syntax elements that are associated with additional coding tools are marked red.

#### Coding tree unit syntax

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> CtbLog2SizeY, yCtb >> CtbLog2SizeY ) |  |
| coding_tree( xCtb, yCtb, CtbSizeX, CtbSizeY, treeType, 0, 0 ) |  |
| } |  |

#### Coding tree syntax

| coding_tree( x0, y0, cbSizeX, cbSizeY, treeType, btDepth, lastSplitDir ) { | Descriptor |
| --- | --- |
| if( canSplit ) |  |
| split_mode( x0, y0, cbSizeX, cbSizeY, treeType, btDepth, lastSplitDir ) |  |
| } |  |
| if( cu_qp_delta_enabled_flag && floor( log2[cbSizeX] ) >= Log2MinCuQpDeltaSize && floor( log2[cbSizeY] ) >= Log2MinCuQpDeltaSize ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| } |  |
| if( split_perpendicular_flag[x0][y0][cbSizeX][cbSizeY] ¦ ¦ split_parallel_flag[x0][y0][cbSizeX][cbSizeY] ) { |  |
| x1 = x0 + ( splitDir[x0][y0][cbSizeX][cbSizeY] = = 0 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : 0 ) |  |
| y1 = y0 + ( splitDir[x0][y0][cbSizeX][cbSizeY] = = 1 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : 0 ) |  |
| cbSizeX0 = cbSizeX − ( splitDir[x0][y0][cbSizeX][cbSizeY] = = 0 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : 0 ) |  |
| cbSizeX1 = ( splitDir[x0][y0][cbSizeX][cbSizeY] = = 0 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : cbSizeX ) |  |
| cbSizeY0 = cbSizeY − ( splitDir[x0][y0][cbSizeX][cbSizeY] = = 1 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : 0 ) |  |
| cbSizeY1 = ( splitDir[x0][y0][cbSizeX][cbSizeY] = = 1 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : cbSizeY ) |  |
| coding_tree( x0, y0, cbSizeX0, cbSizeY0, treeType, qtDepth, btDepth + 1, splitDir[x0][y0][cbSizeX][cbSizeY] ) |  |
| coding_tree( x1, y1, cbSizeX1, cbSizeY1, treeType, qtDepth, btDepth + 1, splitDir[x0][y0][cbSizeX][cbSizeY] ) |  |
| } else { |  |
| coding_unit( x0, y0, cbSizeX, cbSizeY, treeType ) |  |
| } |  |
| } |  |

#### Split mode syntax

| split_mode( x0, y0, cbSizeX, cbSizeY, treeType, btDepth, lastSplitDir ) { | Descriptor |
| --- | --- |
| if( canNotSplit && ( canParlSplit ¦ ¦ canPerpSplit ) ) { |  |
| bt_split_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| if( bt_split_flag[x0][y0] ) { |  |
| if( canParlSplit && canPerpSplit ) { |  |
| split_perpendicular_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| split_parallel_flag_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] = 1 − split_perpendicular_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] |  |
| if( split_perpendicular_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ¦ ¦ split_parallel_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ) { |  |
| splitDir[x0][y0][cbSizeX][cbSizeY] = split_perpendicular_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ? 1 − lastSplitDir : lastSplitDir |  |
| splitSize = splitDir[x0][y0][cbSizeX][cbSizeY] == 0 ? cbSizeX : cbSizeY |  |
| if( canHalfSplit && ( canFourthsSplit ¦ ¦ canEightsSplit ¦ ¦ canThirdsSplit ¦ ¦ canFifthSplit ) ) { |  |
| split_symmetrical_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| if( !split_symmetrical_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ) { |  |
| if( splitSize == ( 1 << log2( splitSize ) ) ) { |  |
| if( canFourthsSplit && canEightsSplit ) { |  |
| split_eights_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| } |  |
| if( canFourthsSplit ¦¦ canEightsSplit ¦¦ canThirdsSplit ) { |  |
| if( ( canThirdsSplit && is13Avail && is23Avail ) ¦¦ ( split_eights_flag[x0][y0][cbSizeX][cbSizeY] && is38Avail && is58Avail ) ¦¦ ( !split_eights_flag[x0][y0][cbSizeX][cbSizeY] && is14Avail && is34Avail ) ) { |  |
| is_split_offset_large[x0][y0][cbSizeX][cbSizeY] | ae(v) |
| } |  |
| splitBase = splitSize / ( canThirdsSplit ? 3 : ( split_eights_flag[x0][y0][cbSizeX][cbSizeY] ? 8 : 4 ) ) |  |
| splitOffset[x0][y0][cbSizeX][cbSizeY] = ( ( canThirdsSplit ¦ ¦ !split_eights_flag[x0][y0][cbSizeX][cbSizeY] ) ? splitBase : 3 * splitBase ) + is_split_offset_large[x0][y0][cbSizeX][cbSizeY] ? ( canThirdsSplit ? 1 : 2 ) * splitBase : 0 |  |
| } else { |  |
| if( ( is25Avail ¦¦ is35Avail ) && ( is15Avail ¦¦ is45Avail ) ) { |  |
| split_fifths_is235_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| fifths_split_id = 1 |  |
| if( split_fifths_is235_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ) { |  |
| if( is25Avail && is35Avail ) { |  |
| split_fifths_is35_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| fifths_split_id = split_fifths_is35_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ? 3 : 2 |  |
| } else { |  |
| if( is15Avail && is45Avail ) { |  |
| split_fifths_is45_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| if( split_fifths_is45_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ) { |  |
| fifths_split_id = 4 |  |
| } |  |
| } |  |
| splitOffset[x0][y0][cbSizeX][cbSizeY] = fifths_split_id * splitSize / 5 |  |
| } |  |
| } else { |  |
| splitOffset[x0][y0][cbSizeX][cbSizeY] = splitSize / 2 |  |
| } |  |
| } |  |
| } else { |  |
| split_perpendicular_flag[x0][y0][cbSizeX][cbSizeY] = 0 |  |
| split_parallel_flag[x0][y0][cbSizeX][cbSizeY] = 0 |  |
| } |  |
| } |  |

The values of the split decision variable canParlSplit, canPerpSplit, canFourthsSplit, canEightsSplit, canThirdsSplit and canFifthsSplit depend on availability constraints such as minimal block sizes, maximal tree depths and the prohibitions of redundant splits (see sec. 2.1.2).

#### Coding unit syntax

| coding_unit( x0, y0, cuWidth, cuHeight, treeMode ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !cu_skip_flag[ x0 ][ y0 ] ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| cu_pred_data_intra( x0, y0, cuWidth, cuHeight, treeMode ) |  |
| else |  |
| cu_pred_data_inter( x0, y0, cuWidth, cuHeight, treeMode ) |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( diffusion_filter_enabled_flag && x0 != 0 && y0 != 0 && !intra_ftm_flag[ x0 ][ y0 ] && mrl_idx[ x0 ][ y0 ] = = 0 && !intra_nn_flag[ x0 ][ y0 ] && ( cuWidth * cuHeight > 256 ) && ( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && NumDiffusionFiltersIntra > 0 ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && NumDiffusionFiltersInter > 0 ) ) && ( ( merge_flag[ x0 ][ y0 ] && fruc_merge_mode[ x0 ][ y0 ] = = 0 ) ¦ ¦ !merge_flag[ x0 ][ y0 ] ) |  |
| cu_diffusion_filter_idx[ x0 ][ y0 ] | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( merge_flag[ x0 ][ y0 ] && cu_diffusion_filter_idx[ x0 ][ y0 ] = = 0 && !additional_hypothesis_flag[ x0 ][ y0 ][ 0 ] && merge_ref_pic_list_idc[ x0 ][ y0 ] = = LIST_NONE ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) |  |
| cu_residual_data( x0, y0, cuWidth, cuHeight, treeMode ) |  |
| if( nsst_enabled_flag && ( ( intra_nn_flag[ x0 ][ y0 ] && nonZeroCoeffsNonTs > 2 ) ¦ ¦ ( treeMode = = DUAL_CHROMA && ( cuWidth >= 4 && cuHeight >= 4 ) ) && ( nonZeroCoeffsNonTsChroma > 1 ) ) ) |  |
| cu_nsst_idx[ x0 ][ y0 ] | ae(v) |
| if( sot_enabled_flag && treeMode != DUAL_CHROMA && cu_nsst_idx[ x0 ][ y0 ] = = 0 && !intra_ftm_flag[ x0 ][ y0 ] && residual_1d_partitions_mode[ x0 ][ y0 ] = = NO_SPLIT && cu_fiffision_filter_idx[ x0 ][ y0 ] = = 0 && !transform_skip_flag[ x0 ][ y0 ][ 0 ] && cbf_luma[ x0 ][ y0 ] && cuWidth <= 64 & cuHeight <= 64 ) { |  |
| cu_sot_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_sot_flag[ x0 ][ y0 ] && ( CuPredMode = = MODE_INTER ¦ ¦ nonZeroCoeffsNonTsLuma > 2 ) ) |  |
| cu_sot_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |

#### Coding unit intra prediction data syntax

| cu_pred_data_intra( x0, y0, cuWidth, cuHeight, treeMode ) { | Descriptor |
| --- | --- |
| if( mrl_enabled_flag && treeMode != DUAL_CHROMA && ( cuWidth − cuHeight ) > 1 ) cuWidth >= 8 && cuHeight >= 8 ) |  |
| mrl_idx[ x0 ][ y0 ] | ae(v) |
| if( pcm_enabled_flag && cuWidth >= MinIpcmCbSizeY && cuWidth <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| if( intra_nn_enabled_flag && treeMode != DUAL_CHROMA && mrl_idx[ x0 ][ y0 ] = = 0 && cuWidth <= 64 && cuHeight <= 64 ) |  |
| intra_nn_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_nn_flag[ x0 ][ y0 ] ) { |  |
| if( cuWidth != 64 && cuHeight != 64 && cuWidth = = cuHeight && cuWidth >= 32 ) |  |
| intra_nn_subsampling_flag[ x0 ][ y0 ] | ae(v) |
| intra_nn_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( intra_rtm_enabled_flag && mrl_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_rtm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_rtm_flag[ x0 ][ y0 ] ) |  |
| intra_rtm_region_idx[ x0 ][ y0 ] | ae(v) |
| else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| if ( treeMode = = SINGLE_LUMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

#### Coding unit inter prediction data syntax

| cu_pred_data_inter( x0, y0, cuWidth, cuHeight, treeMode ) { | Descriptor |
| --- | --- |
| if( !cu_skip_flag[ x0 ][ y0 ] ) |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( fruc_merge_enabled_flag ) |  |
| fruc_merge_mode[ x0 ][ y0 ] | ae(v) |
| if( affine_enabled_flag && fruc_merge_mode[ x0 ][ y0 ] = = 0 && cuWidth > 8 && cuHeight > 8 ) |  |
| affine_flag[ x0 ][ y0 ] | ae(v) |
| if( restricted_merge_enabled_flag && !cu_skip_flag[ x0 ][ y0 ] && !affine_flag[ x0 ][ y0 ] && slice_type = = B ) |  |
| merge_ref_pic_list_idc[ x0 ][ y0 ] | ae(v) |
| if( fruc_merge_mode[ x0 ][ y0 ] = = 0 && MaxNumMergeCand > 1 !affine_flag[ x0 ][ y0 ] ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !cu_skip_flag[ x0 ][ y0 ] && slice_type = = B ) |  |
| multi_hyp_data( x0, y0 ) |  |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( affine_enabled_flag && cuWidth > 8 && cuHeight > 8 ) |  |
| affine_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| if( affine_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 0, AFFINE_LEFT ) |  |
| mvd_coding( x0, y0, 0, AFFINE_RIGHT ) |  |
| } else |  |
| mvd_coding( x0, y0, 0, AFFINE_OFF ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else if( affine_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 1, AFFINE_LEFT ) |  |
| mvd_coding( x0, y0, 1, AFFINE_RIGHT ) |  |
| } else |  |
| mvd_coding( x0, y0, 1, AFFINE_OFF ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( slice_type = = B ) |  |
| multi_hyp_data( x0, y0 ) |  |
| if( imv_enabled_flag && CuHasNonZeroMvd ) |  |
| imv_mode[ x0 ][ y0 ] |  |
| if( obmc_enabled_flag && cuWidth < = 16 ) |  |
| obmc_flag[ x0 ][ y0 ] |  |
| } |  |
| if( lic_enabled_flag && !affine_flag[ x0 ][ y0 ] && fruc_merge_mode[ x0 ][ y0 ] > 0 && !cu_skip_flag[ x0 ][ y0 ] ) |  |
| lic_flag[ x0 ][ y0 ] |  |
| } |  |

#### Multi hypothesis data syntax

| multi_hyp_data( x0, y0 ) { | Descriptor |
| --- | --- |
| if( multi_hypothesis_enabled_flag ) { |  |
| i = 0 |  |
| readMore = 1 |  |
| while( i < MaxNumAdditionalHypothesis && readMode ) { |  |
| additional_hypothesis_flag[ x0 ][ y0 ][ i ] | ae(v) |
| if( additional_hypothesis_flag[ x0 ][ y0 ][ i ] ) { |  |
| ref_idx_add_hyp[ x0 ][ y0 ][ i ] | ae(v) |
| mvd_coding( x0, y0, 2 + i, AFFINE_OFF ) |  |
| mvp_add_hyp_flag[ x0 ][ y0 ][ i ] | ae(v) |
| add_hyp_weight_idx[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| readMore = additional_hypothesis_flag[ x0 ][ y0 ][ i ] |  |
| i+ + |  |
| } |  |
| } |  |
| } |  |

#### Motion vector difference syntax

| mvd_coding( x0, y0, refList, affineMode ) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater1_flag[ 0 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater1_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ] ) |  |
| abs_mvd_minus2[ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 1 ] ) { |  |
| if( abs_mvd_greater1_flag[ 1 ] ) |  |
| abs_mvd_minus2[ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

#### Coding unit residual data syntax

| cu_residual_data( x0, y0, cuWidth, cuHeight, treeMode ) { | Descriptor |
| --- | --- |
| if( residual_1d_partitions_enabled_flag && cu_diffusion_filter_idx = = 0 && mrl_idx[ x0 ][ y0 ] = = 0 && !intra_nn_flag[ x0 ][ y0 ] && treeMode != DUAL_CHROMA && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| residual_1d_partitions_mode[ x0 ][ y0 ] | ae(v) |
| if( treeMode = = SINGLE_LUMA ) { |  |
| cbf_cb[ x0 ][ y0 ] | ae(v) |
| cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ cbf_cb[ x0 ][ y0 ] ¦ ¦ cbf_cr[ x0 ][ y0 ] ) |  |
| cbf_luma[ x0 ][ y0 ] | ae(v) |
| if( thresholding_enabled_flag && && treeMode != DUAL_CHROMA residual_1d_partitions_mode[ x0 ][ y0 ] = = NO_SPLIT && ( cuWidth > = 16 ¦ ¦ cuHeight > = 16 ) && cuHeight = = cuWidth ) { |  |
| thresholding_flag[ x0 ][ y0 ] | ae(v) |
| if( thresholding_flag[ x0 ][ y0 ] ) { |  |
| thresholding_size[ x0 ][ y0 ] | ae(v) |
| thresholding_threshold[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( cbf_luma[ x0 ][ y0 ] ¦ ¦ cbf_cb[ x0 ][ y0 ] ¦ ¦ cbf_cr[ x0 ][ y0 ] ) { |  |
| delta_qp( ) |  |
| if( cbf_cb[ x0 ][ y0 ] ¦ ¦ cbf_cr[ x0 ][ y0 ] && !cu_transquant_bypass_flag ) |  |
| chroma_qp_offset( ) |  |
| if( cbf_luma[ x0 ][ y0 ] ) { |  |
| if( residual_1d_partitions_mode[ x0 ][ y0 ] = = NO_SPLIT ) |  |
| residual_coding( x0, y0, cuWidth, cuHeight, treeMode, 0 ) |  |
| else { |  |
| if( residual_1d_partitions_mode[ x0 ][ y0 ] = = SPLIT_VER ) { |  |
| for( i = 0; i < cuWidth − 1; i+ + ) |  |
| residual_coding( x0 + i, y0, 1, cuHeight, treeMode, 0 ) |  |
| } else if( residual_1d_partitions_mode[ x0 ][ y0 ] = = SPLIT_HOR ) { |  |
| for( i = 0; i < cuHeight − 1; i+ + ) |  |
| residual_coding( x0, y0 + i, cuWidth, 1, treeMode, 0 ) |  |
| } |  |
| } |  |
| } |  |
| if( cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, cuWidth / 2, cuHeight / 2, treeMode, 1 ) |  |
| if( cbf_cr[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, cuWidth / 2, cuHeight / 2, treeMode, 2 ) |  |
| } |  |
| } |  |

#### Residual coding syntax

| residual_coding( x0, y0, tuWidth, tuHeight, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && ( log2TrafoSize <= Log2MaxTransformSkipSize ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( tuWidth > 1 ) |  |
| last_sig_coeff_x_prefix | ae(v) |
| if( tuHeight > 1 ) |  |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| subBlockW = 4 |  |
| subBlockH = 4 |  |
| maxPosInSb = 15 |  |
| if( tuWidth = = 1 ) { |  |
| lastSubBlock = tuHeight >> 2 |  |
| lastScanPos = LastSignificantCoeffY % 4 |  |
| maxPosInSb = 3 |  |
| subBlockW = 1 |  |
| if( tuHeight = = 1 ) |  |
| lastSubBlock = tuWidth >> 2 |  |
| lastScanPos = LastSignificantCoeffX % 4 |  |
| maxPosInSb = 3 |  |
| subBlockH = 1 |  |
| else { |  |
| lastScanPos = 16 |  |
| lastSubBlock = ( tuWdith >> 2 ) * ( tuHeight >> 2 ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = 16 |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = ScanOrder[ tuWidth ][ tuHeight ][ scanIdx ][ lastSubBlock ][ 0 ] |  |
| yS = ScanOrder[ tuWidth ][ tuHeight ][ scanIdx ][ lastSubBlock ][ 1 ] |  |
| xC = ( xS << 2 ) + ScanOrder[ 4 ][ 4 ][ scanIdx ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 4 ][ 4 ][ scanIdx ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| } |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| xS = ScanOrder[ tuWidth ][ tuHeight ][ scanIdx ][ i ][ 0 ] |  |
| yS = ScanOrder[ tuWidth ][ tuHeight ][ scanIdx ][ i ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( ( i < lastSubBlock ) && ( i > 0 ) ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| for( n = ( i = = lastSubBlock ) ? lastScanPos − 1 : maxPosInSb; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ subBlockW ][ subBlockH ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ subBlockW ][ subBlockH ] [ scanIdx ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| } |  |
| for( n = maxPosInSb; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ subBlockW ][ subBlockH ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ subBlockW ][ subBlockH ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| if( coeff_abs_level_greater1_flag[ n ] ) { |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| if( coeff_abs_level_greater2_flag[ n ] ) { |  |
| coeff_abs_level_greater3_flag[ n ] | ae(v) |
| if( coeff_abs_level_greater3_flag[ n ] ) |  |
| coeff_abs_level_greater4_flag[ n ] | ae(v) |
| } |  |
| } |  |
| if (coeff_abs_level_greater4_flag[ n ] ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| } |  |
| } |  |
| for( n = maxPosInSb; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ subBlockW ][ subBlockH ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ subBlockW ][ subBlockH ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * ( coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] coeff_abs_level_greater3_flag[ n ] + coeff_abs_level_greater4_flag[ n ] + coeff_abs_level_remaining[ n ] ) |  |
| } |  |
| } |  |
| } |  |

### Block partitioning

Similar to HEVC and JEM, the video pictures are initially partitioned into fixed-size blocks, which are referred to as coding tree units (CTUs).  The CTUs represent square block which width and height (in luma samples) being equal to an integer power of two.  For the submitted bitstreams, the CTU size was set equal to 128×128 luma samples. The CTUs are partitioned into one or more coding units (CU). As in HEVC and JEM, the partitioning of CTUs into CUs is applied in a recursive fashion. A CTUs can be split into multiple blocks, and each of the resulting blocks can be further partitioned, and so on.

In contrast to HEVC and JEM, a generalized binary splitting (BTS: binary tree with shifting) concept is used. Each block is either not split or it is split into two rectangular blocks (either in horizontal or vertical direction). Both the width and height (in luma samples) of the resulting CUs must represent integer multiples of 4. Let dim represent the width (for vertical splits) or the height (for horizontal splits), in luma samples, of the block to be split.  For both split directions, the following splits are supported (see illustrations in Figure 1 and Figure 2):

- 1/2 split (first row in Figure 1): This split is supported if dim≥k⋅8, k∈N.  It is similar to a binary split for QTBT in JEM.

- 1/4 split and 3/4 split (second row in Figure 1):  These splits are supported if dim represents an integer power of two (dim=2n, n∈N) and dim≥16.

- 3/8 split and 5/8 split (third row in Figure 1):  These splits are supported if dim represents an integer power of two (dim=2n, n∈N) and dim≥32.

- 1/3 split and 2/3 split (first row in Figure 2):  These splits are supported if dim=3⋅2n, n∈N and dim≥12.

- 1/5 split, 2/5 split, 3/5 split and 2/3 split (second and third row in Figure 2):  These splits are supported if dim=5⋅2n, n∈N and dim≥20.

Figure 1:  Supported splits for block sizes that represent integer powers of 2: (first row) binary 1/2 splits; (second row) 1/4 and 3/4 splits with different orientations; (third row) 3/8 and 5/8 splits with different orientations.

At this, the following convention is used. An n/m horizontal split specifies a split for which the ratio of the height of the first resulting block (top block) and the height of the block to be split is equal to n/m. Similarly, an n/m vertical split specifies a split for which the ratio of the width of the first resulting block (left block) and the width of the block to be split is equal to n/m.  Note the following: If the size of the side to be split is not equal to 2n, n∈N, then it is either equal to 3·2n, n∈N or 5·2n, n∈N.

Figure 2:  Supported splits for block sizes that do not represent integer powers of 2: (first row) 1/3 and 2/3 splits with different orientations; (second and third row) 1/5, 2/5, 3/5, and 4/5 splits with different orientations.

A binary split is uniquely determined by its split direction and its split ratio. The direction of the binary splits are coded depending on the previous split, i.e. instead of being signaled as horizontal or vertical split, they are signaled as perpendicular split or parallel split which, however, can always be translated to a horizontal or vertical split. At the root level no previous split was signaled and the first perpendicular split is a horizontal split and the first parallel split is a vertical split. The binary flag perpend_split_flag distinguishes the two possible directions (perpendicular and parallel). The split ratio then describes the location where to split (see Figure 1 and Figure 2). It is coded using a binary decision tree. The full signaling tree for the binary splits with shifting, without any restrictions, is shown in Figure 3. Both, the split direction as well as the split ratio are coded using context-based adaptive arithmetic coding (CABAC). The counting of the binary tree depth starts with the first split that is not a perpendicular 1/2 split.

Figure 3:  Full code tree; the perpendicular part is not shown (it is identical to the sub tree for the parallel case). Branches or leafs of the code tree can be omitted, if the options are not available in a specific context. If a node only contains one child, the decision is not transmitted.

The binary tree with shifting can be configured using high-level parameters such as

- CTUSize: size of the CTU (set equal to 128)

- MaxBTDepth: maximal depth for non-default splits for intra (set equal to 3)

- MaxBTDepthI: maximal depth for non-default splits for inter (set equal to 3)

- MaxBTDepthIChroma: maximal depth for non-default splits for chroma (set equal to 3)

- MaxBTSize: maximal size for non-default splits for intra (set equal to 32)

- MaxBTSizeI: maximal size for non-default splits for inter (set equal to 128)

- MaxBTSizeChroma: maximal size for non-default splits for chroma (set equal to 64)

- MinBTSize: minimal block size for intra (set equal to 4)

- MinBTSizeInter: minimal block size for inter (set equal to 4)

- MinxBTSizeChroma: minimal block size for chroma (set equal to 4)

- AllowFourths: enables usage 1/4 and 3/4 splits (set equal to 1)

- AllowEights: enables usage 3/8 and 5/8 splits (set equal to 1)

- NonLog2Halving: enables that non-power of 2 sized blocks can be split using the 1/2 split (set equal to 1)

The values given in parentheses represent the settings that were used for generating the submitted bitstreams. At this, non-default splits are splits that do not represent a 1/2 perpendicular split. The high-level parameters are transmitted in the sequence parameter set (SPS). Moreover, they can be used to restrict the binary tree with shifting and thereby reduce the full code tree to a subtree. In order to prevent transmitting unnecessary decisions, the binary decision tree is pruned according to the availability of the nodes. Furthermore, parallel redundancies are also prohibited (see Table 1) and therefore lead to smaller subtrees by marking nodes as unavailable. Two separate situations with a limited subtree are shown in Figure 4.

Figure 4:  Signaling tree, with only available options highlighted. The signaling a) would be used at the CTU level for a B or P slice.  Signaling b) would be used for a partition within a binary tree with shifting for which the side perpendicular to the last split is of size 3·2n, n∈N, thus allowing only x/3 and 1/2 splits parallel to the last splits.

The resulting coding units (CUs) are used for both prediction and transform coding. An exception are coding units, which are coded in line-based intra coding mode (see sec. 2.1.9.1). In this mode, the luma signal of the CU is further partitioned into 1d rows or columns, and these 1d sample arrays are predicted and transform coded. Nonetheless, in all coding mode, the same sample blocks are used for prediction and transform coding.

Figure 5:  Notion of different types of layouts presented in Table 1.  All layouts represent the splitting pattern after a binary split is used. (a) First block is to be split and second block not. (b) First block is not split and second part is split. (c) First and second block are split. (d) Split first block and then the second subblock of the resulting block. (e) Split second block and then the first subblock of the resulting block. (f) Split second block and then the second subblock of the resulting block.

All used redundancy prohibitions are summarized in Table 1. The associated split layouts are illustrated in Figure 5.

Table 1: Prohibited split redundancies

| Layout | A-Dir | A-Ratio | B-Dir | B-Ratio | C-Dir | C-Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| a) | * | 2/3 | parallel | 1/2 | - | - |
| a) | * | 3/4 | parallel | 1/3 | - | - |
| a) | * | 3/4 | parallel | 2/3 | - | - |
| a) | * | 3/5 | parallel | 1/3 | - | - |
| a) | * | 3/5 | parallel | 2/3 | - | - |
| a) | * | 2/5 | parallel | 1/2 | - | - |
| a) | * | 4/5 | parallel | 1/2 | - | - |
| a) | * | 5/8 | parallel | 4/5 | - | - |
| a) | * | 5/8 | parallel | 3/5 | - | - |
| a) | * | 4/5 | parallel | 1/4 | - | - |
| a) | * | 4/5 | parallel | 3/4 | - | - |
| b) | * | 1/4 | parallel | 1/3 | - | - |
| b) | * | 3/8 | parallel | 1/5 | - | - |
| c) | * | 1/4 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 3/4 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 1/3 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 2/3 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 1/5 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 2/5 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 3/5 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 4/5 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | ¾ | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 1/3 | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 2/3 | perpendicular | 1/4 | Perpendicular | 1/4 |
| c) | * | 1/5 | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 2/5 | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 3/5 | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 4/5 | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 1/3 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 2/3 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 1/5 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 2/5 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 3/5 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 4/5 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 2/3 | perpendicular | 1/3 | perpendicular | 1/3 |
| c) | * | 1/5 | perpendicular | 1/3 | perpendicular | 1/3 |
| c) | * | 2/5 | perpendicular | 1/3 | perpendicular | 1/3 |
| c) | * | 3/5 | perpendicular | 1/3 | perpendicular | 1/3 |
| c) | * | 4/5 | perpendicular | 1/3 | perpendicular | 1/3 |
| c) | * | 1/5 | perpendicular | 2/3 | perpendicular | 2/3 |
| c) | * | 2/5 | perpendicular | 2/3 | perpendicular | 2/3 |
| c) | * | 3/5 | perpendicular | 2/3 | perpendicular | 2/3 |
| c) | * | 4/5 | perpendicular | 2/3 | perpendicular | 2/3 |
| c) | * | 2/5 | perpendicular | 1/5 | perpendicular | 1/5 |
| c) | * | 3/5 | perpendicular | 1/5 | perpendicular | 1/5 |
| c) | * | 4/5 | perpendicular | 1/5 | perpendicular | 1/5 |
| c) | * | 3/5 | perpendicular | 2/5 | perpendicular | 2/5 |
| c) | * | 4/5 | perpendicular | 2/5 | perpendicular | 2/5 |
| c) | * | 4/5 | perpendicular | 3/5 | perpendicular | 3/5 |
| d) | * | 3/4 | parallel | 1/3 | parallel | 1/3 |
| e) | * | 1/4 | parallel | 2/3 | parallel | 3/4 |
| f) | * | 1/4 | parallel | 1/2 | parallel | 1/3 |

The approach described above was used in the submitted bitstreams. After bitstream submission, the approach was extended to additionally support quadtree splits. For that purpose, the syntax specified in Figure 3 is modified in a way that a first bin indicates whether the square block is split into four equally-sized square blocks (quadtree split) or not. If this bin is equal to 0 (quadtree splitting is not used), the partitioning is specified as specified by the code tree in Figure 3. The generalized splitting with quadtree represents a generalized version of QTBT and is referred to as QT+BTS. A detailed description of this extension is provided in JVET-J0035 [4]. The addition of the quadtree option enables an increased coding efficiency and at the same time it reduces the encoder complexity.

It should be pronounced that the generalized binary splitting approach can be configured via high-level parameters (SPS syntax elements). It is for example possible to set the high-level parameters in a way that all CUs have a width and height that represent integer powers of two.

### Entropy decoding

All low-level syntax elements (slice data and below) are coded using context-based adaptive arithmetic coding (CABAC). There are two modifications relative to CABAC in HEVC:

Coding engine: Similar to JEM, the entropy coding stage employs a counter-based probability estimator, which uses two custom window sizes for each context model, initialized at the beginning of each slice. For coding a binary symbol, the counters are converted to the MPS/LPS representation and used to address a 32x8x8-bit range table for the interval subdivision of the M Coder (see sec. 2.1.3.1).

Context initialization: Similar to JEM, the probability models of inter-coded slices can be initialized using stored probability models for previous slices. But in contrast to JEM, separate sets of probability models are stored for the individual CTU lines of a picture (see sec. 2.1.3.2).

#### Arithmetic coding engine

Conceptually, the arithmetic coding engine is similar to the multi-parameter CABAC engine in JEM. But, it contains the following modifications:

- Context model initialization: Each context model is associated with two separate window sizes for the two counters.

- Counter-based probability estimation with custom window sizes:  Usage of custom (but non-adaptive) window sizes in the probability updates.

- Adjustments to the arithmetic coding engine: The interval subdivision is realized using a smaller look-up table (32×8 range table with 8 bit values).

Context model initialization

Similar to JEM, the counters of the probability estimator are set to an initial value at the beginning of each slice using 8-bit initialization values. The 8-bit initialization values have been updated to adjust the probability estimation stage to the new coding tools. Next to setting the initial counts, the context model initialization process also sets custom window sizes for each of the two counter variables. Similar to the initial probability, an additional 8-bit variable "wspair” is provided which defines the window sizes of the two counters. The derivation process for the correct wspair variable of one context model is dependent on the slice type and the cabac init flag, as outlined in Table 2.

Table 2: Assignment of  wspair values

| sliceType | cabac init flag | wspair |
| --- | --- | --- |
| B | 0 | wspair0 |
|  | 1 | wspair1 |
| P | 0 | wspair1 |
|  | 1 | wspair0 |
| Intra | 0 | wspair2 |

| na |  |  |  | nb |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |

Figure 6:  Bit mask of 8-bit variable wspair.

One wspairvariable contains the window size values for the two counters of one context model.  The structure of the 8-bit value is illustrated in Figure 6. The 4-bit variables na and nb represent the binary logarithm of the associated window sizes for the counters.  As shown in the next paragraph, the variables na and nb are directly used for updating the counters after a bin is encoded or decoded.

Counter-based probability estimation with custom window sizes

Identical to JEM, the probability estimator updates the two counter variables after decoding a bin. With q representing one of the counters, the update is calculated as follows:

qt+1=qt+215-qt≫nq,  if xt=1qt-qt≫nq                ,  if xt=0,

where xt represents the value of the last encoded or decoded bin. In contrast to JEM, both counter variables are updated using custom window sizes with nq in the range of 1 to 11, inclusive. The output of the probability estimator is derived by weighting the two counters according to ct=at+bt≫1, which is identical to JEM.

Adjustments to the arithmetic coding engine

Unlike JEM, the arithmetic coding engine contains a 32×8 range table with pre-calculated 8-bit values for the interval subdivision. The LPS probability interval (0, 0.5) is uniformly quantized with a step size of Δp=0.015625 and a minimum probability of pmin=0.0078125. The table is addressed by the index pStateIdx which is calculated based on the output of the probability estimator ct:

pStateIdx=ct-214 xor valMPS-1≫9,

with valMPS=ct≫14. This operation converts the probability from being represented in the range of (0, 1), as used by the two counter variables from the probability estimator, to a representation in the LPS probability range (0,0.5) along with the value of the MPS.  Therefore, the table containing the pre-calculated range values uses 8 bits per value while keeping the same resolution of the coding engine.

#### Context initialization from previous pictures

Similar to JEM, the initial probability states of context models for inter-coded slices can be initialized by copying states from previously coded pictures. In contrast to JEM, the context states are stored for each CTU line. We found that this modification particularly improves coding efficiency in encoder-only parallelized wavefront processing.

More specifically, after the last CTU of a CTU line is coded, the probability states of all context models are stored for potential use as the initial states of the corresponding context models on later pictures. At the beginning of a CTU-line of an inter-coded slice, the set of initial states is copied from the stored states of the same CTU line of a previously coded picture that has the same slice type and the same slice-level QP as the current slice. At random access points, all stored states are marked as not available.

### Conversion process from entropy decoded data to symbols

#### Intra prediction parameters

Intra prediction parameter for conventional spatial intra prediction are coded as in JEM. The conversion from entropy decoded bins to syntax elements for the additional syntax elements is described in the following.

Syntax elements for line-based intra prediction

The coding unit residual syntax includes the syntax element residual_1d_partitions_mode, which specifies whether the line-based intra coding mode (see sec. 2.1.9.1) is used for the luma signal of the coding unit. The syntax element is coded using two binary decisions. The first bin specifies whether the line-based intra prediction mode is applied for the current coding unit:

- If this first bin is equal to 0, residual_1d_partitions_mode is set equal to 0 (i.e., the line-based intra coding mode is not used).

- Otherwise, a second bin is coded and the following applies:

  - If the second bin is equal to 0, residual_1d_partitions_mode is set equal to 1 (i.e., the line-based intra coding is used with horizontal 1d stripes).

  - Otherwise, residual_1d_partitions_mode is set equal to 2 (i.e., the line-based intra coding is used with vertical 1d stripes).

The first bin is coded using a single context model. The context model that is used for the second bin depends on the luma intra prediction mode. A first context model is used for non-angular modes, a second context model is used for horizontal modes, and a third context model is used for vertical modes.

Syntax elements for Intra prediction with neural networks

The usage of the luma intra predicton mode with neural networks (see sec. 2.1.9.2) is indicated by the binary syntax element intra_nn_flag.  There are three context models for coding the intra_nn_flag.  With ctxId representing an index into the list of these three context models, the context model used is chosen as follows:

ctxId =  intraNNFlagAbove + intraNNFlagLeft,

where intraNNFlagAbove and intraNNFlagLeft represent the values of intra_nn_flag for the block above the current CU and the block left to current CU, respectively.

When intra_nn_flag is equal to 1, parameters for the luma intra prediction are transmitted using the binary syntax element intra_nn_subsampling_flag and the syntax element intra_nn_mpm_idx.

The intra_nn_subsampling_flag is transmitted in the bypass mode of the arithmetic coding engine. When the flag intra_nn_subsampling_flag is not present, it is inferred as follows:

- If the current CU represents a 64×64 CU, intra_nn_subsampling_flag is inferred to be equal to 1;

- Otherwise, intra_nn_subsampling_flag is inferred to be equal to 0.

The binarization used for the mode index intra_nn_mpm_idx is illustrated in Table 3.  The first bin is coded using a single context model. If this bin is equal to 1, one or two further bins are coded in the bypass mode of the arithmetic coding engine (see first three rows in Table 3); otherwise, the following applies:

- If both the width and the height (in luma samples) of the CU are less than 32, five additional bins are coded in the bypass mode of the arithmetic coding engine.

- Otherwise, three additional bins are coded in the bypass mode of the arithmetic coding engine

For CUs with both width and height less than 32, the value of intra_nn_mpm_idx is in the range of 0 to 34, inclusive. For all other CUs, the value of intra_nn_mpm_idx is in the range of 0 to 10, inclusive.

Table 3: Binarization of the syntax element intra_nn_mpm_idx.

|  |  | flag | unary_max |  | fixed_length |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | intra_nn_mpm_idx | CTX | EP |  | LSB |  | EP | MSB |  |
| MPM | 0 | 1 | 0 |  |  |  |  |  |  |
|  | 1 | 1 | 1 | 0 |  |  |  |  |  |
|  | 2 | 1 | 1 | 1 |  |  |  |  |  |
|  | 3 | 0 |  |  | 0 | 0 | 0 | 0 | 0 |
|  | 4 | 0 |  |  | 1 | 0 | 0 | 0 | 0 |
|  | 5 | 0 |  |  | 0 | 1 | 0 | 0 | 0 |
|  | 6 | 0 |  |  | 1 | 1 | 0 | 0 | 0 |
|  | 7 | 0 |  |  | 0 | 0 | 1 | 0 | 0 |
|  | 8 | 0 |  |  | 1 | 0 | 1 | 0 | 0 |
|  | 9 | 0 |  |  | 0 | 1 | 1 | 0 | 0 |
|  | 10 | 0 |  |  | 1 | 1 | 1 | 0 | 0 |
| if width < 32 and height < 32 | 11 | 0 |  |  | 0 | 0 | 0 | 1 | 0 |
|  | 12 | 0 |  |  | 1 | 0 | 0 | 1 | 0 |
|  | 13 | 0 |  |  | 0 | 1 | 0 | 1 | 0 |
|  | 14 | 0 |  |  | 1 | 1 | 0 | 1 | 0 |
|  | 15 | 0 |  |  | 0 | 0 | 1 | 1 | 0 |
|  | 16 | 0 |  |  | 1 | 0 | 1 | 1 | 0 |
|  | 17 | 0 |  |  | 0 | 1 | 1 | 1 | 0 |
|  | 18 | 0 |  |  | 1 | 1 | 1 | 1 | 0 |
|  | 19 | 0 |  |  | 0 | 0 | 0 | 0 | 1 |
|  | 20 | 0 |  |  | 1 | 0 | 0 | 0 | 1 |
|  | 21 | 0 |  |  | 0 | 1 | 0 | 0 | 1 |
|  | 22 | 0 |  |  | 1 | 1 | 0 | 0 | 1 |
|  | 23 | 0 |  |  | 0 | 0 | 1 | 0 | 1 |
|  | 24 | 0 |  |  | 1 | 0 | 1 | 0 | 1 |
|  | 25 | 0 |  |  | 0 | 1 | 1 | 0 | 1 |
|  | 26 | 0 |  |  | 1 | 1 | 1 | 0 | 1 |
|  | 27 | 0 |  |  | 0 | 0 | 0 | 1 | 1 |
|  | 28 | 0 |  |  | 1 | 0 | 0 | 1 | 1 |
|  | 29 | 0 |  |  | 0 | 1 | 0 | 1 | 1 |
|  | 30 | 0 |  |  | 1 | 1 | 0 | 1 | 1 |
|  | 31 | 0 |  |  | 0 | 0 | 1 | 1 | 1 |
|  | 32 | 0 |  |  | 1 | 0 | 1 | 1 | 1 |
|  | 33 | 0 |  |  | 0 | 1 | 1 | 1 | 1 |
|  | 34 | 0 |  |  | 1 | 1 | 1 | 1 | 1 |

Syntax elements for Intra region-based template matching

The usage of intra region-based template matching mode is indicated by the binary syntax element intra_rtm_flag. When intra_rtm_flag is equal to 1, another syntax element intra_rtm_region_idx is transmitted, which indicates the used search region (see sec. 2.1.9.3). The binarization of this syntax element is illustrated in Figure 7. The first bin indicates whether or not the selected region is Region 1.  If this bin is equal to 1, no further bins are transmitted. Otherwise, two more bins are coded for identifying the chosen region (any region from region 2 to 5). All bins are coded using a separate context model.

Figure 7:  Coding of intra_rtm_flag and binarizaion of intra_rtm_region_idx

Syntax elements for Multiple-reference line intra prediction

The usage of multiple-reference line intra prediction (MRL) is indicated by the syntax element mrl_idx. The value of mrl_idx indicates which reference line is used in intra prediction (see sec. 2.1.9.5). Allowed values are 0, 1 and 3 which are coded using truncated unary binarization with 0 for mrl_idx = 0, 10 for mrl_idx = 1, 11 for mrl_idx = 3 and a separate context model for each bin.

#### Motion parameters

Motion parameters are coded in the same way as in JEM. Modifications for multi-hypothesis prediction and the restricted merge mode are described below.

Multi-hypothesis prediction

In contrast to HEVC and JEM, motion-compensated prediction is not limited to uni- and bi-prediction. Instead, additional motion hypotheses can be used (see sec. 2.1.8.1). The maximum number of additional motion hypothesis is indicated by an SPS syntax element MaxNumAdditionalHypotheses.

First conventional motion data for uni- or bi-prediction are transmitted in the same way as in HEVC/JEM.

The prediction unit syntax includes the binary syntax element additional_hypothesis_flag, which is coded using a dedicated context model. When MaxNumAdditionalHypotheses is equal to 0, the flag is not transmitted, but inferred to be equal to 0.

When additional_hypothesis_flag is equal to 1, the following applies:

- An additional set of motion data is transmitted, consisting of a reference index, a motion vector difference, and a motion vector predictor index. The motion data are coded in the same way as in HEVC/JEM.

- An additional syntax element add_hyp_weight_idx is transmitted using truncated unary binarization, where two context models are used (one for the first bin and another for the remaining bins).

- When the maximum supported number of additional motion hypotheses is not reached, another additional_hypothesis_flag is coded as specified above.

For the submitted bitstreams, MaxNumAdditionalHypotheses was set equal to 1 (i.e., one additional motion hypothesis was allowed).

Restricted merge

The prediction unit syntax includes a syntax element merge_ref_pic_list_idc, which indicates which motion hypotheses are used in merge mode (see sec. 2.1.8.2). The syntax element can take three different values. It is coded using a truncated unary binarization. The first bin indicates whether restricted merge or the conventional merge mode is used (as in HEVC/JEM). If the first bin is equal to 1, the second bin indicates which of the two motion hypotheses (list 0 or list 1) is used. Both bins are coded with a dedicated context model.

#### Quantization parameters

In HEVC and JEM, the QP value for a CU is predicted from the top and left neighboring CUs’ QP values, and only the difference between the CU’s actual QP value and the predicted QP value is transmitted. If a neighboring QP value lies outside of the CTU to which the current CU belongs, then the previously coded QP value is assumed for that neighbor in the prediction. This leads to the situation that, for each new CTU line in a slice or picture, no neighboring CUs are accessible in the first CU of the leftmost CTU of that line, and the QP predictor must, therefore, be set equal to the last coded QP value of the rightmost CTU in the previous line. This approach is both inefficient (since QP prediction from the right to left slice/picture boundary is, usually, inaccurate) and cumbersome in a wavefront-parallelized encoder implementation (since the encoding of one CTU line would need to be completed before the encoding of the next CTU line can begin).

In order to avoid these drawbacks, an alternative QP predictor (option “AltDQPCoding =1”) is used. It is based on Martucci’s median adaptive prediction and adopts, as a fallback predictor, the default slice QP value QPslice signaled in the slice header instead of any previously coded QP value. The spatial layout of the neighboring CUs utilized by this improved predictor is as follows:

| c(orner) | a(bove) |
| --- | --- |
| b(efore) | p(redicted) |

If a neighboring CU is not available because the current CU is located at a slice boundary, that CU’s QP value is assumed to equal QPslice.  Given these QP values, the QP predictor p for a current CU is derived according to

p=Clip3 mina; b; maxa; b; a+b-c .

As in HEVC, the delta QP value given by DQPk=QPk-p is transmitted. The coding of the delta QP is not modified in comparison to HEVC or JEM.

#### Transform coefficients

The entropy coding for the transform coefficients uses a similar concept as HEVC and JEM.  For each transform block, the transform coefficient levels are represented by the following syntax elements:

- coded_block_flag indicating whether the transform block includes any transform coefficients unequal to 0.

- x and y coordinate of the last nonzero transform coefficient level in scanning order.

- coded_sub_block_flag‘s indicating whether a subblock includes any transform coefficients unequal to 0.

- binary representation of absolute values of the transform coefficients levels for subblocks with coded_sub_block_flag equal to 1.

- Signs of the nonzero transform coefficients.

Similar to JEM, transform blocks for which both the horizontal and vertical size is larger than 4 are partitioned into 4×4 subblocks, and transform blocks for which the horizontal or the vertical size is equal to 2 are partitioned into 2×2 subblocks.  For the additionally supported 1d transform blocks (see sec. 2.1.9.1), subblocks of size 4×1 or 1×4 are used.

Even though similar syntax elements as in HEVC are coded for specifying the transform coefficient levels, there are differences in the binarization, the order of bins, and the context modeling. The main differences are:

- Context modelling for coded block flags:  The context for the coded block flag of the second chroma is determined by the value of the coded block flag for the first chroma channel. For coding units that are coded in 1d partitioning mode (see sec. 2.1.9), the context is determined by the value of the coded block flags of the previous 1d partition in coding order.

- Coding order of bins in a subblock:  All bins representing the absolute value of a transform coefficient level are coded before moving to the next scanning position.

- Binarization of absolute values:  The first five bins (which represent the absolute values using an unary binarization) are coded in regular mode using adaptive context models; the remainder is coded using Rice codes in bypass mode of the arithmetic coding engine.

- Context modelling for bins of absolute values:  The context for the regular coded bins of the absolute values is determined by the values of already coded absolute values in a local neighborhood. Furthermore, for the first two bins (sig_coeff_flag and coeff_abs_level_greater1_flag), two different context model sets are used for the two scalar quantizers (see sec. 2.1.6).

- Sign bit hiding:  For the generated bitstreams, sign bit hiding is not used. The syntax and software support sign bit hiding when dependent scalar quantization (see sec. 2.1.6) is disabled.

For the following description, it is assumed that the context models are arranged in the memory so that each context can be addressed by a unique global index. Let c be the address of the context model that is used for arithmetic coding of a particular bin. Furthermore, let csyntax be the offset for the particular bin, cset a context model set indicator that represents an additional context offset as multiple of the size nset, and let cidx be the offset within a selected context model set. Then, the context model address can be calculated according to

c=csyntax+nset×cset+cidx.

Note that it is assumed that the offset for the color components is included in the csyntax offset. The commonalities and differences to the transform coefficient coding in HEVC are discussed in the following in more detail.

Coded block flag

For the context modeling of the coded_block_flag, two cases are distinguished:

- The current transform block represents a partition of a coding unit that is coded in line-based intra prediction mode (residual_1d_partitions_mode is larger than 0);

- The current transform block represents a regular 2d transform block (residual_1d_partitions_mode is equal to 0).

The context model selection for the coded_block_flag is illustrated in Figure 8.

In the first case (residual_1d_partitions_mode is larger than 0 and the transform block is a luma block), the luma block of the coding unit is partitioned into horizontal or vertical 1d stripes (of sizes N×1 or 1×M for N×M coding units) and each of the stripes is separately predicted and reconstructed (in a defined order). As further described in sec. 2.1.9.1, each of the 1d stripes is treated as a separate transform block and, thus, has a separate coded_block_flag. For these 1d transform blocks, a separate context model set (set = 2 in Figure 8) is used for coding the coded_block_flag. The context model set consists of two context models.  The context model index cidx is set equal to the value of coded_block_flag for the previous 1d partition (in coding order) of the same coding unit. For the first 1d partition of a coding unit, cidx is set equal to 0.

Figure 8:  Illustration of the context model selection for coded_block_flag.

When 1d intra partitions are not used (residual_1d_partitions_mode is equal to 0 or the transform block is a chroma block), the following applies: A single context model (cset=0 and cidx=0) is used for the coded_block_flag of the luma component. For the coded_block_flag of the Cb component, another single context model is used (cset=1 and cidx=0). However, for the Cr component (i.e., the second chroma component), two context models are used (cset=1 and cidx=1 or 2). The context index cidx is set equal to 1+cbf(Cb), where cbf(Cb) represents the value of coded_block_flag for the Cb component of the same coding unit.

Last significant scanning position

The x and y location of the last nonzero transform coefficient level in scanning order are coded in the same way as in HEVC and JEM.  However, in contrast to HEVC and JEM, the proposed codec additionally supports block widths and heights that do not represent integer powers of two. For these block sizes, the binarization of the x and y coordinates of the last scanning position is modified as shown in Table 4. Table 5 illustrates the resulting binarization for transform sizes of 6, 10, 12, 20, and 24.

Table 4: Modifications of the binarization for the x and y coordinates of the position of the last significant coefficient.

| residual_coding( ) | last_sig_coeff_x_prefix | TR | cMax = ( log2TrafoSize << 1 ) − 1, cRiceParam = 0 |
| --- | --- | --- | --- |
|  | last_sig_coeff_y_prefix | TR | cMax = ( log2TrafoSize << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_x_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_x_prefix >> 1 ) − 1 ) − 1 ) |
|  | last_sig_coeff_y_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_y_prefix >> 1 ) − 1 ) − 1 ) |

Table 5: Complete binarization for selected non-power of two transform sizes.

| value of x or y | TrafoSize=6 | TrafoSize=10 | TrafoSize=12 | TrafoSize=20 | TrafoSize=24 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 10 | 10 | 10 | 10 | 10 |
| 2 | 110 | 110 | 110 | 110 | 110 |
| 3 | 1110 | 1110 | 1110 | 1110 | 1110 |
| 4 | 11110 0 | 11110 0 | 11110 0 | 11110 0 | 11110 0 |
| 5 | 11110 1 | 11110 1 | 11110 1 | 11110 1 | 11110 1 |
| 6 |  | 111110 0 | 111110 0 | 111110 0 | 111110 0 |
| 7 |  | 111110 1 | 111110 1 | 111110 1 | 111110 1 |
| 8 |  | 1111110 00 | 1111110 00 | 1111110 00 | 1111110 00 |
| 9 |  | 1111110 01 | 1111110 01 | 1111110 01 | 1111110 01 |
| 10 |  |  | 1111110 10 | 1111110 10 | 1111110 10 |
| 11 |  |  | 1111110 11 | 1111110 11 | 1111110 11 |
| 12 |  |  |  | 11111110 00 | 11111110 00 |
| 13 |  |  |  | 11111110 01 | 11111110 01 |
| 14 |  |  |  | 11111110 10 | 11111110 10 |
| 15 |  |  |  | 11111110 11 | 11111110 11 |
| 16 |  |  |  | 111111110 000 | 111111110 000 |
| 17 |  |  |  | 111111110 001 | 111111110 001 |
| 18 |  |  |  | 111111110 010 | 111111110 010 |
| 19 |  |  |  | 111111110 011 | 111111110 011 |
| 20 |  |  |  |  | 111111110 100 |
| 21 |  |  |  |  | 111111110 101 |
| 22 |  |  |  |  | 111111110 110 |
| 23 |  |  |  |  | 111111110 111 |

Coded subblock flags

The syntax elements coded_sub_block_flag are coded in the same way as in HEVC/JEM.

Scan patterns

For mapping the 2d arrays of transform coefficient levels into a coding order used, the same scan patterns as in HEVC are used. Note that the modified scan pattern for 8×8 as implemented in JEM is not used. For the additional intra coding modes, the following applies:

- Intra 1d partitions (residual_1d_partitions_mode is larger than 0 and the transform block is a luma block):  The 1d transform blocks use a simple 1d scan.  The transform coefficients are coded in the order from the highest to the lowest frequency component.

- IntraNN mode (intra_nn_flag is equal to 1):  For all transform blocks that are predicted using an IntraNN mode, the diagonal scan as specified in HEVC is used.

Similar to HEVC and JEM, the transform coefficient levels are coded on the basis of subblocks. All bins for the transform coefficient levels are coded before any data of the next subblock in coding order are transmitted.

Binarization for absolute values of transform coefficient levels

In comparison to HEVC, a modified binarization for the absolute values of the transform coefficient levels is used.  Similar to HEVC, the absolute values are binarized using a binarization that consists of a prefix and a suffix part.  While for the prefix part a truncated unary code is used, the suffix part is represented using Rice and Exp-Golomb codes.  But in contrast to HEVC, the first binarization bound, i.e., the transition between the truncated unary code and the Rice codes is fixed, and its value is set equal to five, instead of being adaptive as in HEVC. This configuration results in five bins that are coded in the regular mode, i.e., with context models.  As shown in the syntax diagram in sec. 2.1.1.10), these five bins are referred to as sig_coeff_flag, coeff_abs_level_greater1_flag, coeff_abs_level_greater2_flag, coeff_abs_level_greater3_flag, and coeff_abs_level_greater4_flag, coeff_abs_level_greater1_flag. Other parameters such as the relationship between the Rice and Exp-Golomb order and the interval range that is covered by a Rice code are unchanged in comparison to HEVC. A summary of the differences is illustrated in Figure 9.

Figure 9:  Illustration of the binarization for the absolute values of transform coefficient levels.

Coding order of bins

In contrast to HEVC and JEM, the absolute values of the transform coefficients of a subblock are coded in a single pass (see also syntax diagram in sec. 2.1.1.10).  That means all syntax elements (sig_coeff_flag, coeff_abs_level_greaterX_flag, coeff_abs_level_remaining) for a scan position are coded before any data for the next scan position are transmitted.  The signs for the nonzero transform coefficient levels are coded in a second pass over the subblock.

Context model selection for absolute values of transform coefficient levels

The context model selection for the bins sig_coeff_flag and coeff_abs_level_greaterX_flag (with X being replaced by 1, 2, 3, and 4) as well as the binarization code selection for the syntax element coeff_abs_level_remaining is based on already coded absolute values in a local neighborhood. The size and shape of the local template used is shown in Figure 10(a). It is the same template as used in the JEM software.  Figure 10(a) illustrates the template for an 8×8 transform block using diagonal scan. Each square denotes a scanning position and the square marked with an o denotes the current scanning position. The light grayed scanning positions are those that have been processed and the dark grayed scanning positions are those that have been processed and are covered by the local template T, i.e., the positions that are considered for the evaluation for the current scanning position.

(a)     (b)

table[2][2]={{0,2},{2,0},{3,1}{1,3}}

state = 0

for( subblkId ) {

for( scanId inside subblock ) {

quantId = ( state >> 1 )

code level (using binarization)

state = table[ state ][ level & 1 ]

}

}

Figure 10:  Context modelling for bins specifying the absolute values of the transform coefficient levels: (a) Local neighborhood used for deriving context models; (b) Simplified pseudo-code for deriving the scalar quantizer used (given by quantId) in dependent scalar quantization.

For the actual context selection, the sum of absolute transform coefficient levels Ts and the number of significant neighboring positions Tn inside the template are determined.  These values are used for deriving the context model index cidx and the Rice parameter k.

For the first bin of the Truncated Unary code, also referred to as the significance bin sig_coeff_flag, the context index is set equal to the absolute sum of the neighboring frequencies clipped to a maximum value of five,

cidx=minTTs,5.

Consequently, the context model set size nset is equal to six for the significance bins.  Six context model sets are used for the luma component and four context model sets are used for the chroma components.  The chosen context model set depends on the diagonal position of the current transform coefficient level and the scalar quantizer (see sec. 2.1.6) used.  With d=x+y representing the diagonal position of the current transform coefficient, the context model sets cset are derived according to

cset,luma=3⋅quantId+ 0:d<2 1:2≤d<5 2:5≤d

cset,chroma=2⋅quantId+ 0:d<2 1:2≤d

Here, the variable quantId (with possible values of 0 and 1) specifies the scalar quantizer used.  As illustrated in the pseudo code in Figure 10(b) and described in more detail in sec. 2.1.6, the variable quantId is derived using a state transition process. At the beginning of a transform block, the state is initially set equal to 0.  After an absolute value is completely coded, the state is updated based on the current state and the parity of the absolute transform coefficient level.  The value of quantId is set equal to the second bit of the binary representation of the state.

For the remaining bins of the truncated unary code part, i.e., the bins coeff_abs_level_greaterX_flag (with X being replaced by 1, 2, 3, and 4), the following applies. A separate context model is used for the last significant scanning position. For the other scanning positions, the context index is derived according to

cidx=minTTs-Tn,4

Similar to sig_coeff_flag, the chosen context model set depends on the diagonal position d=x+y of the current transform coefficient level.  For the bin coeff_abs_level_greater1_flag it additionally depends on the scalar quantizer used (see above).  With exception of the last scan position (for which is single context model is used), the context model sets are derived according to

cset,luma=4⋅quantId+ 0:d<1 1:1≤d<4 2:4≤d<11 3:11≤d

cset,chroma=2⋅quantId+ 0:d<11 1:11≤d

For the bin coeff_abs_level_greater1_flag, quantId is derived as specified above. For all other bins coeff_abs_level_greaterX_flag, quantId is set equal to 0.

The cidx value for the remaining bins of the truncated unary code is also used to select the Rice parameter k for the syntax element coeff_abs_level_remaining.  The Rice parameter is derived according to

k= 0:cidx<4 1:4≤cidx<10 2:10≤cidx<22 3:22≤cidx

#### In-loop filter parameters

The coding of in-loop filter parameters is basically identical to JEM. The only exception is that the modified adaptive loop filter (see 2.1.10) uses one additional parameter. The parameter classification_idx, which specifies the classification method applied for the luma signal, is transmitted at the slice level (as the other ALF related parameters in JEM). The syntax element classification_idx can take three values. It is transmitted using truncated unary binarization.

#### Diffusion filter parameters

The coding unit syntax includes the syntax element cu_diffusion_filter_idx, which indicates both the usage of the diffusion filter (see sec. 2.1.11.1) and the parameters for the diffusion filter. The value of cu_diffusion_filter_idx is in the range from 0 to 4, inclusive. A value of 0 indicates that the diffusion filter is not applied. The cu_diffusion_filter_idx is transmitted after the CU prediction data (intra prediction modes or motion data).  If the skip mode is signaled, the diffusion filter is not applied and thus, the cu_diffusion_filter_idx is inferred to be equal to 0. Additionally, the diffusion filter is not applied in case the current CU lies on the left or top picture border. In case the intra prediction mode is DC or Planar, the diffusion is not applied either. In case of merge, the diffusion filter can be merged according to the chosen candidate or tested on top of the merged prediction block. In the latter case, the diffusion filter information has to be sent additionally. There are several options to constrain the usage of diffusion filters in order to reduce encoder run time and signaling which are indicated by SPS syntax elements.

Table 6: Binarization of the syntax element cu_diffusion_filter_idx.

| cu_diffusion_filter_idx | binarization |
| --- | --- |
| 0 | 0 |
| 1 | 100 |
| 2 | 101 |
| 3 | 110 |
| 4 | 111 |

The binarization of cu_diffusion_filter_idx is shown in Table 6. The first bin indicates whether the diffusion filter is applied. The second bin indicates whether the linear or the non-linear variant of the diffusion filter is used, and the third bin specifies the strength of the filter. The first bin is coded using a separate context model for intra, inter or inter-merge. The same distinction is done for the contexts used for the second bins. The last bin has an additional set of contexts (intra, inter and inter-merge) for the linear and nonlinear filter case (as indicated by the second bin).

#### DCT thresholding parameters

The coding unit residual syntax includes the binary syntax element thresholding_flag, which specifies whether the DCT thresholding (see sec. 2.1.11.2) is used for the luma signal of the coding unit. The binary syntax element thresholding_flag is coded using a single context model.

When thresholding_flag is equal to 1, the syntax additional includes the syntax elements thresholding_size and thresholding_threshold.

The syntax element thresholding_size is transmitted using a truncated unary code. The maximum number of bins is specified by an SPS syntax element. For the first four bins, separate context models are used. All remaining bins (if any) are coded using a common context model.

The syntax element thresholding_threshold is also transmitted using a truncated unary code. The maximum number of bins is specified by an SPS syntax element. For the first ten bins, separate context models are used. All remaining bins (if any) are coded using a common context model.

The presence of thresholding syntax elements for a coding unit depends on the block shape of the coding unit and the value of the luma coded block flag. The thresholding_flag is inferred to be equal to 0 if the coding unit has no residual (i.e., the coded block flag is equal to 0) or both the width and the height of the coding unit are less than 16. Additionally, the actual number of available extension sizes in a specific block decreases if its distance to the left or top border of the picture is not sufficient for filling the reconstructed boundary. In that case, the maximum value of thresholding_size equals the index of the largest available extension size. If there is no eligible extension size (if the coding unit is located at top or left border of the picture) the thresholding_flag is not present, but inferred to be equal to 0. Moreover, the SPS includes high-level syntax elements by which DCT thresholding can be completely disabled for all intra and/or inter CUs of a coded video sequence.

### Decoder-side motion refinement

Decoder-side motion refinement is the same as in JEM.  The following two JEM tools that use decoder-side motion estimation are included:

- Pattern matched motion vector derivation (FRUC)

- Decoder-side motion vector refinement (DMVR)

In addition, our video codec includes a tool that uses decoder-side template matching for intra-picture prediction. This tool is described as part of the intra prediction in sec. 2.1.9.3.

### Inverse quantization

The inverse quantization uses a concept we call “dependent scalar quantization”.  The differences to uniform reconstruction quantization (as used in AVC, HEVC, JEM) are:

- Sign hiding (as known from HEVC or JEM) is not used.

- The transform coefficients are reconstructed in scanning order, i.e., in the same order in which they are entropy decoded.

- There are two scalar quantizers (with different mappings of transform coefficient levels to reconstructed transform coefficients).  The scalar quantizer used for a current transform coefficient is determined by the parities (least significant bits) of the transform coefficient levels that precede the current transform coefficient in coding/reconstruction order.

The main effect of the approach used is that, in comparison to conventional independent scalar quantization (as used in HEVC), the admissible reconstruction vectors (given by all reconstructed transform coefficients of a transform block) are packed denser in the N-dimensional vector space (N represents the number of transform coefficients in a transform block).  That means, for a given average number of admissible reconstruction vectors per N-dimensional unit volume, the average distance (or MSE distortion) between an input vector and the closest reconstruction vector is reduced (for typical distributions of input vectors).  Eventually, this effect results in an improved rate-distortion efficiency.

The approach of dependent scalar quantization is realized by: (a) defining two scalar quantizers with different reconstruction levels and (b) defining a process for switching between the two scalar quantizers.

Figure 11:  Illustration of the two scalar quantizers used: The first quantizer Q0 maps the transform coefficient levels (black numbers below the points) to even integer multiples of the quantization step size Δ.  The second quantizer Q1 maps the transform coefficient levels to odd integer multiples of the quantization step size Δ or to zero.

For each transform coefficient, there are two available scalar quantizers, as illustrated in Figure 11.  Similar to conventional uniform reconstruction quantizers (as used in HEVC), for both quantizers (denoted by Q0 and Q1 in Figure 11), the location of the available reconstruction levels is uniquely specified by a quantization step size Δ.  If we neglect the fact that the actual reconstruction of transform coefficients uses integer arithmetic, the two scalar quantizers Q0 and Q1 are characterized as follows:

Q0:	The reconstruction levels of the first quantizer Q0 are given by the even integer multiples of the quantization step size Δ.  When this quantizer is used, a reconstructed transform coefficient t' is calculated according to

t'=2⋅k⋅Δ,

where k denotes the associated transform coefficient level (transmitted quantization index).

Q1:	The reconstruction levels of the second quantizer Q1 are given by the odd integer multiples of the quantization step size Δ and, in addition, the reconstruction level equal to zero.  The mapping of transform coefficient levels k to reconstructed transform coefficients t' is specified by

t'=2⋅k-sgnk ⋅Δ,

where sgn⋅ denotes the signum function

sgnx= 1:x>00:x=0-1:x<0 .

The scalar quantizer used (Q0 or Q1) is not explicitly signaled in the bitstream.  Instead, the quantizer used for a current transform coefficient is determined by the parities of the transform coefficient levels that precede the current transform coefficient in coding/reconstruction order.

Figure 12:  Illustration of the state transition for determining the scalar quantizer used for the transform coefficients: (a) Quantizer selection as state machine; (b) State transition as basic cell of a trellis structure.

The switching between the two scalar quantizers (Q0 and Q1) is realized via a state machine, which is illustrated in Figure 12.  The state can take four different values: 0, 1, 2, 3.  It is uniquely determined by the parities of the transform coefficient levels preceding the current transform coefficient in coding/reconstruction order.  At the start of the inverse quantization for a transform block, the state is set equal to 0.  The transform coefficients are reconstructed in scanning order (i.e., in the same order they are entropy decoded).  After a current transform coefficient is reconstructed, the state is updated as shown in Figure 12, where k denotes the value of the transform coefficient level.  Note that the next state only depends on the current state and the parity (k & 1) of the current transform coefficient level k.  With k representing the value of the current transform coefficient level, the state update can be written as

state = stateTransTable[ state ][ k & 1 ],

where stateTransTable represents the table shown in Figure 12 and the operator & specifies the bit-wise “and” operator in two’s-complement arithmetic.

The state uniquely specifies the scalar quantizer used.  If the state for a current transform coefficient is equal to 0 or 1, the scalar quantizer Q0 is used.  Otherwise (the state is equal to 2 or 3), the scalar quantizer Q1 is used.

stateTransTable[ 2 ][ 2 ] = { { 0, 2 }, { 2, 0 }, { 1, 3 }, { 3, 1 } }

state = 0

for( scanIdx = lastScanIdx; scanIdx >= 0; scanIdx-- )

{

x     = scanIdxToXpos[ scanIdx ]

y     = scanIdxToYpos[ scanIdx ]

k     = TransCoeffLevel[ scanIdx ]

n     = ( k << 1 ) – ( state > 1  ?  sign( k )  :  0 )

state = stateTransTable[ state ][ k & 1 ]

trec  = ( w[ x ][ y ] * levelScale * n + ( 1 << ( shift – 1 ) ) ) >> shift

d[ x ][ y ] = Clip3( coeffMin, coeffMax, trec )

}

Figure 13:  Pseudo-code illustrating the inverse quantization of a transform block.  The reconstructed transform coefficients represented by the array d[ x ][ y ] are calculated in scanning order, which is given by the arrays scanIdToXpos[ ] and scanIdToYPos[ ].  The array TransCoeffLevel[ ] specifies the entropy-decoded transform coefficient levels in scanning order.  The two yellow-marked lines represent the main differences to the inverse quantization in HEVC.

Figure 13 shows C-style pseudo-code for illustrating the inverse quantization process for a transform block.  The entropy-decoded transform coefficient levels are given by the array TransCoeffLevel[ ], which contains the levels in scanning order.  The transform coefficients are reconstructed in scanning order; the arrays scanIdToXPos[ ] and scanIdToYPos[ ] specify the corresponding mapping from a scan index (scanIdx) to the associated location of the transform coefficient (x and y).  The yellow-marked lines represent the main differences to the inverse quantization in HEVC:

- Mapping of a transform coefficient level k onto a multiple n of the quantization step size;

- Update of the state (which specifies the scalar quantizer used).

Given the multiple n of the quantization step size, the inverse quantization proceeds similarly to the inverse quantization in HEVC.  The multiplication with the quantization step size is realized by a multiplication with an integer factor levelScale and a following bit shift (including rounding).  Finally, the calculated reconstructed transform coefficient trec is clipped to the supported dynamic range (given by coeffMin and coeffMax) and stored in the array of reconstructed transform coefficients d[ ][ ].

For completeness, the pseudo-code in Figure 13 includes a multiplication with a quantization weighting matrix specified by w[ x ][ y ].  However, the submitted bitstreams were generated without using quantization weighting matrices (all components w[ x ][ y ] are equal to 16).  Similar to HEVC and JEM, the values of levelScale and shift represent the quantization step size Δ.

Derivation of scaling factors

The core transform matrices used in the proposal are finite precision approximations of DCT/DST matrices.  They are constructed by scaling each matrix element with a particular scaling factor and then rounding the result to the closest integer.  Due to the support of non-square and non-power of two transform blocks, the scaling cannot be solely implemented as a bit shift.  Given the quantization parameter QP, the values of levelScale and shift are derived as follows:

- When dependent scalar quantization is used (as indicated in the SPS), the quantization parameter is modified according to

QP=QP-5

- The values of levelScale and shift are derived (depending on the QP and the transform block size) as specified in sec. 2.1.7 (inverse transforms).

### Inverse transforms

Similar to JEM, the proposed video codec includes multiple inverse transforms. But instead of indicated different primary and secondary transforms (EMT and NSST in JEM), a unified approach is used. The set of available transforms includes EMT and NSST transforms (from JEM) as well as additional transforms.

For each residual block, a list of transform candidates is constructed and if required an index is signaled, which identifies the selected transform among the candidates. Which transforms are considered as candidates depends on the prediction mode, the channel type, and the size of the residual block. While all transforms defined by EMT and NSST (in JEM) can serve as candidates, also new transforms are defined, which have been especially trained for the newly introduced prediction modes and for dedicated block sizes. The set of transforms comprises primary and secondary transforms. Secondary transforms are used for transform blocks for which the width (W) or height (H) exceed 8 and are applied to the low-frequency part of the primary transform coefficients. The size of the secondary transform is given by min(8,W) x min(8,H).

For coding units that are coded in an inter predicted mode, the candidate list comprises the DCT-II transform plus the 4 transforms defined by EMT (as in JEM) for the Inter case.

In the case of line-based intra prediction (sec. 2.1.9.1) the transform is set to be DCT-II if the intra mode is not planar and DST-VII if the intra mode is planar. In the case of intra template matching (sec. 2.1.9.3) the used transform is fixed to DCT-II.

For intraNN prediction modes (sec. 2.1.9.2):

A dedicated intraNN transform set has been trained for intraNN predicted residuals. It comprises primary transforms of sizes in {4x4,4x8,8x8,8x4} and secondary transforms of sizes in {4x8,8x8,8x4} combined with the DCT-II as their primary transform. The candidate list for a residual block is formed by the DCT-II transform plus 4 transforms taken from the intraNN set. A direct mapping from residual block size and intraNN prediction mode to the 4 candidates is given for sizes in S = {4, 8, 16, 32} x {4, 8, 16, 32} by a look-up table. TBs having a size not in S, use the candidates given by the next larger block size s’, which is in S. Precisely, the size s’ is obtained by taking the size in S which is in each, the vertical and horizontal dimension, the smallest in S, but has at least the size of the block. Note, these candidates are given as secondary transforms applied to the low-frequency coefficients of the DCT-II primary transform, which is defined for all occurring sizes.

For DC, Planar and Angular modes:

In addition to the transforms defined by EMT and NSST (in JEM), a dedicated set of secondary transforms with sizes in {4x8,8x8,8x4} are provided for non-quadratic TUs. Which of the transforms serve as candidates for a residual block depends on the prediction mode and the actual TB size. The mapping from block size and intra mode to the candidate list has been found experimentally and is given by a look-up table

The actual identification of the selected transforms among the candidates is done after the CU residual data has been parsed. First, a flag is parsed which indicates if the first candidate in list is chosen. If this is not the case, for intra coded CUs it is checked if the number of non-zero and non-transform-skip luma coefficients is at least 3. Only in this case and for inter coded CUs a transform index is decoded which identifies the selected transform among the remaining 4 candidates. If the required condition on the number of significant coefficient does not hold, the first of the remaining 4 candidates is selected.

For coding the flag a dedicated context model is used for different block sizes. The transform index for the remaining 4 candidates is coded by a fixed length code with two context modeled bins, where a dedicated set of context models is used for intra and inter coded blocks.

Additional details for the adaptive transform selection are presented in JVET-J0040 [9].

Scaling for non-square and non-power of two transforms

The core transform matrices of the current proposal are finite precision approximations of DCT/DST matrices.  They are constructed by scaling each matrix element with a particular scaling factor and then rounding to the closest integer. Due to the support of non-square and non-power of two transform blocks, the scaling cannot be solely implemented as a bit shift

Let N be the size (width or height) of the applied transform in horizontal or vertical direction. N can be represented according to

|  | N=K×P |  |
| --- | --- | --- |

where K∈Z is the power of two part of N and P∈Z is the non-power of two part of N. As examples, consider to following two cases:

N=4 → K=4, P=1

N=12 → K=4, P=3

The scaling factor S that is used for determining the integer transform matrix of a transform of size N is given by

|  | S=28+log2P× KP |  |
| --- | --- | --- |

The integer transform matrices are obtained by first, multiplying the basis functions with the scaling parameters S and then rounding the coefficients to the nearest integer.

For the inverse transform, the following applies:

- The output of the first inverse transform stage is right shifted by

ST1=9+ log2P1,

where P1 specifies the parameter P for the first inverse transform size.

- The output of the second inverse transform stage is right shifted by

|  | ST2=22- B + log2P2 |  |
| --- | --- | --- |

where P2 specifies the parameter P for the second inverse transform size.

The total scaling for the inverse transform is given by,

|  | SITotal=2-15 - B - 12log2K1K2 + 12log2P1P2, |  |
| --- | --- | --- |

where B denotes the bit depth.

The parameters levelScale and shift that are used for inverse quantization (see sec. 2.1.6) are derived as follows.

- Let QP be the quantization parameter for the current transform block.

- The parameter shift is set according to

shift=12log2K1K2-1+B+4,

where the last term (+4) is used for normalizing the quantization weighting matrix. The parameters K1 and K2 denote the power-of-two-parts of the transform sizes in horizontal and vertical direction.

- The parameter levelScale is set according to

levelScale=aIQ×gIQQP%6≪QP6,

where gIQ specifies the list (6 entries) of quantization scales that is used HEVC. The list is indexed using QP%6.  The parameter aIQ is derived according to

αIQ=round28βIQP1P2           with        βIQ=12,  &if K1+K22 ∉Z1,  &otherwise

In order to obtain an integer implementation of the inverse quantization using levelScale and shift, the value βIQP1P2 is first scaled by 28 and then rounded to the nearest integer. The definition of parameter shift is modified accordingly (in relation to HEVC).

The scaling of transform matrices with factors S as well as the calculation of quantization scaling factors αIQ can be done in the program initialization step for all block shapes. The intermediate or transpose buffers can be implemented using 16 bit. Multipliers can be implemented using 16 bits and accumulators using 32 bits.

For the power of 2 sizes partial butterfly implementations of the core transforms are provided. For the non-power of 2 sizes, the transforms are implemented as a matrix multiplication, but a partial butterfly implementation is also possible up to a particular depth.

### Motion compensation

In addition to the JEM tools, the following inter-picture prediction tools are included:

- Multi-hypothesis prediction (sec. 2.1.8.1);

- Restricted merge mode (sec. 2.1.8.2);

- Multi-directional boundary padding (sec. 2.1.8.3).

The corresponding modification of the inter-picture prediction are described in the following subsections.

#### Multi-Hypothesis Inter Prediction

Other than in HEVC and JEM, where inter predicted blocks can use either uni or bi-prediction, more than two motion-compensated prediction signals (“hypotheses”) can be combined for generating the resulting overall prediction signal.  The signaling of the uni/bi-prediction signal itself is unchanged, but it is possible to specify one or more additional inter prediction signal(s).  The syntax elements which are transmitted for each of these additional inter prediction signals are very similar to the ones used for uni/bi-prediction (i.e., reference index, motion vector prediction index, and motion vector differences), as shown below.

| … | Descriptor |
| --- | --- |
| if( ! cu_skip_flag[ x0 ][ y0 ] ) { |  |
| i = 0 |  |
| readMore = 1 |  |
| while( i < MaxNumAdditionalHypotheseis && readMore ) { |  |
| additional_hypothesis_flag[ x0 ][ y0 ][ i ] | ae(v) |
| if( additional_hypothesis_flag[ x0 ][ y0 ][ i ] ) { |  |
| ref_idx_add_hyp[ x0 ][ y0 ][ i ] | ae(v) |
| mvd_coding( x0, y0, 2+i ) |  |
| mvp_add_hyp_flag[ x0 ][ y0 ][ i ] | ae(v) |
| add_hyp_weight_idx[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| readMore = additional_hypothesis_flag[ x0 ][ y0 ][ i ] |  |
| i++ |  |
| } |  |
| } |  |
| … |  |

Note that for the additional prediction signals, the concept of prediction list0/list1 is not applied, and instead one combined list consisting of all distinctive reference frames is used.

The resulting overall prediction signal is accumulated iteratively with each additional prediction signal.  A weighting factor specifies how the so far accumulated prediction signal and the current additional prediction signal are to be combined.  With the uni/bi prediction signal puni/bi and the first additional inter prediction signal h3, the resulting prediction signal p3 is obtained as follows:

p3=(1-α)puni/bi+αh3

The weighting factor α is specified by the syntax element add_hyp_weight_idx, according to the following mapping:

| add_hyp_weight_idx | α |
| --- | --- |
| 0 | 1/4 |
| 1 | -1/8 |

Analogously to above, further additional prediction signals can be used:

pn+1=1-αn+1pn+αn+1hn+1

The resulting overall prediction signal is obtained as the last pn (i.e., the pn having the largest index n).

Note that also for inter prediction blocks using MERGE mode (but not SKIP mode), additional inter prediction signals can be specified.  Further note, that in case of MERGE, not only the uni/bi prediction parameters, but also the additional prediction parameters of the selected merging candidate can be used for the current block.

Additional details on multi-hypotheses prediction are presented in JVET-J0041 [10].

#### Restricted merge mode

In B slices, the merging candidate list can be modified such that it only contains merging candidates using uni prediction.  The application of this “restricted merge mode” as well as the designated prediction list are indicated via a syntax element merge_ref_pic_list_idc, which can take the values LIST_0, LIST_1, and LIST_NONE.  When merge_ref_pic_list_idc != LIST_NONE, the merging candidate list modification process proceeds as specified by the following ordered steps:

- All merging candidates using uni prediction are removed from the merging candidate list.

- For the remaining bi-prediction merging candidates, the prediction list utilization flags are modified, such that the designated prediction list is used (e.g., for merge_ref_pic_list_idc == LIST_0, predFlagL1N is set to 0, with N taking all bi prediction merging candidates). Furthermore, all potentially available additional multi hypothesis prediction parameters are removed from these remaining merging candidates.

- Possibly empty entries at the end of the merging candidate list (due to the removal in step 1) are filled with zero motion vector entries having the appropriate prediction list utilization flag and iterating over all reference frames of this prediction list.

In the case of FRUC or SubPU merge modes, all SubPU blocks using bi-prediction are restricted to the designated prediction list usage. Note that restricted merge mode is not available in SKIP mode.

#### Multi-directional boundary padding (MDBP)

Based on the coded block shape, the given motion vector and the number of interpolation filter taps, a particular area of the reference frame is used for motion compensated prediction. In HEVC and JEM, if this referenced sample area is partially or entirely outside the area of the reconstructed reference frame, a perpendicular extension of the frame border pixels is used, which may not optimally approximate the predicted block. By exploiting different spatial prediction modes to extend the reference frame border, a better continuation might be achieved. Therefore, multi-directional boundary padding (MDBP) uses angular intra prediction to extend the reference frame border, whenever the referenced pixel area is partially or entirely outside the area of the reconstructed reference frame.

In order to reduce signaling cost for the used angular prediction mode, the best fitting mode is estimated at both the encoder and the decoder side. For the estimation, a template area is defined, which lays inside the reconstructed reference frame as shown in Figure 14.

Figure 14:  Template area used for estimating the MDBP angular mode.

In detail, for every possible angular intra prediction mode, the prediction direction is rotated by 180° to point over the available border pixels inside the reference frame. The template area is then predicted from the adjacent border pixels and is compared with the reconstructed reference frame pixels based on the SAD measure. Finally, the angular prediction mode with the smallest template-based SAD measure is chosen, to predict the referenced pixel area outside of the reference frame.

To use the available angular intra prediction for MDBP, some modifications have to be applied. First, for MDBP intra prediction the border pixels are only available at a single side of the predicted area. Therefore, only half of the angular intra prediction modes, e.g. either horizontal or vertical modes, are used depending on the prediction direction. Second, for the top and left border of the reference frame, the angular intra prediction modes have to be rotated by 180° before applying to MDBP border extension.

To provide a complete estimate of the entire referenced pixel area outside the reference frame, two template areas are used in the current implementation. The first template area is determined, based on the outermost pixel line parallel to the reference frame border. The second template area is determined, based on the first pixel line outside the reference frame border as shown in Figure 15.

Figure 15:  The two MDBP template areas T1 and T2 are based on the outermost and the first pixel line respectively, which lay outside of the reference frame.

At the edges of the reference frame, the referenced pixel area overlaps the frame border at two sides. Here MDBP is only applied at one side (the side, which overlaps the frame border by most pixels). The remaining side is padded with the perpendicular frame border padding already available.

### Intra prediction

In addition to the spatial intra prediction specified in HEVC and JEM, the proposal includes three additional intra prediction techniques, which are referred to as line-based intra coding mode (sec. 2.1.9.1), intra prediction with neural networks (sec. 2.1.9.2), and intra region-based template matching (sec. 2.1.9.3). Furthermore, two modifications for the conventional spatial intra prediction are integrated (see sec. 2.1.9.4 and sec. 2.1.9.5).

For the luma component of a coding unit coded in an intra coding mode, the following applies:

- If intra_nn_flag is equal to 1, the luma component is predicted using the intraNN mode described in sec. 2.1.9.2;

- Otherwise, if intra_rtm_flag is equal to 1, the luma component is predicted using the intra region-based template matching mode described in sec. 2.1.9.3;

- Otherwise, the luma component is predicted using conventional spatial intra prediction.

Further, if residual_1d_partitions_mode is equal to 1 and intra_nn_flag is equal to 0, the luma component is predicted using the line-based intra coding mode described in sec. 2.1.9.1 together with one of the following prediction modes:

- If intra_rtm_flag is equal to 1, line-based intra coding is combined with intra region-based template matching mode described in sec. 2.1.9.3;

- Otherwise, if intra_mrl_idx is equal to 0, line-based intra coding is combined with conventional spatial intra prediction without multiple reference lines.

The chroma components of CUs coded in an intra mode are always predicted using conventional spatial intra prediction.

The conventional intra prediction includes the following JEM tools:

- 67 intra prediction modes;

- 4-tap filter for intra prediction;

- Intra boundary filtering;

- Intra Planar PDPC.

In addition, the following modifications are used:

- bilateral filter for smoothing of border samples (sec. 2.1.9.4);

- multi-reference line intra prediction (sec. 2.1.9.5).

#### Line-based intra coding mode

In line-based intra coding mode, the luma block of a coding unit is divided into 1d horizontal or vertical stripes. Hence, a W×H block (where H is the height and W is the width) can be partitioned into H W×1 (horizontal split) or W 1×H (vertical split) stripes, as illustrated in Figure 16. This mode can be applied to all block sizes. The chromas block are coded using conventional spatial intra prediction.

Figure 16:  Example of 1d horizontal and vertical splitting of a 8×4 block into 4 (8×1) partitions (horizontal split) or 8 (1×4) partitions (vertical split).

The line-based intra coding mode is selected at the CU level.  Its usage is indicated by the syntax element residual_1d_partitions_mode (see sec. 2.1.1.9):

- If residual_1d_partitions_mode is equal to 0, the line-based intra coding mode is not used;

- Otherwise, if residual_1d_partitions_mode is equal to 1, line-based intra coding with horizontal lines is used;

- Otherwise (residual_1d_partitions_mode is equal to 2), line-based intra coding with vertical lines is used.

The syntax element residual_1d_partitions_mode is coded using two bins. The first bin indicates whether the line-based intra coding is used for the luma signal of the CU.  When this bin is equal to 1, a second bin is coded, which specifies whether a horizontal or vertical splitting is applied.

If the line-based intra coding mode is used, the W×H luma coding block will comprise n one-dimensional prediction and transform blocks, where n will be defined as the width or the height of the luma coding block, according to the type of split used.

Like any other 2d block, each of the 1d partitions will be predicted and the corresponding residual signal will be transformed, quantized and coded individually in a sequential manner. Therefore, the reconstructed values of each line will be available for generating the prediction signal for the next line. This process is continued until all 1d partitions of the luma signal are coded. Hence, each line will be predicted using neighboring samples that are located at the shortest possible distance. Figure 17 shows an example of this procedure.

Figure 17:  Example of the sequential processing of the 1d partitions of a coded luma block that is split horizontally. The first partition is predicted using the samples of the neighbor blocks. After that, the reconstructed samples of the first line are used to generate the prediction of the second partition and so. The procedure will continue until all lines are coded.

Intra prediction

The line-based intra coding mode is used in combination with any of the conventional spatial intra prediction modes available in HEVC or JEM. To avoid an excessive overhead in signaling, all 1d partitions within a CU must use the same spatial intra prediction mode. Therefore, the intra mode signalization is left unchanged (a single luma intra prediction mode is coded per coding unit). The prediction samples of each line are calculated in the same way as for the first row/column of samples in the 2d case. An exception is the Planar mode, whose equation has been slightly modified and it is described next:

Let pxy be the prediction signal of a line, where the domains of x and y are defined according to the type of split as shown in Figure 18 and p-1-1 points to the top-left neighboring sample of the line. Then, for a horizontal split type the planar prediction equation is specify by

px0=phx0+pvx04W,

where W is the width of the CU and the horizontal and vertical components ph and pv are defined as

phx0=2W-1-xp-10+x+1pW-1

pvx0=Wp-11+px-1.

The vertical split equation for the planar intra mode is obtained by swapping x with y and the vertical component with the horizontal one. Besides, W needs to be replaced with H, the height of the CU.

Finally, if the line-based intra coding mode is used for a CU, then no reference samples or prediction signal filters will be used on the block, since this type of smoothing would probably reduce the quality of the generated samples in the 1d case.

Figure 18:  x and y of the prediction signal p[x][y] and the corresponding reference samples for the vertical (left) and horizontal (right) splits. H and W are the height and the width of the CU respectively

Transform

A 1d Discrete Cosine Transform Type II (DCT-II) is used to transform all 1d residual signals within a CU. Nevertheless, a 1d Discrete Sine Transform Type VII (DST-VII) is employed for the Planar intra mode. Other transform coding tools (Transform-Skip, adaptive transforms, etc.) are disabled if a block is coded in the line-based intra prediction mode.

Processing order

Based on the intra mode and the split utilized, the two different classes of processing orders are used, which are referred to as normal and reversed order.

In normal order, the first line to be processed is the one containing the top-left sample of the CU and then continuing line-wise downwards (horizontal split) or rightwards (vertical split). As a result, reference samples used to generate the 1d prediction signals are only located at the left and above sides of the lines.

On the other hand, the reverse processing order either starts with the line containing the bottom-left sample of the CU and continues upwards or starts with line containing the top-right sample of the CU and continues leftwards.

Figure 19 shows an example of both processing orders for three different intra modes, namely the diagonal (DIAG, 18 in HEVC and 34 in JEM), the horizontal-diagonal (HDIAG, 2 in HEVC and JEM) and the vertical-diagonal (VDIAG, 34 in HEVC and 66 in JEM) modes. The DIAG mode applies the normal processing order for both the horizontal and vertical splits. On the other hand, the HDIAG mode uses the reverse order when the split is horizontal, so that it can utilize reference samples located at the left and below sides of each line. In the vertical split case, however, it uses the normal order. Analogously, VDIAG employs the normal order when the split is horizontal and the reverse one when it is vertical, so that samples located at the top and right sides of each line can be used to generate the 1d prediction signals. Table 7 shows the complete list of processing orders according to the intra mode and the split type.

Figure 19:  Normal and reverse processing order examples. The black color indicates the normal processing order (rightwards or downwards) and the red color the reverse order (leftwards or upwards).

Table 7: Processing order according to the intra mode and the split type.

| Intra Mode | Split Type | Processing Order |
| --- | --- | --- |
| [HDIAG, HOR-1] | Horizontal | Reversed |
| [HDIAG, HOR-1] | Vertical | Normal |
| [VER+1, VDIAG] | Horizontal | Normal |
| [VER+1, VDIAG] | Vertical | Reversed |
| All remaining cases | Both | Normal |

Residual coding (see also sec. 2.1.4.4)

The residual coding is carried out the same way as in the 2d case with the following exceptions:

- There are n coded block flags (CBFs) per CU, where n is the number of 1d partitions in the CU;

- Each line with a non-zero CBF transmits its corresponding 1d last position syntax element (the x coordinate for the horizontal split and the y coordinate for the vertical one);

- The context of each CBF is the value of the CBF of the previously processed line.

- The 4×4 coefficient groups degenerate into 1×4 or 4×1 coefficient lines based on the split type.

- If the 1d split is vertical, then the coefficient scan is vertical. If the 1d split is horizontal, then the coefficient scan is horizontal.

#### Intra prediction using neural networks

For coding units with intra_nn_flag equal to 1, the luma signal is predicted using neural networks, as described below. The chroma signals are predicted using conventional spatial intra prediction. For the purpose of deriving the chroma intra prediction mode, the intra luma prediction mode is treated as planar mode. If for a CU or on a TU the conventional intra prediction is to be performed for the luma component and the list of most probable modes is generated, all neighboring already reconstructed CUs or TUs in neural network intra mode are treated as having the planar prediction mode.

The intra prediction of the luma component proceeds as follows:

- Depending on the neighboring reconstructed samples and the transmitted syntax element intra_nn_mpm_idx, the intraNN prediction mode is derived using a first neural network, which consists of two layers. More accurately, the neural network derives an ordered list of prediction modes, and the transmitted index intra_nn_mpm_idx is used to identify the prediction mode used (in the ordered list).

- Given the intraNN prediction mode, the luma prediction samples for the current CU are derived using a second neural network, which consists of four layers.

When intra_nn_subsampling_flag is equal to 1, the neural networks are not directly applied. Instead, the reconstructed border is downsampled and the neural network is applied to the downsampled signal. Finally, the derived prediction signal is upsampled.

Figure 20:  Overview of the intra prediction with neural networks (predIdx represents the syntax element intra_nn_mpm_idx transmitted in the bitstream).

An overview of the intra prediction using neural network is given in Figure 20. Details are described below.

Derivation of the intraNN prediction mode

The intra prediction with neural networks provides n different prediction modes. For CUs for which both the smallest power of two larger or equal than the CU width and the smallest power of two larger or equal than the CU height are strictly less than 32, n=35 prediction modes are supported; for all other CUs, n=11 prediction modes are supported. The prediction modes are not directly coded in the bitstream. Instead, an index intra_nn_mpm_idx is transmitted. At the decoder side, a fully connected neural network with one hidden layer is used for deriving an ordered list of intraNN prediction modes. The used prediction mode is indicated via index intra_nn_mpm_idx in the ordered mode list.

Figure 21:  Prediction of mode probabilities using a neural network (M and N are assumed to be integer powers of two here).

The approach is illustrated in Figure 21.  For an M×N luma block, let M’ resp. N’ denote the smallest integers powers of two that are larger or equal than M resp. N. The input of the neural network are the reconstructed samples on the two rows of size N’+2 above and the two columns of size M’ left of the block (see figure). Let r' represent these reconstructed samples arranged in a vector of size d = 2*(M’+N’+2).  The ordered list of n prediction modes (with n being equal to 11 or 35, see above) is derived as follows:

- An intermediate vector t1' of d entries is derived according to

t'1=ρ(A1'∙r'+b1'),

where A1' is a constant d×d matrix and b1' is a constant vector with d entries.  The function ρ specifies an element-wise look-up table. For each vector element, ρ represents an integer approximation of the ELU function ρ0v given by

ρ0v=v:v≥0expv-1:v<0.

- Another intermediate vector lgt with n entries is derived according to

lgt=A2'∙t1'+b2' ,

where A2' is a constant matrix with n rows and d columns and b1' is a constant vector with n entries.

- Given the vector lgt, the ordered list of intraNN modes is constructed as follows.  The entries (lgt)k are ordered in decreasing order of their values. At this, if two entries (lgt)k and (lgt)l   are equal for k≠l, (lgt)k is regarded as larger than (lgt)l  if k<l and (lgt)l  is regarded as larger than (lgt)k  otherwise.  Finally, a prediction mode m is placed at the k-th index in the ordered list, if the m-th component of the original vector lgt represents the k-th largest entry in the ordered list of the elements of lgt.

To explain the above procedure, it is remarked that the k-th entry of the vector lgt is to be interpreted, up to a normalization, as the logarithm of the conditional probability of the k-th intra prediction mode given the reference samples r'and thus the index intra_nn_mpm_idx indicates that the intra_nn_mpm_idx-th most probable mode is selected.

Generation of the luma intra prediction signal

On a given M×N luma block with M≤32, N≤32, the generation of the luma prediction signal pred is performed by processing a set of reference samples r through a neural network. Let M’ resp. N’ denote the smallest integer powers of two that are larger or equal than M resp. N. The reference samples r consist of 2 rows of size N’+2 above and 2 columns of size M’ left of the block, as illustrated in Figure 22.

Figure 22:  Prediction of M×N intra luma block from reconstructed neighboring samples using a neural network (M and N are assumed to be integer powers of two here).

The proposed neural network first extracts a vector ftr of features from the reconstructed samples r as follows. Let d0=2*(N’+M’+2) denote the number of samples of reconstructed neighborhood that is used for prediction. And let r represent these samples as a vector of size d0.

The feature vector ftr is an integer vector of d0 entries. It is calculated as specified by the following ordered steps:

- An intermediate integer vector t1 of d0 samples is determined according to

t1=ρ(A1∙r+b1 ),

where A1 is a constant d0×d0 matrix with integer elements and b1 is a constant vector with d0 integer entries.  The function ρ specifies an element-wise look-up table. For each vector element, ρ represents an integer approximation of the ELU function ρ0v given by

ρ0v=v:v≥0expv-1:v<0.

- Another intermediate integer vector t2 of d0 samples is determined according to

t2=ρ(A2∙t1+b2 ),

where A2 is another constant d0×d0 matrix with integer elements and b2 is another constant vector with d0 integer entries.

- Finally, the integer feature vector ftr of size d0 is determined according to

ftr=ρ(A3∙t2+b3),

where A3 is a constant d0×d0 matrix with integer elements and b3 is a constant vector with d0 integer entries.

From the feature vector ftr, the final prediction signal pred is generated using an affine linear map followed by the standard Clipping operation  Clip that depends on the bit-depth. In particular, a predefined matrix A4 with M*N rows and d0 columns and a predefined bias vector b4 of dimension M*N are used to calculate

pred=ClipA4∙ftr+b4 .

The above proposed generation of the prediction signal pred is depicted at the right hand side of Figure 20.

The matrices A1, A2 and A3 and the vectors b1, b2 and b3 are independent of the intraNN prediction mode. Only the matrix A4 and the vector b4  depend on the signalled intraNN prediction mode.

Generation of the luma intra prediction signal with intra_nn_subsampling_flag equal to 1

When intra_nn_subsampling_flag is equal to 1, the following applies.

For blocks of size 64×64, the intra prediction modes for blocks of size 32×32 are reused. For that purpose, for K=2, 2K rows of size 64+2K above the block and 2K columns of size 64 left of the block with reconstructed samples r are taken. Then a (1, 2, 1)/4 low-pass filter in each direction followed by a factor two downsampling in each direction is applied. This yields reconstructed samples rds that can be regarded as an input for the prediction modes on blocks of size 32×32. Let predsmall be a prediction signal of size 32×32 generated by the proposed method using the input rds. Then, the final prediction signal pred is obtained by upsampling predsmall by bilinear interpolation. Using a similar operation, the signalization of the proposed intra prediction modes for blocks of size 32×32 induces a signalization for the proposed intra prediction modes for blocks of size 64×64.

A similar procedure to reuse the prediction modes for blocks of size 16x16 to generate prediction signals on blocks of size 32x32 is additionally supported. Its usage is indicated by the binary syntax element intra_nn_subsampling_flag.

Additional details on intra prediction using neural networks are presented in JVET-J0037 [6].

#### Intra region-based template matching

If intra_rtm_flag is equal to 1, the luma component of the coding unit is predicted using intra region-based template matching.  In intra region-based template matching, the prediction signal for the luma signal is given by a linear combination of already reconstructed blocks inside the current picture. The location of the blocks used is specified by integer-precision displacement vectors. However, the displacement vectors are not explicitly coded.  Instead, a region index is transmitted and the displacement vectors are determined by a defined decoder-side displacement search inside the indicated region.

Figure 23:  Illustration of intra region-based template matching.

The basic concept of intra region-based template matching is illustrated in Figure 23.  If the current CU is processed, all preceding CUs in coding order are already reconstructed. A template refers to the samples above and left of the current luma block. The template is 2-sample wide.  Let the template of the current block be Tc and any possible template in the reconstructed luma area of the current picture be Tr.  The basic idea of a template matching intra mode is to find the best match for Tc by minimizing an error metric.  The error metric used is the sum of squared differences (SSD). Thus, the best template match, Tb , is the template Tr that gives the minimum SSD error,

Tb =argminr SSDTc, Tr  .

The block associated with Tb forms the luma prediction signal for the current CU.  The intra region-based template matching mode searches for the best template match inside a window in the immediate neighborhood of the block to be predicted. The search window is partitioned into five regions as shown in Figure 24. The size of the search regions depends on the frame size of the input video sequence (see Table 8).  Furthermore, at the decoder side, only one region indicated by a syntax element signaled in the bitstream is considered.

Figure 24:  Search window and five regions of intra region-based template matching.

Note that the sizes of the search regions depend on the region index (see Figure 24), the frame width (for regions 4 and 5), and the region size parameter RSP (see Table 8). The region size parameter RSP, mentioned in Table 8, is added to the sequence parameter set of the codec. The value of RSP can be modified for scaling the FTM tool for different combinations of coding gain and decoder complexity. For the submitted bitstream, the region size parameter was set equal to 1.

Table 8: Adaptive search region sizes

| Region size | Region size calculation | frame width w | downsampled search region |
| --- | --- | --- | --- |
| A1 | 4 × (RSP + 1) | all frame widths | no |
| A2 | 3 × A1 | all frame widths | no |
| A3 | 3 × A2 6 × A2 | 0 < w ≤ 1280 otherwise | no yes |

If intra_rtm_flag is equal to 1, the region index intra_rtm_region_idx is transmitted. It specifies which search region is used for the decoder side displacement estimation. Given the search region (1, 2, 3, 4, or 5), all candidate blocks inside the region are processed in a defined order and the three best matches (minimum SSD) are determined. If two blocks yield the same SSD, the one that is processed first is considered the better match.  Let Tb1 , Tb2 and Tb3  denote the three best matches, with SSDTc,Tb1≤SSDTc,Tb2≤SSDTc,Tb3.  The associated predictor blocks of Tb1 , Tb2 and Tb3  are denoted by P1 , P2  and P3 , respectively.  Figure 25 shows an example for Region 4.

Figure 25:  Example for the three best template and the corresponding blocks, for Region 4.

The final luma prediction signal for the CU is derived according to

PFinal= (2⋅P1+ P2+P3) / 4.

For coding units that are predicted in intra region-based template matching mode, the DCT is used for transform coding (adaptive transforms are not supported for this mode). Furthermore, the chroma sample arrays of the coding unit are predicted using conventional spatial intra prediction. For the purpose of deriving the chroma intra prediction mode, the luma intra prediction mode is treated as DC prediction mode.

Search algorithm

Since both encoder and decoder have to conduct the same search for an image patch, the search order needs to be well defined (otherwise, two different patches with the same SSD may be used in encoder and decoder). The intra template matching uses a line-by-line search order. Figure 26 shows the starting position of the search method for each regions.

Figure 26:  Starting position for the search algorithm for different regions.

From the starting position, the search progresses line-by-line towards the outer borders of each region. Region 1 has horizontal and vertical lines. However, Region 2 and 4 have horizontal lines only and Regions 3 and 5 have vertical lines only (Region 2 and 4 are referred as horizontal regions and Region 3 and 5 as vertical regions). For horizontal lines, the search starts from left and continues towards the right border. Similarly, for vertical lines top to bottom approach is used. In each iteration of the search method, one line is searched for all regions except Region1. In the case of Region1, one horizontal line and one vertical line (i.e. two lines) are searched in a single iteration. The search algorithm for different region types is further explained in the following.

For Region 1 (Figure 27), in the first iteration the search starts from (Sx1, Sy1) and continues towards the right border in horizontal direction as well as towards the bottom border in vertical direction. In the second iteration, the start point shifts to one position back in horizontal and vertical directions (S'x1, S'y1). Now, the search starts from (S'x1, S'y1) and progresses towards the right border in horizontal direction as well as towards the bottom border in vertical direction. For horizontal regions, only horizontal lines are searched (Figure 28) whereas for vertical regions only vertical lines are searched (Figure 29). The process is repeated until the given region is completely searched.

Figure 27:  Search algorithm for Region 1, for different iterations.

Figure 28:  Search algorithm for a horizontal region (Regions 2 and 4), for different iterations.

Figure 29:  Search algorithm for a vertical region (Regions 3 and 5), for different iterations.

The objective of the search algorithm is to find the three best template matches Tb1 , Tb2 and Tb3 for the current template Tc. Therefore, for every integer-pel position in each line, the method finds the SSD error between Tc and the template present in the current search position Tr.  Tb1 , Tb2 and Tb3 are determined based on the rule in the flowchart in Figure 30.

It should be noted here that the end of the right border and bottom border of the regions (i.e. endX and endY in Figure 26) varies depending on the position of the current CU with respect to the position of the current CTU. However, these values are limited to the boundary values of the current CTU.

For the case of the CUs in the picture boundary, the search algorithm is not invoked and the prediction signal is set to a DC value.

For the case where the outer regions (2 to 5) are outside the picture boundary, the predictors are set to the first block of the reconstructed picture.

Figure 30:  Rule for finding the best template matches.

Additional details on intra template matching are presented in JVET-J0039 [8].

#### Bilateral filter for intra reference samples

HEVC uses low-pass filtered reference samples for larger blocks and strong intra smoothing for 32×32 blocks. New flexible block partitioning schemes like QTBT, MTT, or the generalized binary split (sec. 2.1.2) have in common that they support larger block sizes and different block shapes (= rectangular) than HEVC. Under these conditions, strong intra smoothing turns out being non-beneficial.

However, intra prediction does still benefit from strongly smoothed reference samples. For this purpose, we included a bilateral reference sample filter described in the following. It combines the two effects of preserving significant textures like sharp edges and avoiding small local details being predicted over several rows/columns of samples.

Figure 31:  Reference samples of a W×H intra block (left) and arrangement for bilateral filtering (right).

1d bilateral filter

The line of reference samples for predicting an intra block can be understood as a 1d array of values I(x), as illustrated in Figure 31. For this, the bilateral filter function is defined as follows:

Ix=∆xi≤swxi∙Ixi∆xi≤swnwxi ,

with weight function

wxi=e-cval∙(Ixi-Ix)2+cpos∙∆xi2

using spatial offset ∆xi=xi-x and filter parameters

- s for filter size,

- cval=12σval2 for Gaussian range kernel, corresponds to level of smoothing,

- cpos=12σpos2 for Gaussian spatial kernel, corresponds to size of smoothing.

In principle, the above equations can be implemented in the intra prediction part of the codec and applied to the reference samples for a given set of parameter values.

Application to reference sample smoothing

The bilateral filter basically replaces strong intra smoothing and is consequently applied to the reference samples of intra blocks that meet the following conditions:

- luma channel,

- block size W×H ≥ 16×16,

- angular prediction mode m with minm-mHOR, m-mVER>1.

The bilateral filter is applied to all reference sample positions x∈[s .. (2W+H-s)] according to Figure 31 (right). Instead of calculating the filter weights w(xi) according to the above equation for every filter step, a lookup table with integer values is used. It has the advantage of avoiding floating-point arithmetic and exponential function calculations. The filter weights w(xi) only depend on the spatial offset ∆xi=xi-x∈[0 .. s] and the sample value difference ∆Ii=Ixi-I(x)∈[0 .. 2b-1] with bit depth b. Given the values of filter parameters s, cval and cpos, the weights can be pre-calculated as integers and stored in a lookup table as follows:

wint∆xi,∆Ii=p∙e-cval∙∆Ii2+cpos∙∆xi2, 
where p is the integer precision factor and  .  is the “round to nearest integer” function. Accordingly, the bilateral filter function can be reformulated as

Ix=∆xi≤swint∆xi,∆Ii∙Ixi∆xi≤swint∆xi,∆Ii, 
where both wint and I are integer values and wint is looked up in the table, thus, avoiding both floating point arithmetic and complex calculations.

The bilateral filter is applied in two different configurations depending on the block size, using the parameters shown in Table 9.

Table 9: Parameters for the bilateral filter

| block size | W×H | <64×64 | ≥64×64 |
| --- | --- | --- | --- |
| filter size | s | 3 | 4 |
| range kernel | cval | 2-8 | 2-12 |
| spatial kernel | cpos | 2-5 | 2-5 |
| num. iterations | N | 1 | 3 |

If the number of iterations N is larger than one, the filter is applied iteratively. This means that the result of the first complete filtering step I is used as the input I to the second filtering step and so on.

With the above parameters, a single lookup table can be used for both configurations, as the weights of the first configuration are a subset of the second configuration. With a precision factor of p=210, the resulting lookup table has an effective size of 5×177 values.

#### Multiple-reference line intra prediction

Conventional intra picture prediction always uses the directly neighboring samples as reference samples for prediction. Multiple-reference line (MRL) extends the conventional prediction to use neighboring samples with one or three samples distance to the left and top of the current prediction block. The conventional directly neighboring reference sample lines and the extended reference sample lines are illustrated in Figure 32. The mrl_idx indicates which line to use in intra-picture prediction of a CU.

Figure 32:  Directly neighboring (nearest) reference samples in blue and extended reference samples in green used in MRL.

Prediction

All three conventional predictions are adapted in a straightforward way to use the extended reference samples with mrl_idx larger than 0.

For DC, the W top and H left reference samples from the reference sample line indicated by mrl_idx are averaged as follows.

p(x,y)= DC with x=0..W-1,y=0..H-1

DCmrl_idx= 1M+Nn=0N-1r(-1-mrl_idx,n)+m=0M-1r(m,-1-mrl_idx)

As in JEM, conventional PLANAR prediction is replaced by a combination of PLANAR and PDPC. For mrl_idx larger than 0, conventional HEVC planar prediction using W top, H left reference samples as well as the top right and bottom left sample is applied as follows:

px,y= W∙pverx,y+ H∙phorx,y  + W∙H2W∙H with x=0..W-1,y=0..H-1

pver,mrl_idxx,y=(H-1-y)∙r(x,-1-mrl_idx)+(y+1)∙r(-1-mrl_idx,H)

phor,mrl_idxx,y=(W-1-x)∙r(-1-mrl_idx,y)+(x+1)∙r(W,-1-mrl_idx)

The JEM ANGULAR prediction with 65 directions and 4-tap interpolation filter is also adapted without modification. Due to the increased distance from the prediction block, one more reference sample for each increment of mrl_idx is needed on the top right and bottom left for the extended reference lines as can be seen in Figure 32.

Reference sample generation and filtering

Reference sample generation, substitution and filtering is the same for the extended reference samples as for the directly neighboring samples. As discussed above, angular prediction requires extension of the reference samples to the top right and bottom left. Consequently, the availability checking and substitution process needs to be extended for these samples.

Boundary and edge filtering

Boundary smoothing and edge filtering in JEM filters prediction samples and directly neighboring reference samples. For the extended reference samples, boundary as well edge filtering is disabled.

### In-loop filtering

There are four in-loop filters that are applied after a picture is completely reconstructed. The filters are applied in the following order:

- Bi-lateral filter (same as in JEM);

- Deblocking filter (same as in HEVC and JEM);

- Sample adaptive offset filter (same as in HEVC and JEM);

- Multiple-feature based adaptive loop filter (MCALF, improved version of GALF).

The bilateral filter is the same as in JEM. The deblocking filter and SAO are the same filter as in HEVC and JEM.  The additional filter called multiple feature-based classifications adaptive loop filter (MCALF) represents an improved version of the geometry transformation-based adaptive loop filter (GALF) in JEM. The main procedure of GALF is to classify each 2×2 sample block into one of 25 classes based on the direction and activity of local gradients of the reconstructed picture s’ followed by the application of Wiener filtering for samples belonging to each of those classes. In fact, the performance of GALF heavily relies on how its classification for each sample behaves. To further improve the overall performance of the adaptive loop filter, the classification for the luma component is modified for MCALF. Other than this new classification for the luma component, MCALF follows the same procedure of GALF. That means, given the classification, the filter coefficients are coded as in JEM, and the filters are applied in the same way as in JEM.

In general, a classification of the reconstructed samples s'(i,j) is given as

CkD=i,j∈I :Di,j=k       for      k=0,…,K-1

where I is a set of all sample locations of the reconstructed samples s'(i,j).  D is a classifier that assign a class index k ∈0,…,K-1 to each sample location (i,j).  And a class CkD is the collection of all sample to which the class index k is assigned by the classifier D. K specifies the number of classes. The MCALF supports three different classifiers, each of which provides K=25 or 27 classes.  The classifier that is used in the decoder is specified by the syntax element classification_idx, which is transmitted at a slice level. Given the classes CkD, with k ∈0,…,K-1, the adaptive loop filter proceeds as in JEM.

Depending on the value of classification_idx, the following applies:

- If classification_idx is equal to 0, the classifier DG specified for GALF in JEM is used. The classifier provides K=25 classes. The samples are classified based on the direction and activity of local gradient in the reconstructed picture.

- Otherwise, if classification_idx is equal to 1, a sample-based feature descriptor DS is used as classifier.  DS(i,j) simply takes the quantized sample value of each sample s'i,j according to

DPi,j= K-12B⋅s'(i,j)

where B is the sample bit depth, the number of classes K is set equal to K=27, and the operator ⋅ specifies rounding to the largest integer not larger than the argument.

- Otherwise (classification_idx is equal to 2), a ranking-based feature descriptor DR is used. The ranking-based feature descriptor DR is specified as follows. Let r8i,j be a descriptor that ranks a sample value s'i,j in order of its magnitude compared to its 8 neighboring samples,

r8i,j=k=i-1i+1l=j-1j+1 s'i,j<s'k,l  ?  1  : 0 .

The descriptor r8i,j can take values in the range of 0 to 8, inclusive. If the sample s'i,j is the largest sample inside the 3×3 block centered around i,j, the value of r8i,j is equal to 0. If s'i,j is the second largest sample, r8i,j is equal to 1, etc.  The ranking-based feature descriptor DRi,j is now defined according to

DRi,j=r8i,j,                                   s'i,j≤2B3 r8i,j+9,            2B3< s'i,j≤2B+13r8i,j+18,                     s'i,j>2B+13 .

That means, the dynamic range for the samples is divided into 3 bands of approximately the same size and inside each band, the rank of the sample in its local neighborhood is used as additional criterion. The ranking-based feature descriptor gives K=27 classes (3 intervals times 9 ranks per interval).

Given the classification of samples, the filtering in MCALF is identical to that in GALF (JEM).

The MCALF version used in the submitted bitstreams supported 2 additional classifiers, which represented combinations of the sample-based and ranking-based classifier specified above. We found, however, that these two additional classifiers do not further improve coding efficiency.

### Additional tools

The proposed codec include two tools, which are applied after intra- or inter-picture prediction of a block and are designed for improving the prediction signal:

- Prediction signal filtering using diffusion filters (sec. 2.1.11.1)

- Prediction signal filtering using DCT thresholding (sec. 2.1.11.2).

Details are described in the following subsections.

#### Prediction signal filtering using diffusion filters

If indicated by cu_diffusion_filter_idx > 0, the initial intra- or inter-prediction samples for the luma component are filtered using a diffusion filter. The final luma prediction signal is formed by the result of the filtering.

For the filtering, the luma prediction signal of the CU is extended by one sample to the left and to the top by using reconstructed samples of already coded/decoded neighboring blocks. To the right and to bottom (non-available samples), it is extended by one sample using a reflection of the boundary values at the edge.  The one-sample boundary extension is illustrated in Figure 33. The extended block of prediction samples forms the input for the diffusion filter.

Figure 33:  Extended prediction block: The blue boundary denotes reconstructed samples of neighboring blocks, the red boundary represents a reflection of the initial prediction signal.

The diffusion filter has two filtering modes: a linear and a non-linear mode.  Both modes specify iterative filters. The filtering mode as well as the number of applied iterations are indicated by the syntax element cu_diffusion_filter_idx. The mapping is specified in Table 10.

Table 10: Interpretation of the syntax element cu_diffusion_filter_idx.

| cu_diffusion_filter_idx | filter type | number of iterations N |  |
| --- | --- | --- | --- |
|  |  | intra | inter |
| 0 | no filter | - | - |
| 1 | linear | 5 | 5 |
| 2 | linear | 15 | 35 |
| 3 | non-linear | 5 | 4 |
| 4 | non-linear | 20 | 8 |

The filtering modes are described in the following.

Linear diffusion filter

The linear diffusion filter is implemented as a symmetric filter mask

h=14 010101010.

Thus, in each iteration of the filter, the filtered prediction u is calculated according to

un+1=h*un

where un specifies the prediction signal after the n-th iteration and “*” denotes the convolution operator. At each convolution step, at the right and lower boundary symmetric boundary conditions are assumed while at the top and left boundary equality with the reconstructed samples is assumed as shown in Figure 33. The first iteration is applied to the original extended prediction block, u0=p, where p specifies the original prediction signal. Whenever possible, all 2D filter masks are implemented as 1D filters. Note that the 1-sample boundaries are not modified in the iterations.

After the last iteration, the filtered prediction uN is cut down to the original size of the current luma block and is used as final block of prediction samples.

Non-linear diffusion filter

While the linear diffusion filter smoothes edges and smoother areas alike, the idea of the non-linear diffusion filter is to identify edges in the given prediction signal and reduce the magnitude of diffusion at those locations. Here, edges and their directions are detected in terms of the gradient of the initial prediction signal. Therefore, the proposed nonlinear diffusion filter performs n convolutions of the prediction signal p with a mask h(x,y) that is different at each position (x,y) and is computed based on the prediction signal p as follows.

First, in order to identify edges, a Gaussian-smoothed diffusion tensor

J∇p =K*(∇p ∇pT)

is constructed, with a Gaussian kernel K. With “*” denoting the convolution operator, the discrete gradient ∇p=EW, NST of the original prediction signal is calculated using finite differences, namely by

EW= -000101000*p           and              NS= 0100000-10*p

using boundary conditions as described above.  That means the gradient components EW and NS are given by simple differences of two sample values.

Then, for each sample location x,y inside the extended prediction block, the 2×2 diffusion tensor is the given by

4⋅∇px,y∇px,yT=EWx,y∙EWx,yEWx,y∙NSx,yEWx,y∙NSx,yNSx,y∙NSx,y.

The Gaussian kernel is chosen as

K=14 010121010

using a reflecting boundary condition. Each of the gradient sample arrays EW∙EW, EW∙NS and NS∙NS is convolved with the kernel K. This way, we obtain a symmetric 2×2 matrix for each sample location x,y inside the extended prediction block area,

J∇p(x,y)=14gxx(x,y)gxy(x,y)gxy(x,y)gyy(x,y)=14((K*(EW∙EW))x,y((K*(EW∙NS))x,y((K*(EW∙NS))x,y((K*(NS∙NS))x,y

In the next step, the matrices Jρ∇p(x,y) are diagonalized according to

λ1(x,y)00λ2(x,y)=cos⁡(θ)-sin⁡(θ)sin⁡(θ)cos⁡(θ) Jρ∇p(x,y) cos⁡(θ)sin⁡(θ)-sin⁡(θ)cos⁡(θ),

where λ1(x,y), λ2(x,y) denote the eigenvalues of Jρ∇p(x,y) and θ represent the associated rotation angle in the coordinate system. Using trigonometric formulas, it can be shown that

λ1(x,y)=14gxx(x,y)-gxy(x,y)⋅tan⁡(θ)

λ2(x,y)=14gyy(x,y)-gxy(x,y)⋅tanθ,

where

tan⁡(2θ) =2gxy(x,y)gyy(x,y)-gxx(x,y)

The determination of the eigenvalues λ1,  λ2 is implemented using integer look-up tables. For that purpose, with a and b representing the rounded integers

a =256⋅2gxygyy-gxx       and      b =256⋅gyy-gxx2gxy

the following expressions are tabulated for a = 0..256 and b = 0..256

tab1 a =256⋅cos 0.5⋅atana256 2

tab2 a =256⋅cos 0.5⋅atana256 ⋅sin 0.5⋅atana256

tab3 a =256⋅sin 0.5⋅atana256 2

tab4 b =256⋅cos 0.5⋅atan256b 2

tab5 b =256⋅cos 0.5⋅atan256b ⋅sin 0.5⋅atan256b

tab6 b =256⋅sin 0.5⋅atan256b 2

where the square brackets [ ] specify rounding to the next integer.

Next, after the eigenvalues λ1, λ2 of J∇p are calculated, a function f is applied to the eigenvalues λ1, λ2, which basically serves as an edge detector: if eigenvalues λ1, λ2 are large, i.e. an edge is identified, function f will be small.  Function f is chosen in case of intra as fs=exp-sμ, and in case of inter as fs=1/1+sμ with parameter μ>0. The factor 14 can be incorporated into function f.  The parameter μ is a fixed value given by

μ=200,  &for intra prediction64,  &for inter prediction and QP<33550,  &for inter prediction and QP≥33

The two functions fs are tabulated using integer tables.

After applying function f, the matrix is retransformed according to

fJρ∇fx=cosθsinθ-sinθcosθfλ100f(λ2)cosθ-sinθsinθcosθ= axxaxyaxyayy(x,y)

Using this sample-wise definition, three 2×2 functions axx(x,y), axy(x,y),ayy(x,y) are defined for each sample location (x,y) inside the extended prediction block area.  Similarly, as described above the rotation of the 2×2 matrix is implemented using the look-up tables given above.

Given the arrays axx(x,y), axy(x,y),ayy(x,y) for each sample location (x,y), the following 8 arrays are derived using a simple averaging of two neighboring samples:

wN=12010010000*ayy

wS=12000010010*ayy

wW=12000110000*axx

wE=12000011000*axx

wO1=14010001000* axy

wO2=14000001010* axy

wO3=14010100000* axy

wO4=14000100010* axy

The integer weighting matrices wN, wS, wW, wE, wO1, wO2, wO3, wO4 are calculated at the beginning of the non-linear diffusion filter process. These matrices are used for the actual filtering in the iteration steps. For summarizing the approach, the integer weighting matrices are derived by the following ordered steps:

- Calculate discrete samples differences EW(x,y) and NS(x,y)

- Calculate products (EW⋅EW)(x,y), EW⋅NSx,y , (NS⋅NS)(x,y)

- Filter the three products with a 5-tap filter kernel for obtaining the gradient arrays gxx(x,y), gxy(x,y), and gyy(x,y)

- Determine the eigenvalues λ1(x,y), λ2(x,y) using look-up tables

- Apply a function f(s) to the eigenvalues using pre-defined look-up tables

- Derive the integer arrays axx(x,y), axy(x,y), and ayy(x,y) using look-up tables

- Derive the integer weighting arrays wN, wS, wW, wE, wO1, wO2, wO3, and wO4 using sample averages.

Given the arrays wN, wS, wW, wE, wO1, wO2, wO3, and wO4 for the sample locations (x,y), the iterative filtering proceeds as follows. Let u0=p represent the initial extended luma prediction signal. In each iteration step, the signal u(x,y) is modified according to

un+1x,y=unx,y+14wNx,y⋅unx,y-1-unx,y+wSx,y⋅unx,y+1-unx,y+wWx,y⋅unx-1,y-unx,y+wEx,y⋅unx+1,y-unx,y+wO1x,y⋅unx+1,y-1-wO2x,y⋅unx+1,y+1-wO3x,y⋅unx-1,y-1+wO4⋅unx-1,y+1,

where n represents the iteration index. This way, diffusion along edges is preferred over diffusion perpendicular to them. In the end, the filtered prediction uN (with N denoting the number of iterations) is cut down to the original size of the current block and is used as the modified prediction block.

For illustration, Figure 34 shows a comparison for the application of the linear and the nonlinear filter on an angular intra prediction signal. It is visible that while the linear filter blurres the edges of the underlying prediction, the nonlinear filter preserves them.

Figure 34:  (left) original intra prediction signal; (middle) linearly filtered intra prediction; (right) non-linearly filtered intra prediction.

Further details on the diffusion filter are presented in JVET-J0038 [7].

#### Prediction signal filtering using DCT thresholding

DCT thresholding refers to an approach for improving an initial prediction signal.  The basic concept of DCT thresholding is illustrated in Figure 35.  Given is an initial prediction signal p0 for an M×N block as well as reconstructed samples of already coded/decoded neighboring blocks. The initial prediction signal p0 is either obtained by intra-picture prediction or inter-picture prediction. The final prediction signal p1 for the M×N block is derived as follows:

- A separable 2d DCT (integer approximation) is applied to a K×L block of samples. As illustrated in Figure 35, the K×L block of samples consists of the M×N block p0 of initial prediction samples as well as reconstructed samples above the current block and left to the current block. The result of the transform is a K×L array c0 of transform coefficients.

- The threshold t is determined as described below.

- All transform coefficients of the K×L array c0 are thresholded using the threshold t. With x representing any single transform coefficient, the thresholding is defined according to

y= 0:x≤tx:x>t .

The results y of the element-wise thresholding form the new K×L array c1 of transform coefficients.

- The K×L array c1 of transform coefficients is inverse transformed (using the inverse of the forward transform, i.e., an integer approximation of the 2d DCT of size K×L).  As result an intermediate K×L array of samples is obtained. From this intermediate array, the M×N block of samples that represent the same area as the original prediction signal are extracted (see Figure 35). These samples form the final prediction signal p1 for the considered M×N block.

Note that the reconstructed samples of neighboring blocks are not replaced. They are solely used to initialize the input of the forward transform.

Figure 35:  Basic concept of prediction signal filtering using the thresholding approach.

The transforms (DCT and IDCT) that are applied are the same integer approximations that are used for transform coding. In the proposed codec, DCT thresholding is only applied to the luma component. Furthermore, DCT thresholding is only supported if the reconstructed samples r in the K-M rows above and the L-N columns on the left of the current luma block are available.

Figure 36 demonstrates how the DCT thresholding method modifies the prediction signal. In this example, the initial prediction signal is obtained by an angular intra prediction.

Figure 36:  Example for thresholding (angular intra prediction): (left) original luma samples; (middle) intra prediction with extended neighborhood; (right) result of the DCT thresholding, where the bottom-left area replaces the initial prediction.

The DCT thresholding is applied to the luma prediction signal if the binary syntax element thresholding_flag for the current coding unit is equal to 1.  In this case, the cu_residual_data() syntax includes the syntax elements thresholding_size and thresholding_threshold, which specify the size of the K×L region that is used as input to the forward transform and the applied threshold, respectively.

sizes[18] = { 2, 4, 6, 8, 10, 12, 16, 20, 24,
              32, 40, 48, 64, 80, 96, 128, 160, 192 }

for( i = 0; i < 18; i++ )

{

if( sizes[ i ] == M )

K = i

if( sizes[ i ] == N  )

L = i

}

K = sizes[ min( K + thresholding_size + 1, 17 ) ]

L = sizes[ min( L + thresholding_size + 1, 17 ) ]

Figure 37:  Pseudo-code showing the derivation of K and L given M, N, and thresholding_size.

Given the size of the current CU (M×N in luma samples) and the syntax element thresholding_size, the size of the K×L region that is used as input to the forward transform is derived as specified by the pseudo-code in Figure 37.

Given the syntax element thresholding_threshold, the threshold t that is used for thresholding the DCT coefficients (as described above) is derived as follows:

- A forward DCT (integer approximation) is applied to the K×L region (the size is determined as specified above).

- The resulting transform coefficients are quantized with a quantization step size Δ, which is determined by the quantization parameter (QP).

- The absolute values of the resulting quantization indices are sorted in ascending order, where duplicated entries are removed. The resulting ordered array of absolute quantization indices is referred to as qIdx[ ].

- The threshold t is set according to

t=Δ⋅qIdx[ thresholding_threshold ]

Note that the derivation of the threshold t is part of the thresholding process specified above.

### Additional algorithmic description topic(s)

As mentioned in sec. 1, we submitted two sets of bitstreams, which differ in the encoder selection of quantization parameters:

- Set 1 uses static quantization parameter settings as the anchor.

- Set 2 uses the CTU-wise QP adaptation described in sec. 3.1.11.1.

## 360° Video

### Projection format (including padding)

We used the equi-angular cubemap (EAC) projection format (see JVET-G1003 [11] for details). In order to reduce seem artifacts visible in rendered viewports, the following two modifications were added:

- Guard bands (of 4 luma samples) are added around the two continuous regions of samples (left+front+right and top+back-bottom) as it has been suggested in JVET-G0071 [12] for the ACP format.

- For the back projection, a linear blending is used for regions that are available in more than one cube face.

Figure 38:  Coding format for 360° picture: The six cube faces of the EAC format are arranged in a 6×1 layout. Guard band with a width of 4 luma samples are added around the two halves of the cube (which represent continuous regions of samples).

The layout of coded video frames is illustrated in Figure 38. The six cube faces are arranged in a 6 × 1 layout. The entire coded frame basically consists of two continuous areas of samples. The first area is formed by the left, front and right cube face. The second area is formed by the top, back, and bottom cube face (all rotated by 90° as illustrated in the figure). Guard bands of 4 luma samples are added around both continuous sample areas. The modifications of the 2d-to-3d and 3d-to-2d coordinate conversion formulas are the same as the ones proposed in JVET- G0071 [12] for the ACP format. For details, the reader is referred to this document.

The face sizes are chosen in a way that the face boundaries coincide with CTU boundaries. Hence, both the face width and height represent a multiple of the CTU size (128 luma samples). For the 6k video, a face size of 1152 was chosen; and for the 8k videos, a face size of 1280 was chosen. The face sizes and the resulting sizes of the coded video frames are summarized in Table 11.

Table 11:  Coded format for 360 videos.

| original ERP format | face size | coded frame size |
| --- | --- | --- |
| 6144 × 3072 | 1152 × 1152 | 6912 × 1152 |
| 8192 × 4096 | 1280 × 1280 | 7680 × 1280 |

The guard bands are not only used for reducing coding artifacts at face boundaries, but also for additionally reducing seem artifacts in rendered viewports. For that purpose, we use a blending for sample regions that are present in more than one of the coded cube faces.

The rendering process that is implemented in the 360Lib (see JVET-G1003 [11]) for cubemap formats basically consists of two steps:

- A border of 16 samples is added around each cube face. The border samples are obtained via geometric mapping from other coded cube faces. They are used later for applying interpolation filters across face boundaries.

- The samples of the cube faces are mapped to the output format. For each sample location of the output format, the cube face that contains the sample location and the sub-sample accurate location inside the cube face are determined. Finally, the output sample is obtained by interpolation of the selected cube face.

Figure 39:  Cube face padding and blending on the example of the left cube face: (left) Decoded cube face including guard bands; (right) Cube face after padding and blending. A padding border (grey area) is added around the decoded cube face for enabling the use of interpolation filter. In addition, we use a linear blending inside the region that is available in more than one decoded cube faces (light blue). The region that is used for the final projection is marked red.

The rendering process is modified in a way that step 1 additionally includes a blending for sample areas that are available in more than one of the cube faces. This is illustrated in Figure 39 for the example of the left cube face. First, a 16 sample boundary is added around each coded cube face (in the same way as in the 360Lib). Then, for sample areas in the coded cube face that are also available in other coded cube faces (guard bands and surrounded area, which is represented by guard bands in other coded cube faces) a linear blending is applied. That means, the samples are projected from a different cube face first. And then, the projected samples in the coded sample inside the cube face are blended. At this the blending weight is linearly decreased from the outer to the inner border of the blending areas (area marked blue in Figure 39). The modified padded cube faces are used for generating the video picture of the output form (in the same was as implemented in the 360Lib).

Table 12:  Conversion only results for chosen projection format (EAC with guard bands of 4 luma samples).

| sequence | WS-PSNR (end-to-end) in dB |  |  |
| --- | --- | --- | --- |
|  | Y | Cb | Cr |
| Balboa | 52.19 | 60.12 | 60.05 |
| Chairlift | 51.44 | 60.18 | 60.22 |
| KiteFlite | 47.81 | 58.18 | 58.19 |
| Harbor | 50.24 | 59.59 | 58.60 |
| Trolley | 46.71 | 58.44 | 58.34 |

Table 12 summarizes the conversion only results (as end-to-end WS-PSNR) that are obtained with the projection format used.

### Quantization handling

We submitted two sets of bitstreams, which differ in the encoder selection of quantization parameters:

- Set 1 uses static quantization parameter settings as the anchor.

- Set 2 uses the CTU-wise QP adaptation described in sec. 3.1.11.1.

### 360° video specific decoding tools

The proposed codec does not include any 360° video specific coding tools.

## HDR

### Post-processing

No post-processing was applied.

### Quantization handling

We submitted two sets of bitstreams, which differ in the encoder selection of quantization parameters:

- Set 1 uses the same quantization parameter settings as the anchor (the QP is adjusted within a frame as a function of the local average luma sample value).

- Set 2 uses the CTU-wise QP adaptation described in sec. 3.1.11.1.

### HDR specific decoding tools

The proposed codec does not include any HDR specific coding tools.

# Encoder algorithm description

## General algorithm description

### Rate-distortion optimization

In the same way as in HM and JEM, all encoder decisions are taken based on a minimization of Lagrangian rate-distortion costs. That means, if the encoder can select among a set of options {p} for coding a block of samples, for each option p, the distortion D(p) between the resulting reconstructed samples and the original samples and the number of bits R(p) required for representing the block are calculated. The encoder selects the option p* that minimizes the Lagrange function of distortion and rate,

p*=argminpDp+λ⋅R(p),

where λ is a Lagrange multiplier. The Lagrange multiplier λ is set depending on the quantization parameter, in the same way as in HM and JEM.

The concept of Lagrangian encoder decision is applied to

- Mode decision (partitioning mode, prediction modes, transform modes);

- Motion estimation;

- Quantization.

#### Distortion metrics

The same distortion measures as in the HM and JEM encoder are used:

- Sum of squared differences (SSD) for all mode decisions and quantization;

- Sum of absolute differences (SAD) for integer-sample accurate motion search;

- Sum of absolute differences after Hadamard transform (SATD) for sub-sample accurate motion search and pre-selection of spatial intra prediction modes.

#### Rate-distortion dependency models

Except for quantization, which is described in sec. 3.1.8, no rate-distortion dependency models are used. That means, in each encoder decision, the impact of a decision on blocks following the current block in coding order are neglected.

### Partitioning structure selection

The encoder uses a depth-first approach, that is, in order to find the optimal split mode decision, the best coding mode in terms of RD-cost is computed for each subblock which leads to the minimal cost of the block as it is the sum of all subblock costs.

As mentioned in sec. 2.1.2, the binary split approach used has several high-level parameters that effect the general availability of certain splits, for instance, neglecting certain split ratios. In addition, several speed-ups are used in the encoder to limit the number of partitioning options tested for determining the best split mode. The rules are as follows:

- For the asymmetric splits (non 1/2 splits) an additional depth is assigned and limited for non-intra slices. Depending on the temporal level TL, the following maximum asymmetric depth are used in the random access configuration:

  - TL 0-2: maximal asymmetric depth 2

  - TL 3-4: maximal asymmetric depth 1

For the low delay configuration, the depths limits depend on the POC. For even POCs, a maximum depth of 2 is used; for odd POCs, a maximum depth of 1 is used. Asymmteric splits are not tested if these depths are reached.

- For non-intra slices, a split mode is not tested if at least two split levels have been tested and for both split levels, the skip mode provided the minimum r-d cost.

- If the current slice is an intra slice and the binary tree depth of the currently best coding unit is zero and has root cbf flag equal to 0, then all splits other than 1/2 are forbidden.

Furthermore, in case of a parallel 1/2 split, the following rules are used:

- If the perpendicular and parallel 1/2 split have been tested, only split modes in the direction of the split that has yield a better r-d cost (i.e., a smaller cost) are tested.

- Gradients in horizontal and vertical directions are computed based on first order finite differences for blocks, for which the height or width is larger than 32 luma samples. The absolute values of the gradients are then summed for the four quadrants shown in Figure 40 of the initial block, which results in 8 values (four gradients of each quadrant for each direction). If the sum of the four horizontal gradients is larger than the sum of the four vertical gradients, then asymmetric splits in horizontal direction are not tested. If the sum of the horizontal gradients in block 2 and 3 are larger than the sum of horizontal gradients in 1 and 4, then vertical splits that are located in quadrant 1 and 4 are not tested. If the sum of the horizontal gradients in block 1 and 4 are larger than the sum of horizontal gradients in 2 and 3, then vertical splits that are located in quadrant 2 and 3 are not tested.

The same speed up technique is used in orthogonal directions with the vertical gradients.

Figure 40:  Quadrants used in gradient-based encoder speed-up.

- No further splits are tested if the skip mode is best mode and the symmetric 1/2 splits did not yield a smaller r-d cost.

- If a current best partitioning exists for a block, then asymmetric split modes are not tested if they yield subblocks smaller or equal to 32x32 that are not covered by the current best partitioning.

### Intra prediction mode estimation and type selection

For conventional spatial intra prediction modes, the same encoder control as in JEM is used. For the additionally introduced modes, relevant aspect of the encoder control as described below. Each of these intra mode is treated as an additional mode in the mode decision process.

Line-based intra coding

Every time the line-base intra coding mode is tested (all luma intra-predicted blocks), the encoder will make three decisions: the 1d split type, the luma intra prediction mode and whether the LIP mode is used for the CU or not. The best combination of these parameters is the one that minimizes the RD cost.

Finding these parameters will increase the encoding time significantly. Hence, to avoid an excessive amount of testing, various speed-up algorithms have been implemented.

First, the line-base intra coding mode is the last intra tool to be tested in the estimation loop. Therefore, the fast intra mode candidate list generated by the 2d DCT-II test mode will be available at the time the line-base intra coding mode is tested and can be reused accordingly.

The encoder will calculate the cost of all combinations of 1d split type and intra mode from the fast candidate list using the following procedure:

- Let Cmin be the minimal cost when the line-base intra coding mode is tested.

- Select a combination of 1d split type and intra mode from the fast candidates list to be tested

- The block is split into N lines and let i denote the index of each of these lines, where i=1,2,…, N.

- As shown in Figure 41 after every line is coded, its sub-cost Ji is calculated. Therefore, the encoder knows the sum of all the sub-costs that are available after the i-th line has been coded, which is Si=k=1iJk.

- After every partition is processed, the expression Si<Cmin is evaluated. If true, the encoder continues coding partitions until the end. Otherwise, it is guaranteed that this test mode is not going to yield a lower RD cost than Cmin, so the process is interrupted and we move on to the next combination of intra mode and split type.

- If all lines are coded, then the test mode is the best mode and Cmin is updated accordingly.

Figure 41:  Accumulation of the lines sub-costs to obtain the final cost of the whole block.

The aforementioned procedure does not have any drawbacks in terms of RD performance. On the other hand, the encoder can decide to skip testing certain intra modes of the list, one of two 1d split types or the whole LIP mode according to different parameters of previously tested modes.

Intra prediction using neural networks

When an intra search is conducted, first the best conventional intra prediction mode (DC-, Planar- or Angular-Prediction-Mode) with corresponding rate distortion (RD) costs is determined as in the HM/JEM encoder implementation. Then, the best proposed intra prediction mode with corresponding rate distortion (RD) costs is determined as described below. If these costs are lower than the costs of the best conventional intra prediction mode, the intraNN mode is chosen.

The following RD-search among the proposed intra prediction modes is proposed. First, the encoder computes the feature vector ftr as described in sec. 2.1.9.2. It is pointed out that this computation needs to be carried out only once since all different intraNN prediction modes use the same feature vector ftr. Second, the encoder computes the vector lgt as described in sec. 2.1.9.2 and thus the signalization costs for each intraNN prediction mode are known to the encoder. Out of the feature vector ftr, the encoder computes all supported intraNN prediction signals pred as described in sec. 2.1.9.2. For each of the intraNN prediction signals, the encoder computes costs that are set to be the weighted sum of the L1 norm of the Hadamard-Transform of the prediction residual and the signalization costs of the mode. The encoder ranks all proposed intra prediction modes according to the latter costs and performs a full RD search for the k best intraNN modes in that ranking. Here, the number k depends on the block size and has the same value as the number of modes for the full RD-search in the conventional intra search with Hadamard search. It is proposed that always the three modes for which the values lgtk are the largest are included in the list for the full RD search, which possibly augments k. This is analogous to the inclusion of the most probable modes (MPMs) into the list for the full RD search in the conventional intra search with Hadamard search.

The encoding process for intraNN is illustrated in Figure 42.

Figure 42:  Block diagram of encoder-side estimation process for intraNN (difference to decoder-side processing is highlighted in blue, predIdx represents the syntax element intra_nn_mpm_idx transmitted in the bitstream).

If the block is of size 32×32, the encoder conducts the above search both for the proposed modes corresponding to size 32×32 (intra_nn_subsampling_flag equal to 0) and for the proposed modes induced from blocks of size 16×16 (intra_nn_subsampling_flag equal to 1) as discussed in sec. 2.1.9.2 and chooses the best result among the two candidates.

Intra region-based template matching

The intra region-based template matching mode is evaluated as an additional intra coding mode. The rate-distortion costs are determined for all 5 supported values of intra_rtm_region_idx. That means, at the encoder, the displacement search is conducted for all five candidate regions (see sec. 2.1.9.3). The encoder selects the region (and thus the associated value of intra_rtm_region_idx) that yields the minimum rate-distortion cost.

For the case of the CUs in the picture boundary, only Region 1 is tested and other regions are skipped. Furthermore, in that case, the search algorithm is not invoked and the prediction signal is set to a DC value.

Intra region-based template matching modes are tested before the line-based intra coding modes. The combination of both is only tested if region-based template matching is the best intra mode for so far.

Multi-reference line intra prediction

The intra prediction mode search for MRL is conducted the same way as the search for conventional intra prediction. This includes a fast search that calculates and compares rate-distortion costs based on SATD distortion and mode signalling cost and a refinement that calculates and compares the full rate-distortion costs of the best N modes. This search is conducted for all allowed values of mrl_idx.

### Motion estimation and motion segment selection

#### General

The motion search proceeds as in HM and JEM. That means, in general, the same Lagrangian rate-distortion function is used as cost measure and the same fast motion estimation strategy (consisting of an integer motion search and the sub-sample refinement is used). Modifications are described in the following subsections.

#### Usage of decoder-side motion refinement

The same decoder-side motion refinement as in JEM is used.

#### Multi-Hypothesis Motion Estimation

First, the inter modes with no explicitly signaled additional inter prediction parameters are tested (e.g., inter with/without IMV/BIO/LIC, affine, merge, FRUC etc.).  For the best of these modes (i.e., the one having lowest RD cost), an additional inter prediction hypothesis is searched.  For that purpose, for all combinations of the following parameters, a motion estimation with a restricted search range of 16 is performed:

- Weighting factor for the additional prediction signal (here: 2 possibilities, namely 1/4 and -1/8)

- Used reference frame (here: 2 possibilities, namely first frame of list0 and list1)

For determining the best combination of these two parameters, a simplified RD cost using Hadamard distortion measure and approximated bit rate is used.  The chosen parameter combination is then used to compute a more accurate RD cost, using forward transform and quantization, which is compared against the so-far best found coding mode for the current block.  If the resulting RD cost is lower than the current best, an additional hypothesis is signaled for the current block.  Note that there is an upper limit for the number of additional hypotheses (here: 1), such that for blocks using MERGE mode, the resulting total number of merged and explicitly signaled additional hypotheses does not exceed this limit. E.g. in the case of this limit equal to 1, for MERGE blocks where the merging candidate already has one additional inter prediction hypothesis, no further additional hypothesis is tested.

#### Multi-directional boundary padding (MDBP)

Multi-directional boundary padding (MDBP) is used in the encoder, whenever the referenced pixel area is partially or entirely outside the reference picture area (for details, see MDBP description above). In order to reduce the computational complexity of MDBP during motion estimation, the best MDBP mode is only estimated once, after the sub-pel motion estimation has been finished. During full-pel and sub-pel motion estimation, the perpendicular frame border padding already available is applied.

#### RD cost based motion vector re-estimation

If the best coding mode found for a CU is non-affine inter prediction, an RD cost based re-estimation of the motion vectors is performed.  The re-estimation is performed in two rounds, each iterating over all explicitly signaled (i.e., non-merge) inter prediction hypotheses.  For each inter prediction hypothesis, having motion vector (dx,dy), all motion vectors (dx’,dy’) with |dx’-dx|+|dy’-dy| <= 1 (for integer-pel motion vector accuracy: |dx’-dx|+|dy’-dy| <= 4; for 4-pel motion vector accuracy: |dx’-dx|+|dy’-dy| <= 16) are evaluated by calculating the actual RD cost (after forward transform and quantization).  The motion vector resulting in the lowest RD cost is used for this prediction hypothesis.  If no improvement in RD performance has been achieved in the first round, the second round is omitted.

### Mode decisions

The mode decision is the same as in HM and JEM. The coding mode for a block is selected based on the Lagrangian rate-distortion costs.

### In-loop filtering type selection and parameter estimation

The proposed codec includes three different loop filters: deblocking filter, SAO and a multiple feature-based classifications adaptive loop filter (MCALF). While the deblocking filter and SAO are the same as those in HEVC, MCALF is additionally applied.

MCALF extends the classification for the luma component in GALF in the sense that instead of applying one specific classifier DG for GALF, multiple classifiers are tested. The classifier that yields the smallest rate-distortion cost is selected in the encoder. MCALF follows the same procedure of GALF at the encoder except for this classification process for luma component.

The MCALF provides three classifiers (see sec. 2.1.10), which are referred to as Cl1,Cl2,Cl3 in the following.  In the encoder, for each video picture, all three classifiers are applied (see Figure 43). That means, for each classifier

- The samples are classified according to the rule given by the classifier;

- The filters are determined;

- The video pictures are filtered and both the distortion and the bit rate for transmitting the filter parameters are determined.

Finally, the classifier that yields the smallest rate-distortion cost is selected. For this, the determination of filter coefficients is identical to the GALF implementation in the JEM software.

Figure 43:  Selection procedure based on RD cost: The chosen classifier Clι is the one that yields the smallest rate-distortion cost among the candidates Cl1,…,Cl3..

### Transforms and transform type selection

The transform type decision for an inter-coded CU is identical to JEM.

For an intra-coded CU, a pre-selection of available modes is performed to reduce the number of RD checks. This pre-selection or rough mode decision is based on SATD cost. Each remaining intra mode is tested with the 5 transform candidates and the RD cost are checked. The candidate list is constructed as described in sec. 2.1.7.

### Quantization and quantization type selection

For all transform blocks, the concept of dependent scalar quantization is used.  Quantization weighting matrices are not used, i.e., the same quantization step size is used for all transform coefficients of a transform block.

Similar to the rate-distortion optimized quantization (RDOQ) implemented in the HM and JEM software, the transform coefficient levels {qk} for a transform block are determined in a way that the Lagrangian rate-distortion cost

J=D+λ⋅R=ktk-tk'qk | qk-1,qk-2,⋯2+λ⋅Rkqk  qk-1,qk-2,⋯)

is minimized.  In the above formula, tk and qk represent the original transform coefficients (obtained as result of the forward transform) and the transform coefficient levels, respectively.  tk'(qk |⋯) denotes the reconstructed transform coefficient that is obtained by choosing the transform coefficient level qk, Rkqk ⋯) specifies the estimated number of bits that is required for transmitting the transform coefficient level qk, and λ is the Lagrange multiplier.  The conditional expressions qk  qk-1,qk-2,⋯) illustrate that both the reconstruction of a particular transform coefficient tk' (due the dependent quantization) and the number of bits Rk required for transmitting a transform coefficient level qk (due to the entropy coding) do not only depend on the associated transform coefficient level qk, but also on the transform coefficient levels qk-1,qk-2,⋯ that precede qk in coding order.

As described in sec. 2.1.6, with dependent scalar quantization, the scalar quantizer used and, thus, the admissible reconstruction values for a transform coefficient depend on the parities of the transform coefficient levels that precede a current transform coefficient in scanning order.  Hence, in comparison to RDOQ in the HM/JEM software, the switching between the scalar quantizers Q0 and Q1 must be additionally taken into account in the quantization process.

In sec. 2.1.6, the switching between the two scalar quantizers Q0 and Q1 is specified using a state machine. As illustrated in Figure 44, the dependencies between the chosen transform coefficient levels and the scalar quantizers used can be described using a trellis structure.  Each node in the trellis represents a potential state (0, 1, 2, or 3; see sec. 2.1.6) for a particular transform coefficient.  The state uniquely determines the quantizer used (Q0 is used for states 0 and 1; Q1 is used for states 2 and 3).  Each node/state is connected with two states for the next transform coefficient in coding order.  Given a certain state for a current transform coefficient tk, the parity of the associated transform coefficient level qk determines the state (and, thus, the quantizer) for the next transform coefficient in coding order. Hence, the set of available transform coefficient levels for a quantizer is basically partitioned into two subsets: The first subset includes the even transform coefficient levels (parity 0) and the second subset includes the odd transform coefficient levels (parity 1). In Figure 11 and Figure 44, the subsets are denoted by A (Q0 with parity 0), B (Q0 with parity 1), C (Q1 with parity 0), and D (Q1 with parity 1).

Figure 44:  Trellis structure representing the dependencies between transform coefficient levels (coding order from left to right).

If we label all connections between two directly connected trellis nodes with the associated rate-distortion costs Jkqk=tk-tk'(qk|⋯)2+λ⋅Rk(qk|⋯), the determination of transform coefficient levels qk that minimize the overall rate-distortion cost is equivalent to finding the path with minimum cost through the trellis.  This minimization problem can be solved using the Viterbi algorithm.

The quantization algorithm used consist of two steps:

- Determination of four candidate transform coefficient levels for each transform coefficient (one for each of the subsets A, B, C, D).

- Viterbi algorithm for determining the sequence of transform coefficient levels qk that minimizes the estimated overall rate-distortion cost (among the pre-determined candidate levels).

Figure 45:  Illustration of the pre-selection of candidate transform coefficient levels.  For both quantizers, the two closest candidates are selected. The two candidate transform coefficient levels for each quantizer differ in their parity.

The determination of candidate transform coefficient levels proceeds as follows. For each original transform coefficient tk and each of the two quantizers Q0 and Q1, the two transform coefficient levels that represent the two closest reconstruction values are selected as candidates. This pre-selection of candidate levels is illustrated in Figure 45. Note that the two candidates for each quantizer differ in their parity. In total, 4 candidate levels are determined (one for each of the subsets A, B, C, D).

Figure 46:  Trellis structure for an example of 8 transform coefficients. The first state (left) represents the initial state, which is set equal to 0.

The Viterbi algorithm is used for determining the sequence of transform coefficient levels (among all possible combinations of pre-selected candidate levels) that minimizes the estimated overall rate-distortion cost. It consists of the following main steps (see Figure 46):

- Set the rate-distortion cost for the initial state equal to 0.

- Process the transform coefficients tk in coding order (i.e., in increasing order of k). For each possible state (0-3) for a transform coefficient tk, do the following

  - Calculate the rate-distortion costs for the two paths that connect a state for the preceding transform coefficient with the current state. The costs are given as the sum of the cost for the preceding state and the cost Jkqk=Dk(qk)+λ⋅Rk(qk), where Dk(qk) and Rk(qk) represent the distortion and rate for choosing the quantization index qk of the subset (A, B, C, or D) that is associated with the considered connection.

  - Assign the minimum of the calculated costs to the current node and prune the connection to the state of the previous transform coefficient that does not represent the path with the minimum rate-distortion cost.

After this step, all four trellis nodes for a current transform coefficient have a single connection to a node for the preceding transform coefficient in coding order.

- Compare the costs of the 4 final nodes (for the last coefficient in coding order) and chose the node with minimum cost. Note that this node is associated with a unique path through the trellis (all other connection were pruned in the previous steps).

- Follow the chosen path (specified by the final node) is reverse order and collect the quantization indices qk that are associated with the connections between the trellis nodes

### Entropy coding

The encoder uses the same probability estimator and coding engine as specified in the decoder’s entropy coding description in sec. 2.1.3. During mode decision, the resolution of the bitrate estimator is identical to that of JEM.

At the encoder side, the same context modeling as at the decoder side (see sec. 2.1.4) is used. The mapping from syntax elements to binary decisions is the inverse of the mapping from binary decisions to syntax elements specified in sec. 2.1.4.

### Additional tools

#### Diffusion filter

Prediction modes with diffusion filter enabled are treated as separate coding modes in the mode decision process.

#### DCT Thresholding

When enabled by SPS parameters, DCT thresholding can be applied for intra- and/or inter-predicted coding units. The number of supported extension sizes and the number of available thresholds is also specified by SPS syntax elements.

When DCT thresholding is enabled by SPS syntax elements, it is evaluated as an additional coding mode in the mode decision process, with the following conditions:

- DCT thresholding is not supported if both the width and the height of the CU is less than 16.

- For inter-coded CUs, thresholding is only enabled if coded block flag is equal to 1.

- For the merge mode, the encoder evaluates DCT thresholding only for the best merge candidate and only if the coded block flag is equal to 1.

Given an initial luma prediction signal (intra or inter prediction signal), the DCT thresholding parameters thresholding_size and thresholding_threshold are determined as follows.

The number of available extension sizes (values of thresholding_size) and thresholds (values of thresholding_threshold) is indicated in the SPS. For each supported value of thresholding_size, the following applies:

- The K×L region is transformed using a 2d DCT. The values of K and L are determined as described in sec. 2.1.11.2.

- The absolute values of the resulting DCT coefficients are quantized and sorted in ascending order, where duplicated entries are removed. The resulting ordered array of absolute quantization indices is referred to as qIdx[ ].

- For all potential values of thresholding_threshold, the following applies (values that exceed the size of qIdx[ ] are not tested):

  - The threshold t is set according to

t=Δ⋅qIdx[ thresholding_threshold ]

  - As described in sec. 2.1.11.2, the DCT coefficients are thresholded using the threshold t, the resulting coefficients are inverse transformed, and the prediction signal is formed by the M×N region of the inverse transform data that corresponds to the luma signal of the current CU.

  - Given the modified prediction signal, the rate-distortion cost J=D+λ⋅R is calculated.

Finally, the parameters that yield the smallest rate-distortion cost J=D+λ⋅R are selected. If the minimum rate-distortion cost for the application of DCT thresholding is small than the rate-distortion cost for the initial prediction signal, thresholding_flag is set equal to 1.

### Additional encoder optimization

We submitted two sets of bitstreams. For the first set, the QP was held constant inside a frame or was adapted in the same way as for the anchor (HDR sequences only). However, for the second set of bitstreams, we use the perceptually optimized QP adaptation described below.

#### Perceptually optimized QP adaptation (QPA)

The perceptually optimized QP adaptation described in the following represents an improved version of the approach presented in JVET-H0047 [13].

For non-reference pictures and picture that have a slice QP of 51, all QP values are set equal to the slice QP.  For reference picture with a slice QP less than 51, the following approach is used.

Let sx,y, with 0≤x<W and 0≤y<H, represent the original luma samples of a video picture, with width W and height H.  A high-pass filtered version of the picture’s luma component, for 0<x<W-1 and 0<y<H-1, is derived according to

hx,y=4⋅sx,y-sx-1,y-sx+1,y-sx,y-1-sx,y+1.

The quantization parameters QPk for a CTU, given by the luma sample block Bk, is derived as follows:

- A local activity measure ak is derived according to

ak=max1; 1|Bk*|x,y∈Bk*h(x,y)2,

where Bk* represents the considered block Bk without the border samples (with  x=0, y=0, x=W-1, or y=H-1) for which we do not have high-pass samples h(x,y).  The term |Bk*| specifies the number of samples in the block Bk*.

- The CTU quantization parameter QPk is calculated according to

QPk=Clip30;51;QPslice+round2∙log2akapic,

The value of apic is given by

apic≈ 2∙2Bfor full HD and smaller sequences, 2Botherwise,

where B denotes the bit depth of the luma component.

- If the video sequence is a HDR sequence, the CTU quantization parameter is modified according to

QPk=Clip30;51;QPk+1-3Jk2∙21-2B

with Jk being the mean luma sample value of the CTU,

Jk=1Bk∙x,y∈Bksx,y.

Note that the additional QP offset represents the Sharp DeltaQP implemented in JEM (see Figure 47).

- Compute the mean absolute error between the CTU’s luma signal and its mean value

I=max1; 1Bk∙[x,y]∈Bksx,y-Jk,

and do the following:

  - If I<α∙qk, then set

QPk=QPk-minround2 log2α∙qkI,3,

where the value of 3 is a hand-tuned parameter and qk is the quantization step size for the quantization parameter QPk (which also depends on the bit depth). α is a constant factor.

  - Otherwise, set QPk=QPk.

- The final CTU quantization parameter QPk is determined by applying a smoothing filter across the CTU-wise QPk values. The filter is specified by

QPk=fQPk=maxQPk,QPk',  QPk'=minQPk-stride,QPk-1,QPk+1,

where QPk' takes the minimum of the spatially above, left, and right-hand neighboring QP values of location k. If a neighboring QP value is unavailable due to an image boundary, it is assumed to be equal to infinity. It is worth noting that f(∙) is applied in-place to obtain the final QPk once QPk+1 has been determined, i.e., the outcome for k depends on the outcome for k-1, k-2, …, k-stride.

Figure 47:  QP offset values (blue curve) for HDR sequences with a bit depth of 10 bits.

Given the CTU quantization parameters QPk, the slice quantization parameter QPslice, and the nominal Lagrange parameter for the slice λslice, the Lagrange parameter for a CTU is derived according to

λk=λslice∙2QPk-QPslice3.

The Lagrange parameter λk is used for all rate-distortion cost calculations for the corresponding CTU.

When the perceptual optimized QP adaptation is used and all of the following conditions are true, the CTU size is reduced from from 128×128 to 64×64:

- The given base QP (quantization parameter specified in the configuration file, as in JEM) is smaller than 42;

- The spatial resolution is smaller than or equal to 2048×1280 luma samples.

### Additional algorithmic description topic(s)

The encoding for generating the submitted bitstreams were done using thread parallel encoding as described in the following. The actually used encoder configurations are summarized in the accompanying Excel documents.

#### Thread parallel encoding

The encoder software includes two different parallelization approaches, which can be used individually or in combination. The number of encoder threads can be configured at runtime. For convenience, OpenMP was used as multi-platform API to implement parallel processing.

Wavefront parallel encoding

To parallelize the encoder on CTU-level a dynamic wavefront approach was chosen that uses an explicit synchronization, where each CTU-line is processed by a single thread, as shown in Figure 48. The parallel processing is limited to the encoder decision path. The entropy coding of all syntax elements is done after the coding parameters have been determined for all CTUs of a slice. The independently operating threads require synchronization to prevent overtaking of the superior thread by the dependent one. The synchronization respects a minimum delay of two CTUs between consecutive CTU-lines to guarantee the availability of the left, above-left, above and above-right CTU-neighbors for prediction of samples and motion parameters. Beside the additional code for synchronization objects and thread scheduling the existing software requires only marginal adaption, and the overhead of the synchronization is negligible.

Figure 48:  Illustration of wavefront parallel encoding.

To enforce deterministic behaviour for the WPP mode search, CABAC context states are initialized at the beginning of each CTU-line in the mode decision path. This initialization scheme is used even in single thread mode, but can be disable at runtime for the latter case. Furthermore, to support content based context adaptation, a CTU linewise buffering of CABAC context states is implemented (see sec. 2.1.3.2), where, if available, the buffered CABAC context states from a previous picture replace the CABAC context states adapted in the current picture, at the beginning of each CTU-line.

Parallel testing of partitioning options

Another implemented parallel approach computes various recursive split option for a particular partition in parallel. Compared to non-split encoding modes, recursive splits have longer execution times and are thus more suited for parallelization. To prevent threads from idling, the thread load has been balanced by selecting an appropriate operation point, which results in similar execution time for the parallel tasks – e.g. different types of splits and the non-split modes. The best operation point to start a parallel split for an unlimited tree partition is at a block size of 16x16 samples, whereas limited BT-splits can always be started with parallel search. This approach requires adaptions in the software. Buffers need to be allocated to mirror the reconstructed samples buffer for each thread. Methods that distribute the current coding state among the threads and collects the data afterwards have to be incorporated.

## 360°

### Usage of projection format adaptation

The projection format used is described in sec. 2.2.1.  Beyond that, no projection format adaptation is used.

### 360° Video specific encoding tools

No 360° video specific encoding tools are used.

### Additional encoder optimization

No additional encoder optimization was used for 360° video sequences.

## HDR

### Pre-processing

No preprocessing is applied.

### HDR specific encoding tools

No HDR specific encoding tools are used.

### Additional encoder optimization

No additional encoder optimization was used for HDR sequences.

# Compression performance

## SDR: Constraint set 1 configuration relative to HM anchor

### Class SDR-A (RD-curves)

The following diagrams show comparisons of the rd-curves obtained by decoding the submitted bitstreams for the category SDR-A (constraint set 1) and the corresponding HM anchor bitstreams. In the diagrams, the average Y-PSNR is plotted over the average bit rate. The diagrams show rd-curves for both sets of submitted bitstreams. It should, however, be noted that one set of bitstreams (labeled “QPA” in the diagrams) was encoded using the perceptually-optimized QP adaptation (QPA) described in sec. 3.1.11.1.  The QP adaptation reduces the PSNR, but often improves perceptual video quality.

### Class SDR-B (RD-curves)

The following diagrams show comparisons of the rd-curves obtained by decoding the submitted bitstreams for the category SDR-B (constraint set 1) and the corresponding HM anchor bitstreams. In the diagrams, the average Y-PSNR is plotted over the average bit rate. The diagrams show rd-curves for both sets of submitted bitstreams. It should, however, be noted that one set of bitstreams (labeled “QPA” in the diagrams) was encoded using the perceptually-optimized QP adaptation (QPA) described in sec. 3.1.11.1.  The QP adaptation reduces the PSNR, but often improves perceptual video quality.

### Overall (BD-rate summary tables)

The following table summarized the measured BD-rate savings for SDR constraint set 1 over the HM anchor bitstreams. The table does not include the BD-rate savings for the set of bitstreams that was generated using the perceptually-optimized QP adaptation (sec. 3.1.11.1), since these bitstreams were not optimized with respect of PSNR (the results are given in the accompanying Excel documents).

## SDR: Constraint set 2 configuration relative to HM anchor

### Class SDR-B

The following diagrams show comparisons of the rd-curves obtained by decoding the submitted bitstreams for the category SDR-B (constraint set 2) and the corresponding HM anchor bitstreams. In the diagrams, the average Y-PSNR is plotted over the average bit rate. The diagrams show rd-curves for both sets of submitted bitstreams. It should, however, be noted that one set of bitstreams (labeled “QPA” in the diagrams) was encoded using the perceptually-optimized QP adaptation (QPA) described in sec. 3.1.11.1.  The QP adaptation reduces the PSNR, but often improves perceptual video quality.

### Overall

The following table summarized the measured BD-rate savings for SDR constraint set 2 over the HM anchor bitstreams. The table does not include the BD-rate savings for the set of bitstreams that was generated using the perceptually-optimized QP adaptation (sec. 3.1.11.1), since these bitstreams were not optimized with respect of PSNR (the results are given in the accompanying Excel documents).

## SDR: Constraint set 1 configuration relative to JEM anchor

### Class SDR-A

The following diagrams show comparisons of the rd-curves obtained by decoding the submitted bitstreams for the category SDR-A (constraint set 1) and the corresponding JEM anchor bitstreams. In the diagrams, the average Y-PSNR is plotted over the average bit rate. The diagrams show rd-curves for both sets of submitted bitstreams. It should, however, be noted that one set of bitstreams (labeled “QPA” in the diagrams) was encoded using the perceptually-optimized QP adaptation (QPA) described in sec. 3.1.11.1.  The QP adaptation reduces the PSNR, but often improves perceptual video quality.

### Class SDR-B

The following diagrams show comparisons of the rd-curves obtained by decoding the submitted bitstreams for the category SDR-B (constraint set 1) and the corresponding JEM anchor bitstreams. In the diagrams, the average Y-PSNR is plotted over the average bit rate. The diagrams show rd-curves for both sets of submitted bitstreams. It should, however, be noted that one set of bitstreams (labeled “QPA” in the diagrams) was encoded using the perceptually-optimized QP adaptation (QPA) described in sec. 3.1.11.1.  The QP adaptation reduces the PSNR, but often improves perceptual video quality.

### Overall

The following table summarized the measured BD-rate savings for SDR constraint set 1 over the JEM anchor bitstreams. The table does not include the BD-rate savings for the set of bitstreams that was generated using the perceptually-optimized QP adaptation (sec. 3.1.11.1), since these bitstreams were not optimized with respect of PSNR (the results are given in the accompanying Excel documents).

## SDR: Constraint set 2 configuration relative to JEM anchor

### Class SDR-B

The following diagrams show comparisons of the rd-curves obtained by decoding the submitted bitstreams for the category SDR-B (constraint set 2) and the corresponding JEM anchor bitstreams. In the diagrams, the average Y-PSNR is plotted over the average bit rate. The diagrams show rd-curves for both sets of submitted bitstreams. It should, however, be noted that one set of bitstreams (labeled “QPA” in the diagrams) was encoded using the perceptually-optimized QP adaptation (QPA) described in sec. 3.1.11.1.  The QP adaptation reduces the PSNR, but often improves perceptual video quality.

### Overall

The following table summarized the measured BD-rate savings for SDR constraint set 2 over the JEM anchor bitstreams. The table does not include the BD-rate savings for the set of bitstreams that was generated using the perceptually-optimized QP adaptation (sec. 3.1.11.1), since these bitstreams were not optimized with respect of PSNR (the results are given in the accompanying Excel documents).

## HDR: Constraint set 1 configuration relative to HM anchor

### Class HDR-A

The following diagrams show comparisons of the rd-curves obtained by decoding the submitted bitstreams for the category HDR-A (constraint set 1) and the corresponding HM anchor bitstreams. In the diagrams, the average Y-PSNR is plotted over the average bit rate. The diagrams show rd-curves for both sets of submitted bitstreams. It should, however, be noted that one set of bitstreams (labeled “QPA” in the diagrams) was encoded using the perceptually-optimized QP adaptation (QPA) described in sec. 3.1.11.1.  The QP adaptation reduces the PSNR, but often improves perceptual video quality.

### Class HDR-B

The following diagrams show comparisons of the rd-curves obtained by decoding the submitted bitstreams for the category HDR-B (constraint set 1) and the corresponding HM anchor bitstreams. In the diagrams, the average Y-PSNR is plotted over the average bit rate. The diagrams show rd-curves for both sets of submitted bitstreams. It should, however, be noted that one set of bitstreams (labeled “QPA” in the diagrams) was encoded using the perceptually-optimized QP adaptation (QPA) described in sec. 3.1.11.1.  The QP adaptation reduces the PSNR, but often improves perceptual video quality.

### Overall

The following table summarized the measured BD-rate savings for HDR over the HM anchor bitstreams. The table does not include the BD-rate savings for the set of bitstreams that was generated using the perceptually-optimized QP adaptation (sec. 3.1.11.1), since these bitstreams were not optimized with respect of PSNR (the results are given in the accompanying Excel documents).

## HDR: Constraint set 1 configuration relative to JEM anchor

### Class HDR-A

The following diagrams show comparisons of the rd-curves obtained by decoding the submitted bitstreams for the category HDR-A (constraint set 1) and the corresponding JEM anchor bitstreams. In the diagrams, the average Y-PSNR is plotted over the average bit rate. The diagrams show rd-curves for both sets of submitted bitstreams. It should, however, be noted that one set of bitstreams (labeled “QPA” in the diagrams) was encoded using the perceptually-optimized QP adaptation (QPA) described in sec. 3.1.11.1.  The QP adaptation reduces the PSNR, but often improves perceptual video quality.

### Class HDR-B

The following diagrams show comparisons of the rd-curves obtained by decoding the submitted bitstreams for the category HDR-B (constraint set 1) and the corresponding JEM anchor bitstreams. In the diagrams, the average Y-PSNR is plotted over the average bit rate. The diagrams show rd-curves for both sets of submitted bitstreams. It should, however, be noted that one set of bitstreams (labeled “QPA” in the diagrams) was encoded using the perceptually-optimized QP adaptation (QPA) described in sec. 3.1.11.1.  The QP adaptation reduces the PSNR, but often improves perceptual video quality.

### Overall

The following table summarized the measured BD-rate savings for HDR over the JEM anchor bitstreams. The table does not include the BD-rate savings for the set of bitstreams that was generated using the perceptually-optimized QP adaptation (sec. 3.1.11.1), since these bitstreams were not optimized with respect of PSNR (the results are given in the accompanying Excel documents).

## 360° Video: Constraint set 1 configuration relative to HM anchor

### Class 360

The following diagrams show comparisons of the rd-curves obtained by decoding the submitted bitstreams for the category 360 video (constraint set 1) and the corresponding HM anchor bitstreams. In the diagrams, the average end-to-end WS-Y-PSNR is plotted over the average bit rate. The diagrams show rd-curves for both sets of submitted bitstreams. It should, however, be noted that one set of bitstreams (labeled “QPA” in the diagrams) was encoded using the perceptually-optimized QP adaptation (QPA) described in sec. 3.1.11.1.  The QP adaptation reduces the PSNR, but often improves perceptual video quality.

### Overall

The following table summarized the measured BD-rate savings for 360 video over the HM anchor bitstreams. The table does not include the BD-rate savings for the set of bitstreams that was generated using the perceptually-optimized QP adaptation (sec. 3.1.11.1), since these bitstreams were not optimized with respect of PSNR (the results are given in the accompanying Excel documents).

## 360° Video: Constraint set 1 configuration relative to JEM anchor

### Class 360

The following diagrams show comparisons of the rd-curves obtained by decoding the submitted bitstreams for the category 360 video (constraint set 1) and the corresponding JEM anchor bitstreams. In the diagrams, the average end-to-end WS-Y-PSNR is plotted over the average bit rate. The diagrams show rd-curves for both sets of submitted bitstreams. It should, however, be noted that one set of bitstreams (labeled “QPA” in the diagrams) was encoded using the perceptually-optimized QP adaptation (QPA) described in sec. 3.1.11.1.  The QP adaptation reduces the PSNR, but often improves perceptual video quality.

### Overall

The following table summarized the measured BD-rate savings for 360 video over the JEM anchor bitstreams. The table does not include the BD-rate savings for the set of bitstreams that was generated using the perceptually-optimized QP adaptation (sec. 3.1.11.1), since these bitstreams were not optimized with respect of PSNR (the results are given in the accompanying Excel documents).

# Decoder complexity analysis

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

The following tables summarized the decoding times of the proposed algorithm relative to the HM and JEM anchors. All decoding times were measured on the same hardware platform with YUV output enabled (and reference input disabled). Details for all test sequences and rate points are included in the accompanying Excel documents.

## Description of computing platform used to determine decoding times reported in section 5.1

The computing platform for the decoding time measurement was the following:

## Memory usage of decoder

The memory usage of the decoder has been analyzed for the highest rate point and the largest resolution for the different classes. The measured results are summarized in Table 13.

Table 13:  Summary of decoder memory usage

## Complexity characteristics of decoder entropy decoding operation

Arithmetic coding engine

The probability estimation stage uses a total of 38 bits per context model, which are made up of 2×15 bits for the counter variables and 2×4 bits for the window size variables. Additionally, 3×8 bits of read-only memory are required for storing the window size initialization values. The arithmetic coding engine uses a table containing 32×8×8 bits values and therefore requires less memory than JEM.

Context Modeling

The coding of transform coefficient levels in a single stage leads to advantages and disadvantages, both however depending on the implementation architecture. The coding in multiple scanning passes approach in HEVC has the drawback that it requires more memory access than the used configuration. Conversely, the usage of the local template requires a more sophisticated state prediction for hardware implementations. However, it is possible since the coding order and the occurrence of the levels are known. Especially due to the coupling of the context sets to the diagonal, only the context index within the context model set used has to be predicted. Another advantage is that the context model indices and the Rice parameter are derived once for each scanning position resulting in less logic. Overall, it is assumed that the throughput performance is similar to the existing scheme in HEVC.

## Complexity characteristics of decoder inverse quantization

In comparison to HEVC and JEM, the complexity of inverse quantization is increased. Even though the basic scaling of transform coefficients is very similar to HEVC/JEM, some additional arithmetic operations (1 bit shift, 1 subtraction, 1 sign operation) are required for non-zero transform coefficient levels. In addition, the transform coefficients have to be constructed in scanning order and a state transition has to be done per coefficient (starting with the first non-zero coefficient). The state transition is implemented using a 2x2x2 bit table, but it can also be implemented using integer arithmetic operation. More details in relation to the inverse quantization process of HEVC are given in the pseudo code in Figure 13.

## Complexity characteristics of decoder inverse transform operation

The set of transforms include implementations of different types. The transforms defined by EMTs and NSST have the same implementation and hence complexity as in JEM. The additional introduced non-separable transforms for non-quadratic transform blocks and the transforms trained for the intraNN modes are implemented using full matrix multiplication. Their maximal transform size is 64. For transform blocks with a higher dimension (W*H), these transforms are applied to the low-frequency part of the coefficients only. The inverse DCT-II transform is then applied to the whole set of coefficients in order to obtain the residual signal in spatial domain.

## Complexity characteristics of decoder-side motion refinement

The complexity of decoder-side motion estimation is the same as in JEM. The following two JEM tools use decode-side motion estimation:

- Pattern matched motion vector derivation (FRUC)

- Decoder-side motion vector refinement (DMVR)

In addition, the video codec includes a tool that uses decoder-side template matching for intra-picture prediction (see sec. 5.9).

## Complexity characteristics of decoder motion compensation

The motion vector accuracy and the interpolation filters used for motion compensation are the same as in JEM.  However, the inter prediction signal may be a weighted superposition of up to three individual motion-compensated prediction signals (compared to up to two in JEM and HEVC).

In addition, for blocks that refer to samples outside the picture area, template matching is used for determining the spatial prediction direction (see sec. 2.1.8.3). For each candidate prediction direction, the SAD has to be calculated for two rows or two columns of samples.

## Complexity characteristics of decoder intra-frame prediction operation

Conventional intra-frame prediction has the same implementation and hence complexity as in JEM except for the bilateral filter that replaces the strong reference sample filter and when multiple-reference line intra prediction is used.

According to the description in section 2.1.9.4, the bilateral filter consists of 2*s+1 taps for each filtered reference sample. Each tap uses the sample value difference ΔI for a table lookup of the weight and updating the two sums. The filtered reference sample value results from dividing the two sums in a last step.

When multiple-reference line intra prediction is used (intra_mrl_idx is larger than 0), the number of operations is exactly the same as for conventional intra-prediction. Memory-wise, two additional lines of reconstructed samples have to be stored. While conventional intra prediction requires 2*(W+H)+1 reconstructed reference samples, MRL requires 2*(W+H)+4*mrl_idx+1 reconstructed reference samples as described in sec. 2.1.9.5.

Line-based intra coding

The decoder complexity is line-based intra coding is considered to be smaller than that of conventional spatial intra coding in HEVC/JEM. The complexity of spatial prediction is basically the same as for conventional intra prediction. Transform coding is applied to individual rows or columns of samples, hence only 1d transform are applied.

Intra prediction using neural networks

Intra prediction using neural networks requires several matrix multiplications and table look-ups. The maximum block size for application of intraNN is 16x16 (larger blocks are downsampled before applying the neural network). For 16x16 blocks, the first three layers of the neural network consist of a matrix multiplication with a 68x68 matrix and element-wise look-up tables. The last layer is given by a 256x256 matrix multiplication. For the derivation of the intraNN mode, another neural network with two layers is used. Here, for a 16x16 block, the first layer consists of multiplication with a 68x68 matrix and element–wise table look-ups. The second layer consists of a matrix multiplication with a 68x35 matrix.

For smaller block sizes, the complexity is reduced correspondingly.  A modification of the tool that applies the prediction in the transform domain and requires less operations is presented in [6].

Intra region-based template matching

For intra template matching, the area of reconstructed samples before in-loop filtering to be stored per block depend on the region sizes A1, A2, A3 as defined in sec. 2.1.9.3 and the size of the prediction block W x H. This results in (A1+A2+A3 + W)*H samples to the left and (A1+A2+A3 + H)*(A1+A2+A3 + H+ W samples above and above right of the current prediction block that are required to be stored.

For the template matching search, the number of SSD calculations between the template of the current block Tc. and the reference template Tr. in the search region depend on the template size and region size. The number of samples in the template depend on the prediction block size and is given by 2*H + 2*(W+2).

For the final prediction, sums and shifts are required to average the best three W x H predictions according to the final prediction calculation given in sec. 2.1.9.3.

## Complexity characteristics of decoder in-loop filtering operation

The complexity of in-loop filtering is very similar to JEM. The following filters are applied: bilateral filter, deblocking filter, SAO, MCALF. While the first three filters are identical to JEM, the last filter (MCALF) represents in extension of the GALF in JEM. The difference is that the MCALF supports three classification options. The two added classifiers have a smaller complexity than the GALF classifier. The actual filtering is identical to that in GALF (JEM).

## Complexity characteristics of additional decoder tools

Diffusion filter

The diffusion filter represents in iterative filter that is applied to intra or inter prediction signals. It has a linear and a non-linear mode. In the linear mode, in each iteration, a sample is replaced with the average of its four direct neighbors. The number of iterations is either 5, 15, or 35 for the linear mode. In the non-linear mode, an individual 3x3 filter mask is derived for each sample in the prediction block. The derivation of the filter masks is calculated using sample differences, sample averages, and table look-ups (see sec. 2.1.11.1) for details. Once the filter masks are determined, in each iteration step, each sample is replaced with a weighted average of the 9 neighboring samples. The number of iterations is either 4, 5, 8, or 20 in the non-linear mode.

DCT thresholding

The DCT thresholding consists of the following operations: A separable DCT-II of an extended prediction signal (which additionally includes reconstructed border samples), a thresholding operation (comparison of the resulting transform coefficients to a threshold), and an inverse DCT-II. The additional complexity is mainly given by the two additional transforms, which represent separable DCTs.

## Complexity characteristics of 360° video specific decoding tools

The video codec does not include any 360° video specific coding tools.

## Complexity characteristics of HDR specific decoding tools

The video codec does not include any HDR specific coding tools.

## Degree of capability for decoder parallel processing

The coding tool related capabilities and restrictions for parallel processing are comparable to those of JEM. The software does not support thread parallel decoding. However, the encoder software does already support thread parallel encoding (wavefront encoding, see 3.1.12.1). Hence, an implementation of wavefront parallel decoding would also be possible.

# Encoder complexity analysis

## Encoding time and measurement methodology

For the actual encodings of the submitted bitstreams, we used thread parallel encoding as described in sec. 3.1.12.1.  Depending on the expected run time for the individual bitstreams (which depends on both the resolution and the bit rate), different configurations with 2 to 24 parallel thread were used.  The accompanying Excel documents contain a sheet, in which the configuration used is specified for each individual bitstream.

In order to report encoding times that can be compared to the single thread encoding times for HM and JEM, we determined scaling factors that represent the ratio between multi- and single-thread encoding times. The scaling factors were determined using a representative number of frames (not the full sequences). Different scaling factors were determined for different pictures resolutions, constraint sets, and thread configurations. The scaling factors used are given in the Excel documents.

In the following tables, we report the encoding times compared to the HM and JEM reference encodings. The tables include two configurations:

- In the columns “thread parallel encoding”, the ratio of the actual encoding times is reported.  At noted above, different thread parallel configurations were used for the individual bitstreams.

- In the columns “estimated single thread encoding”, the determined scaling factors were used for estimating single thread encoder runs times.

## Description of computing platform used to determine encoding times reported in section 6.1

The computing platform used to determine encoding times was:

## Memory usage of encoder

The memory usage of the encoder has been analyzed for the highest rate point and the largest resolution for the different classes. The measured results are summarized in Table 14.  The tables shows the memory consumption for two encoder configurations. The middle column shows the memory consumption for single thread encoding. The right column shows the memory consumption for an encoding with 16 threads. In this configuration 4 WPP threads and, for each of the WPP threads, 4 split mode threads are used (see sec. 3.1.12.1)

Table 14:  Summary of encoder memory usage for two encoder configurations.

## Complexity characteristics of encoder partitioning

The block partitioning used (see sec. 2.1.2) provides 22 different split modes (1/2, 1/4, 3/4, 3/8, 5/8, 1/3, 2/3, 1/5, 2/5, 3/5, 4/5 in horizontal and vertical direction). However, the number of actually possible split modes is significantly smaller due to syntax constraints (for example, constraints on resulting block sizes and prohibitions of redundant split patterns). Moreover, several encoder rules (see sec. 3.1.2) are implemented for speeding up the decision process.

For a fixed maximum binary tree depth, the most complex option is the use of the asymmetric 3/8 and 5/8 splits. Not using these splits also erases the splits 1/5, 2/5, 3/5, and 4/5. Almost 25% time reductions can be obtained if only 1/4, 3/4, 1/3, and 2/3 splits are used in addition to the 1/2 split.

Furthermore, by limiting certain types of asymmetric split modes lead to the non appearance of blocks with certain non-power of 2 sides and hence has other side effects for the complexity, e.g. less matrix multiplications with the primary transforms for certain non power of 2 sizes.

## Complexity characteristics of encoder intra-frame prediction type selection

In addition to conventional spatial intra prediction (as in HEVC and JEM), the codec includes additional options for predicting the luma signal of a block: intra prediction with neural networks (sec. 2.1.9.2), intra region-based template matching (sec. 2.1.9.3), and line-based intra coding (sec. 2.1.9.1). The encoder treats these options as additional coding modes and includes them the mode decisions process.

Details of the encoder operations for the additional modes are described in sec. 3.1.3.

## Complexity characteristics of encoder motion estimation and motion segmentation selection

### General

The encoder uses the same motion search strategy as the HM and JEM encoders. The same blocks are used for prediction and transform coding. Hence, the motion segmentation is determined as described in sec. 6.4. In comparison to JEM, the encoder complexity for motion estimation is only increased due to the usage of multi-hypothesis motion compensation (see sec. 6.6.3).

### Complexity characteristics of usage of decoder-side motion refinement

The complexity of decoder-side motion refinement in the encoder is the same as in JEM. The following two JEM tools are used:

- Pattern matched motion vector derivation (FRUC)

- Decoder-side motion vector refinement (DMVR)

### Complexity characteristics of multi-hypotheses motion estimation

For a given coding unit (CU) to be encoded, first the conventional non-multi hypotheses coding modes are tested.  The best (i.e., lowest RD cost) inter predicted mode is saved.  After all non-multi hypotheses coding modes have been tested, an additional motion estimation stage is performed, based on the best found inter predicted mode for this CU.  The same motion estimation algorithm as used for uni/bi motion estimation is used for this additional motion-compensated prediction hypothesis, however the search range is restricted to 16.  For each additional inter prediction hypothesis to be searched, totally 4 motion estimation operations are performed (see Sec. 3.1.4.3).

## Complexity characteristics of encoder transforms and transform type selection

Similar as JEM, the codec supports multiple transforms. The encoder has to select one of the supported transforms. For inter-predicted blocks, the same transforms (EMT) are tested as in JEM. For intra-predicted blocks, five different transform are tested. In comparison to JEM (in which all combintaions of EMT and NSST transforms are supported), the encoder complexity for selecting the transform is reduced for intra-predicted blocks.

## Complexity characteristics of encoder quantization and quantization type selection

The dependent scalar quantization (sec. 2.1.6) is used for all blocks. Conceptually, the quantization process (see sec. 3.1.8) is similar to rate-distortion optimized quantization (RDOQ) implemented in the HM and JEM encoders. However, for obtaining high coding efficiency, the additionally introduced dependencies between transform coefficients have to be considered during the quantization process. The implemented approach consists of two steps. (1) For each transform coefficient, four transform coefficient levels are pre-selected (one for each of the two quantizers). The pre-selection consists of a multiplication and bit shift. (2) Given the pre-selected levels for all coefficients, the final levels are determined by finding a minimum cost path through a four-state trellis structure using the Viterbi algorithm. In comparison to RDQO, the quantization requires more comparisons of rate-distortion costs. Beside of that, the complexity of the calculation of rate-distortion costs is very similar to RDOQ.

## Complexity characteristics of encoder in-loop filtering type selection and parameter estimation

The complexity of the bilateral filter, the deblocking filter, and SAO is the same as in JEM. The encoder complexity of the MCALF is increased relative to GALF (JEM).  Instead of a single classification of samples, three different classification modes are tested. The mode that yields the smallest rate-distortion cost is selected. For each of the three classification modes, the filters are determined in the same way as in JEM.

## Complexity characteristics of encoder entropy coding type selection

The codec includes only a single type of entropy coding. There are no encoder decisions for entropy coding. The complexity of entropy encoding is virtually the same as for entropy decoding.

## Complexity characteristics of additional encoder tools

The codec includes two prediction improvement filters: a signal-adaptive diffusion filter (sec. 2.1.11.1) and DCT thresholding (2.1.11.2).  Prediction modes with these filters enabled are considered as additional modes in the mode decision process.

## Complexity characteristics of 360° video specific encoding tools

The codec does not include any 360° video specific coding tools.

## Complexity characteristics of HDR specific encoding tools

The codec does not include any HDR specific coding tools.

## Degree of capability for encoder parallel processing

The proposed coding tools provide about the same capabilities for encoder parallel processing as HEVC and JEM.  As discussed in sec. 3.1.12.1, we implemented two approaches for thread parallel processing in the encoder:

- Wavefront-like parallel encoding of CTU lines;

- Parallel testing of split modes.

Both approaches for thread parallel processing can be used together.  The submitted bitstreams were generated using a thread parallel encoder in different configurations (i.e., using different numbers of threads).

# Algorithmic characteristics

## Random access characteristics

The random access characteristics is the same as for the HEVC anchor bitstreams.

For the constraint set 1 bitstreams, we used exactly the same temporal coding structure (hierarchical B pictures with 16 pictures per GOP) as the HEVC anchor bitstreams. The same pictures are coded as CRA pictures. The bitstreams provide clean random access at all CRA pictures.

The constraint set 2 bitstreams, we also used the same coding structure as the HEVC anchor bitstreams. The constraint set 2 bitstreams do not include any random access points except the first picture.

## Delay characteristics

The delay characteristic is the same as for the anchor bitstreams / anchor encodings. We used the same temporal coding structure as the anchor bitstreams. None of the added coding tools increases the encoder or decoder delay.

# Software implementation description

The proposal is based on the publicly available NextSoftware [3], which represents an alternative implementation of the Joint Exploration Model (JEM). The NextSoftware has advantages over the JEM software in various aspects such as readability and code integration. The NextSoftware also features runtime configurability of coding tools and improved debugging capabilities compared to the JEM software. Considerable runtime performance gains with respect to the JEM software, e.g. roughly 50% encoder runtime at a coding penalty below 1% BD-rate for a random access configuration, make the NextSoftware an attractive starting point for standardization.

Apart from extending the public version of the NextSoftware for implementations of the herein proposed coding tools, the software implementation comes with further capabilities for runtime minimization. These efforts go beyond traditional SIMD code optimization and allow for parallelization of the encoding process as detailed in JVET-J0036 [5].

Based on the open source parallel processing framework OpenMP, the software integrates two separate parallel processing approaches. The first proposed approaches, namely Wavefront Parallel Processing (WPP) extents the concept as adopted in HEVC with mechanisms for spatial QP prediction and an advanced CABAC context initialization.  The second proposed approaches, parallel CU split-mode testing, groups the available split options into disjoint subsets which are tested in parallel. Both parallelization approaches are configurable at runtime and can be applied individually or in combination.

The software further supports to partition coded picture into rectangular tile grids with constraints in entropy coding and intra-prediction similar to tiles in HEVC and JEM. The encoder may also be easily configured to constraint Inter-prediction to enable decoding of tiles without cross-tile dependencies similar to motion constrained tile sets in HEVC.

Software is available in the following svn repository:

https://ipbt.hhi.fraunhofer.de/svn/svn_hhi-next-software/tags/NextSoftware-JVET-J0014

This software includes the modifications for the block partitioning (JVET-J0035) as well as improvements of tools described in JVET-J0037 and JVET-J0039.

# Highlighted aspects discussion

## Coding Efficiency / Complexity Analysis of Coding Tools

In the following, we present a coding efficiency / complexity analysis of the proposed tools and the JEM tools integrated into the same software basis. The analysis is similar to that presented in JVET-B0044 [14]. A complexity measure, we used a weighted measure of encoder and decoder run time overhead,

C = ( ( Cenc – 1 ) + 5 * ( Cdec – 1 ) ) / 6,

where Cenc and Cdec represent the ratios of the geometric means of the run times for encoder and decoder, respectively.  The ratios Cenc and Cdec are the complexity measures that are commonly reported in evaluation of coding tools.  The weighting factor w=5 is used for an unequal weighting of encoder and decoder complexity.  Hence, we assumed that the decoder complexity is five times more important than the encoder complexity.

The coding efficiency / complexity trade-off is measured as the slope angle (given by the complexity measure C defined above and the BD delta rate for luma),

α=atan2C, -BDR,

where BDR represents the luma Bjøntegaard delta rate.  We basically followed the JVET common test conditions [15] for the random access scenario with the following modifications:

- The set of test sequences was extended by the CfP test sequences [1] (that means, the union of the JVET common test sequences and the CfP test sequences was used in the tests);

- Only 49 frames of each sequence were encoded (for reducing the simulation time);

- In addition to the QP values {22, 27, 32, 37} (high rates), we also run simulations for the QP values {27, 32, 37, 42} (low rates), because the latter set of QP value represents a better approximation of the QP values actually used for achieving the bit rates specified in the CfP.

As reference configuration, we used an encoder setting in which all proposed and JEM coding tools are disabled, but a modified block partitioning is used. As block partitioning, we used the QT+BTS partitioning described in JVET-J0035 [4], in a configuration that only yields block sizes for which both the block width and height represent integer powers of two. This setting ensures that only block sizes also available with QTBT are used and it provides an increased coding efficiency relative to QTBT. In comparison to HEVC, QT+BTS in the configuration used provides BD rates of -8.1%, -20.3%, -22.8% (Y, U, V) for high rates (QP values 22, 27, 32, 37) and BD rates of -9.3%, -25.3%, -26.7% (Y, U, V) for low rates (QP values 27, 32, 37, 42). In comparison, the measured gains for QTBT are -5.9%, -17.5%, -19.5% (Y, U, V) for high rates and -6.9%, -22.2%, -23.3% (Y, U, V) for low rates. For detailed results, the reader is referred to JVET-J0035 [4].

Each coding tools has been separately enabled in addition to the QT+BTS partitioning and the BD rates and run times have been measured against the QT+BTS reference. The results for QP values {22, 27, 32, 37} (high rates) are summarized in Table 15 and the results for QP values {27, 32, 37, 42} (low rates) are summarized in Table 16. The proposed coding tools are marked blue. The coding tools are ordered according to the measured coding efficiency / complexity slopes introduced above.

Table 15:  Coding efficiency / complexity analysis of proposed coding tools (blue table rows) and JEM coding tools (black table rows) for the random access configuration using QT+BTS as reference and QP values 22, 27, 32, 37 (high bit rates).

Table 16:  Coding efficiency / complexity analysis of proposed coding tools (blue table rows) and JEM coding tools (black table rows) for the random access configuration using QT+BTS as reference and QP values 27, 32, 37, 42 (low bit rates).

Using the same test conditions and test sequences, we additionally analyzed the combination of HHI tools (including modified JEM tools).  Table 17 shows the average results for three configurations: QT+BTS partioning only (same results are reported in JVET-J0035 [4]), all HHI tools except the modified adaptive loop filter (MC-ALF), all HHI tools including the modified adaptive loop filter (i.e., all blue marked tools in Table 15 and Table 16). We want to point out that the proposed coding tools are characterized by a low decoding complexity. Without the adaptive loop filter, the decoder run time is increased to only 130% to 145% of the HM decoder run time. When additionally enabling the adaptive loop filter, the decoder run time increases to about 200-210% of the HM decoder run time.

Table 17:  Coding efficiency / complexity analysis for combinations of HHI coding tools for the random access configuration. The reference is HM-16.14 (same configuration as for anchors, but with fixed QP values).

## Discussion of Chroma QP Selection

For the submitted bitstreams, we did not modify the relationship between luma QP and chroma QP relative to JEM.  The objective results presented in sec. 4 show that, in general, the BD rate gains in chroma are significantly higher than the BD rate gains in luma.  By increasing the chroma QP relative to the luma QP it is possible to obtain a setting in which a larger part of the total bit rate is used for luma.  Such a modification yields results with larger luma BD rate gains and smaller chroma BD rate gains.  In order to demonstrate this effect, we used the luma-to-chroma-QP mapping table suggested in the MediaTek proposal JVET-J0018 [13].  This table specified the following mapping:

| luma QPY | QPY < 30 | 30 ≤ QPY ≤ 34 | QPY > 34 |
| --- | --- | --- | --- |
| chroma QPC | QPY | QPY − 1 | QPY − 2 |

In addition, a chroma QP offset of 1 was used for UHD sequences and a chroma QP offset of 2 was used for HD sequences.  We generated bitstreams according to the CfP for SDR constraint set 1 and HDR.  For these simulations, an updated version of the proposal software was used (which uses the QT+BTS proposed in JVET-J0035 and some complexity reductions for some tools).  The results with and without the chroma QP adaptation are presented in Table 18 and Table 19.  As can be seen, a modification of the relationship between luma QP and chroma QP can yield significant gains in luma BD rate. In particular, the BD rate savings for SDR-A (relative to JEM) could be improved from -6.3% to -15.0%.

Table 18:  Comparison of results with and without chroma QP change for SDR constraint set 1.

Table 19:  Comparison of results with and without chroma QP change for HDR.

# Closing remarks

Some of the described coding tools have been improved (regarding their coding efficiency / complexity trade-off) after the bitstreams were submitted. Improved tools are described in the additional contributions JVET-J0035 [4], JVET-J0037 [6], JVET-J0038 [7], JVET-J0039 [8], JVET-J0040 [9]. In addition, the contribution JVET-J0041 [10] provides further details on multi-hypothesis inter prediction and the document JVET-J0036 [5] provides additional details on encoder parallelization.

# References

- JVET, “Joint Call for Proposals on Video Compression with Capabilities beyond HEVC,” JVET-H1002, Oct. 2017

- JVET, “Algorithmic Description of Joint Exploration Test Model 7 (JEM 7),” JEVT-G1001, July 2017.

- Wieckowski, et. al., “AHG10: Updated NextSoftware as an alternative implementation of the Joint Exploration Model (JEM),” JVET-I0034, Jan. 2018.

- Ma, Wieckowski, George, Hinz, Brandenburg, de Luxan Hernandez, Kirchhoffer, Skupin, Schwarz, Marpe, Schierl, Wiegand, „Quadtree plus binarytree with shifting,“ JVET-J0035, Apr. 2018.

- Wieckowski, Hinz, George, Ma, Brandenburg, Lehmann, Schwarz, Marpe, Schierl, Wiegand, „Thread Parallel Encoding,“ JVET-J0036, Apr. 2018

- Pfaff, Helle, Maniry, Kaltenstadler, Stallenberger, Merkle, Siekmann, Schwarz, Marpe, Wiegand, „Intra Prediction Modes based on Neural Networks,“ JVET-J0037, Apr. 2018.

- Pfaff, Rasch, Schaefer, Schwarz, Winken, Henkel, Siekmann, Marpe, Wiegand, „Signal Adaptive Diffusion Filters for Video Coding,“ JVET-J0038, Apr. 2018.

- Venugopal, Schwarz, Marpe, Wiegand, „Intra Region-based Template Matching,” JVET-J0039, Apr. 2018.

- Siekmann, Stallenberger, Bartnik, Pfaff, Marpe, Schwarz, Wiegand, „Set of Transforms,“ JVET-J0040, Apr. 2018.

- Winken, Bartnik, Schwarz, Marpe, Wiegand, „Multi-Hypothesis Inter Prediction,“ JVET-J0041, Apr. 2018.

- Ye, Alshina, Boyce, " Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 4,” JVET-G1003, July 2017.

- Van der Auwera, Coban, Karczewicz, “AHG8: ACP with padding for 360-degree video,” JVET-G0071, July 2017.

- Bosse, et. al., “Perceptually optimized QP adaptation and associated distortion measure,” JVET-H0047, Oct. 2017.

- Schwarz, Rudat, Siekmann, Bross, Marpe, Wiegand, „Coding Efficiency / Complexity Analysis of JEM 1.0 coding tools for the Random Access Configuration,” JVET-B004, Feb. 2016.

- Suehring, Li, „JVET common test conditions and software reference configurations,” JVET-H1010, Oct. 2017.

- Hsu, et. al., “Description of SDR video coding technology proposal by MediaTek,” JVET-J0018, Apr. 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: Questionnaire

## Codec level questions

- Maximum number of reference frames used by encoder

4 reference frames

- Maximum coding unit (e.g. CTU in HEVC)

luma: 128×128; chroma 64×64

- Minimum and maximum transform block size

Minimum transform block size:  luma: 4×4, 4×1, 1×4; chroma: 2×2

Maximum transform block size:  luma: 128×128; chroma: 64×64

- Minimum and maximum intra prediction block

Minimum intra prediction block size:  luma: 4×4, 4×1, 1×4; chroma: 2×2

Maximum intra prediction block size:  luma: 32×32; chroma: 16×16

- Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length

luma: 4×4, 8-tap filters

chroma: 2×2, 4-tap filters

- List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category)

- template matching in FRUC (JEM tool)

- Local illumination compensation (JEM tool)

- Diffusion filter for prediction improvement

- DCT thresholding for prediction improvement

- List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples)

- 64-bit arithmetic for LIC, BIO and LMC (JEM tools)

- List all tools that use non-rectangular (e.g. triangle) partitions

n/a

- Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa)

n/a

- Indicate whether luma and chroma use separate partitioning structures
Luma and chroma both use the GBT partitioner but can have different partitionings due the dual tree.

Dual partitioning in I slices (as in JEM)

- Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes)

Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation (same as in JEM)

- If the coded picture resolution is different from the input picture resolution, list the coded picture resolution for those sequences (e.g. 360° video, input is 8K ERP, coded video is in 4K CMP converted from the 8K input)

360° 6K:  coded video resolution 6912×1152

360° 8K:  coded video resolution 7680×1280

## Tool level questions

- Partitioning structure

  - Whether the prediction unit partitioning is different from transform unit partitioning:

The transform unit partitioning is identical to the prediction unit partitioning.

  - Maximum tree depth

Maximum BT depth is set to 3

- Entropy coding

  - Total number of context models

702

  - Memory storage size for each context model:

38 bits per context model (two 15-bit counter + two 4-bit values for storing the window sizes)

  - Range table size if table look up is used for range update:

32×8×8 bits for LPS and 32×8 bits for renormalization

  - Initialization table size:

702×6×8 bits (702 context models, three 8-bit values [one per slice type] for initializing the probability counters, three 8-bit values [one per slice type] for setting the window sizes)

  - Other memory associated with context model adaptation:

no

- Control data derivation process

  - Temporal motion data storage granularity

4×4

  - Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-predictive reference picture are used, count this picture as 2)

4 (same as JEM)

- Decoder-side motion refinement

  - Line buffer size in terms of motion vector block lines (assuming horizontal motion vector block size is 4 luma samples)

same as in JEM: BIO/FRUC

  - Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement

same as in JEM: DMVR/FRUC

  - Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size

same as in JEM: DMVR/FRUC

- Intra prediction

  - Line buffer size in terms of luma and chroma sample lines

luma: 2; chroma: 2

Intra region-based template matching requires additional lines (depends on configuration)

  - List all the filters used for intra prediction

Reference sample smoothing: {1,2,1}, {3,7,14,16,14,7,3}, {4,9,12,14,12,9,4}, bilateral filter with up to 9 taps and 3 iterations

Linear diffusion filter: iterative non-adaptive filter with 4-taps

Non-linear diffusion filter: iterative filter with signal-adaptive 3×3 filter masks

DCT thresholding: DCT-II transform + hard thresholding + inverse DCT-II transform

  - Indicate whether there are cross-component dependencies in intra prediction reconstruction

LM-Chroma (same as in JEM)

- Inter prediction

  - Motion vector accuracy

Luma: 1/16-th sample, chroma: 1/32-th sample

  - Storage size for a motion vector component

16 bit

  - List all the interpolation filters used for luma/chroma motion compensation

Luma: 8-tap, 8×16×6 bits

Chroma: 4-tap, 4×32×6 bit

- Quantization

  - Storage size associated with quantization

8 bits (table with 2×2×2 bits)

- Transforms

- List all the combinations of transform block size and transform type for primary transforms

Heights and widths of blocks in luma of the proposed method can be 4, 8, 12,16, 20, 24, 32, 40, 48, 80, 96, and 128. For chroma additionally 2, 6, and 10 are possible. Depending on the configuration not all combinations may be supported.

In intraNN prediction mode:

  - 140x 4x4 non-separable direct matrix multiplication transforms

  - 324x 4x8 and 8x4 non-separable direct matrix multiplication transforms

  - 140x 8x8 non-separable direct matrix multiplication transforms

- List all the combinations of transform block size and transform type for secondary transforms

  - 35×3 4×4 and 8×8 non-separable Hypercube-Givens Transform (HyGT)

  - 175x 4x8 and 179x 8x4 non-separable direct matrix multiplication transforms

  - 568x 8x8 non-separable direct matrix multiplication transforms

- List all the non-separable transforms used

  - 4×4/8×8 HyGT

  - 4x4/4x8/8x4/8x8 non-separable direct matrix multiplication transforms

- Indicate whether transforms can be fully/partially implemented using direct matrix multiply

All newly introduced transforms are implemented using direct matrix multiply

- List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output)

  - 4×4 HyGT 2-interations, 8×8 HyGT 4-interations

- In-loop filtering

- List line buffer size in terms of luma and chroma lines (if any)

Deblocking (luma 4 & chroma 2)

SAO (luma 1 & chroma 1)

ALF (luma 8, chroma 4)

- List all the filters used for in-loop filtering

Deblocking: for luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and

(2 3 1 1 1)/8; for chroma: (1 4 4 –1)/8

ALF: 9×9/7×7/5×5 diamond filter support for luma; and 5×5 diamond filter support for chroma

- Minimum processing unit size

Deblocking (luma 4×4 grid with 4×4 filter decision, chroma 8×8 grid with 2×2 filter decision)

ALF: for luma, filter decision is based on 2×2 block

Bilateral filter: sample level filter switch (luma only)

- “tool-off” and “tool-on” tests

- List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools (see Table 1 below for test procedure)

see analysis in sec. 9