| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0026 |
| --- | --- |

| Title: | Description of SDR and HDR video coding technology proposal by Sharp and Foxconn |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra Jie Zhao Andrew Segall Weijia Zhu Byeongdoo Choi Frank Bossen Michael Horowitz Phil Cowan Yukinobu Yasugi Tomonori Hashimoto Tianyang Zhou Tomohiro Ikai Takeshi Chujoh Tomoko Aono Yao-Jen Chang Hui-Yu Jiang Tsung-Hua Li Yu-Chiao Yang |  |  |
| Source: | Sharp Corporation, Foxconn Technology Group |  |  |

_____________________________

# Abstract

This is document provides the Call for Proposal response from Sharp Corporation and Foxconn Technology Group.  The response focuses on improved coding efficiency for the SDR and HDR categories, and it emphasizes a block based approach with a coding structure that is asserted to be more flexible than the previous HEVC standard.  The flexibility is claimed to allow the codec to better adapt to the local characteristics of a video sequence.  The response incorporates a large subset of the algorithms studied in the JEM software, with additional contributions in the areas of partitioning, motion coding, loop filtering and intra coding.  Additionally, for the HDR category, additional tools in the area of quantization and bit-depth management are incorporated.  The combination of these tools achieves a measureable improvement in coding efficiency relative to an HEVC anchor, with gains of 41.2% and 35.7% for 4K-SDR and HD-SDR sequences, respectively, using the random access configuration; gains of 29.0% for HD-SDR sequences using a low delay configuration, and gains of 34.3% and 32.2% for PQ-HDR and HLG-HDR sequences, respectively, using a random access configuration.  It is asserted that this combination of coding performance with a flexible block structure provides a strong starting point for developing a next generation video coding standard.

# Contents

Abstract	i

Contents	ii

1	Introduction	1

2	Decoder algorithm description	1

2.1	General algorithm description	1

2.1.1	Bitstream structure	1

2.1.2	Entropy decoding	3

2.1.3	Conversion process from entropy decoded data to symbols	3

2.1.4	Decoder-side motion refinement	4

2.1.5	Inverse quantization	6

2.1.6	Inverse transforms	6

2.1.7	Motion compensation	6

2.1.8	Intra prediction	7

2.1.9	In-loop filtering	7

2.1.10	Tiles	7

2.1.11	Additional tools	9

2.1.12	Additional algorithmic description topic(s)	9

2.2	360° Video	9

2.3	HDR	9

2.3.1	Post-processing	9

2.3.2	Quantization handling	9

2.3.3	Internal Bit Depth Increase (IBDI)	11

2.3.4	HDR specific decoding tools	11

3	Encoder algorithm description	11

3.1	General algorithm description	11

3.1.1	Rate-distortion optimization	11

3.1.2	Partitioning structure selection	11

3.1.3	Intra prediction mode estimation and type selection	12

3.1.4	Motion estimation and motion segment selection	12

3.1.5	Mode decisions	12

3.1.6	In-loop filtering type selection and parameter estimation	12

3.1.7	Transforms and transform type selection	12

3.1.8	Quantization and quantization type selection	12

3.1.9	Entropy coding	12

3.1.10	Additional tools	12

3.1.11	Additional encoder optimization	12

3.1.12	Additional algorithmic description topic(s)	12

3.2	360°	12

3.3	HDR	12

3.3.1	Pre-processing	12

3.3.2	HDR specific encoding tools	12

3.3.3	Internal Bit Depth Increase (IBDI)	12

3.3.4	Additional encoder optimization	13

4	Compression performance	13

4.1	SDR: Constraint set 1 configuration relative to HM anchor	13

4.1.1	Class SDR-A (RD-curves)	13

4.1.2	Class SDR-B (RD-curves)	16

4.1.3	Overall (BD-rate summary tables)	18

4.2	SDR: Constraint set 2 configuration relative to HM anchor	19

4.2.1	Class SDR-B	20

4.2.2	Overall	22

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	23

4.3.1	Class SDR-A	23

4.3.2	Class SDR-B	26

4.3.3	Overall	28

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	29

4.4.1	Class SDR-B	30

4.4.2	Overall	32

4.5	HDR: Constraint set 1 configuration relative to HM anchor	33

4.5.1	Class HDR-A	33

4.5.2	Class HDR-B	35

4.5.3	Overall	37

4.6	HDR: Constraint set 1 configuration relative to JEM anchor	38

4.6.1	Class HDR-A	38

4.6.2	Class HDR-B	40

4.6.3	Overall	42

4.7	360° Video: Constraint set 1 configuration relative to HM anchor	43

4.7.1	Class 360	43

4.7.2	Overall	43

4.8	360° Video: Constraint set 1 configuration relative to JEM anchor	43

4.8.1	Class 360	43

4.8.2	Overall	44

5	Decoder complexity analysis	44

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	44

5.2	Description of computing platform used to determine decoding times reported in section 5.1	44

5.3	Memory usage of decoder	44

5.4	Complexity characteristics of decoder entropy decoding operation	44

5.5	Complexity characteristics of decoder inverse quantization	44

5.6	Complexity characteristics of decoder inverse transform operation	44

5.7	Complexity characteristics of decoder-side motion refinement	44

5.8	Complexity characteristics of decoder motion compensation	44

5.9	Complexity characteristics of decoder intra-frame prediction operation	44

5.10	Complexity characteristics of decoder in-loop filtering operation	44

5.11	Complexity characteristics of additional decoder tools	45

5.12	Complexity characteristics of 360° video specific decoding tools	45

5.13	Complexity characteristics of HDR specific decoding tools	45

5.14	Degree of capability for decoder parallel processing	45

6	Encoder complexity analysis	45

6.1	Encoding time and measurement methodology	45

6.2	Description of computing platform used to determine encoding times reported in section 6.1	45

6.3	Memory usage of encoder	45

6.4	Complexity characteristics of encoder intra-frame prediction type selection	46

6.5	Complexity characteristics of encoder motion estimation and motion segmentation selection	46

6.5.1	General	46

6.5.2	Complexity characteristics of usage of decoder-side motion refinement	46

6.6	Complexity characteristics of encoder transforms and transform type selection	46

6.7	Complexity characteristics of encoder quantization and quantization type selection	46

6.8	Complexity characteristics of encoder in-loop filtering type selection and parameter estimation	46

6.9	Complexity characteristics of encoder entropy coding type selection	46

6.10	Complexity characteristics of additional encoder tools	46

6.11	Complexity characteristics of 360° video specific encoding tools	46

6.12	Complexity characteristics of HDR specific encoding tools	46

6.13	Degree of capability for encoder parallel processing	46

7	Algorithmic characteristics	46

7.1	Random access characteristics	46

7.2	Delay characteristics	47

7.3	Additional characteristics discussion topic(s)	47

8	Software implementation description	47

9	Highlighted aspects discussion	47

10	Closing remarks	47

11	Patent rights declaration(s)	47

12	Annex: Questionnaire	48

12.1	Codec level questions	48

12.2	Tool level questions	48

13	Example of filling in the questionnaire: HM vs. JEM	50

13.1	Codec level Q&A	50

13.2	Tool level Q&A	51

13.2.1	Category 1. Partitioning structure	51

13.2.2	Category 2. Entropy coding	51

13.2.3	Category 3. Control data derivation process	51

13.2.4	Category 4. Decoder-side motion refinement	51

13.2.5	Category 5. Intra prediction	52

13.2.6	Category 6. Inter prediction	52

13.2.7	Category 7. Quantization	52

13.2.8	Category 8. Transforms	53

13.2.9	Category 9. In-loop filtering	53

13.2.10	Category 10. “tool-off” and “tool-on” tests	54

# Introduction

We describe a video coding system that is based on a flexible, rectangular partitioning structure.  The flexibility of this structure is derived from four key characteristics: (i) support for a number of rectangular partitions, (ii) adaptive use of different partitioning strategies for luma and chroma in intra-coded regions, (iii) a method for inferring the partitioning at picture boundaries, and (iv) a tile partitioning strategy to better support parallel decoding of high resolution video.  This adaptive structure is then combined with new coding tools as well as tools that are available in the JEM software.  These new tools include improved intra-prediction, loop filtering and motion compensation processes.

The emphasis of the proposal is on standard dynamic range and high dynamic range content.  In the case of high dynamic range content, the proposal includes two coding efficiency tools in addition to the SDR configuration.  These tools are enabled when the content uses the PQ transfer function, which corresponds to the HDR-B category, and consist of a QP inference and bit-depth expansion technology.

The rest of this document provides more detail about the approach and individual tools, and it is organized as follows:  Section 2 contains a description of the decoder; Section 3 contains a description of the encoder, and Section 4 provides coding performance information.  Decoder and encoder complexity analysis is provided in Sections 5 and 6; algorithm characteristics are discussed in Section 7, and a description of the software is in Section 8.  Section 9 provides closing remarks and conclusions.

# Decoder algorithm description

The goal of this section is to provide a detailed description of the decoder algorithms used in the proposed video codec.  The emphasis of this section is on the design of the flexible, rectangular partitioning scheme.  Improved algorithms for motion compensation, loop filtering and Tiles are also described, as are the QP coding and bit-depth management tools developed for high dynamic range content.

## General algorithm description

The algorithm structure has a set of common tools that are proposed to be applied across all content types.  These tools are described in the sub-sections below, while tools specific for high dynamic range video coding are described in a later section.

### Bitstream structure

We employ a block based strategy for our bitstream structure.  A core component of the structure is the coding unit.  In the approach, the coding unit corresponds to a block of samples in the reconstructed frame.  Coding units are rectangular in shape and associated with received prediction and residual information.  For inter-coded regions, a coding unit corresponds to one block of luma samples and two blocks of chroma samples. For intra-coded regions, a coding unit may correspond to one block of luma samples, two blocks of chroma samples, or both.

The bitstream contains the partitioning information to allow a decoder to determine the rectangular region size, as well as the associated prediction and residual information.  This information is signaled as a set of partition trees, where each partition tree corresponds to a square coded tree unit (CTU).  The size of the coded tree unit is received in the bitstream, and the coded tree units are received in raster scan order.  The partition tree is then received as a series of recursive split indicators and no-split indicators.  To be clear, the structure described in this paragraph is similar to what is done in the HEVC standard and JEM software.  The difference is that we support additional split indicators to enable additional rectangular region sizes.

The split indicators supported in the system are as follows: (i) quad-tree partition, (ii) binary split with half offset, (iii) binary split with quarter offset, (iv) binary split with three quarter offset, and (v) ternary split with quarter and three quarter offsets.  The partitions are shown graphically in Figure 2.1.  As can be seen in the Figure, the quad-tree partition divides an NxN region into four equally sized N/2 x N/2 regions, while the binary split with half offset divides an NxM region into either two N/2xM or NxM/2 regions.  Similarly, the binary split with quarter or three quarters offset divides an NxM region into a first partition of N/4xM, NxM/4, 3N/4xM or Nx3M/4 size, with the second partition containing the remainder of the NxM region. The ternary split shown in Figure 2.1 (h) and (i) simultaneously applies binary splits with quarter and three quarters offsets, where sizes of the resulting three partitions are N/4xM, N/2xM and N/4xM, or NxM/4, NxM/2 and NxM/4.  For the binary and ternary split partition types, the direction of split is also received in the bitstream.  Finally, please note that when calculating the size of the first partition of a binary or ternary split, a rounding operation is applied so that its dimension is always a multiple of four.

While the partition sizes are flexible in the structure, there are limitations on the dimensions of the resulting partitions, and the bitstream is structured so that the decoder infers receiving a no split flag when these limits are reached.  Specifically, in this proposal, a binary split with quarter or three quarter offset can only be received when the rounding operation results in a partitioning with two partitions of unequal size.  Similarly, the quad tree partition can only be received when the resulting partitions have equal sizes that are a power of two.  Finally, a ternary split can only be received when the resulting partition size of a quarter split is a power of two.

Figure 2.1 The split types of quad split, binary split with half offset, binary split with quarter offset, and ternary: (a) quad split, (b) vertical binary split with half offset partition (c) horizontal binary split with half offset partition (d) vertical binary split with quarter offset partition (e) horizontal binary split with quarter offset partition (f) vertical binary split with three quarter offset partition (g) horizontal binary split with three quarter offset partition (h) vertical ternary split partition (i) horizontal ternary split partition

Further limitations are also received in the bitstream that control the need to receive a split indicator.  This information includes depth parameters that determine the number of recursive splits that are allowed.  The information is received independently for the quad-tree and binary tree partition types and can also be received separately for luma and chroma partition trees (when allowed as discussed below).  In addition to the depth parameters, maximum and minimum block size parameters are also received to provide limits on block sizes.  All of this control information is signaled in the bitstream, and the syntax and semantics are showing in Table 2.1.  Different sets of these parameters may be signaled.  The set to be used is then determined based on the slice type, chroma channel and temporal layer identifier.  As before, when receiving a split flag in the bitstream would result in a partition that exceeds the transmitted limits, then the split flag is not included in the bitstream, and the decoder infers a no-split state is intended.  Similarly, in the case of the maximum block size information, a split state is inferred if the current partition size has dimensions larger than the maximum block size parameter.

Note, in the proposal, the maximum binary tree depth is received in the bit-stream.  However, for pictures at the higher temporal layers, the depth is further adapted from the maximum binary tree depth used in temporally collocated CTUs.

Table 2.1: Syntax elements in sequence parameter set that limit splitting

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_log2_minQT_non_ISlice_minus2 | ue(v) |
| sps_log2_minQT_ISliceLuma_minus2 | ue(v) |
| sps_log2_minQT_ISliceChroma_minus2 | ue(v) |
| sps_max_bt_depth_non_ISlice | ue(v) |
| sps_max_bt_depth_Islice_luma | ue(v) |
| sps_max_bt_depth_Islice_chroma | ue(v) |
| … |  |
| } |  |

sps_log2_minQT_non_ISlice_minus2 plus 2 specifies the minimum size (in luma samples) of a coding block that may undergo quad split in non-I slice

sps_log2_minQT_ISliceLuma_minus2 plus 2 specifies the minimum size (in luma samples) of a luma coding block that may undergo quad split in I slice

sps_log2_minQT_ISliceChroma_minus2 plus 2 specifies the minimum size (in luma samples) of a chroma coding block that may undergo quad split in I slice

sps_max_bt_depth_non_ISlice specifies the maximum binary tree and ternary tree depth of a coding block in non-I slice

sps_max_bt_depth_ISlice_luma specifies the maximum binary tree and ternary tree depth of a luma coding block in I-slice

sps_max_br_depth_ISlice_chroma specifies the maximum binary tree and ternary tree depth of a chroma coding block in I-slice

An additional system of limitations is used at the picture boundaries.  Specifically, for picture dimensions that are not an integer multiple of the CTU size, CTUs at the right and/or bottom of the picture will contain pixel regions that do not correspond to the intended, reconstructed image.  In the system described here, these CTUs undergo implicit partitioning that make use of the available square as well as rectangular partitionings until all the samples at the leaf of the partitioning tree created using implicit splits do not span across the picture boundary. The implicit splits can include both quad tree and binary tree splits.  Moreover, the quad-tree splitting can be further controlled by receiving a minimum number of required quad-tree splits of the CTU within a parameter set.  After the minimum number of quad tree splits have been performed, the decoder performs a series of implicit binary splits according to the following process.  First, for both the horizontal and vertical directions, the binary tree split offset that results in the split edge being closest to the picture boundary is selected. Then, between the two split direction candidates, the direction that does not partition the block of samples inside the picture boundary is chosen – or if both directions partition the block of samples inside the picture boundaries, the horizontal partition is simply chosen. This process is performed recursively.  Once complete, explicit signaling in the bitstream determines if the resulting partitions will undergo further splitting after the implicit partitioning has been carried out.

The rectangular partitioning described above provides flexibility in spatial partitioning.  As a different aspect of flexibility, the system described here also allows for the flexibility of using different partitionings for luma and chroma. In the JEM, a separate tree partitioning is employed for intra-slices while a shared tree structure is used for inter-slices.  Here though, we introduce the flexibility of adaptively switching between a shared or separate tree structure. The switching is signaled on a coding unit basis, and it is only available for intra coded regions. The selection is received by first receiving the partition information for the CTU, and then, receiving the prediction type for each resulting coding unit.  In the case that the prediction type is intra, a flag is received indicating if the intra region in the coding unit will be further refined with a separate or shared tree structure.

An illustration of the method appears in Figure 2.2.  As can be seen in the Figure, a CTU partitioning can be selectively changed for different coding units.  Moreover, and as above, additional limitations are imposed on the signaling.  This includes that when a coding unit has a block size greater than or equal to 512, then no selection flag is received and a separate luma and chroma partitioning must be used the coding unit.  Alternatively, when the block size is small, then the selection flag is also not received but a shared luma and chroma partitioning is employed.

Figure 2.2 An illustration of combined separate and shared tree partitioning for intra coded regions.

### Entropy decoding

The proposal uses a context-adaptive binary arithmetic coder.  It is the same as what is used in the JEM with the exception that it also includes the double-window adaptation response described in JVET-G0112.

### Conversion process from entropy decoded data to symbols

The binarization of the split and no-split indicators is shown in Figure 2.3.  As can be seen in the Figure, the first bin of the binarization signals if the current block will be further split. If the block will be split, the second bin indicates the splitting direction (horizontal or vertical). The third bin indicates whether a TT partition is selected. If not, the last one or two bins are used to represent the split offset for binary split.

|  | Binarization; Partitioning. 1; QT. 0 0; No Split. 0 1 0 0; Vertical TT. 0 1 0 1 0; Vertical BT 1/2. 0 1 0 1 10; Vertical BT 1/4. 0 1 0 1 11; Vertical BT 3/4. 0 1 1 0; Horizontal TT. 0 1 1 1 0; Horizontal BT 1/2. 0 1 1 1 10; Horizontal BT 1/4. 0 1 1 1 11; Horizontal BT 3/4. |
| --- | --- |

Figure 2.3 Signalling used for the QT, TT, BT

#### Intra prediction parameters

Same as JEM.

#### Motion parameters

Same as JEM.

#### Quantization parameters

Same as JEM.

#### Transform coefficients

Same as JEM.

#### In-loop filter parameters

Same as JEM.

### Decoder-side motion refinement

We use the decoder-side motion refinement tools currently available in the JEM with the following two changes.  First, we employ an enhanced FRUC process. Second, we employ a bit-depth adjusted cost function within the decoder side motion refinement calculation.  These two tools are described in more detail in the following sub-sections.

#### Bit-depth adjusted cost function in decoder side motion refinement (BDCost)

The cost function of the decoder side motion refinement, i.e. FRUC, is modified to measure distortion more correctly and balance motion cost and distortion in range of bitdepth.

where SAD is a distortion measured in output bit depth and w is set equal to 4 << (output bitdepth – 8).

This is different from the JEM where the distortion (SAD) is measured in 8-bit depth and w is set equal to constant 4.

|  | JEM | Proposal |
| --- | --- | --- |
| SAD (measured distortion) | measured in 8-bit (regardless of input/internal bitdepth) | measured in output bit depth (i.e. 10-bit in CfP) |
| w (motion weight) | 4 (constant) | 4 << (output bit depth – 8) |

#### Frame Rate Up Conversion (FRUC) based Merge mode

The FRUC merge mode is a new merge mode based on FRUC algorithm in JEM. On top of the existing features of the FRUC merge mode, the following features are newly proposed in this contribution:

- Asymmetric bilateral matching

In the bilateral matching in JEM, the forward MV an the backward one shall have symmetric MVs. Hence, when the forward MV is equal to (Vx + offset_x, Vy + offset_y), where (Vx, Vy) indicates the origin MV derived from neighboring MVs and (offset_x, offset_y) indicates the shift position from the origin, its paired backward MV shall be equal to (-Vx - offset_x, -Vy - offset_y).

Figure 2.4 Symmetric bilateral matching in JEM

In the proposed method, asymmetric bilateral matching, the backward MV may have the following MV value (V1x - offset_x, V1y - offset_y), while the forward MV is equal to (V0x + offset_x, V0y + offset_y), where (V0x, V0y) and (V1x, V1y) indicate the forward MV and the backward one of the current origin derived from neighboring MVs, respectively.

Figure 2.5: Proposed asymmetric bilateral matching

- New cost function for bilateral matching with side template matching

The bilateral matching can provide good motion vectors if the video sequence contains simple translational motions. However, it does not yield reliable motion trajectories when objects have more complicated motions like rotation or zoom. In this proposal, a new cost function is proposed by combining the bilateral matching distortion and the template matching distortion.

New RD cost = Bilateral matching distortion + λ*Side-template matching distortion

The side-template matching distortion is measured very much like for the template matching. Two differentiating features are the width of template and the recursive refinement of sub-CU MVs. By default, the width of the side-template for the new cost function is equal to two. For more accurate refinement of sub-CU MVs, the previously estimated MVs of top and left sub-CUs are reused to obtain the hypothetical reconstruction pixel data in the side-template region. The hypothetical reconstruction pixel data is compared with the predicted pixel data derived by the current MV candidate, and the side-template matching distortion for the current sub-CU is calculated.

- Improvement of Uni/Bi selection for template matching

In JEM, the forward and backward predictions are independently searched, and then uni- or bi-prediction is selected by comparing each RD cost. In the decision processing, the bidirectional distortion between the forward prediction and the backward prediction is calculated by comparing the forward predicted pixel data with the backward ones using the bilateral matching cost function. In the proposed method, the hypothetical prediction pixel data is constructed by averaging the forward prediction pixel values and the backward prediction ones. Then, the hypothetical prediction pixel data is compared with the reconstructed pixel data in neighboring (top/left) CUs.

### Inverse quantization

Same as JEM with support for added transform sizes (see below). Quantization matrices are currently not supported for the added transform sizes.

### Inverse transforms

The same transform types are used as in JEM, with additional transform sizes supported to enable the flexible partitioning.  Specifically, the transform sizes in the proposal are 2 and multiples of 4.  For example, when using a 128x128 CTU, the transform width or height may be one of: 2, 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 48, 52, 56, 64, 72, 96, 128. For block sizes not supported in the JEM, a generic integer inverse transform is implemented that uses the same basis vector derivation process as the JEM. Please note that as in JEM, transform sizes that are larger than 32 can only include 32 non-zero transform coefficients, with all transform coefficients with row or column index greater than or equal to 32 are set to 0.

### Motion compensation

Same as JEM with added support for new partition sizes.

### Intra prediction

The intra-prediction process is the same as JEM6 with added support for new partition sizes and the following changes:

- The tool combinations described in JVET-G0079 are enabled.

- Unequal Weight Planar Prediction (UW-Planar) are enabled.

- Weighted Angular Prediction for Mode 66 (UW66) are enabled

- Explicit Signaling of RSAF flag and Constrained Adaptive Reference Sample Smoothing (CARSS) are enabled.

- UW-Planar and UW66 modifies Planar mode and Angular mode 66.

- Constraint ARSS only allows ARSS to be applied to a subset of intra modes and RSAF flag is signaled explicitly instead of data hiding.

In addition to the above changes, the proposal also includes a new cross component prediction tool.  This tool is described in the next sub-section.

#### Cross Component Prediction

A cross component linear model prediction process is used in the JEM to predict chroma samples based on the reconstructed luma samples of the same CU. This includes a simple linear model (LM), multiple model LM (MMLM), and multiple filter LM (MFLM). The parameters of these linear models are derived by minimizing the regression error using the reconstructed luma samples and chroma samples in the neighbouring data set.

In JEM, the neighbouring data set for LM is composed of the pairs of luma and chroma samples in line areas B and C as shown in Figure 2.6, and the neighbouring data set for MMLM and MFLM is composed of the pairs of luma and chroma samples in line area B, C, E and F. However, it might not be able to fully reflect the linear relationship between the neighbouring luma and chroma samples to the linear relationships in CU. Therefore, multiple neighbouring data sets for CCLM (MNLM) is proposed to use three neighbouring data sets for LM, MMLM and MFLM to cover various linear relationships between luma samples and chroma samples in CU.

Figure 2.6 Multiple neighbouring data sets are required for various linear relationship between luma and chroma samples in CU

Three neighbouring data sets used in the proposed software are defined as follows:

T0:  B, C 		(used in JEM LM)

T1:  B, C, E, F      	(used in JEM MMLM and MFLM)

T2:  A, B, C, D 	(newly added)

Then the neighbouring data sets and the corresponding binarization for different cross component predictions are shown in the below table:

|  | Binarization | Training data set |
| --- | --- | --- |
| MMLM1 | 1 | T1 |
| LM | 01 | T0 |
| MMLM2 | 001 | T2 |
| MFLM1 | 00001 | T1 |
| MFLM2 | 00010 | T1 |
| MFLM3 | 00000 | T2 |
| MFLM4 | 00011 | T2 |

More various neighbouring data sets are asserted to be able to cover more various luma-chroma linear relationships in CU.

### In-loop filtering

In-loop filtering is the same as the JEM with one modification.  In the proposal, and CTU adaptive band offset method is used.  The CTU adaptive band offset is an extension of HEVC band offset, and it adds offset value based on signaled band position and four continuous offset values. The differences are:

- The band position is adjusted based on min value of the target CTU in addition to signaled band position

- The band range is chosen based on CTU pixel range (i.e. max-min)

While we propose to enable the tool for all sequence times, the motivation for the tool is that the pixel histograms for Cb and Cr are very limited in HDR sequences and therefore the constant band range is not suitable for those sequences. The effectiveness is particularly realized in PQ sequences.

Figure 2.7 Illustration of band position adjustment and band range selection

### Tiles

Tiles are supported in the proposal and includes the ability to operate in the same manner as in HEVC.  Specifically, a picture may be divided into Tiles that can then be encoded in parallel.  These Tiles are then received in raster scan order and maybe decoded by either a parallel or sequential decoder.

In addition to the HEVC Tiles support, the proposal also includes support for a new Tiles structure called an “extractable Tile” (extTile).  This extractable Tile adds an extraction capability functionality to Tile, in which not only parallel decoding in a frame but also independent decoding and/or extraction in a sequence level is supported. The motivation for this functionality is rooted in best supporting 8K or 360 video services that use a 5G network.  In this scenario, it is asserted that only interested regions from the full reconstructed frame sent by the server should be shown.  Of course, this selection could depend on user interaction.

Figure 2.8. Application image

The changes relative to the HEVC Tile design are as follows:

- Non-Restricted Motion Vector in Tile, where the padding is processed in the case of tile boundary rather than picture boundary for all inter predictions (i,e, AMVP, merge, FRUC, ATMVP, Affine….) The padding is implemented by motion vector clipping. If extTile is applied, the motion vector is not clipped by picture boundary but tile boundary.

- Temporal Motion Position Modification in TMVP, FRUC initial vector candidates, ATMVP and STMVP, to ensure referred positions are inside of the current tile. If temporal motion positions are not inside of the current tile, the right-bottom block in collocated block is used.

- Overlapped tile, where some amount tile boundary pixels are covered with two neighboring tiles and used for output by composing pixels from both tile, in order to make tile boundaries less noticeable.

Please note that the proposed method is similar in spirit to the motion constraint tile set (HEVC MCTS).  However, a key difference is that the MCTS design is an encoder only constraint, while here we propose a normative solution.  It is asserted that this will result in easier implementation for both encoders and decoders.

Figure 2.9 extTile (extractable Tile) concept

Figure 2.10 Non-Restricted Motion Vector in Tile

Figure 2.11 Temporal Motion Position Modification

Figure 2.12 Overlapped tile

|  | HEVC motion constraint tile set (MCTS) | Proposal |
| --- | --- | --- |
| Motion vector range | Shall be constrained in encoder side | Non restricted motion vector can be used by tile boundary padding |
| Temporal merge candidates | Shall be disabled in encoder side | Modified position within tile boundary is used in corresponding cases. |
| FRUC, ATMVP and STMVP referencing | N/A | Restricted position within tile boundary is used. |
| Overlapped feature | N/A | Supported to alleviate boundary artifacts |

### Additional tools

None

### Additional algorithmic description topic(s)

None

## 360° Video

No additional tools for 360° coding are proposed.

## HDR

### Post-processing

None

### Quantization handling

Same as the JEM with an added tool that allows the decoder to derive an additional QP offset value for a coding unit based on the average of the predicted pixel values determined by the intra- or inter-prediction processes.  To enable this, the bit-stream contains a look-up table (LUT) to map the average prediction value to a QP offset value.  The decoder then uses the LUT to determine the QP offset at the CU level.  The QP offset is implemented by determining a scale factor and applying the scale factor to the transform coefficients, as shown in Figure 2.13.

Figure 2.13 QP prediction and application process when operating on PQ-HDR content.

The scale factor is received as a QP offset value, and it is then calculated from the equivalent QP offset according to

The scale factor is shifted up by 6 bits to use fixed-point operations. For chroma blocks, the block reuses scaling parameters from collocated luma blocks. For intra-coded regions, where chroma and luma may have different partitions, the scaling parameter is derived from the average equivalent QP offset from all collocated luma blocks.

### Internal Bit Depth Increase (IBDI)

For HDR content, the internal bit depth of the decoder is increased by 1-bit to 11 bits.  This allows decoding the HDR data with fine granularity in luma and chroma components.

|  | JEM (anchor) | Proposal |
| --- | --- | --- |
| IBDI | 10-bit | 11-bit |

### HDR specific decoding tools

None.

# Encoder algorithm description

The goal of this section is to provide a detailed description of the encoder algorithms used in the proposed video codec.  The emphasis of this section is also on the design of the flexible, rectangular partitioning scheme and support for the improved algorithms for motion compensation, loop filtering and Tiles, as are the QP coding and bit-depth management tools developed for high dynamic range content.

## General algorithm description

The algorithm structure has a set of common tools that are proposed to be applied across all content types.  These tools are described in the sub-sections below, while tools specific for high dynamic range video coding are described in a later section.

### Rate-distortion optimization

Same as the JEM with the exception that the internal distortion metrics use a 10-bit distortion calculation  Note that PSNR calculations for the encoder log report is also evaluated in 10-bit.

### Partitioning structure selection

Same as the JEM except that the additional partition sizes are also checked.

Furthermore, to improve the run-time performance of the encoder, some partition checks may be disabled as follows:

- Following the testing of the binary split with half offset: If the best mode of the current CU is the no-split with skip mode and the current binary tree depth is greater than threshold, then further partition testing is omitted

- Following the testing of the binary split with quarter and three quarter offsets and testing the ternary tree: If the best mode of the current CU is the no-split with skip mode and the current binary tree depth is greater than a threshold then further partition testing is omitted

- Following the testing of the ternary split: If the vertical ternary cost is larger than the horizontal ternary cost by a pre-determined factor, then testing of the vertical binary split with quarter and three quarter offsets is not performed.  Similarly, if the vertical ternary cost is less than the horizontal ternary cost by a pre-determined factor, then testing of the horizontal binary split with quarter and three quarter offsets is not performed.

- Quad split testing is omitted if either a binary split or ternary split was tested but the encoder chose not to split the CU.

- Binary splits with quarter offset or three quarter offset are only considered for pictures in the lower temporal layers

Note that these speedup in no way suggest that the tools do not provide coding efficiency improvement in the cases that the encoder currently omits testing.

### Intra prediction mode estimation and type selection

Same as the JEM with the additional limitations that:

- PDPC is applied only to pictures at the lower temporal layers

- EMT is applied only to pictures at the lower temporal layers

- The results of binary split and ternary split RD-check are examined and further splitting of an intra-block is tested only if the number of intra samples in the best RD choice so far exceeds a threshold. The threshold is dependent on temporal layer ID.

### Motion estimation and motion segment selection

#### General

Same as JEM.

#### Usage of decoder-side motion refinement

Same as the JEM with the addition that for bilateral matching, the proposed asymmetric bilateral matching is processed immediately after the symmetric bilateral matching in JEM. The bilateral matching result having the minimum cost is selected.

### Mode decisions

Same a JEM.

### In-loop filtering type selection and parameter estimation

### Same as JEM.Transforms and transform type selection

Same as JEM with the exception that for block sizes not supported in JEM a generic integer transform is implemented. The transform type selection is same as in JEM, except for EMT (see section 3.1.3 above).

### Quantization and quantization type selection

Same as JEM with the exception that the chroma QP offset is set equal to 0 in the random access configuration.

### Entropy coding

Same as JVET-G0112.

### Additional tools

None.

### Additional encoder optimization

The proposal does not rely on additional encoder optimization methods that are not used in the anchor, such as non-automatic means of encoding parameter optimizations, multi-pass encoding, content-adaptive GOP, or special subjective optimization methods.

### Additional algorithmic description topic(s)

## 360°

No additional tools for 360° coding are proposed.

## HDR

### Pre-processing

The response does not use pre-processing.

### HDR specific encoding tools

The HDR encoding tools in the proposal are the same as the JEM.

Please note that this includes the LUT used for mapping an average, predicted luma value to an equivalent QP offset.  It is the same as the default LUT used by the explicit luma adaptive QP control used to generate HDR anchors, where the relationship between luma and QP is expressed as:

| luma intL range | dQP |
| --- | --- |
| intL < 301 | 3 |
| 301 ≤ intL < 367 | 2 |
| 367 ≤ intL < 434 | 1 |
| 434 ≤ intL < 501 | 0 |
| 501 ≤ intL < 567 | -1 |
| 567 ≤ intL < 634 | -2 |
| 634 <= intL < 701 | -3 |
| 701 <= intL < 767 | -4 |
| 767 <= intL < 834 | -5 |
| intL >= 834 | -6 |

Look-up table of the dQP value from the average of the luma value.

### Additional encoder optimization

The following encoder optimizations are used:

- To mirror the proposed decoder, the internal bitdepth of the encoder is increased to 11 bits.

- SAO offset quantization is used by setting log2_sao_offset_scale_luma equal to 3, which effectively signalling SAO offset in 8 bit depth accuracy.

- For HDR PQ sequences coding, when LumaLevelToDeltaQPMode is equal to 2, transform coefficients are scaled before quantization, and scaled back after dequantization.

- The encoder distortion calculation is modified from 8-bit distortion of HM and JEM to 10-bit distortion. PSNR calculation for encoder log report is also evaluated in 10-bit

# Compression performance

We evaluated the performance of the proposed codec using the test conditions prescribed in the Call for Proposals, and we report the results of those tests here.  The section is organized as follows: In 4.1, we provide R-D curves and objective measures for the SDR sequences using tbe random access configuration.  In 4.2, we provide R-D curves and objective metrics for the SDR sequences using the low delay configuration.  In 4.3, we provide R-D curves and objective metrics for the HDR sequences using the random access configuration.

The R-D curves and objective metrics correspond to the bit-streams that were provided for visual evaluation in the middle of Feburary, 2018.  Since delivery of those bit-streams, we have continued to improve and develop our video coding methods, and the result is a further improvement in coding efficiency.  We also provide that objective information here.

For clarity, the configuration of the February and April results are as follows:

| Tool | Short description | February Result | April Result |
| --- | --- | --- | --- |
| Flexible Rectangular Partitioning Structure | Support for quad split, binary split with quarter offset, binary split with half offset, binary split with three quarters offset, and ternary split. | Y | Y |
| Picture Boundary Partitioning | Automatic splitting of CTUs on picture boundaries usng signaled and inferred partitioning. | Y | Y |
| Switchable separate and shared trees for intra coding | Ability for intra-coded regions to use either separate partitioning trees for lum and chrom or a shared partitioning tree for luma an chroma. | Y | Y |
| Bitdepth adjusted cost function in decoder side motion refinement | The cost function used in FRUC is modified to measure distortion more correctly and balance motion cost and distortion in range of bit-depth. 10 bit distortion is newly used in the proposal while 8 bit distortion is used in JEM FRUC module regard less of 10 bit input sequence./ | Y | Y |
| Frame Rate Up Conversion based Merge Mode | Improved frame rate up conversion mdege mode with the following features: (i) asymmetric bilateral matching, (ii) new cost function for bilateral matching with side template matching, and (iii) improvement of selecting between uni-directional and bi-directional modes for template matching. | N | Y |
| Multiple neighboring data sets for cross-component linear model prediction (MNLM) | Multiple neighbouring data sets are used to derive CCLM parameters aiming to reflect various linear relationships between luma samples and chroma samples in CU. | N | Y |
| CTU adaptive band offset (new BO) | Position and band width of SAO band offset is adjusted based on the reconstructed sample range (min value and max) in CTU. | N | Y |
| extractable Tile (extTile) | The extTile provides extraction functionality to Tile, in which not only parallel decoding in a frame but also independent decoding and/or extraction in a sequence level is supported. | N | N |

Additionally, the configuration for the HDR results have the additional tools:

| Tool | Short description | February Result | April Result |
| --- | --- | --- | --- |
| Implicit luma base coefficient scaling in HDR | In HDR category, implicit luma base coefficient scaling is utilized while explicit qp offset is signalled based on luminace in JEM. | Y | Y |
| Internal bit depth increase (IBDI) in HDR | In HDR category, the internal bitdepth is increased to 11-bit to encode the HDR sequences more efficiently with regard to fine granularity in luma and chroma component characteristics. | Y | Y |

Please note that the change in performance between February and April comes from improved encoder design for the existing tools as well some additional coding efficiency improvement from the additional tools.

## SDR: Constraint set 1 configuration relative to HM anchor

Objective results for the random access configuration compared to the HM anchor.

### Class SDR-A (RD-curves)

Figure 4.1.1.1	FoodMarket4

Figure 4.1.1.2	CatRobot1

Figure 4.1.1.3	DaylightRoad2

Figure 4.1.1.4	ParkRunning3

Figure 4.1.1.5	Campfire

### Class SDR-B (RD-curves)

Figure 4.1.2.1	BQTerrace

Figure 4.1.2.2	RitualDance

Figure 4.1.2.3	MarketPlace

Figure 4.1.2.4	BasketballDrive

Figure 4.1.2.5	Cactus

### Overall (BD-rate summary tables)

|  | February Result |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Constraint Set 1 |  |  |  |  |  |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -36.62% | -43.51% | -47.78% | 1.85 | 1.26 | 1.53 |
| CatRobot1 | -41.60% | -55.61% | -48.57% | 1.64 | 1.03 | 1.40 |
| DaylightRoad2 | -43.71% | -61.48% | -46.39% | 1.23 | 1.37 | 0.89 |
| ParkRunning3 | -36.38% | -24.80% | -27.22% | 1.21 | 0.42 | 0.38 |
| Campfire | -38.26% | -34.11% | -51.00% | 1.14 | 1.27 | 1.58 |
| BQTerrace | -35.35% | -49.76% | -58.00% | 1.08 | 0.94 | 1.14 |
| RitualDance | -31.49% | -40.95% | -45.60% | 1.72 | 1.20 | 1.55 |
| MarketPlace | -33.04% | -49.48% | -47.15% | 1.19 | 0.90 | 0.89 |
| BasketballDrive | -35.93% | -50.22% | -46.75% | 1.32 | 1.14 | 1.45 |
| Cactus | -38.92% | -49.84% | -44.81% | 1.64 | 0.82 | 1.19 |
| Average SDR-A | -39.31% | -43.90% | -44.19% | 1.42 | 1.07 | 1.16 |
| Average SDR-B | -34.95% | -48.05% | -48.46% | 1.39 | 1.00 | 1.24 |
| Average all | -37.13% | -45.98% | -46.33% | 1.40 | 1.04 | 1.20 |
| Enc Time[%] | 4517% |  |  |  |  |  |
| Dec Time[%] | 1160% |  |  |  |  |  |
| Max Delta Rate[%] | 0.00% |  |  |  |  |  |
| Min Delta Rate[%] | -0.84% |  |  |  |  |  |
| Max Delta PSNR (YUV) | 2.16 |  | 1.52 |  | 1.86 |  |
| Min Delta PNSR (YUV) | 0.76 |  | 0.38 |  | 0.36 |  |

|  | April Result |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Constraint Set 1 |  |  |  |  |  |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -37.78% | -44.99% | -49.22% | 1.93 | 1.32 | 1.59 |
| CatRobot1 | -44.46% | -58.62% | -51.73% | 1.79 | 1.11 | 1.53 |
| DaylightRoad2 | -46.62% | -63.58% | -49.13% | 1.35 | 1.45 | 0.96 |
| ParkRunning3 | -37.92% | -27.54% | -30.11% | 1.28 | 0.47 | 0.43 |
| Campfire | -38.96% | -34.96% | -52.94% | 1.16 | 1.31 | 1.66 |
| BQTerrace | -35.75% | -50.50% | -58.35% | 1.10 | 0.96 | 1.15 |
| RitualDance | -32.54% | -42.55% | -47.22% | 1.79 | 1.26 | 1.63 |
| MarketPlace | -33.91% | -50.79% | -48.71% | 1.23 | 0.93 | 0.93 |
| BasketballDrive | -36.78% | -51.47% | -47.78% | 1.36 | 1.18 | 1.49 |
| Cactus | -39.40% | -50.40% | -45.49% | 1.66 | 0.84 | 1.22 |
| Average SDR-A | -41.15% | -45.94% | -46.63% | 1.50 | 1.13 | 1.24 |
| Average SDR-B | -35.68% | -49.14% | -49.51% | 1.43 | 1.03 | 1.28 |
| Average all | -38.41% | -47.54% | -48.07% | 1.46 | 1.08 | 1.26 |
| Enc Time[%] | 5133% |  |  |  |  |  |
| Dec Time[%] | 1830% |  |  |  |  |  |
| Max Delta Rate[%] | -0.33% |  |  |  |  |  |
| Min Delta Rate[%] | -6.03% |  |  |  |  |  |
| Max Delta PSNR (YUV) | 2.17 |  | 1.54 |  | 1.93 |  |
| Min Delta PNSR (YUV) | 0.74 |  | 0.40 |  | 0.38 |  |

## SDR: Constraint set 2 configuration relative to HM anchor

### Class SDR-B

Figure 4.2.1.1	BQTerrace

Figure 4.2.1.2	RitualDance

Figure 4.2.1.3	MarketPlace

Figure 4.2.1.4	BasketballDrive

Figure 4.2.1.5	Cactus

### Overall

|  | February Result |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Constraint Set 2 |  |  |  |  |  |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -30.61% | -41.76% | -48.00% | 0.93 | 0.47 | 0.55 |
| RitualDance | -25.56% | -30.60% | -31.56% | 1.34 | 0.71 | 0.85 |
| MarketPlace | -25.35% | -33.63% | -31.82% | 0.83 | 0.41 | 0.43 |
| BasketballDrive | -29.51% | -43.56% | -40.98% | 1.13 | 0.92 | 1.19 |
| Cactus | -31.78% | -38.18% | -34.43% | 1.30 | 0.53 | 0.75 |
| Average SDR-B | -28.56% | -37.55% | -37.36% | 1.11 | 0.61 | 0.75 |
| Enc Time[%] | 2892% |  |  |  |  |  |
| Dec Time[%] | 597% |  |  |  |  |  |
| Max Delta Rate | -0.01% |  |  |  |  |  |
| Min Delta Rate | -1.03% |  |  |  |  |  |
| Max Delta PSNR (YUV) | 1.50 |  | 0.95 |  | 1.22 |  |
| Min Delta PNSR (YUV) | 0.65 |  | 0.33 |  | 0.33 |  |

|  | April Result |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Constraint Set 2 |  |  |  |  |  |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -30.58% | -40.96% | -48.12% | 0.93 | 0.46 | 0.56 |
| RitualDance | -25.73% | -30.52% | -31.78% | 1.35 | 0.71 | 0.86 |
| MarketPlace | -25.81% | -33.02% | -32.41% | 0.85 | 0.41 | 0.44 |
| BasketballDrive | -29.85% | -43.74% | -41.03% | 1.15 | 0.93 | 1.19 |
| Cactus | -32.95% | -38.78% | -34.14% | 1.36 | 0.54 | 0.74 |
| Average SDR-B | -28.99% | -37.40% | -37.50% | 1.13 | 0.61 | 0.76 |
| Enc Time[%] | 3460% |  |  |  |  |  |
| Dec Time[%] | 619% |  |  |  |  |  |
| Max Delta Rate | 0.00% |  |  |  |  |  |
| Min Delta Rate | -3.48% |  |  |  |  |  |
| Max Delta PSNR (YUV) | 1.56 |  | 0.96 |  | 1.23 |  |
| Min Delta PNSR (YUV) | 0.65 |  | 0.32 |  | 0.34 |  |

## SDR: Constraint set 1 configuration relative to JEM anchor

Objective results for the random access configuration compared to the JEM anchor.

### Class SDR-A

Figure 4.3.1.1	FoodMarket4

Figure 4.3.1.2	CatRobot1

Figure 4.3.1.3	DaylightRoad2

Figure 4.3.1.4	ParkRunning3

Figure 4.3.1.5	Campfire

### Class SDR-B

Figure 4.3.2.1	BQTerrace

Figure 4.3.2.2	RitualDance

Figure 4.3.2.3	MarketPlace

Figure 4.3.2.4	BasketballDrive

Figure 4.3.2.5	Cactus

### Overall

|  | February Result |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Constraint Set 1 |  |  |  |  |  |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -5.15% | 0.15% | -2.16% | 0.20 | 0.00 | 0.05 |
| CatRobot1 | -3.93% | -7.78% | -5.72% | 0.11 | 0.08 | 0.11 |
| DaylightRoad2 | -5.79% | -11.58% | -8.90% | 0.11 | 0.15 | 0.13 |
| ParkRunning3 | -7.87% | 0.43% | 0.22% | 0.22 | -0.01 | 0.00 |
| Campfire | -6.29% | 0.66% | -3.03% | 0.14 | -0.02 | 0.06 |
| BQTerrace | -7.26% | -8.88% | -9.52% | 0.17 | 0.12 | 0.11 |
| RitualDance | -5.90% | -6.54% | -8.88% | 0.27 | 0.14 | 0.23 |
| MarketPlace | -5.75% | -5.42% | -5.86% | 0.17 | 0.07 | 0.08 |
| BasketballDrive | -7.18% | -8.96% | -6.08% | 0.20 | 0.14 | 0.14 |
| Cactus | -6.41% | -8.86% | -6.81% | 0.20 | 0.10 | 0.12 |
| Average SDR-A | -5.80% | -3.62% | -3.92% | 0.15 | 0.04 | 0.07 |
| Average SDR-B | -6.50% | -7.73% | -7.43% | 0.20 | 0.11 | 0.14 |
| Average all | -6.15% | -5.68% | -5.68% | 0.18 | 0.08 | 0.10 |
| Enc Time[%] | 567% |  |  |  |  |  |
| Dec Time[%] | 142% |  |  |  |  |  |
| Max Delta Rate[%] |  |  |  |  |  |  |
| Min Delta Rate[%] |  |  |  |  |  |  |
| Max Delta PSNR (YUV) | 0.31 |  | 0.25 |  | 0.35 |  |
| Min Delta PNSR (YUV) | 0.06 |  | -0.12 |  | -0.04 |  |

|  | April Result |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Constraint Set 1 |  |  |  |  |  |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -6.89% | -2.51% | -4.92% | 0.27 | 0.06 | 0.12 |
| CatRobot1 | -8.68% | -14.33% | -11.70% | 0.24 | 0.15 | 0.24 |
| DaylightRoad2 | -10.71% | -16.48% | -13.56% | 0.21 | 0.21 | 0.20 |
| ParkRunning3 | -10.12% | -3.26% | -3.81% | 0.29 | 0.05 | 0.05 |
| Campfire | -7.37% | -0.63% | -6.99% | 0.16 | 0.02 | 0.14 |
| BQTerrace | -7.83% | -10.55% | -10.59% | 0.19 | 0.14 | 0.12 |
| RitualDance | -7.36% | -9.28% | -11.64% | 0.34 | 0.20 | 0.30 |
| MarketPlace | -6.99% | -8.09% | -8.78% | 0.20 | 0.10 | 0.13 |
| BasketballDrive | -8.43% | -11.36% | -7.89% | 0.24 | 0.17 | 0.18 |
| Cactus | -7.15% | -10.07% | -8.13% | 0.22 | 0.11 | 0.15 |
| Average SDR-A | -8.75% | -7.44% | -8.20% | 0.23 | 0.10 | 0.15 |
| Average SDR-B | -7.55% | -9.87% | -9.41% | 0.24 | 0.15 | 0.18 |
| Average all | -8.15% | -8.66% | -8.80% | 0.24 | 0.12 | 0.16 |
| Enc Time[%] | 644% |  |  |  |  |  |
| Dec Time[%] | 223% |  |  |  |  |  |
| Max Delta Rate[%] |  |  |  |  |  |  |
| Min Delta Rate[%] |  |  |  |  |  |  |
| Max Delta PSNR (YUV) | 0.32 |  | 0.27 |  | 0.39 |  |
| Min Delta PNSR (YUV) | 0.04 |  | -0.10 |  | -0.01 |  |

## SDR: Constraint set 2 configuration relative to JEM anchor

Objective results for the low delay configuration compared to the JEM anchor.

### Class SDR-B

Figure 4.4.1.1	BQTerrace

Figure 4.4.1.2	RitualDance

Figure 4.4.1.3	MarketPlace

Figure 4.4.1.4	BasketballDrive

Figure 4.4.1.5	Cactus

### Overall

|  | February Result |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Constraint Set 2 |  |  |  |  |  |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -6.26% | 12.12% | 9.75% | 0.15 | -0.09 | -0.06 |
| RitualDance | -5.00% | 5.75% | 5.59% | 0.23 | -0.11 | -0.13 |
| MarketPlace | -5.98% | 15.68% | 14.60% | 0.18 | -0.14 | -0.15 |
| BasketballDrive | -5.40% | 3.84% | 4.79% | 0.17 | -0.06 | -0.10 |
| Cactus | -5.18% | 7.60% | 8.73% | 0.17 | -0.07 | -0.14 |
| Average SDR-B | -5.57% | 9.00% | 8.69% | 0.18 | -0.10 | -0.12 |
| Enc Time[%] | 311% |  |  |  |  |  |
| Dec Time[%] | 85% |  |  |  |  |  |
| Max Delta Rate |  |  |  |  |  |  |
| Min Delta Rate |  |  |  |  |  |  |
| Max Delta PSNR (YUV) | 0.28 |  | -0.01 |  | 0.00 |  |
| Min Delta PNSR (YUV) | 0.07 |  | -0.20 |  | -0.22 |  |

|  | April Result |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Constraint Set 2 |  |  |  |  |  |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -6.23% | 13.89% | 8.81% | 0.15 | -0.10 | -0.06 |
| RitualDance | -5.17% | 6.18% | 5.48% | 0.24 | -0.12 | -0.12 |
| MarketPlace | -6.56% | 16.77% | 13.45% | 0.20 | -0.15 | -0.14 |
| BasketballDrive | -5.89% | 3.54% | 4.73% | 0.18 | -0.05 | -0.10 |
| Cactus | -6.93% | 6.19% | 9.03% | 0.23 | -0.06 | -0.14 |
| Average SDR-B | -6.16% | 9.32% | 8.30% | 0.20 | -0.10 | -0.11 |
| Enc Time[%] | 372% |  |  |  |  |  |
| Dec Time[%] | 88% |  |  |  |  |  |
| Max Delta Rate |  |  |  |  |  |  |
| Min Delta Rate |  |  |  |  |  |  |
| Max Delta PSNR (YUV) | 0.33 |  | -0.01 |  | -0.01 |  |
| Min Delta PNSR (YUV) | -0.02 |  | -0.19 |  | -0.23 |  |

## HDR: Constraint set 1 configuration relative to HM anchor

Objective results for the random access configuration compared to the HM anchor.

### Class HDR-A

Plot of PSNR vs Bitrate are shown below for the HLG sequences.

Figure 4.5.1.1	DayStreet

Figure 4.5.1.2	PeopleInShoppingCenter

Figure 4.5.1.3	SunsetBeach

### Class HDR-B

Plot of wPSNR vs Bitrate are shown below for the PQ sequences.

Figure 4.5.2.1	Market

Figure 4.5.2.2	ShowGirl2

Figure 4.5.2.3	Hurdles

Figure 4.5.2.4	Starting

Figure 4.5.2.5	Cosmos

### Overall

February Results

April Results

## HDR: Constraint set 1 configuration relative to JEM anchor

Objective results for the random access configuration compared to the JEM anchor.

### Class HDR-A

Plot of PSNR vs Bitrate are shown below for the HLG sequences.

Figure 4.6.1.1	DayStreet

Figure 4.6.1.2	PeopleInShoppingCenter

Figure 4.6.1.3	SunsetBeach

### Class HDR-B

Plot of wPSNR vs Bitrate are shown below for the PQ sequences.

Figure 4.6.2.1	Market

Figure 4.6.2.2	ShowGirl2

Figure 4.6.2.3	Hurdles

Figure 4.6.2.4	Starting

Figure 4.6.2.5	Cosmos1

### Overall

February Results

April Results

## 360° Video: Constraint set 1 configuration relative to HM anchor

No additional tools for 360° coding are proposed.

## 360° Video: Constraint set 1 configuration relative to JEM anchor

No additional tools for 360° coding are proposed.

# Decoder complexity analysis

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

Decoding time comparisons with an HM16.16 decoder are provided below.  To compute these numbers, we employed two measurement methodologies.  For the first methodology, we configured both the HM16.16 and proposed decoder with YUV output enabled and reference input disabled.  For the second methodology, we configured the HM16.16 and proposed decoder with YUV output disabled.

| Decoding time vs HM16.16 | Proposed Decoder (with write) | Proposed Decoder (without write) |
| --- | --- | --- |
| SDR CS1 | 9.47 x | 11.6 x |
| SDR CS2 | 5.78 x | 5.97 x |
| HDR | 10.23 x | 12.9 x |

## Description of computing platform used to determine decoding times reported in section 5.1

The decoder executable was compiled under Linux 64bit using GCC 4.4.7. All decoding time measurements were done on a heterogeneous cluster with clock speeds in the range of 2.6-3.0GHz, where each decoder instance ran as a single thread.

## Memory usage of decoder

The memory usage of the decoder is as follows:

| Memory usage | HM16.16 | JEM7.0 | Proposed decoder (GB) |
| --- | --- | --- | --- |
| SDR CS1 | 1.0 G | 1.3 G | 1.9 G |
| SDR CS2 | 0.24G | 0.37 G | 0.9 G |
| HDR | 1.0 G | 1.3 G | 1.9 G |

## Complexity characteristics of decoder entropy decoding operation

Same as JEM.

## Complexity characteristics of decoder inverse quantization

Same as JEM.

## Complexity characteristics of decoder inverse transform operation

Same as JEM. The added non-power of 2 transforms are no larger than the transforms in JEM and can be implemented using a same generic matrix-multiply function. Additional transform coefficient tables are required for the proposed new transform sizes.

## Complexity characteristics of decoder-side motion refinement

The complexity of decoder-side motion refinement is increased in proportion to the increment of the number points of search patterns. In the minimum case, which indicates the best MV is selected in the first search round, the number of search points is increased up to four times relative to JEM. However, in worst case, the total complexity of the proposed method is limited because the maximum search round is equal to three, while the maximum search round is unlimited in JEM.

## Complexity characteristics of decoder motion compensation

Same as JEM.

## Complexity characteristics of decoder intra-frame prediction operation

Similar to JEM.  (Note intra-coding configuration in 2.1.8 is derived from earlier JEM models but not the same as JEM7.)

## Complexity characteristics of decoder in-loop filtering operation

Same as JEM.

## Complexity characteristics of additional decoder tools

#### extTile (extractable Tile)

The complexity of the extTile feature is increased by 1) padding operation in tile boundary, 2) position check and 3) increased pixel count by tile overlapping and restoration process of duplicated region.

## Complexity characteristics of 360° video specific decoding tools

No additional tools for 360° coding are proposed.

## Complexity characteristics of HDR specific decoding tools

QP inference requires computing the average of luma in each predicted block, a table lookup operation, and scaling of transform coefficients.

IBDI necessitates wider data paths (11 vs 10 in most cases) and a 10% increase in reference frame storage size

## Degree of capability for decoder parallel processing

Same as JEM with the addition that the proposal also includes an extractable tile feature.  This feature better supports decoder parallel architectures where accessing the memory of other parallel decoding units is burdensome.

# Encoder complexity analysis

## Encoding time and measurement methodology

Encoding of proposed encoder is done in parallel. Encoding log files are parsed for each IRAP segment, and the total encoding time are calculated by adding the encoding time of all parallelism segments and then subtracting the encoding times of the first pictures for all segments except the first.   The relative encoding time ratios for the proposal compared to HM anchors is as follows:

| Encoding time vs HM16 | Proposed encoder |
| --- | --- |
| SDR CS1 | 45.2x |
| SDR CS2 | 28.9x |
| HDR | 37.4x |

## Description of computing platform used to determine encoding times reported in section 6.1

The encoder executable was compiled under Linux 64bit using GCC 4.4.7. Encoding time measurements were done on a heterogeneous cluster with clock speeds in the range of 2.6-3.0GHz.  For CS1, bitstreams were coded with parallel encoding.

## Memory usage of encoder

The memory usage of the encoder is as follows:

| Memory usage | HM16.6 | JEM7.0 | Proposed Encoder (GB) |
| --- | --- | --- | --- |
| SDR CS1 | 2.1 G | 6.4 G | 8.0 G |
| SDR CS2 | 0.3 G | 1.0 G | 2.3 G |
| HDR | 2.1 G | 6.4 G | 8.0 G |

## Complexity characteristics of encoder intra-frame prediction type selection

Similar to JEM.  (Note intra-coding configuration in 2.1.8 is derived from earlier JEM models but not the same as JEM7.)

In addition, the RDO cost calculation is increased to support multi neighboring linear model (MNLM) when selecting the chroma intra mode.

## Complexity characteristics of encoder motion estimation and motion segmentation selection

Same as JEM.

### General

### Complexity characteristics of usage of decoder-side motion refinement

The complexity of decoder-side motion refinement is increased in proportion to the increment of the number points of search patterns. In the minimum case, which indicates the best MV is selected in the first search round, the number of search points is increased up to four times relative to JEM. However, in worst case, the total complexity of the proposed method is limited because the maximum search round is equal to three, while the maximum search round is unlimited in JEM.

## Complexity characteristics of encoder transforms and transform type selection

Same as JEM with the exception that for block sizes not supported in JEM a generic integer transform is implemented. The transform type selection is same as in JEM, except for EMT which is disabled for temporal layers higher than a threshold.

## Complexity characteristics of encoder quantization and quantization type selection

Same as JEM.

## Complexity characteristics of encoder in-loop filtering type selection and parameter estimation

Same as JEM.

## Complexity characteristics of encoder entropy coding type selection

Same as JEM.

## Complexity characteristics of additional encoder tools

When extTile is enabled, encoding complexity may be reduced by the decreased search range at the tile boundary.

## Complexity characteristics of 360° video specific encoding tools

No additional tools for 360° coding are proposed.

## Complexity characteristics of HDR specific encoding tools

QP inference requires computing the average of luma in each predicted block, a table lookup operation, and scaling of transform coefficients.

IBDI necessitates wider data paths (11 vs 10 in most cases), though for CPU applications the most 16-bit designs of the JEM do not require modification.  Additionally, IBDI requires a 10% increase in reference frame storage.

## Degree of capability for encoder parallel processing

Same as JEM with the addition that the proposal includes the extractable tile feature.  This feature better supports encoder parallel architectures where accessing to the memory of other parallel encoding units is burdensome and allows sequence level parallelization.

# Algorithmic characteristics

## Random access characteristics

Same as JEM.

## Delay characteristics

Same as JEM.

## Additional characteristics discussion topic(s)

None.

# Software implementation description

The software implementation is written in C++ and is based on JEM. Some functions, mainly transforms, were optimized for speed by using SIMD intrinsics. Additional speed improvements were achieved by avoiding unnecessary dynamic memory allocations, and inlining of small functions. Overall, implementation optimizations may contribute to reducing encoding time by about 10%.

Added tools and optimizations can be switched on or off at compile time using #defines.

# Highlighted aspects discussion

The video coding system described in this document emphasizes a flexible, rectangular partitioning structure.  The flexibility of this structure comes from several features, including (i) support for a number of rectangular partition sizes, (ii) adaptive use of different partitioning strategies for luma and chroma in intra-coded regions, (iii) method for inferring the partitioning at picture boundaries, and (iv) a tile partitioning strategy that better supports parallel decoding of high resolution video.  We observe that this adaptive and flexible structure provides high coding efficiency, and that the coding efficiency increases as the picture size increases.  We assert that this makes the approach both well suited for the increasing global deployment of 4K content, as well as the announced intention to transition to 8K applications in several regions.

We also propose new tools for improved intra-prediction, loop filtering and motion compensation processes, and high dynamic range coding.  We observe that these methods provide further gains in coding efficiency.  Specifically, we observe that the proposed decoder side motion vector derivation process provides over 1.5% bit-rate reduction beyond state-of-the-art. Moreover, the proposed collection of tools emphasizing HDR coding, including a QP inference and bit-depth expansion technology, are well suited for content represented in the PQ transfer function and also provide combined gains in excess of 1.5% .

# Closing remarks

This response to the Call for Proposal response focuses on improved coding efficiency for the SDR and HDR categories.  It emphasizes a block based approach with a coding structure that is asserted to be more flexible than the previous HEVC standard, and this structure is combined with additional contributions in the areas of motion coding, loop filtering and intra coding.  Additionally, for the HDR category, tools in the area of quantization and bit-depth management are incorporated.  Using the evaluation procedure defined in the Call, we observe coding gains relative to an HEVC anchor of 41.2% and 35.7% for 4K-SDR and HD-SDR sequences, respectively, using the random access configuration; 29.0% for HD-SDR sequences using a low delay configuration, and gains of 34.3% and 32.2% for PQ-HDR and HLG-HDR sequences, respectively, using a random access configuration. With a demonstrated performance exceeding 40.0% and a potential to have even higher coding efficiency in emerging applications that use resolutions in excess of 4k, we assert that the approach provided in this proposal provides a strong starting point for developing a next generation standard and propose its use in the first test model.

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Foxconn Technology Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: Questionnaire

## Codec level Q&A

| Codec Level Q&A |  | HM | This proposal |
| --- | --- | --- | --- |
| 1 | Maximum number of reference frames used by encoder | 4 | 4 |
| 2 | Maximum coding unit (e.g. CTU in HEVC) | luma 64×64 chroma 32×32 | luma 128×128 chroma 64×64 |
| 3 | Minimum and maximum transform block size | luma 4×4/32×32 chroma 4×4/16×16 | luma 4×4/128×128 chroma 2×2/64×64 |
| 4 | Minimum and maximum intra prediction block | luma 4×4 chroma 4×4 | luma 4×4 chroma 2×2 |
| 5 | Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length | luma 4×8/8×4 (uni-pred) and 8×8 (bi-pred), 8-tap Chroma 2×4/4×2 (uni-pred) and 4×4 bi-pred, 4-tap | luma 4×4 (uni/bi-pred), 8-tap Chroma 4x4 (uni/bi-pred), 4-tap |
| 6 | List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category) | n/a | template matching in FRUC, LIC |
| 7 | List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples, position unforeseeable) | n/a | 64-bit arithmetic for LIC, BIO and LMC |
| 8 | List all the tools that use non-rectangular (e.g. triangle) partitions | n/a | n/a |
| 9 | Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa) | n/a | n/a |
| 10 | Indicate whether luma and chroma use separate partitioning structures | no | Yes, they may use separate partitioning structures in intra-coded regions |
| 11 | Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes) | Entropy parsing depends on intra prediction mode reconstruction, which requires an intra MPM list derivation | Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation |

## Tool level Q&A

### Category 1. Partitioning structure

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 1. Partitioning structure |  | HM | This proposal |
| 1.a | Whether the prediction unit partitioning is different from transform unit partitioning | Yes, PU and TU partitioning for a CU can differ | No |
| 1.b | Maximum tree depth | CU maximum depth 4 TU maximum depth 3 PU maximum depth 1 | QT maximum depth 5 BT maximum depth 3 |

### Category 2. Entropy coding

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 2. Entropy coding |  | HM | This proposal |
| 2.a | Total number of context models | 186 | 329(JEM)+12 |
| 2.b | Memory storage size for each context model | 8-bit | 30-bit |
| 2.c | Range table size if table look up is used for range update | LPSTable[64×4×8bits]; RenormTable[32× 3bits] | LPSTable[512×64×9bits]; RenormTable[128×3bits] |
| 2.d | Initialization table size | 3×186×8 bits | 3×(329+12)×24 + (329+12)×15×5 bits |
| 2.e | Other memory associated with context model adaptation | LPS transition table: 64*6 bits MPS transition table: 64*6 bits | n/a |

### Category 3. Control data derivation process

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 3. Control data derivation process |  | HM | This proposal |
| 3.a | Temporal motion data storage granularity | 16×16 | 4×4 |
| 3.b | Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-directional reference picture are used, count this picture as 2) | 1 | 4 |

### Category 4. Decoder-side motion refinement

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 4. Decoder-side motion refinement |  | HM | This proposal |
| 4.a | Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement. | n/a | - BIO 2×2D separable 6 taps filter (for gradients), mults per sample in refinement calculation 5+5*4 +1/8+2, minimum processing unit same as regular bi-pred, no SAD calculation - FRUC minimum processing unit 44, sequential PU followed by sub-PU level refinement, number of SAD calculations and additional MC interpolations per unit for MV refinement is increased but constrained in the worst case; bi-linear filter at MV refinement stage |
| 4.b | Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size. | n/a | - DMVR needs to access (H+7+2)×(W+7+2) MC instead of (H+7)×(W+7) in HEVC MC - FRUC needs to access (H+7+16)x(W+7+16)x3 instead of (H+7)×(W+7) in HEVC MC. Note that the number of access in FRUC in JEM is unlimited. |

### Category 5. Intra prediction

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 5. Intra prediction |  | HM | This proposal |
| 5.a | Line buffer size in terms of luma and chroma sample lines | luma 1, chroma 1 | luma 4, chroma 2 |
| 5.b | List all the filters used for intra prediction | Reference sample smoothing: {1,2,1}, Strong intra smoothing filter: 2-tap bilinear filter (only for 32×32 luma TU) Interpolation: 2×32×5 bits (bi-linear 32 phases) | Reference sample smoothing: {1,2,1}, {3,7,14,16,14,7,3}, {4,9,12,14,12,9,4} Interpolation: 2×4×32×8 bits (4 taps, 2 sets, 32 phase) |
| 5.c | Indicate whether there are cross-component dependencies in intra prediction reconstruction | n/a | LM-CHROMA (Cb depends on Y, and Cr depends on Cb&Y). This includes MNLM. |

### Category 6. Inter prediction

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 6. Inter prediction |  | HM | This proposal |
| 6.a | Motion vector accuracy | luma ¼ pel chroma 1/8 pel | luma 1/16 pel chroma 1/32 pel |
| 6.b | Storage size for a motion vector component | 16-bit | 16-bit |
| 6.c | List all the interpolation filters used for luma/chroma motion compensation | luma 8-tap, 8×4×6 bits chroma 4-tap, 4×8×6 bits | luma 8-tap, 8×16×6 bits chroma 4-tap, 4×32×6 bits |

### Category 7. Quantization

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 7. Quantization |  | HM | This proposal |
| 7.a | Storage size associated with quantization | n/a | n/a |

### Category 8. Transforms

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 8. Transforms |  | HM | This proposal |
| 8.a | List all the combinations of transform block size and transform type for primary transforms | - 8-bit 4×4, 8×8, 16×16 and 32×32 DCT-II, - 8-bit 4×4 DST VII | HEVC transform mode (10-bit DCT-II), transform size combinations: - (4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 48, 52, 56, 64, 72, 96, 128) × (4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 48, 52, 56, 64, 72, 96, 128) TUs for luma CTUs - (4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 48, 52, 56, 64, 72, 96, 128) × (4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 48, 52, 56, 64, 72, 96, 128) TUs for inter-coded chroma CTUs - (2, 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 48, 52, 56, 64) × (2, 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 48, 52, 56, 64) TUs for intra-coded chroma CTUs EMT transform mode (10-bit DST VII, DCT-V, DCT-VIII, DST-I), only applied to CTUs up to 64×64, transform size combinations: - (4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 48, 52, 56, 64) × (4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 48, 52, 56, 64) TUs for luma CTUs |
| 8.b | List all the combinations of transform block size and transform type for secondary transforms | n/a | 35×3 4×4 and 8×8 non-separable Hypercube-Givens Transform (HyGT) |
| 8.c | List all the non-separable transforms used | n/a | 4×4/8×8 HyGT |
| 8.d | Indicate whether transforms can be fully/partially implemented using direct matrix multiply | partial factorization equivalent to matrix multiplication | factorized form not equivalent to matrix multiplication |
| 8.e | List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output) | n/a | 4×4 HyGT 2-interations, 8×8 HyGT 4-interations |

### Category 9. In-loop filtering

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 9. In-loop filtering |  | HM | This proposal |
| 9.a | List line buffer size in terms of luma and chroma lines (if any) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) ALF (luma 8, chroma 4) |
| 9.b | List all the filters used for in-loop filtering | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. ALF: 9×9/7×7/5×5 diamond filter support for luma; and 5×5 diamond filter support for chroma |
| 9.c | Minimum processing unit size | Deblocking (luma: 8×8 grid with 4×4 filter decision, chroma 8×8 grid with 4×4 filter decision) | Deblocking (luma 4×4 grid with 4×4 filter decision, chroma 8×8 grid with 2×2 filter decision) ALF: for luma, filter decision is based on 2×2 block Bilateral filter: sample level filter switch (luma only) |

### Category 10. “Tool-on” tests

The objective measurements in Section 4 illustrate the combined performance of the proposed tools.  Here though, we provide supplemental performance information, where we report the performance of each tool individually.  The list of tools to be considered is provided below.

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 10. “tool-off” and “tool-on” tests |  | HM | This proposal |
| 10.a | List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools | None | In addition to JEM tools: General - Binary tree with quarter and three quarter offsets - Ternary tree - Shared separate intra tree - Picture boundary partitioning - FRUC merge mode - MNLM HDR - Inferred QP - IBDI 11-bit with SAO rounding - New band offset Functionality - Tiles |

The performance of the flexible rectangular partitioning an FRUC merge mode are provided in Table 2.  Results are for the random access configuration, and as can be seen from the Table, results are provided independently for Class A and Class B.  For the flexible rectangular partitioning tools, the anchor is the proposed software configured with the majority of proposed and JEM tools disabled.  One notable exception is that both the test and anchor includes a binary tree with half split offset.  For the FRUC merge mode, the anchor is the proposed software configured with other proposed tools disabled, but the JEM tools enabled.  This means that the FRUC algorithm in the JEM software is enabled in both the anchor and test.

Table 2. Summary of tool performance for tools in the proposal

| Tested tool name | “tool-on” test results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Class A |  |  |  |  | Class B |  |  |  |  |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| Binary tree with quarter and three quarter offsets | -4.97% | -6.94% | -7.13% | 432% | 127% | -5.55% | -9.16% | -9.49% | 414% | 156% |
| Ternary Tree | -4.22% | -5.48% | -6.07% | 394% | 128% | -4.77% | -7.04% | -7.03% | 413% | 158% |
| Picture boundary partitioning | -0.17% | -0.15% | -0.08% | 100% | 102% | -0.78% | -1.28% | -1.35% | 101% | 101% |
| FRUC Merge Mode (vs. JEM-like) | -2.06% | -2.77% | -3.16% | 125% | 338% | -1.16% | -2.04% | -1.79% | 128% | 310% |

The performance of tools with an HDR emphasis is reported in Table 3.  Here, results are reported for the inferred QP tool, the new band offset, and IBDI 11-bit with SAO rounding.  The anchor is the proposed software configured with the majority of the JEM and proposed tools disabled, with the notable exception that the test and anchor includes a binary tree with half split offset.  We report the performance as measured using the wPSNR metric.

Table 3. Summary of tool performance for HDR focuses tools in the proposal

| Tested tool name | “tool-on” test results |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Class HDR |  |  |  |  |  |
|  | BD-rate |  |  | Runtime |  |  |
|  | Y | Cb | Cr |  | Enc | Dec |
| Infer QP (HDR) | -1.3% | -0.9% | -1.2% |  | 100% | 100% |
| new BO (HDR) | 0.1% | -1.4 % | -2.2 % |  | 101% | 100% |
| IBDI 11-bit with SAO rounding (HDR vs. JEM-like) | -0.2% | -5.5% | -9.7% |  | 104% | 101% |

The performance of MNLM is reported in Table 4.  The anchor is the proposed software configured with other proposed tools disabled, but the JEM tools enabled.

Table 4. Summary of tool performance for intra focused tools in the proposal

| Tested tool name | “tool-on” test results |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Results (Class A&B) |  |  |  |  |  |
|  | BD-rate |  |  | Runtime |  |  |
|  | Y | Cb | Cr |  | Enc | Dec |
| MNLM (AI, vs. JEM-like) | -0.09% | -1.05% | -1.36% |  | 100% | 100% |

Finally, the performance of the extractable Tile is reported in Table 5.  The anchor is the proposed software configured with all tools on, and the test configures the extractable Tile feature with 8 tiles in a 4x2 layout.  Results are reported for Class A, as the goal of the tool is to better support the coding of higher resolution content.

Table 5. Summary of tool performance for functionality focused tools in the proposal

| Tested tool name | “tool-on” test results |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Class B |  |  |  |  |  |
|  | BD-rate |  |  | Runtime |  |  |
|  | Y | Cb | Cr |  | Enc | Dec |
| extTile (4x2,classA) | 2.2% | 2.0% | 2.0% |  | 96% | 103% |

[end of document]