| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J1026 |
| --- | --- |

| Title: | Description of Core Experiment 6: Transforms and transform signalling |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Amir Said, Xin Zhao | Email: | asaid@qti.qualcomm.com xinzzhao@tencent.com |
| Source: | CE coordinators |  |  |

____________________________

# Introduction

The purpose of this Core Experiment (CE) is to investigate the coding tools related to transforms and transform signaling on top of the Test Model. In this CE, the tradeoff between compression efficiency and complexity of different transformation tools will be investigated. In addition to encoder and decoder run-times, other complexity metrics, e.g., operation counts, storage, transform core bit-precision and internal bit-precision will be also evaluated. The list of tools and experiments to be performed are detailed in this document.

# Participant list

| Name | Affiliation | Email | P/C |
| --- | --- | --- | --- |
| Amir Said Hilmi E. Egilmez | Qualcomm | asaid@qti.qualcomm.com hegilmez@qti.qualcomm.com | P/C |
| Xin Zhao Xiang Li Liang Zhao | Tencent | xinzzhao@tencent.com xlxiangli@tencent.com leolzhao@tencent.com | P/C |
| Mischa Siekmann Heiko Schwarz Santiago de Luxán Hernández | HHI | mischa.siekmann@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de santiago.de.luxan@hhi.fraunhofer.de | P/C |
| Pierrick Philippe | Orange | pierrick.philippe@orange.com |  |
| Fabrice Le Léannec Karam Naser | Technicolor | fabrice.leleannec@technicolor.com karam.naser@technicolor.com | P/C |
| Kiho Choi | Samsung | kiho14.choi@samsung.com | P/C |
| Kiyofumi Abe Kanoh Ryuichi | Panasonic | abe.kiyo@jp.panasonic.com kanoh.ryuichi@jp.panasonic.com | P/C |
| Jungwon Kang Sung-Chang Lim | ETRI | jungwon@etri.re.kr sclim@etri.re.kr | P/C |
| Yung-Lyul Lee | Sejong Univ. | yllee@sejong.ac.kr |  |
| Takeshi Tsukuba Masaru Ikeda | Sony | takeshi.tsukuba@sony.com masaru.ikeda@sony.com | P/C |
| Kei Kawamura Yoshitaka Kidani Sei Naito | KDDI | ki-kawamura@kddi.com yo-kidani@kddi.com se-naitou@kddi.com | P/C |
| Chih-Wei Hsu Yu-Wen Huang Ching-Yeh Chen Han Huang Tzu-Der Chuang Man-Shu Chiang | MediaTek | cw.hsu@mediatek.com yuwen.huang@mediatek.com chingyeh.chen@mediatek.com h.huang@mediatek.com peter.chuang@mediatek.com man-shu.chiang@mediatek.com | P/C |
| Moonmo Koo Mehdi Salehifar Seung Hwan Kim Jaehyun Lim | LG Electronics | moonmo.koo@lge.com mehdi.salehifar@lge.com seunghwan3.kim@lge.com jaehyun.lim@lge.com | P/C |
| Yin Zhao Jianle Chen | Huawei | yin.zhao@huawei.com jianle.chen@huawei.com | P/C |
| Y. Lin Q. Yu J. Zheng | HiSilicon | linyongbing@hisilicon.com yuquanhe@hisilicon.com zhengjianhua@hisilicon.com | P/C |
| Yukinobu Yasugi Takeshi Chujoh Tomohiro Ikai | Sharp | yasugi.yukinobu@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |  |
| Kiran Misra | Sharp | misrak@sharplabs.com |  |

# Core Experiment tasks

The following tasks shall be performed in this Core Experiment (CE):

- Evaluate compression efficiency, computational complexity and memory of transforms studied in this CE;

- Investigate interactions between primary and secondary transform schemes;

- Report the results and conclusions of these experiments.

Experiments are going to be organized according to the following aspects:

- Primary transform process

- Secondary transform process

- Primary and/or secondary transform signaling

# Description of tools

The following tools will be considered in this CE, as summarized in below table.

| Tool # | Proponent | Related Docs. | Summary of the tool | Cross-checkers |
| --- | --- | --- | --- | --- |
| 1 | Qualcomm | JVET-J0021 (CfP) JVET-C0022 JVET-D0126 | Primary transformation: In addition to transform types of AMT in JEM7, DST-4 and Identity transform are also used for intra prediction residual. For inter prediction residual, KLT-1 and KLT-2 are used to replace AMT for small block sizes. | 1. Orange 2. Samsung 3. |
| 2 | Qualcomm | JVET-J0021 (CfP) JVET-D0120 | Secondary transformation: NSST can be applied only when AMT transform flag or index is equal to 0. NSST transform choice is separately signaled for Cb and Cr chroma components. | 1. Samsung 2. HHI 3. |
| 3 | Qualcomm | JVET-J0066 | AMT complexity reduction: The AMT transforms are implemented using the DCT-2 family and adjustment stages. | 1. Orange 2. Technicolor 3. ETRI 4. Samsung 5. Sony 6. Tencent |
| 4 | Qualcomm | JVET-J0062 | Hierarchically Structured Matrix-based Transforms: HyGT based NSST is replaced by hierarchically structured matrix-based transforms (HSMTs). | 1. Technicolor 2. Samsung 3. HHI 4. Tencent 5. |
| 5 | Tencent | JVET-J0054 | Coupled primary and secondary transform: AMT and NSST is coupled and signaled by only one transform index and applied for both luma and chroma components. | 1. Technicolor 2. Qualcomm 3. Samsung 4. Sony 5. |
| 6 | HHI | JVET-J0014, JVET-J0040 | Set of transforms: A set of 5 predefined transform candidates, and each candidate specifies a primary horizontal transform, a primary vertical transform, and a non-separable secondary transform. The transform candidates depend on the block size and the prediction mode. | 1. Qualcomm 2. 3. |
| 7 | Technicolor | JVET-J0022 (CfP) | Transform Reduction: It is proposed to reduce transform types in AMT to DCT-8, DST-4 and DST-7. DST-7 is implemented based on the DCT-8 with flipping and sign changes. | 1. Qualcomm 2. 3. |
| 8 | ETRI/Sejong Univ. | JVET-J0013 (CfP) | Residual flipping: It is proposed to replace the AMT with DCT-II and DST-VII using four types of residual flipping. | 1. Qualcomm 2. 3. |
| 9 | Samsung | JVET-J0024, JVET-J0025 | Reduced AMT transform types: The AMT transforms in JEM7 are replaced with a reduced set of transforms using DCT-8 and DST-7. | 1. Qualcomm 2. 3. |
| 10 | Samsung | JVET-J0024, JVET-J0025 | Primary transformation restriction: When block width or height is over 32 pixels, AMT is not used or signaled. | 1. Qualcomm 2. Tencent 3. |
| 11 | Samsung | JVET-J0024, JVET-J0025 | Secondary transformation: NSST is modified with reduced sets based on grouping intra prediction modes and signaling with on/off flag with 4 modes. | 1. Qualcomm 2. HHI 3. Tencent 4. |
| 12 | Panasonic | JVET-J0020 | AMT and NSST complexity reduction: 1) NSST is used only when DCT2 is used as primary transform. 2) DST7 is used as the horizontal and vertical transform without signaling when width or height is less than or equal to 4. | 1. 2. 3. |
| 13 | Sony | JVET-J0028 | Adaptive Multiple core Transforms for Luma and Chroma: It is proposed to apply AMT, which includes transform replacement matrix, for both luma and chroma. A transform type is signaled by a transform pair index, which indicates horizontal and vertical transform. | 1. 2. 3. |
| 14 | Sony | JVET-J0028 | Transform Matrix Replacement: It is proposed to replace transform matrix of DCT-8, DST-1 and DCT-5 used in JEM with that of alternate transforms as below: - Flipped DST-7 is used instead of DCT8 - Transposed DST-7 is used instead of DST-1 DCT-2 is used instead of DCT-5 | 1. 2. 3. |
| 15 | Tencent | JVET-J0029 (CfP) | Matrix multiplication based NSST: It is proposed to use matrix multiplication based NSST. | 1. 2. |
| 16 | KDDI | JVET-J0016 | Shrink Transform: When the transform block size is equal or larger than 64, inverse transform is realized by two steps: inverse transform with half size and up-sampling. | 1. MediaTek 2. 3. |
| 17 | MediaTek | JVET-J0018 | Adaptive multi-transform (AMT) with constraints: AMT transform type is reduced to DCT-II, DST-VII and flipped DST-VII. AMT is only allowed when both CU width and height are smaller than or equal to 64. For some block sizes, the number of AMT candidates is reduced. | 1. 2. 3. |
| 18 | MediaTek | JVET-J0018 | Non-separable secondary transform (NSST) with constraints: The NSST index signaling depends on luma/chroma, number of nonzero DC/AC coefficient of upper-left 8x8 or 4x4. |  |
| 19 | LG Electronics | JVET-J0017(CfP) | Primary transform: Only DST7 and DCT8 are used for AMT. AMT is applied up to 32. DST7 and DCT8 are implemented using FFT |  |
| 20 | LG Electronics | JVET-J0017(CfP) | 4x4 Secondary transform: 4x4 Layered Givens Transform (LGT) is tested. |  |
| 21 | LG Electronics | JVET-J0017(CfP) | 8x8 Secondary transform: 8x8 Reduced Secondary Transform (RST) keeping 16 bases of the 64 dimensional space (16x64 matrix) with NSST index modification based on RST modification. | 1. Huawei |
| 22 | LG Electronics | JVET-J0017(CfP) | NSST index coding: - NSST index coding update based on RST modification. - Transform kernel mapping between intra prediction modes and transform set. | 1. Huawei |
| 23 | Huawei | JVET-J0024 | Spatial varying transform: For an inter-predicted CU, a TU of half size of the CU may be used, and in this case the TU shape and position information are signaled. | 1. MediaTek |
| 24 | HiSilicon | JVET-J0064 | Prediction dependent Transform - Intra prediction mode dependent transform for luma and chroma - Position dependent transform for residual of HEVC merge mode - DST-7 transform for residual of template matching mode |  |
| 25 | Tencent | JVET-J0029 (CfP) | Block size dependent zero-out transform: When 64-point, 128-point and 256-point inverse primary transform is applied, only coefficients of the first 32, 16 and 4 are used and the remaining coefficients are not used and considered as 0. |  |

Detailed description on each coding tool tested in this CE are provided below.

## Primary Transformation: Extended AMT Transforms [JVET-J0021 (CfP), JVET-C0022, JVET-D0126]

The AMT transforms in JEM7 are replaced with a new set of transforms. In addition to transforms used in JEM, which are DCT-2, DCT-5, DCT-8, DST-1, and DST-7, two more cores are added for each intra and inter coded blocks. Namely, DST-4 and identity transform are added to intra, KLT-1 and KLT-2 are added to inter. It was proposed in JVET-C0022 and JVET-D0126, respectively.

To accommodate more flexible partitioning, where a block size is not power of 2, such as 4x24 and 8x48, corresponding transform cores are included.

In intra coding, the following 7 transform sets, where each set has 4 transform pairs, are defined:

T0, intra = { (DST-4, DST-4), (DST-7, DST-7), (DST-4, DCT-8), (DCT-8, DST-4) }

T1, intra = { (DST-7, DST-7), (DST-7, DCT-5), (DCT-5, DST-7), (DST-1, DCT-5) }

T2, intra = { (DST-7, DST-7), (DST-7, DCT-8), (DCT-8, DST-7), (DCT-5, DCT-5) }

T3, intra = { (DST-4, DST-4), (DST-4, DCT-5), (DCT-8, DST-4), (DST-1, DST-7) }

T4, intra = { (DST-4, DST-7), (DST-7, DCT-5), (DCT-8, DST-7), (DST-1, DST-7) }

T5, intra = { (DST-7, DST-7), (DST-7, DCT-5), (DCT-8, DST-7), (DST-1, DST-7) }

T6, intra = { (DST-7, DST-7), (DST-7, DCT-5), (DCT-5, DST-7), (DST-1, DST-7) }.

The first entry in the transform pair determines vertical transform, and the second entry specifies the horizontal transform.

A look-up table is defined to assign each of the 7 transform sets based on different intra prediction modes and block sizes.

Identity transform is applied for blocks that are not exceeding 16x16 and have intra modes within the proximity of horizontal and vertical intra directions, the proximity is defined by a block size-based threshold. If transform index is equal to 3 and block satisfies the above condition, the horizontal and/or vertical identity transform is applied.

In inter coding, the following 2 transform sets are used, where each set has 4 transform pairs defined as follows:

T0, inter = { (DCT-8, DCT-8), (DCT-8, DST-7), (DST-7, DCT-8), (DST-7, DST-7) }

T1, inter = { (KLT-1, KLT-1), (KLT-1, KLT-2), (KLT-2, KLT-1), (KLT-2, KLT-2) }.

The transform set with KLTs (i.e., T1, inter) is used for blocks that are smaller than or equal to 16x16, and for larger blocks the transforms from T0, inter set are used.

### Test Scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.1.1 | Extended AMT Transforms | Qualcomm | Orange Samsung |

## Secondary Transformation: NSST Signaling [JVET-J0021 (CfP), JVET-D0120]

The non-separable secondary transform (NSST) is the same as in JEM7, including the JEM NSST signaling, with the following three modifications:

- To support additional intra modes, which can be up to 131 directions in JVET-J0021, the look-up table that maps intra mode to NSST transform was extended, where each entry in the table was simply duplicated.

- To lower the complexity, the restriction is imposed on AMT and NSST use that NSST can be applied and signaled only when AMT transform index is equal to 0, it also happens when AMT flag is equal to 0.

- NSST transform choice is separately signaled for Cb and Cr chroma components.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.2.1 | Conditional NSST signaling | Qualcomm | Samsung HHI |

## Primary Transformation: AMT complexity reduction [JVET-J0066]

This is a technique proposed to reduce the complexity of AMT by providing an efficient alternative that approximates the AMT transforms using only a transform of the DCT-2 family and adjustment stages of low complexity. The proposed adjustment stages are defined using sparse block-band orthogonal matrices, which provide good approximation for the set of AMTs used in JEM7.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.3.1 | AMT complexity reduction | Qualcomm | Orange Technicolor ETRI Samsung Sony Tencent |

## Hierarchically Structured Matrix-based Transforms [JVET-J0062]

This contribution presents hierarchically structured matrix-based transforms (HSMTs) for non-separable secondary transformation (NSST) as alternatives to HyGT-based NSST implementations. The proposed set of HSMTs reduces the NSST memory use and provides very similar coding gains when the AMTs in JEM7 are replaced with HSMTs.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.4.1 | Hierarchically Structured Matrix-based Transforms | Qualcomm | Technicolor Samsung HHI Tencent |

## Coupled primary and secondary transform [JVET-J0054]

It is proposed to couple AMT and NSST, the usage of AMT and NSST is indicated by a single transform index. Instead of signaling the indices of primary and secondary transform independently, the primary and secondary transform is coupled and signaled by only one transform index. In addition, the proposed method is applied for both luma and chroma components.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.5.1 | Coupled primary and secondary transform | Tencent | Technicolor Qualcomm Samsung Sony |

## Primary and Secondary Transformation: Set of transforms [JVET-J0014, JVET-J0040]

The transform is selected among a set of 5 predefined transform candidates for each CU. Each of the transform candidates specifies a primary horizontal transform, a primary secondary transform, and a non-separable secondary transform (which may also be equal to the identity transform). The ordered list of transform candidates depends on the block size and the prediction mode. The transform chosen is signaled as follows: When cbf is equal to 1, a flag is transmitted which specifies whether the first transform in the candidate list is used. When this flag is equal to 0, the following applies: If the number of non-zero transform coefficient levels is greater than a threshold, another syntax element is transmitted which indicates the transform candidate used; otherwise, the second transform candidate of the list is used.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.6.1 | Set of transforms | HHI | Qualcomm |

## Primary Transformation: Transform Reduction [JVET-J0022 (CfP)]

In JVET-J022, it is proposed to reduced the set of primary transforms to DCT-2, DCT-8, DST-4, DST-7. The AMT set thus is reduced to DCT-8, DST-4 and DST-7. Moreover, DST-7 is implemented based on the DCT-8 transform matrix, on which flipping, and sign changes operations are applied.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.7.1 | Transform Reduction | Technicolor | Qualcomm |

## Primary transformation: Transform Replacement [JVET-J0013 (CfP)]

It is proposed to replace the adaptive multiple transform (AMT) with DCT-II and DST-VII with four types of residual flipping in order to reduce the number of transform matrix compared to AMT. The four types of residual flipping candidate are as follows: No-Flip (no flipping), H-Flip (horizontal flipping), V-Flip (vertical flipping), HV-Flip (horizontal & vertical flipping). For intra coding, one of the residual flipping candidates in the pre-defined residual flipping candidate set for the given intra prediction mode is selected based on the signalled index. For inter coding, one of four residual flipping candidates is selected based on the signalled index. While four additional transforms are needed for AMT as primary transformation, one additional transform and residual flipping are needed in the proposed method. On residual flipping, inverse residual flipping can be implemented by simply reordering the output of inverse DST-VII transform without any additional process.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.8.1 | Residual flipping | ETRI/Sejong Univ. | Qualcomm |

## Primary transformation: Transform Replacement [JVET-J0024,JVET-J0025]

The AMT transforms in JEM7 are replaced with a reduced set of transforms using DCT-8 and DST-7.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.9.1 | Reduced AMT transform types | Samsung | Qualcomm |

## Primary transformation: Transform Restriction [JVET-J0024,JVET-J0025]

For AMT transforms, if width or height is over 32 pixels in length, the block is not considered to be applied for AMT and not signaled to decoder

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.10.1 | Primary transform restriction | Samsung | Qualcomm Tencent |

## Secondary transformation: Transform Signaling [JVET-J0024, JVET-J0025]

The non-separable secondary transform (NSST) with the following two modifications:

- Reduced angle sets based on grouping intra prediction mode

- Signaling with 1 on/off flag with 4 modes

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.11.1 | Modified NSST sets and signaling | Samsung | Qualcomm HHI Tencent |

## Primary and Secondary Transformation: AMT and NSST Complexity reduction [JVET-J0020]

This contribution proposes to reduce the complexity with the following two modifications. 1) NSST is used only when DCT2 is used as primary transform.  2) AMT (Hor or Ver) selects DST7 without signaling when width or height is less than or equal to 4 independently.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.12.1 | AMT and NSST complexity reduction | Panasonic |  |

## Primary Transformation: AMT for Luma and Chroma [JVET-J0028]

It is proposed to apply AMT, which includes transform replacement matrix, for both luma and chroma. A transform type is signaled by a transform pair index, which indicates horizontal and vertical transform.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.13.1 | Chroma AMT | Sony |  |

## Primary Transformation: Transform Replacement [JVET-J0028]

It is proposed to replace transform matrix of DCT-8, DST-1 and DCT-5 used in JEM with that of alternate transforms as below:

- Flipped DST-7 is used instead of DCT8

- Transposed DST-7 is used instead of DST-1

DCT-2 is used instead of DCT-5

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.14.1 | Transform Matrix Replacement | Sony |  |

## Secondary Transformation: Matrix multiplication based NSST [JVET-J0029 (CfP)]

It is proposed to use matrix multiplication based NSST.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.15.1 | Matrix multiplication based NSST | Tencent |  |

## Primary Transformation: Shrink Transform  [JVET-J0016]

When the transform block size is equal or larger than 64, inverse transform is realized by two steps; inverse transform with half size and up-sampling. The up-sampling ratio is twice with DCT based coefficient, which is the same as in SHVC. Otherwise (the transform block size is less than 64), inverse transform is the same as in the test model.The following aspects are required to evaluate the proposal. Maximum transform size is configured to 64x64 and 128x128. QP range is set as normal (22, 27, 32, and 37) and lower (like 12 or 17) to confirm the effect by the upsampling filter. Proposal is turned on and off. Complexity in hardware implementation or software implementation (SIMD implementation) is also required to be studied.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.16.1 | Shrink Transform | KDDI | MediaTek |

## Primary Transformation: Adaptive multi-transform (AMT) with constraints [JVET-J0018]

In J0018, the adaptive multi-core transform (AMT) can select the horizontal transform and vertical transform from DCT-II, DST-VII and flipped DST-VII (FDST-VII). An AMT flag is defined. AMT flag equal to “0” indicates that DCT-II is used for both horizontal transform and vertical transform, AMT flag equal to “1” indicates that another transform type is used depending on an indicator, AMT index. In J0018, AMT is only allowed when both CU width and height are smaller than or equal to 64. AMT flag is signaled when the number of non-zero coefficients is larger than a threshold. For inter CUs, the threshold is set to 2. For intra CUs, it is set to 0. AMT index is signaled if the number of non-zero coefficients is larger than 2; otherwise, it is inferred as 0. Moreover, for a CU with width, W, and height, H, the number of AMT index is reduced.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.17.1 | AMT with constraints | MediaTek |  |

## Non-separable secondary transform (NSST) with constraints [JVET-J0018]

In J0018, the process of non-separable secondary transform (NSST) is the same as JEM-7.0, but the syntax is different. In intra slices, for an intra luma CU, an NSST index needs to be signaled when the number of non-zero coefficients of upper-left 8x8 or 4x4 luma component is larger than 1, while for an intra chroma CU, an NSST index is signaled when the number of non-zero AC coefficients of upper-left 8x8 or 4x4 chroma components is larger than 0. In inter slices, for an intra luma CU, an NSST index is signaled when the number of non-zero coefficients of upper-left 8x8 or 4x4 luma plus the number of non-zero AC coefficients of upper-left 8x8 or 4x4 chroma components is larger than 2, while for an intra chroma CU, the NSST index is dependent on luma and not signaled. When coding the NSST index, the codeword is the same as JEM-7.0.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.18.1 | NSST with constraints | MediaTek |  |

## Primary transformation: AMT replacement and restriction [JVET-J0017(CfP)]

For core transform, simplified explicit multiple core transform is utilized by adding two transforms (DST7 and DCT8) on top of HEVC, and maximum EMT is restricted to a size of 32. A forward N×N DST7 with 2N + 1 length Discrete Fourier Transform (DFT) is implemented to achieve the N×N DST7. 2N+1 FFT has been reformulated to a two dimensional FFT. DCT8 can be derived from DST7 with sign changes and reordering just before and after the DST7 computation. Hence, DST7 is reused in this implementation for DCT8.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.19.1 | AMT using only DST7 and DCT8 | LG Electronics |  |

## Secondary transformation: Transform replacement [JVET-J0017(CfP)]

For smaller blocks, a Givens rotation based transform called LGT is applied on 4×4 blocks at the top-left 4×4, 4×8, or 8×4 region of a coded block. This consists of 5 Givens rotation layers (GRLs) and 6 permutations. Each GRL is composed of 8 Givens rotations and each permutation shuffles transform inputs or outputs of the previous stage.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.20.1 | Layered Givens Transform (LGT) | LG Electronics |  |

## Secondary transformation: Transform replacement [JVET-J0017 (CfP)]

An 8x8 reduced secondary transform (RST) is proposed. The main idea of a Reduced Transform (RT) is to map an N dimensional vector to an R dimensional vector in a different space, where R/N (R < N) is the reduction factor. In this contribution, the 8×8 RST with a reduction factor of 1/4 is applied. The forward 8x8 RST uses 16×64 matrices so that it produces non-zero coefficients only in the lower 4×4 region within the given 8×8 region. In other words, if RST is applied (i.e. ST_idx is greater than zero) then the lower 8×8 region except the lower 4×4 region (this region is called ROI region) will have to generate only zero coefficients.

As a result, the ST_idx coding method based on the distribution of non-zero coefficients in the ROI region is introduced. Specifically, ST_idx is not coded when RST is used as a secondary transform and any non-zero element is detected within the ROI region because it implies that RST was not applied. In such a case, ST_idx is inferred to be zero.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.21.1 | Reduced Secondary Transform (RST) | LG Electronics | Huawei |

## Secondary transformation: NSST index coding [JVET-J0017 (CfP)]

NSST index coding update based on RST modification. Transform kernel mapping between intra prediction modes and transform set.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.22.1 | NSST index coding | LG Electronics | Huawei |

## Primary Transformation: Spatial varying transform[JVET-J0024]

For an inter-predicted CU with cbf_Y equal to 1, a svt_flag is signaled to indicate whether the whole residual block or a subpart of the residual block is decoded. The latter case is called a spatial varying transform (SVT), in which a part of the residual block is coded and the rest of the residual block is forced to be zero.

When SVT is used for a CU, SVT type and SVT position information are further decoded from the bitstream. There are two SVT types and three SVT positions, as indicated in Fig X. For SVT-V (or SVT-H), the TU width (or height) equals to half of the CU width (or height), and three TU positions are equally spaced as (i/4) of the CU width (or height) to the top-left corner of the CU, where i=0, 1 and 2. The three positions are hence denoted as position 0, 1 and 2.

Both TU and CU boundaries are filtered in de-blocking filtering. SVT-V (or SVTH) is allowed when CU width (or height) is in the range of [8, 64]. To ensure the gap between two filtered boundaries are not smaller than 4 pixels, Position 1 of SVT-V (or SVT-H) is not allowed when CU width (or height) equals to 8.

Position-dependent core transform is applied in SVT. The three positions are associated with different core transforms. More specifically, DCT-8 is used for position 0 while DST-7 is used for position 1 and 2.

Fig. X Two types of SVT TU

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.23.1 | Spatially varying transform | Huawei | MediaTek |

## Primary Transformation: Additional Transforms [JVET-J0064]

The following 3 aspects will be tested in the CE

- Intra prediction mode dependent transform for luma and chroma

- Position dependent transform for residual of HEVC merge mode

- DST-7 transform for residual of template matching mode.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.24.1 | Prediction dependent transform | HiSilicon |  |

## Primary Transformation: Block size dependent zero-out transform [JVET-J0029]

When 64-point, 128-point and 256-point inverse primary horizontal transform is applied, only coefficients of the first 32, 16 and 4 columns are used and the remaining coefficients are not used and considered as 0. When 64-point, 128-point and 256-point inverse primary vertical transform is applied, only coefficients of the first 32, 16 and 4 rows are used and the remaining are not used and considered as 0.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.25.1 | Block size dependent zero-out transform | Tencent |  |

# Test conditions

The following test conditions shall be used for this CE:

## Software

The Test Mode [1] shall be used for these experiments, as specified in JVET-J10xx [2].

## Test configurations

This CE shall use the same encoder and decoder configurations as described in JVET-J10xx [2]. Experiments shall be run without and with enabling the tools in Benchmark Set (BMS). For experiments where the transform precision is involved, results using low-QP configurations, i.e., QP 5, 10, 15 and 20 shall be provided as supplemental information.

## Test sequences

The test sequences as described in JVET-J10xx [2] shall be used for experiments.

# Evaluation Criteria

## Quantitative metrics

To obtain objective rate-distortion measurements, the Test Model [1] and the Benchmark Set (BMS) will be used as two separate anchors. In the BMS anchor, all the coding tools in the BMS are enabled. Proposals will be evaluated using BD rates, encoder and decoder run-time, and experiments will be performed under Common Test Conditions (CTC) as described in JVET-J10xx[2].

## Qualitative evaluation

If sequences coded using a proposed alternative transform exhibit subjective improvements or degradations, proponents shall describe these subjective differences.

## Complexity metrics

For the understanding of complexity, the following shall be provided:

- Operations counts (number of multiplies and additions) for individual transforms

- Memory requirements for storing transform cores

  - Total memory for sets of several transforms

  - Amount of memory needed to compute one transform

- Specify the minimum bit-precision required by the transforms

  - To represent transform coefficients

  - Precision of arithmetic operations needed by the transform

- Other operations and memory requirements relevant to the proposed tool, e.g., scaling matrix modification in quantization

- Encoder and decoder times obtained on the same computing platform for both reference and proposed tools should be provided.

- The bit depth (e.g., 16-bit arithmetic) for intermediate data representation

# Participant task checklist

Participants shall:

- Write source code or receive source code for their corresponding tool. Typically, proponents write the source code and cross-verifiers receive it.

- Examine the software to determine if it is implementing the proposed technique;

- Compile the software and run the tests described in this CE;

- Cross-verifiers shall compare their results to those provided by the proponent;

- Comment on the proposed technique and on the results.

# Timeline

| T1: 2018, May-XX: | Release of the reference software implementation |
| --- | --- |
| T2: 2018, May-04: | Final CE description uploaded |
| T3: 2018, June-01: | Proponents release software packages for the experiments |
| T4: 2018, June-15: | Proponents and cross-checkers report results to the CE coordinators |
| T5: 2018, July-XX: | CE report uploaded |

# References

[1] Test Model

[2] Common Test Condition