| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July, 2018 | Document: JVET-K0027-v2 |
| --- | --- |

| Title: | CE7 Summary Report on Quantization and Coefficient Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Heiko Schwarz, Muhammed Coban | Email: | heiko.schwarz@hhi.fraunhofer.de mcoban@qti.qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The CE report summarizes the test results and cross check reports for the CE7 on quantization and coefficient coding. The following subtests are included in the CE:

Subtest 7.1:	Comparison of 4 proposals for entropy coding of transform coefficient levels;

Subtest 7.2:	Comparison of dependent quantization and sign data hiding;

Subtest 7.3:	Investigation of 3 approaches related the derivation and signalling of quantization step sizes;

Subtest 7.4:	Investigation of 1 approach for modifying the scanning order of transform coefficients;

Subtest 7.5:	Comparison of two configurations for transform domain sign prediction.

# Introduction

The purpose of this core experiment is to explore the coding efficiency and complexity impact of proposed algorithms for coding transform coefficient levels and quantization. Various methods have been proposed for quantization and coding of the residual signal transform coefficient levels. This core experiment evaluates the impact of:

- Quantization of transform coefficient

- Context modelling/selection/coding order for syntax elements related to transform coefficients

- Scanning of transform coefficient levels

The objectives of this CE include:

- Investigation of coding efficiency and complexity of quantization schemes.

- Investigation of the impact of context selection and signalling schemes for transform coefficient levels on the coding efficiency and complexity.

- Investigation of coding efficiency and complexity of transform coefficient scanning order methods.

# Participants List

| Affiliation | Person, Email address | P | C |
| --- | --- | --- | --- |
| Qualcomm | Muhammed Coban (mcoban@qti.qualcomm.com) Dmytro Rusanovskyy (dmytror@qti.qualcomm.com) Jie Dong (jiedong@qti.qualcomm.com) | x | x |
| HHI | Heiko Schwarz (heiko.schwarz@hhi.fraunhofer.de) Tung Nguyen (tung.nguyen@hhi.fraunhofer.de) | x | x |
| Samsung | Yinji Piao (yj1003.piao@samsung.com) Kiho Choi (kiho14.choi@samsung.com) | x | x |
| LG | Seung Hwan Kim (seunghwan3.kim@lge.com) Mehdi Salehifar (mehdi.salehifar@lge.com) Moonmo Koo (moonmo.koo@lge.com) Jaehyun Lim (jaehyun.lim@lge.com) |  | x |
| Huawei | Alexey Filippov (alexey.filippov@huawei.com) Jianle Chen (Jianle.chen@huawei.com) Cheung Auyeung (cheung.auyeung@huawei.com) Yang Haitao (haitao.yang@huawei.com) Yin Zhao (yin.zhao@huawei.com) Sergey Ikonin (Sergey.Ikonin@huawei.com) Biao Wang (biao.wang@huawei.com) Vasily Rufitskiy (vasily.rufitskiy@huawei.com) Alexander Karabutov (karabutov.alexander@huawei.com) | x | x |
| Orange | Pierrick Philippe (pierrick.philippe@orange.com) Felix Henry (felix.henry@orange.com) |  | x |
| Sony | Takeshi Tsukuba (Takeshi.Tsukuba@sony.com) Karl Sharman (karl.sharman@eu.sony.com) |  | x |
| MediaTek | Yu-Wen Huang (yuwen.huang@mediatek.com) Chih-Wei Hsu (cw.hsu@mediate.com) Ching-Yeh Chen (chingyeh.chen@mediatek.com) T.-D. Chuang (peter.chuang@mediatek.com) Shih-Ta Hsiang (shih-ta.hsiang@mediatek.com) Mohammed Sarwer (mohammed.sarwer@mediatek.com) | x | x |
| ETRI | Jungwon Kang (jungwon@etri.re.kr) Sung-Chang Lim (sclim@etri.re.kr) |  | x |
| KDDI | Sei Naito (se-naitou@kddi.com) Kei Kawamura (ki-kawamura@kddi.com) Yoshitaka Kidani (yo-kidani@kddi.com) | x | x |
| NHK | S. Iwamura (iwamura.s-gc@nhk.or.jp) | x |  |
| Sharp | Yukinobu Yasugi (yasugi.yukinobu@sharp.co.jp) Takeshi Chujoh (chujoh.takeshi@sharp.co.jp) Tomohiro Ikai (ikai.tomohiro@sharp.co.jp) Kiran Misra (misrak@sharplabs.com) |  |  |
| InterDigital | Xiaoyu Xiu (xiaoyu.xiu@interdigital.com) Yuwen He (Yuwen.He@InterDigital.com) Saurav Bandyopadhyay (Saurav.Bandyopadhyay@InterDigital.com) |  |  |
| Technicolor | Fabien Racape (fabien.racape@technicolor.com) Ya Chen (Ya.chen@technicolor.com) Edouard Francois (Edouard.francois@technicolor.com) Philippe de Lagrange (Philippe.delagrange@technicolor.com) Leleannec Fabrice (Fabrice.Leleannec@technicolor.com) |  | x |
| Canon | Chris Rosewarne (chris.rosewarne@cisra.canon.com.au) Andrew Dorrell (andrew.dorrell@cisra.canon.com.au) |  |  |
| Tencent | Xin Zhao (xinzzhao@tencent.com) Liang Zhao (leolzhao@tencent.com) Meng Xu (mengxxu@tencent.com) Xiang Li (xlxiangli@tencent.com) |  |  |
| Arris | Yue Yu (Yue.Yu@arris.com) Seungwook Hong (Seungwook.Hong@arris.com) Limin Wang (Limin.Wang@arris.com) |  |  |
| Alibaba | Yu-Chen Sun (yuchen.s@alibaba-inc.com) |  |  |
| Chips&Media | D. Y. Kim (brad.kim@chipsnmedia.com) |  |  |
| Broadcom | Minhua Zhou (minhua.zhou@broadcom.com) Wade Wan (wade.wan@broadcom.com) Peisong Chen (peisong.chen@broadcom.com) |  |  |
| Foxconn | Yao-Jen Chang (yaojen.chang.foxconn@gmail.com) |  |  |
| WILUS Inc. | Dongcheol Kim (dongcheol.kim@wilusgroup.com) |  |  |
| Hulu | Min Gao (min.gao@hulu.com) |  |  |
| Panasonic | Tadamasa Toma (toma.tadamasa@jp.panasonic.com) Kiyofumi Aba (abe.kiyo@jp.panasonic.com) |  |  |

P: proponent, C: cross-checker

# Tools under Test

The following tools is considered in this CE.

|  | Proponent | Document | Tool description |
| --- | --- | --- | --- |
| 1 | Qualcomm | JVET-J0021 | Coefficient Coding (JEM 7.0) |
| 2 | HHI | JVET-J0014 | Coefficient Coding |
| 3 | HHI | JVET-J0014 | Coefficient Coding with dependent quantization |
| 4 | Samsung | JVET-J0024 | Coefficient Coding |
| 5 | Huawei | JVET-J0024 | Transform Domain Residual Sign Prediction |
| 6 | Huawei | JVET-J0025 | Adaptive quantization step size scaling |
| 7 | Huawei | JVET-J0025 | Coefficient Coding (JEM 7.0) with complexity reduction [Note: The tool is not described in JVET-J0025] |
| 8 | MediaTek | JVET-J0018 | Extension of QP value range |
| 9 | MediaTek | JVET-J0018 | Derivation of chroma QP from luma QP |
| 10 | KDDI | JVET-J0016 | Block size dependent coefficient scanning |
| 11 | NHK | JVET-J0027 | Coefficient reordering for transform-skipped and bi-predicted CU |

# List of Contributions

| Proposal Document # | Author(s) | Title |
| --- | --- | --- |
| JVET-K0044 | A. Filippov, A. Karabutov, V. Rufitskiy, J. Chen (Huawei) | CE7: Residual sign prediction in transform domain (Tests 7.5.1 and 7.5.2) |
| JVET-K0069 | M. Coban, J. Dong, T. Hsieh, M. Karzewicz (Qualcomm) | CE7: Coefficient Coding (Test 1.1) |
| JVET-K0071 | H. Schwarz, T. Nguyen, D. Marpe, T. Wiegand (Fraunhofer HHI) | CE7: Transform coefficient coding and dependent quantization (Tests 7.1.2, 7.2.1) |
| JVET-K0138 | Y. Piao, W. Choi, C. Kim (Samsung) | CE7.1.3: Scan Region-based Coefficient Coding |
| JVET-K0140 | Y. Zhao, H. Yang, J. Chen (Huawei) | CE7: Adaptive quantization step size scaling (Test 7.3.1) |
| JVET-K0251 | S.-T. Hsiang, S.-M. Lei (MediaTek) | CE7.3.2: Extension of quantization parameter value range |
| JVET-K0252 | S.-T. Hsiang, S.-M. Lei (MediaTek) | CE7.3.3: Derivation of chroma QP from luma QP |
| JVET-K0321 | C. Auyeung, J. Chen (Huawei) | CE 7.1.4: JEM 7.0 coefficient coding with complexity reduction |
| JVET-K0398 | Y. Kidani, K. Kawamura, S. Naito (KDDI) | CE7: Block size dependent coefficient scanning (CE7.4.1) |
| CE-related contributions |  |  |
| JVET-K0070 | M. Coban, J. Dong, M. Karczewicz (Qualcomm) | Non-CE7: Entropy Coding for Dependent Quantization |
| JVET-K0072 | H. Schwarz, T. Nguyen, D. Marpe, T. Wiegand (Fraunhofer HHI) | Non-CE7: Alternative entropy coding for dependent quantization |
| JVET-K0281 | C. Rudat, P. Haase, H. Schwarz, D. Marpe, T. Wiegand (HHI) | Inter-Component Context Modelling for Coded Block Flag |
| JVET-K0310 | P. Yin, S. McCarthy, F. Pu, T. Lu, W. Husak, T. Chen (Dolby), T. Tsukuba (Sony) | AHG13: Sign Data Hiding |
| JVET-K0319 | J. Dong, M. Coban, M. Karczewicz (Qualcomm) | CE7-related: TCQ with high throughput coefficient coding |
| JVET-K0424 | H. Schwarz (Fraunhofer HHI) | Cross-check of JVET-K0070: Non-CE7: Entropy Coding for Dependent Quantization |
| JVET-K0428 | M. Coban (Qualcomm) | Cross-check of JVET-K0072: Non-CE7: Alternative entropy coding for dependent quantization |

# Test Description

## Software and reference configurations

All tools have to be integrated into the Benchmark software.  Results have to be reported for the following two configurations:

- Test model software (by turning off the benchmark tools via configuration file)

- Benchmark software (benchmark tools are enabled, except JEM coefficient coding)

In total, the following four configurations of the software have to be run:

- VTM reference configuration:

The unmodified Benchmark software is used and all benchmark tools are disabled as specified in the configuration files “encoder_xxx_vtm1.cfg”.

- BMS reference configuration:

The unmodified Benchmark software is used and all benchmark tools are enabled (with the exception specified below) as specified in the configuration files “encoder_xxx_bms1.cfg”.

For subtests 7.1 and 7.2 (i.e., for the subtests that evaluate alternative methods for transform coefficient coding), the JEM coefficient coding is disabled by setting the configuration parameter “AltResiComp” equal to 0.

- VTM test configuration:

The Benchmark software with the integrated tool is used and all benchmark tools are disabled as specified in the configuration files “encoder_xxx_vtm1.cfg”.

The tool to be tested is enabled. (Note: This configuration is the same as the VTM reference configuration with the only difference that the tool to be tested is additionally enabled.)

- BMS test configuration:

The Benchmark software with the integrated tool is used and all benchmark tools are enabled (with the exception specified below) as specified in the configuration files “encoder_xxx_vtm1.cfg”.

For subtests 7.1 and 7.2 (i.e., for the subtests that evaluate alternative methods for transform coefficient coding), the JEM coefficient coding is disabled by setting the configuration parameter “AltResiComp” equal to 0.

The tool to be tested is enabled. (Note: This configuration is the same as the BMS reference configuration with the only difference that the tool to be tested is additionally enabled.)

A separate integration of the investigated tools into the VTM software is not required for the core experiment. The Benchmark software with all benchmark tools disabled and the VTM software produce the same results, with the exception that the Benchmark software codes some additional flags in the sequence parameter set.

## Test conditions

The common test conditions are used for all experiments. Proponent may provide additional results.

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-rate).  In particular for coding tools that are mainly designed for improving perceptual quality, proponents are encouraged to provide additional quality metrics (such as SSIM or MsSSIM).

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the reference configurations);

- Decoding time (compared to the reference configuration).

In addition, proponents are encouraged to provide the following (but not limited to) information for complexity and entropy throughput analysis:

- Number of context models

- Number of context-coded and bypass bins

- Table size of scanning paths

- Analysis of the frequency of context switches

- Analysis of the dependency of context modelling between two consecutive bins

- Analysis of entropy parsing dependency (whether bits to bins conversion depends on complex reconstruction and/or pixel processing process)

- Analysis of entropy de-binarization dependency (whether bins to symbols conversion depends on complex reconstruction and/or pixel processing process)

# Subtest 7.1:  Coefficient coding

The average test results for subtest 7.1 are summarized in the following tables. Table 1 reports the results relative to VTM-1 and BMS-1. Table 2 reports the results relative to BMS-1 with HEVC coefficient coding.

Table 1: Average test results for subtest 7.1 (relative to VTM-1 and BMS-1).

| averages for subtest 7.1 |  | over VTM-1 (BMS-1 in VTM configuration) |  |  |  |  | over BMS-1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | test 7.1.1 | -1.34% | -1.07% | -1.14% | 105% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | test 7.1.2 | -1.75% | -1.67% | -1.84% | 102% | 100% | -0.36% | -0.93% | -0.12% | 94% | 98% |
|  | test 7.1.3 (1) | -0.87% | 0.04% | 0.01% | 105% | 106% | 0.14% | 0.97% | 0.93% | 100% | 102% |
|  | test 7.1.3 (2) | -0.92% | -0.08% | -0.10% | 102% | 102% | 0.08% | 0.78% | 0.79% | 99% | 101% |
|  | test 7.1.4 | -1.30% | -1.06% | -1.09% | 105% | 101% | 0.05% | 0.04% | 0.04% | 100% | 100% |
| RA | test 7.1.1 | -0.87% | -0.59% | -0.64% | 103% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | test 7.1.2 | -1.16% | -1.05% | -1.04% | 101% | 100% | -0.25% | -0.66% | 0.01% | 95% | 99% |
|  | test 7.1.3 (1) | -0.34% | 0.34% | 0.34% | 101% | 105% | 0.30% | 1.01% | 0.87% | 98% | 102% |
|  | test 7.1.3 (2) | -0.49% | 0.02% | 0.04% | 100% | 101% | 0.12% | 0.63% | 0.61% | 98% | 100% |
|  | test 7.1.4 | -0.87% | -0.58% | -0.60% | 106% | 100% | 0.05% | 0.04% | -0.05% | 101% | 100% |
| LB | test 7.1.1 | -0.59% | -0.42% | -0.32% | 105% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | test 7.1.2 | -0.89% | -1.46% | -0.11% | 101% | 101% | -0.27% | -0.79% | 1.01% | 96% | 97% |
|  | test 7.1.3 (1) | 0.58% | 1.16% | 1.23% | 100% | 105% | 0.84% | 1.25% | 1.51% | 96% | 99% |
|  | test 7.1.3 (2) | 0.03% | -0.23% | -0.09% | 99% | 100% | 0.45% | 0.29% | 0.46% | 96% | 100% |
|  | test 7.1.4 | -0.60% | -0.59% | -0.29% | 105% | 100% | -0.04% | -0.02% | 0.13% | 100% | 100% |
| - results reported by cross checker (EP init in tested version, but not in references) - results reported by tester (EP init in both tested version and references) |  |  |  |  |  |  |  |  |  |  |  |

Table 2: Average test results for subtest 7.1 (relative to BMS-1 with HEVC coefficient coding). The given ratios for the encoder and decoder times represent estimated values, obtained by the measures ratios relative to BMS-1 and the measured ratios between BMS-1 and BMS-1 with HEVC coefficient coding.

| averages for subtest 7.1 |  | over BMS-1 with HEVC coefficient coding |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT (estimate) | DecT (estimate) |
| AI | test 7.1.1 | -0.82% | -0.84% | -0.85% | 108% | 102% |
|  | test 7.1.2 | -1.17% | -1.77% | -0.97% | 101% | 100% |
|  | test 7.1.3 (1) | -0.68% | 0.12% | 0.07% | 108% | 103% |
|  | test 7.1.4 | -0.77% | -0.80% | -0.81% | 108% | 102% |
| RA | test 7.1.1 | -0.57% | -0.57% | -0.40% | 107% | 101% |
|  | test 7.1.2 | -0.81% | -1.23% | -0.38% | 101% | 100% |
|  | test 7.1.3 (1) | -0.27% | 0.43% | 0.46% | 105% | 103% |
|  | test 7.1.4 | -0.51% | -0.54% | -0.45% | 108% | 101% |
|  | test 7.1.1 | -0.33% | -0.47% | -0.33% | 106% | 102% |
|  | test 7.1.2 | -0.59% | -1.25% | 0.68% | 102% | 99% |
|  | test 7.1.3 (1) | 0.50% | 0.77% | 1.17% | 101% | 101% |
|  |  | test 7.1.4 |  | -0.37% |  | -0.50% |  | -0.21% |  | 106% |  | 102% |  |
| - results reported by cross checker (EP init in tested version, but not in reference) |  |  |  |  |  |  |  |  |  |  |  |  |  |

## Test 7.1.1:  Coefficient coding (JEM 7.0)

### Brief tool description

Transform units are divided into 4x4 coefficient groups. Last coefficient position coding uses HEVC scheme. Coefficient levels are coded in bitplane by bitplane using a neighboring coefficient template. Significance plane is followed by greater than 1 (gt1) plane and that is followd by greater than 2 (gt2) bit plane. When coding bin i in the i-th scan pass (i being 0, 1, 2), the context index is dependent on the values of the i-th bins of previously coded coefficients in the neighborhood covered by a local template. More specifically, the context index is determined based on the sum of the i-th bins of neighboring coefficients as well as the coefficient position within the TU. The number of regular coded bins for gt1and gt2 bitplane is capped. All remaining bins are bypass coded after the regular coded bins followed by sign information. Details of the scheme can be found in JVET-K0069.

### Test results

The proposed tool was integrated in the BMS-1.0 software. The software was made available to CE7 participant in the following svn branch:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-1.1-0

Simulations were conducted according to the common test conditions [4]. Two configurations were tested:

- the proposed tool was enabled on top of the VTM-1 (using BMS-1 in VTM configuration);

- the proposed tool was enabled on top of the BMS-1.

The test results reported by the proponent are summarized in Table 3 (VTM and BMS configurations).

Table 3: VTM and BMS configuration results of subtest 7.1.1.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.83% | -1.15% | -1.02% | 105% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | -1.21% | -0.95% | -1.04% | 105% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | -1.24% | -0.85% | -0.95% | 105% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | -1.49% | -0.94% | -1.06% | 107% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | -1.92% | -1.65% | -1.78% | 104% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -1.34% | -1.07% | -1.14% | 105% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | -1.62% | -0.89% | -0.93% | 107% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F (optional) | -2.16% | -1.09% | -1.26% | 106% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.63% | -0.79% | -0.64% | 103% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | -0.70% | -0.57% | -0.69% | 103% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | -0.89% | -0.55% | -0.65% | 103% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | -1.17% | -0.51% | -0.59% | 104% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.87% | -0.59% | -0.64% | 103% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | -1.15% | -0.24% | -0.41% | 105% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F (optional) | -1.75% | -0.93% | -0.95% | 104% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.55% | -0.53% | -0.07% | 105% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | -0.70% | -0.58% | -0.54% | 105% | 104% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | -0.52% | -0.03% | -0.46% | 104% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.59% | -0.42% | -0.32% | 105% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | -0.81% | -0.78% | -0.21% | 105% | 103% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F (optional) | -1.35% | -1.08% | -0.56% | 105% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |

Additional results with BMS-HEVC coefficient coding anchor is provided in JVET-K0069.

The following table summarizes the number of contexts used in coefficient coding of 7.1.1.

Table 4: Number of contexts used in coefficient coding of 7.1.1.

| CE-7.1.1 |  |  |
| --- | --- | --- |
| Syntax/Scan | Value | Notes |
| Scan method | Diag |  |
| SigCGFlag | 4 | 2+2 (lum+chr) |
| Last_xy | 58 (29x2) | 25+4 (lum+chr) |
| Sig Luma | 54 (18x3) | TU Size:4x4,8x8,>8x8 |
| Sig Chroma | 12 |  |
| gt1/gt2 Luma | 16 |  |
| gt1/gt2 Chroma | 6 |  |
| Reg. bins | 8GT1/16 |  |
|  | 1GT2/16 |  |
| RiceParam | BMS |  |
| Total # contexts | 150 |  |

Additional results with reduced number (18) of contexs for luma significance is included in JVET-K0069.

### Cross checks

The test results were verified by one cross checker. Details are provided below.

cross check 1 for test 7.1.1:

cross checker: S.-C. Lim (ETRI)

cross check of test results: - the cross checker tested the implementation provided by Qualcomm in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-1.1-0 - simulations were run for both the VTM and BMS configuration and all test cases - some missing simulation results, the rest have been verified. - run times are consistent with the proponent’s reported run times.

source code analysis: - the cross checker checked the source code and confirmed that the source code is consistent with the test description, especially for number of context model and the selection of context models for significant flag, gt1 flag, and gt2 flag

additional remarks: - none

cross check 2 for test 7.1.1:

cross checker: Y. Piao (Samsung)

cross check of test results: - the cross checker tested the implementation provided by Qualcomm in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-1.1-0 - simulations were run for both the VTM and BMS configuration and the test cases “all intra” and “random access” - some missing simulation results, the rest have been verified. - run times are consistent with the proponent’s reported run times.

source code analysis: - the cross checker verified the number of contexts used in coefficient coding.

additional remarks: - none

## Test 7.1.2:  Transform coefficient coding

### Brief tool description

The proposed scheme for coding of transform coefficient levels relative to HEVC includes the following modifications:

- Context modelling for coded block flags:  The context for the coded block flag of the second chroma is determined by the value of the coded block flag for the first chroma channel.

- Coding order of bins in a subblock:  All bins representing the absolute value of a transform coefficient level are coded before moving to the next scanning position.

- Binarization of absolute values:  The first five bins (which represent the absolute values using an unary binarization) are coded in regular mode using adaptive context models; the remainder is coded using Rice codes in bypass mode of the arithmetic coding engine.

- Context modelling for bins of absolute values:  The context for the regular coded bins of the absolute values is determined by the values of already coded absolute values in a local neighborhood.

In comparison to HEVC, a modified binarization for the absolute values of the transform coefficient levels is used.  Similar to HEVC, the absolute values are binarized using a binarization that consists of a prefix and a suffix part.  While for the prefix part a truncated unary code is used, the suffix part is represented using Rice and Exp-Golomb codes.  But in contrast to HEVC, the first binarization bound, i.e., the transition between the truncated unary code and the Rice codes is fixed, and its value is set equal to five, instead of being adaptive as in HEVC. This configuration results in five bins that are coded in the regular mode, i.e., with context models.  Other parameters such as the relationship between the Rice and Exp-Golomb order and the interval range that is covered by a Rice code are unchanged in comparison to HEVC. A summary of the differences is illustrated in the figure below.

In contrast to HEVC and JEM, the absolute values of the transform coefficients of a subblock are coded in a single pass.  That means all syntax elements for a scan position are coded before any data for the next scan position are transmitted.  The signs for the nonzero transform coefficient levels are coded in a second pass over the subblock.

The context model selection for the bins sig_coeff_flag and coeff_abs_level_greaterX_flag (with X being replaced by 1, 2, 3, and 4) as well as the binarization code selection for the syntax element coeff_abs_level_remaining is based on already coded absolute values in a local neighborhood.

### Test results

The proposed tool was integrated in the BMS-1.0 software. The software was made available to CE7 participant in the following svn branch:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-1.2-0

Simulations were conducted according to the common test conditions [4]. Two configurations were tested:

- the proposed tool was enabled on top of the VTM-1 (using BMS-1 in VTM configuration);

- the proposed tool was enabled on top of the BMS-1.

The test results reported by the proponent are summarized in Table 5 (VTM and BMS configurations).

Table 5: VTM and BMS configuration results of subtest 7.1.2.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 in VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.32% | -1.19% | -1.68% | 103% | 102% | -0.45% | 0.00% | -1.11% | 96% | 100% |
| Class A2 | -1.67% | -0.63% | -2.22% | 101% | 100% | -0.38% | 0.39% | -1.11% | 94% | 99% |
| Class B | -1.60% | -1.86% | -1.33% | 102% | 100% | -0.30% | -1.40% | 0.38% | 95% | 98% |
| Class C | -1.93% | -1.77% | -1.84% | 102% | 100% | -0.33% | -1.32% | 0.42% | 93% | 97% |
| Class E | -2.25% | -2.71% | -2.46% | 101% | 98% | -0.37% | -1.88% | 0.27% | 95% | 99% |
| Overall | -1.75% | -1.67% | -1.84% | 102% | 100% | -0.36% | -0.93% | -0.12% | 94% | 98% |
| Class D | -1.98% | -1.47% | -1.63% | 103% | 101% | -0.26% | -0.91% | 0.16% | 92% | 98% |
| Class F (optional) | -2.66% | -1.77% | -2.21% | 101% | 102% | -0.44% | -0.87% | -0.62% | 93% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 in VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.06% | -0.78% | -0.91% | 101% | 98% | -0.39% | 0.09% | -0.49% | 96% | 102% |
| Class A2 | -0.99% | -0.15% | -1.51% | 101% | 99% | -0.23% | 0.35% | -0.72% | 96% | 99% |
| Class B | -1.11% | -1.57% | -0.79% | 101% | 101% | -0.21% | -1.34% | 0.50% | 95% | 99% |
| Class C | -1.42% | -1.29% | -1.10% | 101% | 100% | -0.19% | -1.11% | 0.34% | 94% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.16% | -1.05% | -1.04% | 101% | 100% | -0.25% | -0.66% | 0.01% | 95% | 99% |
| Class D | -1.39% | -0.77% | -0.66% | 102% | 97% | -0.17% | -0.34% | 0.40% | 95% | 99% |
| Class F (optional) | -1.99% | -1.39% | -1.72% | 101% | 99% | -0.26% | -0.69% | 0.08% | 94% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 in VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.77% | -1.55% | 0.25% | 102% | 102% | -0.20% | -1.00% | 1.23% | 97% | 98% |
| Class C | -1.00% | -1.16% | -0.07% | 101% | 100% | -0.25% | -0.52% | 0.91% | 95% | 97% |
| Class E | -0.94% | -1.71% | -0.76% | 100% | 100% | -0.40% | -0.79% | 0.79% | 97% | 97% |
| Overall | -0.89% | -1.46% | -0.11% | 101% | 101% | -0.27% | -0.79% | 1.01% | 96% | 97% |
| Class D | -0.98% | 0.24% | 0.28% | 102% | 100% | -0.09% | 0.60% | 0.89% | 94% | 95% |
| Class F (optional) | -1.59% | -0.88% | -0.65% | 101% | 101% | -0.21% | -0.21% | 0.49% | 96% | 99% |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 in VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.79% | -1.44% | -0.11% | 102% | 101% | -0.21% | -0.73% | 0.95% | 96% | 100% |
| Class C | -1.03% | -1.20% | -0.20% | 101% | 97% | -0.21% | -0.43% | 1.00% | 95% | 97% |
| Class E | -1.07% | -1.99% | -0.07% | 100% | 98% | -0.27% | -0.71% | 0.13% | 96% | 99% |
| Overall | -0.94% | -1.50% | -0.13% | 101% | 99% | -0.22% | -0.63% | 0.76% | 96% | 99% |
| Class D | -1.04% | -0.27% | 0.51% | 103% | 102% | -0.18% | 0.43% | 1.02% | 94% | 98% |
| Class F (optional) | -1.60% | -1.23% | -0.90% | 102% | 103% | -0.10% | 0.33% | 0.39% | 94% | 99% |

The following table summarizes the number of contexts used in coefficient coding of 7.1.2.

Table 6: Number of contexts used in coefficient coding of 7.1.2.

| CE-7.1.2 |  |  |
| --- | --- | --- |
| Syntax/Scan | Value | Notes |
| Scan method | Diag |  |
| SigCGFlag | 4 | 2+2 (lum+chr) |
| Last_xy | 58 (29x2) | 25+4 (lum+chr) |
| Sig Luma | 18 |  |
| Sig Chroma | 12 |  |
| gt1 Luma | 21 |  |
| gt1 Chroma | 11 |  |
| gtX Luma | 21 |  |
| gtX Chroma | 11 |  |
| Reg. bins | No restriction up to gt4 |  |
| RiceParam | JVET-K0071 |  |
| Total # contexts | 156 |  |

### Cross checks

The test results were verified by three cross checkers. Details are provided below.

cross check 1 for test 7.1.2:

cross checker: M. Coban (Qualcomm)

cross check of test results: - the cross checker tested the implementation provided by HHI in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-1.2-0 - simulations were run for both the VTM and BMS configuration and all test cases - all results are verified - run times are consistent with the proponent’s reported run times.

source code analysis: - the cross checker examined the code in detail, consistent with the description.

additional remarks: - this method includes improvement to cbf coding for Cr component. Uses HEVC CG context initialization. As described in the description, levels are coded coefficient by coefficient, interleaving different syntax elements as well as bypass coded bins.

cross check 2 for test 7.1.2:

cross checker: Y. Piao (Samsung)

cross check of test results: - the cross checker tested the implementation provided by HHI in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-1.2-0 - simulations were run for both the VTM and BMS configuration for RA case. - the results are verified. - run times are consistent with the proponent’s reported run times.

source code analysis: - not reported

additional remarks: - none

cross check 3 for test 7.1.2:

cross checker: M. Guo (Hulu)

cross check of test results: - the cross checker tested the implementation provided by HHI in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-1.2-0 - results are verified (two 4k sequences still running) - run times are consistent with the proponent’s reported run times

source code analysis: - not reported

additional remarks: - none

## Test 7.1.3:  Coding of transform coefficient levels

### Brief tool description

Scan region-based coefficient coding proposed in JVET-0024 determines scan region to be scanned in a TU by the scan region (SRx, SRy), where SRx is most right non-zero coefficients’ x–axis and SRy is most bottom non-zero coefficients’ y-axis. Only coefficients in scan region are coded from the right-bottom corner of the scan region in inverse zigzag scan order. A coefficient is presented by GT0flag, GT1flag, GT2flag, remaining level and sign. Context indices of significant flag, GT1flag, GT2flag are derived by number of corresponding flags in neighborhood. Different context sets are used depending on the position in the scan region, color component and transform size. Rice parameter for remaining level coding is same with HEVC. There is no significant group flag in this proposal.

### Test results

The proposed tool was integrated in the BMS-1.0 software. The software was made available to CE7 participant in the following svn branch:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-1.3-0

Simulations were conducted according to the common test conditions [4] and with equal probability initialization of coefficient coding contexts. Two configurations were tested:

- the proposed tool was enabled on top of the VTM-1 (using BMS-1 in VTM configuration);

- the proposed tool was enabled on top of the BMS-1.

The test results reported by the cross checker T. Nguyen are summarized in Table 7 (VTM and BMS configurations).

Table 7: VTM and BMS configuration results for subtest 7.1.3.

|  | Over vtm1.0 |  |  |  |  | Over bms1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | 0.13% | 0.30% | 104% | 108% | 0.32% | 1.05% | 1.06% | 99% | 102% |
| Class A2 | -0.82% | 0.10% | -0.08% | 104% | 106% | 0.08% | 0.83% | 0.60% | 99% | 100% |
| Class B | -0.82% | 0.31% | 0.27% | 105% | 106% | 0.06% | 1.06% | 1.12% | 101% | 102% |
| Class C | -1.12% | -0.25% | -0.31% | 108% | 106% | 0.16% | 0.80% | 0.88% | 103% | 101% |
| Class E | -1.36% | -0.17% | -0.17% | 102% | 102% | 0.15% | 1.11% | 0.87% | 100% | 101% |
| Overall | -0.87% | 0.04% | 0.01% | 105% | 106% | 0.14% | 0.97% | 0.93% | 100% | 102% |
| Class D | -1.14% | 0.00% | -0.04% | 112% | 110% | 0.29% | 0.97% | 0.95% | 106% | 104% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over vtm1.0 |  |  |  |  | Over bms1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.09% | 0.31% | 100% | 101% | 0.35% | 0.86% | 1.07% | 96% | 101% |
| Class A2 | -0.48% | 0.31% | 0.08% | 101% | 101% | 0.06% | 0.81% | 0.76% | 98% | 100% |
| Class B | -0.21% | 0.55% | 0.45% | 101% | 109% | 0.33% | 1.15% | 0.95% | 98% | 102% |
| Class C | -0.57% | 0.44% | 0.40% | 102% | 105% | 0.42% | 1.08% | 0.69% | 100% | 104% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.34% | 0.34% | 0.34% | 101% | 105% | 0.30% | 1.01% | 0.87% | 98% | 102% |
| Class D | -0.43% | 0.65% | 0.28% | 103% | 101% | 0.60% | 0.94% | 0.82% | 102% | 105% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over vtm1.0 |  |  |  |  | Over bms1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.47% | 1.37% | 1.38% | 99% | 102% | 0.74% | 1.37% | 1.47% | 96% | 99% |
| Class C | 0.70% | 1.33% | 1.17% | 101% | 107% | 1.05% | 1.49% | 1.12% | 97% | 99% |
| Class E | 0.61% | 0.59% | 1.08% | 100% | 108% | 0.71% | 0.73% | 2.09% | 95% | 100% |
| Overall | 0.58% | 1.16% | 1.23% | 100% | 105% | 0.84% | 1.25% | 1.51% | 96% | 99% |
| Class D | 0.58% | 2.95% | 2.76% | 101% | 104% | 1.36% | 3.64% | 3.17% | 96% | 99% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Additional test results reported by the proponent are summarized in Table 8 (VTM and BMS configurations with EP context initialization).

Table 8: VTM and BMS configuration with EP context initialization results for subtest 7.1.3.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0_ctxInitEP |  |  |  |  | Over BMS-1.0_ctxInitEP |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | 0.07% | 0.28% | 101% | 102% | 0.31% | 1.00% | 1.06% | 97% | 101% |
| Class A2 | -0.84% | 0.07% | -0.12% | 101% | 102% | 0.06% | 0.76% | 0.58% | 97% | 105% |
| Class B | -0.86% | 0.18% | 0.16% | 103% | 102% | 0.02% | 0.86% | 0.96% | 98% | 100% |
| Class C | -1.21% | -0.39% | -0.44% | 105% | 102% | 0.05% | 0.55% | 0.58% | 101% | 100% |
| Class E | -1.45% | -0.43% | -0.43% | 100% | 101% | 0.03% | 0.78% | 0.69% | 98% | 99% |
| Overall | -0.92% | -0.08% | -0.10% | 102% | 102% | 0.08% | 0.78% | 0.79% | 99% | 101% |
| Class D | -1.41% | -0.45% | -0.49% | 107% | 103% | -0.02% | 0.21% | 0.34% | 102% | 100% |
| Class F (optional) | -1.73% | -0.59% | -0.50% | 103% | 102% | 0.31% | 0.63% | 0.76% | 103% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0_ctxInitEP |  |  |  |  | Over BMS-1.0_ctxInitEP |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.23% | 0.14% | 99% | 101% | 0.26% | 0.65% | 0.84% | 97% | 100% |
| Class A2 | -0.52% | 0.30% | -0.03% | 98% | 101% | -0.02% | 0.67% | 0.55% | 98% | 100% |
| Class B | -0.38% | 0.09% | 0.07% | 100% | 101% | 0.14% | 0.69% | 0.62% | 98% | 101% |
| Class C | -0.83% | -0.09% | -0.04% | 101% | 101% | 0.09% | 0.51% | 0.47% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.49% | 0.02% | 0.04% | 100% | 101% | 0.12% | 0.63% | 0.61% | 98% | 100% |
| Class D | -0.91% | -0.09% | -0.28% | 102% | 100% | 0.01% | -0.06% | 0.19% | 101% | 101% |
| Class F (optional) | -1.32% | -0.21% | -0.30% | 101% | 100% | 0.24% | 0.62% | 0.51% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0_ctxInitEP |  |  |  |  | Over BMS-1.0_ctxInitEP |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.21% | 0.33% | 0.27% | 99% | 99% | 0.50% | 0.66% | 0.55% | 96% | 99% |
| Class C | 0.09% | -0.03% | -0.11% | 100% | 100% | 0.48% | 0.16% | 0.07% | 97% | 101% |
| Class E | -0.33% | -1.42% | -0.66% | 97% | 100% | 0.32% | -0.16% | 0.84% | 96% | 101% |
| Overall | 0.03% | -0.23% | -0.09% | 99% | 100% | 0.45% | 0.29% | 0.46% | 96% | 100% |
| Class D | -0.29% | -0.21% | -0.26% | 101% | 100% | 0.24% | -0.41% | -0.25% | 98% | 99% |
| Class F (optional) | -0.55% | -0.53% | -0.33% | 99% | 100% | 0.39% | 0.14% | 0.46% | 98% | 99% |

The contribution JVET-K0138 contain additional results with context initialization; these results have not been verified in the cross checks.

The following table summarizes the number of contexts used in coefficient coding of 7.1.3.

Table 9: Number of contexts used in coefficient coding of 7.1.3.

| CE-7.1.3 |  |  |
| --- | --- | --- |
| Syntax/Scan | Value | Notes |
| Scan method | Diag | Different scan region |
| SigCGFlag |  |  |
| Last_xy | 58 (29x2) | 25+4 (lum+chr) |
| Sig | 47 | Firs scan pos, different offset |
| gt1/gt2 | 18 |  |
| Reg. bins | max(scanR/2, 8)GT1 max(scanR/16,1)GT2 |  |
| RiceParam | VTM/BMS |  |
| Total # contexts | 123 |  |

### Cross checks

The test results were verified by at least one cross checker. Details are provided below.

cross check 1 for test 7.1.3:

cross checker: Y. Chen (Technicolor)

cross check of test results: - the cross checker tested the implementation provided by Samsung in the attachment - simulations were run for the VTM configuration for RA case - for both the proposal and the VTM reference, the context models related to transform coefficient coding were initialized with an equal probability distribution (154) - partial verification (incomplete data). - run times are consistent with the proponent’s reported run times.

source code analysis: - the cross checker studied the software and confirmed that the implementation matches the description in the CE document (even though not all implementation details are described in the CE description, the CE description covers the most important aspects) - Context modelling of the significant flag: reduce the number of context indexes into 2 in the top-left region of a TU; for the remaining two regions, reduce the number of context indexes inside each region into 5 - Context modelling of the GT1/2 flag: there are 4 context indexes used only for DC coefficient, which are independent and separate from other GT1/2 coefficients - For the last significant coefficient inside the scan region: context index of its significant flag, GT1 flag and GT2 flag, is independent and separate from other significant coefficients

additional remarks: - none

cross check 2 for test 7.1.3:

cross checker: T. Nguyen (HHI)

cross check of test results: - the cross checker tested the implementation provided by HHI in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-1.3-0 - simulations were run for both the VTM and BMS configuration and all test cases - all results are verified - run times are consistent with the proponent’s reported run times.

source code analysis: - the cross checker studied the software and confirmed that the implementation matches the description in the CE document

additional remarks: - for the results provided by the proponent, the context models related to transform coefficient coding were initialized with an equal probability distribution (0.5, 0.5), for both the proposal and the VTM/BMS reference - the cross checker obtained identical rate and PSNR values for the proposal; for the comparison, the cross checker used the actual VTM/BMS results (without EP init in the references)

## Test 7.1.4:  Coefficient Coding

### Brief tool description

This proposal is intended to reduce the complexity for context model selection for transform coefficient levels. In this proposal, the context model selection method is the same as the description in subtest 7.1.1 except that the transform coefficient positional based context switching is constrained to the first 4x4 coefficient group in order to reduce computation complexity of other coefficient groups.

### Test results

The proposed tool was integrated in the BMS-1.0 software. The software was made available to CE7 participant in the following svn branch:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-1.4-0

Simulations were conducted according to the common test conditions [4]. Two configurations were tested:

- the proposed tool was enabled on top of the VTM-1 (using BMS-1 in VTM configuration);

- the proposed tool was enabled on top of the BMS-1.

The test results reported by the proponent are summarized in Table 10 (VTM and BMS configurations).

Table 10: VTM and BMS configuration results for subtest 7.1.4.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.79% | -1.12% | -0.97% | 104% | 100% | 0.06% | 0.06% | -0.06% | 100% | 100% |
| Class A2 | -1.14% | -0.92% | -1.03% | 106% | 101% | 0.07% | 0.03% | -0.01% | 100% | 101% |
| Class B | -1.20% | -0.88% | -0.96% | 106% | 101% | 0.06% | 0.09% | 0.07% | 100% | 100% |
| Class C | -1.48% | -0.89% | -1.02% | 107% | 101% | 0.01% | 0.07% | 0.10% | 100% | 100% |
| Class E | -1.91% | -1.70% | -1.56% | 104% | 100% | 0.03% | -0.08% | 0.03% | 99% | 100% |
| Overall | -1.30% | -1.06% | -1.09% | 105% | 101% | 0.05% | 0.04% | 0.04% | 100% | 100% |
| Class D | -1.61% | -0.85% | -0.98% | 108% | 102% | 0.07% | 0.19% | 0.08% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.64% | -0.87% | -0.58% | 105% | 100% | 0.06% | 0.01% | 0.03% | 101% | 100% |
| Class A2 | -0.73% | -0.39% | -0.66% | 106% | 100% | 0.05% | -0.14% | 0.01% | 101% | 101% |
| Class B | -0.86% | -0.58% | -0.58% | 106% | 100% | 0.04% | 0.03% | -0.03% | 101% | 100% |
| Class C | -1.15% | -0.49% | -0.59% | 106% | 100% | 0.06% | 0.19% | -0.20% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.87% | -0.58% | -0.60% | 106% | 100% | 0.05% | 0.04% | -0.05% | 101% | 100% |
| Class D | -1.13% | -0.28% | -0.55% | 107% | 101% | 0.07% | -0.30% | 0.17% | 101% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.53% | -0.43% | -0.05% | 105% | 100% | 0.01% | -0.07% | 0.28% | 101% | 100% |
| Class C | -0.66% | -0.59% | -0.62% | 106% | 100% | 0.03% | 0.46% | 0.12% | 100% | 99% |
| Class E | -0.62% | -0.84% | -0.23% | 104% | 100% | -0.22% | -0.59% | -0.13% | 100% | 100% |
| Overall | -0.60% | -0.59% | -0.29% | 105% | 100% | -0.04% | -0.02% | 0.13% | 100% | 100% |
| Class D | -0.77% | -0.65% | -1.28% | 106% | 101% | 0.04% | 0.58% | 0.14% | 101% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

### Cross checks

The test results were verified by one cross checker. Details are provided below.

cross check 1 for test 7.1.4:

cross checker: T. Tsukubo (Sony)

cross check of test results: - the cross checker tested the implementation provided by Huawei in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-1.4-0 - simulations were run for both the VTM and BMS configuration and all test cases - all results are verified

source code analysis: - the cross checker analysed the source code and confirmed that the implementation matches the description

additional remarks: - the cross checker remarked: It looks beneficial to simplify context index assignments for luma’s sig_coef/gr1/gr2 flag used in BMS (--AltResiComp=1) - To avoid mismatches for RA parallel simulation, it would be helpful to specify the common procedure to calcurate bitrate and psnrs in CTC

cross check 2 for test 7.1.4:

cross checker: Y. Piao (Samsung)

cross check of test results: - the cross checker tested the implementation provided by Huawei in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-1.4-0 - simulations were run for both the VTM and BMS configuration for RA case. - the results are verified. - run times are consistent with the proponent’s reported run times

source code analysis: - not reported

additional remarks: - none

# Subtest 7.2:  Quantization of a transform block

In this subtest, 7.2.1 dependent quantization as well as sign data hiding (SDH) in combination with subtests 7.1.1, 7.1.2 and 7.1.4 are presented. The crosscheck reports for 7.1.1, 7.1.2 and 7.1.4 from previous section covers the SDH case as well. SDH method used in this subtest is the same and based on HEVC SDH scheme.

The average test results for subtest 7.2 are summarized in the following tables. Table 11 reports the results relative to VTM-1 and BMS-1. Table 12 reports the results relative to BMS-1 with HEVC coefficient coding.

Table 11: Average test results for subtest 7.2 (relative to VTM-1 and BMS-1).

| averages for subtest 7.2 |  | over VTM-1 (BMS-1 in VTM configuration) |  |  |  |  | over BMS-1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| AI | test 7.2.1 (7.1.2+DQ) | -4.99% | -1.98% | -1.96% | 115% | 98% | -2.85% | -2.39% | -0.78% | 113% | 97% |
|  | add. test: 7.1.1+SDH | -2.12% | -2.75% | -2.78% | 109% | 102% | -0.71% | -1.59% | -1.63% | 105% | 102% |
|  | add. test: 7.1.2+SDH | -2.53% | -3.36% | -3.48% | 106% | 100% | -1.06% | -2.56% | -1.74% | 99% | 99% |
|  | add. test: 7.1.4+SDH | -2.07% | -2.74% | -2.79% | 107% | 101% | -0.65% | -1.56% | -1.61% | 101% | 100% |
| RA | test 7.2.1 (7.1.2+DQ) | -3.40% | -1.82% | -1.72% | 110% | 99% | -2.16% | -1.74% | -0.39% | 113% | 99% |
|  | add. test: 7.1.1+SDH | -1.45% | -1.57% | -1.69% | 107% | 100% | -0.54% | -1.01% | -1.17% | 104% | 100% |
|  | add. test: 7.1.2+SDH | -1.74% | -2.08% | -2.10% | 104% | 99% | -0.80% | -1.67% | -1.14% | 100% | 100% |
|  | add. test: 7.1.4+SDH | -1.44% | -1.64% | -1.77% | 107% | 100% | -0.51% | -1.01% | -1.17% | 102% | 100% |
| LB | test 7.2.1 (7.1.2+DQ) | -2.70% | -2.34% | -0.66% | 111% | 99% | -1.88% | -2.30% | 0.38% | 112% | 96% |
|  | add. test: 7.1.1+SDH | -1.18% | -1.87% | -1.55% | 108% | 103% | -0.66% | -1.14% | -0.86% | 103% | 99% |
|  | add. test: 7.1.2+SDH | -1.43% | -2.36% | -1.33% | 106% | 102% | -0.85% | -1.75% | -0.27% | 101% | 99% |
|  | add. test: 7.1.4+SDH | -1.15% | -1.69% | -1.36% | 106% | 100% | -0.58% | -1.11% | -1.11% | 102% | 100% |

Table 12: Average test results for subtest 7.2 (relative to BMS-1 with HEVC coefficient coding). The given ratios for the encoder and decoder times represent estimated values, obtained by the measures ratios relative to BMS-1 and the measured ratios between BMS-1 and BMS-1 with HEVC coefficient coding.

| averages for subtest 7.2 |  | over BMS-1 with HEVC coefficient coding |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT |
| AI | test 7.2.1 (7.1.2+DQ) | -3.65% | -3.21% | -1.63% | 119% | 99% |
|  | add. test: 7.1.1+SDH | -1.53% | -2.42% | -2.46% | 111% | 104% |
|  | add. test: 7.1.2+SDH | -1.87% | -3.38% | -2.58% | 105% | 101% |
|  | add. test: 7.1.4+SDH | -1.47% | -2.39% | -2.44% | 107% | 102% |
| RA | test 7.2.1 (7.1.2+DQ) | -2.71% | -2.30% | -0.78% | 121% | 100% |
|  | add. test: 7.1.1+SDH | -1.11% | -1.58% | -1.57% | 111% | 101% |
|  | add. test: 7.1.2+SDH | -1.36% | -2.24% | -1.53% | 107% | 101% |
|  | add. test: 7.1.4+SDH | -1.07% | -1.58% | -1.56% | 109% | 101% |
| LB | test 7.2.1 (7.1.2+DQ) | -2.20% | -2.77% | 0.05% | 118% | 98% |
|  | add. test: 7.1.1+SDH | -0.99% | -1.61% | -1.19% | 109% | 101% |
|  | add. test: 7.1.2+SDH | -1.18% | -2.22% | -0.61% | 107% | 101% |
|  | add. test: 7.1.4+SDH | -0.91% | -1.57% | -1.44% | 108% | 102% |

## Test 7.2.1:  Dependent quantization and coding of transform coefficient levels

### Brief tool description

The approach of dependent scalar quantization is realized by: (a) defining two scalar quantizers with different reconstruction levels and (b) defining a process for switching between the two scalar quantizers.

For each transform coefficient, there are two available scalar quantizers, as illustrated in the figure above.  Similar to conventional uniform reconstruction quantizers (as used in HEVC), for both quantizers (denoted by Q0 and Q1), the location of the available reconstruction levels is uniquely specified by a quantization step size Δ.  If we neglect the fact that the actual reconstruction of transform coefficients uses integer arithmetic, the two scalar quantizers Q0 and Q1 are characterized as follows:  The reconstruction levels of the first quantizer Q0 are given by the even integer multiples of the quantization step size  Δ.  When this quantizer is used, the reconstructed transform coefficient t’ is obtained according to

where k denotes the associated transform coefficient level (transmitted quantization index).  The reconstruction levels of the second quantizer Q1 are given by the odd integer multiples of the quantization step size Δ and, in addition, the reconstruction level equal to zero. The mapping of transform coefficient levels k to reconstructed transform coefficients t’ is given by

with

sgnx= 1:x>00:x=0-1:x<0 .

The scalar quantizer (Q0 or Q1) used for a current transform coefficient is determined by the parities of the transform coefficient levels that precede the current transform coefficient in coding/reconstruction order. The switching between the two scalar quantizers (Q0 and Q1) is realized via a state machine.  The state can take four different values: 0, 1, 2, 3.  It is uniquely determined by the parities of the transform coefficient levels preceding the current transform coefficient in coding/reconstruction order.  At the start of the inverse quantization for a transform block, the state is set equal to 0.  The transform coefficients are reconstructed in scanning order (i.e., in the same order they are entropy decoded).  After a current transform coefficient is reconstructed, the state is updated as specified by the table below. The next state only depends on the current state and the parity (k & 1) of the current transform coefficient level k.

The state uniquely specifies the scalar quantizer used.  If the state for a current transform coefficient is equal to 0 or 1, the scalar quantizer Q0 is used.  Otherwise (the state is equal to 2 or 3), the scalar quantizer Q1 is used.

At the encoder side, similar to the rate-distortion optimized quantization (RDOQ) implemented in the HM and JEM software, the transform coefficient levels for a transform block are determined in a way that the Lagrangian rate-distortion cost is minimized.  In comparison to RDOQ in the HM/JEM software, the switching between the scalar quantizers Q0 and Q1 must be additionally taken into account in the quantization process. The dependencies between the chosen transform coefficient levels and the scalar quantizers used can be described using a trellis structure.  Each node in the trellis represents a potential state (0, 1, 2, or 3) for a particular transform coefficient.  The state uniquely determines the quantizer used (Q0 is used for states 0 and 1; Q1 is used for states 2 and 3).  Each node/state is connected with two states for the next transform coefficient in coding order.  Given a certain state for a current transform coefficient, the parity of the associated transform coefficient level q[k] determines the state (and, thus, the quantizer) for the next transform coefficient in coding order. Hence, the set of available transform coefficient levels for a quantizer is basically partitioned into two subsets: The first subset includes the even transform coefficient levels (parity 0) and the second subset includes the odd transform coefficient levels (parity 1).

If we label all connections between two directly connected trellis nodes with the associated rate-distortion costs, the determination of transform coefficient levels that minimize the overall rate-distortion cost is equivalent to finding the path with minimum cost through the trellis.  This minimization problem can be solved using the Viterbi algorithm.

The quantization algorithm used consist of two steps:

- Determination of four candidate transform coefficient levels for each transform coefficient (one for each of the subsets A, B, C, D).

- Viterbi algorithm for determining the sequence of transform coefficient levels qk that minimizes the estimated overall rate-distortion cost (among the pre-determined candidate levels).

The coefficient coding scheme used with this method is based on subtest 7.1.2 with the addition of two different context model sets are used for the two scalar quantizers for coding of (sig_coeff_flag and coeff_abs_level_greater_flag).

### Test results

The proposed tool was integrated in the BMS-1.0 software. The software was made available to CE7 participant in the following svn branch:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-2.1-0

Simulations were conducted according to the common test conditions [4]. Two configurations were tested:

- the proposed tool was enabled on top of the VTM-1 (using BMS-1 in VTM configuration);

- the proposed tool was enabled on top of the BMS-1.

The test results reported by the proponent are summarized in Table 13 (VTM and BMS configurations).

Table 13: VTM and BMS configuration results for subtest 7.2.1.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 in VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.87% | -3.64% | -3.26% | 108% | 97% | -3.49% | -3.29% | -3.35% | 107% | 98% |
| Class A2 | -5.46% | -2.23% | -3.15% | 115% | 97% | -3.40% | -1.83% | -1.96% | 114% | 98% |
| Class B | -4.84% | -1.90% | -1.29% | 115% | 98% | -2.79% | -2.47% | -0.34% | 113% | 97% |
| Class C | -5.02% | -0.78% | -0.98% | 124% | 98% | -2.45% | -2.10% | 0.65% | 119% | 95% |
| Class E | -4.84% | -1.82% | -1.87% | 111% | 98% | -2.31% | -2.28% | 0.34% | 111% | 99% |
| Overall | -4.99% | -1.98% | -1.96% | 115% | 98% | -2.85% | -2.39% | -0.78% | 113% | 97% |
| Class D | -5.07% | -0.02% | -0.26% | 128% | 100% | -2.40% | -1.05% | 1.16% | 121% | 97% |
| Class F (optional) | -4.25% | -0.11% | -1.55% | 119% | 102% | -1.63% | -0.46% | 0.41% | 120% | 98% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 in VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.28% | -2.37% | -1.48% | 104% | 100% | -2.52% | -1.63% | -1.00% | 106% | 100% |
| Class A2 | -2.95% | -1.19% | -2.79% | 110% | 100% | -1.97% | -1.03% | -1.77% | 114% | 102% |
| Class B | -3.70% | -1.87% | -1.04% | 110% | 99% | -2.39% | -1.76% | 0.61% | 112% | 98% |
| Class C | -3.47% | -1.83% | -1.94% | 115% | 98% | -1.74% | -2.31% | -0.14% | 118% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.40% | -1.82% | -1.72% | 110% | 99% | -2.16% | -1.74% | -0.39% | 113% | 99% |
| Class D | -3.45% | -0.09% | -1.39% | 118% | 96% | -1.69% | -1.27% | 1.03% | 119% | 98% |
| Class F (optional) | -3.60% | -0.83% | -1.83% | 110% | 99% | -1.44% | -0.87% | 0.25% | 115% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 in VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -3.11% | -3.08% | -0.38% | 111% | 98% | -2.25% | -2.92% | 0.14% | 111% | 93% |
| Class C | -2.80% | -3.84% | -2.59% | 116% | 99% | -1.84% | -3.36% | -1.48% | 118% | 98% |
| Class E | -1.89% | 0.90% | 1.46% | 106% | 101% | -1.30% | 0.13% | 3.25% | 107% | 97% |
| Overall | -2.70% | -2.34% | -0.66% | 111% | 99% | -1.88% | -2.30% | 0.38% | 112% | 96% |
| Class D | -2.62% | -2.23% | -3.29% | 117% | 97% | -1.60% | -1.29% | -1.31% | 116% | 95% |
| Class F (optional) | -2.56% | -2.70% | -3.09% | 112% | 101% | -0.88% | -1.98% | -0.75% | 113% | 98% |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 in VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.83% | -2.80% | -1.06% | 114% | 98% | -2.19% | -2.41% | -0.34% | 113% | 93% |
| Class C | -2.80% | -3.92% | -2.96% | 119% | 97% | -1.77% | -3.41% | -1.78% | 120% | 97% |
| Class E | -1.58% | 0.97% | 1.70% | 107% | 96% | -1.16% | 1.04% | 2.14% | 110% | 100% |
| Overall | -2.51% | -2.23% | -1.00% | 114% | 97% | -1.79% | -1.88% | -0.20% | 114% | 96% |
| Class D | -2.74% | -3.11% | -3.17% | 123% | 102% | -1.66% | -1.72% | -2.34% | 121% | 98% |
| Class F (optional) | -2.50% | -2.63% | -3.17% | 117% | 104% | -0.89% | -1.82% | -1.72% | 114% | 99% |

The following table summarizes the number of contexts used in coefficient coding of 7.2.1.

Table 14: Number of contexts used in coefficient coding of 7.2.1.

| CE-7.2.1 |  |  |
| --- | --- | --- |
| Syntax/Scan | Value | Notes |
| Scan method | Diag |  |
| SigCGFlag | 4 | 2+2 (lum+chr) |
| Last_xy | 58 (29x2) | 25+4 (lum+chr) |
| Sig Luma | 36 (2x18) |  |
| Sig Chroma | 24 (2x12) |  |
| gt1 Luma | 42( 2x21) |  |
| gt1 Chroma | 22 (2x11) |  |
| gtX Luma | 21 |  |
| gtX Chroma | 11 |  |
| Reg. bins | No restriction up to gt4 |  |
| RiceParam | JVET-K0071 |  |
| Total # contexts | 218 |  |

### Cross checks

The test results were verified by three cross checkers. Details are provided below.

cross check 1 for test 7.2.1:

cross checker: M. Coban (Qualcomm)

cross check of test results: - the cross checker tested the implementation provided by HHI in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-2.1-0 - simulations were run for both the VTM and BMS configuration and all test cases - all results are verified - run times are consistent with the proponent’s reported run times.

source code analysis: - the cross checker examined the code in detail, consistent with the description, well optimized implementation.

additional remarks: - the coeffcient coding used in this scheme is unified with the 7.1.2 scheme. This method requires complex RD algorithm for achieving gains, a fallback scheme for low complexity encoders is necessary.

cross check 2 for test 7.2.1:

cross checker: Y. Piao (Samsung)

cross check of test results: - the cross checker tested the implementation provided by HHI in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-2.1-0 - simulations were run for both the VTM and BMS configuration and RA case. - results are verified. - run times are consistent with the proponent’s reported run times

source code analysis: - not reported

additional remarks: - none

cross check 3 for test 7.2.1:

cross checker: F. Racape (Technicolor)

cross check of test results: - VTM partial results have been run - VTM results from the cross checker math the results provided in the svn branch of test 7.2.1

source code analysis: - not reported

additional remarks: - none

cross check 4 for test 7.2.1:

cross checker: A. Filippov (Huawei)

cross check of test results: - the cross checker tested the implementation provided by HHI in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-2.1-0 - simulations were run for both the VTM and BMS configuration (all test cases). - results are verified for VTM (there are similar issues with precision in the 4th decimal position of PSNR values for RA as reported by other cross-checkers); results for BMS are not finished yet.

source code analysis: - the source code was inspected, to be described in the CE document the process of skipping SG inside quantization process, distortion calculation and rate-distortion optimization

additional remarks: - the cross checker recommends to study dependent quantization on top of the BMS coefficient coding in CE7

## Sign data hiding with Subtests 7.1.1, 7.1.2, 7.1.4

Sign data hiding is combined with coefficient coding schemes 7.1.1, 7.1.2 and 7.1.4.

Below tables show results for VTM and BMS cases for these coefficient coding methods in combination with sign data hiding (SDH).

Table 15: VTM and BMS configuration results for subtest 7.1.1 + SDH.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.73% | -2.49% | -2.37% | 107% | 104% | -0.84% | -1.30% | -1.59% | 104% | 103% |
| Class A2 | -2.10% | -2.42% | -2.31% | 110% | 102% | -0.81% | -1.33% | -1.18% | 104% | 101% |
| Class B | -2.10% | -2.63% | -2.78% | 110% | 100% | -0.74% | -1.74% | -1.76% | 105% | 102% |
| Class C | -2.16% | -2.78% | -2.90% | 112% | 102% | -0.63% | -1.79% | -1.72% | 106% | 101% |
| Class E | -2.52% | -3.51% | -3.50% | 108% | 101% | -0.55% | -1.64% | -1.78% | 104% | 102% |
| Overall | -2.12% | -2.75% | -2.78% | 109% | 102% | -0.71% | -1.59% | -1.63% | 105% | 102% |
| Class D | -2.32% | -2.84% | -2.89% | 113% | 103% | -0.58% | -1.79% | -1.79% | 106% | 101% |
| Class F (optional) | -2.17% | -2.51% | -2.59% | 111% | 101% | -0.10% | -1.38% | -1.63% | 106% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.37% | -1.39% | -1.40% | 106% | 100% | -0.70% | -0.80% | -1.02% | 103% | 100% |
| Class A2 | -1.22% | -1.32% | -1.43% | 107% | 100% | -0.49% | -0.78% | -0.57% | 104% | 100% |
| Class B | -1.48% | -1.64% | -1.87% | 106% | 99% | -0.58% | -1.16% | -1.42% | 103% | 100% |
| Class C | -1.64% | -1.83% | -1.88% | 107% | 101% | -0.42% | -1.15% | -1.43% | 104% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.45% | -1.57% | -1.69% | 107% | 100% | -0.54% | -1.01% | -1.17% | 104% | 100% |
| Class D | -1.65% | -1.56% | -1.81% | 108% | 101% | -0.46% | -1.10% | -1.18% | 105% | 100% |
| Class F (optional) | -1.85% | -2.10% | -2.07% | 107% | 100% | -0.16% | -1.16% | -1.12% | 105% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.27% | -1.87% | -1.48% | 108% | 101% | -0.74% | -1.24% | -0.60% | 103% | 100% |
| Class C | -1.38% | -1.85% | -1.75% | 108% | 103% | -0.69% | -0.98% | -1.26% | 103% | 99% |
| Class E | -0.76% | -1.89% | -1.39% | 107% | 105% | -0.49% | -1.17% | -0.76% | 102% | 98% |
| Overall | -1.18% | -1.87% | -1.55% | 108% | 103% | -0.66% | -1.14% | -0.86% | 103% | 99% |
| Class D | -1.35% | -1.59% | -2.25% | 108% | 100% | -0.53% | -0.90% | -0.90% | 103% | 101% |
| Class F (optional) | -1.53% | -2.48% | -1.99% | 108% | 101% | -0.24% | -1.49% | -1.56% | 103% | 98% |

Table 16: VTM and BMS configuration results for subtest 7.1.2 + SDH.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 in VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.19% | -2.63% | -3.06% | 106% | 104% | -1.28% | -1.46% | -2.66% | 100% | 100% |
| Class A2 | -2.58% | -2.13% | -3.52% | 105% | 99% | -1.21% | -0.93% | -2.29% | 99% | 99% |
| Class B | -2.46% | -3.65% | -3.11% | 106% | 100% | -1.03% | -3.27% | -1.39% | 100% | 98% |
| Class C | -2.61% | -3.63% | -3.67% | 108% | 101% | -0.95% | -3.07% | -1.33% | 99% | 98% |
| Class E | -2.86% | -4.46% | -4.22% | 105% | 97% | -0.90% | -3.42% | -1.42% | 100% | 100% |
| Overall | -2.53% | -3.36% | -3.48% | 106% | 100% | -1.06% | -2.56% | -1.74% | 99% | 99% |
| Class D | -2.69% | -3.50% | -3.64% | 110% | 102% | -0.86% | -2.78% | -1.80% | 98% | 98% |
| Class F (optional) | -2.66% | -3.10% | -3.48% | 106% | 102% | -0.52% | -2.37% | -2.04% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 in VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.78% | -1.37% | -1.81% | 104% | 99% | -1.08% | -0.71% | -1.56% | 101% | 102% |
| Class A2 | -1.51% | -1.09% | -2.17% | 104% | 97% | -0.74% | -0.42% | -1.32% | 100% | 103% |
| Class B | -1.74% | -2.68% | -1.99% | 104% | 100% | -0.78% | -2.52% | -0.82% | 100% | 99% |
| Class C | -1.90% | -2.60% | -2.42% | 105% | 98% | -0.65% | -2.28% | -1.08% | 99% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.74% | -2.08% | -2.10% | 104% | 99% | -0.80% | -1.67% | -1.14% | 100% | 100% |
| Class D | -1.90% | -2.12% | -2.05% | 106% | 98% | -0.62% | -2.08% | -1.18% | 100% | 99% |
| Class F (optional) | -2.10% | -2.65% | -2.77% | 105% | 100% | -0.40% | -1.84% | -1.31% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 in VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.52% | -2.53% | -0.72% | 109% | 106% | -0.89% | -2.15% | 0.04% | 102% | 99% |
| Class C | -1.62% | -2.28% | -1.43% | 105% | 100% | -0.90% | -1.53% | -0.74% | 100% | 99% |
| Class E | -1.01% | -2.16% | -2.22% | 104% | 100% | -0.72% | -1.39% | -0.17% | 100% | 99% |
| Overall | -1.43% | -2.36% | -1.33% | 106% | 102% | -0.85% | -1.75% | -0.27% | 101% | 99% |
| Class D | -1.52% | -1.50% | -1.28% | 106% | 99% | -0.74% | -0.19% | -0.47% | 99% | 97% |
| Class F (optional) | -1.77% | -2.24% | -2.04% | 106% | 101% | -0.25% | -1.26% | -0.58% | 100% | 101% |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 in VTM config |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.37% | -2.60% | -1.10% | 106% | 100% | -0.85% | -1.91% | -0.16% | 101% | 102% |
| Class C | -1.58% | -2.31% | -2.16% | 105% | 97% | -0.84% | -1.59% | -0.66% | 100% | 97% |
| Class E | -0.94% | -2.04% | -1.52% | 105% | 98% | -0.71% | -1.66% | -1.13% | 100% | 99% |
| Overall | -1.33% | -2.36% | -1.56% | 105% | 99% | -0.81% | -1.74% | -0.57% | 100% | 100% |
| Class D | -1.52% | -1.33% | -1.12% | 108% | 103% | -0.65% | -0.82% | -0.02% | 99% | 98% |
| Class F (optional) | -1.74% | -2.71% | -1.81% | 106% | 102% | -0.27% | -1.27% | -1.01% | 99% | 98% |

Table 17: VTM and BMS configuration results for subtest 7.1.4 + SDH.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.67% | -2.45% | -2.37% | 105% | 100% | -0.76% | -1.32% | -1.58% | 101% | 100% |
| Class A2 | -2.04% | -2.42% | -2.30% | 107% | 100% | -0.75% | -1.29% | -1.17% | 101% | 100% |
| Class B | -2.04% | -2.63% | -2.78% | 107% | 101% | -0.67% | -1.72% | -1.80% | 102% | 100% |
| Class C | -2.13% | -2.78% | -2.90% | 109% | 101% | -0.60% | -1.71% | -1.68% | 102% | 100% |
| Class E | -2.48% | -3.50% | -3.56% | 105% | 100% | -0.51% | -1.62% | -1.68% | 101% | 100% |
| Overall | -2.07% | -2.74% | -2.79% | 107% | 101% | -0.65% | -1.56% | -1.61% | 101% | 100% |
| Class D | -2.30% | -2.91% | -2.91% | 110% | 102% | -0.58% | -1.57% | -1.74% | 103% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.37% | -1.58% | -1.51% | 106% | 100% | -0.65% | -0.81% | -1.07% | 102% | 100% |
| Class A2 | -1.22% | -1.40% | -1.35% | 106% | 100% | -0.49% | -0.89% | -0.69% | 102% | 100% |
| Class B | -1.46% | -1.73% | -2.02% | 106% | 100% | -0.53% | -1.21% | -1.36% | 102% | 99% |
| Class C | -1.64% | -1.74% | -1.96% | 107% | 101% | -0.39% | -0.99% | -1.36% | 102% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.44% | -1.64% | -1.77% | 107% | 100% | -0.51% | -1.01% | -1.17% | 102% | 100% |
| Class D | -1.65% | -1.59% | -2.01% | 108% | 101% | -0.38% | -1.15% | -1.13% | 102% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.22% | -1.83% | -1.55% | 106% | 100% | -0.69% | -1.27% | -0.78% | 102% | 100% |
| Class C | -1.35% | -1.81% | -1.77% | 107% | 101% | -0.69% | -0.87% | -1.18% | 102% | 100% |
| Class E | -0.77% | -1.31% | -0.47% | 105% | 100% | -0.26% | -1.15% | -1.56% | 101% | 99% |
| Overall | -1.15% | -1.69% | -1.36% | 106% | 100% | -0.58% | -1.11% | -1.11% | 102% | 100% |
| Class D | -1.33% | -1.43% | -1.97% | 107% | 102% | -0.56% | -0.53% | -0.63% | 102% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

# Subtest 7.3:  Quantization Step Size

The average test results for subtest 7.3 are summarized in the following table. Since the test 7.3.1 targets an improvement of subjective quality, for this test, additional bit rate savings were calculated using the MS-SSIM (JEM implementation) as distortion metric.

Table 18: Average test results for subtest 7.3.

## Test 7.3.1:  Adaptive quantization step size scaling

### Tool description (copied from JVET-J1027 [5])

Visual distortion of a block is influenced by the background of the block. Luminance masking and contrast masking effect of the background are considered in the quantizer design, in order to make a more perceptually uniform quantizer. The supporting background are two lines around the current CU.

A luminance factor fl(Ba) is derived based on the average luminance of the background Ba (normalized to 8-bit),

flBa=1+60-Ba150,       Ba∈0,60   1,                      Ba∈60,170       1+Ba-170425,    Ba∈170,255.

A contrast masking factor fcBc is derived based on the Mean Absolute Difference (MAD) between the background luminance and the average luminance (i.e., a way to measure the contrast Bc).

fcBc=1,            Bc≤5Bc+510,     Bc>5.

The scaler is based on the product of the two factors and is constrained in a range of [2/2,2]. The scaler is then applied to scale the quantization step size. For the flexibility of the quantizer, a slice-header parameter s is signaled to further adjust the scaler. At encoder, the parameter s is derived from the geometric mean of masking factors calculated from original luma samples of 8x8 blocks inside the slice, and is written into the slice header.

[CE coordinator: The cross checker noted that the coded uses 16x16 instead of 8x8 blocks, but the 16x16 blocks are subsampled by a factor of two in horizontal and vertical direction.]

qs'= qsQP×scaler=qsQP×clipflBa×fcBc×s2, 0.5, 2.0

To avoid floating-point calculation, the factors and scaler are all integerized into a 9-bit integers, and the squared root operation is realized by a look-up table with 33 entries.

The quantization step size scaler is derived based on the reconstructed luma samples around the current CU, which is the same as that in the decoder side. For a same position CU reached from different split routes, the surrounding reconstructed pixels may be different and hence the scaler may be different. The scaler is then used in the quantization process to adjust the quantization step size.

### Test results

The proposed tool was integrated in the BMS-1.0 software. The software was made available to CE7 participant in the following svn branch:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-3.1-0

Simulations were conducted according to the common test conditions [4]. Two configurations were tested:

- the proposed tool was enabled on top of the VTM-1 (using BMS-1 in VTM configuration);

- the proposed tool was enabled on top of the BMS-1.

Two different versions of the adaptive quantization step size scaling were implemented (one of the two versions can be selected in the software by setting a corresponding macro) and tested:

- version 1: CU-adaptive scaling of the quantization step size (as described in sec. 8.1.1);

- version 2: CU-adaptive scaling of the reconstructed residual (after inverse quantization).

Version 2 uses the same scaling factor, but applies the scaling at a different point in the reconstruction process. Conceptually, both approaches are the same (only due to rounding in the inverse transform, the results are slightly different).

Since the goal of the adaptive quantization step size/residual scaling is too increase the perceptual quality, the reconstruction quality was not only measure using PSNR, but also using MS-SSIM (source code from JEM was re-used), which is considered to provide a better metric for the perceptual quality.

The test results reported by the proponent are summarized in Table 19 (VTM configuration with quantization step size scaling), Table 20 (VTM configuration with residual scaling), Table 21 (BMS configuration with quantization step size scaling), and Table 22 (BMS configuration with residual scaling).

Table 19:  VTM test results of test 7.3.1 (version 1: adaptive scaling of quantization step size).

Table 20:  VTM test results of test 7.3.1 (version 2: adaptive scaling of residual).

Table 21:  BMS test results of test 7.3.1 (version 1: adaptive scaling of quantization step size).

Table 22:  BMS test results of test 7.3.1 (version 2: adaptive scaling of residual).

### Cross checks

Both the test results and the implementation were verified by at least one cross checker. Details are provided below.

cross check 1 for test 7.3.1:

cross checker: H. Schwarz (Fraunhofer HHI)

cross check of test results: - the cross checker tested the implementation provided by Huawei in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-3.1-0 - simulations were run for both the VTM and BMS configuration and all test cases - both versions, version 1 with adaptive quantization step size scaling and version 2 with adaptive residual scaling were tested - the cross checker reported that the test results (rate, PSNR, and MS-SSIM values) could be reproduced; some individual MS-SSIM values differed in the last digit of precision (by 1), which is as a result of using different approaches for averaging the frame-based values - the cross checker reported that the measured encoding and decoding time increases are similar to those reported by the tester (i.e., the deviations are in the expected range)

source code analysis: - the cross checker studied the software and confirmed that the implementation matches the description in the CE document (even though not all implementation details are described in the CE description, the CE description covers the most important aspects) - the scaling of the residual signal (after inverse transform), which was not described in the CE specification, represents a straightforward modification of the original approach (instead of scaling the quantization step size, the reconstructed residual is scaled with the same factor) - both the scaling of the quantization step size and the scaling of the reconstructed residual yield virtually the same results

additional remarks: - the cross checker remarked that a similar effect might also be achieved by an encoder only approach, i.e., by deriving and transmitting delta QP values (using the same concept as for the derivation of quantization scaling factors) - the cross checker suggests that the following aspects are investigated in future experiments: - Are scaling factors with fractional QP precision required for achieving the improvement in subjective quality? - How large is the rate increase that would be observed if delta QP values were transmitted (instead of changing the quantization step size in the decoding process)? - In an encoder only approach that actual signal of a CU (instead of border samples) could be used for deriving suitable delta QP values. Couldn’t this have a positive effect on subjective quality? - in a visual comparison, it was difficult to see any differences between the proposed version and the fixed QP reference (bit rates deviate) - it is suggested to compare the approach with an encoder-only version of the approach and other encoder-only QP adaptation technologies - for measuring MS-SSIM, the cross checker suggest to implement the metric into a separate tool, which can be run after decoding; using encoder outputs for quality measurement is inappropriate

cross check 2 for test 7.3.1:

cross checker: P. de Lagrange (Technicolor)

cross check of test results: - the cross checker tested the implementation provided by Huawei in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-3.1-0 - the results provided by proponent of CE7-3.1 have been checked in Random Access, VTM, classes B+C+D, default settings [details will be provided] - runtime was not checked because of inconsistent execution environment leading to inaccurate timings. - Reconstructed sequences were not viewed yet; relevance of psycho-visual adaptation has not been checked

source code analysis: - Setting aside the JVET_F0064_MSSSIM macro, 503 lines in 18 files are impacted. - It has been verified that the code contains the features described in the CE document (JVET-J1027) and in an updated description document provided by the proponent. - Code review has clarified the following: - fl and fc tables are computed at startup and the computation is consistent with the document - Ba and Bc are normalized to 8-bit range (using SPS bit depth information), Ba is further divided by 4 and Bc is clipped to 64, which limits fl and fc tables to 65 entries (0-64 range) - the derivation of Ba, Bc, and scaler is performed in CodingUnit::deriveQSFactor() and is consistent with the document: Ba is the average luminance of reconstructed pixels in two lines above and two columns to the left of the current CU. Bc is the average absolute difference of pixels in the border and Ba. scaler (qsFactor in the code) is then derived as described. - on decoding side, scaling is applied either to quantizer scale if AQS_RESIDUAL_SCALE is zero, or residual coefficients otherwise (after inverse transform) - scaling applies to luma samples only - the additional slice-level normalization syntax element is an index addressing two different hard-coded tables depending on its sign

additional remarks: - Proposed technology is an implicit psycho-visual adaptive quantization technique: a per-CU quantizer adjustment is derived locally, not transmitted. - This saves the bits that would be needed to transmit this adjustment, at the expense of a fixed adjustment law, that must work on pixels already available when decoding the current CU. In order to separate these three aspects it is suggested to run further tests: - perform the same adjustment at encoder side, and by making use of traditional delta-QP signaling, evaluate the bit savings - use the same adjustment law at encoder side, but using current CU source pixels instead of reconstructed border, and still transmitting the adjustment. This way the impact of the restriction to CU border could be judged - Quantizer adjustment at the finest granularity (CU-level) is regarded as valuable; restriction to luma may be questioned. - It is thought that the problem of decoding pipeline, because of an additional dependency of quantizer scale on surrounding CUs reconstruction, and the impact of applying the scaler on either the quantizer step or residual, may need more explanations or further study. - Semantics of the slice header syntax element added to transmit a normalization value could be simpler.

## Test 7.3.2:  Extension of QP value range

### Tool description (copied from JVET-J1027 [5])

The quantization parameter value range is further extended to the maximum value equal to 57 from 51. It is suggested to use the QP values 32, 37, 42, and 47 for testing this particular tool.

### Test results

The proposed tool was integrated in the BMS-1.0 software. The software was made available to CE7 participant in the following svn branch:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-3.2-0

Simulations were conducted according to the common test conditions [4] with modifications noted below. Two configurations were tested:

- the proposed tool was enabled on top of the VTM-1 (using BMS-1 in VTM configuration);

- the proposed tool was enabled on top of the BMS-1.

The tool was only tested for the “random access” configuration; it does not have any effect for “all intra” and the “low delay B/P” configurations. Furthermore, instead of using the QP values 22, 27, 32, 37 specified in the common test conditions, the tool was tested for the QP values32, 37, 42, 47.

The test results reported by the proponent are summarized in Table 23.

Table 23:  Test results of test 7.3.2 (extension of QP value range).

### Cross checks

The test results (for all test cases and classes) were verified by at least one cross checker. Details are provided below.

cross check 1 for test 7.3.2:

cross checker: Y. Kidani (KDDI)

cross check of test results: - the cross checker tested the implementation provided by KDDI in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-3.2-0 - simulations were run for both the VTM and BMS configuration and the test cases “random access” (the tool does not have any effect for the other test cases); the optional class F was not tested - the cross checker reported that the test results (rate and PSNR values) could be reproduced exactly - the cross checker reported that the measured encoding and decoding time increases are similar to those reported by the tester (i.e., the deviations are in the expected range)

source code analysis: - not reported

additional remarks: - none

cross check 2 for test 7.3.2:

cross checker: P. de Lagrange (Technicolor)

cross check of test results: - the cross checker tested the implementation provided by KDDI in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-3.2-0 - The results provided by proponent of CE7-3.2 have been checked in random access, VTM, B+C+D classes; Bitrate and PSNRs were found to match proponent results; Runtime was not checked because of inconsistent execution environment leading to inaccurate timings - Reconstructed sequences were not viewed yet; visually checking the effect of high QP on some pictures may be useful. The quality of high-QP encodings may be far from realistic use cases

source code analysis: Source code inspection reveals the following: - 32 lines in 7 files are impacted - MAX_QP has been raised from 51 to 57 - QP-indexed tables in bilateral filter and deblocking filter have been extended by linear extrapolation of the last few values - reading and writing of delta-QP has been modified to reflect change of modulo bounds. However the modification seems incomplete. It is thought that it should read qpY = ((predQP + DQp + (MAX_QP + 1) + 2 * qpBdOffsetY) % ((MAX_QP + 1) + qpBdOffsetY)) - qpBdOffsetY; instead of qpY = ((predQP + DQp + 52 + 2 * qpBdOffsetY) % ((MAX_QP + 1) + qpBdOffsetY)) - qpBdOffsetY; for reading, and DQp = (DQp + (MAX_QP + 1) + (MAX_QP + 1)/2 + qpBdOffsetY + (qpBdOffsetY / 2)) % ((MAX_QP + 1) + qpBdOffsetY) - (MAX_QP + 1)/2 - (qpBdOffsetY / 2); instead of DQp = (DQp + 78 + qpBdOffsetY + (qpBdOffsetY / 2)) % ((MAX_QP + 1) + qpBdOffsetY) - 26 - (qpBdOffsetY / 2); for writing. Note that this has no impact in common test conditions, where local QP adjustment is not used

additional remarks: - QP range extension is regarded as welcome. The cross-checker had experience in a professional encoder manufacturing company, where QP range extension (up to 75) was used through quantization matrix scaling, which was helping bitrate regulation in extreme cases, and graceful degradation in very hard to encode scenes (very complex, or noise). The visual improvement was obvious. - Extended values for Tc table (DBF) may need review. - Inspection of rate-distortion curves reveals that the bottom of the curve is significantly altered sometimes, which is not always matching bdrate numbers. - Obvious effects were noted for Tango2, CampfireParty2, RitualDance, BaskballDrive, and BasketballDrill. - Inspection of per-picture QPs reveals saturation at 51 for anchor: Anchor Proposed % of total pictures \ Seq QP 32 37 42 47 32 37 42 47 Pic type I 29 34 39 44 29 34 39 44 2.2 B0 33 38 43 48 33 38 43 48 4.2 B1 35 41 46 51 35 41 46 51 6.2 B2 38 44 49 51 38 44 49 54 12.4 B3 40 45 50 51 40 45 50 55 25.0 B4 41 46 51 51 41 46 51 56 50.0 - Inspection of partial RD curves per picture type (temporal level), provided for C_BasketballDrill in the following figures, gives more details about the effect of QP saturation: - left-bottom part of the 3 lower curves (for B2,B3,B4 picture) are distorted on anchor (colored lines), and smooth in proposal (dashed lines) - Accumulated rates and distortion, still for C_BasketballDrill, are represented in the following figure and show the result of combining the different pictures types. The bottom curve is the final one.

## Test 7.3.3:  Derivation of chroma QP from luma QP

### Tool description (copied from JVET-J1027 [5])

Derivation of the chroma quantization parameter from the associated luma quantization parameter is further controlled by the specified maxDeltaDiffQpc, which determines the maximum difference between the input index qPi and the output value Qpc of the chroma qp mapping function. The chroma quantization parameter is derived by

Qpc = qPi – min(maxDiffQpc, qPi - Qpc )

maxDeltaDiffQpc equal to 5, 4, 3, and 2, respectively, will be signaled in the PPS and tested with the chroma QP offset parameters pps_cb_qp_offset and pps_cr_qp_offset equal to 1 under the RA configuration, and equal to 0 under the LB configuration. The further results generated by only adjusting the chroma QP offset parameters with pps_cb_qp_offset and pps_cr_qp_offset equal to 0, 1, and 2, respectively, will be provided.

### Test results

The proposed tool was integrated in the BMS-1.0 software. The software was made available to CE7 participant in the following svn branch:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-3.3-0

Simulations were conducted according to the common test conditions [4]. Two configurations were tested:

- the proposed tool was enabled on top of the VTM-1 (using BMS-1 in VTM configuration);

- the proposed tool was enabled on top of the BMS-1.

The test results reported by the proponent are summarized in Table 24.

Table 24:  Test results of test 7.3.3 (derivation of chroma QP from luma QP).

### Cross checks

The test results (for all test cases and classes) were verified by at least one cross checker. Details are provided below.

cross check 1 for test 7.3.3:

cross checker: Y. Kidani (KDDI)

cross check of test results: - the cross checker tested the implementation provided by KDDI in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-3.3-0 - simulations were run for both the VTM and BMS configuration and all normative test cases “all intra”, “random access”, and “low delay B”; the optional class F was not tested - the cross checker reported that the test results (rate and PSNR values) could be reproduced exactly - the cross checker reported that the measured encoding and decoding time increases are similar to those reported by the tester (i.e., the deviations are in the expected range)

source code analysis: - not reported

additional remarks: - none

cross check 2 for test 7.3.3:

cross checker: P. de Lagrange (Technicolor)

cross check of test results: - the cross checker tested the implementation provided by KDDI in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-3.3-0 - The results provided by proponent of CE7-3.3 have been checked in random access mode, VTM, A+B+C+D classes, default settings. - Bitrate and PSNRs were found to match proponent results. - Runtime was not checked because of inconsistent execution environment leading to inaccurate timings. - Reconstructed sequences were not viewed yet; relevance of luma/chroma quantization balance has not been checked. The would be useful since the proposal balances out chroma PSNR in favor of luma, with significant success for RA class A1. Success seems low in LDB and moderate in AI

source code analysis: - Setting aside configuration files, 200 lines in 15 files are impacted - Source code inspection reveals the following: - new luma to chroma Qp mapping is implemented in ChromaFormat.h, getScaledChromaQP(), and matches the CE description document: - first, the difference between luma Qp and old mapping is computed - then, this difference is clipped to the provided maximum value (additional "maxDeltaQp" parameter) - the returned chroma Qp is the luma Qp minus the clipped difference - most of the remaining modifications relates to the addition of PPS and slice syntax elements to control the mapping (maxDeltaQp) for Cb and Cr components - the rest relates to initialization and getScaledChromaQP() calls with correct additional parameter - Default configuration files, which have been used to check proponent results, set CbQpOffset and CrQpOffset to zero whereas CE description document states that the proposal would be tested with those settings set to 1 for RA. The proponent explained that this change matches the evolution of VTM common test conditions

additional remarks: - Study of the luma to chroma Qp mapping function seems useful. - This mapping seems useful for local Qp adjustment only, since PPS and Slice cb/cr chroma Qp adjustment is already available. - It was asked whether the purpose of this test is to select the best mapping, or to make it adjustable, to which the proponent answered by the second option. - It has been noted that many offsets and adjustments apply to chroma Qp, with different rules for deblocking filter (local adjustment ignored), which may need review. - The proponent has provided more results with other maxDeltaQp settings, and a comparison with pps_cb/cr_qp_offset=1 without mapping modification, advocating that the proposed technique is superior. However, for a fair comparison, pps and slice cb/cr_qp_offset shall be adjusted for every QP to follow the competing mapping, so that slice chroma QPs are exactly the same

# Subtest 7.4:  Scanning Order

The average test results for subtest 7.4 are summarized in the following table.

Table 25: Average test results for subtest 7.4.

## Test 7.4.1:  Block size dependent coefficient scanning

### Tool description (copied from JVET-J1027 [5])

The proposed coefficient scanning is modified based on the ratio of block size. When the ratio of horizontal and vertical size is equal or larger than four, mode-dependent coefficient scanning is applied. The MDCS itself is the same as in benchmark test set.

Benchmark test set and the proposed decision maps are shown in the following figure.

It is noted that transform block size 64x64 luma and 32x32 chroma are excluded in benchmark set, so these blocks are removed from shown following figure.

|  | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- |
| 4 | M | M | D | D | D |
| 8 | M | M | D | D | D |
| 16 | D | D | D | D | D |
| 32 | D | D | D | D | D |
| 64 | D | D | D | D | D |

Decision table based on block sizes in bechmark set

|  | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- |
| 4 | M | M | D | D | D |
| 8 | M | M | D | M | M |
| 16 | D | D | D | D | M |
| 32 | D | M | D | D | D |
| 64 | D | M | M | D | D |

Proposed decision table based on block sizes

Regarding a test, both normal QP range and lower QP range are required to study the effectiveness.

### Test results

The proposed tool was integrated in the BMS-1.0 software. The software was made available to CE7 participant in the following svn branch:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-4.1-0

Simulations were conducted according to the common test conditions [4]. Two configurations were tested:

- the proposed tool was enabled on top of the VTM-1 (using BMS-1 in VTM configuration);

- the proposed tool was enabled on top of the BMS-1.

The test results reported by the proponent are summarized in Table 26.

Table 26:  Test results of test 7.4.1 (block size dependent coefficient scanning).

### Cross checks

The test results (for all test cases and classes) were verified by at least one cross checker. Details are provided below.

cross check 1 for test 7.4.1:

cross checker: M. Koo (LG)

cross check of test results: - the cross checker tested the implementation provided by KDDI in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-4.1-0 - the cross checker ran all required simulations under both VTM and BMS settings and confirmed that there exist no problems in the proponent results. - The cross checker found that the fourth or fifth digit of BD-rate percentage numbers in some of sequences differed from the proponent ones, but also confirmed that it was just caused by rounding errors during the accumulation of results from parallel encoding. - the cross checker confirmed all the results for all test conditions (AI, RA, and LD under both VTM and BMS settings)

source code analysis: - the cross checker commented: The SW implementation was very concise and straightforward. I also confirmed what it was doing and it was perfectly matched with what it was intended

additional remarks: the cross checker remarked: - Basically, in my opinion, optimal scannings for transform units of diverse block shapes which are derived from different prediction modes should be continuously investigated. - Therefore, I encourage that any new scannings including the approach of CE 7.4.1 should be further studied in the same CE or a newly established CE

cross check 2 for test 7.4.1:

cross checker: B. Wang (Huawei)

cross check of test results: - the cross checker tested the implementation provided by KDDI in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-4.1-0 - simulations were run for both the VTM and BMS configuration and all normative test cases “all intra”, “random access”, and “low delay B” - the cross checker reported that the test results (rate and PSNR values) could be reproduced exactly - the cross checker reported that the measured encoding and decoding time increases are similar to those reported by the tester (i.e., the deviations are in the expected range)

source code analysis: - not reported

additional remarks: - none

## Test 7.4.2 [withdrawn]

Test 7.4.2 was withdrawn.

# Subtest 7.5:  Sign Prediction

The average test results for subtest 7.5 are summarized in the following table.

Table 27: Average test results for subtest 7.5.

## Test 7.5.1:  JVET-J0025 sign prediction in transform domain for both intra- and inter-picture predicted blocks

### Tool description (copied from JVET-J1027 [5])

Technology was proposed in JVET-D0031. Transform domain residual sign prediction (TD-RSP) has similar parsing processes.  As the result of parsing process, decoded sign values’ positions and corresponding sign prediction errors of coefficients are obtained and stored in sign_pred_pos_list.

Main difference between RSP (JVET-D0031) and TD-RSP is that all calculations are performed in transform domain. The proposed technology comprises the following steps:

- Preparation prediction of boundary residuals

- Calculation spectrum of boundary line

- Calculation cost function for hypothesis

- Sign values decoding

Preparation prediction of boundary residuals

Pixels used to derive cost value

The first step to get prediction of boundary residuals is to predict boundary pixels of current block. To realize it, a linear extrapolation of lines (Xi,j)/columns (Zi,j) is used:

PXi=2Xi,-1-Xi,-2PZi=2Z-1,i-Z-2,i

where PXi and PZi are linear extrapolations.

Pixels of a current block could be presented as a sum of residuals and an inter/intra-predictor:

Yi,j=Ri,j+Pi,j

where Ri,j and Pi,j denote residuals and a predictor, respectively.

The second step is to remove an inter/intra predictor and to perform forward transform:

xn=TPXi-Ri,0zm=TPZi-R0,i

[CE coordinator: The cross checker indicated that there is a typo in the formula above. Ri,0 should be replaced with Pi,0, and R0,i should be replaced with P0,i.]

Calculation spectrum of boundary line

Spectrum of neighboring boundary could be calculated as follows:

lyn=m=0M-1ωmRn,mcosπm2M,

tym=n=0N-1ωnRn,mcosπn2N,

where ωi=1M, i=02M, i>0, M is block height.

Calculation cost function for hypothesis

Cost function values should be calculated for every combination of signs to be predicted. So, the number of hypotheses is 2max_num_pred_signs . SSE is used as cost function:

costk=n=0N-1xn-lyn2+m=0M-1zm-tym2

Sign values decoding

Similarly to RSP (as described in JVET-D0031), sign positions are consequently fetched from sign_pred_pos_list starting from the 1st one. At every step, hypothesis having minimal cost function value is chosen and coefficient signs are assigned in accordance with corresponding sign prediction errors. If a sign prediction error value is 1, a coefficient sign is assigned as specified by the hypothesis. Otherwise, the sign is inverted, i.e. “+” is changed to “–” and vice versa. Only those hypotheses are further checked that specify the same signs as previously restored using sign prediction error values as described above. This process is repeated for each element of sign_pred_pos_list.

TD-RSP is applied to restore signs of luma quantized transform coefficients only. Chroma coding blocks are not processed by TD-RSP.

In this first test, the sign prediction is used for both intra- and inter-picture predicted blocks.

### Test results

The proposed tool was integrated in the BMS-1.0 software. The software was made available to CE7 participant in the following svn branch:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-5.1-0

Simulations were conducted according to the common test conditions [4]. Two configurations were tested:

- the proposed tool was enabled on top of the VTM-1 (using BMS-1 in VTM configuration);

- the proposed tool was enabled on top of the BMS-1.

The test results reported by the proponent are summarized in Table 28.

Table 28:  Test results of test 7.5.1 (transform domain sign prediction for intra- and inter blocks).

### Cross checks

Both the test results (for all test cases and classes) and the implementation were verified by at least one cross checker. Details are provided below.

cross check 1 for test 7.5.1:

cross checker: Felix Henry (Orange)

cross check of test results: - the cross checker tested the implementation provided by Huawei in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-5.1-0 - simulations were run for both the VTM and BMS configuration and all test cases “all intra”, “random access”, “low delay B”, “low delay P” (including the optional class F) - the cross checker reported that the test results (rate and PSNR values) could be reproduced exactly - the cross checker reported that the measured encoding and decoding time increases are similar to those reported by the tester (i.e., the deviations are in the expected range)

source code analysis: - the cross checker carefully studied the software and confirmed that the implementation matches the description in the CE document (except one typo in the CE description, see note in sec. 10.1.1)

additional remarks: - We have been checking the impact of reducing the number of reference samples needed to perform the sign prediction and we have observed, based on a limited set of coding points because of our cluster load, that it has almost no impact on compression performance. We will make a contribution if we can obtain full results on our cluster. - The proposal make the residue decoding stage more serial because it relies on neighboring reconstructed samples to decode the signs of the residue. If this is deemed unacceptable, we would recommend to keep Residue Sign Prediction in CE7 and ask the proponent to study this issue in a new experiment, as there seem to be different approaches that can solve or mitigate the problem

cross check 2 for test 7.5.1:

cross checker: M. Gao (Hulu)

cross check of test results: - the cross checker tested the implementation provided by Huawei in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-5.1-0 - partial verification (incomplete data).

source code analysis: - not reported

additional remarks: - none

## Test 7.5.2:  JVET-J0025 sign prediction in transform domain for intra-picture predicted blocks

### Tool description

The sign prediction is the same as the one described in sec. 10.1.1.  The difference between test 7.5.1 and test 7.5.2 is that, in test 7.5.2, the sign prediction is only used for intra-picture predicted blocks.

### Test results

The proposed tool was integrated in the BMS-1.0 software. The software was made available to CE7 participant in the following svn branch:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-5.2-0

Simulations were conducted according to the common test conditions [4]. Two configurations were tested:

- the proposed tool was enabled on top of the VTM-1 (using BMS-1 in VTM configuration);

- the proposed tool was enabled on top of the BMS-1.

The test results reported by the proponent are summarized in Table 29.

Table 29:  Test results of test 7.5.2 (transform domain sign prediction for intra blocks).

### Cross check

Both the test results (for all normative cases and sequence classes) and the implementation were verified by the cross checker. Details are provided below.

cross check for test 7.5.2:

cross checker: Tung Nguyen (Fraunhofer HHI)

cross check of test results: - the cross checker tested the implementation provided by Huawei in the svn branch https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE7/CE7-5.2-0 - simulations were run for both the VTM and BMS configuration and the normative test cases “all intra”, “random access”, and “low delay B”; the optional test case “low delay P” and the optional class F were not tested - the cross checker reported that the test results (rate and PSNR values) could be reproduced exactly - the cross checker reported that the measured encoding and decoding time increases are similar to those reported by the tester (i.e., the deviations are in the expected range)

source code analysis: - the cross checker studied the software and confirmed that the implementation matches the description in the CE document

additional remarks: - all signs of a transform block are coded after all absolute values of the transform block have been coded - after the reconstruction of the absolute levels for a transform block, only positions with absolute transform coefficients (after scaling) greater than 300 are considered as candidates for sign prediction - sign prediction is conducted for up to 5 candidates per transform block - sign prediction is supported for transform blocks up to a size of 32

# References

- ITU-T and ISO/IEC, “High Efficiency Video Coding (HEVC),” ITU-T Rec. H.265 and ISO/IEC 23008-2, version 5, Feb. 2018.

- B. Bross, “Versatile Video Coding (Draft 1),“ Joint Video Experts Team, doc. JVET-J1001, San Diego, CA, USA, Apr. 2018.

- J. Chen, E. Alshina, “Algorithmic Description for Versatile Video Coding and Test Model 1 (VTM 1),” Joint Video Experts Team, doc. JVET-J1002, San Diego, CA, USA, Apr. 2018.

- J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Joint Video Experts Team, doc. JVET-J1010, San Diego, CA, USA, Apr. 2018.

- H. Schwarz, M. Coban, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” Joint Video Experts Team, doc. JVET-J1027, San Diego, CA, USA, Apr. 2018.