| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0045-v1 |
| --- | --- |

| Title: | CE3.7.2: Distance-Weighted Directional Intra Prediction (DWDIP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Jianle Chen | Tel: Email: | +7-495-660-44-59 alexey.filippov@huawei.com vasily.rufitskiy@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

According to test results, the tool provides average -0.3% of luma BD rate gain for VTM software and -0.4% of luma BD rate gain for BMS software in all intra configuration. Encoding and decoding time increase for all intra configuration are reported to be approximately 105% and 102%, respectively, for both VTM and BMS.

# Introduction

This proposal reports test results for the experiment defined in [1]. A tool being studied is a bi-directional prediction technique that is applicable to all the directional intra prediction modes. For negative angles, secondary reference samples are generated and block prediction process utilizes both primary and secondary reference samples’ set. In the rest cases prediction is performed from samples on the left and top sides of the block being predicted.

# Tool summary

Distance-Weighted Directional Intra Prediction (DWDIP) is defined for all directional intra prediction modes, used in VTM and BMS software. A decision whether to apply DWDIP or not is signalled by a flag at a PU level.

Figure 1 illustrates the process of intra prediction, when DWDIP is enabled for a PU. Prediction signal is obtained using the following formula:

,

denotes a reference sample that is used by conventional intra prediction,

denotes a reference sample that is located on the opposite side relative to .

Figure 1. Overview of DWDIP

Weight  is not calculated directly, but is determined for a first row of predicted samples using lookup table and is updated for each row and column positions using addition and shift operations. Weight value for the first row depends on the number of rows to predict. For some positions within a row of samples weight gradually changes. The amount of this change is defined by intra prediction angle and is stored in a LUT.

If intra prediction mode defined for a PU corresponds to positive angle prediction angle, sample is predicted from top and left sides of reference samples.

If intra prediction mode defined for a PU corresponds to negative angle prediction angle, sample is predicted from either left or top side and secondary reference samples. Secondary reference samples are generated as shown in Figure 2.

Figure 2. Generation of secondary reference samples

Directionally predicted values are calculated using intra prediction process. Combination of directional and gradual components is performed by calculation of their weighted sum:

.

Gradually interpolated values are calculated by the means of linear interpolation, i.e. calculating the value of step  and using this step value to compute the gradually interpolated values:

.

Value  is stored in a look-up table.

# Experimental results

Experimental results given in Table 1 were obtained using test conditions [2].

Table 1. Results over VTM and BMS

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -0.43% | -0.36% | 107% | 100% | -0.55% | -0.50% | -0.42% | 106% | 99% |
| Class A2 | -0.21% | -0.24% | -0.18% | 107% | 100% | -0.31% | -0.48% | -0.45% | 106% | 100% |
| Class B | -0.23% | -0.28% | -0.23% | 106% | 101% | -0.31% | -0.33% | -0.40% | 106% | 100% |
| Class C | -0.24% | -0.38% | -0.43% | 105% | 100% | -0.25% | -0.44% | -0.40% | 104% | 99% |
| Class E | -0.32% | -0.36% | -0.26% | 106% | 100% | -0.41% | -0.40% | -0.57% | 105% | 99% |
| Overall | -0.28% | -0.33% | -0.29% | 106% | 100% | -0.35% | -0.42% | -0.44% | 105% | 99% |
| Class D | -0.17% | -0.32% | -0.17% | 105% | 99% | -0.18% | -0.29% | -0.20% | 104% | 98% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.49% | -0.35% | 101% | 99% | -0.38% | -0.50% | -0.66% | 101% | 100% |
| Class A2 | -0.12% | -0.07% | -0.06% | 100% | 99% | -0.17% | -0.34% | -0.19% | 100% | 101% |
| Class B | -0.17% | -0.15% | -0.36% | 100% | 99% | -0.23% | -0.34% | -0.50% | 100% | 100% |
| Class C | -0.22% | -0.29% | -0.37% | 99% | 98% | -0.15% | -0.28% | -0.59% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.20% | -0.24% | -0.30% | 100% | 99% | -0.23% | -0.36% | -0.49% | 101% | 100% |
| Class D | -0.10% | -0.14% | -0.41% | 99% | 97% | -0.04% | -0.40% | -0.34% | 101% | 100% |

# References

- G. Van der Auwera, J. Heo, A. Filippov, JVET-1023 “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding”, 10th Meeting: San Diego, USA, 10–20 Apr. 2018, JVET-J1023.

- J. Boyce, K. Suehring, X. Li, V. Seregin, JVET-J1010, “JVET common test conditions and software reference configurations”, April 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).