| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0061-v1 |
| --- | --- |

| Title: | CE3: Bilateral intra reference sample filter (Test 2.1.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Vadim Seregin Adarsh Krishnan Ramasubramonian Marta Karczewicz | Email: | geertv@qti.qualcomm.com vseregin@qti.qualcomm.com aramasub@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The results of test CE3-2.1.1 on bilateral filtering of intra reference samples are reported. It is asserted that the bilateral filter reduces noise while preserving edges. In the test, the decision whether to use reference sample filtering is kept identical to that in VTM1 and BMS1. The software implementation is stated to be flexible in that it allows for future optimizations of the filter by tuning the filter strength parameters.

For test CE3-2.1.1 the reported BD-rates over BMS-VTM-1.0 are:

- AI: 0.03% (Y), −0.02% (U), −0.04% (V) with runtimes 100% (Enc) and 101% (Dec)

- RA: 0.00% (Y), 0.10% (U), 0.06% (V) with runtimes 100% (Enc) and 101% (Dec)

The reported BD-rates over BMS-1.0 are:

- AI: −0.13% (Y), −0.16% (U), −0.17% (V) with runtimes 100% (Enc) and 99% (Dec)

- RA: −0.04% (Y), 0.09% (U), −0.06% (V) with runtimes 100% (Enc) and 99% (Dec)

The AI and RA results of CE3-2.1.1 and its source code have been cross-checked by HHI [1].

# Proposal

The bilateral filter reduces noise while preserving edges. The experiment evaluates the bilateral filter application to the luma reference samples for intra prediction. The 5-tap bilateral filter taps are given by:

f = {f-2, f-1, f0, f1, f2}/( f-2+f-1+f0+f1+f2), where

with , the unfiltered reference at position , ,   and .

To save computational complexity, values of  and  are stored in look-up tables. The decision whether to use filtering is identical to VTM1 and BMS1. The software implementation is flexible and allows for future optimizations of the filter by optimizing the filter strength parameters.

# Test scenarios

The following test was included in CE3 [5]:

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.1.1 | Bilateral reference sample filter | Geert Van der Auwera (Qualcomm) | Philipp Merkle (HHI) |

# Results

The following results are obtained according to the JVET common test conditions [4] and the CE3 test guidelines [5]. The BMS1.0 software is used for both the BMS and VTM results. The anchors are also generated using the BMS1.0 software.

## ‘All Intra’, ‘Random Access’ results over BMS-VTM-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.10% | 0.06% | 101% | 100% |
| Class A2 | 0.07% | -0.01% | 0.00% | 100% | 99% |
| Class B | 0.08% | 0.01% | -0.02% | 101% | 102% |
| Class C | -0.10% | -0.14% | -0.10% | 101% | 103% |
| Class E | 0.08% | -0.06% | -0.13% | 98% | 99% |
| Overall | 0.03% | -0.02% | -0.04% | 100% | 101% |
| Class D | 0.03% | 0.07% | 0.00% | 102% | 102% |
| Class F (optional) | -0.12% | -0.10% | -0.07% | 102% | 103% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.21% | 0.12% | 99% | 99% |
| Class A2 | -0.04% | 0.09% | 0.06% | 100% | 100% |
| Class B | 0.03% | 0.07% | 0.07% | 100% | 100% |
| Class C | 0.00% | 0.05% | -0.01% | 102% | 103% |
| Overall | 0.00% | 0.10% | 0.06% | 100% | 101% |
| Class D | 0.06% | 0.10% | -0.13% | 102% | 105% |
| Class F (optional) | -0.06% | 0.07% | 0.06% | 101% | 101% |

## ‘All Intra’, ‘Random Access’ results over BMS-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.08% | 0.01% | 100% | 101% |
| Class A2 | -0.09% | -0.20% | -0.13% | 100% | 99% |
| Class B | -0.04% | -0.08% | -0.15% | 100% | 101% |
| Class C | -0.32% | -0.43% | -0.34% | 100% | 97% |
| Class E | -0.08% | -0.13% | -0.18% | 100% | 97% |
| Overall | -0.13% | -0.16% | -0.17% | 100% | 99% |
| Class D | -0.01% | 0.03% | -0.03% | 100% | 100% |
| Class F (optional) | -0.27% | -0.43% | -0.55% | 97% | 95% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.15% | 0.03% | 100% | 101% |
| Class A2 | -0.08% | 0.05% | 0.10% | 100% | 101% |
| Class B | -0.03% | 0.02% | -0.08% | 101% | 100% |
| Class C | -0.01% | 0.17% | -0.23% | 99% | 96% |
| Overall | -0.04% | 0.09% | -0.06% | 100% | 99% |
| Class D | 0.02% | 0.09% | -0.16% | 100% | 100% |
| Class F (optional) | -0.08% | 0.01% | -0.26% | 100% | 101% |

# Acknowledgement

We are grateful to Fraunhofer HHI for conducting the cross-check for this test.

# References

- G. Van der Auwera, J. Heo, A. Filippov, “CE3: Summary Report on Intra Prediction and Mode Coding,” 11th JVET Meeting, Ljubljana, SI, July 2018, JVET-K0023

- J. Chen, E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1)”, 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1002

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of Joint Exploration Test Model 7,” 7th JVET Meeting, Torino, Italy, July 2017, JVET-G1001

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1010

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1023

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).