| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0176 |
| --- | --- |

| Title: | CE2: SAO Modification (CE2.3.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen Kiho Choi Chanyul Kim | Tel: Email: | +86-10-8443-9777-1548 jie1222.chen@samsung.com |
| Source: | Samsung Electronics Co.,Ltd |  |  |

_____________________________

- Abstract

This contribution presents some modifications of the SAO in VTM1.0. These modifications are firstly presented in JVET-J0024[1] and then further studied in the CE2[2]. Firstly, the SAO parameter regions don’t align with CTU in this contribution but are shifted from the CTU to the left and up by four pixels. Secondly, the classification condition of EO are modified by adding an offset and the range of EO offset is changed as well. Thirdly, in BO mode, the offset bands are not signaled but fixed to the extreme bands and the offsets to be added are quantized to save the bits. All the individual modifications are tested in this contributions and results are reported.

Introduction

The key idea of SAO is to reduce mean sample distortion of a region by first classifying region samples into multiple categories with a selected classifier, obtaining an offset for each category, and then adding the offset to each sample of category, where the classifier index and the offsets of the region are coded in the bitstream while the classification of each sample is performed at both the encoder and decoder with the selected classifier to save the amount of side information to be signaled.

To achieve low latency of only one Coding tree unit (CTU), a CTU-based syntax design is specified to adapt SAO parameters for each CTU. A CTU-based optimization algorithm can be used to derive SAO parameters of each CTU, and the SAO parameters for each CTU are interleaved into slice data. To further reduce the bits dedicated on SAO parameters, a merge flag is employed to indicate whether the SAO parameters of the current CTU is exact the same with its neighbors. When merge flag is enabled, all the following SAO parameters are not signaled but inferred from neighbors.

SAO uses different offset values for each sample in a CTU depending on the sample classification, and SAO parameters are adapted from CTU to CTU, that is, SAO parameters are derived and signaled for each CTU. Two types of classifiers are adopted in VTM1.0 (or HEVC): edge offset mode (EO) and band offset mode (BO). In edge offset mode, each sample in a CTU is classified into one of five categories based on the current sample and its neighbors specified by one of four pre-defined patterns shown in Figure 1, and the classification rule is specified in table 1. The offset values to be added to the reconstructed samples are signaled for each category except the category 0 in which the edge offset is not used. In band offset mode, the whole sample value range is divided into 32 bands, and the bands to be offset and the offset values are derived and signaled shown as Figure 2.

Figure 1. Four 1-D EO classification pattern

Table 1. EO classification rule

| Category | Condition | Offset Range |
| --- | --- | --- |
| 1 | c < a && c < b | 0 <= offset <= 7 |
| 2 | ( c < a && c=b) ¦¦ (c=a && c < b) | 0 <= offset <= 7 |
| 3 | ( c > a && c=b) ¦¦ ((c=a && c > b) | –7 <= offset <= 0 |
| 4 | c > a && c > b | –7<= offset <= 0 |
| 0 | None of the above | None |

Figure 2. Bands to be offset in BO

Proposed modification

EO classification modification (CE2.3.4.a)

The classification rule of EO proposed in this contribution is similar as before. Four 1-D patterns in Figure 1 are used for pixel value comparison and the class index depends on the comparison results. Only one change is that a threshold with value of 3 is introduced in the value comparison as in Table 2.

Table 2. Proposed classification condition and offset range for EO

| Category | Condition | Offset Range |
| --- | --- | --- |
| 1 | c < a -3 && c < b -3 | 0 <= offset <= 10 |
| 2 | (c < a -3 && ¦c-b¦≤3) ¦¦ (¦c - a¦≤3 && c < b -3) | 0 <= offset <= 10 |
| 3 | (c > a +3 && ¦c-b¦≤3) ¦¦ ((¦c - a¦≤3 && c > b +3 ) | –10 <= offset <= 0 |
| 4 | c > a +3 && c > b +3 | –10 <= offset <= 0 |
| 0 | None of the above | None |

BO offset quantization (CE2.3.4.b)

In band offset (BO) mode, the offset derived is quantized first and then transmitted in the bitstream. The offset value added to the pixels is the value after inv-quantization. The quantization method used in the contribution is directly right shift 2 bits.

Fixed Offset band (CE2.3.4.c)

In VTM/BMS SAO, four bands of total thirty-two bands are selected and signaled as offset bands by encoder, while in this contribute the fixed offset band is proposed. The whole sample value range is equally divided into 32 bands. For 10-bit samples ranging from 0 to 1023, the width of a band is 32, and sample values from 32k to 32k+31 belong to band k, where k ranges from 0 to 31. The offset bands are fixed to extreme band (e.g., band 0, band 1, band 30 and band 31) shown as Figure 5. Thus, there is no need to signal which band to be offset in the bitstream.

Figure 5. Proposed offset bands

Shifted structure (test 2.3.4.d)

SAO is applied after de-block filtering. In block-based pipeline scheme, when the current block is reconstructed, the right boundary and the bottom boundary cannot be de-block filtered until the right neighboring block and bottom neighboring block are reconstructed, since the mode information as well as reconstructed pixel values of the two neighboring blocks are needed in the deblocking of right and bottom boundary of the current block.

So, in practical implementation, the block for de-blocking filtering is shifted to the left-top compared with currently reconstructed block as shown in Figure 3. In Figure 3, the block with bold line is a block pipelined and the blue region is the currently reconstructed block. In Figure 3(a), the red line is the boundaries need to be de-block filtered in the currently reconstructed block. However, due to the de-blocking order and unavailability of right and bottom neighboring blocks, the actual boundaries being de-block filtered in the current stage is shown as red line in Figure 3(b). Because of the shifting of de-blocking region, the SAO region should also be shifted as Figure 3(c). In figure 3(c), the gray block is the current SAO processing region and the dotted line block gives the data region needed for current SAO processing region. Obviously, the current SAO processing region crosses four bold blocks and is divided into four sub-regions shown in Figure 3(d). These four sub-regions may have different SAO parameters, which leads to high complexity of SAO process and more memory is needed to store the different parameters for SAO1, SAO2, SAO3 and SAO4.

Figure 3. A practical implementation of SAO

To align the SAO parameter region with practical processing region, a shifted structure is proposed for SAO. In this structure, SAO parameters are derived and applied in CTU-level as before while the applied region is not exact a CTU but a region with the same size as CTU and shifted four pixels from a CTU to left-top as Figure 4 shows.

Figure 4. Proposed shifted structure

Offset range extension (CE2.3.4.e)

Anther modification is that the maximum value of offset range is extended to 10. For offset of EO mode the offset range is [-10,0] or [0,10] dependent on the class as in table 2, and for offset of BO mode the range offset is [-10,10].

Test results

The modifications of SAO were implemented on top of BMS1.0 s/w. Tests were conducted with VTM configuration and BMS configuration using one s/w under common test condition [3].

The complete test results could be found in the svn repository [4] and here a summary is provided.

Results of EO classification (Test 2.3.4.a)

The coding results of EO classification modification are give in the following tables.

Table 7. Test results of EO classification modification only in AI (Test 2.3.4.a)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -1.04% | -1.21% | 100% | 106% | -0.13% | -1.76% | -2.20% | 99% | 96% |
| Class A2 | -0.75% | -1.83% | -1.23% | 101% | 109% | -0.25% | -1.19% | -0.86% | 99% | 98% |
| Class B | -0.46% | -1.62% | -2.66% | 99% | 104% | -0.09% | -1.65% | -2.81% | 99% | 98% |
| Class C | -0.48% | -1.84% | -2.82% | 100% | 101% | -0.13% | -1.93% | -2.51% | 99% | 101% |
| Class E | -0.50% | -2.55% | -3.35% | 100% | 102% | -0.10% | -1.93% | -2.58% | 99% | 102% |
| Overall | -0.52% | -1.76% | -2.33% | 100% | 104% | -0.13% | -1.70% | -2.28% | 99% | 99% |
| Class D | -0.36% | -1.36% | -1.75% | 99% | 98% | -0.07% | -0.96% | -1.42% | 100% | 100% |
| Class F (optional) | -1.18% | -2.25% | -3.22% | 99% | 99% | -1.13% | -3.79% | -4.81% | 99% | 100% |
| Table 8. Test results of EO classification modification only in RA (Test 2.3.4.a) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.22% | -0.79% | -1.07% | 98% | 100% | -0.36% | -0.83% | -1.23% | 98% | 97% |
| Class A2 | -1.98% | -2.27% | -1.48% | 98% | 100% | -0.97% | -2.32% | -1.92% | 98% | 96% |
| Class B | -1.78% | -2.50% | -2.15% | 97% | 97% | -0.71% | -2.38% | -2.61% | 99% | 99% |
| Class C | -0.64% | -1.59% | -2.32% | 98% | 95% | -0.32% | -1.71% | -2.14% | 98% | 95% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.41% | -1.87% | -1.84% | 98% | 98% | -0.59% | -1.88% | -2.07% | 98% | 97% |
| Class D | -0.11% | -0.94% | -0.79% | 98% | 95% | -0.24% | -0.93% | -1.04% | 99% | 96% |
| Class F (optional) | -1.78% | -2.37% | -3.50% | 98% | 97% | -1.63% | -3.43% | -4.45% | 99% | 98% |
| Table 9. Test results of EO classification modification only in LDB (Test 2.3.4.a) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.36% | -4.35% | -5.12% | 98% | 99% | -1.14% | -4.17% | -5.43% | 101% | 101% |
| Class C | -0.83% | -3.61% | -6.24% | 99% | 100% | -1.03% | -4.43% | -6.24% | 99% | 100% |
| Class E | -0.97% | -2.01% | -3.58% | 99% | 99% | -1.28% | -2.25% | -3.49% | 100% | 100% |
| Overall | -1.09% | -3.51% | -5.11% | 99% | 99% | -1.14% | -3.77% | -5.21% | 100% | 101% |
| Class D | -0.30% | -2.13% | -1.91% | 99% | 96% | -0.57% | -2.59% | -2.60% | 100% | 100% |
| Class F (optional) | -4.43% | -6.19% | -8.61% | 98% | 101% | -4.48% | -7.50% | -10.68% | 97% | 99% |
| Table 10. Test results of EO classification modification only in LDP(Test 2.3.4.a) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -8.88% | -8.43% | -10.04% | 98% | 103% | -1.35% | -5.13% | -6.56% | 98% | 97% |
| Class C | -4.82% | -6.14% | -8.85% | 98% | 102% | -1.23% | -5.10% | -6.93% | 99% | 99% |
| Class E | -5.65% | -6.71% | -8.15% | 98% | 98% | -1.20% | -3.75% | -3.50% | 100% | 100% |
| Overall | -6.72% | -7.23% | -9.17% | 98% | 102% | -1.27% | -4.77% | -5.92% | 99% | 98% |
| Class D | -2.80% | -3.22% | -3.80% | 99% | 98% | -0.77% | -3.41% | -3.08% | 100% | 100% |
| Class F (optional) | -5.34% | -7.62% | -9.10% | 98% | 100% | -4.95% | -7.63% | -9.10% | 99% | 99% |

Results of BO offset quantization (Test 2.3.4.b)

The coding results of BO offset quantization are given in the following tables

Table 11. Test results of BO offset quantization only in AI (Test 2.3.4.b)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.78% | -0.83% | 100% | 107% | -0.06% | -1.45% | -1.61% | 100% | 105% |
| Class A2 | -0.67% | -1.19% | -0.98% | 102% | 111% | -0.26% | -0.75% | -0.70% | 100% | 105% |
| Class B | -0.47% | -1.00% | -1.67% | 100% | 106% | -0.15% | -0.94% | -1.77% | 100% | 107% |
| Class C | -0.48% | -1.52% | -2.41% | 99% | 101% | -0.19% | -1.54% | -2.09% | 99% | 102% |
| Class E | -0.49% | -1.69% | -2.16% | 100% | 103% | -0.21% | -1.32% | -1.65% | 104% | 110% |
| Overall | -0.48% | -1.23% | -1.66% | 100% | 105% | -0.17% | -1.19% | -1.62% | 100% | 106% |
| Class D | -0.38% | -1.08% | -1.49% | 109% | 98% | -0.14% | -0.74% | -1.20% | 100% | 101% |
| Class F (optional) | -1.22% | -1.80% | -2.76% | 99% | 99% | -1.10% | -2.78% | -3.49% | 99% | 104% |
| Table 12. Test results of BO offset quantization only in RA (Test 2.3.4.b) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.05% | -0.84% | -0.82% | 99% | 103% | -0.27% | -0.89% | -0.81% | 101% | 106% |
| Class A2 | -1.87% | -2.06% | -1.52% | 99% | 104% | -0.95% | -1.96% | -1.71% | 101% | 101% |
| Class B | -1.75% | -1.78% | -1.27% | 98% | 101% | -0.72% | -1.51% | -1.62% | 99% | 101% |
| Class C | -0.64% | -1.33% | -2.21% | 98% | 95% | -0.34% | -1.53% | -1.88% | 98% | 97% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.34% | -1.53% | -1.48% | 99% | 100% | -0.58% | -1.48% | -1.55% | 100% | 101% |
| Class D | -0.24% | -0.51% | -0.39% | 100% | 96% | -0.43% | -0.95% | -0.89% | 99% | 97% |
| Class F (optional) | -1.89% | -1.89% | -3.19% | 98% | 98% | -1.56% | -2.31% | -2.89% | 99% | 100% |
| Table 13. Test results of BO offset quantization only in LDB (Test 2.3.4.b) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.39% | -4.32% | -5.22% | 102% | 116% | -1.13% | -4.23% | -5.79% | 101% | 101% |
| Class C | -0.86% | -4.31% | -7.03% | 99% | 101% | -1.11% | -4.86% | -6.98% | 100% | 111% |
| Class E | -1.20% | -2.87% | -3.47% | 100% | 104% | -1.42% | -2.34% | -3.52% | 100% | 110% |
| Overall | -1.16% | -3.95% | -5.39% | 101% | 108% | -1.20% | -3.96% | -5.62% | 100% | 107% |
| Class D | -0.41% | -2.42% | -2.40% | 99% | 99% | -0.68% | -3.30% | -3.05% | 101% | 101% |
| Class F (optional) | -4.83% | -5.78% | -8.64% | 99% | 104% | -4.67% | -5.79% | -10.17% | 98% | 108% |
| Table 14. Test results of BO offset quantization only in LDP (Test 2.3.4.b) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -8.27% | -8.53% | -9.81% | 100% | 113% | -1.36% | -5.53% | -6.99% | 101% | 103% |
| Class C | -4.66% | -6.80% | -9.51% | 100% | 104% | -1.23% | -5.57% | -7.66% | 100% | 106% |
| Class E | -5.50% | -6.65% | -7.57% | 99% | 102% | -1.34% | -4.51% | -4.27% | 99% | 102% |
| Overall | -6.37% | -7.49% | -9.15% | 100% | 107% | -1.31% | -5.29% | -6.53% | 100% | 104% |
| Class D | -2.82% | -3.74% | -4.65% | 99% | 100% | -0.84% | -3.78% | -4.02% | 99% | 101% |
| Class F (optional) | -5.61% | -6.67% | -8.61% | 99% | 104% | -5.09% | -7.04% | -8.48% | 99% | 107% |

Results of BO fixed offset band (Test 2.3.4.c)

The coding results of BO fixed offset band are given in the following tables.

Table 15. Test results of BO offset band modification only in AI (Test 2.3.4.c)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.67% | -0.67% | 101% | 111% | -0.06% | -1.12% | -1.41% | 101% | 100% |
| Class A2 | -0.69% | -0.86% | -0.76% | 101% | 113% | -0.28% | -0.39% | -0.47% | 101% | 101% |
| Class B | -0.46% | -0.62% | -1.00% | 101% | 105% | -0.15% | -0.47% | -1.03% | 102% | 105% |
| Class C | -0.52% | -0.81% | -1.18% | 100% | 101% | -0.25% | -0.75% | -0.93% | 100% | 109% |
| Class E | -0.54% | -0.96% | -1.36% | 101% | 104% | -0.27% | -0.32% | -0.52% | 102% | 119% |
| Overall | -0.50% | -0.76% | -1.00% | 101% | 106% | -0.20% | -0.60% | -0.89% | 101% | 106% |
| Class D | -0.35% | -0.83% | -1.15% | 100% | 99% | -0.10% | -0.41% | -0.67% | 101% | 101% |
| Class F (optional) | -0.96% | -0.63% | -0.62% | 100% | 101% | -1.09% | -1.13% | -1.20% | 100% | 110% |
| Table 16. Test results of BO offset band modification only in RA (Test 2.3.4.c) |  |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.99% | -0.65% | -0.67% | 101% | 113% | -0.20% | -0.75% | -0.57% | 102% | 102% |
| Class A2 | -1.68% | -1.34% | -0.76% | 101% | 115% | -0.89% | -1.71% | -1.38% | 102% | 104% |
| Class B | -1.47% | -1.37% | -0.79% | 99% | 101% | -0.61% | -1.16% | -1.23% | 99% | 104% |
| Class C | -0.54% | -0.52% | -0.66% | 98% | 96% | -0.34% | -1.03% | -1.09% | 99% | 97% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.17% | -0.99% | -0.73% | 100% | 105% | -0.51% | -1.16% | -1.09% | 100% | 102% |
| Class D | -0.06% | -0.19% | -0.04% | 102% | 96% | -0.30% | -0.65% | -0.64% | 98% | 97% |
| Class F (optional) | -1.20% | -0.81% | -0.58% | 98% | 98% | -1.36% | -1.20% | -1.13% | 98% | 100% |
| Table 17. Test results of BO offset band modification only in LDB (Test 2.3.4.c) |  |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.84% | -3.41% | -3.63% | 103% | 117% | -0.92% | -3.15% | -4.01% | 100% | 102% |
| Class C | -0.45% | -1.77% | -2.80% | 99% | 97% | -0.95% | -3.18% | -4.13% | 100% | 117% |
| Class E | -0.54% | -1.79% | -2.71% | 99% | 100% | -1.23% | -1.44% | -2.94% | 100% | 109% |
| Overall | -0.63% | -2.46% | -3.12% | 101% | 106% | -1.01% | -2.73% | -3.78% | 100% | 108% |
| Class D | -0.23% | -1.64% | -1.27% | 99% | 97% | -0.64% | -2.59% | -2.10% | 99% | 99% |
| Class F (optional) | -2.04% | -2.43% | -2.40% | 99% | 105% | -3.79% | -2.06% | -5.61% | 99% | 114% |
| Table 18. Test results of BO offset band modification only in LDP (Test 2.3.4.c) |  |  |  |  |  |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -8.03% | -7.67% | -8.44% | 100% | 115% | -1.14% | -4.29% | -5.66% | 100% | 100% |
| Class C | -4.37% | -4.49% | -5.70% | 100% | 102% | -1.14% | -4.47% | -5.10% | 100% | 112% |
| Class E | -5.11% | -6.08% | -7.20% | 99% | 102% | -1.34% | -2.64% | -2.84% | 103% | 112% |
| Overall | -6.08% | -6.21% | -7.22% | 100% | 107% | -1.19% | -3.94% | -4.77% | 101% | 107% |
| Class D | -2.63% | -3.06% | -2.98% | 98% | 97% | -0.78% | -2.77% | -3.11% | 98% | 98% |
| Class F (optional) | -3.08% | -4.28% | -2.22% | 100% | 106% | -4.11% | -4.56% | -3.31% | 101% | 111% |

Results of shifted structure (Test 2.3.4.d)

The coding results of shifted structure is shown in the following tables.

Table 3. Test results of shifted structure only in AI (Test 2.3.4.d)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0, SAO =0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -1.07% | -1.12% | 100% | 106% | -0.12% | -2.32% | -2.08% | 99% | 98% |
| Class A2 | -0.64% | -1.74% | -1.18% | 102% | 108% | -0.22% | -1.35% | -0.88% | 99% | 96% |
| Class B | -0.41% | -1.69% | -2.72% | 100% | 104% | -0.09% | -1.82% | -2.99% | 99% | 98% |
| Class C | -0.41% | -1.84% | -2.79% | 100% | 101% | -0.12% | -2.02% | -2.62% | 99% | 101% |
| Class E | -0.43% | -2.31% | -3.21% | 100% | 102% | -0.11% | -2.19% | -3.10% | 100% | 102% |
| Overall | -0.44% | -1.74% | -2.30% | 100% | 104% | -0.13% | -1.93% | -2.42% | 99% | 99% |
| Class D | -0.34% | -1.33% | -1.67% | 99% | 99% | -0.08% | -1.00% | -1.44% | 101% | 101% |
| Class F (optional) | -1.11% | -2.32% | -3.31% | 100% | 99% | -1.08% | -3.89% | -4.96% | 99% | 100% |
| Table 4. Test results of shifted structure only in RA (Test 2.3.4.d) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.11% | -1.08% | -1.27% | 99% | 100% | -0.40% | -1.39% | -1.29% | 98% | 97% |
| Class A2 | -1.87% | -2.70% | -1.75% | 99% | 100% | -0.93% | -2.75% | -2.12% | 98% | 96% |
| Class B | -1.68% | -2.64% | -2.44% | 98% | 97% | -0.68% | -2.45% | -2.90% | 99% | 98% |
| Class C | -0.59% | -1.71% | -2.60% | 98% | 96% | -0.33% | -1.84% | -2.22% | 98% | 95% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.31% | -2.09% | -2.11% | 98% | 98% | -0.58% | -2.14% | -2.24% | 98% | 97% |
| Class D | -0.13% | -0.90% | -0.75% | 99% | 96% | -0.24% | -1.07% | -1.37% | 99% | 97% |
| Class F (optional) | -1.73% | -2.31% | -3.50% | 98% | 97% | -1.56% | -3.51% | -4.64% | 99% | 99% |
| Table 5. Test results of shifted structure only in LDB (Test 2.3.4.d) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO =0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.35% | -5.19% | -6.39% | 98% | 98% | -1.13% | -4.87% | -6.34% | 98% | 98% |
| Class C | -0.83% | -4.00% | -7.14% | 99% | 99% | -1.07% | -5.03% | -6.88% | 99% | 101% |
| Class E | -1.05% | -2.76% | -4.62% | 99% | 100% | -1.35% | -2.61% | -4.48% | 100% | 103% |
| Overall | -1.10% | -4.19% | -6.20% | 99% | 99% | -1.16% | -4.36% | -6.05% | 99% | 101% |
| Class D | -0.32% | -2.81% | -2.51% | 99% | 97% | -0.57% | -2.75% | -3.01% | 99% | 100% |
| Class F (optional) | -4.48% | -6.18% | -9.44% | 98% | 101% | -4.38% | -7.38% | -10.45% | 98% | 103% |
| Table 6. Test results of shifted structure only in LDP (Test 2.3.4.d) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -8.29% | -9.25% | -10.89% | 97% | 103% | -1.34% | -6.30% | -7.85% | 97% | 98% |
| Class C | -4.60% | -6.76% | -9.38% | 98% | 102% | -1.17% | -5.80% | -7.72% | 99% | 100% |
| Class E | -5.59% | -6.96% | -8.97% | 98% | 101% | -1.34% | -4.04% | -5.60% | 99% | 100% |
| Overall | -6.39% | -7.85% | -9.91% | 98% | 102% | -1.29% | -5.57% | -7.24% | 98% | 99% |
| Class D | -2.71% | -3.73% | -4.45% | 98% | 98% | -0.73% | -3.97% | -4.22% | 99% | 98% |
| Class F (optional) | -5.11% | -7.65% | -9.07% | 99% | 100% | -5.02% | -8.19% | -9.44% | 99% | 102% |

Results of offset range extension (Test 2.3.4.e)

The coding results of offset range extension are given in the following tables.

Table 19. Test results of range modification only in AI (Test 2.3.4.e)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.96% | -1.00% | 101% | 109% | -0.12% | -2.17% | -1.98% | 99% | 96% |
| Class A2 | -0.63% | -1.62% | -1.09% | 101% | 110% | -0.23% | -1.21% | -0.80% | 99% | 98% |
| Class B | -0.40% | -1.58% | -2.57% | 101% | 109% | -0.09% | -1.71% | -2.85% | 100% | 98% |
| Class C | -0.42% | -1.76% | -2.63% | 100% | 104% | -0.12% | -1.95% | -2.52% | 100% | 105% |
| Class E | -0.42% | -2.14% | -2.99% | 101% | 109% | -0.12% | -2.03% | -2.88% | 101% | 106% |
| Overall | -0.44% | -1.62% | -2.14% | 101% | 108% | -0.13% | -1.81% | -2.30% | 100% | 101% |
| Class D | -0.31% | -1.29% | -1.67% | 100% | 104% | -0.06% | -0.98% | -1.44% | 100% | 105% |
| Class F (optional) | -0.95% | -2.23% | -3.05% | 100% | 103% | -0.95% | -3.80% | -4.76% | 100% | 104% |
| Table 20. Test results of range modification only in RA (Test 2.3.4.e) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.08% | -0.97% | -1.10% | 103% | 104% | -0.38% | -1.22% | -1.28% | 100% | 99% |
| Class A2 | -1.84% | -2.50% | -1.58% | 100% | 103% | -0.93% | -2.52% | -1.82% | 99% | 97% |
| Class B | -1.67% | -2.48% | -2.34% | 99% | 97% | -0.67% | -2.31% | -2.78% | 99% | 100% |
| Class C | -0.60% | -1.58% | -2.36% | 99% | 96% | -0.34% | -1.74% | -2.09% | 98% | 96% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.30% | -1.94% | -1.94% | 100% | 99% | -0.58% | -1.98% | -2.11% | 99% | 98% |
| Class D | -0.13% | -1.04% | -0.78% | 100% | 96% | -0.22% | -1.08% | -1.17% | 98% | 97% |
| Class F (optional) | -1.52% | -2.23% | -3.19% | 99% | 98% | -1.47% | -3.32% | -4.40% | 98% | 99% |
| Table 21. Test results of range modification only in LDB (Test 2.3.4.e) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.30% | -4.90% | -6.10% | 101% | 102% | -1.11% | -4.72% | -6.12% | 98% | 99% |
| Class C | -0.83% | -4.04% | -6.70% | 101% | 109% | -1.05% | -4.50% | -6.45% | 100% | 108% |
| Class E | -1.13% | -2.29% | -4.02% | 101% | 108% | -1.46% | -1.97% | -4.45% | 101% | 108% |
| Overall | -1.10% | -3.96% | -5.78% | 101% | 106% | -1.18% | -3.96% | -5.81% | 99% | 104% |
| Class D | -0.30% | -2.57% | -2.18% | 101% | 106% | -0.63% | -3.26% | -3.10% | 101% | 107% |
| Class F (optional) | -3.92% | -6.03% | -8.46% | 101% | 110% | -4.19% | -7.10% | -10.87% | 99% | 106% |
| Table 22. Test results of range modification only in LDP (Test 2.3.4.e) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -8.21% | -9.03% | -10.72% | 101% | 109% | -1.34% | -5.74% | -7.52% | 98% | 98% |
| Class C | -4.58% | -6.47% | -9.23% | 101% | 111% | -1.20% | -5.50% | -7.09% | 100% | 108% |
| Class E | -5.54% | -7.00% | -8.13% | 101% | 109% | -1.32% | -4.16% | -4.78% | 100% | 106% |
| Overall | -6.33% | -7.67% | -9.58% | 101% | 110% | -1.29% | -5.26% | -6.69% | 99% | 103% |
| Class D | -2.64% | -3.83% | -4.28% | 100% | 107% | -0.67% | -4.08% | -3.89% | 101% | 107% |
| Class F (optional) | -4.62% | -7.38% | -9.05% | 101% | 109% | -4.71% | -8.02% | -9.47% | 100% | 106% |

Results of all modifications (Test 2.3.4.f)

The coding results of all the modification proposed in the contribution are given in the following tables.

Table 23. Test results of all modifications in AI (Test 2.3.4.f)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.42% | -0.85% | -0.96% | 101% | 108% | -0.07% | -0.71% | -1.65% | 100% | 101% |
| Class A2 | -0.81% | -1.15% | -0.97% | 100% | 107% | -0.31% | -0.39% | -0.56% | 103% | 101% |
| Class B | -0.53% | -0.71% | -1.15% | 100% | 105% | -0.16% | -0.35% | -0.94% | 100% | 98% |
| Class C | -0.60% | -0.91% | -1.32% | 100% | 100% | -0.27% | -0.68% | -0.83% | 100% | 101% |
| Class E | -0.61% | -1.51% | -2.02% | 101% | 103% | -0.26% | -0.17% | -0.17% | 101% | 103% |
| Overall | -0.59% | -0.98% | -1.27% | 101% | 104% | -0.21% | -0.46% | -0.85% | 101% | 100% |
| Class D | -0.40% | -0.95% | -1.32% | 100% | 98% | -0.12% | -0.38% | -0.68% | 101% | 101% |
| Class F (optional) | -1.07% | -0.73% | -0.67% | 100% | 100% | -1.08% | -1.09% | -1.09% | 100% | 101% |
| Table 24. Test results of all modifications in RA (Test 2.3.4.f) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.17% | -0.53% | -0.63% | 98% | 99% | -0.21% | -0.37% | -0.55% | 97% | 98% |
| Class A2 | -1.82% | -1.00% | -0.61% | 99% | 101% | -0.93% | -1.36% | -1.48% | 97% | 96% |
| Class B | -1.67% | -1.40% | -0.53% | 98% | 97% | -0.70% | -1.19% | -0.99% | 98% | 100% |
| Class C | -0.59% | -0.47% | -0.58% | 98% | 96% | -0.36% | -0.97% | -1.03% | 98% | 97% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.31% | -0.90% | -0.58% | 98% | 98% | -0.56% | -1.00% | -1.01% | 97% | 98% |
| Class D | -0.15% | -0.06% | 0.04% | 99% | 95% | -0.43% | -0.62% | -0.59% | 98% | 97% |
| Class F (optional) | -1.27% | -0.81% | -0.51% | 98% | 97% | -1.40% | -1.23% | -1.08% | 98% | 99% |
| Table 25. Test results of all modifications in LDB (Test 2.3.4.f) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.93% | -2.18% | -2.16% | 98% | 98% | -0.98% | -2.16% | -2.69% | 96% | 99% |
| Class C | -0.48% | -1.34% | -2.52% | 99% | 100% | -1.03% | -3.44% | -4.07% | 98% | 102% |
| Class E | -0.46% | -1.79% | -2.35% | 99% | 99% | -1.02% | -0.77% | -0.93% | 98% | 101% |
| Overall | -0.66% | -1.80% | -2.33% | 99% | 99% | -1.01% | -2.24% | -2.71% | 97% | 100% |
| Class D | -0.30% | -1.13% | -1.01% | 99% | 98% | -0.75% | -2.22% | -1.89% | 99% | 101% |
| Class F (optional) | -2.35% | -1.79% | -2.46% | 99% | 101% | -3.85% | -2.02% | -6.23% | 96% | 101% |
| Table 26. Test results of all modifications in LDP (Test 2.3.4.f) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -8.77% | -6.78% | -7.29% | 98% | 104% | -1.21% | -3.38% | -3.85% | 96% | 98% |
| Class C | -4.59% | -4.08% | -5.04% | 99% | 102% | -1.18% | -4.27% | -5.08% | 98% | 101% |
| Class E | -5.33% | -6.39% | -7.18% | 99% | 100% | -0.95% | -1.83% | -2.11% | 98% | 101% |
| Overall | -6.52% | -5.79% | -6.51% | 98% | 102% | -1.14% | -3.29% | -3.82% | 97% | 100% |
| Class D | -2.96% | -2.59% | -3.61% | 98% | 98% | -0.89% | -3.07% | -2.79% | 99% | 99% |
| Class F (optional) | -3.37% | -3.57% | 1.18% | 99% | 100% | -4.28% | -4.16% | -3.66% | 97% | 101% |

Results of EO classification + BO offset quantization + shifted structure (Test 2.3.4.g)

The coding results of all the modification proposed in the contribution are given in the following tables.

Table 23. Test results of all modifications in AI (Test 2.3.4.g)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.42% | -0.94% | -1.13% | 101% | 109% | -0.07% | -0.96% | -1.88% | 100% | 98% |
| Class A2 | -0.79% | -1.48% | -1.18% | 101% | 110% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.53% | -1.10% | -1.82% | 101% | 106% | -0.16% | -0.85% | -1.73% | 100% | 96% |
| Class C | -0.54% | -1.64% | -2.58% | 100% | 101% | -0.19% | -1.52% | -2.06% | 100% | 101% |
| Class E | -0.56% | -2.25% | -2.79% | 101% | 104% | -0.19% | -1.24% | -1.34% | 101% | 103% |
| Overall | -0.56% | -1.45% | -1.93% | 101% | 106% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.40% | -1.18% | -1.63% | 102% | 99% | -0.12% | -0.74% | -1.26% | 101% | 100% |
| Class F (optional) | -1.33% | -1.90% | -2.85% | 100% | 101% | -1.19% | -2.78% | -3.46% | 100% | 101% |
| Table 24. Test results of all modifications in RA (Test 2.3.4.g) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.23% | -0.67% | -0.83% | 100% | 104% | -0.29% | -0.39% | -0.75% | 97% | 98% |
| Class A2 | -2.03% | -1.89% | -1.43% | 100% | 105% | -1.01% | -1.86% | -1.84% | 97% | 97% |
| Class B | -1.89% | -1.86% | -1.02% | 99% | 103% | -0.74% | -1.60% | -1.45% | 98% | 101% |
| Class C | -0.69% | -1.38% | -2.21% | 98% | 98% | -0.37% | -1.58% | -1.77% | 98% | 97% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.46% | -1.50% | -1.38% | 99% | 102% | -0.60% | -1.41% | -1.48% | 98% | 98% |
| Class D | -0.25% | -0.26% | -0.34% | 105% | 96% | -0.41% | -0.69% | -0.88% | 98% | 97% |
| Class F (optional) | -2.01% | -1.97% | -3.18% | 98% | 99% | -1.70% | -2.33% | -2.88% | 98% | 100% |
| Table 25. Test results of all modifications in LDB (Test 2.3.4.g) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.44% | -3.48% | -3.80% | 99% | 100% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.89% | -3.93% | -6.83% | 99% | 102% | -1.08% | -4.54% | -6.80% | 98% | 104% |
| Class E | -1.12% | -2.83% | -3.36% | 100% | 104% | -1.31% | -2.38% | -2.64% | 99% | 104% |
| Overall | -1.18% | -3.47% | -4.70% | 99% | 102% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.36% | -2.25% | -1.87% | 99% | 99% | -0.68% | -2.73% | -3.07% | 99% | 101% |
| Class F (optional) | -4.93% | -5.26% | -8.05% | 99% | 104% | -4.95% | -5.90% | -8.94% | 96% | 100% |
| Table 26. Test results of all modifications in LDP (Test 2.3.4.f) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0, SAO = 0 |  |  |  |  | Over BMS-1.0, SAO = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -8.77% | -6.78% | -7.29% | 98% | 104% | -1.21% | -3.38% | -3.85% | 96% | 98% |
| Class C | -4.59% | -4.08% | -5.04% | 99% | 102% | -1.18% | -4.27% | -5.08% | 98% | 101% |
| Class E | -5.33% | -6.39% | -7.18% | 99% | 100% | -0.95% | -1.83% | -2.11% | 98% | 101% |
| Overall | -6.52% | -5.79% | -6.51% | 98% | 102% | -1.14% | -3.29% | -3.82% | 97% | 100% |
| Class D | -2.96% | -2.59% | -3.61% | 98% | 98% | -0.89% | -3.07% | -2.79% | 99% | 99% |
| Class F (optional) | -3.37% | -3.57% | 1.18% | 99% | 100% | -4.28% | -4.16% | -3.66% | 97% | 101% |

Conclusion

SAO in VTM1.0 is modified in this contribution. To simplify the hardware implementation of SAO the shifted structure is proposed. The experimental results show than the shifted structure doesn’t affect the coding performance and running time. Besides, the EO classification, BO offset quantization, fixed BO offset band and offset range extension are proposed as well. Considering the coding performance and the complexity, it is suggested to adopt shifted structure, EO classification modification and BO offset quantization in VVC.

Reference

- JVET, “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario”, JVET-J0024, 10th JVET Meeting, San Diego, US, 10-20 Apr., 2018

- JVET, “Description of Core Experiment 2 (CE2): In-Loop filters”, JVET-J1022, 10th JVET Meeting, San Diego, US, 10-20 Apr., 2018

- JVET, “JVET common test conditions and software reference configurations”, JVET-J1010, 10th JVET Meeting, San Diego, US, 10-20 Apr., 2018

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE2/CE2-3.4/

Patent rights declaration(s)

Samsung Electronics Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

.