| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0183 |
| --- | --- |

| Title: | AHG8: Face boundary filtering for 360° video |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuchang Huangfu Yule Sun Bin Wang Lu Yu | Tel: Email: | +86 57187953107 huangfuxc@zju.edu.cn sunyule@zju.edu.cn 3130100819@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

- Abstract

This contribution proposes a face boundary filtering method for 360 video to reduce the seam artifacts in reconstructed viewports. Blocks which are adjacent on the sphere but nonadjacent in the frame-packed image are filtered. The subjective quality of reconstructed viewports has been improved significantly after filtering. The performance of in-loop filtering and post-filtering is similar with each other objectively and subjectively.

Introduction

The 360° video is actually a spherical video. Adjacent blocks on the sphere may be separated while projecting from the sphere to a projection format. As shown in Figure 1, block A and block B are adjacent on the sphere but nonadjacent in the image with cubemap-like frame-packed projection format. On the contrary, block C and D are packed adjacent whereas they are nonadjacent on the sphere. The adjacent block of block D on the sphere is block E. Cubemap-like frame packing method could result in discontinuous face boundaries. As shown in Figure 2, seam artifacts appear at these discontinuous face boundaries after coding. Padding pixels at discontinuous face boundaries in the frame-packed image before coding is recommended in [1] to reduce seam artifacts. However redundant pixels will affect coding efficiency. Additionally, as reported in [2], padding cannot resolve the seam artifacts for blocks which are nonadjacent in the frame-packed image but adjacent on the sphere. Therefore, we propose a face boundary filtering method for those blocks.

Figure 1. Adjacent blocks on the sphere：A and B, D and E; adjacent blocks on coding projection format: C and D

Figure 2. Seam artifacts in the reconstructed viewport of ChairLift (QP=37, frame 46) in degree (-135, 35)

Face boundary filter

We propose a face boundary filtering method to handle the seam artifacts caused by those nonadjacent blocks in the frame-packed image. The proposed filter imitates deblocking filter in HEVC. Figure 3 illustrates the regions processed by the proposed method in frame-packed image. In Figure 3 pixels in the same numbered regions are filtered to reduce the seam artifacts and pixels in the green rectangle are padded to avoid improper deblocking. For instance, when a block in region 1 (red color) is filtered its spherical adjacent block in region 1 (blue color) is fetched. These nonadjacent blocks can be processed with in-loop filtering or post filtering. When blocks are processed with in-loop filtering the boundary strength (BS) is determined by prediction mode of blocks. When the proposed method is used in post processing, the BS is set a fixed strength of 1 and the QP in slice level is used to determine the parameters τc and β. Considering the different sampling rates of PAU projection format [3] in horizontal and vertical directions, an alignment between two boundary blocks could be conducted.

Figure 3. Regions processed by proposed filtering method

Experiment Results

The proposed filtering method is implemented into VTM and 360Lib-6.0 for investigation. PAU is selected as encoding projection format and encoded using the Random Acess configuration. The QP set {22, 27, 32, 37}are used. Both conditions are tested when nonadjacent blocks in the frame-packed image are processed with in-loop filtering and post filtering. The anchor is set with the same projection format without filtering process. Table 1 reports the BD-rate for PAU with filtering. In-loop filtering and post filtering can provide an average BD-rate savings of 0.03% and 0.02% for Y component respectively. As shown in Figure 4 and Figure 5, the proposed method can significantly reduce the seam artifacts and the subjective quality of reconstructed viewports after in-loop filtering and post filtering is similar.

Table 1.Comparison of PAU with and without in-loop filtering or post filtering

| Sequence | E2E WS-PSNR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | In-loop filtering |  |  | Post filtering |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| SkateboardInLot | -0.05% | 0.11% | -0.05% | -0.05% | 0.00% | 0.00% |
| ChairLift | -0.05% | 0.03% | -0.13% | -0.06% | 0.00% | 0.00% |
| KiteFlite | 0.02% | -0.02% | -0.03% | 0.00% | 0.00% | 0.00% |
| Harbor | 0.04% | -0.02% | -0.06% | 0.01% | 0.00% | 0.00% |
| Trolley | 0.02% | 0.01% | 0.07% | 0.01% | 0.00% | 0.00% |
| GasLamp | 0.06% | 0.04% | 0.01% | 0.03% | 0.00% | 0.00% |
| Balboa | -0.11% | 0.02% | -0.34% | -0.05% | 0.00% | 0.00% |
| Broadway | -0.09% | -0.20% | 0.04% | -0.05% | 0.00% | 0.00% |
| Landing2 | -0.07% | -0.18% | -0.03% | -0.05% | 0.00% | 0.00% |
| BranCastle2 | -0.05% | 0.00% | -0.08% | -0.04% | 0.00% | 0.00% |
| Overall | -0.03% | -0.02% | -0.06% | -0.02% | 0.00% | 0.00% |

(a)

(b)

(c)

Figure 4. Reconstructed viewport of ChairLift (QP=37, frame 46) in degree (-135, 35) (a) without filtering (b) with in-loop filtering (c) with post filtering

(a)

(b)

(c)

Figure 5. Reconstructed viewport of Trolley (QP=37, frame 143) in degree (-135, 35) (a) without filtering (b) with in-loop filtering (c) with post filtering

Conclusion

A face boundary filtering method is proposed to resolve the seam artifacts in cubemap-like projection. Experiment finds that seam artifacts in the reconstructed viewport are reduced significantly by using the proposed filtering method. The subjective and objective performance of in-loop filtering and post filtering is similar. In order to keep the coding process of 360° video consistent with general video, we suggest using post filtering to resolve the seam artifacts.

Reference

- G. Van der Auwera, M. Coban, M. Karczewicz,“AHG8: ACP with padding for 360-degree video,”JVET-G0071, July 2017, Torino, IT.

- P. Hanhart, Y. He, Y. Ye, “AHG8: Geometry padding for PERP,” JVET-J0044, Apr. 2018, San Diego, USA.

- Y.Sun, X.Huangfu, R.Zheng, B.Wang. L.Yu, “Description of 360° video coding technology proposal by Zhejiang University,” JVET-J0044, Apr. 2018, San Diego, USA.

Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).