| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0185-v1 |
| --- | --- |

| Title: | CE4: Affine inter prediction (Test 1.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Maxim Sychev Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com Sychev.Maxim@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of integrating affine inter prediction in JVET-J0024 [1] to BMS1.0. In this contribution, affine inter prediction is improved by 4 aspects: 1) model based affine candidates are inserted into affine AMVP candidates with the same scan order of AMVP in HEVC; 2) affine MVD zero flag is signaled to indicate whether affine MVDs is zero or not; 3) additional 6-parameter affine model is added and different models are adaptively selected in CU level; 4) Enhanced bi-linear Interpolation Filter (EIF) is used for affine motion compensation, when the width or height of affine sub-block is less than 8.

For affine AMVP improvement, experimental results reportedly show on average 0.26%/0.06% luma BD-rate gain in RA/LB configurations over to VTM with affine on. For EIF, experimental results reportedly show on average 0.25%/0.41% luma BD-rate gain with 9%/6% decoding time reduction in RA/LB configurations over VTM with affine on.

Introduction

Affine inter prediction in BMS is improved by following aspects:

- model based affine candidates are inserted into affine AMVP candidates with the same scan order of AMVP in HEVC

- affine MVD zero flag is signaled to indicate whether affine MVDs is zero or not

- additional 6-parameter affine model is added and different models are adaptively selected in CU level

- Enhanced bi-linear Interpolation Filter (EIF) is used for affine motion compensation, when the width or height of affine sub-block is less than 8

Test specification

The tests for this contribution are descripted in Table 1.

Test a to test e are descripted tests in JVET-J1024 [4]. Additional tests f and g are conducted to enable EIF when the width or height of affine sub-block is less than 4.

Table 1 test specification for this contribution

| Test # | Test description |
| --- | --- |
| Test a | Affine MVP construction |
| Test b | Test a + affine MVD coding |
| Test c | Test b + 4-para/6-para switching |
| Test d | EIF on top of BMS affine (width or height of sub-block is less than 8) |
| Test e | EIF on top of Test c (width or height of sub-block is less than 8) |
| *Test f | EIF on top of BMS affine (width or height of sub-block is less than 4) |
| *Test g | EIF on top of Test c (width or height of sub-block is less than 4) |

Simulation results

The contribution has been implemented on top of BMS1.0 [3]. The test condition descripted in JVET-J1024 [4] is used for verifying the contribution. The results are summarized in Table 2 and Table 3. More detailed results are provided from Table 4 to Table 17.

For test a, the results reportedly show on average 0.26%/0.06% luma BD-rate gain in RA/LB configurations, compared to VTM with affine on, and 0.19%/0.09% luma BD-rate gain in RA/LB configurations, compared to BMS1.0.

For test d, the results reportedly show on average 0.25%/0.41% luma BD-rate gain with 9%/6% decoding time reduction in RA/LB configurations, compared to VTM with affine on.

Table 2 Summary of overall performance in RA/LB configuration (over VTM and BMS – AFFINE)

| Test a | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -3.24% | -2.41% | -2.47% | 138% | 110% | -2.10% | -1.54% | -1.53% | 110% | 101% |
| LB | -2.13% | -1.26% | -1.59% | 169% | 107% | -2.07% | -1.69% | -1.57% | 126% | 103% |
| Test b | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -3.23% | -2.41% | -2.43% | 139% | 110% | -2.10% | -1.52% | -1.53% | 110% | 101% |
| LB | -2.11% | -1.40% | -1.51% | 170% | 107% | -2.07% | -1.72% | -1.86% | 126% | 104% |
| Test c | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -3.83% | -2.86% | -2.89% | 144% | 111% | -2.65% | -1.91% | -1.94% | 111% | 102% |
| LB | -2.36% | -1.54% | -1.47% | 174% | 108% | -2.31% | -1.69% | -2.13% | 128% | 104% |
| Test d | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -3.23% | -2.41% | -2.52% | 132% | 102% | -1.85% | -1.42% | -1.44% | 109% | 99% |
| LB | -0.98% | -0.64% | -0.86% | 131% | 101% | -0.62% | -0.51% | -0.29% | 108% | 99% |
| Test e | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -4.20% | -3.08% | -3.30% | 137% | 103% | -2.67% | -2.00% | -2.07% | 109% | 99% |
| LB | -2.95% | -1.84% | -2.02% | 165% | 102% | -2.21% | -1.60% | -1.73% | 124% | 99% |
| *Test f | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -3.33% | -2.41% | -2.56% | 131% | 104% | -1.98% | -1.47% | -1.51% | 107% | 100% |
| LB | -2.47% | -1.67% | -1.73% | 159% | 103% | -2.13% | -1.66% | -1.77% | 122% | 100% |
| *Test g | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -4.32% | -3.14% | -3.32% | 137% | 104% | -2.81% | -2.04% | -2.10% | 109% | 99% |
| LB | -1.66% | -0.96% | -1.24% | 140% | 100% | -1.18% | -1.13% | -1.00% | 109% | 98% |

Table 3 Summary of overall performance in RA/LB configuration (over VTM + AFFINE and BMS)

| Test a | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -0.26% | -0.23% | -0.27% | 100% | 98% | -0.19% | -0.20% | -0.17% | 102% | 99% |
| LB | -0.06% | 0.07% | -0.07% | 101% | 100% | -0.09% | -0.16% | 0.12% | 101% | 99% |
| Test b | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -0.25% | -0.23% | -0.23% | 101% | 99% | -0.19% | -0.19% | -0.18% | 102% | 100% |
| LB | -0.05% | -0.07% | 0.01% | 101% | 100% | -0.09% | -0.20% | -0.17% | 101% | 100% |
| Test c | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -0.88% | -0.70% | -0.70% | 105% | 99% | -0.76% | -0.59% | -0.60% | 103% | 100% |
| LB | -0.30% | -0.21% | 0.06% | 104% | 100% | -0.34% | -0.16% | -0.44% | 103% | 100% |
| Test d | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -0.25% | -0.23% | -0.32% | 96% | 91% | 0.07% | -0.08% | -0.09% | 100% | 98% |
| LB | -0.41% | -0.27% | -0.42% | 94% | 94% | 0.10% | -0.05% | 0.05% | 98% | 97% |
| Test e | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -1.28% | -0.93% | -1.13% | 100% | 92% | -0.78% | -0.67% | -0.73% | 101% | 97% |
| LB | -0.90% | -0.51% | -0.50% | 98% | 95% | -0.23% | -0.07% | -0.04% | 100% | 96% |
| *Test f | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -0.36% | -0.23% | -0.36% | 95% | 93% | -0.07% | -0.13% | -0.16% | 99% | 98% |
| LB | -0.42% | -0.34% | -0.21% | 95% | 96% | -0.15% | -0.14% | -0.07% | 98% | 96% |
| *Test g | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -1.40% | -0.99% | -1.15% | 100% | 93% | -0.92% | -0.71% | -0.76% | 100% | 97% |
| LB | -0.89% | -0.64% | -0.53% | 98% | 96% | -0.48% | -0.39% | -0.34% | 99% | 96% |

Test a

Table 4 Coding performance of test a (over VTM and BMS - AFFINE)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.12% | -1.73% | -1.86% | 135% | 106% | -1.21% | -0.96% | -0.96% | 110% | 101% |
| Class A2 | -6.38% | -5.15% | -4.94% | 139% | 119% | -3.89% | -2.86% | -2.79% | 111% | 103% |
| Class B | -3.54% | -2.40% | -2.58% | 139% | 110% | -2.38% | -1.69% | -1.74% | 111% | 100% |
| Class C | -1.32% | -0.87% | -0.93% | 137% | 106% | -1.09% | -0.80% | -0.73% | 110% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.24% | -2.41% | -2.47% | 138% | 110% | -2.10% | -1.54% | -1.53% | 110% | 101% |
| Class D | -1.87% | -1.12% | -1.21% | 138% | 105% | -1.62% | -1.25% | -1.08% | 109% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.60% | -1.76% | -1.91% | 166% | 109% | -2.64% | -1.86% | -1.85% | 123% | 103% |
| Class C | -1.44% | -0.57% | -0.74% | 149% | 105% | -1.42% | -1.03% | -0.86% | 118% | 103% |
| Class E | -2.24% | -1.35% | -2.19% | 205% | 106% | -1.98% | -2.28% | -2.07% | 144% | 101% |
| Overall | -2.13% | -1.26% | -1.59% | 169% | 107% | -2.07% | -1.69% | -1.57% | 126% | 103% |
| Class D | -2.96% | -2.10% | -2.29% | 142% | 107% | -3.04% | -2.49% | -2.72% | 118% | 103% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 5 Coding performance of test a (over VTM + AFFINE and BMS)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.15% | -0.22% | 100% | 99% | -0.07% | -0.16% | -0.08% | 102% | 99% |
| Class A2 | -0.55% | -0.44% | -0.52% | 101% | 100% | -0.48% | -0.38% | -0.37% | 102% | 100% |
| Class B | -0.30% | -0.25% | -0.26% | 100% | 98% | -0.23% | -0.21% | -0.24% | 102% | 99% |
| Class C | -0.06% | -0.11% | -0.13% | 100% | 95% | -0.01% | -0.09% | -0.02% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.26% | -0.23% | -0.27% | 100% | 98% | -0.19% | -0.20% | -0.17% | 102% | 99% |
| Class D | -0.09% | -0.10% | 0.00% | 100% | 95% | -0.04% | 0.04% | 0.01% | 101% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.12% | -0.28% | -0.14% | 101% | 101% | -0.12% | -0.38% | 0.20% | 101% | 99% |
| Class C | 0.00% | 0.19% | -0.14% | 101% | 99% | -0.02% | 0.16% | 0.16% | 101% | 100% |
| Class E | -0.06% | 0.50% | 0.14% | 101% | 98% | -0.13% | -0.22% | -0.06% | 101% | 97% |
| Overall | -0.06% | 0.07% | -0.07% | 101% | 100% | -0.09% | -0.16% | 0.12% | 101% | 99% |
| Class D | -0.03% | -0.10% | -0.03% | 100% | 98% | -0.10% | 0.32% | 0.00% | 101% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test b

Table 6 Coding performance of test b (over VTM and BMS - AFFINE)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.13% | -1.75% | -1.86% | 137% | 107% | -1.21% | -0.96% | -0.96% | 109% | 101% |
| Class A2 | -6.41% | -5.26% | -4.99% | 140% | 119% | -3.89% | -2.91% | -2.86% | 110% | 104% |
| Class B | -3.51% | -2.39% | -2.56% | 141% | 110% | -2.36% | -1.69% | -1.72% | 111% | 101% |
| Class C | -1.31% | -0.79% | -0.79% | 138% | 107% | -1.09% | -0.70% | -0.75% | 110% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.23% | -2.41% | -2.43% | 139% | 110% | -2.10% | -1.52% | -1.53% | 110% | 101% |
| Class D | -1.83% | -1.10% | -1.20% | 138% | 105% | -1.55% | -1.25% | -1.20% | 109% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.60% | -1.91% | -2.00% | 167% | 110% | -2.64% | -1.69% | -1.92% | 122% | 104% |
| Class C | -1.44% | -0.55% | -0.59% | 150% | 105% | -1.38% | -1.18% | -1.14% | 118% | 104% |
| Class E | -2.19% | -1.67% | -1.93% | 208% | 107% | -2.04% | -2.48% | -2.73% | 145% | 103% |
| Overall | -2.11% | -1.40% | -1.51% | 170% | 107% | -2.07% | -1.72% | -1.86% | 126% | 104% |
| Class D | -2.89% | -1.65% | -1.97% | 142% | 105% | -2.89% | -2.19% | -3.11% | 117% | 103% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 7 Coding performance of test b (over VTM + AFFINE and BMS)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.18% | -0.21% | 102% | 100% | -0.07% | -0.16% | -0.07% | 102% | 100% |
| Class A2 | -0.58% | -0.55% | -0.57% | 101% | 100% | -0.49% | -0.44% | -0.44% | 101% | 100% |
| Class B | -0.27% | -0.23% | -0.23% | 101% | 99% | -0.21% | -0.21% | -0.21% | 102% | 99% |
| Class C | -0.04% | -0.03% | 0.02% | 101% | 97% | -0.01% | 0.02% | -0.04% | 101% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.25% | -0.23% | -0.23% | 101% | 99% | -0.19% | -0.19% | -0.18% | 102% | 100% |
| Class D | -0.04% | -0.08% | 0.01% | 100% | 94% | 0.03% | 0.03% | -0.12% | 100% | 97% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.11% | -0.44% | -0.24% | 101% | 101% | -0.12% | -0.22% | 0.14% | 101% | 100% |
| Class C | 0.00% | 0.22% | 0.01% | 101% | 99% | 0.02% | 0.00% | -0.12% | 101% | 100% |
| Class E | -0.01% | 0.17% | 0.41% | 102% | 99% | -0.18% | -0.43% | -0.74% | 101% | 99% |
| Overall | -0.05% | -0.07% | 0.01% | 101% | 100% | -0.09% | -0.20% | -0.17% | 101% | 100% |
| Class D | 0.06% | 0.38% | 0.34% | 101% | 96% | 0.06% | 0.61% | -0.45% | 100% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test c

Table 8 Coding performance of test c (over VTM and BMS - AFFINE)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.55% | -2.17% | -2.21% | 139% | 107% | -1.57% | -1.20% | -1.26% | 110% | 102% |
| Class A2 | -7.96% | -6.43% | -6.08% | 146% | 120% | -5.41% | -4.07% | -3.92% | 112% | 104% |
| Class B | -4.02% | -2.73% | -2.94% | 147% | 111% | -2.80% | -1.90% | -2.07% | 112% | 101% |
| Class C | -1.46% | -0.85% | -0.93% | 144% | 107% | -1.20% | -0.84% | -0.80% | 111% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.83% | -2.86% | -2.89% | 144% | 111% | -2.65% | -1.91% | -1.94% | 111% | 102% |
| Class D | -1.92% | -1.23% | -1.28% | 145% | 106% | -1.69% | -1.37% | -1.17% | 111% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.87% | -2.03% | -1.85% | 171% | 110% | -2.92% | -1.89% | -2.45% | 124% | 104% |
| Class C | -1.53% | -0.74% | -0.55% | 152% | 106% | -1.51% | -1.22% | -1.37% | 119% | 104% |
| Class E | -2.61% | -1.78% | -2.05% | 216% | 107% | -2.36% | -1.96% | -2.63% | 148% | 103% |
| Overall | -2.36% | -1.54% | -1.47% | 174% | 108% | -2.31% | -1.69% | -2.13% | 128% | 104% |
| Class D | -2.98% | -2.32% | -2.29% | 145% | 107% | -3.06% | -2.88% | -3.20% | 119% | 103% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 9 Coding performance of test c (over VTM + AFFINE and BMS)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.60% | -0.61% | -0.57% | 103% | 100% | -0.44% | -0.41% | -0.38% | 102% | 100% |
| Class A2 | -2.24% | -1.78% | -1.72% | 106% | 101% | -2.07% | -1.63% | -1.53% | 103% | 100% |
| Class B | -0.79% | -0.58% | -0.62% | 106% | 99% | -0.66% | -0.43% | -0.57% | 103% | 100% |
| Class C | -0.20% | -0.09% | -0.12% | 105% | 96% | -0.13% | -0.13% | -0.09% | 103% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.88% | -0.70% | -0.70% | 105% | 99% | -0.76% | -0.59% | -0.60% | 103% | 100% |
| Class D | -0.14% | -0.21% | -0.07% | 106% | 95% | -0.11% | -0.09% | -0.09% | 102% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.39% | -0.56% | -0.08% | 104% | 102% | -0.41% | -0.43% | -0.40% | 102% | 100% |
| Class C | -0.09% | 0.03% | 0.05% | 103% | 99% | -0.11% | -0.04% | -0.34% | 102% | 100% |
| Class E | -0.44% | 0.06% | 0.29% | 106% | 99% | -0.51% | 0.12% | -0.64% | 104% | 99% |
| Overall | -0.30% | -0.21% | 0.06% | 104% | 100% | -0.34% | -0.16% | -0.44% | 103% | 100% |
| Class D | -0.04% | -0.34% | -0.03% | 102% | 97% | -0.11% | -0.13% | -0.57% | 101% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test d

Table 10 Coding performance of test d (over VTM and BMS - AFFINE)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.17% | -1.54% | -1.58% | 130% | 101% | -1.13% | -0.83% | -0.80% | 108% | 100% |
| Class A2 | -6.55% | -5.07% | -5.13% | 132% | 105% | -3.63% | -2.55% | -2.55% | 109% | 100% |
| Class B | -3.62% | -2.63% | -2.91% | 133% | 102% | -2.22% | -1.84% | -1.91% | 109% | 98% |
| Class C | -1.05% | -0.80% | -0.77% | 131% | 102% | -0.59% | -0.48% | -0.50% | 108% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.23% | -2.41% | -2.52% | 132% | 102% | -1.85% | -1.42% | -1.44% | 109% | 99% |
| Class D | -0.98% | -0.64% | -0.86% | 131% | 101% | -0.62% | -0.51% | -0.29% | 108% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.82% | -2.10% | -2.29% | 156% | 103% | -2.49% | -1.88% | -2.22% | 119% | 100% |
| Class C | -1.75% | -1.70% | -2.04% | 142% | 101% | -1.14% | -1.19% | -1.13% | 115% | 101% |
| Class E | -2.85% | -0.68% | -1.20% | 188% | 100% | -1.90% | -1.58% | -1.37% | 138% | 99% |
| Overall | -2.47% | -1.61% | -1.94% | 158% | 102% | -1.89% | -1.58% | -1.64% | 122% | 100% |
| Class D | -2.52% | -1.88% | -1.92% | 135% | 102% | -1.78% | -1.54% | -2.47% | 115% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 11 Coding performance of test d (over VTM + AFFINE and BMS)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | 0.05% | 0.07% | 97% | 95% | 0.01% | -0.03% | 0.09% | 100% | 98% |
| Class A2 | -0.73% | -0.34% | -0.72% | 96% | 88% | -0.21% | -0.06% | -0.13% | 100% | 97% |
| Class B | -0.38% | -0.48% | -0.60% | 96% | 91% | -0.07% | -0.38% | -0.41% | 101% | 97% |
| Class C | 0.23% | -0.03% | 0.04% | 96% | 92% | 0.50% | 0.23% | 0.22% | 100% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.25% | -0.23% | -0.32% | 96% | 91% | 0.07% | -0.08% | -0.09% | 100% | 98% |
| Class D | 0.85% | 0.40% | 0.37% | 96% | 91% | 1.00% | 0.80% | 0.81% | 99% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.34% | -0.62% | -0.53% | 95% | 95% | 0.04% | -0.40% | -0.17% | 98% | 96% |
| Class C | -0.31% | -0.93% | -1.45% | 95% | 95% | 0.27% | -0.01% | -0.10% | 99% | 97% |
| Class E | -0.68% | 1.18% | 1.15% | 92% | 93% | -0.04% | 0.48% | 0.63% | 97% | 95% |
| Overall | -0.41% | -0.27% | -0.42% | 94% | 94% | 0.10% | -0.05% | 0.05% | 98% | 97% |
| Class D | 0.46% | 0.15% | 0.45% | 96% | 93% | 1.30% | 1.33% | 0.26% | 98% | 96% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test e

Table 12 Coding performance of test e (over VTM and BMS - AFFINE)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.83% | -2.00% | -2.17% | 133% | 101% | -1.62% | -1.20% | -1.25% | 108% | 99% |
| Class A2 | -9.05% | -6.73% | -6.87% | 138% | 105% | -5.85% | -4.04% | -3.96% | 109% | 99% |
| Class B | -4.50% | -3.26% | -3.72% | 140% | 103% | -2.93% | -2.33% | -2.54% | 110% | 98% |
| Class C | -1.23% | -0.92% | -0.92% | 138% | 102% | -0.75% | -0.64% | -0.69% | 109% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -4.20% | -3.08% | -3.30% | 137% | 103% | -2.67% | -2.00% | -2.07% | 109% | 99% |
| Class D | -1.13% | -0.68% | -0.92% | 139% | 101% | -0.70% | -0.78% | -0.53% | 109% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -3.38% | -2.54% | -2.61% | 162% | 104% | -2.89% | -2.19% | -2.37% | 121% | 100% |
| Class C | -2.05% | -2.24% | -2.64% | 146% | 102% | -1.23% | -1.13% | -1.04% | 116% | 100% |
| Class E | -3.41% | -0.13% | -0.22% | 200% | 101% | -2.38% | -1.23% | -1.59% | 142% | 98% |
| Overall | -2.95% | -1.84% | -2.02% | 165% | 102% | -2.21% | -1.60% | -1.73% | 124% | 99% |
| Class D | -2.55% | -1.58% | -2.25% | 139% | 102% | -1.74% | -1.53% | -1.90% | 116% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 13 Coding performance of test e (over VTM + AFFINE and BMS)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.88% | -0.43% | -0.54% | 99% | 95% | -0.49% | -0.40% | -0.37% | 101% | 98% |
| Class A2 | -3.41% | -2.10% | -2.55% | 100% | 88% | -2.52% | -1.60% | -1.57% | 100% | 96% |
| Class B | -1.31% | -1.14% | -1.44% | 101% | 92% | -0.81% | -0.88% | -1.05% | 101% | 96% |
| Class C | 0.05% | -0.15% | -0.11% | 101% | 92% | 0.34% | 0.07% | 0.03% | 101% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.28% | -0.93% | -1.13% | 100% | 92% | -0.78% | -0.67% | -0.73% | 101% | 97% |
| Class D | 0.69% | 0.37% | 0.31% | 101% | 91% | 0.92% | 0.53% | 0.57% | 101% | 97% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.92% | -1.08% | -0.86% | 99% | 95% | -0.38% | -0.73% | -0.34% | 100% | 96% |
| Class C | -0.61% | -1.47% | -2.04% | 98% | 96% | 0.18% | 0.06% | -0.01% | 100% | 96% |
| Class E | -1.25% | 1.73% | 2.15% | 98% | 94% | -0.53% | 0.85% | 0.41% | 100% | 94% |
| Overall | -0.90% | -0.51% | -0.50% | 98% | 95% | -0.23% | -0.07% | -0.04% | 100% | 96% |
| Class D | 0.45% | 0.48% | 0.07% | 98% | 93% | 1.34% | 1.34% | 0.88% | 99% | 95% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test f

Table 14 Coding performance of test f (over VTM and BMS - AFFINE)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.18% | -1.52% | -1.60% | 127% | 102% | -1.13% | -0.92% | -0.85% | 107% | 99% |
| Class A2 | -6.57% | -5.02% | -5.02% | 130% | 108% | -3.71% | -2.44% | -2.49% | 107% | 100% |
| Class B | -3.69% | -2.66% | -2.93% | 133% | 105% | -2.29% | -1.80% | -1.94% | 108% | 99% |
| Class C | -1.32% | -0.82% | -0.97% | 132% | 103% | -0.93% | -0.74% | -0.73% | 108% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.33% | -2.41% | -2.56% | 131% | 104% | -1.98% | -1.47% | -1.51% | 107% | 100% |
| Class D | -1.51% | -0.89% | -1.16% | 133% | 103% | -1.16% | -0.87% | -0.77% | 108% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.89% | -2.29% | -1.96% | 155% | 105% | -2.63% | -2.06% | -2.14% | 119% | 100% |
| Class C | -1.78% | -1.34% | -1.39% | 143% | 102% | -1.48% | -1.12% | -1.30% | 115% | 101% |
| Class E | -2.71% | -1.09% | -1.77% | 190% | 101% | -2.15% | -1.71% | -1.77% | 138% | 100% |
| Overall | -2.47% | -1.67% | -1.73% | 159% | 103% | -2.13% | -1.66% | -1.77% | 122% | 100% |
| Class D | -3.17% | -2.42% | -2.92% | 137% | 104% | -2.85% | -2.59% | -3.02% | 115% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 15 Coding performance of test f (over VTM + AFFINE and BMS)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | 0.07% | 0.05% | 95% | 96% | 0.00% | -0.11% | 0.04% | 99% | 98% |
| Class A2 | -0.76% | -0.30% | -0.60% | 94% | 91% | -0.30% | 0.05% | -0.06% | 99% | 97% |
| Class B | -0.45% | -0.51% | -0.62% | 95% | 93% | -0.14% | -0.34% | -0.45% | 99% | 98% |
| Class C | -0.05% | -0.06% | -0.16% | 97% | 93% | 0.15% | -0.03% | -0.02% | 99% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.36% | -0.23% | -0.36% | 95% | 93% | -0.07% | -0.13% | -0.16% | 99% | 98% |
| Class D | 0.29% | 0.14% | 0.06% | 97% | 93% | 0.44% | 0.43% | 0.32% | 100% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.41% | -0.82% | -0.21% | 95% | 97% | -0.11% | -0.60% | -0.09% | 98% | 97% |
| Class C | -0.34% | -0.57% | -0.79% | 96% | 96% | -0.08% | 0.06% | -0.28% | 99% | 97% |
| Class E | -0.54% | 0.76% | 0.56% | 93% | 94% | -0.30% | 0.34% | 0.24% | 97% | 96% |
| Overall | -0.42% | -0.34% | -0.21% | 95% | 96% | -0.15% | -0.14% | -0.07% | 98% | 96% |
| Class D | -0.25% | -0.40% | -0.63% | 97% | 95% | 0.12% | 0.21% | -0.32% | 98% | 96% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test g

Table 16 Coding performance of test g (over VTM and BMS - AFFINE)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.81% | -1.92% | -2.28% | 132% | 103% | -1.62% | -1.30% | -1.28% | 107% | 100% |
| Class A2 | -9.09% | -6.78% | -6.80% | 137% | 107% | -5.93% | -3.96% | -3.91% | 108% | 100% |
| Class B | -4.58% | -3.32% | -3.62% | 140% | 105% | -3.02% | -2.34% | -2.55% | 109% | 98% |
| Class C | -1.54% | -1.10% | -1.11% | 138% | 102% | -1.09% | -0.77% | -0.81% | 109% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -4.32% | -3.14% | -3.32% | 137% | 104% | -2.81% | -2.04% | -2.10% | 109% | 99% |
| Class D | -1.66% | -0.96% | -1.24% | 140% | 100% | -1.18% | -1.13% | -1.00% | 109% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -3.39% | -2.68% | -2.55% | 161% | 105% | -3.04% | -2.32% | -2.71% | 120% | 100% |
| Class C | -2.04% | -1.69% | -2.06% | 146% | 102% | -1.61% | -1.44% | -1.35% | 115% | 101% |
| Class E | -3.34% | -1.15% | -1.16% | 201% | 101% | -2.58% | -1.84% | -1.78% | 141% | 98% |
| Overall | -2.93% | -1.97% | -2.04% | 165% | 103% | -2.45% | -1.91% | -2.03% | 124% | 100% |
| Class D | -3.30% | -2.49% | -2.54% | 139% | 101% | -2.98% | -2.55% | -2.84% | 116% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 17 Coding performance of test g (over VTM + AFFINE and BMS)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.86% | -0.35% | -0.64% | 98% | 96% | -0.49% | -0.51% | -0.39% | 100% | 98% |
| Class A2 | -3.46% | -2.15% | -2.47% | 99% | 90% | -2.60% | -1.51% | -1.52% | 99% | 97% |
| Class B | -1.39% | -1.20% | -1.34% | 101% | 94% | -0.90% | -0.88% | -1.07% | 100% | 96% |
| Class C | -0.27% | -0.34% | -0.31% | 101% | 92% | -0.01% | -0.06% | -0.10% | 101% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.40% | -0.99% | -1.15% | 100% | 93% | -0.92% | -0.71% | -0.76% | 100% | 97% |
| Class D | 0.14% | 0.07% | -0.02% | 102% | 91% | 0.42% | 0.17% | 0.08% | 101% | 96% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.94% | -1.22% | -0.81% | 98% | 97% | -0.53% | -0.86% | -0.68% | 99% | 96% |
| Class C | -0.60% | -0.93% | -1.47% | 98% | 96% | -0.22% | -0.26% | -0.33% | 99% | 97% |
| Class E | -1.18% | 0.70% | 1.19% | 98% | 94% | -0.74% | 0.21% | 0.21% | 99% | 95% |
| Overall | -0.89% | -0.64% | -0.53% | 98% | 96% | -0.48% | -0.39% | -0.34% | 99% | 96% |
| Class D | -0.38% | -0.47% | -0.23% | 99% | 92% | -0.02% | 0.25% | -0.13% | 99% | 95% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusions

This contribution reports the results of integrating affine inter prediction in JVET-J0024 to BMS1.0. In this contribution, affine inter prediction is improved by 4 aspects.

For aspect 1 (affine AMVP improvement), experimental results reportedly show on average 0.26%/0.06% luma BD-rate gain in RA/LB configurations over to VTM with affine on. For aspect 2(EIF), experimental results reportedly show on average 0.25%/0.41% luma BD-rate gain with 9%/6% decoding time reduction in RA/LB configurations over VTM with affine on.

We propose to adopt affine AMVP improvement and EIF into the test model.

References

- “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario”, JVET-J0024, April 2018

- J. Chen, E. Alshina, G. J. Sullivan, J-R. Ohm, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7)”, JVET-G1001, July 2017.

- BMS reference software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- H. Yang, S. Liu, K. Zhang, “Description of CE4: Inter prediction and motion vector coding”, JVET-J1024, April 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).