| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-Kxxxx-v1 |
| --- | --- |

| Title: | CE1: Asymmetric Binary Tree (tests 1.0.1, 1.0.2, 1.0.3, 1.0.4, 8.0.1 and 8.0.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Fabrice Le Léannec, Tangi Poirier, Franck Galpin. | Tel: Email: | +33(0)299273198 fabrice.leleannec@technicolor.com tangi.poirier@technicolor.com franck.galpin@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the results of Asymmetric Binary Tree (ABT) partitioning method, described in JVET-J0022, implemented within the JVET BMS.

# Introduction

Asymmetric Binary Tree partitioning was proposed in JVET-J0022. This contribution reports the results of ABT in various configurations, when implemented in the BMS software with the Common Tests Conditions.

The CE1 tests reported in this document are the following ones: 1.0.1, 1.0.2, 1.0.3, 1.0.4, 8.0.1, 8.0.2.

# Description

## Partitioning method used

Four new Binary Tree splitting modes were introduced into the QTBT framework, so as to allow new splitting configurations. So-called Asymmetric Binary Tree split modes (ABT) are employed in addition to the splitting modes already available in QTBT, as shown by Error! Reference source not found..

According to the added splitting modes, a coding unit with size S is divided into 2 sub-CU with sizes S/4 and 3.S/4, either in the horizontal or in the vertical direction. In practice the added available CU sizes are 6, 12, 24 and 48.

Overall, a CTU can be divided into square or rectangle CUs, according to various split options, as illustrated in Figure 4: quadtree (QT), symmetric binary tree (BT), and a new type of split, asymmetric binary tree (ABT).

Figure 1: Split modes (tree types) supported in the QTBT+ABT framework

A CU resulting from an asymmetric splitting may be further split in a recursive way, as shown on Figure 5 This includes binary symmetric split, asymmetric binary split along the direction which size is a power of two, and optionally asymmetric binary split in the orientation where the CU dimension is a multiple of 3. In the latest case, the divided CU horizontal or vertical sizes are respectively equal to s/3 and 2*s/3, where s is the parent CU horizontal or vertical size.

Figure 2: Spatial splitting optionally adaptive to the parent CU Size

## ABT signaling

In the QTBT+ABT framework up to 2 bins are added to the QTBT signaling a CU’s split mode, as illustrated on Figure 7. The first bin indicates if ABT is used, in case a CU is partitioned in non-quad-tree mode. If ABT is used for the considered coding tree node, then the second added bin indicates the asymmetric split type: 0 for left/top (vertical split), 1 for right/bottom (horizontal split). The two added bins are arithmetically with one single context.

Figure 3: ABT signaling

## Non-power of 2 adaptations

### Quantization

With the Asymmetric Binary Tree, blocks may have sizes equal to 3×2n in width and/or height. This leads to an overall block surface potentially equal to 3×2n×2m=3×2n+m or 3×2n×3×2n=9×2n+m. To properly handle all these cases, one needs to choose the post-inverse-quantization scale factors, based on the following analysis.

Error! Reference source not found. depicts the inverse transform and inverse quantization process, for any rectangular block with generic size noted (s1,s2). Therefore, the successively scaling operations applied during this overall process are the following ones.

- 26: scale factor applied during the inverse quantization stage

- SIQ: scale factor applied after the inverse quantization stage

- s1×26 : scale factor used during the first (horizontal) inverse transform stage

- SIT1: scale factor applied after the first inverse transform stage

- s2×26 : scale factor applied during the second (vertical) inverse transform stage

- SIT2: scale factor applied after the second inverse transform stage.

Figure 4: Inverse Transform and inverse quantization steps, with associated respective scaling factors used for the HEVC-like integer-based implementation, for a generic rectangular transform block with size (s1,s2)

Figure 2 provides the values of each scale factor involved and the overall scale factor resulting from the product of all scale factors applied in the inverse 2D transform. It is equal to s1s2×2-(15-B).

Figure 5: value of each scaling factor in the HEVC-like inverse transform process, with generic rectangular blocks, where (s1,s2) is the size of the rectangular transform block and B is the signal bit-depth.

Therefore, to ensure the preservation of the residual norm for Qstep=1 i.e. QP=4, post-quantization scaling factor SIQ are chosen such that SIQ×g4×s1s2×2-(15-B)=1

Which leads to:

SIQ=1s1s2 ×2-(B-9)

The practical implementation of such scaling factor with only multiplications and shift operations, needs to differentiate 6 different configurations for the 2D block sizes, which are listed in Figure 3.

Figure 6: post-inverse quantization scaling factor SIQ which is needed to preserve the residual norm

### Coding of last significant coefficient position

Let us note c the block size equal to a multiple of three in width or height, i.e. of the form c=3×2n. The syntax elements that use block size dependent contexts are the following ones.

- The LastSignificantXY syntax element coding for block size c, employs the context that would normally be used for the closest block size smaller than c and equal to a power of two, i.e. 3×c/4=2n+1. Apart from this context sharing between different block sizes, the coding of LastSignificantXY is identical to that of the VTM1.0.

### Additional primary transforms to support non-power of 2 block sizes

The set of primary transforms contained in the BMS is extended to includes the following transforms with sizes 6, 12, 24 and 48: DCT2, DCT5, DCT8, DST1 DST7.

# Results

The performances, encoding times and decoding times of each of the following tests are reported: 1.0.1, 1.0.2, 1.0.3, 1.0.4, 8.0.1, 8.0.2.

## Test 1.0.1: QTBT+ABT with new block size 12,24,48 (luma)

Apart from the entropy coding and post-quantization scaling described in section 2.3, the overall VTM1.0 configuration parameters used in this test are the following ones.

- Minimum and maximum CTU size: same as VTM1.0

- Minimum and maximum TU size: 2 to 64

- Minimum and maximum PU size: 2 to 64 (no partitioning of CUs into PUs)

- Additional transform types and sizes:

  - VTM1.0: DCT2 is extended to sizes 6,12,24,48

  - BMS1.0: the EMT set of transforms is extended to sizes 6,12,24,48

### VTM-1.0

### BMS-1.0

## Test 1.0.2: QTBT+ABT with new block size 12,24 (luma)

Apart from the entropy coding and post-quantization scaling described in section 2.3, the overall VTM1.0 configuration parameters used in this test are the following ones.

- Minimum and maximum CTU size: same as VTM1.0

- Minimum and maximum TU size: 2 to 64

- Minimum and maximum PU size: 2 to 64 (no partitioning of CUs into PUs)

- Additional transform types and sizes:

  - VTM1.0: DCT2 is extended to sizes 6,12,24

  - BMS1.0: the EMT set of transforms is extended to sizes 6,12,24

### VTM-1.0

### BMS-1.0

## Test 1.0.3: QTBT+ABT, with new block sizes 12,24,48 and allow mode ratio (1/3,2/3)

### VTM-1.0

### BMS-1.0

## Test 1.0.4: Code/Decode CU with size 12,24,48 either with null residual, or by dividing the CU into 2 power of 2 transform blocks

### VTM-1.0

### BMS-1.0

## Test 8.0.1: QTBT+ABT, medium complexity level

### VTM-1.0

### BMS-1.0

## Test 8.0.2: QTBT+ABT, medium complexity level

### VTM-1.0

### BMS-1.0

# Discussion

The overall curve associated to the BD-rate/encoding time trade-offs provided by the proposed ABT tool are illustrated on figure, for the VTM Random Access configuration.

…..

Figure 7: various BD-rate/Enc. Time trade-offs provided by ABT

The BD-rate-complexity combined slope values, as specified in document JVET-J0094 are provided in

Table 1: coding efficiency/complexity slopes of the delivered ABT operating points

# Conclusion

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).