| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET- K0212-v2 |
| --- | --- |

| Title: | Improved Cyclic Intra Refresh |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jean-Marc Thiesse, Didier Nicholson VITEC 99 rue Pierre Semard 92320 Chatillon, France | Tel: Email: | +33 1 46 73 06 06 jean-marc.thiesse@vitec.com didier.nicholson@vitec.com |
| Source: | VITEC |  |  |

_____________________________

# Abstract

This contribution proposes a technique to improve the use of Cyclic Intra Refresh, a technique that is commonly used for real-time implementation of HEVC video encoders. Intra Refresh provides several desirable features, such as bit-rate smoothing, maintaining Intra data bit-rate under control, low delay and natural error resilience limiting temporal error propagation along frames. For video encoder’s manufacturers this feature is considered as mandatory for real-life applications.

In VVC Requirements, as contained in VCEG-BD03 and WG11/ N17074, low latency and error resilience requirements are present but have not been addressed by the responses to the CfP. One big factor of latency apart picture ordering and related dependencies, is the importance of Intra picture data amongst other, with an increasing Intra picture data size versus Inter picture data size ratio at each video codec generation. Cyclic Intra Refresh can be used for fulfilling these requirements.

But this feature, as applied by encoder manufacturer in previous standards, implies some encoding restrictions, limiting the coding efficiency, as well as Random Access capability. The proposed technique through appropriate signaling enables to leverage these limitations, allowing less coding restrictions. In this document results provided show an improvement toward classic use of Cyclic Intra Refresh together with usual LD configuration as a reference. Authors recommend considering the proposed technique for a potential inclusion in VVC future standard with further study to be conducted in an appropriate Core Experiment.

# Introduction

In VVC Requirements [1][2], low latency and error resilience requirements are present have not been addressed by the responses to the CfP. One big factor of latency apart picture ordering and related dependencies, is the importance of Intra picture data amongst other, with an increasing Intra picture data size versus Inter picture data size ratio at each video codec generation. This phenomenon is due to the fact that reached coding gain on Intra pictures is less than on Inter pictures. The bigger size of Intra picture is generating a latency issue when sent to a fixed data rate network, and furthermore can generate network congestions and packet losses. For reducing the impact of Intra picture size it is common on practical real time implementation of video encoders to use Cyclic Intra Refresh [3] to spread Intra data along the Inter frames, as depicted in the figure 1, smoothing bitrate variation. In addition to solving instantaneous high rate issue, it provides additional error robustness capabilities [4].

Figure 1: Intra Refresh principle.

This is represented on figures 2 and 3 extracted from an HEVC analyzer which respectively shows the sizes of pictures for a classical IB structure with and without Intra Refresh. The Intra pictures are in red while B pictures are in green. In figure 2, the third I pictures is 8 times higher than following B pictures taking much more time to be send and requiring a bigger buffer which increase the latency. In figure 3, using Cyclic Intra Refresh, the picture size is homogenous and the latency is reduced.

Figure 2: Size of picture in a classical IB structure.

Figure 3: Size of picture in a classical IB structure with Intra Refresh.

As reported in figure 1, two regions are defined, a Clean which corresponds to the pixels which have been refreshed during the current cycle and the Dirty region which will be refreshed. In yellow, the Intra column is represented here. The principle is to ensure that pixels from Clean region are reconstructed using pixels coming only from refreshed area of the current cycle. Several restrictions in coding are added in order to ensure this, decreasing the coding efficiency.

In addition to the blocks forced to Intra coding, the usual restrictions used in order to insure the video refresh after a cycle includes:

- Enabling Constrained Intra Prediction (CIP) so that each Intra blocks used pixels independent from previously encoded pictures.

- Restricting the motion vectors on the Clean region to used only pixels from Clean regions of previously encoded pictures (including take care of half and quarter-pel vectors at the column boundary).

- Taking care of old references when re-looping in case of column.

This document proposes an improved Cyclic Intra Refresh method, including some signalling that allows keeping good coding efficiency while making CIR well integrated in the standard.

# Proposed Improved Intra Refresh

## Description of the method

The proposed solution adds the minimum signalization needed to determinate the position of Intra Refreshed pixels and then exploits this position knowledge to improve the quality within this low-latency use-case. The modifications at the encoder side involved:

- Avoid the transmission of pred_mode_flag for Coding Unit with at least one pixel within the Intra Refresh Area.

- Forcing splits of CU to reach the Intra Refresh Area.

- Enable the Constrained Intra Prediction (CIP) tool adaptively depending on the position and direction of the Intra Refresh Area.

- Take care of the Intra Refresh position for the motion vectors derivation.

- Define two deltas QP by slice depending on the Intra Refresh Area position.

## Signalization

The proposed additional signalization is:

- at PPS level:

  - intra_refresh_enabled_flag specifies if the Intra Refresh tool is enabled.

  - intra_refresh_mode specifies the mode of Intra Refresh (Column, Line, pseudo-random, …)

  - intra_refresh_size  specifies the size of the refresh pattern (width of the Column, height of the Line, width x height for block-based refresh). This can be signaled as a multiple of the CU min size (0 for 8 pixels, 1 for 16 pixels, …).

  - intra_refresh_delta_qp specifies a delta QP for the Intra Refresh blocks. This allows to optimize the encoder quality balancing between Intra Refresh blocks and none.

- at Slice level:

  - intra_refresh_direction specifies the direction of the refresh (right to left/left to right for Column, top to bottom/bottom to top for Line, …). This is used to determine motion vectors constraints during the encoding/decoding.

  - intra_refresh_position specifies the position of the Intra Refresh blocks, this is given according to intra_refresh_size.

The corresponding draft modifications are:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_qt_size_intra_slices_minus2 | ue(v) |
| log2_min_qt_size_inter_slices_minus2 | ue(v) |
| max_mtt_hierarchy_depth_inter_slices | ue(v) |
| max_mtt_hierarchy_depth_intra_slices | ue(v) |
| intra_refresh_enabled_flag | u(1) |
| if( intra_refresh_enabled_flag ){ |  |
| intra_refresh_mode | ue(v) |
| intra_refresh_size | ue(v) |
| intra_refresh_delta_qp | ue(v) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if ( slice_type != I ) |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if (intra_refresh_enabled_flag ){ |  |
| intra_refresh_direction | u(1) |
| intra_refresh_position | ue(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

| coding_unit( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if( slice_type != I && !intra_refresh_cu) { |  |
| pred_mode_flag | ae(v) |
| } |  |
| … |  |

….

CuIntraRefresh[ xNbY ][ yNbY ] is derived at frame level according to intra_refresh_position value. CuIntraRefresh[ xNbY ][ yNbY ] is equal to 1 if the CU belongs to the Intra Refresh area.

….

- Each sample p[ x ][ y ] is derived as follows:

– If one or more of the following conditions are true, the sample p[ x ][ y ] is marked as "not available for intra prediction":

– The variable availableN is equal to FALSE.

– CuPredMode[ xNbY ][ yNbY ] is not equal to MODE_INTRA and constrained_intra_pred_flag is equal to 1.

– CuPredMode[ xNbY ][ yNbY ] is not equal to MODE_INTRA and CuIntraRefresh[ xNbY ][ yNbY ]  is equal to 1.

– Otherwise, the sample p[ x ][ y ] is marked as "available for intra prediction" and the sample at the location ( xNbCmp, yNbCmp ) is assigned to p[ x ][ y ].

Figure 4: Proposed signalization and modification.

# Results

## Intra Refresh against Reference

First, Intra Refresh as it can be implemented classically on top of VTM 1.1 has been tested with JVET conditions for Low DelayB-10 configuration on Class B and Class E sequences. An Intra Refresh using a column scanning from left to right has been used and in that test only Intra forcing and CIP are implemented. Two sizes of column have been tested (16 and 64).

|  |  | 16 |  |  | 64 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| Class B | MarketPlace | 5.82% | 7.62% | 8.16% | 14.00% | 13.17% | 13.17% |
|  | RitualDance | 8.37% | 12.77% | 13.67% | 13.56% | 17.01% | 17.82% |
|  | Cactus | 7.99% | 10.54% | 10.72% | 19.18% | 20.09% | 20.60% |
|  | BasketBallDrive | 8,95% | 17,37% | 16,92% | 13,36% | 20,22% | 19,47% |
|  | BQTerrace | 10.12% | 10.90% | 10.86% | 27.97% | 26.50% | 27.51% |
|  | Average | 8,25% | 11,84% | 12,07% | 17,62% | 19,40% | 19,71% |
| Class E | FourPeople | 28.61% | 28.40% | 28.79% | 83.90% | 82.79% | 82.85% |
|  | Johnny | 34.92% | 35.54% | 35.92% | 101.97% | 105.39% | 106.82% |
|  | KristenAndSara | 25.38% | 25.07% | 24.34% | 78.42% | 76.75% | 76.61% |
|  | Average | 29,64% | 29,67% | 29,68% | 88,10% | 88,31% | 88,76% |

Table 1 Low Delay B, Reference Intra Refresh against Reference VVC.

Results are reported on Table 1 with a significant loss, especially for Class E sequences. This confirms the need for an improved handling of Intra Refresh. The loss logically increases with the size of the column and it is important to notice that having a 64 width column refresh is usually used. Losses are also higher as the sequence is easy to encode (small Inter picture).

Actually these strong losses comes in part from a non-fair comparison since the reference LDB-10 has only one Intra picture at the beginning while the tested Intra Reference configuration actually corresponds to an Intra period of 120 and 30 respectively for column width of 16 and 64. A test with LDB-10 and an Intra period of 20 (to match the GOP size of 4) has been consequently reported on table 2 for reference against Intra Refresh of with 64. The losses are actually in the range of 25.66% in Y, 42.66% in U and 41.61% in V components. Corresponding losses are expected for Class B sequences, keeping the needs for an improved Intra Refresh solution.

|  |  | 64 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| Class E | FourPeople | 25,33% | 38,23% | 37,15% |
|  | Johnny | 25,77% | 48,09% | 43,10% |
|  | KristenAndSara | 25,87% | 41,64% | 44,57% |
|  | Average | 25,66% | 42,66% | 41,61% |

Table 2 Low Delay B with Intra Period of 20, Reference Intra Refresh against Reference VVC.

## Improved Intra Refresh (early results)

In this section, the proposed Improved Intra Refresh is evaluated against the Reference Intra Refresh (without signalization). The modifications which are evaluated here include:

- Intra Refresh signalization at PPS and Slice level.

- No transmission of cu_skip_flag as well as pred_mode_flag on Intra Refresh CU.

- Enabling of Constrained Intra Prediction only for Intra Refresh CU.

|  |  | 16 |  |  | 64 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| Class B | MarketPlace | -1,31% | -3,27% | -3,79% | -1,50% | -2,75% | -2,94% |
|  | RitualDance | -5,13% | -8,93% | -9,35% | -5,20% | -8,29% | -8,85% |
|  | Cactus | -2,91% | -4,99% | -5,20% | -3,00% | -4,58% | -5,11% |
|  | BasketBallDrive | -5,34% | -11,08% | -11,58% | -5,24% | -9,95% | -10,59% |
|  | BQTerrace | -1,14% | -2,33% | -1,60% | -1,38% | -1,43% | -1,49% |
|  | Average | -3,16% | -6,12% | -6,30% | -3,26% | -5,40% | -5,80% |
| Class E | FourPeople | -2.74% | -2.94% | -3.56% | -2.90% | -2.55% | -2.87% |
|  | Johnny | -2,42% | -2,67% | -1,75% | -2,70% | -2,34% | -2,42% |
|  | KristenAndSara | -2,34% | -3,21% | -3,04% | -2,73% | -2,75% | -2,46% |
|  | Average | -2,50% | -2,94% | -2,78% | -2,78% | -2,55% | -2,58% |

Table 3 Low Delay B, Improved Intra Refresh against Reference Intra Refresh.

# Conclusion

Low latency is classically achieved in real life video encoder implementations with Intra Refresh at a cost of significant coding efficiency loss (that can be very significant e.g. on Class E test sequences 29.64% loss on Y component with column width of 16). In this document, a method for an improved Cyclic Intra Refresh method was proposed, including some normative signalling that allows keeping good coding efficiency.

- Minimal signalization at PPS and Slice level.

- Small modification within encoder and decoder to exploit the IR position.

Results show an IR improvement of -3.16% and -2.50% in Luma for Class B and E respectively while providing all IR advantages for latency reduction (and error robustness improvement).

# Recommendation

It is recommended to consider it for a potential inclusion in VVC future standard with further study to explore this solution that could be conducted in an Ad Hoc Group (e.g. AHG on Low Latency). Studies that could be conducted may include, but not limited to:

- Definition of CTC and appropriate test set

- Appropriate Core Experiment.

- Explore/cross-check the proposed modifications as well as improvements

- To integrate the proposed modifications within VTM.

# Acknowledgement

The research leading to this work has been supported by the FUI23 EFIGI project, BPIFrance and Region Ile de France.

# References

- “Requirements for Future Video Coding (FVC)”, ITU-TSG16/Q6 VCEG56th meeting, Torino, Italy, July2017, Doc. VCEG-BD03 (available at http://wftp3.itu.int/av-arch/video-site/1707_Tor/).

- “Requirements for a Future Video Coding Standard v5”, ISO/IEC JTC1/SC29/WG11 MPEG, 119th meeting, Torino, Italy, July 2017, Doc.N17074(available at http://mpeg.chiariglione.org/meetings/119).

- Tri D. Tran, Lurng-Kuo Liu, Peter H. Westering: “Low-delay MPEG-2 video coding”, Proc. SPIE - Int. Soc. Opt. Eng. (USA), USA: SPIE-Int. Soc. Opt. Eng, vol.3309, 1997 pp. 510-16.

- Haoming Chen, Chen Zhao, Ming-Ting Sun a, Aaron Drake, “Adaptive intra-refresh for low-delay error-resilient video coding”, J. Vis. Commun. Image R. 31 (2015) 294–304

# Patent rights declaration(s)

VITEC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).