| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0245-v1 |
| --- | --- |

| Title: | CE4.2.8: Merge mode enhancement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ling Hsiao, Zhi-Yi Lin, Tzu-Der Chuang, Chun-Chia Chen, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yuling.hsiao, zhi-yi.lin, peter.chuang, chunchia.chen, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents the test results of Core Experiment 4.2.8 (CE4.2.8), including affine mode with integrating affine merge candidates into one single merge list, more spatial and temporal candidates, pairwise-average candidates to replace the combined candidates from HEVC, and alternative temporal motion vector prediction (ATMVP) mode with adding one additional ATMVP candidate and performing redundancy check. Results are summarized as follows.

Affine mode with integrating affine merge candidates into one single merge list
VTM-1.0-RA: {Y-BD-rate = -3.36%, EncT=132%, DecT=122%} (no affine mode in anchor)
VTM-1.0-LB: {Y-BD-rate = -2.23%, EncT=159%, DecT=110%} (no affine mode in anchor)
BMS-1.0-RA: {Y-BD-rate = -2.12%, EncT=110%, DecT=108%} (disable JEM affine mode in anchor)
BMS-1.0-LB: {Y-BD-rate = -2.21%, EncT=126%, DecT=113%} (disable JEM affine mode in anchor)

More spatial and temporal candidates
VTM-1.0-RA: {Y-BD-rate = -0.29%, EncT=104%, DecT=99%} (HEVC spat. & temp. cand. in anchor)
VTM-1.0-LB: {Y-BD-rate = -0.39%, EncT=104%, DecT=99%} (HEVC spat. & temp. cand. in anchor)
BMS-1.0-RA: {Y-BD-rate = -0.09%, EncT=103%, DecT=100%} (HEVC spat. & temp. cand. in anchor)
BMS-1.0-LB: {Y-BD-rate = -0.10%, EncT=103%, DecT=99%} (HEVC spat. & temp. cand. in anchor)

Pairwise-average candidates to replace the combined candidates from HEVC
VTM-1.0-RA: {Y-BD-rate = -0.41%, EncT=103%, DecT=100%} (combined candidates in anchor)
VTM-1.0-LB: {Y-BD-rate = -0.13%, EncT=102%, DecT=100%} (combined candidates in anchor)
BMS-1.0-RA: {Y-BD-rate = -0.51%, EncT=103%, DecT=101%} (combined candidates in anchor)
BMS-1.0-LB: {Y-BD-rate = -0.27%, EncT=102%, DecT=101%} (combined candidates in anchor)

ATMVP with adding one additional ATMVP candidate and performing redundancy check
VTM-1.0-RA: {Y-BD-rate = -1.00%, EncT=105%, DecT=118%} (no ATMVP in anchor)
VTM-1.0-LB: {Y-BD-rate = -1.19%, EncT=104%, DecT=109%} (no ATMVP in anchor)
BMS-1.0-RA: {Y-BD-rate = -0.77%, EncT=102%, DecT=106%} (disable JEM ATMVP in anchor)
BMS-1.0-LB: {Y-BD-rate = -1.15%, EncT=101%, DecT=105%} (disable JEM ATMVP in anchor)

# Introduction

In the 10th JVET meeting, four merge mode modifications were presented in JVET-J0018 [1] and planned in CE4.2.8, JVET-J1024 [2]. The first one is adding affine merge candidate into merge list and studied in CE4.2.8.A. The second one is allowing more spatial and temporal candidates in merge list, which is studied in CE4.2.8.B. CE4.2.8.C is to use pairwise-average candidates in merge list to replace the combined candidates from HEVC. The last one, CE4.2.8.D, is related to AMVTP modifications. More details are introduced in the next sections.

# Proposed methods

## CE4.2.8.A: Affine merge candidates

In this test, the separate affine merge candidate list in BMS is removed and the affine candidates are added to merge candidate list. There are two kinds of affine candidates. The first one is spatial inherited affine candidate. The second one is corner derived affine candidates. Spatial inherited affine candidates use affine model from one of neighboring coding units (CUs) located at A1, B1, B0, A0, and B2 to derive current CU’s affine model as shown in Figure 1. In this proposal, spatial inherited affine candidates only use 6-parameter affine model. For each spatial location, if the spatial inherited affine candidate is available, then its original spatial inherited candidates will not be added to the merge candidate list. The spatial affine candidates searching order goes as A1  B1  B0 A0 B2.

Corner derived affine candidates simply set three spatial candidates as control points. For example, the motion vector (MV) sets of block {(B2, A1, B1), (B2, A0, B1), (B2, A1, B0), (B2, A0, B0)} as shown in Figure 1. Available motion triplets of spatial neighbors with the same reference picture are inserted. In this proposal, corner derived affine candidates only use 6-parameter affine model. At most four sets of control points {(B2, A1, B1), (B2, A0, B1), (B2, A1, B0), (B2, A0, B0)} will be added to the candidate list as corner derived affine candidates, and the corner derived affine candidates are inserted after spatial candidate at location B2.

Figure 1. Position for spatial inherited affine and corner derived affine candidates.

## CE4.2.8.B: Middle spatial and multiple temporal candidates

In additional to the spatial candidates located at A1, B1, B0, A0 and B2 in HEVC, two middle spatial candidates located at ML and MT are added as shown in Figure 2. The middle spatial candidates at the middle positions of left and top edges are added by following the order {ML, MT}, and they are added after temporal candidates. In contrast to temporal candidates in HEVC (RB or CT if RB is not available), four temporal candidates are added by following the order {RB, CT, LB, RT} as shown in Figure 2.

Figure 2. Positions for spatial (darkest blocks) and temporal (brightest blocks) candidates.

## CE4.2.8.C: Pairwise-average candidates as replacement of HEVC combined candidates

Pairwise average candidates are generated by averaging predefined pairs of candidates in the current merge candidate list, and the predefined pairs are defined as {(0, 1), (0, 2), (1, 2), (0, 3), (1, 3), (2, 3)}, where the numbers denote the merge indices to the merge candidate list. The averaged motion vectors are calculated separately for each reference list; if both MVs are available in one list, the MV with the larger merge index is scaled to the reference picture of the merge candidate with the smaller merge index; if only one MV is available, use the MV directly; if no MV is available, keep this list invalid. The pairwise average candidates replace the combined candidates from HEVC.

## CE4.2.8.D: Double ATMVP candidates with uniformity check and decoder-side speed-up

ATMVP is basically the same as JEM-7.0; however, there are modifications here. Two MVs from merge candidate list instead of one MV are used as temporal MVs to generate two ATMVP candidates. The first and second temporal MVs are the first and second available MVs in the merge candidate list, respectively. There are two types of uniformity checks; one is to check whether the motion information at the four corners of ATMVP sub-blocks is the same as the motion information at the center, and treat it as a normal temporal candidate if they are the same; the other is to check whether the motion information at each of the four corners is the same for the two corresponding ATMVP sub-blocks, and the second ATMVP candidate is not inserted if they are the same at all the four corners. To save decoding time at the decoder side, it only derives motion information at the four corner sub-blocks for uniformity check instead of all sub-blocks.

# Experiment results

The proposed modifications have been implemented on top of BMS-1.0. The common test conditions [3] are used for evaluation. Note that the simulations in VTM test condition are performed by using VTM encoder configuration settings without MACRO changes in the software. Table 1, Table 2, Table 3, and Table 4 summarize the results of CE4.2.8.A, CE4.2.8.B, CE4.2.8.C, and CE4.2.8.D, respectively.

Table 1. CE4.2.8.A: Affine merge candidates.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 w/o Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.37% | -2.09% | -2.18% | 130% | 121% | -1.29% | -1.05% | -1.00% | 108% | 109% |
| Class A2 | -6.46% | -5.38% | -5.15% | 130% | 135% | -3.82% | -2.86% | -2.85% | 108% | 116% |
| Class B | -3.64% | -2.55% | -2.71% | 134% | 125% | -2.42% | -1.67% | -1.74% | 111% | 104% |
| Class C | -1.42% | -0.88% | -0.95% | 134% | 110% | -1.10% | -0.74% | -0.67% | 110% | 106% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.36% | -2.58% | -2.62% | 132% | 122% | -2.12% | -1.53% | -1.53% | 110% | 108% |
| Class D | -1.87% | -1.25% | -1.38% | 137% | 113% | -1.61% | -1.39% | -1.16% | 112% | 110% |
| Class F (optional) | -1.48% | -1.47% | -1.52% | 131% | 105% | -1.23% | -1.33% | -1.32% | 109% | 107% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 w/o Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.63% | -1.94% | -1.96% | 153% | 107% | -2.69% | -1.78% | -1.89% | 127% | 119% |
| Class C | -1.49% | -0.60% | -0.52% | 142% | 111% | -1.50% | -1.32% | -1.28% | 118% | 111% |
| Class E | -2.53% | -2.11% | -2.71% | 197% | 113% | -2.34% | -2.52% | -2.14% | 138% | 105% |
| Overall | -2.23% | -1.53% | -1.67% | 159% | 110% | -2.21% | -1.81% | -1.75% | 126% | 113% |
| Class D | -3.05% | -2.42% | -2.40% | 135% | 102% | -3.10% | -2.59% | -3.41% | 117% | 113% |
| Class F (optional) | -1.81% | -1.71% | -1.50% | 156% | 107% | -1.56% | -1.41% | -2.36% | 113% | 103% |

Table 2. CE4.2.8.B: Middle spatial and multiple temporal candidates.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.33% | -0.30% | 103% | 99% | -0.04% | -0.13% | 0.03% | 102% | 99% |
| Class A2 | -0.30% | -0.31% | -0.27% | 103% | 100% | -0.08% | -0.15% | -0.09% | 103% | 100% |
| Class B | -0.28% | -0.31% | -0.19% | 104% | 98% | -0.11% | -0.09% | -0.12% | 103% | 100% |
| Class C | -0.26% | -0.21% | -0.20% | 105% | 100% | -0.11% | -0.05% | -0.17% | 104% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.29% | -0.29% | -0.23% | 104% | 99% | -0.09% | -0.10% | -0.10% | 103% | 100% |
| Class D | -0.32% | -0.23% | -0.32% | 105% | 100% | -0.24% | -0.17% | -0.25% | 104% | 100% |
| Class F (optional) | 0.02% | -0.08% | -0.13% | 104% | 99% | 0.00% | -0.10% | -0.10% | 103% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.38% | -0.34% | -0.10% | 104% | 99% | -0.03% | 0.01% | 0.33% | 103% | 100% |
| Class C | -0.25% | -0.26% | -0.16% | 105% | 98% | -0.05% | 0.07% | 0.14% | 103% | 99% |
| Class E | -0.60% | -0.47% | -0.79% | 102% | 99% | -0.28% | -0.17% | 0.43% | 102% | 99% |
| Overall | -0.39% | -0.35% | -0.30% | 104% | 99% | -0.10% | -0.02% | 0.29% | 103% | 99% |
| Class D | -0.16% | -0.59% | -0.46% | 106% | 99% | -0.03% | 0.18% | -0.10% | 103% | 99% |
| Class F (optional) | -0.14% | -0.24% | -0.32% | 104% | 99% | 0.10% | 0.50% | 0.67% | 102% | 100% |

Table 3. CE4.2.8.C: Pairwise-Average Candidates instead of Combined Candidates.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.32% | -0.41% | 102% | 100% | -0.46% | -0.40% | -0.31% | 103% | 100% |
| Class A2 | -0.63% | -0.60% | -0.63% | 102% | 101% | -0.62% | -0.49% | -0.54% | 103% | 103% |
| Class B | -0.43% | -0.35% | -0.30% | 103% | 100% | -0.55% | -0.32% | -0.33% | 103% | 102% |
| Class C | -0.20% | -0.26% | -0.27% | 104% | 100% | -0.41% | -0.32% | -0.32% | 104% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.41% | -0.37% | -0.38% | 103% | 100% | -0.51% | -0.37% | -0.36% | 103% | 101% |
| Class D | -0.41% | -0.26% | -0.42% | 104% | 102% | -0.65% | -0.45% | -0.55% | 104% | 101% |
| Class F (optional) | 0.01% | -0.04% | -0.09% | 103% | 99% | -0.10% | -0.18% | -0.19% | 103% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.12% | 0.20% | 0.27% | 102% | 101% | -0.20% | 0.00% | 0.26% | 102% | 101% |
| Class C | 0.02% | -0.02% | 0.31% | 103% | 100% | -0.17% | -0.17% | -0.12% | 102% | 101% |
| Class E | -0.36% | -0.17% | -0.41% | 102% | 99% | -0.52% | -0.63% | -1.04% | 102% | 100% |
| Overall | -0.13% | 0.03% | 0.11% | 102% | 100% | -0.27% | -0.21% | -0.19% | 102% | 101% |
| Class D | -0.11% | 0.19% | -0.30% | 103% | 101% | -0.28% | 0.37% | -0.16% | 102% | 101% |
| Class F (optional) | -0.15% | -0.10% | 0.31% | 104% | 101% | -0.08% | 0.50% | 0.08% | 102% | 100% |

Table 4. CE4.2.8.D: Double ATMVP candidates with uniformity check and decoder-side speed-up.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 w/o ATMVP |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.84% | -0.78% | -0.78% | 104% | 118% | -0.60% | -0.66% | -0.53% | 102% | 105% |
| Class A2 | -1.08% | -1.01% | -1.00% | 105% | 124% | -0.63% | -0.45% | -0.39% | 103% | 106% |
| Class B | -0.89% | -0.74% | -0.66% | 106% | 113% | -0.61% | -0.49% | -0.54% | 102% | 106% |
| Class C | -1.20% | -1.16% | -1.21% | 105% | 118% | -1.20% | -1.06% | -0.98% | 101% | 108% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.00% | -0.92% | -0.90% | 105% | 118% | -0.77% | -0.67% | -0.62% | 102% | 106% |
| Class D | -1.30% | -0.93% | -1.16% | 106% | 122% | -1.35% | -1.00% | -0.99% | 102% | 108% |
| Class F (optional) | -0.38% | -0.42% | -0.48% | 104% | 106% | -0.33% | -0.40% | -0.43% | 102% | 102% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 w/o ATMVP |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.63% | -0.70% | -0.51% | 107% | 109% | -0.57% | -0.47% | -0.55% | 102% | 104% |
| Class C | -0.95% | -1.36% | -1.23% | 104% | 109% | -0.95% | -1.04% | -1.00% | 101% | 106% |
| Class E | -2.42% | -2.54% | -2.81% | 101% | 109% | -2.40% | -3.22% | -3.69% | 99% | 107% |
| Overall | -1.19% | -1.38% | -1.32% | 104% | 109% | -1.15% | -1.35% | -1.48% | 101% | 105% |
| Class D | -1.05% | -1.13% | -1.32% | 105% | 108% | -1.30% | -0.97% | -1.16% | 100% | 106% |
| Class F (optional) | -0.76% | -1.03% | -0.95% | 105% | 104% | -0.39% | -0.84% | -0.95% | 102% | 103% |

# Conclusion

The simulation results of Core Experiment 4.2.8 (CE4.2.8), including affine merge candidate, more spatial and temporal candidates, pairwise-average candidates as replacement of combined candidates from HEVC, and modifications of ATMVP are summarized in this contribution. Affine mode with integrating affine merge candidates into one single merge list can achieve -3.36%, -2.23%, -2.12%, and -2.21%, luma BD-rates for VTM-1.0-RA, VTM-1.0-LB, BMS-1.0-RA, and BMS-1.0-LB, respectively. Adding more spatial and temporal candidates can provide -0.29% -0.39%, -0.09%, and -0.10% luma BD-rates for VTM-1.0-RA, VTM-1.0-LB, BMS-1.0-RA, and BMS-1.0-LB, respectively. The encoding time increases are 3% and 4% for VTM-1.0 and BMS-1.0, respectively, and no decoding time increases are observed. Using pairwise-average candidates to replace combined candidates from HEVC can achieve -0.41%, -0.13%, -0.51%, and -0.27% luma BD-rates with 3%, 2%, 3% and 2% encoding time increases for VTM-1.0-RA, VTM-1.0-LB, BMS-1.0-RA, and BMS-1.0-LB, respectively. The decoding time increases are less than 1%. ATMVP with adding one additional ATMVP candidate and performing redundancy check can bring -1.00%, -1.19%, -0.77%, and -1.15% luma BD-rate gains, for VTM-1.0-RA, VTM-1.0-LB, BMS-1.0-RA, and BMS-1.0-LB, respectively. The encoding time increases are 5%, 4%, 2%, and 1% for VTM-1.0-RA, VTM-1.0-LB, BMS-1.0-RA, and BMS-1.0-LB, respectively. The decoding time increases are 18%, 9%, 6%, and 5% for VTM-1.0-RA, VTM-1.0-LB, BMS-1.0-RA, and BMS-1.0-LB, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei, “Description of SDR video coding technology proposal by MediaTek,” Document of Joint Video Experts Team, JVET-J0018.

- H. Yang, S. Liu, K. Zhangi, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” Document of Joint Video Experts Team, JVET-J1024.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-J1010.