| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-Kxxxx-v1 |
| --- | --- |

| Title: | CE6.2 Secondary Transformation: NSST Signaling (test 6.2.1.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Fabrice Urban, Fabrice Le Léannec, Edouard Francois | Tel: Email: | firstname.lastname@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the results of conditional NSST signaling, depending on the AMT transform.

# Introduction

The non-separable secondary transform (NSST) is the same as in JEM7, including the JEM NSST signaling.

# Description and encoder options

The following modifications are proposed:

- To lower the complexity, the restriction is imposed on AMT and NSST use that NSST can be applied and signaled only when AMT transform index is equal to 0.

- To lower the complexity even further, NSST can be applied and signaled only when AMT flag is equal to 0.

# Results

As this proposition applies only when AMT and NSST are available, the results are only proposed for BMS

## restriction is imposed on AMT and NSST use that NSST can be applied and signaled only when AMT transform index is equal to 0, (AMT flag ON)

### VTM

Ref1: VTM + EMT

Ref 2: VTM + EMT + NSST

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 + EMT |  |  |  |  | Over VTM 1.1 + EMT + NSST |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.52% | -5.02% | -5.45% | 231% | 96% | 0.24% | 0.01% | 0.14% | 87% | 100% |
| Class A2 | -1.15% | -3.33% | -3.43% | 239% | 98% | 0.34% | 0.13% | 0.03% | 84% | 100% |
| Class B | -1.30% | -3.37% | -3.24% | 236% | 99% | 0.26% | 0.11% | 0.10% | 82% | 99% |
| Class C | -2.06% | -2.86% | -3.05% | 244% | 100% | 0.31% | -0.27% | -0.03% | 80% | 99% |
| Class E | -2.03% | -2.69% | -2.79% | 219% | 99% | 0.28% | -0.09% | -0.09% | 83% | 99% |
| Overall | -1.60% | -3.41% | -3.52% | 234% | 99% | 0.28% | -0.02% | 0.03% | 83% | 99% |
| Class D | -2.22% | -2.47% | -2.48% | 248% | 100% | 0.33% | -0.27% | -0.29% | 77% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 + EMT |  |  |  |  | Over VTM 1.1 + EMT + NSST |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.82% | -4.63% | -4.80% | 131% | 93% | 0.11% | 0.10% | 0.24% | 84% | 97% |
| Class A2 | -0.53% | -2.32% | -2.31% | 123% | 98% | 0.05% | 0.24% | 0.11% | 86% | 99% |
| Class B | -0.69% | -3.26% | -3.03% | 128% | 100% | 0.12% | 0.41% | 0.22% | 85% | 101% |
| Class C | -0.91% | -2.05% | -2.27% | 130% | 103% | 0.17% | 0.18% | 0.07% | 86% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.74% | -3.02% | -3.04% | 128% | 99% | 0.12% | 0.25% | 0.16% | 85% | 100% |
| Class D | -0.84% | -1.64% | -1.30% | 132% | 101% | 0.16% | 0.23% | 0.09% | 86% | 100% |

### BMS

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS |  |  |  |  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.26% | 0.06% | 0.15% | 87% | 98% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | 0.34% | 0.08% | 0.02% | 85% | 97% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.23% | 0.09% | 0.03% | 84% | 99% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.24% | -0.05% | 0.08% | 81% | 99% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class E | 0.16% | -0.13% | -0.09% | 85% | 99% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | 0.25% | 0.02% | 0.04% | 84% | 98% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.25% | -0.11% | -0.12% | 79% | 99% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS |  |  |  |  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.27% | 0.13% | 89% | 100% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | 0.07% | 0.15% | 0.06% | 91% | 100% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.08% | 0.34% | 0.25% | 90% | 99% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.20% | 0.24% | -0.07% | 89% | 100% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | 0.11% | 0.26% | 0.10% | 90% | 100% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.13% | 0.02% | 0.07% | 89% | 99% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

## restriction is imposed on AMT and NSST use that NSST can be applied and signaled only when AMT flag is equal to 0

### VTM

Ref1: VTM + EMT

Ref 2: VTM + EMT + NSST

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 + EMT |  |  |  |  | Over VTM 1.1 + EMT + NSST |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.31% | -3.82% | -4.32% | 170% | 94% | 0.46% | 1.30% | 1.34% | 64% | 98% |
| Class A2 | -0.87% | -2.52% | -2.57% | 174% | 95% | 0.62% | 0.98% | 0.92% | 61% | 97% |
| Class B | -0.95% | -2.29% | -2.24% | 173% | 98% | 0.61% | 1.24% | 1.13% | 60% | 98% |
| Class C | -1.59% | -2.28% | -2.53% | 176% | 99% | 0.80% | 0.33% | 0.50% | 58% | 98% |
| Class E | -1.36% | -1.83% | -1.77% | 166% | 99% | 0.96% | 0.80% | 0.97% | 63% | 99% |
| Overall | -1.21% | -2.50% | -2.63% | 172% | 97% | 0.69% | 0.93% | 0.96% | 61% | 98% |
| Class D | -1.66% | -1.81% | -1.99% | 179% | 99% | 0.90% | 0.41% | 0.23% | 56% | 99% |

### BMS

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS |  |  |  |  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.48% | 0.83% | 1.01% | 64% | 99% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | 0.56% | 0.72% | 0.84% | 62% | 94% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.49% | 0.83% | 0.73% | 62% | 98% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.60% | 0.32% | 0.51% | 59% | 98% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class E | 0.70% | 0.54% | 0.55% | 65% | 98% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | 0.56% | 0.65% | 0.72% | 62% | 97% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.66% | 0.25% | 0.40% | 57% | 99% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS |  |  |  |  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.23% | 0.66% | 0.85% | 78% | 100% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | 0.13% | 0.61% | 0.67% | 83% | 100% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.22% | 1.00% | 0.82% | 81% | 99% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.38% | 0.61% | 0.36% | 79% | 99% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | 0.25% | 0.75% | 0.67% | 81% | 99% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.34% | 0.53% | 0.68% | 79% | 98% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusion

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).