| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0280-v1 |
| --- | --- |

| Title: | CE1: QTBTS partitioning and boundary handling (1.0.5, 1.0.6, 1.0.7, 1.0.8, 2.0.7) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jackie Ma Adam Wieckowski Heiko Schwarz Detlev Marpe Thomas Wiegand | Email: | jackie.ma@hhi.fraunhofer.de adam.wieckowski@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the results of SubCE1.0.5-1.0.8 and its combination with SubCE2.0.7. The combinations of these two tools have the following overall BD-rate gains over VTM:

# Introduction

The experiments SubCE1.0.5-1.0.8 in [1] are on QTBTS a partitioning method originally proposed in [2]. QTBTS extends the quad split and the binary 1/2 split by the following binary splits: 1/4, 3/4, 3/8, 5/8, 1/3, 2/3, 1/5, 2/5, 3/5, and 4/5.

SubCE1.0.7 allows the use of all splits available in QTBTS. SubCE1.0.8 use the same split modes, but the splitting mechanism is forced to result in a final picture partitioning that does not contain non-power of 2 block sizes.

SubCE1.0.5 restricts the set of splits onto a subset of the general split modes by prohibiting the binary split modes 3/8, 5/8, 1/5, 2/5, 3/5, and 4/5. SubCE1.0.6 is the version of SubCE1.0.5 that contains the splitting mechanism that return a final picture partition that has only blocks with block sizes that are integer powers of 2.

# Partitioning

## Test 1.0.5, 1.0.6, 1.0.7, 1.0.8

Test 1.0.5 – 1.0.8 represent different complexities of the QTBTS partitioner. The characteristics of each test are summarized in Table 1. The configurations are detailed in Table 4.

Table 1: Characteristics of SubCE1.0.5-1.0.8

|  | Test 1.0.5 | Test 1.0.6 | Test 1.0.7 | Test 1.0.8 |
| --- | --- | --- | --- | --- |
| 1/4 and 3/4 splits | YES | YES | YES | YES |
| 3/8 and 5/8 splits | NO | NO | YES | YES |
| Non-power of 2 blocks | YES | NO | YES | NO |
| Configuration name | 3-3 | 2-3 | 5-3 | 4-3 |

## Additional information

Further results are reported with different configurations for the Test 1.0.6 and 1.0.8. Both configurations do not use non-power of 2 blocks. Different complexities can be obtained by decreasing or increasing, respectively, the maximum binary tree depth. In Table 2 the additional tests and the characteristics of them are shown and the detailed configurations are shown in Table 4.

Table 2: Table characteristics of additional tests

|  | Test 1.0.6_BTD2 | Test 1.0.6_BTD4 | Test 1.0.8_BTD4 |
| --- | --- | --- | --- |
| 1/4 and 3/4 splits | YES | YES | YES |
| 3/8 and 5/8 splits | NO | NO | YES |
| Non-power of 2 blocks | NO | NO | NO |
| Configuration name | 2-2 | 2-4 | 4-4 |

# Picture boundary handling

## Test 2.0.7

The boundary handling in [2] extends the HEVC boundary handling by a binary 1/2 split if only two corners of the current block extent the picture boundary. This technology is added on top of Test 1.0.6 in SubCE2.0.7 in CE1.

## Additional information

As an additional information we report results for the boundary handling tested in Test 2.0.7 on top of  Test 1.0.6_BTD2, Test 1.0.6_BTD4, and Test 1.0.8_BTD4.

Table 3: Overview of  tests results that are reported with the picture boundary handling

|  | Test 1.0.6_BTD2 | Test 1.0.6 | Test 1.0.6_BTD4 | Test 1.0.8 |
| --- | --- | --- | --- | --- |
| 1/4 and 3/4 splits | YES | YES | YES | YES |
| 3/8 and 5/8 splits | NO | NO | NO | YES |
| Non-power of 2 blocks | NO | NO | NO | NO |
| Configuration name | 2-2-edge | 2-3-edge | 2-4-edge | 4-3-edge |

# Software and settings

All of the tests described in this document were performed using the same software basis, which can be found in the following repository:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE1/CE1-1.0.6

All tests described in this document except for the tests 2.0.7 [XX] were performed without further code adaptations and the uploaded implementations are exact copies of the software for the test 1.0.6. For the test 2.0.7 the value of the macro setting HM_QTBT_ONLY_QT_IMPLICIT was set to ‘0’. This software can be found in the following repository:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE1/CE1-2.0.7

The implementation of the partitioning and the required logic is marked with the macro USE_GBS. Additional logic required by the partitioner, but necessarily related to the partitioning (e.g. non-power of two block and transformation handling) itself is marked with additional macro settings dependent on the usage of the USE_GBS macro.

Some changes to the code were performed, which do not have any impact on the codec functionality. Most notably, the affine motion vector difference information was moved from MotionInfo to the PredictionUnit type. Such cosmetic changes are not marked with a macro. When USE_GBS macro is disabled, the generated results equal those of VTM and BMS.

The QTBTS partitioner is implemented as a separate partitioner implementation independent of the available HEVC and QTBT+MTT partitioners. Also, due to different semantics, a separate encoder controller is implemented. For all not-split logic, the functionality of the QTBT+MTT controller was copied from the existing implementation.

To enable the QTBTS partitioner following options have to applied on top of the default VTM and BMS configuration:

QTBT:  0 # disable QTBT and MTT
MTT:    0
SaveLoadEncInfo: 0 # disable QTBT specific speed-ups
GenBinSplit:   1 # enable QTBTS
GBSFast:       1 # enable fast search for QTBTS
GBSAllowFourths: 1 # enable ¼ and ¾ splits for QTBTS
MaxAsymTSize:  64 # set maximal blocks size for asymmetric splits to 64
MaxAsymTSizeI: 64 
AnisoTVTh: 1.1 # set threshold for gradient based speed-ups

The parameters varying between tests are described in the tables below.

Table 4: Parameter configration values for Test 1.0.5-1.0.8 (without boundary handling, software from CE1-1.0.6)

| Parameter name | 2-2 | 2-3 | 2-4 | 4-3 | 3-3 | 5-3 |
| --- | --- | --- | --- | --- | --- | --- |
| GBSAllowEights | 0 | 0 | 0 | 1 | 0 | 1 |
| GBSNonLog2CUs | 0 | 0 | 0 | 0 | 1 | 1 |
| GBSForceSplitToLog2 | 1 | 1 | 1 | 1 | 0 | 0 |
| MaxBTDepth | 2 | 3 | 4 | 3 | 3 | 3 |
| MaxBTDepthI MaxBTDepthISliceL | 2 | 3 | 4 | 3 | 3 | 3 |

Table 5: Parameter confiration values for Test 2.0.7 (with boundary handling, software from CE1-2.0.7)

| Parameter name | 2-2-edge | 2-3-edge | 2-4-edge | 4-3-edge |
| --- | --- | --- | --- | --- |
| GBSAllowEights | 0 | 0 | 0 | 1 |
| GBSNonLog2CUs | 0 | 0 | 0 | 0 |
| GBSForceSplitToLog2 | 1 | 1 | 1 | 1 |
| MaxBTDepth | 2 | 3 | 4 | 3 |
| MaxBTDepthI MaxBTDepthISliceL | 2 | 3 | 4 | 3 |

# Results

In this section we report the overall gains over VTM, i.e. BMS software with VTM config, and BMS, i.e. BMS software with BMS config.

## CTC results for partitioning only

Figure 1: Overall results over VTM for partitioning only

Figure 2: Overall results over BMS for partitioning only

## CTC results for partitioning with boundary handling

Figure 3: Overall results over VTM with boundary handling

Figure 4: Overall results over BMS with boundary handling

Figure 5: Visualization of trade-offs for overall luma gains over VTM for partitioning only and with boundary handling.

# Conclusion

The QTBTS partitioner covers a wide range of trade-offs that can be obtained by different config parameters that determine the splitting topology. In medium configuration -0.18% gain in luma are achieved over VTM for 76% encoding time. By additionally using a picture boundary handling the same configuration obtains -0.52% gain in luma over VTM for 79% encoding time. Further, -1.58% gain in luma can be obtained for 143% encoding runtime without using non-power of 2 block sizes. Hence, we propose to adopt QTBTS and investigate this technique further with respect to its trade-off of gains vs encoder runtime.

# References

- J. Ma, F. Le Léannec, M. W. Park, “Description of Core Experiment 1 (CE1): Partitioning,” JVET-J1021, Apr. 2018, San Diego.

- J. Ma, A. Wieckowski, V. George, T. Hinz, J. Brandenburg, S. de Luxan Hernandez, H. Kirchhoffer, R. Skupin, H. Schwarz, D. Marpe, T. Schierl, T. Wiegand, “Quadtree plus binarytree with shifting,“ JVET-J0035, Apr. 2018, San Diego.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).