| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0294-v1 |
| --- | --- |

| Title: | Tencent test sequences, and Class F test set restructure |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Ye, Stephan Wenger, Xiang Li, Shan Liu, Lori Wu, Chemin Xie, Keane Liu, Berton Wang, Ping Liu, Kane Dong, Ying Kuang, Weizhong Feng Tencent America LLC 661 Bryant St, Palo Alto, CA 94301 | Tel: Email: | +1 650-798-3300 jingye@tencent.com swenger@tencent.com xlxiang@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Four test sequences are presented to JVET for inclusion into the test set.  All are in 1080p YUV 4:2:0 resolution.  Three of the four sequences are captured screen content of eSport/gaming applications and appropriate for the Class F test set.  One sequence is part of a music video and offered for the class B test set.  All sequences are characterized by objective and subjective test results.  The copyright holder offers the sequences free of charge and under a copyright license claimed to be suitable for use in standardization projects.

The proponents point out that the current Class F test set is outdated and does not reflect today’s screen content in both resolution and frame rate.  Therefore, it is proposed to replace the current class F sequences with those of the SCC 4:2:0 TGM test set.  The proponents further observe that eSport/gaming content is currently underrepresented.  Accordingly, it is further proposed to include one eSport/gaming sequences to the Class F test set, either by adding or perhaps by replacing one of the SCC4:2:0 TGM sequences with the new material.

# Introduction

Music videos have been popular on TV since at least the 1980s, and the streaming of eSports/gaming content has seen dramatic rise in recent years.  Both forms of content are not present in the current test set.  Accordingly, we recommend to reflect these developments in the JVET test conditions.

In the Gwangju meeting (January 2018), experts expressed a desire for the inclusion of concert and gaming content into the test set.  In San Diego (April 2018), Tencent presented JVET-J0052 [1], offering 10 sequences at various HD resolutions and 25 Hz /30 Hz frame rates.  One observation that was made was that the sequences were in HD format, but with only 25Hz or 30 Hz frame rate, which was claimed to be too low.  No change of the test set was made in San Diego; instead, the study of the sequences was assigned to an ad hoc group.

Since the San Diego meeting, Tencent has obtained the rights and content of the four sequences proposed now.  Three of these sequences are in 1080p60 YUV 4:2:0 format.  The concert sequence is in 1080p25 format as that is the maximum frame rate obtainable from the RAW format source.  All four sequences use BT.709 color primaries and are 10 seconds in length.

The set of four proposed sequences are part of a much larger set that have been subject to an evaluation process by an evaluation house experienced in video coding standard evaluation work and retained by Tencent, during which sequences less suitable for subjective and/or objective quality testing were eliminated.  Further, in order to ease the work load of JVET, prior work has been carried out by the same evaluation house, including a subjective characterization of the test sequences with results presented below in section 5.  We also include objective test results.  For both objective and subjective tests, results are based on the use of HM16.16 and BMS_rc17.0.

# Sequence description

The four sequences are in 1080p resolution and in YUV 4:2:0 format.  All sequences have a length of 10 seconds, and BT.709 color primaries.

| Sequence | Source | Frame rate | Number of frames |
| --- | --- | --- | --- |
| Tencent_AOV5_1920x1080_60_8bit_420.yuv | Screen capture AOV | 60 fps | 600 |
| Tencent_AOV6_1920x1080_60_8bit_420.yuv | Screen capture AOV | 60 fps | 600 |
| Tencent_AOV7_1920x1080_60_8bit_420.yuv | Screen capture AOV | 60 fps | 600 |
| Tencent_Concert1_1920x1080_25_8bit_420.yuv | Atomos Shogun Studio | 25 fps | 250 |

Table 1 – Test set

All sequences have in common that they contain challenging video characteristics for codecs, including:

- camera panning,

- Frequently light changing,

- challenging texture structures,

- saturated colour,

- large contrast/dynamic range.

| Tencent_AOV5_1920x1080_60_8bit_420.yuv |  |
| --- | --- |
| Tencent_AOV6_1920x1080_60_8bit_420.yuv |  |
| Tencent_AOV7_1920x1080_60_8bit_420.yuv |  |
| Tencent_Concert1_1920x1080_25_8bit_420.yuv |  |

# Conversion process

The one concert sequence was captured by Atomos Shogun Studio as a 115Mb/s MOV file. FFMPEG [2] was used to generate the YUV from the MOV files. The command line is as below:

ffmpeg -i input.mov -c:v rawvideo -pix_fmt yuv422p -deinterlace output.yuv

Then the YUV 4:2:2 has been converted into YUV 4:2:0 by FFMPEG, and the command line is as below:

ffmpeg.exe -y -pix_fmt yuv422p -s 1920x1080 -r 25 -i 1520014897673_ yuv422.yuv -f rawvideo -pix_fmt yuv420p -s 1920x1080 -r 25 output_420.yuv

The three 1920x1080 eSports sequences were captured by monitor/phone/tablet screen with OBS studio [3] that can capture the YUV information via the GPU and store the YUV in the local HDD. The captured YUV is 4:4:4 format, and it is converted into 4:2:0 format by the HDRtools [4].

# Objective Evaluation

RD performance of those test sequences are evaluated by HM16.16 [5] and BMS_rc17.0[6] platform. They have been encoded using RA and LDB configurations. For each sequence and configuration, two set of QP are used. QP values 25-35-42-47 have been tested as set 1. QP values 27-37-45-50 have been tested as set 2. We have used the configuration files provided on Aachen server. The test results are in Table 1 and Table 2.

The intra period in RA configuration has been set to:

- 32 for 25 fps content, and

- 64 for 60 fps content

Table 1. BMS and HM BD rate of the proposed test sequences using QP set 1

Table 2. BMS and HM BD rate of the proposed test sequences using QP set 2

The RD curves of the sequences are depicted in Fig. 1 to Fig 4.

Fig. 1 Rate-Distortion curves of proposed sequence (RA and LDB QP set1)

Fig. 2 Rate-Distortion curves of proposed sequence (RA and LDB QP set1)

Fig. 3 Rate-Distortion curves of proposed sequence (RA and LDB QP set1)

Fig. 4 Rate-Distortion curves of proposed sequences (RA and LDB QP set1)

For QP set 2, the RD curve can be found in the excel sheet. We didn’t list all the RD curves here.

- Subjective Characterization

Subjective characterization was performed using ten non-expert test subjects under professional viewing conditions and professional supervision.

As an “anchor”, the well-known Basket sequence, coded using HM, was included in the testing session.  The new sequences were encoded at various rate points (QPs) as shown in the graphs, using HM as well as BMS in both LD and RA configurations.  Details of the encoding and decoding process were already described above.

The test sequences show a behavior that makes them, in our view, suitable for subjective testing.  In particular, there are no critical outliers in the rate / subjective quality relationship, and the (comparatively narrow) confidence intervals show that the confidence of the evaluator team is reasonably high—which we think are two key characteristics for useful sequences.

For those interested in the details of the evaluation results, we include the xls of the evaluation as received by the testing house.  (Included therein is also data related to one other sequence (S11) that is not further discussed herein).

Below the subjective results.

## Basket Anchor Sequence

## Sequence Tencent_AOV5_1920x1080_60_8bit

## Sequence Tencent_AOV6_1920x1080_60_8bit

## Sequence Tencent_AOV7_1920x1080_60_8bit

## Sequence Tencent_Concert1_1920x1080_25_8bit_420

# Proposals

## Replace current Class F sequences with the SCC 4:2:0 TGM Sequences

The current Class F sequences have been used by the community for a number of years, however are now becoming outdated in that they neither have the resolutions, nor the frame rates current systems use.

| Category | Resolution | Sequence name | Fps |
| --- | --- | --- | --- |
| Class F | 832x480 1024x768 1280x720 1280x720 | BasketballDrillText ChinaSpeed SlideEditing SlideShow | 50 30 30 20 |

We propose to replace those sequences in their entirety with the SCC 4:2:0 sequences, in particular:

| Category | Resolution | Sequence name | Fps |
| --- | --- | --- | --- |
| SCC 4:2:0 TGM | 1920x1080 1920x1080 1920x1080 1920x1080 | FlyingGraphics* Desktop Console ChineseEditing | 60 60 60 60 |

The proposal acknowledges that a simple extension of the existing test set creates additional work on the already high workload, and therefore we propose the removal of the outdated sequences.  We also acknowledge that replacing arguably outdated 720p30 (or lower) sequences with a 1080p60 sequences slightly increases the workload for class F.  Note, however, that class F is currently optional for testing and, more importantly, the replaced sequences are in a format that, today, has little or no practical applications anymore.

## Add one or more of the Tencent eSports/Gaming sequences to class F

As already discussed, eSport/gaming sequences are not present in the current test set.  We suggest that it would enrich the class F test set to add at least one and perhaps two gaming sequences to it.  As the current SCC 4:2:0 TGM sequences bear certain similarities, it may be possible, rather than adding sequences, to replace one of the SCC 4:2:0 TGM sequences with one of the proposed eSports/gaming sequences.  In particular, we suggest to include AOV5 as a typical representatives of currently widely distributed gaming screen content. Once again, in order to limit the JEVET experimentation work, the new sequences can replace two SCC 4:2:0 TGM sequences for forming the new class F, e.g.:

| Category | Resolution | Sequence name | Fps |
| --- | --- | --- | --- |
| (New) Class F | 1920x1080 1920x1080 1920x1080 1920x1080 | FlyingGraphics* Desktop Console AOV5 | 60 60 60 60 |

- Copyright information

The aforementioned video sequences are made available under the following copyright disclaimer.  The terms of the disclaimer are the same as in the one used in JVET-J0052 [1]; editorial changes were made with respect to the company names of the rightsholders.

Copyright © 1998 - 2018 Tencent. All Rights Reserved.

This sequence and all intellectual property rights therein remain the property of Shenzhen Tencent Computer System Co. Ltd. and Tencent America LLC. This sequence may only be used for the purpose of developing, testing and promulgating technology standards (the Purpose). Shenzhen Tencent Computer System Co. Ltd. and Tencent America LLC. make no warranties with respect to this sequence and expressly disclaim any warranties regarding its fitness for any purpose. This disclaimer shall remain attached to this sequence except when this sequence is used for the Purpose.

- References

[1] JVET-J0052

- [2] https://www.ffmpeg.org/

[3] https://obsproject.com/

[4] HDRTools0.16, https://gitlab.com/standards/HDRTools/tags/v0.16

[5] https://vceg.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6

[6] https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0rc1

The evaluation house performed sequence selection and evaluation work on the condition of anonymity.

Arena of Valor (AOV) ©Tencent Games