| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0319-v1 |
| --- | --- |

| Title: | CE7-Related: TCQ with High Throughput Coefficient Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong, Muhammed Coban, Marta Karczewicz | Tel: Email: | +1-408-533-9515 jiedong@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Trellis coded quantization (TCQ) together with a coupled coefficient coding scheme was proposed in [1] . It may degrade CABAC throughput compared with the coefficient coding in HEVC or the current JEM design. To improve the throughput and maintain most of the gain reported in CE7 [2] , this contribution proposes a modified TCQ with a coupled coefficient coding scheme.

# Introduction

Trellis coded quantization (TCQ) was proposed in [1] , where two scalar quantizers are switchably used for quantization. The scalar quantizer used for quantizing a current transform coefficient is determined by the parity (the least significant bit) of the quantized coefficient that precedes the current transform coefficient in the scanning order.

A coefficient coding scheme coupled with TCQ was also proposed in [1] , by which the context modeling for a quantized coefficient depends on the quantizer it used. Specifically, each of the significance flag (SIG) and the greater than one flag (gt1) has two sets of context models, and the set selected for a particular SIG or gt1 depends on the quantizer used for the associated coefficient. Therefore, the coefficient coding proposed in [1]  requires full reconstruction of the absolution level (absLevel) of the quantized coefficient before moving to the next scanning position, in order to know the parity, which is used to determine the quantizer and thus the context sets for the next coefficient. This is one of the main differences from HM/JEM coefficient coding.

The other main difference is that the bins of SIG and greater than x flags (gtx, x = 1, 2, 3, and 4) are regular coded and the sets of context models include two sets for SIG, two sets for gt1, and a separate set shared by gt2, gt3, and gt4.

# Problem Statement

The new design of the coefficient coding in [1]  may degrade the throughput compared with the design in HEVC or JEM, analyzed as below.

- Regular coded bins are increased. Regular coded bins are processed much slower than bypass coded bins, due to context selection and interval subdivision calculation. With the coefficient coding in [1] , the number of regular coded bins in a CG is up to 80, compared with 25 in JEM (16 for SIG, 8 for gt1, and 1 for gt2).

- Bypass coded bins are not grouped. Grouping bypass bins together into longer chains increases the number of bins processed per cycle and thus reduces the number of cycles required to process a single bypass bin. However, with the coefficient coding in [1] , all the bypass coded bins in a CG are not grouped (see Fig. 1); instead, they are interleaved with regular coded bins per coefficient.

- Regular coded bins with different sets of contexts are interleaved. When bins that use the same logic for context selection are grouped together, fewer speculative calculations are needed to process multiple bins per cycle. With the coefficient coding in [1] , SIG, gt1, and gtx (x=2, 3, 4), which use different sets of contexts, are interleaved per coefficient (as illustrated in Fig. 1).

Fig. 1 Order of coefficient bins in a CG, as proposed in [1]

Besides the impact on throughput, memory requirement for storing contexts is increased. Totally 156 contexts (60 for SIG, 64 for gt1, and 32 for gt2/gt3/gt4) are used in [1] , compared with 74 contexts (44 for SIG, 24 for gt1, and 6 for gt2) in HEVC.

# Proposed TCQ and Coefficient Coding

The goal of this contribution is to achieve the same level of throughput as the coefficient coding design in HM and JEM, while maintaining most of the gain provided by TCQ [2] .

## TCQ with SIG-based state transition

The parity-based state transition as proposed in [1]  is replaced by SIG-based state transition in this contribution, as shown in Fig. 2, while other TCQ design in [1]  remains unchanged. By doing this, the scalar quantizer used for quantizing a current transform coefficient is determined by the SIG of the quantized coefficient that precedes the current transform coefficient in the scanning order.

|  | current state; next state for …. sig == 0; sig==1. 0; 0; 2. 1; 2; 0. 2; 1; 3. 3; 3; 1. |
| --- | --- |

Fig. 2 SIG-based state transition

## Coefficient coding

The coefficient coding in this contribution is basically JEM coefficient coding. The difference is that each of the SIG and gt1 has two sets of context models, and the entropy coder selects the context set for a particular SIG or gt1, according to the quantizer the associated coefficient used. Since the entropy coder knows the previous SIG and thus the current quantizer, the right context set can be selected without reconstructing the previous absLevel. Therefore, changing the state transition of TCQ from parity-based to SIG-based enables the similar high throughput design to JEM coefficient coding. The proposed order of coefficient bins in a CG is shown in Fig. 3.

Fig. 3 Proposed order of coefficient bins in a CG

Other minor changes include (1) gt2, which shares the same set of context models with gt1 in JEM, also shares the two sets of context models of gt1 in this contribution and switches the set in the same way as gt1 does; (2) the context modeling for SIG does not consider the TU size, and therefore the number of contexts per set is reduced from 54 to 18.

In short, with the proposed coefficient coding, the regular coded bins are up to 25 per CG, which remains the same as HM and JEM coefficient coding; all the bypass coded bins in a CG are grouped together; the regular coded bins with the same set of contexts are grouped together. Regarding the memory requirement, totally 104 contexts (60 for SIG and 44 for gt1/gt2) are used, which are 2/3 of the contexts used in [1] .

# Experimental Results

# Reference

- “Description of SDR, HDR and 360° video coding technology proposal by Fraunhofer HHI”, JVET-J0014, San Diego, USA, Apr. 10-20, 2018.

- “CE7 summary report on quantization and coefficient coding”, JVET-K0027, Ljubljana, SI, Jul. 10-18, 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).