| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0361-v1 |
| --- | --- |

| Title: | CE9-related: Harmonization between CE9.2.6 (DMVR with Template-free Bilateral Matching, JVET-K0359) and CE9.2.9 (DMVR with Bilateral Matching, JVET-K0217) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen, Wei-Jung Chien, Marta Karczewicz (Qualcomm) Semih Esenlik, Ivan Krasnov, Zhijie Zhao, Ma Xiang, Haitao Yang, Jianle Chen (Huawei) | Tel: Email: | +886-3-566-1980 {chunchic,wchien,martak}@qti.qualcomm.com {semih.esenlik, jianle.chen}@huawei.com |
| Source: | Qualcomm Inc. and Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

This document proposes a harmonization method of CE9.2.6 (DMVR with Template-free Bilateral Matching, JVET-K0359) and CE9.2.9 (DMVR with Bilateral Matching, JVET-K0217). The followings are applied to the achieve the harmonized design: 1.) bilateral matching based on MVD mirroring without scaling; 2.) adaptive search pattern; 3.) MRSAD as the cost function; 4.) refined MVs not for spatial MV prediction; 5.) 2-tap bilinear filter for interpolating search range; 6.) early-skip condition based on MVD between merge candidates. When compared with the VTM anchor (i.e. BMS-1.0 with VTM cfg.) of CE9, the proposed method achieves an average Y BD-rate saving of x.x%, with xx% and xx% increase in encoding and decoding time, respectively. When compared with the BMS anchor (i.e. BMS-1.0 with BMS cfg.), it shows an average Y BD-rate saving of x.x%, with xx% and xx% increase in encoding and decoding time, respectively.

# Introduction

In the context of CE 9 [1], two tests, CE 9.2.6 [2] and 9.2.9 [3], were conducted to modify Decoder-side Motion Vector Refinement (DMVR) in BMS-1.0 [4]. There are some common grounds shared between them. They both applies a bilateral matching method which takes a mirroring MVD pair (i.e. (dx,dy) and (-dx,-dy)) without POC-distance-based scaling to refine the original bi-prediction MVs of merge blocks. Refined MVs can be used for spatial MV prediction, temporal MV prediction and boundary strength calculation in deblocking filter. The 8-tap DCTIF is used to perform motion compensation based on the refined MVs. Differences between them include search pattern, cost function, interpolation used for search, number of search iterations and early-skip method.

In CE9.2.6, a 9-point square search is applied for integer-pel search (w/ 26 iterations) and half-pel search (w/ 1 iteration). Its cost function is a block-adaptive MRSAD value plus 4 times the 1-norm of MVD. A 2-tap bilinear filter is used to interpolate the samples on the search range. An early-skip condition based on the MVDs between merge candidates is imposed on top of DMVR.

In CE9.2.9, an adaptive 6-point square for integer-pel (w/ 2 iterations) and 4-point cross for half-pel (w/ 1 iteration) is applied. The cost function takes high-prediction MRSAD which computes the MRSAD value based on un-clipped 14-bit interpolated samples within the search range. The 8-tap DCTIF is used to interpolate the samples on the search range.

The aforementioned common features and differences of the two CE tests are summarized in Table 1. The harmonized solution proposed in this contribution will be described in the next section.

- Search configurations of CE 9.2.6 and CE 9.2.9.

|  | CE 9.2.6 | CE 9.2.9 |
| --- | --- | --- |
| Signaled initial MVs | Yes |  |
| Matching method | bilateral based on MVD mirroring without scaling |  |
| Search pattern | Fixed pattern (9-point square for int-pel and half-pel) | Adaptive pattern (adaptive 6-point square for int-pel, 4-point cross for half-pel) |
| Cost function | Block-adaptive MRSAD + motion cost | High-precision MRSAD |
| Refined MV for spatial MV prediction | Yes |  |
| Refined MV for temporal MV prediction | Yes |  |
| Refined MV for deblocking filter | Yes |  |
| Search range | [-2,2] w/ 2-tap bilinear filter | [-2,2] w/ 8-tap DCTIF |
| Search iteration | 26 for integer-pel, 1 for half-pel | 2 for integer-pel, 1 for half-pel |
| Motion compensation | 8-tap DCTIF |  |
| Additional early-skip condition | MVD between merge candidates | no |

# Technical Description

To achieve a harmonized design, this contribution takes the following search configurations to remove the difference between CE 9.2.6 and CE 9.2.9.

- Adaptive search pattern;

- MRSAD as the cost function;

- Original MVs for spatial MV prediction;

- Search range within [-2,2] interpolated by using a 2-tap bilinear filter;

- Two integer-pel search at most and one half-pel iterations;

- MVD between merge candidates as an extra early-skip condition.

The resulting search configurations are summarized in Table 2.

- Harmonized search configurations with harmonized items highlighted.

|  | Harmonization of CE 9.2.6 and CE 9.2.9 |
| --- | --- |
| Signaled initial MVs | Yes |
| Matching method | bilateral based on MVD mirroring without scaling |
| Search pattern | Adaptive pattern (adaptive 6-point square for int-pel, 4-point cross for half-pel) |
| Cost function | MRSAD (computed in internal bit-depth) |
| Refined MV for spatial MV prediction | no |
| Refined MV for temporal MV prediction | yes |
| Refined MV for deblocking filter | yes |
| Search range | [-2,2] w/ 2-tap bilinear filter |
| Search iteration | 2 for integer-pel, 1 for half-pel |
| Motion compensation | 8-tap DCTIF |
| Additional early-skip condition | MVD between merge candidates |

# Experimental Results

The experiments for the proposed harmonization method of CE 9.2.6 and CE 9.2.9 were conducted using the BMS-1.0 software [4] and the common test conditions [5] to collect BD-rate numbers, encoding-time ratio and decoding-time ratio. BMS and VTM anchors were generated using the same BMS-1.0 software but with BMS and VTM configurations, respectively. None of the original macros in the software was changed. Parallel encoding and decoding were applied to Random Access cases for anchors and tests. The search configurations of proposed harmonization method are recorded in Table 3. The performance results generated on top of the VTM and BMS anchors are summarized in Table 4.

- Search configurations of DMVR, CE 9.2.6, CE 9.2.9 and this contribution.

|  | Initial MV signaled | Sub-CU refinement | Neighboring recon. samples used | Max # of SAD calculation | Max. SR | Cost Function | Interpolation filter/tap no. | Search Patterns |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR | yes | no | no | 9 | 1 | SAD | DCTIF/8 | 9-point for int-pel search no half-pel search |
| CE9.2.6 | yes | no | no | less than 9 + 5 + 11*2 + 8 = 44 | 2 | >64: MRSAD ≤64: SAD | ME: Bilinear/2 MC: DCTIF/8 | 9-point for int-pel search 9-point for half-pel search |
| CE9.2.9 | yes | no | no | 6 + 3 + 4 =13 | 2 | Hi.-prec. MRSAD | DCTIF/8 | 6-point for int-pel search 4-point for half-pel search |
| Proposed | yes | no | no | 6 + 3 + 4 =13 | 2 | MRSAD | ME: Bilinear/2 MC: DCTIF/8 | 6-point for int-pel search 4-point for half-pel search |

- BD-rate results and processing time ratios relative to VTM and BMS anchors.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Proposed vs. VTM anchor |  |  |  |  | Proposed vs. BMS anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -1.82% | -1.94% | -1.95% | 106% | 125% |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -2.04% | -2.04% | -2.19% | 108% | 125% |  |  |  |  |  |

# Conclusion

The joint proposal is proposed to harmonize the coding tools in CE9.2.6: DMVR with Template-free Bilateral Matching (JVET-K0359) and CE9.2.9: DMVR with Bilateral Matching (JVET- K0217). We would like to recommend the committee to consider adoption in the next release of VTM, BMD and working draft.

# References

- S. Esenlik, Y. W. Chen and et. al., “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1029, Apr. 2018.

- C.-C. Chen, W.-J. Chien and M. Karczewicz, “CE9.2.5/9.2.6: DMVR with Template-free Bilateral Matching,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0359, July 2018.

- S. Esenlik, I. Krasnov, Z. Zhao, M. Xiang, Y. Haitao, J. Chen, “CE9.2.9: DMVR with Bilateral Matching,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0217, July 2018.

- BMS-1.0 software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- J. Boyce, K. Suehring, and et. al., “JVET Common Test Conditions and Software Reference Configurations,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1010, Apr. 2018.

# Patent rights declarations

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).