| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0386-v1 |
| --- | --- |

| Title: | CE2: Luma-adaptive deblocking filter (Test 2.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shimpei Nemoto Shunsuke Iwamura Atsuro Ichigaya | Tel: Email: | +81-3-5494-3354 nemoto.s-fy@nhk.or.jp iwamura.s-gc@nhk.or.jp ichigaya.a-go@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution reports test results of CE2-2.2.2 on luma-adaptive deblocking filter. The filter strength of the proposed deblocking filter is controlled based on the averaged luma level of the reconstructed samples. The proposed method is applied to an only brighter area, therefore it slightly impacts on averaging results. The results show that the proposed method achieves 0.00%, 0.01%, and 0.03% BD-Rate for AI, RA, and LDB compared to VTM-1.0 and 0.01%, 0.01%, and -0.04% BD-Rate for AI, RA, and LDB compared to BMS-1.0. In the case of BQSquare sequence which has typical brighter area, the proposed method achieves 0.05%, -2.59%, and -1.45% for each component, for LDB coding condition.

Proposed Method

The proposed deblocking filter applies variable filter strength depending on the level of reconstructed samples before in-loop filters. The distortion of decoded pictures is caused by a quantization process. The distortion is displayed through nonlinear process called Electro-Optical Transfer Function (EOTF) in the linear light domain. Additionally, the scene referred video system such as conventional SDR format and one of the HDR formats, Hybrid Log-Gamma (HLG), requires an additional nonlinear process called Opto-Optical Transfer Function (OOTF) also known as a system gamma. The displayed subjective distortion is enhanced by the nonlinear transfer function depending on the video signal level. Such mechanism makes block distortion visible at brighter areas, even if the quantization noise is uniformly distributed at all video level. Especially, for HDR formats, this enhanced distortion becomes salient due to its strong nonlinearity.

In HEVC, the filter strength of the deblocking filter is controlled by the variables β and tC which are derived from the averaged quantization parameters qPL. The proposed deblocking filter controls the strength of the deblocking filter by adding offset to qPL according to the luma level of the reconstructed samples.

The reconstructed luma level LL is derived as follow:

LL= ( ( p0,0 + p0,3 + q0,0 + q0,3 ) >> 2 ) / ( 1 << iBitDepth ),

where, the sample values pi,k and qi,k with i = 0..3 and k = 0 and 3 are derived as shown in Fig.1.

Fig.1 Sample position of pi,k and qi,k

The variable qPL is derived as follows:

qPL = ( ( QpQ + QpP +1 ) >> 1 ) + qpOffset,

where QpQ and QpP denote the quantization parameters of the coding units containing the sample q0,0 and p0,0, respectively. The offset qpOffset should be derived according to the transfer characteristics of the contents since the EOTF and OOTF vary among video formats. Therefore, the condition of qpOffset is defined for each transfer characteristics as shown in Table 1. VTM and BMS software currently do not support VUI parameters indicating the transfer characteristics of the contents, an additional flag “transfer_characteristics” is signaled in SPS.

Table 1 Specification of qpOffset as a function of LL

| transfer_characteristics == 16 (PQ format) |  | transfer_characteristics == 18 (HLG format) |  | Otherwise (conventional SDR format) |  |
| --- | --- | --- | --- | --- | --- |
| Conditions | qpOffset | Conditions | qpOffset | Conditions | qpOffset |
| LL <= 0.84 | 0 | LL <= 0.8 | 0 | LL <= 0.875 | 0 |
| 0.84 < LL <= 0.91 | 1 | 0.8 < LL <= 0.9 | 1 | 0.875 < LL | 1 |
| 0.91 < LL <= 0.99 | 2 | 0.9 < LL | 2 |  |  |
| 0.99 < LL | 3 |  |  |  |  |

Experimental Results

The proposed method is implemented on VTM-1.0 and BMS-1.0. The test is under CTC and the results are summarized in Table 2-7. More detailed results can be found in the accompanied Excel tables.

Table 2 Comparison of Proposed method with VTM 1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.01% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class E | 0.01% | 0.00% | 0.00% | 101% | 105% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 99% |

Table 3 Comparison of Proposed method with VTM 1.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.03% | 100% | 99% |
| Class A2 | 0.00% | -0.03% | 0.02% | 100% | 101% |
| Class B | 0.03% | -0.01% | 0.06% | 100% | 100% |
| Class C | 0.00% | -0.10% | -0.08% | 100% | 101% |
| Overall | 0.01% | -0.04% | 0.01% | 100% | 100% |
| Class D | 0.02% | -0.07% | -0.01% | 101% | 108% |

Table 4 Comparison of Proposed method with VTM 1.0

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class B | 0.01% | 0.03% | 0.09% | 101% | 103% |
| Class C | 0.03% | -0.04% | -0.06% | 100% | 100% |
| Class E | 0.07% | 0.10% | 0.06% | 100% | 98% |
| Overall | 0.03% | 0.02% | 0.03% | 100% | 101% |
| Class D | 0.01% | -0.75% | -0.46% | 100% | 95% |

Table 5 Comparison of Proposed method with BMS 1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.00% | 102% | 105% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Class B | 0.01% | 0.00% | 0.00% | 101% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class E | 0.01% | 0.00% | -0.01% | 101% | 101% |
| Overall | 0.01% | 0.00% | 0.00% | 101% | 102% |
| Class D | 0.01% | 0.00% | 0.00% | 101% | 103% |

Table 6 Comparison of Proposed method with BMS 1.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.01% | 0.02% | 100% | 100% |
| Class A2 | 0.00% | 0.02% | 0.02% | 100% | 101% |
| Class B | 0.00% | -0.02% | -0.09% | 100% | 100% |
| Class C | 0.04% | -0.05% | -0.07% | 100% | 100% |
| Overall | 0.01% | -0.02% | -0.04% | 100% | 100% |
| Class D | 0.05% | 0.04% | -0.05% | 100% | 100% |

Table 7 Comparison of Proposed method with BMS 1.0

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class B | -0.01% | -0.23% | 0.12% | 99% | 102% |
| Class C | 0.01% | 0.10% | -0.37% | 99% | 91% |
| Class E | -0.15% | 0.12% | -0.02% | 99% | 93% |
| Overall | -0.04% | -0.03% | -0.08% | 99% | 96% |
| Class D | 0.03% | -0.22% | 0.00% | 101% | 100% |

Conclusions

This contribution proposes luma-adaptive deblocking filter. The filter strength of the proposed deblocking filter is controlled based on the averaged luma level of the reconstructed samples. The proposed method is applied to an only brighter area, therefore it slightly impacts on averaging results. In the case of brighter sequences, the proposed method achieves coding efficiency.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).