| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0511 |
| --- | --- |

| Title: | Cross-check of JVET-K0161: Spatial-temporal merge mode (non subblock STMVP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Guichun Li Xiaozhong Xu | Tel: Email: | +81-43-299-8526 guichunli@tencent.com xiaozhongxu@tencent.com |
| Source: | Tencent America |  |  |

_____________________________

# Abstract

This contribution is a cross-check report of JVET-K0161, Spatial-temporal merge mode (non subblock STMVP). The software was studied and the crosscheck was carried out under the JVET common test condition (CTC).  The results of JVET-K0161 are confirmed.

# Introduction

In JVET-K0161[1], a spatial-temporal merge mode which is a simplified version of STMVP mode was proposed.

# Simulation results

The software on top of BMS1.0 was provided by the proponents [1]. Crosscheck experiments were performed under the JVET common test conditions (CTC). In addition to the CTC, as instructed by the proponents, for the VTM configuration, encoder parameter “--SubPuMvp=2--MaxNumMergeCand=6” was used; for the BMS configuration, “--SubPuMvp=3--MaxNumMergeCand=7” is used.

Table 1 and Table 2 report the results of the cross-check experiments under VTM configuration. Table 3 and Table-4 report the results under BMS configuration.

Table 1-Simulation result with VTM configuration, Random Access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.83% | -0.85% | -0.81% | 106% | 123% |
| Class A2 | -1.12% | -1.12% | -1.12% | 107% | 128% |
| Class B | -0.80% | -0.71% | -0.64% | 107% | 117% |
| Class C | -0.53% | -0.56% | -0.65% | 108% | 118% |
| Class E |  |  |  |  |  |
| Overall | -0.80% | -0.78% | -0.78% | 107% | 120% |
| Class D | -0.64% | -0.45% | -0.58% | 108% | 117% |
| Class F (optional) | -0.17% | -0.24% | -0.26% | 107% | 110% |

Table 2-Simulation result with VTM configuration, Low Delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.42% | 0.08% | 0.53% | 107% | 114% |
| Class C | -0.32% | -0.03% | 0.05% | 105% | 109% |
| Class E | 0.02% | 0.13% | -0.39% | 110% | 117% |
| Overall | -0.28% | 0.06% | 0.14% | 107% | 113% |
| Class D | -0.40% | -0.17% | -0.08% | 107% | 113% |
| Class F (optional) | -0.51% | -0.35% | -0.36% | 105% | 107% |

Table 3-Simulation result with VTM configuration, Random Access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | -0.39% | -0.25% | -0.42% | 100% | 104% |
| Class B | -0.41% | -0.25% | -0.32% | 101% | 104% |
| Class C | -0.26% | -0.18% | -0.24% | 102% | 106% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.46% | -0.38% | -0.36% | 101% | 105% |
| Class F (optional) | -0.03% | -0.17% | -0.14% | 103% | 101% |

*Note: BMS test is not fully completed yet, the results will be updated when the tests are finished.

# Conclusion

It was confirmed that the the simulation results of bitrate and PSNR match with those provided by the proponent.

# References

[1] TianYang Zhou, Tomohiro Ikai, “Spatial-temporal merge mode (non subblock STMVP)”, JVET-K0161, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 11th Meeting: Ljubljana, SI, 10–18 July 2018