| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0007 |
| --- | --- |

| Title: | JVET AHG report: Coding of HDR/WCG Material (AHG7) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Segall W. Husak E. Francois D. Rusanovskyy | Email: | asegall@sharplbas.com WJH@dolby.com Edouard.Francois@technicolor.com dmytror@qti.qualcomm.com |
| Source: | AHG7 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG7: Coding of HDR/WCG Material between the 12th meeting in Macao, CN (3 – 12 October 2018) and the 13th meeting in Marrakech, MA (9 – 18 January 2019).

# Mandates

The AHG was established with the following mandates:

- Study and evaluate available HDR/WCG test content.

- Study objective metrics for quality assessment of HDR/WCG material, including investigation of the correlation between subjective and objective results of the CfP responses.

- Compare the performance of the VTM and HM for HDR/WCG content.

- Prepare for expert viewing of HDR content at the 13th JVET meeting if feasible.

- If feasible, arrange a demonstration event for viewing of JVET-L0205 and JVET-L0245 coded material and possibly other material.

- Coordinate implementation of HDR anchor aspects in the test model software with AHG3.

- Study additional aspects of coding HDR/WCG content.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, with an [AHG7] indication on message headers.  The primary activity of the AhG was related to the mandates of arranging for a demonstration event for viewing JVET-L0205 and JVET-L0245 and maintaining test content. This work is described in the following subsection.

## Demonstration Event

In the previous AHG study (as well as the previous JVET meeting), it was determined that it was unlikely that HDR viewing equipment would be present at the Marrakech meeting due to meeting logistics.  In response to that, some HDR proponents proposed to have a demonstration event during the recent AHG period.  The goal of the demonstration event was to facilitate the viewing of JVET-L-0205 and JVET-L0245.

During the AHG study period, several companies and locations were contacted to schedule such an  event.  However, all contacts reported scheduling issues due to year-end HDR movie title releases as well as industry preparation for the Consumer Electronics Show (CES).  Because of these scheduling issues, no event was held.

It was observed that the situation continues to enforce the importance of HDR content in current, consumer entertainment and consumer electronic industries.

## Maintaining Test Content

During the recent meetings, it has been determined that there are some inconsistencies in how the copyright statements are managed within the HDR test sequences.  For example, some sequences contain the copyright statement as a text file, while other sequences include the copyright sequences in the last frame of the sequence.  During the AHG period, there was an effort to clean up and unify the copyright handling, with emphasis on the HLG content.  While this did not affect the content of any sequence, it did result in changing the md5sums for the sequences (in cases that had copyright frames).  These changes were also reflected in the CTCs.

# Contributions

There are 6 contributions related to HDR video coding:

| JVET-M0032 | m46249 | CE12: Summary report on mapping functions | E. Francois, P. Yin |
| --- | --- | --- | --- |
| JVET-M0109 | m45368 | CE12-related: block-based in-loop reshaping | E. Francois, C. Chevance, F. Hiron (Technicolor) |
| JVET-M0427 | m45700 | CE12: Mapping functions (test CE12-1 and CE12-2) | T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen (Dolby) |
| JVET-M0580 | m45876 | Crosscheck of JVET-M0109 (CE12-related: block-based in-loop luma reshaping) | T. Lu, P. Yin (Dolby) |
| JVET-M0640 | m45964 | CE12-related: in-loop reshaping with approximate inverse mapping function | E. Francois (Technicolor) |
| JVET-M0703 | m46179 | Crosscheck of JVET-M0640 (CE12-related: in-loop luma reshaping with approximate inverse mapping function) | T. Lu, P. Yin |

# Recommendations

The AHG recommends the following:

- Review all input contributions

- Discuss if a demonstration and/or viewing event should be performed prior to (or at the) JVET 14th meeting