| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0026-v2 |
| --- | --- |

| Title: | CE6: Summary Report on Transforms and Transform Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Amir Said Xin Zhao | Tel: Email: | asaid@qti.qualcomm.com xinzzhao@tencent.com |
| Source: | CE6 coordinators |  |  |

_____________________________

# Abstract

This contribution summarizes the activities of Core Experiment (CE) on Transforms and Transform Signaling. The goal of this CE is to study transform design and signaling for the VVC standard. The CE studies were divided into three categories, including:

(1) CE6.1: Transform core design (19 tests, 7 proposals)

(2) CE6.2: Fast transform (3 tests, 3 proposals)

(3) CE6.3: Transform selection and signalling (10 tests, 5 proposals)

(4) CE6.4: Sub-block transform (8 tests, 4 proposals)

(5) CE6.5: Secondary transform (1 tests, 1 proposal)

In this CE all experiments were done using based on the BMS-3.0 SW with a bug-fix for low QP configuration. This document summarizes the test results, brief experiment definition, cross-check reports and complexity measurements.

# Introduction

The purpose of this Core Experiment (CE) is to investigate the coding tools related to transforms and transform signaling on top of the VTM-3.0. In this CE, the tradeoff between coding efficiency and complexity of different transformation tools will be investigated.

# Experiment List

Core experiments (CEs) are organized according to the following five categories, and the following table lists all the experiments in each category, and the corresponding input document to the Marrakesh meeting.

| CE6.1: Transform core design |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test # | Docs. | Description | Tester | Cross-checker |
| 6-1.1a | JVET-M0496 | JVET-L0287: Replacing 4-pt DST-7 / DCT-8 by 4-pt DST-4 / DCT-4 used in MTS | X. Zhao (Tencent) | A. Karabutov (Huawei) |
| 6-1.1b |  | JVET-L0287: Replacing 4-pt and 8-pt DST-7 / DCT-8 by 4-pt and 8-pt DST-4 / DCT-4 used in MTS, respectively. | X. Zhao (Tencent) | A. Karabutov (Huawei) |
| 6-1.1c |  | JVET-L0287: Compound Orthonormal Transform | X. Zhao (Tencent) | A. Karabutov (Huawei) |
| 6-1.1d |  | 6-1.1c + 6.2.3a | X. Zhao (Tencent) | A. Karabutov (Huawei) |
| 6-1.1e | JVET-M0084 | JVET-L0262: Replacing all DST-7 / DCT-8 by DST-4 / DCT-4 used in MTS | K. Abe (Panasonic) Sony Technicolor | A. Karabutov (Huawei) |
| 6-1.2a | JVET-M0200 | JVET-L0060: Unified matrix for transform | K. Choi (Samsung) | X. Zhao (Tencent) |
| 6-1.3a | JVET-M0244 | JVET-L0353: MTS using DST-4 and transposed DCT-2 | Y. Lin (HiSilicon) | H. Egilmez (Qualcomm) |
| 6-1.3b |  | MTS using DCT-2 like transforms | Y. Lin (HiSilicon) | H. Egilmez (Qualcomm) |
| 6-1.4a | JVET-M0538 | JVET-L0386, JVET-L0682: TAF for 32-pt MTS | A. Said (Qualcomm) | A. Karabutov (Huawei) X. Zhao (Tencent) |
| 6-1.4b |  | JVET-L0386, JVET-L0682: TAF for 32-pt and 64-pt MTS | A. Said (Qualcomm) | A. Karabutov (Huawei) |
| 6-1.4c |  | JVET-L0386, JVET-L0682: TAF for 16-pt and 32-pt MTS | A. Said (Qualcomm) | A. Karabutov (Huawei) X. Zhao (Tencent) |
| 6-1.4d |  | JVET-L0386, JVET-L0682: TAF for 16-pt, 32-pt and 64-pt MTS | A. Said (Qualcomm) | A. Karabutov (Huawei) |
| 6-1.5a | JVET-M0080 | JVET-L0135: TAF simplification for 32-pt MTS | P. Philippe (Orange) | LGE |
| 6-1.5b |  | JVET-L0135: TAF simplification for 32-pt and 64-pt MTS | P. Philippe (Orange) | LGE |
| 6-1.5c |  | JVET-L0135: TAF simplification for 16-pt and 32-pt MTS | P. Philippe (Orange) | LGE |
| 6-1.5d |  | JVET-L0135: TAF simplification for 16-pt, 32-pt and 64-pt MTS | P. Philippe (Orange) | LGE |
| 6-1.6a | JVET-M0521 | JVET-L0395:4-pt DST-4 and DCT-4 replacing DST-7 and DCT-8 used in MTS | H. Egilmez (Qualcomm) | Y. Lin (HiSilicon) |
| 6-1.6b |  | JVET-L0395: 4-pt and 8-pt DST-4 and DCT-4 replacing DST-7 and DCT-8 used in MTS | H. Egilmez (Qualcomm) | Y. Lin (HiSilicon) |
| 6-1.6c |  | JVET-L0395: 4-pt, 8-pt and 16-pt DST-4 and DCT-4 replacing DST-7 and DCT-8 used in MTS | H. Egilmez (Qualcomm) | Y. Lin (HiSilicon) |

| CE6.2: Fast transform |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test # | Docs | Description | Tester | Cross-checker |
| 6-2.1a | JVET-M0288 | Fast DST-7/DCT-8 based on DFT - Simplification of transform kernel coefficients, intermediate variables, operations, etc. - Implementation-friendly architecture based on regular matrix multiply | M. Koo (LGE) | P. Philippe (Orange, bcom) |
| 6-2.2a | JVET-M0372 | JVET-L0421: Fast DST-7/DCT-8 based on DFT and matrix multiplication | K. Naser (Technicolor) | H. Gao (Huawei) |
| 6-2.3a | JVET-M0497 | JVET-L0286: Fast DST-7/DCT-8 with dual implementation support | X. Zhao (Tencent) | K. Choi (Samsung) |

| CE6.3: Transform signaling |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test # | Docs | Description | Tester | Cross-checker |
| 6-3.1a | JVET-M0303 | Shape adaptive transform selection (up to 32-point DST-7) | J. Lainema (Nokia) | H. Egilmez (Qualcomm) |
| 6-3.1b |  | Shape adaptive transform selection (up to 16-point DST-7) | J. Lainema (Nokia) | H. Egilmez (Qualcomm) |
| 6-3.2a | JVET-M0522 | Applying MTS to smaller side in case of Nx64 and 64xN) for N < 64, where max. MTS size is set to 32 by default | H. Egilmez (Qualcomm) | J. Lainema (Nokia) |
| 6-3.2b |  | Applying MTS for MxN in a way that DCT2 is used for M,N > 16, and MTS is applied for M,N <= 16. | H. Egilmez (Qualcomm) | X. Zhao (Tencent) |
| 6-3.2c |  | MTS with max. MTS size is set to 16 | H. Egilmez (Qualcomm) | J. Lainema (Nokia) |
| 6-3.3a | JVET-M0319 | For 64xN and Nx64 luma transform blocks (N < 64), MTS can be applied to the shorter side. | J. Jung (Wilus) | A. Karabutov (Huawei) |
| 6-3.3b |  | For 64xN and Nx64 luma transform blocks (N<64), MTS can be applied to the shorter side, with the increased max_BT and max_TT sizes for I-slices to 64. | J. Jung (Wilus) | A. Karabutov (Huawei) |
| 6-3.7a | JVET-M0079 | MTS size restriction to 16 | P. Philippe (Orange) | X. Zhao (Tencent) |
| 6-3.8a | JVET-M0498 | Signal horizontal transform if width is less than or equal to 16-point and height is less than or equal to 32, and signal vertical transform if height is less than or equal to 16-point and and width is less than or equal to 32 | J. Jung (Wilus) X. Zhao (Tencent) | P. Philippe (Orange) |
| 6-3.8b |  | Signal horizontal transform if width is less than or equal to 16-point and height is less than or equal to 64, and signal vertical transform if height is less than or equal to 16-point and and width is less than or equal to 64 | J. Jung (Wilus) X. Zhao (Tencent) | P. Philippe (Orange) |

| CE6.4: Sub-block transform |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test # | Docs | Description | Tester | Cross-checker |
| 6-4.1a | JVET-M0140 | Sub-block Transform (SBT) for inter blocks - transform type of residual TU inferred | Y. Zhao (Huawei) | X. Zhao (Tencent) |
| 6-4.1b |  | Sub-block Transform (SBT) for inter blocks - transform type of residual TU always DCT-2 | Y. Zhao (Huawei) | C.-M. Tsai (MediaTek) |
| 6-4.1c |  | Sub-block Transform (SBT) for inter blocks - transform type of residual TU signaled (two transform candidates each TU) | Y. Zhao (Huawei) | K. Choi (Samsung) |
| 6-4.1d |  | Sub-block Transform (SBT) for inter blocks - transform type of residual TU signaled (four transform candidates each TU, like inter MTS but with more complex context modeling) | Y. Zhao (Huawei) | M. Ikeda (Sony) |
| 6-4.1e |  | Sub-block Transform (SBT) for inter blocks - with Quad-tree split (max TU depth still 1) - transform type of residual TU inferred | Y. Zhao (Huawei) | X. Zhao (Tencent) |
| 6-4.2a | JVET-M0523 | RQT-like concept of transform sub-block splitting | Qualcomm | Y. Zhao (Huawei) |
| 6-4.3a | JVET-M0499 | RQT-like concept of transform sub-block splitting | X. Zhao (Tencent) | Y. Zhao (Huawei) |
| 6-4.4a | JVET-M0141 | RQT-like concept of transform sub-block splitting | Y. Zhao (Huawei) | H. Egilmez (Qualcomm) |

| CE6.5: Secondary transform |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test # | Docs | Description | Tester | Cross-checker |
| 6-5.1 | JVET-M0292 | Reduced Secondary Transform (RST) - Searching best configurations of RST parameters, such as the number of transform kernels, transform set, and matrix dimension. - RST index signalling optimization | M. Koo (LGE) | P. Philippe (Orange, bcom) |

# CE6.1: Transform core design

## Summary of results

The following table summarizes the results for CE6-1 using CTC configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-1.1a | JVET-M0496 | -0.17% | -0.14% | -0.11% | 101% | 99% | -0.06% | 0.05% | 0.04% | 101% | 99% | 0.02% | 0.30% | 0.13% | 100% | 98% |
| CE6-1.1b | JVET-M0496 | -0.10% | -0.17% | -0.18% | 102% | 99% | -0.01% | 0.14% | 0.05% | 100% | 100% | 0.05% | 0.00% | 0.01% | 100% | 97% |
| CE6-1.1c | JVET-M0496 | -0.03% | -0.09% | -0.09% | 101% | 100% | 0.07% | 0.12% | 0.11% | 101% | 100% | 0.15% | 0.20% | -0.42% | 99% | 98% |
| CE6-1.1d | JVET-M0496 | -0.04% | -0.11% | -0.13% | 96% | 91% | 0.06% | 0.08% | 0.04% | 99% | 99% | 0.10% | 0.27% | -0.05% | 100% | 100% |
| CE6-1.1e | JVET-M0084 | 0.23% | 0.03% | 0.07% | 103% | 98% | 0.16% | 0.30% | 0.26% | 101% | 101% | 0.04% | 0.22% | -0.17% | 101% | 100% |
| CE6-1.2a | JVET-M0200 | -0.06% | -0.14% | -0.12% | 101% | 99% | 0.07% | 0.14% | 0.11% | 99% | 99% | 0.17% | 0.32% | 0.18% | 97% | 95% |
| CE6-1.3a | JVET-M0244 | 0.08% | 0.11% | 0.10% | 98% | 97% | 0.07% | 0.25% | 0.19% | 100% | 99% | -0.02% | 0.32% | 0.33% | 100% | 101% |
| CE6-1.3b | JVET-M0244 | 0.22% | 0.34% | 0.32% | 97% | 93% | 0.17% | 0.39% | 0.36% | 99% | 99% | 0.03% | 0.34% | 0.10% | 100% | 101% |
| CE6-1.4a | JVET-M0538 | 0.00% | 0.07% | 0.05% | 98% | 94% | -0.01% | 0.08% | 0.13% | 100% | 99% | -0.02% | 0.09% | 0.11% | 99% | 99% |
| CE6-1.4b | JVET-M0538 | -0.07% | 0.01% | 0.00% | 98% | 94% | -0.12% | -0.12% | -0.22% | 100% | 98% | 0.01% | 0.28% | 0.01% | 99% | 97% |
| CE6-1.4c | JVET-M0538 | 0.12% | 0.17% | 0.15% | 95% | 91% | 0.04% | 0.09% | 0.18% | 99% | 99% | 0.04% | 0.20% | 0.19% | 99% | 100% |
| CE6-1.4d | JVET-M0538 | 0.05% | 0.09% | 0.08% | 95% | 90% | -0.06% | -0.11% | -0.14% | 99% | 98% | 0.01% | -0.03% | 0.01% | 98% | 96% |
| CE6-1.5a | JVET-M0080 | 0.07% | 0.06% | 0.06% | 96% | 88% | 0.06% | 0.11% | 0.11% | 99% | 99% | 0.03% | 0.18% | 0.16% | 100% | 100% |
| CE6-1.5b | JVET-M0080 | 0.02% | -0.03% | -0.02% | 98% | 88% | -0.03% | -0.14% | -0.12% | 101% | 99% | -0.01% | 0.04% | -0.15% | 101% | 100% |
| CE6-1.5c | JVET-M0080 | 0.15% | 0.09% | 0.09% | 94% | 85% | 0.09% | 0.19% | 0.25% | 99% | 98% | 0.06% | 0.16% | 0.14% | 100% | 100% |
| CE6-1.5d | JVET-M0080 | 0.09% | 0.01% | 0.02% | 96% | 85% | 0.01% | -0.07% | -0.07% | 101% | 98% | 0.07% | 0.04% | 0.05% | 101% | 100% |
| CE6-1.6a | JVET-M0521 | -0.17% | -0.14% | -0.11% | 99% | 103% | -0.06% | 0.05% | 0.04% | 100% | 101% | 0.02% | 0.30% | 0.13% | 100% | 99% |
| CE6-1.6b | JVET-M0521 | -0.12% | -0.18% | -0.16% | 99% | 101% | -0.02% | 0.16% | 0.02% | 100% | 100% | 0.02% | 0.11% | -0.07% | 100% | 100% |
| CE6-1.6c | JVET-M0521 | 0.08% | -0.06% | -0.04% | 101% | 101% | 0.09% | 0.14% | 0.18% | 100% | 100% | 0.07% | 0.00% | 0.18% | 100% | 100% |

The following table summarizes the results for CE6-1 using low QP configuration and VTM-3.0 as anchor.

|  |  |  |  | All Intra |  |  |  |  |  |  | Random Access |  |  |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-1.1a | JVET-M0496 | 0.00% | -0.06% | -0.06% | 99% | 100% | 0.00% | -0.02% | -0.01% | 100% | 100% | -0.01% | 0.00% | -0.01% | 101% | 101% |
| CE6-1.1b | JVET-M0496 | 0.24% | 0.00% | 0.01% | 101% | 101% | 0.06% | -0.03% | -0.01% | 100% | 101% | 0.02% | -0.01% | 0.00% | 100% | 101% |
| CE6-1.1c | JVET-M0496 | 0.26% | 0.03% | 0.04% | 102% | 101% | 0.12% | 0.04% | 0.06% | 100% | 101% | 0.05% | 0.06% | 0.07% | 99% | 101% |
| CE6-1.1d | JVET-M0496 | 0.28% | 0.04% | 0.04% | 99% | 100% | 0.13% | 0.05% | 0.06% | 100% | 100% | 0.05% | 0.08% | 0.06% | 100% | 102% |
| CE6-1.1e | JVET-M0084 | 0.41% | 0.20% | 0.20% | 101% | 101% | 0.11% | 0.05% | 0.08% | 100% | 100% | 0.04% | 0.03% | 0.03% | 100% | 100% |
| CE6-1.2a | JVET-M0200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.3a | JVET-M0244 | 0.20% | 0.17% | 0.18% | 99% | 100% |  |  |  |  |  |  |  |  |  |  |
| CE6-1.3b | JVET-M0244 | 0.21% | 0.18% | 0.18% | 99% | 99% |  |  |  |  |  |  |  |  |  |  |
| CE6-1.4a | JVET-M0538 | 0.03% | 0.04% | 0.05% | 87% | 101% | 0.01% | 0.04% | 0.04% | 95% | 104% | 0.00% | 0.01% | 0.01% | 91% | 102% |
| CE6-1.4b | JVET-M0538 | 0.03% | 0.04% | 0.05% | 87% | 102% | 0.01% | 0.04% | 0.05% | 95% | 105% | 0.00% | 0.01% | 0.01% | 91% | 101% |
| CE6-1.4c | JVET-M0538 | 0.13% | 0.18% | 0.18% | 85% | 100% | 0.05% | 0.09% | 0.10% | 94% | 103% | 0.02% | 0.05% | 0.04% | 90% | 101% |
| CE6-1.4d | JVET-M0538 | 0.13% | 0.18% | 0.18% | 85% | 101% | 0.05% | 0.09% | 0.09% | 95% | 104% | 0.02% | 0.04% | 0.03% | 90% | 101% |
| CE6-1.5a | JVET-M0080 | 0.05% | 0.07% | 0.07% | 98% | 100% | 0.02% | 0.08% | 0.07% | 99% | 100% |  |  |  |  |  |
| CE6-1.5b | JVET-M0080 | 0.05% | 0.07% | 0.07% | 99% | 100% | 0.02% | 0.07% | 0.06% | 100% | 100% |  |  |  |  |  |
| CE6-1.5c | JVET-M0080 | 0.23% | 0.28% | 0.28% | 96% | 100% | 0.10% | 0.16% | 0.19% | 99% | 100% |  |  |  |  |  |
| CE6-1.5d | JVET-M0080 | 0.23% | 0.28% | 0.28% | 97% | 100% | 0.10% | 0.16% | 0.19% | 99% | 100% |  |  |  |  |  |
| CE6-1.6a | JVET-M0521 | 0.00% | -0.06% | -0.06% | 94% | 97% | 0.00% | -0.02% | -0.01% | 99% | 105% | -0.01% | 0.00% | -0.01% | 96% | 102% |
| CE6-1.6b | JVET-M0521 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.6c | JVET-M0521 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The following table summarizes the results for CE6-1 using CTC w/ Inter MTS configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-1.1a | JVET-M0496 | -0.17% | -0.14% | -0.11% | 101% | 100% | -0.07% | 0.01% | 0.03% | 101% | 100% | -0.07% | 0.31% | 0.03% | 100% | 98% |
| CE6-1.1b | JVET-M0496 | -0.10% | -0.16% | -0.18% | 102% | 99% | -0.05% | 0.03% | -0.04% | 101% | 99% | -0.13% | -0.11% | -0.20% | 100% | 97% |
| CE6-1.1c | JVET-M0496 | -0.03% | -0.09% | -0.09% | 101% | 100% | 0.03% | 0.08% | 0.08% | 101% | 100% | -0.03% | 0.14% | -0.29% | 100% | 100% |
| CE6-1.1d | JVET-M0496 | -0.04% | -0.11% | -0.12% | 96% | 91% | 0.02% | -0.01% | -0.04% | 98% | 98% | -0.06% | 0.08% | -0.33% | 96% | 97% |
| CE6-1.1e | JVET-M0084 | 0.23% | 0.03% | 0.07% | 103% | 98% | 0.13% | 0.15% | 0.11% | 102% | 100% | -0.16% | -0.20% | -0.49% | 102% | 101% |
| CE6-1.2a | JVET-M0200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.3a | JVET-M0244 | 0.08% | 0.11% | 0.10% | 98% | 97% | 0.12% | 0.20% | 0.13% | 99% | 99% | -0.03% | 0.28% | 0.03% | 99% | 98% |
| CE6-1.3b | JVET-M0244 | 0.22% | 0.34% | 0.32% | 97% | 93% | 0.21% | 0.32% | 0.27% | 98% | 98% | 0.06% | 0.16% | -0.09% | 98% | 97% |
| CE6-1.4a | JVET-M0538 | 0.00% | 0.07% | 0.05% | 98% | 94% | 0.00% | 0.12% | 0.15% | 98% | 98% | 0.05% | 0.13% | -0.01% | 97% | 96% |
| CE6-1.4b | JVET-M0538 | -0.07% | 0.01% | 0.00% | 98% | 94% | -0.15% | -0.06% | -0.15% | 102% | 97% | -0.01% | 0.22% | 0.09% | 102% | 95% |
| CE6-1.4c | JVET-M0538 | 0.12% | 0.17% | 0.15% | 95% | 91% | 0.09% | 0.08% | 0.21% | 96% | 96% | 0.16% | 0.13% | -0.07% | 95% | 95% |
| CE6-1.4d | JVET-M0538 | 0.05% | 0.09% | 0.08% | 95% | 90% | -0.07% | -0.04% | 0.01% | 101% | 95% | 0.09% | 0.18% | 0.07% | 100% | 93% |
| CE6-1.5a | JVET-M0080 | 0.07% | 0.06% | 0.06% | 96% | 88% | 0.05% | 0.09% | 0.10% | 97% | 97% | 0.05% | 0.26% | -0.22% | 120% | 103% |
| CE6-1.5b | JVET-M0080 | 0.02% | -0.03% | -0.02% | 98% | 88% | -0.08% | -0.07% | -0.04% | 104% | 96% | -0.09% | 0.24% | 0.02% | 107% | 104% |
| CE6-1.5c | JVET-M0080 | 0.15% | 0.09% | 0.09% | 94% | 85% | 0.09% | 0.15% | 0.23% | 96% | 96% | 0.05% | 0.15% | -0.05% | 98% | 101% |
| CE6-1.5d | JVET-M0080 | 0.09% | 0.01% | 0.02% | 96% | 85% | -0.04% | 0.00% | 0.01% | 103% | 96% | -0.09% | 0.13% | 0.01% | 106% | 102% |
| CE6-1.6a | JVET-M0521 | -0.17% | -0.14% | -0.11% | 99% | 103% | -0.07% | 0.01% | 0.03% | 100% | 102% | -0.07% | 0.31% | 0.03% | 100% | 99% |
| CE6-1.6b | JVET-M0521 | -0.12% | -0.18% | -0.16% | 99% | 101% | -0.04% | -0.04% | -0.10% | 100% | 101% | -0.08% | 0.05% | -0.36% | 100% | 98% |
| CE6-1.6c | JVET-M0521 | 0.08% | -0.06% | -0.04% | 101% | 101% | 0.06% | 0.05% | 0.11% | 100% | 100% | -0.14% | -0.06% | 0.04% | 100% | 99% |

The following table summarizes the results for CE6-1 using low QP w/ Inter MTS and VTM-3.0 as anchor.

|  |  |  |  | All Intra |  |  |  |  |  |  | Random Access |  |  |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-1.1a | JVET-M0496 | 0.00% | -0.06% | -0.06% | 100% | 100% | 0.00% | -0.03% | -0.03% | 101% | 101% | -0.01% | -0.03% | -0.03% | 101% | 100% |
| CE6-1.1b | JVET-M0496 | 0.24% | 0.00% | 0.01% | 101% | 101% | 0.11% | -0.08% | -0.06% | 100% | 100% | 0.10% | -0.07% | -0.06% | 100% | 100% |
| CE6-1.1c | JVET-M0496 | 0.26% | 0.03% | 0.04% | 102% | 101% | 0.13% | -0.06% | -0.04% | 100% | 101% | 0.11% | -0.06% | -0.05% | 100% | 101% |
| CE6-1.1d | JVET-M0496 | 0.28% | 0.04% | 0.04% | 100% | 100% | 0.14% | -0.05% | -0.03% | 98% | 98% | 0.12% | -0.06% | -0.06% | 98% | 97% |
| CE6-1.1e | JVET-M0084 | 0.41% | 0.20% | 0.20% | 100% | 99% | 0.21% | 0.01% | 0.04% | 100% | 99% | 0.19% | 0.06% | 0.07% | 100% | 100% |
| CE6-1.2a | JVET-M0200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.3a | JVET-M0244 | 0.20% | 0.17% | 0.18% | 99% | 99% |  |  |  |  |  | 0.15% | 0.06% | 0.08% | 98% | 96% |
| CE6-1.3b | JVET-M0244 | 0.21% | 0.18% | 0.18% | 99% | 99% |  |  |  |  |  |  |  |  |  |  |
| CE6-1.4a | JVET-M0538 | 0.03% | 0.04% | 0.05% | 93% | 99% | 0.03% | 0.07% | 0.07% | 95% | 98% | 0.02% | 0.05% | 0.05% | 91% | 97% |
| CE6-1.4b | JVET-M0538 | 0.03% | 0.04% | 0.05% | 95% | 99% | 0.02% | 0.07% | 0.06% | 98% | 99% | 0.01% | 0.06% | 0.05% | 93% | 97% |
| CE6-1.4c | JVET-M0538 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.4d | JVET-M0538 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.5a | JVET-M0080 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.5b | JVET-M0080 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.5c | JVET-M0080 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.5d | JVET-M0080 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.6a | JVET-M0521 | 0.00% | -0.06% | -0.06% | 94% | 97% | 0.00% | -0.03% | -0.03% | 98% | 99% | -0.01% | -0.03% | -0.03% | 92% | 97% |
| CE6-1.6b | JVET-M0521 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.6c | JVET-M0521 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## Test definition

CE6-1.1 a/b/c/d: JVET-L0287 CE6: Compound Orthonormal Transform (Test 6.1.7 f/g) [X. Zhao, Z. Zhang, X. Li, S. Liu (Tencent)]

Two aspects have been proposed, including:

- Use DST-4 / DCT-4 to replace DST-7 / DCT-8 for 4-point and 8-point transforms used in MTS

- Embedding 16-point and 32-point DST-7 / DCT-8 into 64-point DCT-2, and

With both two proposed aspects, all the transform types used in VVC can be extracted from one single 64x64 matrix.

CE6-1.1 e:JVET-L0262 CE 6-1.7a: MTS simplification by reusing DCT-2 partial butterfly - Change MTS transform to DST-4/DCT-4 [K. Abe, T, Toma (Panasonic), M. Ikeda, T. Tsukuba (Sony), K. Naser, F. Leannec, E. Francois (Technicolor)]

In this proposal, a direct replacement is considered, where the DST-7 and DCT-8 basis functions of all transforms sizes (4, 8, 16 and 32) are replaced by the basis functions of DST-4 and DCT-4.

CE6-1.2a: JVET-L0060 CE6-related: Unified matrix for transform [K. Choi, K. P. Choi (Samsung)]

This CE describes a unified matrix designed with 8-bit representation proposed in JVET-L0060. The unified matrix is based on 64x64 that can derive all kernel types with all block sizes used in VTM. The proposed matrix in this CE is shown in Table 1.

DCT2 can be derived by sampling a subset of coefficients in the matrix as follows:

    - DCT2_NxN[l, k] = matrix_64x64[l, 2(6- log2N) *k]

    - where DCT2_NxN[l, k] represents the l-th element of the k-th basis vector of DCT-2 for NxN matrix and N is a size of matrix from 64 to 2.

DCT8 can be derived by sampling a subset of coefficients in the matrix as follows:

    - DCT8_NxN[l, k] = matrix_64x64[l, 2(6- log2N) *k +2(5 - log2N)]

    - where DCT8_NxN[l, k] represents the l-th element of the k-th basis vector of DCT-8 for NxN matrix and N is a size of matrix from 32 to 4.

DST7 can be derived by sampling a subset of coefficients in the matrix as follows:

    - DST7_NxN[l, k] = (-1)^k * DCT8_NxN[N – l - 1, k]

    - DST7_NxN[l, k] = (-1)^k * matrix_64x64[N – l - 1, 2(6- log2N) *k +2(5 - log2N)]

    - where DST7_NxN[l, k] represents the l-th element of the k-th basis vector of DST-7 for NxN matrix and N is a size of matrix from 32 to 4.

Table 1. Proposed unified 64x64 matrix for transform

64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 64, 90, 90, 89, 89, 88, 87, 86, 84, 82, 81, 79, 76, 74, 71, 69, 66, 63, 60, 56, 53, 49, 46, 42, 38, 34, 30, 26, 22, 17, 13, 9, 4, -4, -9,-13,-17,-22,-26,-30,-34,-38,-42,-46,-49,-53,-56,-60,-63,-66,-69,-71,-74,-76,-79,-81,-82,-84,-86,-87,-88,-89,-89,-90,-90, 89, 88, 87, 84, 81, 77, 73, 67, 62, 55, 48, 41, 33, 25, 17, 9, -9,-17,-25,-33,-41,-48,-55,-62,-67,-73,-77,-81,-84,-87,-88,-89,-89,-88,-87,-84,-81,-77,-73,-67,-62,-55,-48,-41,-33,-25,-17, -9, 9, 17, 25, 33, 41, 48, 55, 62, 67, 73, 77, 81, 84, 87, 88, 89, 90, 88, 84, 79, 71, 63, 53, 42, 30, 17, 4, -9,-22,-34,-46,-56,-66,-74,-81,-86,-89,-90,-89,-87,-82,-76,-69,-60,-49,-38,-26,-13, 13, 26, 38, 49, 60, 69, 76, 82, 87, 89, 90, 89, 86, 81, 74, 66, 56, 46, 34, 22, 9, -4,-17,-30,-42,-53,-63,-71,-79,-84,-88,-90, 90, 87, 80, 70, 58, 43, 26, 9, -9,-26,-43,-58,-70,-80,-87,-90,-90,-87,-80,-70,-58,-43,-26, -9, 9, 26, 43, 58, 70, 80, 87, 90, 90, 87, 80, 70, 58, 43, 26, 9, -9,-26,-43,-58,-70,-80,-87,-90,-90,-87,-80,-70,-58,-43,-26, -9, 9, 26, 43, 58, 70, 80, 87, 90, 89, 84, 74, 60, 42, 22, 0,-22,-42,-60,-74,-84,-89,-89,-84,-74,-60,-42,-22, 0, 22, 42, 60, 74, 84, 89, 89, 84, 74, 60, 42, 22,-22,-42,-60,-74,-84,-89,-89,-84,-74,-60,-42,-22, 0, 22, 42, 60, 74, 84, 89, 89, 84, 74, 60, 42, 22, 0,-22,-42,-60,-74,-84,-89, 88, 81, 67, 48, 25, 0,-25,-48,-67,-81,-88,-88,-81,-67,-48,-25, 25, 48, 67, 81, 88, 88, 81, 67, 48, 25, 0,-25,-48,-67,-81,-88,-88,-81,-67,-48,-25, 0, 25, 48, 67, 81, 88, 88, 81, 67, 48, 25,-25,-48,-67,-81,-88,-88,-81,-67,-48,-25, 0, 25, 48, 67, 81, 88, 89, 79, 60, 34, 4,-26,-53,-74,-87,-90,-82,-66,-42,-13, 17, 46, 69, 84, 90, 86, 71, 49, 22, -9,-38,-63,-81,-89,-88,-76,-56,-30, 30, 56, 76, 88, 89, 81, 63, 38, 9,-22,-49,-71,-86,-90,-84,-69,-46,-17, 13, 42, 66, 82, 90, 87, 74, 53, 26, -4,-34,-60,-79,-89, 89, 75, 50, 18,-18,-50,-75,-89,-89,-75,-50,-18, 18, 50, 75, 89, 89, 75, 50, 18,-18,-50,-75,-89,-89,-75,-50,-18, 18, 50, 75, 89, 89, 75, 50, 18,-18,-50,-75,-89,-89,-75,-50,-18, 18, 50, 75, 89, 89, 75, 50, 18,-18,-50,-75,-89,-89,-75,-50,-18, 18, 50, 75, 89, 88, 71, 42, 4,-34,-66,-86,-89,-76,-49,-13, 26, 60, 82, 90, 81, 56, 22,-17,-53,-79,-90,-84,-63,-30, 9, 46, 74, 89, 87, 69, 38,-38,-69,-87,-89,-74,-46, -9, 30, 63, 84, 90, 79, 53, 17,-22,-56,-81,-90,-82,-60,-26, 13, 49, 76, 89, 86, 66, 34, -4,-42,-71,-88, 87, 67, 33, -9,-48,-77,-89,-81,-55,-17, 25, 62, 84, 88, 73, 41,-41,-73,-88,-84,-62,-25, 17, 55, 81, 89, 77, 48, 9,-33,-67,-87,-87,-67,-33, 9, 48, 77, 89, 81, 55, 17,-25,-62,-84,-88,-73,-41, 41, 73, 88, 84, 62, 25,-17,-55,-81,-89,-77,-48, -9, 33, 67, 87, 87, 63, 22,-26,-66,-88,-86,-60,-17, 30, 69, 89, 84, 56, 13,-34,-71,-89,-82,-53, -9, 38, 74, 90, 81, 49, 4,-42,-76,-90,-79,-46, 46, 79, 90, 76, 42, -4,-49,-81,-90,-74,-38, 9, 53, 82, 89, 71, 34,-13,-56,-84,-89,-69,-30, 17, 60, 86, 88, 66, 26,-22,-63,-87, 87, 58, 9,-43,-80,-90,-70,-26, 26, 70, 90, 80, 43, -9,-58,-87,-87,-58, -9, 43, 80, 90, 70, 26,-26,-70,-90,-80,-43, 9, 58, 87, 87, 58, 9,-43,-80,-90,-70,-26, 26, 70, 90, 80, 43, -9,-58,-87,-87,-58, -9, 43, 80, 90, 70, 26,-26,-70,-90,-80,-43, 9, 58, 87, 86, 53, 0,-53,-86,-86,-53, 0, 53, 86, 86, 53, 0,-53,-86,-86,-53, 0, 53, 86, 86, 53, 0,-53,-86,-86,-53, 0, 53, 86, 86, 53,-53,-86,-86,-53, 0, 53, 86, 86, 53, 0,-53,-86,-86,-53, 0, 53, 86, 86, 53, 0,-53,-86,-86,-53, 0, 53, 86, 86, 53, 0,-53,-86, 84, 48, -9,-62,-88,-77,-33, 25, 73, 89, 67, 17,-41,-81,-87,-55, 55, 87, 81, 41,-17,-67,-89,-73,-25, 33, 77, 88, 62, 9,-48,-84,-84,-48, 9, 62, 88, 77, 33,-25,-73,-89,-67,-17, 41, 81, 87, 55,-55,-87,-81,-41, 17, 67, 89, 73, 25,-33,-77,-88,-62, -9, 48, 84, 84, 42,-22,-74,-89,-60, 0, 60, 89, 74, 22,-42,-84,-84,-42, 22, 74, 89, 60, 0,-60,-89,-74,-22, 42, 84, 84, 42,-22,-74,-89,-60, 60, 89, 74, 22,-42,-84,-84,-42, 22, 74, 89, 60, 0,-60,-89,-74,-22, 42, 84, 84, 42,-22,-74,-89,-60, 0, 60, 89, 74, 22,-42,-84, 84, 35,-35,-84,-84,-35, 35, 84, 84, 35,-35,-84,-84,-35, 35, 84, 84, 35,-35,-84,-84,-35, 35, 84, 84, 35,-35,-84,-84,-35, 35, 84, 84, 35,-35,-84,-84,-35, 35, 84, 84, 35,-35,-84,-84,-35, 35, 84, 84, 35,-35,-84,-84,-35, 35, 84, 84, 35,-35,-84,-84,-35, 35, 84, 82, 30,-42,-87,-76,-17, 53, 89, 69, 4,-63,-90,-60, 9, 71, 89, 49,-22,-79,-86,-38, 34, 84, 81, 26,-46,-88,-74,-13, 56, 90, 66,-66,-90,-56, 13, 74, 88, 46,-26,-81,-84,-34, 38, 86, 79, 22,-49,-89,-71, -9, 60, 90, 63, -4,-69,-89,-53, 17, 76, 87, 42,-30,-82, 81, 25,-48,-88,-67, 0, 67, 88, 48,-25,-81,-81,-25, 48, 88, 67,-67,-88,-48, 25, 81, 81, 25,-48,-88,-67, 0, 67, 88, 48,-25,-81,-81,-25, 48, 88, 67, 0,-67,-88,-48, 25, 81, 81, 25,-48,-88,-67, 67, 88, 48,-25,-81,-81,-25, 48, 88, 67, 0,-67,-88,-48, 25, 81, 81, 17,-60,-90,-49, 30, 86, 74, 4,-69,-88,-38, 42, 89, 66, -9,-76,-84,-26, 53, 90, 56,-22,-82,-79,-13, 63, 89, 46,-34,-87,-71, 71, 87, 34,-46,-89,-63, 13, 79, 82, 22,-56,-90,-53, 26, 84, 76, 9,-66,-89,-42, 38, 88, 69, -4,-74,-86,-30, 49, 90, 60,-17,-81, 80, 9,-70,-87,-26, 58, 90, 43,-43,-90,-58, 26, 87, 70, -9,-80,-80, -9, 70, 87, 26,-58,-90,-43, 43, 90, 58,-26,-87,-70, 9, 80, 80, 9,-70,-87,-26, 58, 90, 43,-43,-90,-58, 26, 87, 70, -9,-80,-80, -9, 70, 87, 26,-58,-90,-43, 43, 90, 58,-26,-87,-70, 9, 80, 79, 4,-74,-82,-13, 69, 86, 22,-63,-88,-30, 56, 89, 38,-49,-90,-46, 42, 90, 53,-34,-89,-60, 26, 87, 66,-17,-84,-71, 9, 81, 76,-76,-81, -9, 71, 84, 17,-66,-87,-26, 60, 89, 34,-53,-90,-42, 46, 90, 49,-38,-89,-56, 30, 88, 63,-22,-86,-69, 13, 82, 74, -4,-79, 77, 0,-77,-77, 0, 77, 77, 0,-77,-77, 0, 77, 77, 0,-77,-77, 77, 77, 0,-77,-77, 0, 77, 77, 0,-77,-77, 0, 77, 77, 0,-77,-77, 0, 77, 77, 0,-77,-77, 0, 77, 77, 0,-77,-77, 0, 77, 77,-77,-77, 0, 77, 77, 0,-77,-77, 0, 77, 77, 0,-77,-77, 0, 77, 76, -9,-84,-66, 26, 89, 53,-42,-90,-38, 56, 88, 22,-69,-82, -4, 79, 74,-13,-86,-63, 30, 89, 49,-46,-90,-34, 60, 87, 17,-71,-81, 81, 71,-17,-87,-60, 34, 90, 46,-49,-89,-30, 63, 86, 13,-74,-79, 4, 82, 69,-22,-88,-56, 38, 90, 42,-53,-89,-26, 66, 84, 9,-76, 75,-18,-89,-50, 50, 89, 18,-75,-75, 18, 89, 50,-50,-89,-18, 75, 75,-18,-89,-50, 50, 89, 18,-75,-75, 18, 89, 50,-50,-89,-18, 75, 75,-18,-89,-50, 50, 89, 18,-75,-75, 18, 89, 50,-50,-89,-18, 75, 75,-18,-89,-50, 50, 89, 18,-75,-75, 18, 89, 50,-50,-89,-18, 75, 74,-22,-89,-42, 60, 84, 0,-84,-60, 42, 89, 22,-74,-74, 22, 89, 42,-60,-84, 0, 84, 60,-42,-89,-22, 74, 74,-22,-89,-42, 60, 84,-84,-60, 42, 89, 22,-74,-74, 22, 89, 42,-60,-84, 0, 84, 60,-42,-89,-22, 74, 74,-22,-89,-42, 60, 84, 0,-84,-60, 42, 89, 22,-74, 73,-25,-89,-33, 67, 77,-17,-88,-41, 62, 81, -9,-87,-48, 55, 84,-84,-55, 48, 87, 9,-81,-62, 41, 88, 17,-77,-67, 33, 89, 25,-73,-73, 25, 89, 33,-67,-77, 17, 88, 41,-62,-81, 9, 87, 48,-55,-84, 84, 55,-48,-87, -9, 81, 62,-41,-88,-17, 77, 67,-33,-89,-25, 73, 71,-34,-89,-13, 82, 56,-53,-84, 9, 89, 38,-69,-74, 30, 90, 17,-81,-60, 49, 86, -4,-88,-42, 66, 76,-26,-90,-22, 79, 63,-46,-87, 87, 46,-63,-79, 22, 90, 26,-76,-66, 42, 88, 4,-86,-49, 60, 81,-17,-90,-30, 74, 69,-38,-89, -9, 84, 53,-56,-82, 13, 89, 34,-71, 70,-43,-87, 9, 90, 26,-80,-58, 58, 80,-26,-90, -9, 87, 43,-70,-70, 43, 87, -9,-90,-26, 80, 58,-58,-80, 26, 90, 9,-87,-43, 70, 70,-43,-87, 9, 90, 26,-80,-58, 58, 80,-26,-90, -9, 87, 43,-70,-70, 43, 87, -9,-90,-26, 80, 58,-58,-80, 26, 90, 9,-87,-43, 70, 69,-46,-84, 17, 90, 13,-86,-42, 71, 66,-49,-82, 22, 90, 9,-87,-38, 74, 63,-53,-81, 26, 89, 4,-88,-34, 76, 60,-56,-79, 30, 89,-89,-30, 79, 56,-60,-76, 34, 88, -4,-89,-26, 81, 53,-63,-74, 38, 87, -9,-90,-22, 82, 49,-66,-71, 42, 86,-13,-90,-17, 84, 46,-69, 67,-48,-81, 25, 88, 0,-88,-25, 81, 48,-67,-67, 48, 81,-25,-88, 88, 25,-81,-48, 67, 67,-48,-81, 25, 88, 0,-88,-25, 81, 48,-67,-67, 48, 81,-25,-88, 0, 88, 25,-81,-48, 67, 67,-48,-81, 25, 88,-88,-25, 81, 48,-67,-67, 48, 81,-25,-88, 0, 88, 25,-81,-48, 67, 66,-56,-74, 46, 81,-34,-86, 22, 89, -9,-90, -4, 89, 17,-87,-30, 82, 42,-76,-53, 69, 63,-60,-71, 49, 79,-38,-84, 26, 88,-13,-90, 90, 13,-88,-26, 84, 38,-79,-49, 71, 60,-63,-69, 53, 76,-42,-82, 30, 87,-17,-89, 4, 90, 9,-89,-22, 86, 34,-81,-46, 74, 56,-66, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 64,-64,-64, 64, 63,-66,-60, 69, 56,-71,-53, 74, 49,-76,-46, 79, 42,-81,-38, 82, 34,-84,-30, 86, 26,-87,-22, 88, 17,-89,-13, 89, 9,-90, -4, 90,-90, 4, 90, -9,-89, 13, 89,-17,-88, 22, 87,-26,-86, 30, 84,-34,-82, 38, 81,-42,-79, 46, 76,-49,-74, 53, 71,-56,-69, 60, 66,-63, 62,-67,-55, 73, 48,-77,-41, 81, 33,-84,-25, 87, 17,-88, -9, 89,-89, 9, 88,-17,-87, 25, 84,-33,-81, 41, 77,-48,-73, 55, 67,-62,-62, 67, 55,-73,-48, 77, 41,-81,-33, 84, 25,-87,-17, 88, 9,-89, 89, -9,-88, 17, 87,-25,-84, 33, 81,-41,-77, 48, 73,-55,-67, 62, 60,-74,-42, 84, 22,-89, 0, 89,-22,-84, 42, 74,-60,-60, 74, 42,-84,-22, 89, 0,-89, 22, 84,-42,-74, 60, 60,-74,-42, 84, 22,-89, 89,-22,-84, 42, 74,-60,-60, 74, 42,-84,-22, 89, 0,-89, 22, 84,-42,-74, 60, 60,-74,-42, 84, 22,-89, 0, 89,-22,-84, 42, 74,-60, 58,-80,-26, 90, -9,-87, 43, 70,-70,-43, 87, 9,-90, 26, 80,-58,-58, 80, 26,-90, 9, 87,-43,-70, 70, 43,-87, -9, 90,-26,-80, 58, 58,-80,-26, 90, -9,-87, 43, 70,-70,-43, 87, 9,-90, 26, 80,-58,-58, 80, 26,-90, 9, 87,-43,-70, 70, 43,-87, -9, 90,-26,-80, 58, 56,-81,-22, 90,-17,-82, 53, 60,-79,-26, 90,-13,-84, 49, 63,-76,-30, 89, -9,-86, 46, 66,-74,-34, 89, -4,-87, 42, 69,-71,-38, 88,-88, 38, 71,-69,-42, 87, 4,-89, 34, 74,-66,-46, 86, 9,-89, 30, 76,-63,-49, 84, 13,-90, 26, 79,-60,-53, 82, 17,-90, 22, 81,-56, 55,-81,-17, 89,-25,-77, 62, 48,-84, -9, 88,-33,-73, 67, 41,-87, 87,-41,-67, 73, 33,-88, 9, 84,-48,-62, 77, 25,-89, 17, 81,-55,-55, 81, 17,-89, 25, 77,-62,-48, 84, 9,-88, 33, 73,-67,-41, 87,-87, 41, 67,-73,-33, 88, -9,-84, 48, 62,-77,-25, 89,-17,-81, 55, 53,-86, 0, 86,-53,-53, 86, 0,-86, 53, 53,-86, 0, 86,-53,-53, 86, 0,-86, 53, 53,-86, 0, 86,-53,-53, 86, 0,-86, 53, 53,-86, 86,-53,-53, 86, 0,-86, 53, 53,-86, 0, 86,-53,-53, 86, 0,-86, 53, 53,-86, 0, 86,-53,-53, 86, 0,-86, 53, 53,-86, 0, 86,-53, 50,-89, 18, 75,-75,-18, 89,-50,-50, 89,-18,-75, 75, 18,-89, 50, 50,-89, 18, 75,-75,-18, 89,-50,-50, 89,-18,-75, 75, 18,-89, 50, 50,-89, 18, 75,-75,-18, 89,-50,-50, 89,-18,-75, 75, 18,-89, 50, 50,-89, 18, 75,-75,-18, 89,-50,-50, 89,-18,-75, 75, 18,-89, 50, 49,-89, 22, 71,-79, -9, 86,-60,-38, 90,-34,-63, 84, -4,-81, 69, 26,-89, 46, 53,-88, 17, 74,-76,-13, 87,-56,-42, 90,-30,-66, 82,-82, 66, 30,-90, 42, 56,-87, 13, 76,-74,-17, 88,-53,-46, 89,-26,-69, 81, 4,-84, 63, 34,-90, 38, 60,-86, 9, 79,-71,-22, 89,-49, 48,-88, 25, 67,-81, 0, 81,-67,-25, 88,-48,-48, 88,-25,-67, 81,-81, 67, 25,-88, 48, 48,-88, 25, 67,-81, 0, 81,-67,-25, 88,-48,-48, 88,-25,-67, 81, 0,-81, 67, 25,-88, 48, 48,-88, 25, 67,-81, 81,-67,-25, 88,-48,-48, 88,-25,-67, 81, 0,-81, 67, 25,-88, 48, 46,-90, 42, 49,-90, 38, 53,-89, 34, 56,-89, 30, 60,-88, 26, 63,-87, 22, 66,-86, 17, 69,-84, 13, 71,-82, 9, 74,-81, 4, 76,-79, 79,-76, -4, 81,-74, -9, 82,-71,-13, 84,-69,-17, 86,-66,-22, 87,-63,-26, 88,-60,-30, 89,-56,-34, 89,-53,-38, 90,-49,-42, 90,-46, 43,-90, 58, 26,-87, 70, 9,-80, 80, -9,-70, 87,-26,-58, 90,-43,-43, 90,-58,-26, 87,-70, -9, 80,-80, 9, 70,-87, 26, 58,-90, 43, 43,-90, 58, 26,-87, 70, 9,-80, 80, -9,-70, 87,-26,-58, 90,-43,-43, 90,-58,-26, 87,-70, -9, 80,-80, 9, 70,-87, 26, 58,-90, 43, 42,-89, 60, 22,-84, 74, 0,-74, 84,-22,-60, 89,-42,-42, 89,-60,-22, 84,-74, 0, 74,-84, 22, 60,-89, 42, 42,-89, 60, 22,-84, 74,-74, 84,-22,-60, 89,-42,-42, 89,-60,-22, 84,-74, 0, 74,-84, 22, 60,-89, 42, 42,-89, 60, 22,-84, 74, 0,-74, 84,-22,-60, 89,-42, 41,-88, 62, 17,-81, 77, -9,-67, 87,-33,-48, 89,-55,-25, 84,-73, 73,-84, 25, 55,-89, 48, 33,-87, 67, 9,-77, 81,-17,-62, 88,-41,-41, 88,-62,-17, 81,-77, 9, 67,-87, 33, 48,-89, 55, 25,-84, 73,-73, 84,-25,-55, 89,-48,-33, 87,-67, -9, 77,-81, 17, 62,-88, 41, 38,-87, 74, -9,-63, 90,-53,-22, 81,-82, 26, 49,-89, 66, 4,-71, 88,-42,-34, 86,-76, 13, 60,-90, 56, 17,-79, 84,-30,-46, 89,-69, 69,-89, 46, 30,-84, 79,-17,-56, 90,-60,-13, 76,-86, 34, 42,-88, 71, -4,-66, 89,-49,-26, 82,-81, 22, 53,-90, 63, 9,-74, 87,-38, 35,-84, 84,-35,-35, 84,-84, 35, 35,-84, 84,-35,-35, 84,-84, 35, 35,-84, 84,-35,-35, 84,-84, 35, 35,-84, 84,-35,-35, 84,-84, 35, 35,-84, 84,-35,-35, 84,-84, 35, 35,-84, 84,-35,-35, 84,-84, 35, 35,-84, 84,-35,-35, 84,-84, 35, 35,-84, 84,-35,-35, 84,-84, 35, 34,-82, 84,-38,-30, 81,-86, 42, 26,-79, 87,-46,-22, 76,-88, 49, 17,-74, 89,-53,-13, 71,-89, 56, 9,-69, 90,-60, -4, 66,-90, 63,-63, 90,-66, 4, 60,-90, 69, -9,-56, 89,-71, 13, 53,-89, 74,-17,-49, 88,-76, 22, 46,-87, 79,-26,-42, 86,-81, 30, 38,-84, 82,-34, 33,-81, 84,-41,-25, 77,-87, 48, 17,-73, 88,-55, -9, 67,-89, 62,-62, 89,-67, 9, 55,-88, 73,-17,-48, 87,-77, 25, 41,-84, 81,-33,-33, 81,-84, 41, 25,-77, 87,-48,-17, 73,-88, 55, 9,-67, 89,-62, 62,-89, 67, -9,-55, 88,-73, 17, 48,-87, 77,-25,-41, 84,-81, 33, 30,-76, 89,-63, 9, 49,-86, 84,-46,-13, 66,-90, 74,-26,-34, 79,-89, 60, -4,-53, 87,-82, 42, 17,-69, 90,-71, 22, 38,-81, 88,-56, 56,-88, 81,-38,-22, 71,-90, 69,-17,-42, 82,-87, 53, 4,-60, 89,-79, 34, 26,-74, 90,-66, 13, 46,-84, 86,-49, -9, 63,-89, 76,-30, 26,-70, 90,-80, 43, 9,-58, 87,-87, 58, -9,-43, 80,-90, 70,-26,-26, 70,-90, 80,-43, -9, 58,-87, 87,-58, 9, 43,-80, 90,-70, 26, 26,-70, 90,-80, 43, 9,-58, 87,-87, 58, -9,-43, 80,-90, 70,-26,-26, 70,-90, 80,-43, -9, 58,-87, 87,-58, 9, 43,-80, 90,-70, 26, 26,-69, 89,-81, 46, 4,-53, 84,-88, 63,-17,-34, 74,-90, 76,-38,-13, 60,-87, 86,-56, 9, 42,-79, 90,-71, 30, 22,-66, 89,-82, 49,-49, 82,-89, 66,-22,-30, 71,-90, 79,-42, -9, 56,-86, 87,-60, 13, 38,-76, 90,-74, 34, 17,-63, 88,-84, 53, -4,-46, 81,-89, 69,-26, 25,-67, 88,-81, 48, 0,-48, 81,-88, 67,-25,-25, 67,-88, 81,-48, 48,-81, 88,-67, 25, 25,-67, 88,-81, 48, 0,-48, 81,-88, 67,-25,-25, 67,-88, 81,-48, 0, 48,-81, 88,-67, 25, 25,-67, 88,-81, 48,-48, 81,-88, 67,-25,-25, 67,-88, 81,-48, 0, 48,-81, 88,-67, 25, 22,-60, 84,-89, 74,-42, 0, 42,-74, 89,-84, 60,-22,-22, 60,-84, 89,-74, 42, 0,-42, 74,-89, 84,-60, 22, 22,-60, 84,-89, 74,-42, 42,-74, 89,-84, 60,-22,-22, 60,-84, 89,-74, 42, 0,-42, 74,-89, 84,-60, 22, 22,-60, 84,-89, 74,-42, 0, 42,-74, 89,-84, 60,-22, 18,-50, 75,-89, 89,-75, 50,-18,-18, 50,-75, 89,-89, 75,-50, 18, 18,-50, 75,-89, 89,-75, 50,-18,-18, 50,-75, 89,-89, 75,-50, 18, 18,-50, 75,-89, 89,-75, 50,-18,-18, 50,-75, 89,-89, 75,-50, 18, 18,-50, 75,-89, 89,-75, 50,-18,-18, 50,-75, 89,-89, 75,-50, 18, 17,-49, 74,-88, 89,-76, 53,-22,-13, 46,-71, 87,-89, 79,-56, 26, 9,-42, 69,-86, 90,-81, 60,-30, -4, 38,-66, 84,-90, 82,-63, 34,-34, 63,-82, 90,-84, 66,-38, 4, 30,-60, 81,-90, 86,-69, 42, -9,-26, 56,-79, 89,-87, 71,-46, 13, 22,-53, 76,-89, 88,-74, 49,-17, 17,-48, 73,-87, 88,-77, 55,-25, -9, 41,-67, 84,-89, 81,-62, 33,-33, 62,-81, 89,-84, 67,-41, 9, 25,-55, 77,-88, 87,-73, 48,-17,-17, 48,-73, 87,-88, 77,-55, 25, 9,-41, 67,-84, 89,-81, 62,-33, 33,-62, 81,-89, 84,-67, 41, -9,-25, 55,-77, 88,-87, 73,-48, 17, 13,-38, 60,-76, 87,-90, 86,-74, 56,-34, 9, 17,-42, 63,-79, 88,-90, 84,-71, 53,-30, 4, 22,-46, 66,-81, 89,-89, 82,-69, 49,-26, 26,-49, 69,-82, 89,-89, 81,-66, 46,-22, -4, 30,-53, 71,-84, 90,-88, 79,-63, 42,-17, -9, 34,-56, 74,-86, 90,-87, 76,-60, 38,-13, 9,-26, 43,-58, 70,-80, 87,-90, 90,-87, 80,-70, 58,-43, 26, -9, -9, 26,-43, 58,-70, 80,-87, 90,-90, 87,-80, 70,-58, 43,-26, 9, 9,-26, 43,-58, 70,-80, 87,-90, 90,-87, 80,-70, 58,-43, 26, -9, -9, 26,-43, 58,-70, 80,-87, 90,-90, 87,-80, 70,-58, 43,-26, 9, 9,-26, 42,-56, 69,-79, 86,-89, 90,-87, 81,-71, 60,-46, 30,-13, -4, 22,-38, 53,-66, 76,-84, 89,-90, 88,-82, 74,-63, 49,-34, 17,-17, 34,-49, 63,-74, 82,-88, 90,-89, 84,-76, 66,-53, 38,-22, 4, 13,-30, 46,-60, 71,-81, 87,-90, 89,-86, 79,-69, 56,-42, 26, -9, 9,-25, 41,-55, 67,-77, 84,-88, 89,-87, 81,-73, 62,-48, 33,-17, 17,-33, 48,-62, 73,-81, 87,-89, 88,-84, 77,-67, 55,-41, 25, -9, -9, 25,-41, 55,-67, 77,-84, 88,-89, 87,-81, 73,-62, 48,-33, 17,-17, 33,-48, 62,-73, 81,-87, 89,-88, 84,-77, 67,-55, 41,-25, 9, 4,-13, 22,-30, 38,-46, 53,-60, 66,-71, 76,-81, 84,-87, 89,-90, 90,-89, 88,-86, 82,-79, 74,-69, 63,-56, 49,-42, 34,-26, 17, -9, 9,-17, 26,-34, 42,-49, 56,-63, 69,-74, 79,-82, 86,-88, 89,-90, 90,-89, 87,-84, 81,-76, 71,-66, 60,-53, 46,-38, 30,-22, 13, -4,

CE6-1.3 a/b: JVET-L0353 CE6-related: MTS using DST-4 and transposed DCT-2 [Y. Lin, J. Zheng, Q. Yu, N. Zhang (HiSilicon), C. Zhu (UESTC)]

It is proposed to replace core transforms (DST-7 and DCT-8) of MTS with DST-4 and transposed DCT-2 to enable reusing hardware design of legacy DCT-2 transform. The DCT-2 like transform( i.e. transposed DCT-2) can be implemented with partial butterfly for further complexity reduction.

CE6-1.4 a/b/c/d: JVET-L0386 CE6.1.6: Efficient Implementations of AMT with Transform Adjustment Filters (TAF) [A. Said, H. Egilmez, Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm)]

CE6-1.4 a/b/c/d: JVET-L0682 CE6-related: Low-complexity approximations with 8-bit Transform Adjustment Filters (TAF) [H. Egilmez, Y.-H. Chao, A. Said, V. Seregin, M. Karczewicz (Qualcomm)]

MTS transforms (DST-7 and DCT-8) are approximated using DCT-2 and transform adjustment filters (TAF). In CEs 6-1.4a-d, low-complexity approximations for different transform sizes will be investigated.

CE6-1.5 a/b/c/d: CE6-1.1: JVET-L0135 CE6: Further simplification of AMT with adjustment stages (Test CE6.1.6b) [P. Philippe (Orange), V. Lorcy (bcom)]

In CE 6.1.5, TAF is further simplified as follows:

- The adjustment stage used for the forward and inverse transform contain a maximum of 5 taps per row each coefficient is represented on 8 bits.

- Further, symmetries are imposed in the adjustment matrices for the size-32 and above, leading to a reduced storage impact.

CE6-1.6 a/b/c: JVET-L0395 CE6-related: MTS with 4-point DST/DCT-4 and large block support [H. Egilmez, A. Gadde, V. Seregin, M. Karczewicz, A. Said (Qualcomm)]

This proposal introduces a transform table replacing 4-point, 8-point or 16-point DST-7 and DCT-8 with DST-4 and DCT-4.

## Crosscheck reports

| Test # | Crosscheckers | Implementation is aligned with the algorithm description? | Does crosschecking confirm whether ALL data points match with the results provided by proponent? | Additional comments? |
| --- | --- | --- | --- | --- |
| CE6-1.1a | Huawei |  | All finished results matched. LQP results are partial, will be updated. |  |
| CE6-1.1b | Huawei |  |  |  |
| CE6-1.1c | Huawei |  |  |  |
| CE6-1.1d | Huawei |  |  |  |
| CE6-1.1e | Huawei |  | CTC results for normal QP are confirmed to be matching with the proponents' results. Results of 1.1e (LQP) I will send later. |  |
| CE6-1.2a | Tencent | Yes | CTC results for normal QP are confirmed to be matching with the proponents' results |  |
| CE6-1.3a | Qualcomm |  | Yes |  | Crosschecker's CTC results match the results provided by the proponent |  |  |  |
| CE6-1.3b | Qualcomm | Yes | Crosschecker's CTC results match the results provided by the proponent |  |
| CE6-1.4a | Huawei, Tencent |  | RA results slightly not matched (about 0.01%) that relates to problem of parsing PSNR. It observed for: • 1.4a CTC for Class A2, • 1.4a CTC with Inter MTS for Class A1 and D, • 1.4c CTC with Inter MTS for Class A2, • 1.4d CTC for Class C and F, • 1.4d CTC with Inter MTS for Class A2. For 1.4 (all tests for LQP) encoder complexity gap between my and Amir’s results from 10% to 15%. |  |
| CE6-1.4b | Huawei |  |  |  |
| CE6-1.4c | Huawei, Tencent |  |  |  |
| CE6-1.4d | Huawei |  |  |  |
| CE6-1.5a | LGE | Yes | The proponents’ results are confirmed to be matched with the cross-checker’s ones under both CTC and the condition with inter MTS on. | none |
| CE6-1.5b | LGE | Yes | The proponents’ results are confirmed to be with the cross-checker’s ones under both CTC and the condition with inter MTS on. | none |
| CE6-1.5c | LGE | Yes | The proponents’ results are confirmed to be with the cross-checker’s ones under both CTC and the condition with inter MTS on. | none |
| CE6-1.5d | LGE | Yes | The proponents’ results are confirmed to be with the cross-checker’s ones under both CTC and the condition with inter MTS on. | none |
| CE6-1.6a | HiSilicon | Yes | 1. For normal QP tests, all data points are confirmed except for RA configuration. The mismatch for RA is due to different simulation platforms used by proponent and cross-checker, and the issue is related to VTM-3.0 bug (https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/134#comment:5 ) 2. For low QP tests, only AI and LDB results are confirmed. |  |
| CE6-1.6b | HiSilicon |  |  |  |
| CE6-1.6c | HiSilicon |  |  |  |

## Related contributions

| Document | Related CE Test | Title |
| --- | --- | --- |
| JVET-M0046 |  | CE6-related: A study of primary transforms |
| JVET-M0072 |  | Non-CE6: On transform skip for lager block |
| JVET-M0269 |  | Non-CE6 : Extension of transform skip block size to 8x8 |
| JVET-M0275 |  | Non-CE6 : On transform skip conditions |
| JVET-M0297 |  | CE6-related: 32 point MTS based on skipping high frequency coefficients |
| JVET-M0354 |  | CE6-related: MTS with Haar transform for Screen Contents Coding |
| JVET-M0396 |  | CE6-related: MTS kernel derivation for efficient memory usage |
| JVET-M0397 |  | CE6-related: DST-3 based transform kernels derivation |
| JVET-M0398 | CE6-1.5 | CE6-related Further simplification of CE6-1.5 |
| JVET-M0480 |  | CE6-related: Implicit transform selection for Multi directional LM |
| JVET-M0482 |  | CE6-related: Implicit transform selection for Multi-hypothesis inter-intra mode |
| JVET-M0501 |  | CE6 related: Unification of Transform Skip mode and MTS |
| JVET-M0539 |  | CE6-related: Efficient computation of MTS transform combinations |
| JVET-M0540 |  | CE6-related: Software tool for computing transform throughput |

# CE6.2: Fast transform

## Summary of results

The following table summarizes the results for CE6-2 using CTC configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-2.1a | JVET-M0288 | 0.00% | 0.02% | 0.00% | 96% | 90% | -0.01% | 0.01% | 0.07% | 99% | 98% | 0.02% | 0.26% | -0.06% | 100% | 99% |
| CE6-2.2a | JVET-M0372 | 0.08% | 0.05% | 0.04% | 118% | 112% | 0.01% | 0.07% | 0.12% | 116% | 105% | 0.04% | 0.22% | -0.17% | 101% | 100% |
| CE6-2.3a | JVET-M0497 | 0.01% | -0.01% | 0.01% | 96% | 91% | -0.01% | -0.03% | -0.02% | 100% | 100% | 0.00% | 0.22% | -0.22% | 99% | 101% |

The following table summarizes the results for CE6-2 using low QP configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-2.1a | JVET-M0288 | 0.08% | 0.08% | 0.09% | 98% | 99% |  |  |  |  |  |  |  |  |  |  |
| CE6-2.2a | JVET-M0372 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-2.3a | JVET-M0497 | 0.02% | 0.00% | 0.00% | 98% | 99% | 0.01% | 0.00% | 0.01% | 99% | 99% | 0.00% | -0.01% | 0.00% | 100% | 100% |

The following table summarizes the results for CE6-2 using CTC w/ Inter MTS configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-2.1a | JVET-M0288 | 0.00% | 0.02% | 0.00% | 96% | 90% | -0.01% | -0.03% | 0.03% | 97% | 98% | 0.01% | 0.11% | -0.19% | 97% | 96% |
| CE6-2.2a | JVET-M0372 | 0.08% | 0.05% | 0.04% | 118% | 112% | 0.03% | 0.00% | 0.12% |  |  | 0.07% | 0.13% | -0.18% |  |  |
| CE6-2.3a | JVET-M0497 | 0.01% | -0.01% | 0.01% | 96% | 91% | 0.00% | -0.04% | -0.04% | 98% | 100% | 0.00% | 0.18% | -0.37% | 97% | 97% |

The following table summarizes the results for CE6-2 using low QP w/ Inter MTS and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-2.1a | JVET-M0288 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-2.2a | JVET-M0372 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-2.3a | JVET-M0497 | 0.02% | 0.00% | 0.00% | 98% | 99% |  |  |  |  |  | 0.00% | -0.01% | -0.01% | 98% | 96% |

Additional results of CE6-2.2a with 8-bit representation of DFT matrices:

| CTC |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-2.2a | JVET-M0372 | 0.09% | 0.05% | 0.09% | 118% | 112% | 0.00% | 0.11% | 0.04% | 116% | 104% | 0.04% | 0.22% | -0.17% | 101% | 100% |

| CTC, w/ Inter MTS |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-2.2a | JVET-M0372 |  |  |  |  |  | 0.04% | 0.07% | 0.04% |  |  |  |  |  |  |  |

## Test definition

CE6-2.1a: JVET-L0132 CE 6-1.1 (c,d): Fast DST-7/DCT-8 based on DFT and 32 point MTS based on skipping high frequency coefficients [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

In the last JVET meeting, it is commented that fast algorithms should provide implementation-friendly features for both HW and SW.  For example, allowing at most 16-bit operations or enabling regular vector operations could be more useful for practical implementation as well as low computational complexity and memory usage. Therefore, it is planned to simplify our current design accordingly. Moreover, design reusability and unification among transforms are high priority aspects being considered, so that unification of our primary transform designs will be investigated together.

CE6-2.2a: JVET-L0421 CE6-related: fast implementation of MTS transforms using matrix multiplication [K. Naser, G. Rath, E. Francois (Technicolor)]

This contribution proposes to decompose the MTS DST-7/DCT-8 transforms basis into smaller matrices using the prime factor algorithm of DFT for real input data and imaginary output data. The decomposition is generic for 16- and 32- point MTS transforms, and results into matrix multiplications to implement the transforms. The approach consists in performing a N-point transform using a (2N+1) point DFT. The (2N+1) point DFT is decomposed into 2 stages, involving N2 DFTs of length N1 then N1 DFTs of length N2, with 2N+1=N1xN2, and N1 and N2 being mutually prime.

The algorithm is made of the following steps:

- N2 calls of DFT_RI involving N1-DFTs

- 1 call of DFT_RI_IO involving N2-DFTs

- (N1+1)/2 calls of DFT_IO involving N2-DFTs

- Plus permutations of input/output

where DFT_RI, DFT_RI_IO and DFT_IO are generic elementary blocks.

Test 6-x.xa will evaluate variants of the approach in terms of internal bit-depth, and reduction of the number of operations.

CE6-2.3a: JVET-L0286 CE6: Fast DST-7/DCT-8 with dual implementation support (Test 6.1.4) [X. Zhao, Z. Zhang, X. Li, S. Liu (Tencent)]

A fast DST-7/DCT-8 method based on the characteristics of DST-7 / DCT-8 bases. The proposed method supports dual implementations, i.e., identical results between full matrix multiplication and fast method.

## Crosscheck reports

| Test # | Crosscheckers | Implementation is aligned with the algorithm description? | Does crosschecking confirm whether ALL data points match with the results provided by proponent? | Additional comments? |
| --- | --- | --- | --- | --- |
| CE6-2.1a | Orange, bcom | Yes | Results were verified for CTC (AI,RA & LD) and when MTS is enabled for Inter coding modes. Bd-rate results are exact; the execution times are within the measurement error range. |  |
| CE6-2.2a | Huawei |  |  |  |
| CE6-2.3a | Samsung | Yes | Testing results were verifeid and all Bd-reates are exactly matched. |  |

## Related contributions

| Document | Related CE Test | Title |
| --- | --- | --- |
| JVET-M0085 | CE6-1.1, CE6-1.6 | CE6-related: Fast algorithm for DST-4/DCT-4 as alternative transforms for MTS |
| JVET-M0379 | CE6-2.2 | CE6-related: Further Simplification on top of CE6-2.2 |

# CE6.3: Transform signaling

## Summary of results

The following table summarizes the results for CE6-3 using CTC configuration and VTM-3.0 as anchor.

|  |  |  |  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  | Low Delay B |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-3.1a | JVET-M0303 | -0.08% | 0.02% | 0.00% | 99% | 102% | -0.03% | 0.01% | 0.07% | 101% | 103% | 0.02% | 0.37% | 0.24% | 100% | 99% |
| CE6-3.1b | JVET-M0303 | -0.06% | -0.02% | -0.08% | 100% | 99% | -0.03% | 0.05% | 0.02% | 100% | 101% | 0.02% | 0.18% | -0.01% | 99% | 99% |
| CE6-3.2a | JVET-M0522 | 0.00% | 0.00% | 0.00% | 99% | 99% | -0.04% | -0.09% | -0.16% | 99% | 98% | 0.02% | -0.02% | 0.10% | 101% | 99% |
| CE6-3.2b | JVET-M0522 | 0.33% | 0.40% | 0.38% | 92% | 93% | 0.18% | 0.39% | 0.48% | 97% | 99% | 0.07% | 0.27% | 0.19% | 99% | 99% |
| CE6-3.2c | JVET-M0522 | 0.55% | 0.62% | 0.63% | 86% | 92% | 0.32% | 0.62% | 0.68% | 95% | 99% | 0.12% | 0.49% | 0.16% | 98% | 99% |
| CE6-3.3a | JVET-M0319 | 0.00% | 0.00% | 0.00% | 100% | 100% | -0.04% | -0.09% | -0.16% | 101% | 100% | 0.02% | -0.02% | 0.10% | 100% | 100% |
| CE6-3.3b | JVET-M0319 | -0.65% | -1.51% | -1.56% | 154% | 100% | -0.18% | -0.66% | -0.71% | 103% | 100% | 0.00% | -0.48% | -0.57% | 101% | 100% |
| CE6-3.7a | JVET-M0079 | 0.55% | 0.62% | 0.63% | 86% | 87% | 0.32% | 0.62% | 0.68% | 95% | 98% | 0.12% | 0.49% | 0.16% | 98% | 99% |
| CE6-3.8a | JVET-M0498 | 0.33% | 0.40% | 0.38% | 91% | 92% | 0.18% | 0.39% | 0.48% | 98% | 99% | 0.07% | 0.27% | 0.19% | 99% | 100% |
| CE6-3.8b | JVET-M0498 | 0.33% | 0.40% | 0.38% | 91% | 93% | 0.17% | 0.29% | 0.43% | 99% | 99% | 0.06% | 0.29% | 0.22% | 99% | 100% |

The following table summarizes the results for CE6-3 using CTC w/ Inter MTS configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-3.1a | JVET-M0303 | -0.08% | 0.02% | 0.00% | 99% | 102% | -0.04% | 0.03% | 0.04% | 100% | 101% | -0.03% | 0.19% | 0.09% | 99% | 101% |
| CE6-3.1b | JVET-M0303 | -0.06% | -0.02% | -0.08% | 100% | 99% | -0.04% | 0.01% | 0.01% | 101% | 100% | -0.03% | 0.06% | -0.25% | 100% | 101% |
| CE6-3.2a | JVET-M0522 | 0.00% | 0.00% | 0.00% | 99% | 99% | -0.04% | -0.06% | -0.03% | 102% | 100% | -0.02% | 0.10% | -0.20% | 101% | 99% |
| CE6-3.2b | JVET-M0522 | 0.33% | 0.40% | 0.38% | 92% | 93% | 0.28% | 0.30% | 0.38% | 91% | 97% | 0.26% | 0.20% | -0.01% | 90% | 95% |
| CE6-3.2c | JVET-M0522 | 0.55% | 0.62% | 0.63% | 86% | 92% | 0.45% | 0.43% | 0.43% | 85% | 97% | 0.35% | 0.15% | -0.10% | 84% | 95% |
| CE6-3.3a | JVET-M0319 | 0.00% | 0.00% | 0.00% | 100% | 100% | -0.04% | -0.06% | -0.03% | 104% | 100% | -0.02% | 0.10% | -0.20% | 104% | 100% |
| CE6-3.3b | JVET-M0319 | -0.65% | -1.51% | -1.56% | 154% | 100% | -0.18% | -0.66% | -0.66% | 105% | 100% | -0.03% | -0.31% | -0.54% | 104% | 101% |
| CE6-3.7a | JVET-M0079 | 0.55% | 0.62% | 0.63% | 86% | 87% | 0.45% | 0.43% | 0.43% | 85% | 96% | 0.35% | 0.15% | -0.10% | 85% | 101% |
| CE6-3.8a | JVET-M0498 | 0.33% | 0.40% | 0.38% | 92% | 92% | 0.28% | 0.30% | 0.38% | 91% | 98% | 0.26% | 0.20% | -0.01% | 90% | 96% |
| CE6-3.8b | JVET-M0498 | 0.33% | 0.40% | 0.38% | 92% | 93% | 0.26% | 0.25% | 0.35% | 93% | 98% | 0.24% | 0.11% | 0.06% | 92% | 97% |

The following table summarizes the results for CE6-3.1 using CTC w/ MTS = 0 configuration for both test and the VTM-3.0 anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-3.1a | JVET-M0303 | -1.66% | -2.31% | -2.28% | 99% | 119% | -0.75% | -0.87% | -0.96% | 100% | 108% | -0.09% | -0.18% | -0.33% | 99% | 102% |
| CE6-3.1b | JVET-M0303 | -1.61% | -2.11% | -2.12% | 101% | 111% | -0.71% | -0.81% | -0.84% | 100% | 102% | -0.14% | -0.02% | -0.33% | 100% | 100% |

## Test definition

CE6-3.1 a/b: JVET-L0134 CE6-related: Shape adaptive transform selection [J. Lainema (Nokia)]

This experiment measures performance of the shape adaptive transform selection approach proposed in JVET-L0134. Proposed tool decides horizontal and vertical transforms for Intra coded CUs based on the dimensions of the CU. If a CU is rectangular, DST-7 is selected for the shorter side of the CU and DCT-2 is used for the longer one. The test is performed both with and without enabling MTS and in the case of MTS off the results are reported with respect to both CTC anchor and anchor that also has MTS disabled.

Meaningful combinations with other tools in the CE may be tested. For example, the maximum length of DST-7 may be limited to 16 points, different MTS configurations may be enabled and different transforms for the rectangular blocks may be used.

CE6-3.2 a/b/c: JVET-L0395 CE6-related: MTS with 4-point DST/DCT-4 and large block support [H. Egilmez, A. Gadde, V. Seregin, M. Karczewicz, A. Said (Qualcomm)]

In the current VTM (where max. MTS size = 32), to incorporate DST-7 or DCT-8 for large rectangular blocks with sizes Nx64 and 64xN for N<64, the 64-point DCT-2 is used to transform the longer side of the block, and N-point DCT-8 or N-point DST-7 is used for the shorter side. One bit is signalled to determine whether DST-7 or DCT-8 is used for the shorter side of the block, and no signalling is required for the side using DCT-2. Moreover, since current VTM has both DST-7 and DCT-8 available for N<64, no additional memory or transform kernels are required to implement this method. This method will be tested in CE 6-3.2a.

In CE 6-3.2b, the same idea with max. MTS size = 16 will also be tested where DST-7 and DCT-8 are used for Nx32 and 32xN for N<32 with one-bit signalling. The MTS with max MTS size = 16 will be tested in CE 6-3.2c.

In CE 6-3.4, inter MTS is not applied for 4x4 TUs, and MTS overhead is not signaled for those blocks.

In CE 6-3.5, the intra MTS coefficient threshold scheme is applied to inter MTS, i.e. if number of non-zero coefficients is less than 2, MTS index is not signaled, and transform index is inferred equal to 0.

CE6-3.3 a/b: JVET-L0195 CE6-related: MTS for non-square CUs [J. Jung, D. Kim, G. Ko, J. Son, J. Kwak (WILUS), Y. Lee (Humax)]

Enable MTS (Multiple Transform Selection) to the shorter side of 64xN and Nx64 luma transform blocks (N<64). Currently, MTS is applied to a luma transform block only when both of the width and the height are smaller than or equal to 32. It is proposed to loosen the restriction to enable MTS to the shorter side of rectangular blocks if it is smaller than or equal to 32 even though the longer side is larger than 32. The shorter side of large rectangular blocks such as 64xN and Nx64 (N<64) can exploit selection of the transform kernels among DST-VII and DCT-VIII.

CE6-3.7 a / CE6-3.8a/b: MTS restriction using up to 16-length transforms

A possible restriction of maximum complexity could probably be achieved by restricting the block size further (as it is currently already restricted to maximum of 32-length transforms). Data for knowing what the disadvantage in compression by imposing further restriction would be shall be collected in the CE, along with experimenting on fast algorithms.

## Crosscheck reports

| Test # | Crosscheckers | Implementation is aligned with the algorithm description? | Does crosschecking confirm whether ALL data points match with the results provided by proponent? | Additional comments? |
| --- | --- | --- | --- | --- |
| 6-3.1a | Qualcomm | Yes | Yes |  |
| 6-3.1b | Qualcomm | Yes | Yes |  |
| 6-3.2a | Nokia | Yes | Yes |  |
| 6-3.2b | Tencent |  | CTC results confimed to be matching with proponents' results. CTC w/ Inter MTS results from proponent are not complete, but available results match |  |
| 6-3.2c | Nokia | Yes | Yes |  |
| 6-3.3a | Huawei |  | RA results slightly not matched (about 0.01%) that relates to problem of parsing PSNR. It observed for: • 3.3a CTC and CTC with Inter MTS for Class A2, • 3.3b CTC for Class F, • 3.3b CTC with Inter MTS for Class A1 and A2. |  |
| 6-3.3b | Huawei |  |  |  |
| 6-3.7a | Tencent | Yes | Yes |  |
| 6-3.8a | Orange | Yes | Configurations CE6-3.8a and CE6-3.8b were tested. Results were verified for CTC (AI,RA & LD) and when MTS is enabled for Inter coding modes in RA conditions. Bd-rate results are exact; the execution times are within the measurement error range. |  |
| 6-3.8b | Orange |  |  |  |

## Related contributions

| Document | Related CE Test | Title |
| --- | --- | --- |
| JVET-M0201 |  | CE6-related: Syntax clean-up related to MTS |
| JVET-M0202 |  | CE6-related: Simplification related to MTS with reduced modes |
| JVET-M0280 |  | CE6-related: Context selection for entropy coding the MTS flag |
| JVET-M0304 |  | CE6-related: 2-mode MTS with shape adaptive transform selection |
| JVET-M0340 |  | CE6-related：Simplification on MTS for intra residual coding |
| JVET-M0347 |  | CE6-related: Simplification on MTS CU flag coding |

# CE6.4: Transform signaling

## Summary of results

The following table summarizes the results for CE6-4 using CTC configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-4.1a | JVET-M0140 |  |  |  |  |  | -0.47% | -0.16% | 0.00% | 108% | 101% | -0.83% | -0.98% | -0.06% | 113% | 102% |
| CE6-4.1b | JVET-M0140 |  |  |  |  |  | -0.25% | -0.16% | -0.07% | 107% | 101% | -0.42% | -0.89% | -0.04% | 111% | 101% |
| CE6-4.1c | JVET-M0140 |  |  |  |  |  | -0.54% | -0.20% | 0.08% | 110% | 101% | -0.94% | -0.75% | 0.23% | 116% | 102% |
| CE6-4.1d | JVET-M0140 |  |  |  |  |  | -0.52% | -0.10% | 0.14% | 114% | 101% | -0.90% | -0.70% | 0.25% | 121% | 102% |
| CE6-4.1e | JVET-M0140 |  |  |  |  |  | -0.55% | -0.26% | 0.00% | 109% | 101% | -0.93% | -0.91% | 0.07% | 115% | 102% |
| CE6-4.2a | JVET-M0523 |  |  |  |  |  | -0.36% | 0.09% | 0.35% | 122% | 102% | -0.59% | -0.65% | 0.47% | 129% | 101% |
| CE6-4.2a + InterMTS | JVET-M0523 |  |  |  |  |  | -0.62% | 0.61% | 0.80% | 146% | 105% | -0.94% | 0.68% | 1.41% | 161% | 107% |
| CE6-4.3a | JVET-M0499 | -0.29% | -0.16% | -0.17% | 192.17% | 107% | -0.42% | -0.30% | -0.10% | 143% | 103% | -0.45% | -0.64% | -0.01% | 142% | 104% |
| CE6-4.4a | JVET-M0141 |  |  |  |  |  | -0.59% | -0.35% | 0.10% | 120% | 102% | -1.01% | -1.77% | -0.19% | 127% | 103% |

The following table summarizes the results for CE6-4 using CTC w/ Inter MTS configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-4.1a | JVET-M0140 |  |  |  |  |  | -0.28% | -0.21% | -0.06% | 98% | 101% | -0.43% | -0.72% | -0.26% | 96% | 100% |
| CE6-4.1b | JVET-M0140 |  |  |  |  |  | -0.12% | -0.15% | -0.06% | 97% | 101% | -0.20% | -0.67% | -0.21% | 95% | 101% |
| CE6-4.1c | JVET-M0140 |  |  |  |  |  | -0.33% | -0.25% | 0.03% | 100% | 100% | -0.53% | -0.88% | -0.28% | 98% | 101% |
| CE6-4.1d | JVET-M0140 |  |  |  |  |  | -0.29% | -0.14% | 0.03% | 103% | 100% | -0.47% | -0.64% | -0.06% | 102% | 100% |
| CE6-4.1e | JVET-M0140 |  |  |  |  |  | -0.34% | -0.23% | -0.06% | 99% | 101% | -0.55% | -0.73% | -0.21% | 97% | 101% |
| CE6-4.2a | JVET-M0523 |  |  |  |  |  | -0.25% | 0.06% | 0.24% | 120% | 103% | -0.40% | -0.23% | 0.31% | 124% | 101% |
| CE6-4.3a | JVET-M0499 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-4.4a | JVET-M0141 |  |  |  |  |  | -0.30% | -0.29% | 0.01% | 107% | 101% | -0.52% | -1.32% | -0.54% | 105% | 102% |

## Test definition

CE6-4.1 a/b/c/d/e: JVET-L0358 CE6: Sub-block transform for inter blocks (CE6.1.2) [Y. Zhao, H. Yang, J. Chen (Huawei)]

For an inter-predicted CU with cu_root_cbf equal to 1, cu_sbt_flag may be signaled to indicate whether the whole residual block or a sub-part of the residual block is decoded. In the former case, inter MTS information is parsed to determine the transform type of the CU. In the latter case, a part of the residual block is coded with inferred adaptive transform and the other part of the residual block is zeroed out.

When SBT is used for a CU, SBT type and SBT position information are further decoded from the bitstream. There are two SBT types and two SBT positions, as indicated in Figure 1. For SBT-V (or SBT-H), the TU width (or height) may equal to half of the CU width (or height) or 1/4 of the CU width (or height), signaled by another flag, resulting in 2:2 split or 1:3/3:1 split. If one side of CU is 8 in luma samples, the 1:3/3:1 split is not allowed. Hence, there are at most 8 SVT modes for a CU.

SBT-V (or SBT-H) is allowed for CU with width and height both no larger than maxSbtSize. The maxSbtSize is signaled in SPS. For HD and 4K sequences, maxSbtSize is set as 64 by encoder; for other smaller resolution sequences, maxSbtSize is set as 32.

Position-dependent transform is applied in SBT. The two positions of SBT-H and SBT-V are associated with different core transforms. More specifically, the horizontal and vertical transforms for each SBT position is specified in Figure 1. For example, the horizontal and vertical transforms for SBT-V position 0 is DCT-8 and DST-7, respectively. When one side of the residual TU is greater than 32, the corresponding transform is set as DCT-2.

Therefore, the sub-block transform jointly specifies the TU tiling, cbf, and horizontal and vertical transforms of a residual block, which may be considered a syntax shortcut for the cases that the major residual of a block is at one side of the block.

Some fast algorithms have been enabled in RDO of SBT. For each SBT mode, a RD cost is estimated based on the SSD of the residual-skipped part. An SBT mode is skipped in RDO if the estimated RD cost of the SBT mode is larger than the actual RD cost of the best mode. In addition, only the best 4 SBT modes in terms of the estimated RD cost are tried in RDO. To further reduce encoder run-time, encoder fast algorithm is going to be studied.

Figure 1 Illustration of Sub-block Transform modes

CE6-4.2 a/ CE6-4.3a / CE6-4.4a: RQT-like concept of transform sub-block splitting

A sub-block transform scheme is tested, which allows multiple transform blocks within one coding block.

## Crosscheck reports

| Test # | Crosscheckers | Implementation is aligned with the algorithm description? | Does crosschecking confirm whether ALL data points match with the results provided by proponent? | Additional comments? |
| --- | --- | --- | --- | --- |
| CE6-4.1a | Tencent | Yes | CTC results are confirmed |  |
| CE6-4.1b | MediaTek |  | Yes |  |
| CE6-4.1c | Samsung | Yes | Testing results were verifeid and all Bd-reates are exactly matched. |  |
| CE6-4.1d | Sony | Yes | RA/LB results exactly matches with those of the proponent. |  |
| CE6-4.1e | Tencent | Yes | CTC results are confirmed |  |
| CE6-4.2a | Huawei | Yes | All data points are confirmed |  |
| CE6-4.3a | Huawei | Yes | All data points are confirmed |  |
| CE6-4.4a | Qualcomm | Yes | Crosschecker's results match the results provided by the proponent |  |
| CE6-1.6c | Tencent | Yes | CTC results are confirmed |  |

## Related contributions

| Document | Related CE Test | Title |
| --- | --- | --- |

# CE6.5: Secondary transform

## Summary of results

The following table summarizes the results for CE6-1 using CTC configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-5.1 (Test 1) | JVET-M0292 | -1.59% | -2.75% | -3.22% | 128% | 98% | -0.88% | -1.88% | -2.27% | 108% | 100% | -0.24% | -0.83% | -1.19% | 104% | 101% |
| CE6-5.1 (Test 2) | JVET-M0292 | -1.40% | -2.60% | -3.09% | 128% | 97% | -0.76% | -1.83% | -2.25% | 108% | 100% | -0.21% | -0.78% | -0.85% | 104% | 98% |
| CE6-5.1 (Test 3) | JVET-M0292 | -1.25% | -2.48% | -2.88% | 125% | 97% | -0.69% | -1.59% | -2.03% | 107% | 100% | -0.18% | -0.68% | -1.11% | 103% | 100% |
| CE6-5.1 (Test 4) | JVET-M0292 | -1.34% | -2.50% | -2.95% | 129% | 97% | -0.71% | -1.60% | -2.02% | 107% | 100% | -0.18% | -0.35% | -1.00% | 104% | 99% |

The following table summarizes the results for CE6-1 using CTC w/ Inter MTS configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-5.1 (Test 1) | JVET-M0292 |  |  |  |  |  | -0.91% | -1.94% | -2.34% | 106% | 100% | -0.29% | -0.84% | -1.37% | 103% | 100% |
| CE6-5.1 (Test 2) | JVET-M0292 |  |  |  |  |  | -0.80% | -1.89% | -2.27% | 106% | 99% | -0.24% | -0.66% | -1.21% | 103% | 98% |
| CE6-5.1 (Test 3) | JVET-M0292 |  |  |  |  |  | -0.74% | -1.61% | -2.03% | 105% | 99% | -0.22% | -0.64% | -1.21% | 102% | 100% |
| CE6-5.1 (Test 4) | JVET-M0292 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## Test definition

CE6-5.1: JVET-L0133 CE 6-2.1: Reduced Secondary Transform (RST) [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

It will be performed to search best configurations of RST parameters, such as the number of transform kernels, transform set, and matrix dimension, while maintaining reasonable complexity. Furthermore, RST index coding is also to be optimized by exploiting the best configurations in order to maximize coding performance.

In JVET-M0292, test results of CE 6-5.1 on reduced secondary transform (RST) are reported. According to the request of the previous 12th JVET meeting, all tests were performed without normative change of MTS signaling, which are described as the following:

- Test 1:  (A),

- Test 2:  (A) + (B),

- Test 3:  (A) + (B) + (C),

- Test 4:  (A) + (B) + (D),  where

| Feature | Description |
| --- | --- |
| (A) | 4 transform sets (instead of 35). 2 transforms per set. |
| (B) | Restricted RST to allow maximum 8 multiplications/pixel |
| (C) | RST is disabled for 4x4 TU. |
| (D) | 16x48 matrices are employed instead of 16x64 ones. |

## Crosscheck reports

| Test # | Crosscheckers | Implementation is aligned with the algorithm description? | Does crosschecking confirm whether ALL data points match with the results provided by proponent? | Additional comments? |
| --- | --- | --- | --- | --- |
| CE6-5.1 | Orange, bcom | The software matches the CE description | Results were verified for CTC (AI,RA & LD) and when MTS is enabled for Inter coding modes for test1, test 2 and test3. (combination of features A, A+B and A+B+C resp.) Bd-rate results are exact; the execution times are within the measurement error range. | none |

## Related contributions

| Document | Related CE Test | Title |
| --- | --- | --- |

# References

- A. Said and X. Zhao, “Core Experiment 6: Arithmetic Coding Engine,” JVET-L1026, 12th meeting, Macao, CN, 3–12 Oct. 2018.