| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0028-v3 |
| --- | --- |

| Title: | CE8: Summary Report on Screen Content Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Yung-Hsuan Chao Yu-Chen Sun Jizheng Xu | Tel: Email: | xiaozhongxu@tencent.com yunghsua@qti.qualcomm.com yuchen.s@alibaba-inc.com xujizheng@bytedance.com |
| Source: | CE8 coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report of Core Experiment 8 on screen content coding. 18 tests have been tested in CE8 in between JVET-L and JVET-M meetings, to study and evaluate technologies related to screen content coding. In this report, coding performance and complexity of these tests are reported and analyzed. In particular, test results against 1) VTM-3.0+CPR; 2) VTM-3.0+CPR+PLT; 3) VTM-3.0+ PLT anchors are provided to show the coding efficiency and complexity trade-off of each tool. Crosschecking results for the performed tests are integrated in this contribution.

Changes in this revision relative to the v1 version are:

- Update on summary spread sheet to include more crosscheck reports.

- Include one more CE8 related contribution

# Introduction

As is agreed in Macao meeting, the following tests in table 1 are performed in CE8.

Table 1: Summary of tests performed in CE8

| Test | Tester | Document | Tool description | Cross checker |
| --- | --- | --- | --- | --- |
| 8.1.1a | J. Nam (LGE) | JVET-M0332 | Block vector prediction for merge mode and AMVP mode using default positions. | X. Xu (Tencent) |
| 8.1.1b | J. Nam (LGE) | JVET-M0332 | Block vector prediction for merge mode and AMVP mode. Consider alternative candidates with various positions | A. Karabutov (Huawei) |
| 8.1.2a | X. Xu (Tencent) | JVET-M0407 | Reference sample memory reuse, update reference sample memory on 64x64 basis, keep the requirement for total memory (4 64x64 blocks) unchanged | A. Karabutov (Huawei) |
| 8.1.2b | X. Xu (Tencent) | JVET-M0408 | Reference sample memory reuse, update reference sample memory on 64x64 basis, reduce the requirement for total memory to 3 64x64 blocks | J. Nam (LGE) |
| 8.1.2c | X. Xu (Tencent) | JVET-M0408 | Reference sample memory reuse, update reference sample memory on 64x64 basis, reduce the requirement for total memory to 2 64x64 blocks | J. Nam (LGE) |
| 8.1.2d | X. Xu (Tencent) | JVET-M0407 | Reference sample memory reuse, update reference sample memory on CU basis, keep the requirement for total memory (4 64x64 blocks) unchanged | A. Karabutov (Huawei) |
| 8.1.3 | L. P. Van (Qualcomm) | JVET-M0474 | CPR using extended search range with line buffers from top of CTU and the left columns of the current CTU |  |
| 8.2.1 | Y.-H. Chao (Qualcomm) Y.-C. Sun (Alibaba) | JVET-M0050 | Palette mode as in HEVC SCC, CE base software | R. Chernyak (Huawei) |
| 8.2.2 | Y.-C. Sun (Alibaba) | JVET-M0051 | Palette mode and intra mode combination | Y.-W. Chen (Kwai) |
| 8.2.3 | J. Ye (Tencent) | JVET-M0455 | Palette index map scan order constraints | R. Chernyak (Huawei) |
| 8.2.4b | J. Ye (Tencent) | JVET-M0456 | Apply palette mode on separate chroma CU only when corresponding luma samples are all coded in palette mode | Y.-C. Sun (Alibaba) |
| 8.2.5a | R. Chernyak (Huawei) Y.-H. Chao (Qualcomm) Y.-C. Sun (Alibaba) | JVET-M0052 | Separate palette coding for luma and chroma Sub test 1: palette coding is applied separately on luma and chroma components in both intra and inter slices when dual tree is disabled in configuration (SPS) | J. Ye (Tencent) |
| 8.2.5b | R. Chernyak (Huawei) Y.-H. Chao (Qualcomm) Y.-C. Sun (Alibaba) | JVET-M0052 | Separate palette coding for luma and chroma Sub test 2: palette coding is applied separately on luma and chroma components in both intra and inter slices when dual tree is enabled in configuration (SPS) | J. Ye (Tencent) |
| 8.2.6 | J. Ye (Tencent) | JVET-M0457 | Palette predictor list enhancement by using palette coded spatial neighbors | Y.-C. Sun (Alibaba) |
| 8.3.1a | F. Henry (Orange) | JVET-M0056 | BDPCM with throughput reduction, investigating: - Block size limitation of BDPCM and impact on throughput - Independently decodable regions inside BDPCM block (in a manner similar to RDPCM throughput reduction proposed in JCT-VC M0439 [16]) and impact on throughput | B. Bross (HHI) |
| 8.3.1b | F. Henry (Orange) | JVET-M0057 | Same as 8.3.1a with vertical and horizontal predictors (in a way similar to RDPCM), allowing the reconstruction to proceed line by line / column by column and thus increase throughput | B. Bross (HHI) |
| 8.3.2 | F. Henry (Orange) | JVET-M0058 | BDPCM bin/context reduction - Use different approaches to encode the BDPCM residual in order to reduce the number of bins and associated contexts | B. Bross (HHI) |

The detailed description of each tools is provided in the next section.

# Description of CE tools

## CE8.1.1 (JVET-M0332)

In this test, alternative candidates for CPR is proposed. It is added in both AMVP and merge mode. For AMVP mode, when reference picture of current coding block indicated by reference index is same as current picture and the number of candidates in the constructed list is smaller than the maximum number of candidates, default candidates are inserted into MVP candidate list. For Merge mode, when the current picture exists in the reference picture list and the number of candidates in the constructed list is smaller than the maximum number of candidates, default candidates are added to merge candidate list. Various positions for alternative candidates will be tested.

In subtest 1, (-2W, 0) and (0, -2H) are tested, where (W, H) is the size of current coding block

In subtest 2, (-mid, 0) and (0, -mid)) are tested, which is the middle positions between the CTU boundary and current block position.

## CE8.1.2 (JVET-M0407, JVET-M0408)

X

Curr

X

X

Curr

X

X

X

X

Curr

X

X

Curr

X

Currently, the search range of CPR mode is constrained to be within the current CTU. The effective memory requirement to store reference samples for CPR mode is 1 CTU size of samples. Considering the existing reference sample memory to store reconstructed samples in current 64x64 region, 3 more 64x64 sized reference sample memory are required. Based on this fact, the proposed method in JVET-L0297 extends the effective search range of the CPR mode to some part of the left CTU while the total memory requirement for storing reference pixels are kept unchanged (1 CTU size, 4 64x64 reference sample memory in total). This is done by updating the stored reference samples from the left CTU to the reconstructed samples from the current CTU:

1:) The update process is done on a 64x64 luma sample basis, for each of the four 64x64 block regions in the CTU size memory, the reference samples in the same region from the left CTU can be used to predict the coding block in current CTU with CPR mode until any of the blocks in the same region of the current CTU is being coded or has been coded.

2:) Similar as in 1:), but the required reference sample memory size is reduced. For example, in addition to the 64x64 size memory for storing reconstructed samples of the current 64x64 region, additional 2 (or 1) 64x64 size memory can be used to store previously coded regions. Therefore, the total requirement reference sample memory is reduced from 4 64x64 size to 3 (or 2) 64x64 size.

3:) Similar as in 1:), but the update process is done on a CU basis. The reference samples in the left CTU can be used to predict the coding block in current CTU with CPR mode until the block in the same location of the current CTU is being coded or has been coded.

## CE8.1.3 (JVET-M0474)

The current CPR reference area is limited to the reconstructed samples of the current CTU. However, neighbor samples around a CTU are required for intra prediction, that samples can be available for CPR reference as well. In the CE, the following tests are performed:

1. The search range is the current CTU and 1 line above, and 1 left column to the current CTU

## CE8.2.1 (JVET-M0050)

Palette coding as in HEVC SCC.

For dual tree enabled in SPS, palette coding is applied separately for luma tree and chorma tree in intra slice/P slice where current picture is the only reference and jointly for luma and chroma in inter slice. For dual tree disabled in SPS, palette coding is applied jointly for both intra and inter slice.

## CE8.2.2 (JVET-M0051)

In this test, the method combining palette mode and intra prediction is tested. The decoder first derives the prediction block based on the intra prediction information. Then, the decoder decodes a palette and an index map. Using the decoding palette information, the decoder refines the prediction block and reconstructs the block.

## CE8.2.3 (JVET-M0455)

In this test, index map scan order is constraint by the block shape. it is proposed when the block height / width ratio is greater than a threshold, only vertical traverse scan is used in the index map scan. If the block width / height ratio is greater than or equal to a threshold, only horizontal traverse scan is used. Different thresholds will be tested. In the CE proposal, a threshold equal to 8 is tested. For these blocks, a scan order starting along the longer side of the block is used. For example, a 64x8 block will use horizontal traverse scan.

## CE8.2.4 (JVET-M0456)

In this test, palette mode with dual tree coding structure is investigated.

Sub test b: apply palette mode to luma plane when dual tree is enabled. When coding chroma plane, if the co-located luma block are all palette mode, the chroma block are also have the flexibility to use palette mode. A flag is signaled whether the chroma block using palette mode or not. If the chroma block using palette mode, the corresponding palette mode syntax will be signaled.

## CE8.2.5 (JVET-M0052)

In the palette anchor (test CE8.2.1), when dual tree is enabled in the configuration (SPS), palette coding is applied separately on luma tree and chroma tree in intra slices and jointly on luma/chroma in inter slices. When dual tree is disabled in the configuration, palette mode is applied jointly for both luma and chroma in all slice types.

In this test, separated palette coding for luma/chroma components are investigated based on the same palette coding functions in anchor software:

Sub test 1: palette coding is applied separately on luma and chroma components in both intra and inter slices when dual tree is disabled in configuration (SPS)

Sub test 2: palette coding is applied separately on luma and chroma components in both intra and inter slices when dual tree is enabled in configuration (SPS)

## CE8.2.6 (JVET-M0457)

In this test, the palette predictor will be derived from previously palette coded coding blocks.

1. Derive the spatial palette predictor：

To derive the spatial palette predictor, the adjacent neighboring and non-adjacent neighboring are both checked from the neighboring blocks that are close to the current block to the blocks that are far away. The left block (Ai) and above block (Bi) are checked. The non-adjacent neighboring candidates are in a virtual box that surrounding the current block. The virtual block size and position are illustrated in Fig.1. In the current implementation, the gridX is block width and gridY is block height. The number of search rounds will be tested. Pruning of palette entries will be tested. The orders for checking each candidate will be tested.

2. Combine the spatial palette predictor and the HEVC SCC palette predictor:

After deriving the spatial palette predictor, the spatial palette predictor will be inserted into the palette predictor list for the current block first. If the size of palette predictor list for the current block doesn’t exceed the maximum palette predictor size, the HEVC SCC palette predictor is inserted into the palette predictor list. If the list size exceeds the maximum palette predictor size, the rest palette entry will be discarded. The combined palette predictor will be used to code the palette table entry of the current block.

## CE8.3.1 (JVET-M0056, JVET-M0057)

It is contribution proposes to use a classical DPCM approach at the block level.

A bdpcm_flag is transmitted at the CU level whenever it is a luma intra CU having each dimension smaller or equal to 32. This flag indicates whether regular intra coding or DPCM is used. This flag is encoded using a single CABAC context.

Block DPCM uses the Median Edge Detector of LOCO-I. For a current pixel X having pixel A as left neighbor, pixel B as top neighbor and C as top-left neighbor, the prediction P(X) is determined by

P(X)=	min(A,B) if C≥max(A,B)

max(A,B) if C≤min(A,B)

A+B-C otherwise

The predictor uses unfiltered reference pixels when predicting the top row and left column of the CU. The predictor then uses reconstructed pixels for the rest of the CU. Pixels are processed in raster-scan order inside the CU. The prediction error is quantized in the spatial domain, after rescaling, in a way identical to the Transform Skip quantizer. Each pixel is reconstructed by adding the dequantized prediction error to the prediction. Thus, the reconstructed pixels are used to predict the next pixels in raster-scan order. Amplitude and signs of the quantized prediction error are encoded separately.  A cbf_bdpcm_flag is coded. If it is equal to 0, it indicates that all amplitudes of the block are to be decoded as zero. If the flag is equal to 1, all amplitudes of the block are encoded individually in raster-scan order. In order to keep complexity low, the amplitude is limited to at most 31 (inclusive). The amplitude is encoded using unary binarization, with three contexts for the first bit, then one context for each additional bin until the 12th bin, and one context for all remaining bins. A sign is encoded in bypass mode for each non-zero residue.

In order to maintain the coherence of the regular intra mode prediction, the first mode in the MPM list is associated with a Block-DPCM CU (without being transmitted) and is available for MPM generation for subsequent blocks.

The deblocking filter is de-activated on a border between two Block-DPCM blocks, since neither of the blocks uses the transform stage usually responsible for blocking artifacts.

Block-DPCM does not use any other step than the ones described here. In particular it does not use any transform.

This test will investigate the throughput improvements of BDPCM and the interaction with other SCC tools.

Additional details about the tests are provided in the following subsections.

### Encoder modifications

The following encoder-only modifications were implemented in all tests:

- Full RD check of intra modes: BDPCM is systematically checked before the intra modes during the full-RD check. Then the first intra mode is checked. If BDPCM is better, the process is stopped.

- BDPCM RDOQ: a simple form of RDOQ is implemented for test 8.3.1b as the horizontal/vertical predictor simplifies the RD cost computation of alternate quantization levels.

### Test 8.3.1a

The smaller blocks (4x4, 4x8 and 8x4) are divided into two independently decodable areas, along a diagonal in the middle of the block (see below diagram). Pixels from one area cannot use pixels from another one, they use reference pixels instead.

### Test 8.3.1b

In this test the JPEG-LS predictor is replaced with a simple vertical or horizontal predictor (chosen and signalled at block level). The shape of the independently decodable regions has been changed to reflect the geometry of the predictor (see diagram below).

## CE8.3.2 (JVET-M0058)

In this test the BDPCM residue amplitude is limited to 28, and it is encoded with truncated unary binarization for the first 12 bits, followed by order-2 Exp-Golomb EP bits for the remainder (using the existing encodeRemAbsEP() function). As a basis for this test we used 8.3.1b (in order to limit the number of configurations to crosscheck).

# CE results summary

Tt is agreed that VTM version VTM-3.0 shall be used for these experiments. The PLT software used in this CE is also based on VTM-3.0.

- The proposed methods in this CE shall be tested with VTM configurations and CPR=1.

- The proposed methods in this CE should be compared with CPR turned on (VTM config + CPR=1) and CPR turned off (VTM config).

The mandatory common test conditions (CTC) as specified in the JVET-L1010 document from this meeting shall be followed to perform the simulations in this CE.

The mandatory test sequences as described in common test conditions shall be used for these tests.

In addition to that, to evaluate the benefit of proposed tools on screen content materials, the sequences in TGM YUV 4:2:0 1080p category from HEVC SCC CTC JCT-VC-U1015 document and class F in common test conditions shall be tested. The details of these sequences are listed in the table below.

Table 3: screen content test sequences used in CE8

| Category | Resolution | Sequence name | fps | Bit depth | Frames to be encoded | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4:2:0 TGM | 1920x1080 1920x1080 1920x1080 1920x1080 | FlyingGraphics* Desktop Console ChineseEditing | 60 60 60 60 | 8 8 8 8 | 0-299* 0-599 0-599 0-599 | M M M M | M M M M | O O O O |
| Class F | 832x480 1024x768 1280x720 1280x720 | BasketballDrillText ChinaSpeed SlideEditing SlideShow | 50 30 30 20 | 8 8 8 8 | 0-499 0-499 0-299 0-499 | M M M M | M M M M | O O O O |
| *Note that only the first 300 frames of this sequence are used. M refers to mandatory test in the configuration; O refers to optional test in the configuration |  |  |  |  |  |  |  |  |

The followings are summary tables of the tests in this CE. To incorporate different tests together for easy discussion, all the results shown in this report are compared against VTM-3.0 (with CPR=0, PLT=0)

Table 4: CE8 test results with VTM+CPR=1

|  |  |  |  |  | AI Over VTM-3.0 |  |  |  |  |  |  |  |  |  | RA Over VTM-3.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test# |  | Y |  | U |  | V |  | EncT |  | DecT |  | Y |  | U |  | V |  | EncT |  |  | DecT |
| CTC overall | VTM+CPR | -0.27% | -0.40% | -0.33% | 137% | 100% | 0.09% | -0.01% | 0.05% | 100% | 100% |
|  |  |  |  | CE8.1.1a |  | -0.34% |  | -0.43% |  | -0.41% |  | 144% |  | 104% |  | 0.05% |  | -0.06% |  | 0.00% |  | 101% |  |  | 101% |
|  |  |  |  | CE8.1.1b |  | -0.34% |  | -0.41% |  | -0.37% |  | 144% |  | 104% |  | 0.05% |  | 0.00% |  | 0.05% |  | 101% |  |  | 102% |
|  |  |  |  | CE8.1.2a |  | -0.35% |  | -0.46% |  | -0.42% |  | 142% |  | 102% |  | 0.05% |  | -0.04% |  | -0.03% |  | 100% |  |  | 99% |
|  |  |  |  | CE8.1.2b |  | -0.31% |  | -0.41% |  | -0.35% |  | 143% |  | 98% |  | 0.07% |  | -0.04% |  | 0.04% |  | 99% |  |  | 98% |
|  |  |  |  | CE8.1.2c |  | -0.22% |  | -0.30% |  | -0.30% |  | 139% |  | 100% |  | 0.11% |  | -0.07% |  | 0.08% |  | 99% |  |  | 99% |
|  |  |  |  | CE8.1.2d |  | -0.39% |  | -0.48% |  | -0.48% |  | 143% |  | 99% |  | 0.05% |  | 0.01% |  | 0.02% |  | 99% |  |  | 99% |
|  |  |  |  | CE8.1.3 |  | -0.29% |  | -0.37% |  | -0.34% |  | 137% |  | 103% |  | 0.08% |  | 0.01% |  | 0.02% |  | 100% |  |  | 100% |
|  |  |  |  | CE8.2.1 |  | -0.19% |  | -0.26% |  | -0.26% |  | 146% |  | 105% |  | 0.20% |  | 0.07% |  | 0.20% |  | 108% |  |  | 104% |
|  |  |  |  | CE8.2.1* |  | -0.36% |  | -0.65% |  | -0.81% |  | 142% |  | 100% |  | 0.16% |  | -0.30% |  | -0.22% |  | 107% |  |  | 102% |
|  |  |  |  | CE8.2.2 |  | -0.19% |  | -0.24% |  | -0.26% |  | 147% |  | 105% |  | 0.22% |  | 0.07% |  | 0.18% |  | 106% |  |  | 103% |
|  |  |  |  | CE8.2.3 |  | -0.19% |  | -0.26% |  | -0.25% |  | 142% |  | 104% |  | 0.20% |  | 0.05% |  | 0.15% |  | 100% |  |  | 99% |
|  |  |  |  | CE8.2.4 |  | -0.21% |  | -0.30% |  | -0.26% |  | 141% |  | 105% |  | 0.20% |  | 0.10% |  | 0.22% |  | 101% |  |  | 99% |
|  |  |  |  | CE8.2.5a |  | -0.36% |  | -0.63% |  | -0.81% |  | 142% |  | 100% |  | 0.15% |  | -0.32% |  | -0.24% |  | 106% |  |  | 101% |
|  |  |  |  | CE8.2.5b |  | -0.19% |  | -0.26% |  | -0.26% |  | 146% |  | 105% |  | 0.20% |  | 0.05% |  | 0.13% |  | 105% |  |  | 103% |
|  |  |  |  | CE8.2.6 |  | -0.19% |  | -0.27% |  | -0.26% |  | 143% |  | 104% |  | 0.20% |  | 0.08% |  | 0.18% |  | 101% |  |  | 100% |
|  |  |  |  | CE8.3.1a |  | -0.24% |  | -0.42% |  | -0.36% |  | 134% |  | 104% |  | 0.11% |  | 0.03% |  | 0.09% |  | 101% |  |  | 102% |
|  |  |  |  | CE8.3.1b |  | -0.34% |  | -0.37% |  | -0.32% |  | 137% |  | 106% |  | 0.07% |  | -0.01% |  | 0.09% |  | 102% |  |  | 102% |
|  |  |  |  | CE8.3.2 |  | -0.34% |  | -0.37% |  | -0.33% |  | 137% |  | 106% |  | 0.07% |  | -0.01% |  | 0.08% |  | 102% |  |  | 102% |
| Class F | VTM+CPR | -12.09% | -12.04% | -12.10% | 158% | 99% | -9.89% | -9.92% | -9.95% | 107% | 99% |
|  | CE8.1.1a | -12.42% | -12.24% | -12.27% | 161% | 104% | -10.10% | -9.99% | -10.12% | 108% | 102% |
|  | CE8.1.1b | -12.41% | -12.37% | -12.34% | 162% | 106% | -10.07% | -9.94% | -10.16% | 108% | 102% |
|  | CE8.1.2a | -14.47% | -14.32% | -14.26% | 175% | 103% | -11.62% | -11.63% | -11.49% | 109% | 98% |
|  | CE8.1.2b | -13.66% | -13.52% | -13.49% | 172% | 97% | -11.04% | -11.05% | -11.07% | 109% | 97% |
|  | CE8.1.2c | -12.11% | -12.04% | -11.94% | 164% | 97% | -9.88% | -9.89% | -10.04% | 108% | 98% |
|  | CE8.1.2d | -15.34% | -15.20% | -15.15% | 178% | 96% | -12.34% | -12.28% | -12.22% | 109% | 97% |
|  |  |  | CE8.1.3 |  |  | -12.21% |  | -12.14% |  | -12.19% |  | 159% |  | 104% |  | -9.96% |  | -10.10% |  | -9.99% |  | 106% |  | 101% |
|  |  |  | CE8.2.1 |  |  | -15.51% |  | -14.05% |  | -14.13% |  | 174% |  | 97% |  | -11.90% |  | -11.77% |  | -11.99% |  | 119% |  | 103% |
|  | CE8.2.1* | -13.16% | -13.63% | -13.62% | 159% | 96% | -10.48% | -11.17% | -11.20% | 118% | 102% |
|  | CE8.2.2 | -15.53% | -14.03% | -14.11% | 174% | 97% | -11.94% | -11.72% | -11.94% | 119% | 103% |
|  | CE8.2.3 | -15.50% | -14.03% | -14.09% | 173% | 100% | -11.90% | -11.72% | -11.92% | 110% | 98% |
|  |  |  |  | CE8.2.4 |  | -15.62% |  | -13.29% |  | -13.31% |  | 169% |  | 99% |  | -12.03% |  | -11.00% |  | -11.12% |  | 110% |  |  | 98% |
|  |  |  |  | CE8.2.5a |  | -14.59% |  | -14.28% |  | -14.61% |  | 167% |  | 93% |  | -11.43% |  | -11.08% |  | -11.70% |  | 118% |  |  | 99% |
|  | CE8.2.5b | -15.51% | -14.05% | -14.13% | 174% | 97% | -11.97% | -11.64% | -11.89% | 118% | 104% |
|  |  |  |  | CE8.2.6 |  | -15.49% |  | -13.98% |  | -14.09% |  | 175% |  | 100% |  | -11.87% |  | -11.53% |  | -11.90% |  | 110% |  |  | 99% |
|  |  |  |  | CE8.3.1a |  | -13.09% |  | -13.59% |  | -13.60% |  | 136% |  | 103% |  | -10.41% |  | -10.92% |  | -10.98% |  | 109% |  |  | 101% |
|  |  |  |  | CE8.3.1b |  | -15.75% |  | -14.12% |  | -14.14% |  | 139% |  | 106% |  | -12.48% |  | -11.27% |  | -11.42% |  | 111% |  |  | 102% |
|  | CE8.3.2 | -15.68% | -14.05% | -14.06% | 139% | 106% | -12.43% | -11.30% | -11.34% | 110% | 102% |
| SCC 1080p | VTM+CPR | -36.92% | -36.11% | -36.36% | 143% | 90% | -22.03% | -21.58% | -22.10% | 103% | 96% |
|  |  |  |  | CE8.1.1a |  | -37.81% |  | -36.76% |  | -37.03% |  | 139% |  | 102% |  | -22.77% |  | -22.27% |  | -22.78% |  | 104% |  |  | 100% |
|  |  |  |  | CE8.1.1b |  | -37.80% |  | -36.76% |  | -37.03% |  | 138% |  | 101% |  | -22.69% |  | -22.18% |  | -22.73% |  | 104% |  |  | 100% |
|  |  |  |  | CE8.1.2a |  | -42.96% |  | -41.68% |  | -41.99% |  | 147% |  | 90% |  | -25.89% |  | -25.17% |  | -25.82% |  | 103% |  |  | 95% |
|  |  |  |  | CE8.1.2b |  | -41.02% |  | -39.81% |  | -40.12% |  | 151% |  | 86% |  | -24.55% |  | -23.96% |  | -24.54% |  | 104% |  |  | 94% |
|  |  |  |  | CE8.1.2c |  | -36.76% |  | -35.55% |  | -35.83% |  | 150% |  | 89% |  | -21.43% |  | -20.80% |  | -21.35% |  | 105% |  |  | 94% |
|  |  |  |  | CE8.1.2d |  | -45.00% |  | -43.77% |  | -44.06% |  | 151% |  | 85% |  | -27.25% |  | -26.61% |  | -27.20% |  | 103% |  |  | 94% |
|  |  |  |  | CE8.1.3 |  | -37.14% |  | -36.32% |  | -36.57% |  | 144% |  | 97% |  | -22.17% |  | -21.75% |  | -22.27% |  | 105% |  |  | 100% |
|  |  |  |  | CE8.2.1 |  | -44.77% |  | -41.06% |  | -41.27% |  | 157% |  | 75% |  | -25.67% |  | -24.47% |  | -24.82% |  | 110% |  |  | 96% |
|  |  |  |  | CE8.2.1* |  | -39.08% |  | -38.45% |  | -38.66% |  | 149% |  | 80% |  | -22.67% |  | -22.57% |  | -22.80% |  | 107% |  |  | 93% |
|  |  |  |  | CE8.2.2 |  | -45.18% |  | -41.22% |  | -41.45% |  | 155% |  | 76% |  | -26.03% |  | -24.68% |  | -25.04% |  | 110% |  |  | 95% |
|  |  |  |  | CE8.2.3 |  | -44.75% |  | -41.03% |  | -41.26% |  | 156% |  | 77% |  | -25.64% |  | -24.50% |  | -24.80% |  | 102% |  |  | 90% |
|  |  |  |  | CE8.2.4 |  | -44.78% |  | -41.04% |  | -41.26% |  | 158% |  | 80% |  | -25.65% |  | -24.40% |  | -24.77% |  | 102% |  |  | 91% |
|  |  |  |  | CE8.2.5a |  | -41.75% |  | -40.55% |  | -40.56% |  | 157% |  | 70% |  | -24.88% |  | -24.46% |  | -24.44% |  | 106% |  |  | 91% |
|  |  |  |  | CE8.2.5b |  | -44.77% |  | -41.06% |  | -41.27% |  | 157% |  | 75% |  | -26.03% |  | -24.75% |  | -24.95% |  | 108% |  |  | 95% |
|  |  |  |  | CE8.2.6 |  | -44.78% |  | -41.04% |  | -41.26% |  | 158% |  | 80% |  | -25.65% |  | -24.40% |  | -24.77% |  | 102% |  |  | 91% |
|  |  |  |  | CE8.3.1a |  | -38.67% |  | -38.38% |  | -38.57% |  | 104% |  | 94% |  | -23.03% |  | -22.87% |  | -23.29% |  | 105% |  |  | 98% |
|  |  |  |  | CE8.3.1b |  | -42.26% |  | -39.36% |  | -39.70% |  | 107% |  | 101% |  | -25.50% |  | -23.29% |  | -23.84% |  | 106% |  |  | 99% |
|  |  |  |  | CE8.3.2 |  | -41.97% |  | -39.22% |  | -39.55% |  | 107% |  | 102% |  | -25.45% |  | -23.27% |  | -23.78% |  | 104% |  |  | 99% |

* PLT mode in HEVC SCC, when dual-tree SPS is off

Table 5: CE8 test results with VTM+CPR=1+PLT=1

|  |  |  |  | AI Over VTM-3.0 |  |  |  |  |  | RA Over VTM-3.0 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test# |  | Y |  | U | V | EncT | DecT | Y |  |  | U |  | V | EncT | DecT |
| CTC overall | VTM+CPR+PLT | -0.19% | -0.26% | -0.26% | 145% | 103% | 0.20% | 0.07% | 0.20% | 105% | 102% |
|  |  | CE8.1.1a |  | -0.25% |  | -0.32% | -0.36% | 151% | 104% | 0.17% |  |  | 0.06% |  | 0.05% | 106% | 102% |
|  |  | CE8.1.1b |  | -0.24% |  | -0.30% | -0.30% | 151% | 105% | 0.18% |  |  | 0.09% |  | 0.04% | 105% | 101% |
|  |  | CE8.1.2a |  | -0.26% |  | -0.33% | -0.37% | 149% | 101% | 0.19% |  |  | 0.11% |  | 0.17% | 104% | 99% |
|  |  | CE8.1.2b |  | -0.22% |  | -0.31% | -0.30% | 152% | 104% | 0.21% |  |  | 0.05% |  | 0.15% | 106% | 102% |
|  |  | CE8.1.2c |  | -0.14% |  | -0.19% | -0.21% | 150% | 105% | 0.23% |  |  | 0.07% |  | 0.17% | 106% | 102% |
|  |  | CE8.1.2d |  | -0.30% |  | -0.37% | -0.38% | 152% | 105% | 0.18% |  |  | 0.08% |  | 0.12% | 105% | 102% |
|  |  | CE8.1.3 |  | -0.19% |  | -0.31% | -0.25% | 144% | 103% | 0.21% |  |  | 0.05% |  | 0.16% | 104% | 100% |
|  |  | CE8.2.1 |  | -0.19% |  | -0.26% | -0.26% | 146% | 105% | 0.20% |  |  | 0.07% |  | 0.20% | 108% | 104% |
|  |  | CE8.2.2 |  | -0.19% |  | -0.24% | -0.26% | 147% | 105% | 0.22% |  |  | 0.07% |  | 0.18% | 106% | 103% |
|  |  | CE8.2.3 |  | -0.19% |  | -0.26% | -0.25% | 142% | 104% | 0.20% |  |  | 0.05% |  | 0.15% | 100% | 99% |
|  |  | CE8.2.4 |  | -0.21% |  | -0.30% | -0.26% | 141% | 105% | 0.20% |  |  | 0.10% |  | 0.22% | 101% | 99% |
|  |  | CE8.2.5a |  | -0.36% |  | -0.63% | -0.81% | 142% | 100% | 0.15% |  |  | -0.32% |  | -0.24% | 106% | 101% |
|  |  | CE8.2.5b |  | -0.19% |  | -0.26% | -0.26% | 146% | 105% | 0.20% |  |  | 0.05% |  | 0.13% | 105% | 103% |
|  |  | CE8.2.6 |  | -0.19% |  | -0.27% | -0.26% | 143% | 104% | 0.20% |  |  | 0.08% |  | 0.18% | 101% | 100% |
|  |  | CE8.3.1a |  | -0.16% |  | -0.30% | -0.32% | 170% | 116% | 0.24% |  |  | 0.07% |  | 0.17% | 141% | 122% |
|  |  | CE8.3.1b |  | -0.27% |  | -0.29% | -0.24% | 141% | 109% | 0.18% |  |  | 0.14% |  | 0.24% | 104% | 105% |
|  |  | CE8.3.2 |  | -0.27% |  | -0.28% | -0.24% | 141% | 109% | 0.18% |  |  | 0.15% |  | 0.26% | 105% | 106% |
| Class F | VTM+CPR+PLT | -15.51% | -14.05% | -14.13% | 173% | 97% | -11.90% | -11.77% | -11.99% | 115% | 100% |
|  | CE8.1.1a | -15.61% | -14.25% | -14.33% | 176% | 97% | -12.01% | -11.77% | -12.05% | 116% | 100% |
|  | CE8.1.1b | -15.62% | -14.18% | -14.25% | 176% | 98% | -12.02% | -11.78% | -11.95% | 116% | 100% |
|  | CE8.1.2a | -16.95% | -15.79% | -15.92% | 182% | 95% | -13.09% | -13.00% | -13.18% | 115% | 97% |
|  | CE8.1.2b | -16.44% | -15.15% | -15.25% | 185% | 99% | -12.66% | -12.51% | -12.73% | 118% | 101% |
|  | CE8.1.2c | -15.29% | -13.87% | -13.97% | 179% | 97% | -11.79% | -11.59% | -11.76% | 117% | 100% |
|  | CE8.1.2d | -17.71% | -16.58% | -16.68% | 191% | 100% | -13.66% | -13.50% | -13.75% | 118% | 101% |
|  |  | CE8.1.3 |  | -15.57% |  | -14.12% | -14.27% | 171% | 96% | -12.00% |  | -11.84% |  | -12.07% |  | 112% | 97% |
|  |  | CE8.2.1 |  | -15.51% |  | -14.05% | -14.13% | 174% | 97% | -11.90% |  | -11.77% |  | -11.99% |  | 119% | 103% |
|  | CE8.2.2 | -15.53% | -14.03% | -14.11% | 174% | 97% | -11.94% | -11.72% | -11.94% | 119% | 103% |
|  | CE8.2.3 | -15.50% | -14.03% | -14.09% | 173% | 100% | -11.90% | -11.72% | -11.92% | 110% | 98% |
|  |  | CE8.2.4 |  | -15.62% |  | -13.29% | -13.31% | 169% | 99% | -12.03% |  |  | -11.00% |  | -11.12% | 110% | 98% |
|  |  | CE8.2.5a |  | -14.59% |  | -14.28% | -14.61% | 167% | 93% | -11.43% |  |  | -11.08% |  | -11.70% | 118% | 99% |
|  |  | CE8.2.5b |  | -15.51% |  | -14.05% | -14.13% | 174% | 97% | -11.97% |  |  | -11.64% |  | -11.89% | 118% | 104% |
|  |  | CE8.2.6 |  | -15.49% |  | -13.98% | -14.09% | 175% | 100% | -11.87% |  |  | -11.53% |  | -11.90% | 110% | 99% |
|  |  | CE8.3.1a |  | -15.81% |  | -14.80% | -14.98% | 175% | 113% | -12.12% |  |  | -11.98% |  | -12.30% | 147% | 119% |
|  |  | CE8.3.1b |  | -16.81% |  | -15.47% | -15.63% | 143% | 104% | -12.95% |  |  | -12.24% |  | -12.56% | 113% | 104% |
|  | CE8.3.2 | -16.80% | -15.44% | -15.62% | 143% | 105% | -12.93% | -12.26% | -12.57% | 114% | 105% |
| SCC 1080p | VTM+CPR+PLT | -44.77% | -41.06% | -41.27% | 158% | 77% | -25.67% | -24.49% | -24.84% | 106% | 93% |
|  |  | CE8.1.1a |  | -45.08% |  | -41.40% | -41.60% | 155% | 80% | -26.14% |  |  | -24.92% |  | -25.30% | 108% | 95% |
|  |  | CE8.1.1b |  | -45.09% |  | -41.40% | -41.61% | 155% | 80% | -26.12% |  |  | -24.91% |  | -25.30% | 108% | 95% |
|  |  | CE8.1.2a |  | -48.16% |  | -44.73% | -44.97% | 156% | 75% | -28.04% |  |  | -26.75% |  | -27.18% | 104% | 91% |
|  |  | CE8.1.2b |  | -46.92% |  | -43.43% | -43.67% | 164% | 79% | -27.13% |  |  | -25.86% |  | -26.32% | 107% | 94% |
|  |  | CE8.1.2c |  | -43.87% |  | -40.16% | -40.35% | 164% | 77% | -24.73% |  |  | -23.54% |  | -23.90% | 109% | 94% |
|  |  | CE8.1.2d |  | -49.80% |  | -46.50% | -46.74% | 159% | 78% | -29.13% |  |  | -27.90% |  | -28.39% | 106% | 93% |
|  |  | CE8.1.3 |  | -44.89% |  | -41.21% | -41.42% | 157% | 77% | -25.74% |  |  | -24.58% |  | -24.94% | 107% | 95% |
|  |  | CE8.2.1 |  | -44.77% |  | -41.06% | -41.27% | 157% | 75% | -25.67% |  |  | -24.47% |  | -24.82% | 110% | 96% |
|  |  | CE8.2.2 |  | -45.18% |  | -41.22% | -41.45% | 155% | 76% | -26.03% |  |  | -24.68% |  | -25.04% | 110% | 95% |
|  |  | CE8.2.3 |  | -44.75% |  | -41.03% | -41.26% | 156% | 77% | -25.64% |  |  | -24.50% |  | -24.80% | 102% | 90% |
|  |  | CE8.2.4 |  | -45.03% |  | -40.16% | -40.41% | 158% | 82% | -25.77% |  |  | -23.77% |  | -24.18% | 102% | 91% |
|  |  | CE8.2.5a |  | -41.75% |  | -40.55% | -40.56% | 157% | 70% | -24.88% |  |  | -24.46% |  | -24.44% | 106% | 91% |
|  |  | CE8.2.5b |  | -44.77% |  | -41.06% | -41.27% | 157% | 75% | -26.03% |  |  | -24.75% |  | -24.95% | 108% | 95% |
|  |  | CE8.2.6 |  | -44.78% |  | -41.04% | -41.26% | 158% | 80% | -25.65% |  |  | -24.40% |  | -24.77% | 102% | 91% |
|  |  | CE8.3.1a |  | -45.25% |  | -42.15% | -42.34% | 130% | 93% | -24.20% |  |  | -23.13% |  | -23.45% | 133% | 111% |
|  |  | CE8.3.1b |  | -46.17% |  | -42.67% | -42.88% | 106% | 89% | -25.31% |  |  | -23.46% |  | -23.88% | 104% | 102% |
|  |  | CE8.3.2 |  | -46.16% |  | -42.65% | -42.86% | 106% | 90% | -25.30% |  |  | -23.48% |  | -23.88% | 105% | 99% |

Table 6: CE8 PLT related test results with VTM+PLT=1

|  |  |  |  |  | AI Over VTM-3.0 |  |  |  |  |  |  |  |  |  |  | RA Over VTM-3.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test# |  |  | Y |  |  | U |  | V |  | EncT |  | DecT |  | Y |  |  | U |  |  |  |  |  | V | EncT | DecT |
| CTC overall | VTM+PLT | 0.09% | 0.10% | 0.06% | 107% | 101% | 0.14% | 0.20% | 0.09% | 106% | 102% |
|  |  | CE8.2.1 |  |  | 0.09% |  |  | 0.10% |  | 0.06% |  | 107% |  | 101% |  | 0.14% |  |  | 0.20% |  |  |  |  |  | 0.09% | 106% | 102% |
|  |  | CE8.2.1* |  |  | 0.07% |  |  | 0.07% |  | 0.01% |  | 108% |  | 101% |  | 0.14% |  |  | 0.09% |  |  |  |  |  | 0.08% | 106% | 101% |
|  |  | CE8.2.2 |  |  | 0.08% |  |  | 0.09% |  | 0.11% |  | 108% |  | 103% |  | 0.14% |  |  | 0.18% |  |  |  |  |  | 0.10% | 106% | 102% |
|  |  | CE8.2.3 |  |  | 0.08% |  |  | 0.10% |  | 0.07% |  | 106% |  | 101% |  | 0.12% |  |  | 0.17% |  |  |  |  |  | 0.13% | 100% | 98% |
|  |  | CE8.2.4 |  |  | 0.06% |  |  | 0.07% |  | 0.05% |  | 106% |  | 102% |  | 0.13% |  |  | 0.16% |  |  |  |  |  | 0.10% | 100% | 98% |
|  |  | CE8.2.5a |  |  | 0.07% |  |  | 0.09% |  | 0.03% |  | 104% |  | 98% |  | 0.13% |  |  | 0.08% |  |  |  |  |  | 0.10% | 104% | 99% |
|  |  | CE8.2.5b |  |  | 0.09% |  |  | 0.10% |  | 0.06% |  | 111% |  | 105% |  | 0.11% |  |  | 0.18% |  |  |  |  |  | 0.12% | 105% | 101% |
|  |  | CE8.2.6 |  |  | 0.09% |  |  | 0.10% |  | 0.06% |  | 106% |  | 101% |  | 0.13% |  |  | 0.20% |  |  |  |  |  | 0.11% | 100% | 98% |
| Class F | VTM+PLT | -11.43% | -8.94% | -9.04% | 117% | 89% | -8.52% | -8.02% | -8.31% | 110% | 101% |
|  | CE8.2.1 | -11.43% | -8.94% | -9.04% | 117% | 89% | -8.52% | -8.02% | -8.31% | 110% | 101% |
|  | CE8.2.1* | -5.55% | -6.57% | -7.23% | 114% | 95% | -3.89% | -5.15% | -5.83% | 111% | 102% |
|  | CE8.2.2 | -11.65% | -9.08% | -9.19% | 118% | 91% | -8.80% | -8.28% | -8.66% | 110% | 101% |
|  | CE8.2.3 | -11.41% | -8.92% | -8.98% | 118% | 93% | -8.47% | -8.06% | -8.26% | 103% | 95% |
|  |  | CE8.2.4 |  |  | -11.50% |  |  | -8.30% |  | -8.35% |  | 117% |  | 93% |  | -8.56% |  |  | -7.60% |  |  |  |  |  | -7.78% | 102% | 95% |
|  |  | CE8.2.5a |  |  | -9.75% |  |  | -9.29% |  | -10.06% |  | 113% |  | 86% |  | -7.51% |  |  | -7.08% |  |  |  |  |  | -8.08% | 107% | 96% |
|  |  | CE8.2.5b |  |  | -11.43% |  |  | -8.94% |  | -9.04% |  | 121% |  | 94% |  | -8.65% |  |  | -7.95% |  |  |  |  |  | -8.32% | 110% | 102% |
|  |  | CE8.2.6 |  |  | -11.43% |  |  | -8.83% |  | -8.89% |  | 116% |  | 92% |  | -8.54% |  |  | -8.01% |  |  |  |  |  | -8.25% | 102% | 95% |
| SCC 1080p | VTM+PLT | -33.06% | -27.59% | -27.70% | 131% | 68% | -15.71% | -14.68% | -14.53% | 106% | 93% |
|  |  | CE8.2.1 |  |  | -33.06% |  |  | -27.59% |  | -27.70% |  | 131% |  | 68% |  | -15.71% |  |  | -14.68% |  |  |  |  |  | -14.53% | 106% | 93% |
|  |  | CE8.2.1* |  |  | -17.97% |  |  | -18.43% |  | -18.73% |  | 122% |  | 81% |  | -9.03% |  |  | -10.12% |  |  |  |  |  | -10.01% | 106% | 96% |
|  |  | CE8.2.2 |  |  | -34.47% |  |  | -28.70% |  | -28.82% |  | 122% |  | 70% |  | -16.56% |  |  | -15.23% |  |  |  |  |  | -15.12% | 105% | 94% |
|  |  | CE8.2.3 |  |  | -33.03% |  |  | -27.60% |  | -27.66% |  | 126% |  | 67% |  | -15.73% |  |  | -14.64% |  |  |  |  |  | -14.50% | 99% | 87% |
|  |  | CE8.2.4 |  |  | -33.15% |  |  | -26.95% |  | -27.08% |  | 127% |  | 69% |  | -15.77% |  |  | -14.33% |  |  |  |  |  | -14.23% | 99% | 88% |
|  |  | CE8.2.5a |  |  | -27.02% |  |  | -26.01% |  | -25.52% |  | 126% |  | 62% |  | -14.78% |  |  | -15.28% |  |  |  |  |  | -14.62% | 102% | 88% |
|  |  | CE8.2.5b |  |  | -33.06% |  |  | -27.59% |  | -27.70% |  | 135% |  | 69% |  | -16.64% |  |  | -15.24% |  |  |  |  |  | -15.02% | 108% | 94% |
|  |  | CE8.2.6 |  |  | -33.13% |  |  | -27.64% |  | -27.75% |  | 128% |  | 71% |  | -15.73% |  |  | -14.59% |  |  |  |  |  | -14.50% | 99% | 89% |

* PLT mode in HEVC SCC, when dual-tree SPS is off

Table 7: CE8 BDPCM related test results against VTM-3.0 anchor

|  |  | AI Over VTM-3.0 |  |  |  |  | RA Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# |  | Y |  | U |  | V |  | EncT |  | DecT |  | Y |  | U |  | V |  | EncT |  |  |  | DecT |
| CTC overall |  |  |  |  |  |  |  |  |  |  |  |
|  | CE8.3.1a | -0.03% | -0.08% | -0.04% | 101% | 97% | -0.01% | 0.05% | 0.02% | 101% | 97% |
|  | CE8.3.1b |  | -0.08% |  | 0.01% |  | 0.04% |  | 106% |  | 98% |  | -0.03% |  | 0.14% |  | 0.10% |  |  |  |  | 102% | 97% |
|  | CE8.3.2 |  | -0.08% |  | 0.00% |  | 0.04% |  | 105% |  | 98% |  | -0.03% |  | 0.16% |  | 0.12% |  |  |  |  | 102% | 97% |
| Class F |  |  |  |  |  |  |  |  |  |  |  |
|  | CE8.3.1a |  | -6.67% |  | -6.04% |  | -6.05% |  | 81% |  | 88% |  | -4.81% |  | -4.52% |  | -4.58% |  |  |  |  | 100% | 93% |
|  | CE8.3.1b |  | -9.16% |  | -6.79% |  | -6.77% |  | 88% |  | 96% |  | -6.93% |  | -5.21% |  | -5.16% |  |  |  |  | 103% | 95% |
|  | CE8.3.2 |  | -9.00% |  | -6.50% |  | -6.48% |  | 88% |  | 96% |  | -6.74% |  | -4.97% |  | -4.97% |  |  |  |  | 102% | 95% |
| SCC 1080p |  |  |  |  |  |  |  |  |  |  |  |
|  | CE8.3.1a |  | -21.30% |  | -18.64% |  | -18.76% |  | 62% |  | 81% |  | -9.66% |  | -8.23% |  | -8.18% |  |  |  |  | 100% | 93% |
|  | CE8.3.1b |  | -24.70% |  | -19.08% |  | -19.36% |  | 73% |  | 100% |  | -12.59% |  | -8.46% |  | -8.63% |  |  |  |  | 102% | 96% |
|  | CE8.3.2 |  | -24.45% |  | -19.08% |  | -19.37% |  | 73% |  | 99% |  | -12.06% |  | -8.05% |  | -8.17% |  |  |  |  | 102% | 95% |

| Table 7: CE8 BDPCM related test results against VTM-3.0 anchor |  | Table 7: CE8 BDPCM related test results against VTM-3.0 anchor |  | Table 7: CE8 BDPCM related test results against VTM-3.0 anchor |  |  | Table 7: CE8 BDPCM related test results against VTM-3.0 anchor |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Crosschecker reports

Crosscheckers reported that the software/results in CE8 are generally in good shape, no outstanding issues were identified. So minor differences were found, which should be due to the identified CPR related bugs in VTM-3.0.

Table 8: crosscheck reports that are associated with the tests in this CE.

| # | Tester | Cross-checker | Software review/studied? (Y/N) | Any significant inconsistencies between the description and the provided software? (Y/N) |
| --- | --- | --- | --- | --- |
| CE8.1.1a | J. Nam | X. Xu | Y | N |
|  | (LGE) | (Tencent) |  |  |
| CE8.1.1b | J. Nam | A. Karabutov | Y | N |
|  | (LGE) | (Huawei) |  |  |
| CE8.1.2a | X. Xu | A. Karabutov | Y | N |
|  | (Tencent) | (Huawei) |  |  |
| CE8.1.2b | X. Xu | J. Nam | Y | N |
|  | (Tencent) | (LGE) |  |  |
| CE8.1.2c | X. Xu | J. Nam | Y | N |
|  | (Tencent) | (LGE) |  |  |
| CE8.1.2d | X. Xu | A. Karabutov | Y | N |
|  | (Tencent) | (Huawei) |  |  |
| CE8.1.3 | L.P. Van | S. Paluri | Y | N |
|  | (Qualcomm) | (LGE) |  |  |
| CE8.2.1 | Y.-H. Chao/Y.-C. Sun | R. Chernyak | Y | N |
|  | (Qualcomm/Alibaba) | (Huawei) |  |  |
| CE8.2.2 | Y.-C. Sun | Y.-W. Chen | Y | N |
|  | (Alibaba) | (Kwai) |  |  |
| CE8.2.3 | J. Ye | R. Chernyak | Y | N |
|  | (Tencent) | (Huawei) |  |  |
| CE8.2.4 | J. Ye | Y.-C. Sun | Y | N |
|  | (Tencent) | (Alibaba) |  |  |
| CE8.2.5a | R. Chernyak/Y.-H. Chao/Y.-C. Sun | J. Ye | Y | N |
|  | (Huawei/Qualcomm/Alibaba) | (Tencent) |  |  |
| CE8.2.5b | R. Chernyak/Y.-H. Chao/Y.-C. Sun | J. Ye | Y | N |
|  | (Huawei/Qualcomm/Alibaba) | (Tencent) |  |  |
| CE8.2.6 | J. Ye | Y.-C. Sun | Y | N |
|  | (Tencent) | (Alibaba) |  |  |
| CE8.3.1a | F. Henry | B. Bross | Y | N |
|  | (Orange) | (HHI) |  |  |
| CE8.3.1b | F. Henry | B. Bross | Y | N |
|  | (Orange) | (HHI) |  |  |
| CE8.3.2 | F. Henry | B. Bross | Y | N |
|  | (Orange) | (HHI) |  |  |

| # | Results available (Full/Partial) | Resutls matched? (Y/N) | Runtime behavior matched? (Y/N) | Additional comments |
| --- | --- | --- | --- | --- |
| CE8.1.1a | Full | Y | Y | minor different results by CPR related bug fixes |
| CE8.1.1b | Partial | Y | Y | minor different results by CPR related bug fixes |
|  | (Full except LDB) |  |  |  |
| CE8.1.2a | Partial | Y | Y | minor different results by CPR related bug fixes |
|  | (Full except LDB) |  |  |  |
| CE8.1.2b | Full | Y | Y | minor different results by CPR related bug fixes |
| CE8.1.2c | Full | Y | Y | minor different results by CPR related bug fixes |
| CE8.1.2d | Partial | Y | Y | minor different results by CPR related bug fixes |
|  | (Full except LDB) |  |  |  |
| CE8.1.3 | Full | Y | Y |  |
| CE8.2.1 | Full | Y | Y | minor different results by CPR related bug fixes |
| CE8.2.2 | Full | Y | Y |  |
| CE8.2.3 | Full | Y | N | minor different results by CPR related bug fixes |
| CE8.2.4 | Full | Y | Y | minor different results by CPR related bug fixes |
| CE8.2.5a | Full | Y | Y | minor different results by CPR related bug fixes |
| CE8.2.5b | Full | Y | Y | minor different results by CPR related bug fixes |
| CE8.2.6 | Full | Y | Y | minor different results by CPR related bug fixes |
| CE8.3.1a | Partial | Y | Y |  |
|  | (Full except LDB and PLT1+CPR1 still running) |  |  |  |
| CE8.3.1b | Partial | Y | Y | CPR=0 encoder runtimes not reliable due to cluster inconsistencies |
|  | (Full except LDB and PLT1+CPR1 still running) |  |  |  |
| CE8.3.2 | Partial | Y | Y |  |
|  | (Full except LDB and PLT1+CPR1 still running) |  |  |  |

# Input contributions for CE8 and CE8 related topics

## CE contributions

Table 9: input contributions for tests that are evaluated and studied in this CE

| CE tests | Title | Contribution # |
| --- | --- | --- |
| CE8.1.1a | CE8: Block vector prediction for CPR (test 8.1.1a and test 8.1.1b) | JVET-M0332 |
| CE8.1.1b |  |  |
| CE8.1.2a | CE8: CPR reference memory reuse without increasing memory requirement (CE8.1.2a and CE8.1.2d) | JVET-M0407 |
| CE8.1.2d |  |  |
| CE8.1.2b | CE8: CPR reference memory reuse with reduced memory requirement (CE8.1.2b and CE8.1.2c) | JVET-M0408 |
| CE8.1.2c |  |  |
| CE8.1.3 | CE8.1.3: Extended CPR reference with 1 buffer line | JVET-M0474 |
| CE8.2.1 | CE8: Palette Mode in HEVC (test 8.2.1) | JVET-M0050 |
| CE8.2.2 | CE8: Palette Mode and Intra Mode Combination (test 8.2.2) | JVET-M0051 |
| CE8.2.3 | CE8: Palette index map scan order constraints (Test 8.2.3) | JVET-M0455 |
| CE8.2.4 | CE8: palette mode when dual-tree is enabled (Test 8.2.4) | JVET-M0456 |
| CE8.2.5a | CE8: Separate Palette Coding for Luma and Chroma (test 8.2.5) | JVET-M0052 |
| CE8.2.5b |  |  |
| CE8.2.6 | CE8: Palette predictor list enhancement (Test 8.2.6) | JVET-M0457 |
| CE8.3.1a | CE8.3.1a: BDPCM with LOCO-I and independently decodable areas | JVET-M0056 |
| CE8.3.1b | CE8.3.1b: BDPCM with horizontal/vertical predictor and independently decodable areas | JVET-M0057 |
| CE8.3.2 | CE8.3.2: BDPCM with modified binarization | JVET-M0058 |

## CPR related contributions

- JVET-M0151, CE8-related: Virtual search area for current picture referencing (CPR), L. Pham Van, T. Hsieh, W.-J. Chien, V. Seregin, H. Wang, M. Karczewicz (Qualcomm)

- JVET-M0174, CE8-related: Removal of subblock-based chroma MC in CPR, C.-Y. Lai, T.-D. Chuang, Y.-L. Hsiao, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-M0175, CE8-related: Clarification on interaction between CPR and other inter coding tools, C.-Y. Lai, T.-D. Chuang, Y.-L. Hsiao, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-M0254, Non-CE8: Subblock Operation Removal for Chroma CPR, J. Xu, K. Zhang, L. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong (Bytedance)

- JVET-M0325, CE8-related: CPR with previous CTU availability at frame and tile edge, C. Rosewarne, A. Dorrell (Canon)

- JVET-M0326, CE8-related: Remove the redundancy of CPR-related syntax coding, S. Ye, F. Chen, L. Wang (Hikvision)

- JVET-M0327, CE8-related: A new CPR syntax scheme, S. Ye, F. Chen, L. Wang (Hikvision)

- JVET-M0333, Non-CE8: Coding on block vector difference, J. Nam, J. Lim, S. Kim (LGE)

- JVET-M0334, Non-CE8: Removal of redundant syntax between CPR and other inter coding tools, J. Nam, J. Lim, S. Kim (LGE)

- JVET-M0335, Non-CE8: modification on SbTMVP process regarding with CPR, H.Jang, J.Nam, S.Kim, J.Lim(LGE)

- JVET-M0336, Non-CE11: Considering boundary strength on CPR coded block boundary, H.Jang, J.Nam, S.Kim, J.Lim(LGE)

- JVET-M0341, Non-CE8: MMVD harmonization with CPR, H.Jang, J.Nam, S.Kim, J.Lim(LGE)

- JVET-M0393, Non-CE8: chroma block vector initialization for CPR in dual tree, T. Poirier, F. Le Léannec, F. Galpin (Technicolor)

- JVET-M0402, Non-CE8: Proposed Cleanup for Current Picture Referencing, B. Heng, M. Zhou, W. Wan (Broadcom)

- JVET-M0409, Non-CE8: Mismatch between text specification and reference software on ATMVP candidate derivation when CPR is enabled, X. Xu, X. Li, S. Liu (Tencent), W.-J. Chien, M. Karczewicz (Qualcomm)

- JVET-M0410, Non-CE8: CPR flag signaling at slice level, X. Xu, X. Li, S. Liu (Tencent)

- JVET-M0411, Non-CE8: Inter mode related flag signaling when current picture is the only reference picture, X. Xu, X. Li, S. Liu (Tencent)

- JVET-M0418, CE8-related: Context modeling on pred_mode_flag when current picture is the only reference picture (CPR), Yu-Chen Sun, Jian Lou (Alibaba)

- JVET-M0483, CE8-related: CPR mode signaling and interaction with inter coding tools, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-M0541, Non-CE8: Combination of MMVD and CPR mode, Y. Li, Z. Chen (Wuhan Univ.), X. Xu, S. Liu (Tencent)

- JVET-M0542, Non-CE8: Combination of Multi Hypothesis Intra and CPR mode, Y. Li, Z. Chen (Wuhan Univ.), X. Xu, S. Liu (Tencent)

- JVET-M0544, Non-CE8: CPR with chroma 4x4 sub-block size when dual-tree is on, X. Xu, X. Li, S. Liu (Tencent)

- JVET-M0634, Affine motion mode in intra coding, S.-Q. Cao, H.-Y. Han, J. Wang, F. Liang(SYSU), Y.-F. Yu, Y. Liu(OPPO)

## Palette related contributions

- JVET-M0417, CE8-related: Combination test of CE8.2.2 and CE8.2.5, Yu-Chen Sun, Jian Lou (Alibaba)

- JVET-M0419, CE8-related: Context modeling on palette mode flag, Yu-Chen Sun, Jian Lou (Alibaba)

## Other related contributions

- JVET-M0253, Non-CE8: Hash-based Motion Search, J. Xu, J. Li, K. Zhang, L. Zhang (Bytedance), R. Xiong (Peking University)

- JVET-M0365, Non-CE3: modified PDPC for horizontal and vertical modes, A. Filippov, V. Rufitskiy, J. Chen (Huawei)

- JVET-M0449, CE8-related: BDPCM entropy coding with reduced number of context coded bins, M. Xu, X. Li, X. Xu, M. Gao, S. Liu (Tencent)

- JVET-M0464, Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip, B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand (HHI)

- JVET-M0765, CE8-related: Unified Screen Content and Multiview Video Coding - Experimental results	Jarosław Samelak, Marek Domański (Poznań University of Technology)

## Crosscheck reports for CE8 and CE8 related contributions

- JVET-M0543,	Crosscheck of M0474: CE8.1.3- Extended CPR reference with 1 buffer line, S.Paluri, S. Kim (LGE)

- JVET-M0556, Crosscheck of JVET-M0417 (CE8-related: Combination test of CE8.2.2 and CE8.2.5), C.-Y. Lai (MediaTek)

- JVET-M0615, Crosscheck of JVET-M0326 (CE8-related: Remove the redundancy of CPR-related syntax coding), Y.-C. Sun (Alibaba)

- JVET-M0616, Crosscheck of JVET-M0327 (CE8-related: A new CPR syntax scheme), Y.-C. Sun (Alibaba)

- JVET-M0635, Crosscheck of JVET-M0411 (Non-CE8: Inter mode related flag signaling when current picture is the only reference picture), C.-Y. Lai (MediaTek)

- JVET-M0648, Crosscheck of JVET-M0449 (CE8-related: BDPCM entropy coding with reduced number of context coded bins), F. Henry, G. Clare (Orange)

- JVET-M0650, Crosscheck of JVET-M0464 (Non-CE8: Unified Transform Type Signaling and Residual Coding for Transform Skip, test TSRC-CCB8), F. Henry, G. Clare (Orange)

- JVET-M0669, Crosscheck of JVET-M0544 (Non-CE8: CPR with chroma 4x4 sub-block size when dual-tree is on), J. Nam (LGE)

- JVET-M0670, Crosscheck of JVET-M0410 (Non-CE8: CPR flag signaling at slice level), J. Nam (LGE)

- JVET-M0696, Crosscheck of JVET-M0542 (Non-CE8: Combination of Multi Hypothesis Intra and CPR mode), T. Poirier (Technicolor)

- JVET-M0704, crosscheck for JVET-M0253: Non-CE8: Hash-based Motion Search, X. Xu (Tencent)

- JVET-M0705, crosscheck for JVET-M0333: Non-CE8: Coding on block vector difference, X. Xu (Tencent)

- JVET-M0706, crosscheck for JVET-M0335: Non-CE8: modification on SbTMVP process regarding with CPR, X. Xu (Tencent)

- JVET-M0707, crosscheck for JVET-M0188: CE7-related: On quantization parameter signalling considering CU area, Y. Wang, X. Xu (Tencent)

- JVET-M0708, Crosscheck of JVET-M0464 (Non-CE8: Unified Transform Type Signaling and Residual Coding for Transform Skip, test TS32Y/uniMTS), T. Tsukuba (Sony)

- JVET-M0711, Crosscheck of JVET-M0418 (CE8-related: Context modeling on pred_mode_flag when current picture is the only reference picture (CPR)), S. Ye (Hikvision)

- JVET-M0732, Crosscheck of JVET-M0409 (Non-CE8: Mismatch between text specification and reference software on ATMVP candidate derivation when CPR is enabled), T.-S. Chang, Y.-C. Sun, J. Lou (Alibaba)

- JVET-M0751, crosscheck for JVET-M0393: Non-CE8: chroma block vector initialization for CPR in dual tree, X. Xu (tencent)

- JVET-M0767, Crosscheck of JVET-M0334 (Non-CE8: Removal of redundant syntax between CPR and other inter coding tools), X. Xu (Tencent)

- JVET-M0768, Crosscheck of JVET-M0341 (Non-CE8: MMVD harmonization with CPR), X. Xu (Tencent)