| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0051v2 |
| --- | --- |

| Title: | CE8: Palette mode and intra mode combination (Test8.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Jian Lou | Tel: Email: | +1-858-349-2818 yuchen.s@alibaba-inc.com |
| Source: | Alibaba Cloud Computing Ltd. |  |  |

_____________________________

- Abstract

This document proposes a method that combines palette mode and intra prediction (CE8.2.2). The tests are performed on top of CE8.2.1 (JVET-M0050). Compared with CE8.2.1, the results of CE8.2.2 show that: When the method is tested with CPR turned off in the setting,

- For 4:2:0 TGM sequences, the proposed method provides 2.5%, 1.1%, and 0.1% BD-rate gains for luma with 93%, 99% and 101% for encoding time, and 102%, 100% and 103% in decoding time under AI, RA, and LD configurations, respectively.

- For Class F sequences, the proposed method provides 0.3%, 0.4%, and 0.2% BD-rate gains for luma, with 101%, 100% and 100% for encoding time, and 102%, 100% and 98% in decoding time under AI, RA, and LD configurations, respectively.

When the method is tested with CPR turned on in the setting,

- For 4:2:0 TGM sequences, the proposed method provides 0.9%, 0.5%, and 0.1% BD-rate gains for luma with 98%, 100% and 102% for encoding time, and 100%, 98% and 101% in decoding time under AI, RA, and LD configurations, respectively.

- For Class F sequences, the proposed method provides 0.0%, 0.1%, and 0.3% BD-rate gains for luma, with 100%, 101% and 102% for encoding time, and 100%, 100% and 103% in decoding time under AI, RA, and LD configurations, respectively.

CE Description

CE8.2.2 tests a method that combines palette mode and intra prediction. In CE8.2.1 (HEVC palette mode), a flag is transmitted for each CU to signal whether the palette mode is used in the current CU. If a block is encoded by the palette mode, it will be decoded independently without using any prediction method as Figure 1.

Figure 1. CE8.2.1: Palette mode in HEVC.

To obtain the benefits of both palette mode and traditional prediction methods, this document proposes a method combining palette mode and the intra prediction methods.

Description of techniques

In the proposed combination, the decoder first derives the prediction block based on the intra prediction methods. Then, the decoder decodes a palette and an index map. Using the decoding palette information, the decoder refines the prediction block and reconstructs the block.

Figure 2 shows an example. Compared with CE8.2.1, the proposed method combines the conventional intra prediction methods and the palette method.

Figure 2. Proposed method.

Figure 3 shows an example combining the palette information and the intra prediction information. First, the decoder generates the prediction block (with pixel values A0~A15). And, the decoder decodes an index map in the palette mode. To reconstruct the block, if the decoded indices equal to “0”, the corresponding pixels are marked as “P” as shown in Figure 3. The pixels marked as “P” is reconstructed by pixel values in the intra prediction block. Otherwise, the pixels are reconstructed by palette colors, e.g. C0, C1, as CE8.2.1.

# Figure 3. Method that refines prediction block according to the palette information

The proposed method is implemented on top of the separate palette structure and enabled only when the area of the CU is larger than or equal to 256. Unlike the palette base software in JVET-L0213 [1], the CE8 base software (CE8.2.1) applies the separate palette structure only on intra slices, so the proposed method is only enabled in intra slices. However, if the separate palette structure is extended to inter slices, i.e. CE8.2.5, the proposed method also works for inter slices.

Simulation results

The proposed method is evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations with the VTM-3.0 software.

CE8.2.2 is tested with/without CPR. Table 1 shows the results with CPR turned off in the setting. The results are compared with VTM3.0 and CE8.2.1.

Table 1: Results of the CE8.2.2 test (CPR is turned off)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.09% | 109% | 101% | -0.01% | -0.05% | 0.01% | 101% | 102% |
| Class A2 | 0.09% | 0.05% | 0.11% | 108% | 102% | -0.01% | -0.09% | 0.03% | 101% | 103% |
| Class B | 0.08% | 0.08% | 0.09% | 107% | 103% | -0.01% | -0.01% | 0.02% | 101% | 104% |
| Class C | 0.09% | 0.09% | 0.06% | 109% | 106% | -0.01% | 0.00% | 0.14% | 101% | 100% |
| Class E | 0.11% | 0.23% | 0.21% | 108% | 101% | -0.01% | 0.07% | 0.01% | 100% | 100% |
| Overall | 0.08% | 0.09% | 0.11% | 108% | 103% | -0.01% | -0.01% | 0.05% | 101% | 102% |
| Class D | 0.12% | 0.10% | 0.14% | 108% | 105% | 0.01% | 0.03% | 0.12% | 101% | 101% |
| Class F | -11.65% | -9.08% | -9.19% | 118% | 91% | -0.30% | -0.19% | -0.19% | 101% | 102% |
| Class SCC 1080p | -34.47% | -28.70% | -28.82% | 122% | 70% | -2.45% | -1.71% | -1.72% | 93% | 102% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.21% | 0.14% | 105% | 101% | -0.02% | 0.05% | 0.02% | 101% | 101% |
| Class A2 | 0.10% | 0.18% | 0.31% | 106% | 102% | -0.01% | -0.05% | 0.09% | 101% | 101% |
| Class B | 0.12% | 0.27% | -0.06% | 106% | 100% | -0.01% | -0.01% | 0.04% | 100% | 100% |
| Class C | 0.21% | 0.06% | 0.11% | 108% | 106% | 0.04% | -0.04% | -0.09% | 100% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.14% | 0.18% | 0.10% | 106% | 102% | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class D | 0.21% | 0.04% | 0.17% | 109% | 105% | 0.02% | 0.20% | -0.08% | 100% | 101% |
| Class F | -8.80% | -8.28% | -8.66% | 110% | 101% | -0.36% | -0.30% | -0.40% | 100% | 100% |
| Class SCC 1080p | -16.56% | -15.23% | -15.12% | 105% | 94% | -1.06% | -0.68% | -0.73% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.10% | -0.26% | -0.16% | 107% | 98% | 0.02% | -0.25% | -0.22% | 101% | 100% |
| Class C | 0.16% | 0.34% | 0.46% | 107% | 101% | 0.00% | -0.09% | -0.02% | 101% | 106% |
| Class E | 0.38% | 0.48% | 0.18% | 108% | 97% | 0.12% | -0.75% | -0.27% | 100% | 102% |
| Overall | 0.19% | 0.13% | 0.13% | 107% | 99% | 0.04% | -0.32% | -0.16% | 101% | 103% |
| Class D | 0.25% | 0.25% | 0.53% | 110% | 103% | -0.02% | -0.27% | 0.13% | 101% | 100% |
| Class F | -4.06% | -4.70% | -5.28% | 109% | 98% | -0.17% | -0.26% | -0.48% | 100% | 98% |
| Class SCC 1080p | -7.10% | -6.64% | -6.35% | 106% | 98% | -0.08% | 0.03% | -0.01% | 101% | 103% |

Table 2 shows the results with CPR turned on in the setting. The results of CE8.2.2 are compared with VTM3.0 and CE8.2.1.

Table 2: Results of the CE8.2.2 test (CPR is turned on)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.17% | 0.02% | 111% | 103% | -0.01% | 0.00% | -0.06% | 101% | 98% |
| Class A2 | 0.09% | 0.10% | 0.11% | 110% | 103% | 0.00% | 0.03% | 0.00% | 101% | 98% |
| Class B | 0.08% | 0.12% | 0.11% | 109% | 104% | -0.01% | 0.04% | 0.05% | 103% | 102% |
| Class C | 0.10% | 0.20% | 0.10% | 107% | 102% | 0.02% | 0.09% | 0.03% | 100% | 101% |
| Class E | 0.13% | 0.18% | 0.02% | 107% | 101% | -0.01% | -0.10% | 0.00% | 101% | 100% |
| Overall | 0.09% | 0.15% | 0.08% | 109% | 103% | 0.00% | 0.02% | 0.01% | 101% | 100% |
| Class D | 0.08% | 0.31% | 0.06% | 110% | 100% | -0.01% | 0.12% | 0.11% | 101% | 97% |
| Class F | -4.49% | -2.41% | -2.46% | 112% | 96% | -0.03% | 0.05% | 0.05% | 100% | 100% |
| Class SCC 1080p | -13.43% | -8.20% | -8.16% | 111% | 86% | -0.92% | -0.31% | -0.36% | 98% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.21% | 0.13% | 106% | 104% | 0.04% | -0.03% | -0.01% | 98% | 99% |
| Class A2 | 0.11% | 0.04% | 0.04% | 106% | 105% | 0.00% | -0.04% | 0.01% | 98% | 99% |
| Class B | 0.12% | 0.11% | 0.15% | 106% | 101% | 0.03% | 0.10% | 0.02% | 98% | 99% |
| Class C | 0.16% | -0.02% | 0.18% | 109% | 101% | 0.00% | -0.07% | -0.12% | 100% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.13% | 0.08% | 0.13% | 107% | 102% | 0.02% | 0.00% | -0.02% | 99% | 99% |
| Class D | 0.24% | 0.28% | 0.20% | 111% | 100% | -0.03% | 0.04% | 0.40% | 101% | 96% |
| Class F | -2.66% | -2.29% | -2.52% | 110% | 102% | -0.05% | 0.07% | 0.08% | 101% | 100% |
| Class SCC 1080p | -5.20% | -3.91% | -3.73% | 104% | 95% | -0.51% | -0.29% | -0.30% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.09% | 0.48% | 0.06% | 110% | 106% | -0.02% | 0.40% | -0.19% | 99% | 100% |
| Class C | 0.16% | 0.25% | 0.22% | 106% | 107% | -0.01% | 0.10% | 0.18% | 98% | 104% |
| Class E | 0.07% | 0.44% | 0.07% | 105% | 100% | -0.12% | 0.46% | 0.24% | 100% | 101% |
| Overall | 0.11% | 0.39% | 0.11% | 108% | 105% | -0.04% | 0.32% | 0.04% | 99% | 102% |
| Class D | 0.21% | 0.64% | -0.14% | 107% | 103% | -0.04% | -0.18% | -0.10% | 99% | 102% |
| Class F | -1.47% | -1.89% | -2.54% | 106% | 101% | -0.30% | -0.24% | -0.67% | 102% | 103% |
| Class SCC 1080p | -1.88% | -1.32% | -1.06% | 104% | 97% | -0.06% | -0.01% | 0.08% | 102% | 101% |

Reference

- Y.-C. Sun, etc,	“Combination of palette mode and intra prediction”, JVET-L0213, 12th JVET meeting, Oct. 2018.

- X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu, "Description of Core Experiment 8: Screen Content Coding Tools”, JVET-L1028, 2018.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).