Added texts are highlighted.

#### Derivation process for motion vectors and reference indices in subblock merge mode

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the luma coding block.

Outputs of this process are:

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flag arrays predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ],

- the luma subblock motion vector arrays in 1/16 fractional-sample accuracy mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1,

- the chroma subblock motion vector arrays in 1/32 fractional-sample accuracy mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1,

- the bi-prediction weight index gbiIdx.

The variables numSbX, numSbY and the subblock merging candidate list, subblockMergeCandList are derived by the following ordered steps:

- When sps_sbtmvp_enabled_flag is equal to 1, the following applies:

- The derivation process for merging candidates from neighbouring coding units as specified in clause 8.4.2.3 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the luma coding block width as inputs, and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1 and refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and predFlagLXB2, and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1 and mvLXB2, with X being 0 or 1.

- The derivation process for subblock-based temporal merging candidates as specified in clause 8.4.4.3 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight , the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1 as inputs and the output being the availability flag availableFlagSbCol, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the reference indices refIdxLXSbCol, the luma motion vectors mvLXSbCol[ xSbIdx ][ ySbIdx ] and the prediction list utilization flags predFlagLXSbCol[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1 and X being 0 or 1.

- When sps_affine_enabled_flag is equal to 1, the sample locations ( xNbA0, yNbA0 ), ( xNbA1, yNbA1 ), ( xNbA2, yNbA2 ), ( xNbB0, yNbB0 ), ( xNbB1, yNbB1 ), ( xNbB2, yNbB2 ), ( xNbB3, yNbB3 ), and the variables numSbX and numSbY are derived as follows:

( xA0, yA0 ) = ( xCb − 1, yCb + cbHeight )		(8-472)

( xA1, yA1 ) = ( xCb − 1, yCb + cbHeight − 1 )		(8-473)

( xA2, yA2 ) = ( xCb − 1, yCb )		(8-474)

( xB0, yB0 ) = ( xCb + cbWidth , yCb − 1 )		(8-475)

( xB1, yB1 ) = ( xCb + cbWidth − 1, yCb − 1 )		(8-476)

( xB2, yB2 ) = ( xCb − 1, yCb − 1 )		(8-477)

( xB3, yB3 ) = ( xCb, yCb − 1 )		(8-478)

numSbX = cbWidth >> 2		(8-479)

numSbY = cbHeight >> 2		(8-480)

- When sps_affine_enabled_flag is equal to 1, the variable availableFlagA is set equal to FALSE and the following applies for ( xNbAk, yNbAk ) from ( xNbA0, yNbA0 ) to ( xNbA1, yNbA1 ):

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbAk, yNbAk ) as inputs, and the output is assigned to the block availability flag availableAk.

- When availableAk is equal to TRUE and MotionModelIdc[ xNbAk ][ yNbAk ] is greater than 0 and CbWidth[ xNbAk ][ yNbAk ] is larger or equal to 16 and CbHeight[ xNbAk ][ yNbAk ] is larger or equal to 16 and availableFlagA is equal to FALSE, the following applies:

    - The variable availableFlagA is set equal to TRUE, motionModelIdcA is set equal to MotionModelIdc[ xNbAk ][ yNbAk ], ( xNb, yNb ) is set equal to ( CbPosX[ xNbAk ][ yNbAk ], CbPosY[ xNbAk ][ yNbAk ] ), nbW is set equal to CbWidth[ xNbAk ][ yNbAk ], nbH is set equal to CbHeight[ xNbAk ][ yNbAk ], numCpMv is set equal to MotionModelIdc[ xNbAk ][ yNbAk ] + 1, and gbiIdxA is set equal to GbiIdx[ xNbAk ][ yNbAk ].

    - For X being replaced by either 0 or 1, the following applies:

      - When PredFlagLX[ xNbAk ][ yNbAk ] is equal to 1, the derivation process for luma affine control point motion vectors from a neighbouring block as specified in clause 8.4.4.5 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width and height (cbWidth, cbHeight), the neighbouring luma coding block location ( xNb, yNb ), the neighbouring luma coding block width and height (nbW, nbH), and the number of control point motion vectors numCpMv as input, the control point motion vector predictor candidates cpMvLXA[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 as output.

      - The following assignments are made:

predFlagLXA = PredFlagLX[ xNbAk ][ yNbAk ]	(8-481)

refIdxLXA = RefIdxLX[ xNbAk ][ yNbAk ]	(8-482)

- When sps_affine_enabled_flag is equal to 1, the variable availableFlagB is set equal to FALSE and the following applies for ( xNbBk, yNbBk ) from ( xNbB0, yNbB0 ) to ( xNbB2, yNbB2 ):

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbBk, yNbBk ) as inputs, and the output is assigned to the block availability flag availableBk.

- When availableBk is equal to TRUE and MotionModelIdc[ xNbBk ][ yNbBk ] is greater than 0 and CbWidth[ xNbBk ][ yNbBk ] is larger or equal to 16 and CbHeight[ xNbBk ][ yNbBk ] is larger or equal to 16 and availableFlagB is equal to FALSE, the following applies:

    - The variable availableFlagB is set equal to TRUE, motionModelIdcB is set equal to MotionModelIdc[ xNbBk ][ yNbBk ], ( xNb, yNb ) is set equal to ( CbPosX[ xNbAB ][ yNbBk ], CbPosY[ xNbBk ][ yNbBk ] ), nbW is set equal to CbWidth[ xNbBk ][ yNbBk ], nbH is set equal to CbHeight[ xNbBk ][ yNbBk ], numCpMv is set equal to MotionModelIdc[ xNbBk ][ yNbBk ] + 1, and gbiIdxB is set equal to GbiIdx[ xNbBk ][ yNbBk ].

    - For X being replaced by either 0 or 1, the following applies:

      - When PredFlagLX[ xNbBk ][ yNbBk ] is equal to TRUE, the derivation process for luma affine control point motion vectors from a neighbouring block as specified in clause 8.4.4.5 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width and height (cbWidth, cbHeight), the neighbouring luma coding block location ( xNb, yNb ), the neighbouring luma coding block width and height (nbW, nbH), and the number of control point motion vectors numCpMv as input, the control point motion vector predictor candidates cpMvLXB[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 as output.

      - The following assignments are made:

predFlagLXB = PredFlagLX[ xNbBk ][ yNbBk ]	(8-483)

refIdxLXB = RefIdxLX[ xNbBk ][ yNbBk ]	(8-484)

#### Derivation process for luma affine control point motion vector predictors

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the current luma coding block,

- the reference index of the current coding unit refIdxLX, with X being 0 or 1,

- the number of control point motion vectors numCpMv.

Output of this process are the luma affine control point motion vector predictors mvpCpLX[ cpIdx ] with X being 0 or 1, and cpIdx = 0 .. numCpMv − 1.

For the derivation of the control point motion vectors predictor candidate list, cpMvpListLX with X being 0 or 1, the following ordered steps apply:

- The number of control point motion vector predictor candidates in the list numCpMvpCandLX is set equal to 0.

- The variables availableFlagA and availableFlagB are both set equal to FALSE.

- The sample locations ( xNbA0, yNbA0 ), ( xNbA1, yNbA1 ), ( xNbA2, yNbA2 ), ( xNbB0, yNbB0 ), ( xNbB1, yNbB1 ), and ( xNbB2, yNbB2 ) are derived as follows:

( xA0, yA0 ) = ( xCb − 1, yCb + cbHeight )		(8-605)

( xA1, yA1 ) = ( xCb − 1, yCb + cbHeight − 1 )		(8-606)

( xB0, yB0 ) = ( xCb + cbWidth , yCb − 1 )		(8-607)

( xB1, yB1 ) = ( xCb + cbWidth − 1, yCb − 1 )		(8-608)

( xB2, yB2 ) = ( xCb − 1, yCb − 1 )		(8-609)

- The following applies for ( xNbAk, yNbAk ) from ( xNbA0, yNbA0 ) to ( xNbA1, yNbA1 ):

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbAk, yNbAk ) as inputs, and the output is assigned to the block availability flag availableAk.

- When availableAk is equal to TRUE and MotionModelIdc[ xNbAk ][ yNbAk ] is greater than 0 and CbWidth[ xNbAk ][ yNbAk ] is larger or equal to 16 and CbHeight[ xNbAk ][ yNbAk ] is larger or equal to 16 and availableFlagA is equal to FALSE, the following applies:

    - The variable ( xNb, yNb ) is set equal to ( CbPosX[ xNbAk ][ yNbAk ], CbPosY[ xNbAk ][ yNbAk ] ), nbW is set equal to CbWidth[ xNbAk ][ yNbAk ],and nbH is set equal to CbHeight[ xNbAk ][ yNbAk ].

    - If PredFlagLX[ xNbAk ][ yNbAk ] is equal to 1 and DiffPicOrderCnt( RefPicListX[ RefIdxLX[ xNbAk ][ yNbAk ] ], RefPicListX[ refIdxLX ] ) is equal to 0, the following applies:

      - The variable availableFlagA is set equal to TRUE

      - The derivation process for luma affine control point motion vectors from a neighbouring block as specified in clause 8.4.4.5 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width and height (cbWidth, cbHeight), the neighbouring luma coding block location ( xNb, yNb ), the neighbouring luma coding block width and height (nbW, nbH), and the number of control point motion vectors numCpMv as input, the control point motion vector predictor candidates cpMvpLX[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 as output.

      - The rounding process for motion vectors as specified in clause 8.4.2.14 is invoked with mvX set equal to cpMvpLX[ cpIdx ], rightShift set equal to 2, and leftShift set equal to 2 as inputs and the rounded cpMvpLX[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 as output.

      - The following assignments are made:

cpMvpListLX[ numCpMvpCandLX ][ 0 ] = cpMvpLX[ 0 ]	(8-610)

cpMvpListLX[ numCpMvpCandLX ][ 1 ] = cpMvpLX[ 1 ]	(8-611)

cpMvpListLX[ numCpMvpCandLX ][ 2 ] = cpMvpLX[ 2 ]	(8-612)

numCpMvpCandLX = numCpMvpCandLX + 1	(8-613)

    - Otherwise if PredFlagLY[ xNbAk ][ yNbAk ] (with Y = !X) is equal to 1 and DiffPicOrderCnt( RefPicListY[ RefIdxLY[ xNbAk ][ yNbAk ] ], RefPicListX[ refIdxLX ] ) is equal to 0, the following applies:

      - The variable availableFlagA is set equal to TRUE

      - The derivation process for luma affine control point motion vectors from a neighbouring block as specified in clause 8.4.4.5 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width and height (cbWidth, cbHeight), the neighbouring luma coding block location ( xNb, yNb ), the neighbouring luma coding block width and height (nbW, nbH), and the number of control point motion vectors numCpMv as input, the control point motion vector predictor candidates cpMvpLY[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 as output.

      - The rounding process for motion vectors as specified in clause 8.4.2.14 is invoked with mvX set equal to cpMvpLY[ cpIdx ], rightShift set equal to 2, and leftShift set equal to 2 as inputs and the rounded cpMvpLY[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 as output.

      - The following assignments are made:

cpMvpListLX[ numCpMvpCandLX ][ 0 ] = cpMvpLY[ 0 ]	(8-614)

cpMvpListLX[ numCpMvpCandLX ][ 1 ] = cpMvpLY[ 1 ]	(8-615)

cpMvpListLX[ numCpMvpCandLX ][ 2 ] = cpMvpLY[ 2 ]	(8-616)

numCpMvpCandLX = numCpMvpCandLX + 1	(8-617)

- The following applies for ( xNbBk, yNbBk ) from ( xNbB0, yNbB0 ) to ( xNbB2, yNbB2 ):

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbBk, yNbBk ) as inputs, and the output is assigned to the block availability flag availableBk.

- When availableBk is equal to TRUE and MotionModelIdc[ xNbBk ][ yNbBk ] is greater than 0 and CbWidth[ xNbBk ][ yNbBk ] is larger or equal to 16 and CbHeight[ xNbBk ][ yNbBk ] is larger or equal to 16 and availableFlagB is equal to FALSE, the following applies:

    - The variable ( xNb, yNb ) is set equal to ( CbPosX[ xNbBk ][ yNbBk ], CbPosY[ xNbBk ][ yNbBk ] ), nbW is set equal to CbWidth[ xNbBk ][ yNbBk ],and nbH is set equal to CbHeight[ xNbBk ][ yNbBk ].

    - If PredFlagLX[ xNbBk ][ yNbBk ] is equal to 1 and DiffPicOrderCnt( RefPicListX[ RefIdxLX[ xNbBk ][ yNbBk ] ], RefPicListX[ refIdxLX ] ) is equal to 0, the following applies:

      - The variable availableFlagB is set equal to TRUE

      - The derivation process for luma affine control point motion vectors from a neighbouring block as specified in clause 8.4.4.5 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width and height (cbWidth, cbHeight), the neighbouring luma coding block location ( xNb, yNb ), the neighbouring luma coding block width and height (nbW, nbH), and the number of control point motion vectors numCpMv as input, the control point motion vector predictor candidates cpMvpLX[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 as output.

      - The rounding process for motion vectors as specified in clause 8.4.2.14 is invoked with mvX set equal to cpMvpLX[ cpIdx ], rightShift set equal to 2, and leftShift set equal to 2 as inputs and the rounded cpMvpLX[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 as output.

      - The following assignments are made:

cpMvpListLX[ numCpMvpCandLX ][ 0 ] = cpMvpLX[ 0 ]	(8-618)

cpMvpListLX[ numCpMvpCandLX ][ 1 ] = cpMvpLX[ 1 ]	(8-619)

cpMvpListLX[ numCpMvpCandLX ][ 2 ] = cpMvpLX[ 2 ]	(8-620)

numCpMvpCandLX = numCpMvpCandLX + 1	(8-621)

    - Otherwise if PredFlagLY[ xNbBk ][ yNbBk ] (with Y = !X) is equal to 1 and DiffPicOrderCnt( RefPicListY[ RefIdxLY[ xNbBk ][ yNbBk ] ], RefPicListX[ refIdxLX ] ) is equal to 0, the following applies:

      - The variable availableFlagB is set equal to TRUE

      - The derivation process for luma affine control point motion vectors from a neighbouring block as specified in clause 8.4.4.5 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width and height (cbWidth, cbHeight), the neighbouring luma coding block location ( xNb, yNb ), the neighbouring luma coding block width and height (nbW, nbH), and the number of control point motion vectors numCpMv as input, the control point motion vector predictor candidates cpMvpLY[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 as output.

      - The rounding process for motion vectors as specified in clause 8.4.2.14 is invoked with mvX set equal to cpMvpLY[ cpIdx ], rightShift set equal to 2, and leftShift set equal to 2 as inputs and the rounded cpMvpLY[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 as output.

      - The following assignments are made:

cpMvpListLX[ numCpMvpCandLX ][ 0 ] = cpMvpLY[ 0 ]	(8-622)

cpMvpListLX[ numCpMvpCandLX ][ 1 ] = cpMvpLY[ 1 ]	(8-623)

cpMvpListLX[ numCpMvpCandLX ][ 2 ] = cpMvpLY[ 2 ]	(8-624)

numCpMvpCandLX = numCpMvpCandLX + 1	(8-625)