| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0058 |
| --- | --- |

| Title: | CE8: BDPCM with modified binarization (Test 8.3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohsen Abdoli Gordon Clare Félix Henry Pierrick Philippe Orange Labs 4 rue Clos Courtel 35510 Cesson Sevigne France b<>com 1219 Avenue des Champs Blancs, 35510 Cesson-Sévigné France | Tel: Email: | +33 2 99 12 45 58 mohsen.abdoli@orange.com gordon.clare@orange.com gordon.clare@b-com.com felix.henry@orange.com felix.henry@b-com.com pierrick.philippe@orange.com |
| Source: | Orange, b<>com |  |  |

_____________________________

# Abstract

This contribution describes CE test 8.3.2. This test uses CE test 8.3.1b and modifies the coefficient coding binarization. It is asserted that this modification increases bin throughput. The modification consists of using a combination of truncated unary coding to the 12th bin, and second order Exp-Golomb binarization of the remainder. The performance of this test in Y-BDR gain in All Intra is as follows.

CPR Off, Class F:        -9,0% (Y), 88% (EncT),	96% (DecT)

CPR Off, Class SCC: -24,5% (Y), 73% (EncT),  99% (DecT)

CPR On, Class F:         -4,5% (Y), 92% (EncT), 107% (DecT)

CPR On, Class SCC:    -8,0% (Y), 79% (EncT), 110% (DecT)

# Reference description

In CE test 8.3.1b, the amplitude of the quantized residual of BPDCM is encoded using the following process:

- First, the highest value the amplitude can take is 32.

- Second, the amplitude is encoded using truncated unary binarization. The first bin is context coded with 3 contexts, the next 11 bins are coded with one context each, and all remaining bins are encoded with a single context.

# Test description

In this contribution, the quantized residual of BPDCM is encoded with the following modifications:

- First, the highest value the amplitude can take is 28.

- Second, the amplitude is encoded using truncated unary binarization up to level 12. The first bin is context coded with 3 contexts, the next 11 bins are coded with one context each, and all remaining bins are encoded with equiprobable bits binarized using second order Exp-Golomb codes.

# Performance

The modified software has been run under the CE8 test conditions [2] with to different configurations: CPR off and CPR on. The main goal of testing BDCM with CPR off is to show that BDPCM can reach a substantial proportion of the gains of other Screen Content tools at a fraction of the complexity.

Table 1 shows the performance of BDPCM in test 8.3.2 with CPR off in All Intra

Table 2 shows the performance of BDPCM in test 8.3.2 with CPR off in Random Access

Table 3 shows the performance of BDPCM in test 8.3.2 with CPR on in All Intra

Table 4 shows the performance of BDPCM in test 8.3.2 with CPR on in Random Access

Table 1: performance of BDPCM in test 8.3.2 with CPR off in All Intra

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,09% | 0,16% | 106% | 98% |
| Class A2 | 0,07% | 0,06% | 0,10% | 106% | 101% |
| Class B | 0,01% | 0,04% | 0,11% | 105% | 98% |
| Class C | -0,27% | -0,12% | -0,22% | 103% | 96% |
| Class E | -0,19% | -0,05% | 0,10% | 105% | 97% |
| Overall | -0,08% | 0,00% | 0,04% | 105% | 98% |
| Class D | -0,31% | -0,16% | -0,23% | 99% | 98% |
| Class F | -9,00% | -6,50% | -6,48% | 88% | 96% |
| Class TGM | -24,45% | -19,08% | -19,37% | 73% | 99% |

Table 2: performance of BDPCM in test 8.3.2 with CPR off in Random Access

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | 0,16% | 0,23% | 102% | 99% |
| Class A2 | 0,02% | 0,13% | 0,23% | 102% | 100% |
| Class B | -0,01% | 0,29% | -0,05% | 102% | 96% |
| Class C | -0,12% | 0,02% | 0,17% | 102% | 93% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | 0,16% | 0,12% | 102% | 97% |
| Class D | -0,08% | -0,28% | 0,06% | 102% | 97% |
| Class F | -6,74% | -4,97% | -4,97% | 102% | 95% |
| Class TGM | -12,06% | -8,05% | -8,17% | 102% | 95% |

Table 3: performance of BDPCM in test 8.3.2 with CPR on in All Intra

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | 0,24% | 0,11% | 105% | 103% |
| Class A2 | 0,05% | 0,01% | 0,11% | 104% | 104% |
| Class B | 0,00% | 0,08% | 0,06% | 104% | 103% |
| Class C | -0,28% | -0,20% | -0,16% | 103% | 106% |
| Class E | -0,08% | 0,07% | -0,05% | 103% | 99% |
| Overall | -0,07% | 0,03% | 0,01% | 104% | 103% |
| Class D | -0,37% | -0,17% | -0,29% | 101% | 103% |
| Class F | -4,55% | -2,60% | -2,54% | 92% | 107% |
| Class TGM | -7,95% | -5,06% | -5,17% | 79% | 110% |

Table 4: performance of BDPCM in test 8.3.2 with CPR on in Random Access

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | 0,05% | 0,04% | 102% | 99% |
| Class A2 | 0,02% | -0,01% | -0,05% | 102% | 99% |
| Class B | 0,04% | -0,07% | 0,02% | 103% | 98% |
| Class C | -0,10% | 0,04% | 0,09% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0,01% | 0,00% | 0,03% | 102% | 99% |
| Class D | -0,08% | 0,08% | 0,07% | 103% | 96% |
| Class F | -3,22% | -1,82% | -1,84% | 101% | 102% |
| Class TGM | -4,38% | -2,18% | -2,16% | 101% | 103% |

# Conclusion

CE test 8.3.2 shows the performance of BDPCM with a modified binarization of the quantized values. It is proposed to adopt BDPCM in the VVC draft text and VTM-4.0.

# Bibliography

[1] JVET-L0078, AHG11: Block DPCM for Screen Content Coding, M. Abdoli, G. Clare, F. Henry, P. Philippe (Orange)

[2] JVET-L1028 Description of Core Experiment 8 (CE8): Screen Content Coding Tools, X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu

Orange and b<>com may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).