| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0068 |
| --- | --- |

| Title: | Non-CE4: MMVD scaling fix |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Eiichi Sasaki Tomohiro Ikai | Tel: Email: | +81-43-299-8526 eiichi.sasaki@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a fix on MMVD motion vector calculation. VTM3.0 MMVD flips / scales the motion vector difference (MVD) two times, i.e. with POC direction and with POC distance between L0 and L1 reference picture. This two time scaling creates the case where not mirrored motion vector is used (when the POC direction is mirrored and POC distance is different) while MMVD uses mirrored motion vectors in other most cases. This two time scaling causes confusions and it is reported this non mirrored motion usage is not efficient.

Experimental results shows -0.02% luma BD-rate change in RA and there is no change in LB.

WD change

We just moved POC direction part otherwise part not to scale/flip twice. POC distance part is also modified accordingly where the shorter MVD is derived from the larger one.

8.3.2.7 Derivation process for merge motion vector difference

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Outputs of this process are the luma merge motion vector differences in 1/16 fractional-sample accuracy mMvdL0 and mMvdL1.

The variable currPic specifies the current picture.

The luma merge motion vector differences mMvdL0 and mMvdL1 are derived as follows:

- If both predFlagL0 and predFlagL1 are equal to 1, the following applies:

currPocDiffL0  =  DiffPicOrderCnt( currPic, RefPicList0[ refIdxL0 ] )	(8-272)

currPocDiffL1  =  DiffPicOrderCnt( currPic, RefPicList1[ refIdxL1 ] )	(8-273)

- If −currPocDiffL0 * currPocDiffL1 is greater than 0, the following applies:

mMvdL0[ 0 ]  =  MmvdOffset[ 0 ]		(8-274)

mMvdL0[ 1 ]  =  MmvdOffset[ 1 ]		(8-275)

mMvdL1[ 0 ]  =  −MmvdOffset[ 0 ]		(8-276)

mMvdL1[ 1 ]  =  −MmvdOffset[ 1 ]		(8-277)

  - If currPocDiffL0 is equal to currPocDiffL1Otherwise ( −currPocDiffL0 * currPocDiffL1 is less than 0 ), the following applies:

mMvdL0[ 0 ]  =  MmvdOffset[ 0 ]		(8-278)

mMvdL0[ 1 ]  =  MmvdOffset[ 1 ]		(8-279)

mMvdL1[ 0 ]  =  MmvdOffset[ 0 ]		(8-280)

mMvdL1[ 1 ]  =  MmvdOffset[ 1 ]		(8-281)

  - Otherwise if Abs( currPocDiffL0 ) is greater than Abs( currPocDiffL1 ), the following applies:

td = Clip3( −128, 127, currPocDiffL0 )		(8-282)

tb = Clip3( −128, 127, currPocDiffL1 )		(8-283)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(8-284)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-285)

mMvdL0[ 0 ]  =  MmvdOffset[ 0 ]		(8-278)

mMvdL0[ 1 ]  =  MmvdOffset[ 1 ]		(8-279)

mMvdL1[ 0 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL0L1[ 0 ] )  *  	(8-286)
	( ( Abs( distScaleFactor * mMvdL0L1[ 0 ] ) + 127 )  >>  8 ) )

mMvdL1[ 1 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL0L1[ 1 ] )  *  	(8-287)
	( ( Abs( distScaleFactor * mMvdL0L1[ 1 ] ) + 127 )  >>  8 ) )

- Otherwise if Abs( currPocDiffL0 ) is less than or equal to Abs( currPocDiffL1 ), the following applies:

td = Clip3( −128, 127, currPocDiffL1 )		(8-288)

tb = Clip3( −128, 127, currPocDiffL0 )		(8-289)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(8-290)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-291)

mMvdL1[ 0 ]  =  MmvdOffset[ 0 ]		(8-280)

mMvdL1[ 1 ]  =  MmvdOffset[ 1 ]		(8-281)

mMvdL0[ 0 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL1L0[ 0 ] )  *  	(8-292)
	( ( Abs( distScaleFactor * mMvdL1L0[ 0 ] ) + 127 )  >>  8 ) )

mMvdL0[ 1 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL1L0[ 1 ] )  *  	(8-293)
	( ( Abs( distScaleFactor * mMvdL1L0[ 1 ] ) + 127 )  >>  8 ) )

- Otherwise ( predFlagL0 or predFlagL1 are equal to 1 ), the following applies for X being 0 and 1:

mMvdLX[ 0 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ 0 ]  :  0	(8-294)

mMvdLX[ 1 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ 1 ]  :  0	(8-295)

Experimental Results

The results of proposed fix is shown in Table 1.

Table 1. Results of proposed fix

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.06% | 0.04% | 100% | 99% |
| Class A2 | -0.03% | -0.06% | -0.01% | 100% | 100% |
| Class B | -0.04% | -0.03% | -0.10% | 100% | 100% |
| Class C | -0.03% | 0.00% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.01% | -0.02% | 100% | 100% |
| Class D | -0.07% | -0.08% | 0.06% | 100% | 100% |
| Class F | -0.01% | -0.01% | 0.01% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

Conclusions

In this contribution, a MMVD motion calculation fix about flipping / scaling is proposed.

References

- M. W. Park, Y. Piao, M. Park, K. Choi, “CE4 Ultimate motion vector expression,” JVET-L0054, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).