| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0116 |
| --- | --- |

| Title: | CE2-related: ATMVP simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu Du Liu Kenneth Andersson Rickard Sjöberg | Email: | ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In VTM-3.0, the advanced temporal motion vector prediction (ATMVP) includes a process for deriving a motion candidate from spatial neighboring blocks to identify a collocated block. This derivation process requires four neighboring block accesses for motion information in the worst case. The contribution describes two different methods to simplify the derivation process and proposes that one of them is adopted to the VVC draft.

The first method always selects the block to the left of the current block, also known as block A1.

The second method always selects the last entry from the history-based motion vector prediction (HMVP) table and uses that as the motion-shift candidate.

Both methods were tested under the VTM-3.0 random access (RA) and low delay B (LDB) common test configurations. The BD-rate results for the first method are 0.00% RA, 0.02% LDB. The BD-rate results for the second method are 0.00% RA, 0.01% LDB. It is asserted by the proponents that both methods simplify the derivation while having negligible impact on the BD-rate performance.

# Background

ATMVP is similar to the temporal motion vector prediction (TMVP) in HEVC. It fetches motion information from a collocated picture. The collocated picture is signaled in the slice header. The ATMVP scheme in VTM-3.0 consists of the following two steps.

The first step is to find a motion vector to be used for identifying a collocated block in the collocated picture.

A motion-shift candidate is first selected by scanning the spatial neighboring blocks in a predefined order. There are four candidate neighboring blocks as shown in Figure 1. The order of scanning is A1 – B1 – B0 – A0. The motion information from the first block in scan order that is an inter block is selected as the motion-shift candidate. In the worst case, the number of accesses to spatial neighboring blocks is 4.

After the motion-shift candidate is selected, a condition check follows. It checks if any of the motion vectors (mvL0 and mvL1) from the motion-shift candidate points to the collocated picture (i.e. uses the collocated picture as the reference picture). The order of checking mvL0 and mvL1 depends on the current slice’s reference pictures POC values and the collocated_from_l0_flag in the slice header.  The first motion vector that fulfils the criterion is used for identifying a collocated block in the collocated picture. If no motion vector fulfils the criterion, zero motion vector is used.

Figure 1. Spatial neighboring blocks in the derivation process

The second step split the current block into sub-blocks with size of 8x8 and derive motion information for each sub-block from a corresponding block in the collocated picture. The corresponding block is identified by the motion vector found from the first step.

An example is shown in Figure 2. In the example, a current block T with size of 16x16 is split in to sub-blocks T0, T1, T2 and T3. For each sub-block, its motion information is fetched (with proper motion scaling applied similar to TMVP) from the collocated sub-blocks in the collocated picture. The positions of the collocated sub-blocks are identified by the motion vector found in the first step. If the current sub-block has its center position at (xC, yC) and the motion vector (in 16th pel resolution) from the first step is (mvX, mvY), then the collocated position colC (xColC, yColC) is calculated by the following equation, where the value of the shift variable is equal to 4 to adjust the motion vector to full-pel resolution.

- xColC = ( (xC + (mvX >> shift) ) >> 3 ) << 3

- yColC = ( (yC + (mvY >> shift) ) >> 3 ) << 3

The sub-block in the collocated picture that covers the position colC is used for deriving motion information of the current sub-block.

Figure 2. An example of second step in ATMVP

# Proposal

In this contribution, two different methods are proposed to address the worst-case number of accessing spatial neighboring block in the current ATMVP.

Both methods only change the motion-shift candidate derivation in ATMVP. The remaining part of ATMVP is kept unmodified.

## Method 1 - Restrict to access one spatial neighboring block

The method restricts the ATMVP process to access only one spatial neighboring block. Specifically, only the left (A1 in Figure 1) spatial neighboring block is used. Blocks A0, B0, B1 are never accessed.

## Method 2 - Fetch the last entry of HMVP table

The HMVP method in VTM maintains a table of HMVP candidates. The HMVP candidates corresponds to the motion information from already encoded or decoded inter blocks. The table is updated during the decoding process.

The method fetches the last entry from the HMVP table and use that as the motion-shift candidate. In other words, the most recent HMVP candidate is used instead of any of the A0, A1, B0, B1 blocks.

# Results

The two proposed methods are implemented in the VTM-3.0 reference software [1] and tested using the VTM common test configuration [2].

In addition, a third test which always uses zero motion vector for finding the collocated block in the collocated picture was done. The purpose of that test is for information.

## Results of method 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.03% | 103% | 101% |
| Class A2 | 0.00% | 0.04% | 0.04% | 100% | 102% |
| Class B | 0.00% | -0.03% | -0.01% | 99% | 99% |
| Class C | 0.02% | -0.01% | -0.01% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.01% | 101% | 100% |
| Class D | 0.00% | 0.04% | 0.03% | 105% | 101% |
| Class F | -0.01% | -0.02% | -0.01% | 98% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.22% | 0.13% | 101% | 104% |
| Class C | -0.01% | 0.18% | 0.26% | 100% | 98% |
| Class E | 0.06% | -0.25% | -0.16% | 103% | 101% |
| Overall | 0.02% | -0.09% | 0.10% | 101% | 101% |
| Class D | -0.02% | 0.00% | -0.10% | 94% | 94% |
| Class F | -0.01% | -0.03% | -0.49% | 92% | 94% |

## Results of method 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.00% | 99% | 99% |
| Class A2 | -0.02% | 0.02% | 0.07% | 100% | 97% |
| Class B | 0.00% | -0.02% | -0.03% | 94% | 102% |
| Class C | -0.01% | -0.03% | -0.08% | 97% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.02% | 97% | 100% |
| Class D | 0.02% | -0.16% | 0.03% | 101% | 101% |
| Class F | 0.00% | 0.03% | 0.03% | 99% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.23% | 0.00% | 99% | 104% |
| Class C | -0.02% | 0.15% | 0.06% | 99% | 96% |
| Class E | 0.03% | 0.05% | -0.06% | 99% | 98% |
| Overall | 0.01% | -0.03% | 0.01% | 99% | 100% |
| Class D | -0.04% | 0.02% | -0.01% | 85% | 84% |
| Class F | -0.01% | 0.06% | 0.46% | 90% | 88% |

## Additional results of always using zero motion vector

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.02% | 0.03% | 83% | 91% |
| Class A2 | 0.06% | 0.03% | 0.09% | 80% | 94% |
| Class B | 0.03% | 0.05% | -0.02% | 81% | 87% |
| Class C | 0.10% | 0.09% | 0.07% | 83% | 89% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.04% | 0.04% | 82% | 90% |
| Class D | 0.02% | -0.02% | 0.00% | 82% | 90% |
| Class F | 0.01% | -0.02% | -0.03% | 80% | 88% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.06% | 0.06% | 82% | 90% |
| Class C | 0.07% | 0.29% | 0.08% | 105% | 106% |
| Class E | 0.02% | 0.51% | -0.10% | 81% | 87% |
| Overall | 0.04% | 0.20% | 0.03% | 89% | 95% |
| Class D | -0.05% | -0.47% | 0.15% | 88% | 86% |
| Class F | 0.06% | -0.10% | -0.27% | 70% | 74% |

# Conclusion

In this contribution, two different methods are proposed to address the worst-case number of accessing spatial neighboring block in the current ATMVP. We recommend adopting one of the proposed method into next VTM and VVC working draft.

# References

[1]. VTM-3.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0.

[2]. F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).