| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0122-v1 |
| --- | --- |

| Title: | Non-CE3: On block size restrictions for PDPC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Jianle Chen | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co. |  |  |

_____________________________

# Abstract

This document proposes block size restrictions for PDPC. In Test 1-3, PDPC is disabled for small and large blocks that reportedly shows the following overall results (Test 3): 0.01% (Y), -0.03% (U), 0.00% (V) and 0.02% (Y), 0.07% (U), 0.04% (V) for AI and RA configurations, respectively. The running time is changed negligibly at both encoder and decoder sides for AI and RA configurations as compared with VTM-3.0.

# Introduction

PDPC (Position Dependent Prediction Combination) is an efficient tool used to improve compression performance of conventional intra prediction modes. However, PDPC can increase worst-case complexity. In this contribution, we propose several ways of how to reduce its adverse impact on worst-case complexity by disabling PDPC for some block sizes and intra prediction modes. To compensate for some coding efficiency loss caused by disabling PDPC, we modify calculation of PDPC scales to make them mode-dependent.

# Proposal

In this contribution, we propose to disable PDPC for blocks that meet the condition sets described in Table 1. The proposal is aimed at disabling PDPC for the worst case intra prediction modes (the planar one and the angular ones that require interpolation filtering).

Table 1. Summary of tests for the proposed block size restrictions

| Test # | Description |
| --- | --- |
| 1 | Disable PDPC for blocks with (width + height <= 8) or (width + height > 64) |
| 2 | Disable PDPC for blocks with - (width + height <= 8), - (width + height > 64) for the worst case intra prediction modes |
| 3 | Disable PDPC for blocks with (width + height <= 8) or (width + height > 64) for the worst case intra prediction modes |

To compensate for coding efficiency loss caused by disabling PDPC, we made calculation of PDPC scales mode-dependent as shown in Table 2.

Table 2. Summary of how new PDPC scales are calculated for different intra prediction mode

| Modes index | Description |
| --- | --- |
| DC_IDX, HOR_IDX or VER_IDX | scale = (g_aucLog2[iWidth] + g_aucLog2[iHeight] > 6) ? 1 : 0; Variable scale is used to compute weights wT and wL: wT = 32 >> std::min(31, ((y << 1) >> scale)); wL = 32 >> std::min(31, ((x << 1) >> scale)); |
| 2 or VDIA_IDX | scaleX = (g_aucLog2[width ] > 3) ? 1 : 0; scaleY = (g_aucLog2[height] > 3) ? 1 : 0; Variables scaleX and scaleY are used to compute weights wL and wT, respectively: wL = 16 >> std::min(31, ((x << 1) >> scaleX)); wT = 16 >> std::min(31, ((y << 1) >> scaleY)); |
| The worst case intra prediction modes | scaleX = (g_aucLog2[iWidth ] - 2 + 1) >> 1; scaleY = (g_aucLog2[iHeight] - 2 + 1) >> 1; Variables scaleX and scaleY are used to compute weights wL and wT, respectively: wL = 32 >> std::min(31, ((x << 1) >> scaleX)); wT = 32 >> std::min(31, ((y << 1) >> scaleY)); |

Tests to evaluate mode-dependent PDPC scales are described in Table 3.

Table 3. Summary of tests related to mode-dependent PDPC scales

| Test # | Description |
| --- | --- |
| 4 | Mode-dependent PDPC scales as described in Table 2 on top of VTM-3.0 |
| 5 | Mode-dependent PDPC scales as described in Table 2 on top of Test 1 |
| 6 | Mode-dependent PDPC scales as described in Table 2 on top of Test 2 |
| 7 | Mode-dependent PDPC scales as described in Table 2 on top of Test 3 |

# Experiment Results

The results reportedly obtained for tests 1-7 using the common test conditions [1] are presented in Table 4-Table 10 for AI and RA configurations.

Table 4. Experimental results for Test 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.01% | -0.02% | 100% | 99% |
| Class A2 | 0.05% | 0.06% | 0.03% | 100% | 100% |
| Class B | 0.04% | 0.02% | 0.01% | 100% | 99% |
| Class C | 0.04% | 0.02% | 0.01% | 99% | 99% |
| Class E | 0.08% | 0.09% | 0.05% | 100% | 99% |
| Overall | 0.03% | 0.04% | 0.02% | 100% | 99% |
| Class D | 0.06% | -0.04% | -0.03% | 99% | 100% |
| Class F | -0.17% | -0.25% | -0.13% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.13% | 0.17% | 100% | 100% |
| Class A2 | 0.04% | 0.07% | 0.14% | 99% | 99% |
| Class B | 0.02% | 0.07% | 0.02% | 99% | 100% |
| Class C | 0.04% | -0.13% | 0.02% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.03% | 0.07% | 99% | 100% |
| Class D | 0.05% | -0.42% | 0.05% | 99% | 99% |
| Class F | -0.20% | -0.11% | -0.04% | 99% | 100% |

Table 5. Experimental results for Test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.02% | 0.07% | 99% | 99% |
| Class A2 | 0.04% | 0.05% | -0.02% | 99% | 100% |
| Class B | 0.04% | 0.04% | 0.08% | 99% | 99% |
| Class C | 0.05% | 0.00% | -0.01% | 100% | 100% |
| Class E | 0.08% | 0.11% | 0.08% | 99% | 100% |
| Overall | 0.03% | 0.03% | 0.04% | 99% | 100% |
| Class D | 0.07% | -0.04% | -0.04% | 100% | 100% |
| Class F | -0.18% | -0.21% | -0.19% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.11% | 0.07% | 99% | 100% |
| Class A2 | 0.01% | 0.10% | 0.09% | 99% | 100% |
| Class B | 0.03% | 0.16% | 0.10% | 99% | 100% |
| Class C | 0.05% | -0.14% | -0.08% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.06% | 0.05% | 99% | 100% |
| Class D | 0.03% | -0.44% | 0.10% | 100% | 100% |
| Class F | -0.21% | -0.14% | -0.12% | 100% | 100% |

Table 6. Experimental results for Test 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.05% | 0.05% | 100% | 99% |
| Class A2 | 0.04% | 0.04% | -0.02% | 99% | 99% |
| Class B | 0.02% | 0.01% | -0.01% | 100% | 100% |
| Class C | -0.01% | -0.10% | -0.08% | 100% | 100% |
| Class E | 0.04% | -0.09% | 0.07% | 99% | 99% |
| Overall | 0.01% | -0.03% | 0.00% | 99% | 99% |
| Class D | 0.04% | -0.14% | -0.21% | 100% | 100% |
| Class F | -0.13% | -0.21% | -0.04% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.18% | 0.20% | 100% | 100% |
| Class A2 | 0.02% | 0.08% | 0.07% | 100% | 100% |
| Class B | 0.02% | 0.08% | -0.01% | 100% | 99% |
| Class C | 0.04% | -0.04% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.07% | 0.04% | 100% | 100% |
| Class D | 0.02% | -0.45% | -0.03% | 100% | 100% |
| Class F | -0.13% | -0.18% | -0.30% | 100% | 99% |

Table 7. Experimental results for Test 4

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.13% | 0.04% | 101% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.04% | -0.06% | -0.06% | 99% | 100% |
| Class C | -0.05% | -0.03% | -0.23% | 100% | 100% |
| Class E | -0.05% | 0.01% | -0.08% | 100% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.03% | -0.16% | 0.00% | 100% | 101% |
| Class F | -0.02% | -0.06% | -0.04% | 102% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.03% | 0.09% | 0.03% | 101% | 100% |
| Class C | -0.02% | -0.14% | 0.18% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.04% | -0.40% | -0.07% | 100% | 100% |
| Class F | -0.03% | -0.13% | 0.04% | 101% | 102% |

Table 8. Experimental results for Test 5

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.10% | -0.09% | 100% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0.00% | -0.06% | -0.01% | 99% | 100% |
| Class C | 0.00% | -0.12% | -0.11% | 100% | 100% |
| Class E | 0.01% | -0.06% | -0.13% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.04% | 0.06% | -0.09% | 100% | 101% |
| Class F | -0.21% | -0.26% | -0.11% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0.00% | 0.14% | 0.02% | 100% | 100% |
| Class C | 0.05% | -0.17% | 0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.02% | -0.59% | -0.08% | 100% | 100% |
| Class F | -0.21% | -0.11% | 0.05% | 101% | 102% |

Table 9. Experimental results for Test 6

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0.00% | -0.04% | 0.01% | 99% | 100% |
| Class C | -0.01% | -0.11% | -0.11% | 100% | 100% |
| Class E | 0.01% | 0.06% | -0.06% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.04% | 0.05% | -0.09% | 100% | 101% |
| Class F | -0.22% | -0.25% | -0.06% | 102% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0.00% | 0.09% | 0.01% | 101% | 100% |
| Class C | 0.06% | -0.23% | 0.21% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.03% | -0.58% | -0.03% | 100% | 101% |
| Class F | -0.20% | -0.12% | -0.06% | 102% | 102% |

Table 10. Experimental results for Test 7

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.04% | 0.01% | 100% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.01% | -0.05% | -0.13% | 99% | 100% |
| Class C | -0.05% | -0.12% | -0.27% | 100% | 100% |
| Class E | -0.04% | -0.09% | -0.05% | 100% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.02% | -0.21% | -0.16% | 100% | 100% |
| Class F | -0.17% | -0.25% | -0.11% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.02% | 0.09% | -0.22% | 100% | 100% |
| Class C | 0.02% | -0.16% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.01% | -0.65% | -0.40% | 100% | 100% |
| Class F | -0.15% | -0.04% | -0.13% | 101% | 102% |

# Conclusion

We propose to adopt the proposed block size restrictions for PDPC into the VTM software and the VVC spec draft.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, 12th JVET Meeting, Macao, China, October 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).