| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0124 |
| --- | --- |

| Title: | CE4: Methods of Reducing Number of Pruning Checks of History Based Motion Vector Prediction (Test 4.1.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, U.S.A | Tel: Email: | 1-201-816-2000 jie.zhao@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

This document reports result of CE4.1.1 – “Methods of Reducing Number of Pruning Checks of History Based Motion Vector Prediction”. Goal of the proposed methods is to reduce HMVP worst case number of pruning checks by limiting the number of HMVP candidates to check depending on the existing number of candidates in merge list.

In VTM3, the worst case number of pruning checks, denoted as Mprune, to add HMVP candidates to merge list is 10 [1]. All methods proposed in this CE reduce the worst case number of prune checks with little impact on coding performance and run time. Some tests even have small gains.

CE4.1.1a) reduces Mprune to 6, with performance impact of (0.00%, 0.07%, 0.02%) for RA YUV components, and (0.07%, 0.14%, 0.22%) for LD, YUV components respectively.

CE4.1.1b) reduces Mprune to 8, with performance impact of (0.00%, 0.02%, 0.06%) for RA YUV components, and (0.02%, 0.01%, 0.00%) for LD, YUV components respectively.

CE4.1.1c) reduces Mprune to 7, with performance impact of (-0.04%, 0.00%, -0.01%) for RA YUV components, and (0.05%, 0.06%, 0.00%) for LD, YUV components respectively.

CE4.1.1d) reduces Mprune to 8, with performance impact of (-0.02%, 0.00%, 0.01%) for RA YUV components, and (0.03%, -0.04%, -0.02%) for LD, YUV components respectively.

# Introduction

In Oct. 2018 JVET meeting, History-based Motion Vector Prediction (HMVP) [1] was adopted into VTM3. A table with 6 HMVP candidates is maintained during the encoding/decoding process, and updated when there is an inter-coded non-affine block. When updating the HMVP table, redundancy check is applied to find whether there is an identical HMVP in the table. If found, the identical HMVP is removed from the table and all the HMVP candidates afterwards are moved forward, i.e., with indices reduced by 1.

HMVP candidates are used in merge candidate list construction and AMVP candidate list construction process. Pruning is applied on the HMVP candidates relative to the existing spatial or temporal merge candidate in the merge list before an HMVP candidate being added into the merge or AMVP list.

Once the total number of available merge candidates reaches the MaxNumMergeCand minus 1, the merge candidate list construction process from HMVP list is terminated. MaxNumMergeCand is the maximum number of merging MVP candidate, and shall be in the range of 1 to 6, inclusive. In VTM3 common test condition, MaxNumMergeCand is 6, therefore HMVP candidate is added only up to merge candidate 5.

To understand how the number of pruning checks of HMVP is calculated, the following paragraph is copied from JVET-L0266.

“For the computational complexity, it is mainly from the additional pruning operations. Two procedures of pruning operations are required by HVMP.

- One is invoked in the table updating process when inserting a new HMVP candidate, wherein up to 6 times of pruning operations are required.

- The other one is invoked in the merge list construction process. Thanks to the pruning process in the table updating process, all HMVP candidates in the table are always different from each other. Therefore, in the merge list construction process, there is no need for a second HMVP candidate to compare with a spatial or temporal candidate if it has been marked as identical to a first HMVP candidate. The number of additional pruning operations could be defined as C(NST, NmrgToBeAdded, NHMVP) wherein NST indicates the number of spatial or temporal merge candidates already in the merge candidate list, NmrgToBeAdded indicates the largest number of candidates from HMVP may be added to the merge list, and NHMVP indicates the number of HMVP candidates to be checked, respectively. In this contribution, NmrgToBeAdded is set to Nmrg - 1.

- When NHMVP is not smaller than NmrgToBeAdded, the worst case happens when the first NmrgToBeAdded - 1, candidates are all different from the first NST candidates and the remaining HMVP candidates are identical to one of the first NST candidates. In total, it requires  times of pruning operations.

- When NHMVP is smaller than NmrgToBeAdded, the worst case happens when all NHMVP, candidates are different from the first NST candidates which requires times of pruning operations.

”

In VTM3, the worst case number of pruning checks needed to insert HMVP candidates to merge list (denoted as Mprune ) is 10.   The details of pruning operations for each value of number of existing spatial or temporal merge candidates denoted by NST in the worst case is shown Table 1. Here the number of HMVP checked NHMVPchecked equals to the number of available HMVP candidates, denoted as NHMVP.

Table 1. Additional pruning operations introduced by HMVP in worst case

| C(NST , NmrgToBeAdded , NHMVP ) | Mprune |
| --- | --- |
| C(0, 5, 6) | 0 |
| C(1, 4, 6) =1*(4-1)+1 | 4 |
| C(2, 3, 6) =2*(3-1)+2+1 | 7 |
| C(3, 2, 6) = 3*(2-1)+3+2+1 | 9 |
| C(4, 1, 6) = 4*(1-1)+4+3+2+1 | 10 |

# Proposed

In JVET-L0309 [2], a method was proposed to limit the number of HMVP candidates to check based on how many spatial and temporal candidates already exist in the merge list. The idea is that if there are not many spots left in the merge list for the HMVP candidates, it may not need to check all the available HMVP candidates in the table. The benefit is to reduce the number of pruning checks when inserting HMVP to the merge list.

Four subtests with different thresholds are tests as listed below.

- NHMVPChecked = 5 - NST  .    Threshold 5 is picked because it equals to maxNumMergeCand -1 , which is the max number of HMVP candidates allowed to be added to the merge list, so 5- NST is how many spots left in the merge list for HMVP candidates, i.e. NmrgToBeAdded

- NHMVPChecked = 6 - NST  .       6 is picked because it equals the maxNumMergeCand

- NHMVPChecked = (NST >=3) ? 5 - NST  :  NHMVP   .  This is applying a) when NST >=3

- NHMVPChecked = (NST >=3) ? 6 - NST  :  NHMVP  .  This is applying b) when NST >=3

Mprune -- the number of pruning checks in worst case for each of subtests are listed below. As reference, currently in VTM3, the worst case number of pruning checks for HMVP is 10.

Table 2 Worst case pruning operations introduced by HMVP with proposed methods

| a) NHMVPChecked = 5 - NST | b) NHMVPChecked = 6 - NST |
| --- | --- |
| C(NST, NmrgToBeAdded,NHMVPChecked ); Mprune. C(0, 5, 5); 0. C(1, 4, 4) = 1*(4-1)+1; 4. C(2, 3, 3) = 2*(3-1)+2; 6. C(3, 2, 2) = 3*(2-1)+3; 6. C(4, 1, 1) = 4*(1-1)+4; 4. | C(NST, NmrgToBeAdded, NHMVPChecked); Mprune. C(0, 5, 6); 0. C(1, 4, 5) = 1*(4-1)+1; 4. C(2, 3, 4) = 2*(3-1)+2+1; 7. C(3, 2, 3) = 3*(2-1)+3+2; 8. C(4, 1, 2) = 4*(1-1)+4+3; 7. |
| c) NHMVPChecked = (NST >=3) ? 5 - NST : NHMVP | - NHMVPChecked = (NST >=3) ? 6 - NST : NHMVP |
| (NST , NmrgToBeAdded, NHMVPChecked ); Mprune. C(0, 5, 6); 0. C(1, 4, 6) =1*(4-1)+1; 4. C(2, 3, 6) =2*(3-1)+2+1; 7. C(3, 2, 2) = 3*(2-1)+3; 6. C(4, 1, 1) = 4*(1-1)+4; 4. | (NST , NmrgToBeAdded, NHMVPChecked ); Mprune. C(0, 5, 6); 0. C(1, 4, 6) =1*(4-1)+1; 4. C(2, 3, 6) =2*(3-1)+2+1; 7. C(3, 2, 3) = 3*(2-1)+3+2; 8. C(4, 1, 2) = 4*(1-1)+4+3; 7. |

Table 3. Change in the number of candidates comparisons/pruning operations

| CE4.1.1 | Worst case of # of HMVP prunings | Difference in the number of candidate comparison (+/- xx) | Max number of pruning stages | Others |
| --- | --- | --- | --- | --- |
| a | 6 | -4 | unchanged |  |
| b | 8 | -2 | unchanged |  |
| c | 7 | -3 | unchanged |  |
| d | 8 | -2 | unchanged |  |

# Experimental Results

Proposed methods in this CE are implemented into VTM3, and tested following CE4 description [3] and JVET common test condition [4].

Table 4.1 Result of CE4.1.1.a (NHMVPChecked = 5 - NST)

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.04% | -0.01% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.02% | 0.11% | 0.13% | 100% | 99% |  |  |  |  |  |
| Class B | 0.05% | 0.09% | 0.00% | 100% | 100% | 0.15% | -0.02% | 0.18% | 101% | 101% |
| Class C | -0.02% | 0.09% | 0.00% | 101% | 102% | 0.04% | 0.27% | 0.38% | 101% | 97% |
| Class E |  |  |  |  |  | -0.02% | 0.25% | 0.06% | 100% | 97% |
| Overall | 0.00% | 0.07% | 0.02% | 100% | 100% | 0.07% | 0.14% | 0.22% | 101% | 99% |
| Class D | -0.09% | -0.23% | -0.05% | 101% | 103% | 0.00% | 0.65% | -0.01% | 101% | 102% |
| Class F | 0.06% | -0.01% | -0.03% | 101% | 102% | 0.12% | -0.09% | -0.11% | 100% | 98% |

Table 4.2 Result of CE4.1.1.b  (NHMVPChecked = 6 - NST)

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.02% | 0.01% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.02% | 0.04% | 0.08% | 100% | 100% |  |  |  |  |  |
| Class B | 0.01% | 0.05% | 0.07% | 100% | 100% | 0.05% | -0.12% | -0.01% | 101% | 101% |
| Class C | 0.01% | 0.00% | 0.09% | 101% | 102% | 0.01% | 0.39% | 0.14% | 101% | 98% |
| Class E |  |  |  |  |  | -0.02% | -0.28% | -0.17% | 101% | 99% |
| Overall | 0.00% | 0.02% | 0.06% | 100% | 101% | 0.02% | 0.01% | 0.00% | 101% | 100% |
| Class D | -0.04% | 0.05% | -0.08% | 101% | 102% | 0.05% | 0.45% | 0.29% | 101% | 103% |
| Class F | 0.02% | -0.02% | 0.00% | 101% | 101% | 0.24% | 0.20% | 0.44% | 101% | 101% |

Table 4.2 Result of CE4.1.1.c (NHMVPChecked = (NST >=3) ? 5 - NST  :  NHMVP)

./

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.06% | -0.01% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.04% | 0.09% | 0.11% | 100% | 99% |  |  |  |  |  |
| Class B | -0.01% | 0.00% | -0.08% | 100% | 100% | 0.10% | -0.16% | 0.03% | 100% | 97% |
| Class C | -0.06% | -0.03% | -0.01% | 101% | 102% | -0.01% | 0.21% | 0.22% | 101% | 101% |
| Class E |  |  |  |  |  | 0.05% | 0.21% | -0.35% | 101% | 100% |
| Overall | -0.04% | 0.00% | -0.01% | 100% | 100% | 0.05% | 0.06% | 0.00% | 101% | 99% |
| Class D | -0.10% | -0.06% | -0.10% | 102% | 101% | 0.01% | 0.36% | 0.69% | 101% | 102% |
| Class F | 0.01% | -0.02% | 0.00% | 101% | 102% | 0.16% | 0.15% | 0.52% | 101% | 99% |

Table 4.4 Result of CE4.1.1.d (NHMVPChecked = (NST >=3) ? 6 - NST  :  NHMVP)

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.08% | -0.10% | 99% | 99% |  |  |  |  |  |
| Class A2 | 0.00% | -0.01% | 0.07% | 99% | 99% |  |  |  |  |  |
| Class B | -0.01% | 0.09% | 0.02% | 100% | 97% | 0.04% | -0.12% | -0.04% | 100% | 99% |
| Class C | -0.04% | -0.03% | 0.05% | 100% | 100% | 0.00% | 0.11% | 0.42% | 100% | 101% |
| Class E |  |  |  |  |  | 0.04% | -0.11% | -0.60% | 100% | 101% |
| Overall | -0.02% | 0.00% | 0.01% | 100% | 99% | 0.03% | -0.04% | -0.02% | 100% | 100% |
| Class D | -0.02% | -0.04% | 0.09% | 101% | 102% | 0.05% | 0.04% | 0.63% | 100% | 101% |
| Class F | -0.02% | -0.08% | -0.01% | 100% | 99% | -0.05% | 0.10% | -0.13% | 100% | 101% |

# Conclusion

Methods to reduce the worst case number of HMVP pruning checks are proposed. Results of different threshold are presented. They all reduce the worst case number of HMVP pruning checks with little impact on performance. Based on the reduction of pruning checks and results, we propose to adopt method a or c.

With method a), the number of HMVP to check is set to the number of HMVP candidates to be added, the worst case number of HMVP pruning checks is reduced to 6 from 10 with little impact on performance --  (0.00%, 0.07%, 0.02%) for RA YUV components, and (0.07%, 0.14%, 0.22%) for LD, YUV components  respectively .

With method c), the rule of a) is applied when the number of existing candidates is greater or equal to 3. The worst case number of HMVP pruning checks is reduced to 7 from 10 with little impact on performance (-0.04%, 0.00%, -0.01%) for RA YUV components, and (0.05%, 0.06%, 0.00%) for LD, YUV components respectively. There is even a small gain for RA.

# Reference

- L. Zhang, K. Zhang, H. Liu, Y. Wang, P. Zhao, D. Dong, “CE4: History-based Motion Vector Prediction (Test 4.4.7),” JVET-L0266, Macao, CN, Oct 2018.

- J. Zhao, S. Paluri, S. Kim, “CE4-related: Simplification to History Based Motion Vector Prediction,” JVET-L0309, Macao, CN, Oct 2018.

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVT-L1024, Macao, CN, Oct 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-L1010 Macao, CN, Oct 2018.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).