| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0126 |
| --- | --- |

| Title: | CE4: Modification on History-based Motion Vector Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu Han, Wei-Jung Chien, Chun-Chi Chen, Han Huang, Chao-Hsiung Hung, Yan Zhang, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1-858-845-1793 yuhan@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents 3 changes on History-based Motion Vector Prediction (HMVP) mode [1] and pairwise candidates not using HMVP predictors which reduce the complexity in both encoder side and decoder side. For the “VTM3.0 as reference” tests in RA case with the size of merge list equal to 6, the presented method with HMVP buffer size equal to 10, the sub test a) “first 3 HMVPs pruned by left and above spatial candidates” achieved 0.03% loss in terms of average luma BD-rate; the sub test b) “first 2 HMVPs pruned by left and above spatial candidates” achieved 0.03% loss in terms of average luma BD-rate with 59% time saving for HMVP table update and adding HMVP candidates in merge list in the decoder side.

# Introduction

The HMVP method for merge mode [1] is firstly presented in 11th JVET meeting and studied in CE4. In this method, a table with multiple HMVP candidates is generated during encoder/decoding process. A First-In-First-Out (FIFO) rule is applied to updating the table, and redundancy check is also use when inserting a new HMVP candidate to the table. The table must be updated at every time encoding/decoding an inter CU. HMVP candidates are pruned by all available candidates in the merge list, so parallel derivation is not possible.

Pairwise candidates derived from the merge candidates in the merge list which has dependency on HMVP predictors.

This contribution presents simplification methods to reduce the complexity of HMVP mode and achieves comparable coding gain.

# Technical description

## Update HMVP table without pruning

In VTM3.0, the size of HMVP table is 6, the size of merge list is 6 and the last merge candidate can not be HMVP. In the worst case, 4 pruning operations are need for updating the HMVP table at every time of decoding an inter CU in the decoder side. The pruning operations include reference direction, reference POC, and motion vector comparisons between two motion vector predictors.

Since the HMVP table is used only when the CU is coded in merge mode and refer to a HMVP candidate, it is not necessary to update the HMVP table at every time of encoding/decoding an inter CU. In this conurbation, redundancy check is removed from HMVP table update. Without redundancy check, the HMVP table can be easily implemented in circle buffer which is hardware friendly design.

The run time saving for HMVP table update and adding HMVP to merge list is as below (calculate the run time of functions of addMotionInfoToLUTs() and addMergeHMVPCand()). In the decoder side, comparing with VTM3.0, sub test CE4.1.2.b achieved 59% time saving in RA case, achieved 62% time saving in LB case.

timeOfHMVP=timeOfaddMotionInfoToLUTs()+timeOf(addMergeHMVPCand())

timeSave=1-timeOfHMVPproposedtimeOfHMVPVTM3.0

Table 1. Time saving in decoder side

## Subsampling HMVP table when adding HMVP candidates in merger list

In this contribution, the size of HMVP table is increased to 10, and one out of every 3 is picked to add in the merge list. At most 4 HMVP candidates can be used.

In order to reduce the number of pruning, partial pruning is used when adding a new HMVP candidate in merge list. In the sub test CE4.1.2.a, only the first 3 HMVP candidates are pruned by the left and above spatial candidates. In the sub test CE4.1.2.b, only the first 2 HMVP candidates are pruned by the left and above spatial candidates. The complexity comparison is shown in the table below. In the case of merge list size equal to 6, the maximum number of pruning of CE4.1.2.a is 6, the maximum number of pruning of CE4.1.2.b is 4. The maximum number of pruning of VTM3.0 is 14. The details of pruning number comparation is shown in the table below:

Table 2. Number of pruning

|  | stage number | pruning number |
| --- | --- | --- |
| VTM3.0 | 1+4=5 | 4+(4+3+2+1)=14 |
| CE4.1.2.a | 3 | 2+2+2=6 |
| CE4.1.2.b | 2 | 2+2=4 |

## HMVP in AMVP mode

The size of HMVP table is increased to 10, and one out of every 3 HMVP candidates is picked to add in the AMVP list.

## Remove dependency between HMVP and pairwise candidates

In VTM3.0, pairwise candidates are derived from the candidates in the merge list which brings dependency between pairwise predictors and HMVP predictors. In this contribution, pairwise candidates are generated not using HMVP predictors.

# Simulation results

As descripted in CE4 [2], the simulation is based on VTM3.0 with the size of merge list equal to 6.

## The test results of CE4.1.2.a

Simulation results as below, the anchor is VTM3.0 with default setting and only the first 3 HMVP candidates are pruned by the left and above spatial candidates and pairwise candidates generation not using HMVP predictors:

## The test results of CE4.1.2.b

Simulation results as below, the anchor is VTM3.0 with default setting and only the first 2 HMVP candidates are pruned by the left and above spatial candidates and pairwise candidates generation not using HMVP predictors:

# Conclusion

This contribution presented simplification methods to reduce the complexity of HMVP mode and pairwise candidates generation not using HMVP predictors. For the “VTM3.0 as reference” tests in RA case with the size of merge list equal to 6, the presented method with HMVP buffer size equal to 10, the sub test a) “first 3 HMVPs pruned by left and above spatial candidates” achieved 0.03% loss in terms of average luma BD-rate; the sub test b) “first 2 HMVPs pruned by left and above spatial candidates” achieved 0.03% loss in terms of average luma BD-rate with 59% time saving for HMVP table update and adding HMVP candidates in merge list in the decoder side.

# References

- L. Zhang, K. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong, JVET-K0104, “CE4-related: History-based Motion Vector Prediction”, Ljubljana, July 2018.

- H. Yang, S. Liu, K. Zhang, JVET-L1024, “Description of CE4: Inter prediction and motion vector coding”, Macao, October 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).