| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0126 |
| --- | --- |

| Title: | Suggested specification changes for JVET-M0126 |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Yu Han, Wei-Jung Chien, Chun-Chi Chen, Han Huang, Chao-Hsiung Hung, Yan Zhang, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1-858-845-1793 yuhan@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document describes the suggested specification changes on top of version 9 of JVET-L1001. The newly added parts are highlighted in yellow and the text to be removed is marked as red with strikethrough

# Decoding process

#### 8.4.2.6 Derivation process for history-based merging candidates

Inputs to this process are:

- a merge candidate list mergeCandList,

- the number of available merging candidates in the list numCurrMergeCand.

Outputs to this process are:

- the modified merging candidate list mergeCandList,

- the modified number of merging candidates in the list numCurrMergeCand.

Each element of the array isPruned[ i ] with i = 0..numCurrMergeCand − 1 is set equal to FALSE and the variable numOrigMergeCand is set equal to numCurrMergeCand.

For each candidate in HmvpCandList[ hMvpIdx ] with index hMvpIdx = 1..NumHmvpCand, the following ordered steps are repeated until numCurrMergeCand is equal to ( MaxNumMergeCand − 1):

- The variable sameMotion is derived as follows:

  - If all of the following conditions are true for any i = 0..numOrigMergeCand − 1, sameMotion and isPruned[ i ] are both set equal to TRUE:

  - The candidate HmvpCandList[NumHmvpCand − hMvpIdx] is equal to the merging candidate mergeCandList[ i ].

  - hMvpIdx <= 2.

  - The candidate HmvpCandList[NumHmvpCand − hMvpIdx] is equal to the merging candidate A1 or B1.

  - isPruned[ i ] is equal to FALSE.

  - Otherwise, sameMotion is set equal to FALSE.

- When sameMotion is equal to FALSE, the candidate HmvpCandList[NumHmvpCand − hMvpIdx] is added to the merging candidate list as follows:

mergeCandList[ numCurrMergeCand++ ] = HmvpCandList[ NumHmvpCand − hMvpIdx ] 	(8-296)