| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0130-v1 |
| --- | --- |

| Title: | On tile grouping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Hendry 10180 Telesis Court San Diego, CA, 92121, USA Jianle Chen 2330 Central Expy Santa Clara, CA 95050, USA Maxim Sychev Building 1/7, Altufievskoe Shosse, Moscow 127106, Russia | Tel: Email: | +1-908-803-3888 yekui.wang@huawei.com +1-858-375-8065 fnu.hendry@huawei.com +1-858-999-4243 jianle.chen@huawei.com +7-925-008-1124 sychev.maxim@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a tile grouping scheme where each tile group contains a rectangular region of a picture. It is asserted that having tile group containing a sequence of tiles in tile raster scan of a picture cannot sufficiently support applications in which tiles may be used for region of interest (ROI) or sub-picture based picture access, wherein the access is asserted to be usually on a rectangular basis.

It is also proposed that information of which tiles are included in a tile group is signalled either in the PPS or in the tile group header, but never both.

# Introduction

## Background

The current tiling scheme for VVC is the same as in HEVC, where tiles are always in tile rows and tile columns. A tile is a sequence of CTUs that cover a rectangular region of a picture.

This document discusses tile grouping based on the current tiling scheme.

In the current tile grouping scheme for VVC, each tile group contains a sequence of tiles in tile raster scan of a picture. An example of such tile grouping is shown in Figure 1.

Figure 1 – A picture with 18 by 12 luma CTUs that is partitioned into 12 tiles and 3 tile groups

## Tile group shape

The purposes of tiles are parallel processing as well as region of interest (ROI) or sub-picture based picture access, which has recently become an important tool for enabling viewport-dependent omnidirectional video delivery optimizations. In applications with ROI support, usually each ROI is a rectangular region of a picture. Same for applications with support of sub-picture based picture access such as viewport-dependent omnidirectional video delivery.

However, the current tiling scheme does not support lots of cases with tile groups covering rectangular regions, e.g., as shown in Figure 2. Without rectangular tile group scheme, those tile groups in middle tile group row of the picture cannot be realized.

Figure 2 – A picture with 18 by 12 luma CTUs that is partitioned into 24 tiles and 9 tile groups

Then, does it make sense in any scenario for a tile group not to contain a rectangular region of a picture? One such scenario mentioned during discussion at the previous JVET meeting was support of real-time ultralow latency applications with sub-picture-interval delay, e.g., remote controlling of a drone or wireless screen sharing. Such applications could operate according to the following principles:

- They encode each picture as multiple VCL NAL units.

- Each VCL NAL unit is transmitted immediately after its encoding has been completed.

- At the receiving end, each received VCL NAL unit is decoded as soon as possible.

However, rectangular-shaped tile groups can also enable the above operation. Therefore, we propose that there is only rectangular-shaped tile groups are supported.

## Signalling of which tiles are included in a tile group

Information of which tiles are included in a tile group can be signalled in the PPS or in tile group headers.

For applications with ROI support or applications with support of sub-picture based picture access, it usually makes sense to use MCTSs and each MCTS is included in its own NAL unit. The motion constraints for MCTSs usually apply across a sequence of pictures. Therefore, for these applications, it makes more sense to signal the information of which tiles are included in a tile group in the PPS.

For real-time ultralow latency applications with sub-picture-interval delay, for which the operation is discussed above in subclause 1.2, because the partitioning of a picture into tile groups needs to be determined when encoding of the picture, it would make more sense to signal the information of which tiles are included in a tile group can be signalled in tile group headers.

It is therefore suggested to allow signalling of the information of which tiles are included in a tile group either in the PPS or in tile group headers.

# Proposed tile grouping scheme

The proposed tile grouping scheme is summarized as follows:

- Each tile group always covers a rectangular region of a picture.

- The information of which tiles are included in a tile group is signalled either in the PPS or in the tile group header, but never both.

  - If tile group information is signalled in the PPS, a loop over the tile groups is singalled, and for each tile group, the addresses of its first and last tiles are signalled. In this case, in the tile group header, only the address of the tile group's first tile is signalled.

  - Otherwise, (tile group information is not signalled in the PPS but in the tile group header), the addresses of each tile group's first and last tiles are signalled in tile group header.

The specification text, relative to the basis text in JVET-L0686-v2, is provided below, where the texts in the basis text that are not mentioned below apply as they are, and modified texts relative to the basis text are highlighted in yellow.

### Partitioning of pictures into tile groups and tiles

This subclause specifies how a picture is partitioned into tile groups and tiles.

Pictures are divided into tile groups and tiles. A tile group is a group of tiles that cover a rectangular region of a picture. A tile is a sequence of CTUs that cover a rectangular region of a picture.

For example, a picture may be divided into 24 tiles (6 tile columns and 4 tile rows) and 9 tile groups as shown in Figure 6-4.

Figure 6-4 – A picture with 18 by 12 luma CTUs that is partitioned into 24 tiles and 9 tile groups (informative)

When a picture is coded using three separate colour planes (separate_colour_plane_flag is equal to 1), a tile group contains only CTUs of one colour component being identified by the corresponding value of colour_plane_id, and each colour component array of a picture consists of tile groups having the same colour_plane_id value. Coded tile groups with different values of colour_plane_id within a picture may be interleaved with each other under the constraint that for each value of colour_plane_id, the coded tile group NAL units with that value of colour_plane_id shall be in the order of increasing CTU address in tile scan order for the first CTU of each coded tile group NAL unit.

NOTE 1 – When separate_colour_plane_flag is equal to 0, each CTU of a picture is contained in exactly one tile group. When separate_colour_plane_flag is equal to 1, each CTU of a colour component is contained in exactly one tile group (i.e., information for each CTU of a picture is present in exactly three tile groups and these three tile groups have different values of colour_plane_id).

### Picture parameter set RBSP syntax and semantics

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| single_tile_per_tile_group_flag | u(1) |
| tile_group_info_in_pps_flag | u(1) |
| if( tile_group_info_in_pps_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| pps_first_tile_id[ i ] | u(v) |
| if( !single_tile_per_tile_group_flag ) |  |
| pps_last_tile_id[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

...

single_tile_per_tile_group_flag equal to 1 specifies that each tile group that refers to the PPS contains exactly one tile. single_tile_per_tile_group_flag equal to 0 specifies that each tile group that refers to the PPS contains one or more tiles.

tile_group_info_present_flag equal to 1 specifies that tile group information of which tiles are included in a tile group is present in the PPS and not present in tile group headers referring to the PPS. tile_group_info_present_flag equal to 0 specifies that tile group information is not present in the PPS and present in tile group headers referring to the PPS.

num_tile_groups_in_pic_minus1 plus 1, when tile_group_info_in_pps_flag is equal to 1, specifies the number of tile groups in each picture referring to the PPS. The value of num_tile_groups_in_pic_minus1 shall be in the range of 0 to NumTilesInPic − 1, inclusive.

pps_first_tile_id[ i ] specifies the tile ID of the first tile of the i-th tile group. The length of pps_first_tile_id[ i ] is Ceil( Log2( NumTilesInPic ) ) bits. The value of pps_first_tile_id[ i ] shall not be equal to the value of pps_first_tile_id[ j ] for any i not equal to j.

pps_last_tile_id[ i ] specifies the tile ID of the tile of the last tile of the i-th tile group. The length of pps_last_tile_id[ i ] is Ceil( Log2( NumTilesInPic ) ) bits. When single_tile_per_tile_group_flag is equal to 1, the value of pps_last_tile_id[ i ] is inferred to be equal to pps_first_tile_id[ i ].

...

### Tile group header RBSP syntax and semantics

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| if( !single_tile_per_tile_group_flag ) |  |
| single_tile_in_tile_group_flag | u(1) |
| first_tile_id | u(v) |
| if( !single_tile_in_tile_group_flag && !tile_group_info_present_flag ) |  |
| last_tile_id | u(v) |
| } |  |
| ... |  |
| } |  |

...

tile_group_address specifies the tile address of the first tile in the tile group. The length of tile_group_address is Ceil( Log2 ( NumTilesInPic ) ) bits. The value of tile_group_address shall be in the range of 0 to NumTilesInPic − 1, inclusive, and the value of tile_group_address shall not be equal to the value of tile_group_address of any other coded tile group NAL unit of the same coded picture. When tile_group_address is not present it is inferred to be equal to 0.

num_tiles_in_tile_group_minus1 plus 1 specifies the number of tiles in the tile group. The value of num_tiles_in_tile_group_minus1 shall be in the range of 0 to NumTilesInPic − 1, inclusive. When not present, the value of num_tiles_in_tile_group_minus1 is inferred to be equal to 0.

single_tile_in_tile_group_flag equal to 1 specifies that there is only one tile in the tile group. single_tile_in_tile_group_flag equal to 0 specifies that there may be more than one tile in the tile group. When single_tile_per_tile_group_flag is equal to 1, the value of single_tile_in_tile_group_flag is inferred to be equal to 1.

first_tile_id specifies the tile ID of the first tile of the tile group. The length of first_tile_id is Ceil( Log2( NumTilesInPic ) ) bits. The value of first_tile_id of a tile group shall not be equal to the value of first_tile_id of any other tile group of the same picture. When not present, the value of first_tile_id is inferred to be equal to the tile ID of the first tile of the current picture.

last_tile_id specifies the tile ID of the last tile of the tile group. The length of last_tile_id is Ceil( Log2( NumTilesInPic ) ) bits.

When NumTilesInPic is equal to 1 or single_tile_in_tile_group_flag is equal to 1, the value of last_tile_id is inferred to be equal to first_tile_id. When tile_group_info_in_pps_flag is equal to 1, the value of last_tile_id is inferred to be equal to the value of pps_last_tile_id[ i ] where i is the value such that first_tile_id is equal to pps_first_tile_id[ i ].

NOTE – The first_tile_id is the tile ID of the tile located at the top-left corner of the tile group, and the last_tile_id is the tile ID of the tile located at the bottom-right corner of the tile group.

The variable NumTilesInTileGroup, which specifies the number of tiles in the tile group, and TgTileIdx[ i ], which  specifies the tile index of the i-th tile in the tile group, are derived as follows:

deltaTileIdx = last_tile_idx − first_tile_idx
numTileRows = ( deltaTileIdx / ( num_tile_columns_minus1 + 1 ) ) + 1
numTileColumns = ( deltaTileIdx % ( num_tile_columns_minus1 + 1 ) ) + 1
NumTilesInTileGroup = numTileRows * numTileColumns	(7-x)
tileIdx = first_tile_id
for( j = 0, tIdx = 0; j < numTileRows; j++, tileIdx  +=  num_tile_columns_minus1 + 1 )
	for( i = 0, currTileIdx = tileIdx; i < numTileColumns; i++, currTileIdx++, tIdx++ )
		TgTileIdx[ tIdx ] = currTileIdx

...

### General tile group data syntax

| tile_group_data( ) { | Descriptor |
| --- | --- |
| tileIdx = tile_group_address |  |
| for( i = 0; i <= num_tiles_in_tile_group_minus1; i++, tileIdx++ ) { |  |
| for( i = 0; i < NumTilesInTileGroup; i++ ) { |  |
| ctbAddrInTs = FirstCtbAddrTs[ tileIdxTgTileIdx[ i ] ] |  |
| for( j = 0; j < NumCtusInTile[ tileIdxTgTileIdx[ i ] ]; j++, ctbAddrInTs++ ) { |  |
| CtbAddrInRs = CtbAddrTsToRs[ ctbAddrInTs ] |  |
| coding_tree_unit( ) |  |
| } |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| if( i < num_tiles_in_tile_group_minus1NumTilesInTileGroup − 1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).