| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0132-v1 |
| --- | --- |

| Title: | AHG17: On header parameter set (HPS) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Hendry 10180 Telesis Court San Diego, CA, 92121, USA Jianle Chen 2330 Central Expy Santa Clara, CA 95050, USA | Tel: Email: | +1-908-803-3888 yekui.wang@huawei.com +1-858-375-8065 fnu.hendry@huawei.com +1-858-999-4243 jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a design of header parameter set (HPS) for carriage of ALF parameters only. When tile_group_alf_enabled_flag is equal to 1, an HPS is referenced by a tile group header, and the ALF parameters carried in the HPS is used by the tile group.

In the latest VVC WD, ALF parameters are directly signalled in the tile group header. It is asserted that this causes unnecessary redundant transmission of ALF parameters when there are multiple tile groups of a picture while at least two of the tile groups contain the same ALF parameters and when multiple pictures use the same ALF parameters. It asserted that using HPSs can avoid such unnecessary redundant transmission of ALF parameters.

It is asserted that allowing inclusion of other types of parameters in the HPS would require either of the following:

- A complicated partial HPS update mechanism, which was asserted to be a headache when designing the adaptation parameter set (APS) during HEVC development, and an agreeable technical solution was not found finally, as before that problem was solved, the headache was gone when ALF was removed from the HEVC draft text.

- Multiple HPS types, each carrying one type of parameters; this approach would require the tile group header to reference multiple HPS IDs or one HPS ID plus an HPS type.

# Introduction

In the latest VVC WD, ALF parameters are directly signalled in the tile group header. This causes unnecessary redundant transmission of ALF parameters when there are multiple tile groups of a picture while at least two of the tile groups contain the same ALF parameters and when multiple pictures use the same ALF parameters. Due to that the size of coded ALF parameters can be big, there is a need to avoid such unnecessary redundant transmission of ALF parameters.

This was also an issue during the development of HEVC years ago. There were HEVC proposals on the design of the adaptation parameter set (APS), triggered for carriage of ALF parameters, but most if not all of the proposals also targeted the carriage of other tile group header parameters. There was a sticky and complicated issue, partial update, to enable update only some part of the APS content, while other parts that remain unchanged are not resent and can be obtained by the decoder through referencing to an earlier-received APS. The partial update issue attracted quite some APS proposals at certain point back then. It was a headache. An agreeable technical solution was not finally found, as before that problem was solved, the headache went away when ALF was decided to be removed from the HEVC draft text, fortunately or unfortunately.

There can be different ways avoiding unnecessary redundant transmission of ALF parameters. Using a design of APS, or probably better named header parameter set (HPS), is one. Another approach is to use something like MPEG-4 Visual header extension code (HEC), to optionally signal the ALF parameters in the tile group headers, and when not signalled while ALF is enabled then the ALF parameters are inferred to be the same as another tile group, e.g., the first tile group of the same picture.

In the following, through a discussion of the use of VVC in Dynamic Adaptive Streaming over HTTP (DASH), we try to argue that ALF parameters should be carried in an HPS instead of being embedded in the tile group header, even when HEC-like mechanism mentioned above is in use.

A key advantage of using HPS for carriage of ALF parameters is that HPSs can be sent out-of-band, i.e., provided by external means. This enables an optimal delivery of VVC video in streaming applications as follows. In the ISO base media file format (ISOBMFF) environment, HPSs containing ALF parameters can be carried in a time-synchronized track, e.g., a parameter set track or a timed metadata track. This way, the tile groups each containing a group of complete tiles may be carried in their own file format tracks, e.g., named tile tracks, and the HPSs are carried in a time-synchronized track. Consequently, these tracks can each be carried in a DASH representation, and for decoding and rendering of a subset of the tile tracks, the DASH representations containing the subset of the tile tracks as well as the DASH representation containing the HPSs may be requested by the client, segments by segments. This way, both sources of ALF parameters redundancy can be avoided nicely.

Therefore, we believe that carrying ALF parameters in HPSs instead of embedded in tile group headers as in the latest VVC WD is a better design.

Allowing inclusion of other types of parameters in the HPS would require either of the following:

- An HPS partial update mechanism, which can be very complicated as shown by the HEVC history of APS development.

- Multiple HPS types, each carrying one type of parameters; this approach would require the tile group header to reference multiple HPS IDs or one HPS ID plus an HPS type. For some people, this may also be considered as one particular solution for HPS partial update.

Therefore, we suggest that HPSs only carry ALF parameters, nothing else.

# Proposal

We propose a design of HPS for carriage of ALF parameters only. When tile_group_alf_enabled_flag is equal to 1, an HPS is referenced by a tile group header, and the ALF parameters carried in the HPS is used by the tile group.

The specification text, relative to the basis text in JVET-L1001-v7 (with anticipation of "slice" being replaced by "tile group" and "slice_" being replaced by "tile_group_"), is provided below, where the texts in the basis text that are not mentioned below apply as they are, and modified texts relative to the basis text are highlighted in yellow, and some removals are shown strikethroughs in red fonts.

### NAL unit header semantics

...

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | NON_IRAP_NUT | Coded tile group of a non-IRAP picture tile_group_layer_rbsp( ) | VCL |
| 1 | IRAP_NUT | Coded tile group of an IRAP picture tile_group_layer_rbsp( ) | VCL |
| 2-15 | RSV_VCL_NUT | Reserved VCL NAL Units | VCL |
| 16 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 17 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 18 | HPS_NUT | Header parameter set header_parameter_set_rbsp( ) | non-VCL |
| 19 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 20 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 21, 22 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 23-26 | RSV_NVCL | Reserved | non-VCL |
| 27-31 | UNSPEC | Unspecified | non-VCL |

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit. The value of nuh_temporal_id_plus1 shall not be equal to 0.

The variable TemporalId is derived as follows:

TemporalId = nuh_temporal_id_plus1 − 1		(7-1)

When nal_unit_type is equal to IRAP_NUT, the coded tile group belongs to an IRAP picture, TemporalId shall be equal to 0.

The value of TemporalId shall be the same for all VCL NAL units of an access unit. The value of TemporalId of a coded picture or an access unit is the value of the TemporalId of the VCL NAL units of the coded picture or the access unit.

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	If nal_unit_type is equal to SPS_NUT, TemporalId shall be equal to 0 and the TemporalId of the access unit containing the NAL unit shall be equal to 0.

–	Otherwise, if nal_unit_type is equal to HPS_NUT, TemporalId shall be equal to that of the access unit containing the NAL unit.

–	Otherwise, if nal_unit_type is equal to EOS_NUT or EOB_NUT, TemporalId shall be equal to 0.

–	Otherwise, TemporalId shall be greater than or equal to the TemporalId of the access unit containing the NAL unit.

NOTE – When the NAL unit is a non-VCL NAL unit, the value of TemporalId is equal to the minimum value of the TemporalId values of all access units to which the non-VCL NAL unit applies. When nal_unit_type is equal to PPS_NUT, TemporalId may be greater than or equal to the TemporalId of the containing access unit, as all picture parameter sets (PPSs) may be included in the beginning of a bitstream, wherein the first coded picture has TemporalId equal to 0. When nal_unit_type is equal to PREFIX_SEI_NUT or SUFFIX_SEI_NUT, TemporalId may be greater than or equal to the TemporalId of the containing access unit, as an SEI NAL unit may contain information that applies to a bitstream subset that includes access units for which the TemporalId values are greater than the TemporalId of the access unit containing the SEI NAL unit.

nuh_reserved_zero_7bits shall be equal to '0000000'. Other values of nuh_reserved_zero_7bits may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore (i.e. remove from the bitstream and discard) NAL units with values of nuh_reserved_zero_7bits not equal to '0000000'.

### Header parameter set syntax and semantics

| header_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| header_parameter_set_id | ue(v) |
| hps_seq_parameter_set_id | ue(v) |
| alf_data( ) |  |
| } |  |

header_parameter_set_id identifies the HPS for reference by other syntax elements. The value of hdr_parameter_set_id shall be in the range of 0 to 63, inclusive.

hps_seq_parameter_set_id specifies the value of sps_seq_parameter_set_id for the active SPS. The value of hps_seq_parameter_set_id shall be in the range of 0 to 15, inclusive.

### General tile group header syntax and semantics

| tile_group_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( sps_alf_enabled_flag ) { |  |
| tile_group_alf_enabled_flag | u(1) |
| if( tile_group_alf_enabled_flag ) |  |
| alf_data( ) tile_group_hps_id | ue(v) |
| } |  |
| ... | u(1) |
| } |  |

...

tile_group_hps_id specifies the header_parameter_set_id of the HPS that the tile group refers to. The value of tile_group_hps_id shall be in the range of 0 to 63, inclusive. The TemporalId of the HPS NAL unit having header_parameter_set_id equal to tile_group_hps_id shall be less than or equal to the TemporalId of the coded tile group NAL unit.

...

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. does not have any current or pending patent rights relating to the technology proposed in this contribution (to the extent of the personal awareness of the authors).