| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0145-v1 |
| --- | --- |

| Title: | Non-CE2: Motion vector clipping in affine sub-block motion vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

In affine motion prediction, the motion field is derived for each sub-block based on the control point motion vectors. Even if the control point motion vectors are in the range specified by the motion field storage bit depth, the derived sub-block motion vectors may be outside of the range of values that can be represented within the motion field bit depth, resulting in arithmetic underflow and/or overflow. However, no motion vector clipping to the motion field storage bit depth (e.g., 16-bit in VTM) is considered in the VTM-3.0 software nor in the current VVC draft. To ensure the same behavior between different implementations and to avoid encoder crash in some cases, it is proposed to apply clipping to motion field storage bit depth to both sub-block motion vectors and control point motion vectors after the sub-block motion vector derivation in affine mode. Simulation results show that the proposed fix provides similar performance as VTM-3.0 in terms of BD-rate and timing.

# Problem statement

Affine motion prediction [1] has been adopted in VTM to capture complex kinds of motion that cannot be captured by the simple translational motion model. There are two models in affine prediction mode: a four-parameter model and a six-parameter model. For the sake of clarity, we focus on the four-parameter model in the following, but the same problems occur with the six-parameter model.

The four-parameter affine motion model is coded using two motion vectors as one pair at two control point positions defined at top-left corner and top-right corner of current CU. As shown in Figure 1 (a), the affine motion field of the block is described by two control point motion vectors (v0, v1). Based on the control point motion, the motion field (vx, vy) of one affine coded CU is described as:

|  | (1) |
| --- | --- |

where (v0x, v0y) is the motion vector of the top-left corner control point, and (v1x, v1y) is motion vector of the top-right corner control point, as shown in Figure 1 (a). Additionally, when one block is coded in the affine prediction mode, its motion field is derived based on 4x4 sub-block. Specifically, to derive the motion vector of each sub-block, the motion vector of the center sample of each sub-block (as shown in Figure 1 (b)) is calculated according to (1), and rounded to 1/16-pel accuracy.

Figure 1. four-parameter affine mode: (left) the affine model; (right) the sub-block-level motion derivation for affine blocks

Each derived sub-block motion vector is stored in the motion field for spatial and temporal motion vector prediction. However, when deriving a sub-block motion vector, the calculated motion vector may be outside of the range of values that can be represented considering the motion field storage bit depth (16-bit in VTM), resulting in arithmetic underflow and/or overflow problems, even when the control point motion vectors are in the range specified by the motion field storage bit depth. The problem is that there is no clipping to the motion field bit depth in the VTM-3.0 software nor in the current VVC draft [2].

In addition to the conformance issue (for example if a higher bit depth is used to store the motion field at the encoder side compared to the decoder side), very large motion vectors (i.e., beyond the 16-bit range) may create some problems when using predictive coding to code a motion vector difference of neighboring CU, e.g., to code a motion vector predicted from a neighboring affine CU. If the difference after predictive coding is too large, this may even cause the encoder to crash as the difference may not be coded using 32-bit represented in order-1 Exponential-Golomb codeword.

# Proposed solution

To ensure the same behavior between different implementations and to avoid encoder crash in some cases, it is proposed to add clipping to motion field storage bit depth to both sub-block motion vectors and control point motion vectors after the sub-block motion vector derivation in affine mode. The motion vector vxi,j, vyi,j of the i, j sub-block is clipped as

| vxi,j=Clip3(-32768, 32767,vxi,j) vyi,j=Clip3⁡(-32768, 32767,vyi,j) | (2) |
| --- | --- |

# Simulation results

The software is implemented based on VTM-3.0. The encoding was done under SDR video CTC [3].

Table 1. BD-rate for random access configuration.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 2. BD-rate for low delay B configuration.

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Class F | -0.06% | 0.19% | 0.03% | 100% | 100% |

Simulation results show similar performance to VTM-3.0 in terms of BD-rate and timing.

# Conclusion

In this contribution, it is proposed to add clipping to motion field storage bit depth after the sub-block motion vector derivation in affine mode. This can ensure the same behavior between different implementations and avoid encoder crash in some cases. Simulation results show that the proposed fix provides similar performance as VTM-3.0 in terms of BD-rate and timing.

# References

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3)”, JVET-L1002, Oct. 2018, Macao, CN.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3)”, JVET-L1001, Oct. 2018, Macao, CN.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Oct. 2018, Macao, CN.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).