| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0147-v6 |
| --- | --- |

| Title: | CE9: Results of DMVR related Tests CE9.2.1 and CE9.2.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sriram Sethuraman | Email: | Sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_____________________________

# Abstract

In this proposal, the results of sub-tests CE9.2.1 and 9.2.2 are summarized. In CE9.2.1b0, a simplified base with refinement disabled for coding units with luma sample counts larger than 1024, integer grid samples based refinement, SAD as the cost function, and use of refined MVs for only the MC of the current CU is considered. The progressive impact of the key elements of DMVR design such as forced partitioning of large CUs into sub-CUs that have a constraint on their maximum width and height, type of interpolation done for refinement, mean-removed SAD as cost function, and use of refined MVs for purposes other than MC for current CU are studied in the other sub-tests of CE9.2.1. The results indicate that DMVR can provide average BDRATE gains of up to -1.13%, -1.33%, -1.44% over VTM3 for the combination of (a) sub-CUs of maximum width and height of 16 luma samples, (b) bilinear interpolation for refinement, (c) block mean removed SAD as cost function, and (d) use of refined MVs for de-blocking, temporal MV prediction, and spatial MV prediction from top and top-left CTU neighbors. The average encoding and decoding time ratios have increased to up to 103% and 118% respectively due to performing motion compensation and BDOF at sub-CU granularity. The results also indicate that use of refined MVs for purposes other than MC provides more than one-third of the coding gain offered by DMVR.

In CE9.2.2, an alternative cost function that replaces block level mean removed SAD cost function with a row-level mean removed SAD cost function is studied at two different sub-PU level early exit thresholds. The results indicate that half of the BDRATE gain provided by block-level MR-SAD over SAD can be achieved using row-level mean removed SAD cost function. Further study may be required to study variants of such cost functions and suitable early exit thresholds that can provide a larger reduction in average decoding time increase at minimal impact to the coding gains. One result is also provided when BIO is disabled to show an average BDRATE gain of -1.51%, -1.58%, -1.59% over VTM3 with BIO disabled.

Complexity analysis for aspects such as pre-fetch cache accesses, internal memory requirements, and worst-case operation count are provided for the various choices. The analysis results indicate that sub-CUs of maximum width and height of 16 luma samples provide the least internal memory requirements while not increasing the pre-fetch cache accesses in the worst-case. The average luma BDRATE drop for SAD as a cost function when compared to MR-SAD as the cost function is seen to be only ~0.11%.

# Introduction

At the Macao meeting, JVET-L0173 presented results for DMVR tests under CE9 and one of the joint tests CE9.2.6 was selected as the anchor method for further study within CE. Though CE9.2.6 offered -1.89% average luma BDRATE gain and similar chroma BDRATE gains, it had high internal memory requirements or required a higher rate of read accesses from a high latency reference sample pre-fetch cache commonly used in hardware implementations due to the fact that refinement was performed at the CU level. Further, the adoption of Merge with MVD coding tool and bi-directional optical flow coding tools adoptions at the last meeting were expected to erode the BDRATE gains of DMVR. The base version software for DMVR is based on the joint test CE9.2.6 from the Macao meeting (JVET-L0173) with the restriction that DMVR is disabled for coding units with luma sample count larger than 1024.

Section 2 provides a detailed description of the test conditions of CE9.2.1 and CE9.2.2. CE9.2.1 studies the aspect of force partitioning a larger coding unit into multiple sub-partitioned-units (sub-PUs) each with width and height less than a specified value. CE9.2.1b through CE9.2.1e study the impact of various aspects within DMVR such as the choice of costs function, the type of interpolation performed for refinement, and the use of refined MVs only for MC of current CU vs. using it for spatial MV prediction, temporal MV prediction, and deblocking for sub-PUs with width and height constrained to 32. Given that VTM3.0 allows a VPDU-level pipeline, CE9.2.1f explores the additional gains possible with use of refined MVs from VPDUs processed two VPDUs before the current VPDU in the pipeline in decoding order. CE9.2.1g explores the impact of sub-PUs with width and height constrained to 16. In CE9.2.2, the block mean removed SAD is replaced by a previous row mean removed SAD cost function that enables DMVR to have a row-level pipeline between interpolation for refinement and cost function evaluations. Also, the impact of changing the sub-CU level early exit SAD threshold is also studied. In CE9.2.2f1, the impact of turning off use of refined MVs in CE9.2.2c is shown. In CE9.2.2f2, the impact of using SAD as the cost function is shown. CE9.2.2f also shows the performance of DMVR in the absence of BDOF against VTM3.0 minus BDOF.

The results section captures first the summary of all the results and then provides detailed class by class results against VTM3.0.

# Description of the tests

## CE9.2.1 sub-tests:

CE9.2.1(a) – Base software for DMVR:

- Avoiding increase in memory access relative to bi-prediction by performing interpolation with padded sample for sample positions outside (CU_w + 7)*(CU_h + 7) as first presented in JVET-K0275.

- Use of bilinear interpolation for refinement (proposed in JVET-K0041 and JVET-K0359) to avoid the increase in DCTIF based MC complexity due to refinement search range

- Restriction of DMVR to CU sizes to.

- Reducing the cost evaluation complexity by computing cost only on every other row (K0105)

- DMVR restricted from happening:

  - For CUs with height >= 8 and CU_w*CU_h >= 64 8x8 (as recommended by K0480 and K0105) in order to cap the worst-case compute operations

  - For CUs with luma sample count greater than 1024 samples in order to limit the internal memory requirements of DMVR (recommendation from Macao meeting notes)

  - When the MV distance from the MV corresponding to the signaled merge index to MVs of previous merge indices is less than a threshold  (first proposed in JVET-K0358)

  - When the SAD cost between L0 and L1 interpolated samples with zero delta-MV is less than a threshold (as proposed in JVET-K0342)

CE9.2.1b0 – a simplified version of base software that uses nearest integer grid of samples in L0 and L1 (as per K0275), SAD as the cost function, and restricts use of refined MV only for MC of current CU

CE9.2.1b – In CE9.2.1b0, remove the restriction of disabling DMVR for CUs with luma sample count greater than 1024 samples by force partitioning these CUs in to sub-CUs with maximum width of 32 and maximum height of 32.

CE9.2.1c – In CE9.2.1b, replace SAD cost function with MR-SAD cost function

CE9.2.1d – In CE9.2.1c, replace use of integer grid samples for refinement with use of bilinear interpolation at 1/16th of a pixel precision for refinement

CE9.2.1e – In CE9.2.1d, enable use of refined MVs for performing deblocking, for temporal MV prediction in future access units, and for spatial MV prediction (in merge, AMVP, and affine-MVP) from top-left and top-CTU neighbors.

CE9.2.1f – In CE9.2.1e, enable use of refined MVs also within a CTU from neighbor CUs in VPDUs that were processed 2 VPDUs earlier in scan order.

CE9.2.1f1 – In CE9.2.1f, restrict the motion compensation to not require samples beyond (64+7)*(64+7) in CUs of size greater than 64x64.

CE9.2.1g – In CE9.2.1e, restrict max sub-CU width and height to 16 instead of 32.

## CE9.2.2 sub-tests

MR-SAD requires the entire block of interpolated samples for the two references for a given sub-PU to be buffered before the cost calculations can begin. For example, with SAD as the cost function, only 5 lines of interpolated samples for each reference will need to be maintained for a vertical +/-2 refinement range as compared to (sub_PU_h + 4) lines for each reference required by MR-SAD. In order to get the benefit of mean-removed SAD while having the buffer requirement of SAD, the block mean removed SAD is replaced by a row-mean removed SAD. Specifically, the mean of the previous row of samples within the block for the current refinement position is removed from all samples of the current row at the corresponding refinement positions in L0 and L1 (based on symmetric bilateral matching). This method is hence referred to as previous-row mean-removed SAD (PR-MR-SAD). Both SAD and PR-MR-SAD allow interpolation for refinement and cost calculations for all search positions to happen in a row-level pipeline. In addition, with the cost function being evaluated only on alternate rows of the sub-PU, only odd vertical or even vertical refinement positions require cost evaluation update at the row level.

CE9.2.2(c) studies the impact of replacing MR-SAD with PR-MR-SAD with an early exit per-pixel SAD threshold of 64.

CE9.2.2(d) studies the same with an early exit per-pixel SAD threshold of 192.

CE9.2.2(f) presents the results of DMVR with BDOF.

CE9.2.2(f1) presents the results of CE9.2.2(c) with refined MV usage restricted to MC of current CU.

CE9.2.2.(f2) present the results of replacing PR-MR-SAD in CE9.2.2c by simple SAD as the cost function.

# Results

Following table provides a summary of all the tests with the averages across classes A, B, and C.

| S.No | Test case | Description | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CE9.2.1a | CE9.2.6 with DMVR disabled for w*h > 1024 | -0.71% | -0.81% | -0.87% | 110.39% | 117.81% |
| 2 | CE9.2.1.b0 | Integer DMVR, SAD, and refined MV used only for MC in Base | -0.30% | -0.39% | -0.44% | 101.40% | 102.58% |
| 3 | CE9.2.1.b | 2 + Enable 32x32 forced split + enabled DMVR for w*h > 1024 also | -0.58% | -0.86% | -0.86% | 101.76% | 106.42% |
| 4 | CE9.2.1.c | 3 + Replace SAD with MR-SAD | -0.66% | -0.88% | -0.92% | 101.73% | 108.31% |
| 5 | CE9.2.1.d | 4 + Replace Integer DMVR with Bilinear MC based | -0.74% | -1.02% | -1.07% | 102.28% | 111.22% |
| 6 | CE9.2.1.e | 5 + use of refined MV for deblocking, TMVP and for spatial MV prediction from top/top-left CTU neighbors | -1.12% | -1.25% | -1.30% | 103.01% | 113.33% |
| 7 | CE9.2.1.f | 6 + use of refined MV also within CTU row from VPDUs that were processed 2 VPDUs behind current VPDU | -1.25% | -1.33% | -1.43% | 102.87% | 113.30% |
| 8 | CE9.2.1.f1 | In 7, MC restricted to use samples within (64+7)*(64+7) for VPDU of CUs larger than 64x64 | -1.25% | -1.33% | -1.42% | 102.75% | 113.40% |
| 9 | CE9.2.1g | In 6, sub-PU size changed from 32x32 to 16x16 | -1.13% | -1.33% | -1.44% | 103.24% | 118.11% |
| 10 | CE9.2.2c | In 9.2.1g, MR-SAD replaced by PR-MR-SAD | -1.07% | -1.27% | -1.36% | 103.20% | 117.69% |
| 11 | CE9.2.2d | In 10, increase early exit threshold for w*h > 1024 to assess DecT impact | -1.04% | -1.27% | -1.36% | 103.20% | 117.27% |
| 12 | CE9.2.2f | Results for 9.2.2(c) when BIO is disabled in both VTM3 and with DMVR + DMVR is enabled for ATMVP and MMVD | -1.51% | -1.58% | -1.59% | 115.98% | 125.70% |
| 13 | CE9.2.2f1 | Disabling refined MV usage for anything other than MC in 9.2.2c | -0.72% | -1.05% | -1.11% | 102.30% | 115.33% |
| 14 | CE9.2.2f2 | Replacing PR-MR-SAD with SAD in 9.2.2c | -1.02% | -1.29% | -1.35% | 102.94% | 115.27% |

Table 1: Summary of CE9.2.1 and CE9.2.2 test results

Following are the key observations from this summary table:

- CE9.2.1g provides a BDRATE gain of 1.13% over VTM3.0.

- Use of refined MV for purposes other than MC of current CU (e.g. deblocking, temporal MV prediction, and spatial merge/AMVP/affine-MVP from top-left and top CTU boundary neighbors shows a BDRATE gain of 0.38% (CE9.2.1d vs. CE9.2.1e) and 0.35% (CE9.2.2c vs. CE9.2.2f1) and hence forms a significant part of the coding gains offered by DMVR.

- Use of refined MV in a VPDU pipeline offers an additional 0.12% BDRATE gain. This indicates that use of refined MVs in cases5 with max-CTU of 64x64 will be close 0.5%.

- MR-SAD cost function offers a 0.11% BDRATE gain over SAD as the cost function. PR-MR-SAD shows in between gains of around 0.05% over SAD.

- Bilinear interpolation with 1/16th pel accuracy provides close to 0.1% BDRATE gain over integer DMVR

- Performing sub-PU level DMVR increases the overall average decoding time as MC and BDOF are then performed at sub-PU level. 32x32 sub-PU size shows close to 2% increase in decoding time while 16x16 sub-PU shows a 5% increase in decoding time over 32x32 sub-PU size. However, sub-PU size of 16x16 incurs the smallest internal memory footprint of (sPUw + 5)x(sPUh + 4 + 7) for each reference to pass on the integer grid samples accessed from a pre-fetch cache to the final MC in order to avoid accessing these again. In addition, cost functions such as PR-MR-SAD and SAD require only 2x5 line buffers of 20 samples each to hold the interpolated samples.

- Performing DMVR for MMVD increases the encoding time significantly as up to 32 DMVRs are performed during the SATD stage. Performing DMVR for SbTMVP increases the decoding time significantly as an 8x8 sub-PU size is used in these cases. For these reasons, barring 9.2.1a and 9.2.2f, DMVR for MMVD and SbTMVP are disabled.

Table 2: Test results for CE9.2.1a

Table 3: Test results for CE9.2.1b0

Table 4: Test results of CE9.2.1b

Table 5: Test results of CE9.2.1c

Table 6: Test results of CE9.2.1d

Table 7: Test results of CE9.2.1e

Table 8: Test results of CE9.2.1f

Table 9: Test results of CE9.2.1f1

Table 10: Test results of CE9.2.1g

Table 11: Test results of CE9.2.2c

Table 12: Test results of CE9.2.2d

Table 13: Test results of CE9.2.2f

Table 14: Test results of CE9.2.2f1

Table 15: Test results of CE9.2.2f2

# Supplementary Results

During the CE9 discussion it was suggested that the test CE9.2.1g with refined motion vectors only applied in temporal MV prediction is a promising candidate. In this section the supplementary results are provided based on to suggestions from the discussion. In the results, CE9.2.1g test is used with further modifications.

## CE9.2.1g Variant 1

CE9.2.1g with refined MVs used only for motion compensation and temporal MV prediction. The spatial MV prediction and deblocking filtering use non-refined motion vectors.

Table 16: Test results for CE9.2.1g with refined MVs used only for MC and temporal MV prediction

The crosscheck report for Variant 1 is presented in document JVET-M0887.

## CE9.2.1g Variant 2

## CE9.2.1g with refined MVs used for motion compensation, spatial MV prediction from top and top-left CTU and temporal MV prediction. Deblocking filtering use non-refined motion vectors.

Table 17: Test results for CE9.2.1g with refined MVs not used for deblocking

.

## CE9.2.1g Variant 3

CE9.2.1g with refined MVs used only for motion compensation and temporal MV prediction. The spatial MV prediction and deblocking filtering use non-refined motion vectors. Furthermore MRSAD in CE9.2.1g is replaced with SAD.

Table 18: Test results for CE9.2.1g with refined MVs used only for MC and temporal MV prediction. MRSAD replaced with SAD.

The crosscheck report for Variant 3 is presented in document JVET-M0887.

## CE9.2.1g Variant 4

During the Track-B revisit of this proposal, variant-3 was chosen for adoption with the additional restriction that the MVD based early exit check be removed due to hardware issues that may be there with it. Following are the results after removing this aspect from variant-3.

Table 19: Test results for CE9.2.1g with refined MVs used only for MC and temporal MV prediction, MRSAD replaced with SAD, and MVD based early exit check disabled.

The cross-checked results for this will be available in JVET-M0903.

# Conclusion

The CE9.2.1 test results provide a clear break-down of the BDRATE impact and decoder complexity contributions from the different aspects of DMVR such as the cost function, the type of refinement used for interpolation, the type of force splitting done for larger CUs, and the purposes for which refined MVs are used. The best result is achieved for CE9.2.1g which offers a -1.13% average luma BDRATE gain over VTM3.0 with similar gains for chroma. The use of refined MVs for purposes other than MC of current CU provides the biggest coding gain among all aspects. Sub-PU size of 16x16 offers the lowest memory footprint. It is recommended that 9.2.2c be considered for adoption of DMVR into the next VTM.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).