| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0153 |
| --- | --- |

| Title: | AHG17: On leading picture for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Tel: Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes the following NAL unit types and restrictions on leading picture types:

Proposal 1: One NAL unit type is assigned to indicate a leading picture type.

Proposal 2: RADL is used for IDR only. (An RADL picture is not associated with a CRA picture.)

Proposal 3: RASL is used for CRA only.

Proposal 4: (if Proposals 2 & 3 are agreed) The leading picture type (RASL or RADL) is identified by the associated IRAP picture type.

# Introduction

In JVET-L0064 discussion, it was agreed that a functionality to support an open GOP structure and that some basic picture type information indicating open/closed prediction structuring should be in the NAL unit header, not in an SEI message. Through several meetings, experts inclined to agree to have the reduced number of NAL unit type for VVC. In JVET-M0101, multiple companies propose some discussion points on random access point and leading picture indication.

In this contribution, agreeing the suggestions in JVET-M0101 in principle, technical details on leading picture indication methods are proposed. The proposals regarding random access point pictures are described in JVET-M0152.

# Discussion on Proposal 1

Proposal 1: One NAL unit type is assigned to indicate a leading picture type.

Discussion: It is not expensive to consume one NAL unit type to indicate that the picture in the current NAL unit is a leading picture. By easily checking the leading picture from the field nal_unit_type in NUH, a splicer or middle box can discard unnecessary leading pictures in system level. If a CRA picture is handled as a BLA picture, the associated RASL pictures are needed to discarded in system level, before the bistream is fed into a decoder.

# Discussion on Proposal 2

Proposal 2: RADL is used for IDR only. (An RADL picture is not associated with a CRA picture.)

Discussion: The CRA enables an open GOP structure with RASL pictures, while the IDR provides a closed GOP structure only, with RADL pictures. Hence, RASL pictures are needed to support a prediction across a GOP boundary, while RADL pictures are not needed for a closed GOP structure. In CTC or other implementations, RADL pictures are unlikely used for an open GOP structure with a CRA picture. It is reasonable to restrict the uncommon use case that a RADL picture is associated with a CRA picture, for simplification of VVC text.

# Discussion on Proposal 3

Proposal 3: RASL is used for CRA only.

Discussion: RASL pictures are used for support of a prediction across a GOP boundary. RASL pictures can/shall not be used with IDR for a closed GOP structure. It is reasonable to associate RASL with CRA only.

# Discussion on Proposal 4

Proposal 4: (if Proposals 2 & 3 are agreed) The leading picture type (RASL or RADL) is identified by the associated IRAP picture type.

Discussion: If Proposals 2 and 3 are agreed, one NAL unit type for indication of the leading picture (as Proposal 1) is enough to distinguish RASL pictures from RADL pictures. Any leading picture associated with IDR is always RADL, while any leading picture associated with CRA is always RASL. In conclusion, an additional indication of the leading picture types (RASL or RADL) is not necessary.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).