| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0155 |
| --- | --- |

| Title: | AHG12: On tile group identification for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The contribution proposes a syntax element (nuh_tile_group_id) in the NAL unit header, which has the same value with the syntax element (tile_group_id) signalled in tile group header. The proposed syntax element (nuh_tile_group_id) is an identifier for each tile group and can be used for the tile group identification at the system level. It is asserted that a network element or a video editor can easily identify a tile group carried by NAL units and remove the corresponding NAL units, therebyextracting a sub-bitstream including the NAL units belonging to the target tile group(s), by parsing and interpreting NAL unit headers only.

# Problem statement

The identification of a tile group for each NAL unit is essential not only for decoding the corresponding tile group bitstream, but also for interpreting and distinguishing which VCL NAL units belong to a specific tile group at the system level. A media-aware network element (MANE) or a video editor can identify a tile group carried by NAL units and can remove the corresponding NAL units or extract a sub-bitstream including the NAL units belonging to the target tile group(s). In HEVC, the identification of a tile requires following the parameter set activation and parsing syntax elements coded as variable length codewords. For VVC, if the tile identification information is present in a very high-level syntax structure like the NAL unit header and coded as a fixed length codeword, then a MANE or any system entity can easily identify NAL units corresponding to a specific tile group. Then, the system behaviour with the tile identification can be significantly simplified.

# Proposal

The contribution proposes a syntax element (nuh_tile_group_id) in NAL unit header, which has the same value with the syntax element (tile_group_id) signalled in tile group header. The proposed syntax element (tile_group_id) signalled in tile group header is an identifier for each tile group. The value of tile_group_id shall not be equal to the value of tile_group_id of any other coded tile group NAL unit of the same coded picture. The reason why the tile group identifer is duplicatedly signaled in a tile group header is that the tile group id needs to be mapped to a tile group address (tile_group_address) signalled in a tile group header. It is not desirable to directly signal the tile group address in a NAL unit header, because tile_group_address is coded as a variable length codeword and needs many bits depending on the number of tiles in a picture. A tile_group_id is coded as a fixed length codeword and one-to-one matched to a tile_group_address, while tile_group_id may or may not indicate the tile address of the first tile in the tile group.

#### NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(32) |
| nuh_reserved_zero_7bits | u(7) |
| nuh_tile_group_id | u(8) |
| } |  |

Or, in accordance with the NAL unit header design in JVET-M0520:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nuh_unit_class | u(4) |
| nuh_temporal_id_plus1 | u(3) |
| . . . |  |
| if( nuh_unit_class == 9 ) { /* Complex */ | u(1) |
| nuh_tile_group_id_present_flag | u(1) |
| if( nuh_tile_group_id_present_flag ) { |  |
| nuh_tile_group_id | u(8) |
| } |  |
| } |  |
| byte_align() |  |
| } |  |

nuh_tile_group_id specifies the tile group id of the NAL unit. The value of nuh_tile_group_id shall be equal to the value of tile_group_id in tile_group_header ().

### Tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_address | u(v) |
| tile_group_id | u(8) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| tile_group_type | ue(v) |
| if ( tile_group_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_sbtmvp_enabled_flag ) { |  |
| sbtmvp_size_override_flag | u(1) |
| if( sbtmvp_size_override_flag ) |  |
| log2_sbtmvp_active_size_minus2 | u(3) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( num_tiles_in_tile_group_minus1 > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_tiles_in_tile_group_minus1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

tile_group_id specifies the tile group id of the tile group. The value of tile_group_id shall be in the range of 0 to 255, inclusive, and the value of tile_group_id shall not be equal to the value of tile_group_id of any other coded tile group NAL unit of the same coded picture. When tile_group_id is not present, the value of tile_group_id is inferred to be equal to 0.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).