| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0159-v1 |
| --- | --- |

| Title: | AHG9: Convolutional neural network loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ling Hsiao, Ching-Yeh Chen, Tzu-Der Chuang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yuling.hsiao, chingyeh.chen, peter.chuang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document presents two modifications of convolution neural network loop filter (CNNLF) introduced in JVET-K0222. The first modification is to reduce two 4-layer networks separate for luma and chroma to only one 3-layer network shared by luma and chroma. The second modification is to conditionally signal the CNNLF parameters in the I-slice header. Compared with VTM3.0, the proposed CNNLF reportedly achieves -1.23% (Y), -10.11% (Cb), and -9.96% (Cr) BD-rates with 42% decoding time increase in random access (RA) condition without using any GPUs. After shifting coding gain from chroma to luma by increasing chroma quantization parameter (QP) offsets, the BD-rates are -2.47% (Y), +2.90% (Cb), and +3.01% (Cr). It is shown that Class C (small resolution, 832x480) has no coding gain because of the relatively “expensive” side information bits of CNNLF parameters while Class A (large resolution, 3840x2160) has higher coding gains (-3.7% luma BD-rate and +3% chroma BD-rate). Further research on CNNLF to reduce complexity and enhance training for improving coding efficiency is suggested.

# Introduction

In JVET-K0222 [1], convolution neural network (CNN) is applied as one in-loop filter stage and called convolutional neural network loop filter (CNNLF). The inputs of CNNLF are adaptive loop filter (ALF) output samples and are called reconstructed samples (Reco) in this contribution. The outputs of CNNLF are called the restored reconstructed samples. A 4-layer neural network for luma and another 4-layer neural network for chroma are designed, as shown in Figure 1 (a) and (b), respectively, where the numbers in a layer, MxMxNxP, are used to represent HorizontalSpatialFilterSize x VerticalSpatialFilterSize x NumberOfInputChannels x NumberOfOutputChannels. The operation in each layer is filtering with a rectified linear unit (ReLu) right after each filter, except the last layer. For the luma neural network, the 6 input channels are 4 x Y-Reco, Cb-Reco, and Cr-Reco. The data of 4 x Y-Reco are generated by 2x2 subsampling a luma block into 4 different subblocks, and each subblock is treated as one input channel. For example, if one to-be-processed luma block is Width x Height, then 2x2 subsampling is applied to generate 4 subblocks of size Width/2 x Height/2, in Packing Luma. After finishing the processes of 4 layers, Unpacking Luma generates a filtered block with size equal to Width x Height by integrating the 4 subblocks into one block and adding the filtered results on top of Y-Reco. As for chroma, Cb and Cr are processed simultaneously. The 12 input channels are 4 x AvgY-Reco, 4 x Cb-Reco, and 4 x Cr-Reco, where AvgY-Reco is generated by averaging 4 luma samples first if the YUV format is 420. Unpacking Chroma generates the two filtered chroma blocks, after finishing the processes of 4 layers.

Figure 1.  The network design of CNNLF in JVET-K0222; (a) for luma; (b) for chroma.

In JVET-K0222, the parameters of CNNLF are signalled in the I-slice header of each random access segment (RAS, roughly 1 sec in the conducted tests), and the parameters are trained by the pictures in this RAS with temporal ID equal to 0 or 1; and multi-level on/off control is applied at slice level and coding tree block (CTB) level for each color component. At the slice header, one syntax element is signalled to indicate the mode selection among 3 supported modes, {SliceAllOff, SliceAllOn, CTBOnOff}. If SliceAllOff is selected, the CNNLF is disabled for the current component in the current slice. If SliceAllOn is selected, the CNNLF is applied for all samples of the current component in the current slice. If CTBOnOff is selected, whether the CNNLF is applied or not depends on the syntax elements signalled at CTB level. At CTB level, there are also three on/off modes, {CTBAllOff, CTBAllOn, BlockOnOff}. If CTBAllOff is selected, the CNNLF is disabled for the current component in the current CTB. If CTBAllOn is selected, the CNNLF is applied for all samples in the current CTB for the current component. If BlockOnOff is selected, whether the CNNLF is applied or not depends on the on/off flags signalled at 32x32 block level.

# Proposed methods

## One shared network for luma and chroma

To reduce CNNLF parameters signalling overhead and decoding time, it is proposed to apply one network shared by luma and chroma, as shown in Figure 2. In our design, each coding tree unit (CTU) is independently processed by CNNLF. In Packing, every 2x2-sample block of luma are treated as one sample of four channels, so there are six channels in total (Y0, Y1, Y2, Y3, Cb, Cr). After Packing, the size of the reconstructed samples becomes 64x64x6. After 3 layers convolution, the size of the results is still 64x64x6. In Unpacking, the first 4 channels ensemble the luma coding tree block (CTB), and last 2 channels are for the two chroma CTBs. Finally, the CNNLF output samples are calculated by summing ALF output samples and the Unpacking output samples.

Figure 2.  The proposed network design of CNNLF in this contribution.

## CNNLF parameters conditional signalling

The parameters of CNNLF are always signalled in the I-slice header of each RAS in JVET-K0222. To improve coding efficiency, when the mode selection of the intra slice is SliceAllOff, it proposed to not signal CNNLF parameters, and the remaining slices in this RAS are all set as SliceAllOff.

# Training and inference stage analysis

## Training stage

To reduce the computation overhead of generating the required data for CNNLF training process, only pictures with temporal ID equal to 0 or 1 are used to derive CNNLF parameters in the training process. That is, only these pictures are required to be encoded twice. The first round is to generate the required data for CNNLF training process and derive the CNNLF parameters. The second round is to generate the final bitstream by enabling CNNLF with the derived parameters. Table 1 shows the training stage information.

Table 1. Training stage settings

| Learning rate | 0.1 (halved per 5000 epochs) |
| --- | --- |
| Optimizer | ADAM |
| Batch size | All patches |
| Epoch | 25000 |
| Loss function | L2 (Y:U:V=8:1:1) |
| Training GPU | GTX 1080ti |
| Training time |  |
| └ Class A | 5.744 hours per network (one RAS) |
| └ Class B | 3.333 hours per network (one RAS) |
| └ Class C | 0.593 hours per network (one RAS) |
| Framework | TensorFlow 1.8.0 |
| Patch size | 128x128, grid cropped |
| Dataset | CTC sequences |
| QP set for generating training dataset | 22, 27, 32, 37 |

## Inference stage

The encoder and decoder have no dependency on any other frameworks. Table 2 shows the inference stage information.

Table 2. Inference stage information

| Total conv. layers | 3 |
| --- | --- |
| Total FC layers | 0 |
| Framework | NA |
| Param. num | 1506 |
| └ Weight param. num | 1440 (6-bit) |
| └ Scale, shift, bias param. num | 66 (32-bit) |
| GFLOPs | 24.983 (H=3840, W=2160) |
| Mem.P (MB) | 0.001282 (1344 Bytes) |
| Mem.T (MB) | 0.344 (H=128, W=128, CTU based) |
| Test GPU | NA |
| Test CPU | Xeon E5-2643 v3 |
| OS | Red Hat 6.7 |
| Fix-point operation | Yes |

# Experiment results

The proposed methods are implemented on top of VTM3.0. The common test conditions [2] are used for evaluation. Table 3 summarizes the results. Results of shifting coding gain from chroma to luma by increasing chroma QP offsets are shown in Table 4.

Table 3.  Results of the proposed CNNLF

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.93% | -4.92% | -6.91% | 100% | 177% |
| Class A2 | -1.73% | -13.16% | -10.95% | 100% | 131% |
| Class B | -1.63% | -15.41% | -14.36% | 100% | 151% |
| Class C | 0.17% | -5.09% | -6.01% | 100% | 118% |
| Class E |  |  |  |  |  |
| Overall | -1.23% | -10.11% | -9.96% | 100% | 142% |

Table 4.  Results of the proposed CNNLF with coding gain shift from chroma to luma

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.58% | 5.93% | 2.40% | 100% | 174% |
| Class A2 | -3.82% | 1.00% | 2.61% | 99% | 128% |
| Class B | -2.41% | -1.72% | 0.31% | 99% | 148% |
| Class C | -0.72% | 7.84% | 7.13% | 100% | 118% |
| Class E |  |  |  |  |  |
| Overall | -2.47% | 2.90% | 3.01% | 99% | 140% |

# Conclusion

This document presents a CNNLF that achieves -1.23% (Y), -10.11% (Cb), and -9.96% (Cr) BD-rates with 142% decoding time in RA condition on VTM3.0. Further research on CNNLF to reduce complexity and enhance training for improving coding efficiency is suggested.

# References

[1] Y.-L. Hsiao, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “AHG9: Convolution neural network loop filter,” Document of Joint Video Experts Team, JVET-K0222.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).