| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0165-v1 |
| --- | --- |

| Title: | CE2.5.1: Simplification of SbTMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chia Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chunchia.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the test result of Core Experiment 2.5.1 (CE2.5.1). In hardware decoder designs, subblock-based temporal motion vector prediction (SbTMVP) merge mode is challenging for small coding units (CUs) because a small CU has to be processed within a limited number of clock cycles to meet the real-time requirement. It is proposed to disable the SbTMVP merge mode for small CUs. Moreover, in this CE, for small CUs, the affine merge mode is also disabled. The luma BD-rates are reportedly 0.00% and 0.00% for RA and LB, respectively

# Problem statement

SbTMVP is more complex than traditional spatial and temporal merge candidates. Sequential steps are:

- Find an initial vector according to the spatial merge candidates

- Constrain and modify the initial vector

- Find the corresponding block pointed by the modified initial vector

- Check the availability of the center motion vector (MV) of the corresponding block

- Find MVs of all collocated subblocks; if the MV of a subblock is not available, use the center MV

- Scale collocated subblock MVs to reference index 0

If the center MV is unavailable in Step 4, the process is terminated and SbTMVP is not allowed. Table 1 shows the clock cycle budgets for small CUs to support real-time decoding. In hardware designs, the clock cycle budgets are already very tight to support real-time 8K 60fps or 120fps decoding for traditional merge modes. It is even more challenging for SbTMVP. Moreover, to achieve higher coding gain, merge mode is getting more and more complex, and therefore simplifying SbTMVP is highly suggested.

Table 1. Clock cycle budgets for one CU

| Assuming Clock @ 1GHz | 8K 60fps decoder | 8K 120fps decoder |
| --- | --- | --- |
| 4x4 CU | 8 | 4 |
| 4x8 or 8x4 CU | 16 | 8 |
| 8x8 CU | 32 | 16 |
| Assuming Clock @ 500MHz | 8K 60fps decoder | 8K 120fps decoder |
| 4x4 CU | 4 | 2 |
| 4x8 or 8x4 CU | 8 | 4 |
| 8x8 CU | 16 | 8 |

Since currently SbTMVP is integrated with affine merge list and unified as a subblock-based merge list in VTM3.0, SbTMVP merge mode and affine merge mode will be disabled for width or height equal to 4 now. Therefore, 4x4, 4x8, and 8x4 CUs currently already disables SbTMVP. However, for 8x8 CU, SbTMVP can be activated, therefore the cycle budget problem still exists for 8x8 CU. Moreover, the subblock of SbTVMP is 8x8, therefore, the SbTMVP merge mode will become a non-subblock merge mode for 8x8 CU.

# Proposed method

It is proposed to disable the SbTMVP merge mode for 8x8 CU. Moreover, the affine merge mode is also disabled for 8x8 CU.

# Simulation results

The proposed method is integrated on VTM3.0. All tests were conducted under the common test condition [1]. Table 2 shows the results.

Table 2.  Test results of CE 2.5.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | -0.03% | 101% | 99% |
| Class A2 | -0.02% | -0.01% | 0.07% | 100% | 100% |
| Class B | -0.01% | 0.00% | 0.02% | 101% | 101% |
| Class C | 0.01% | 0.00% | 0.15% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.05% | 100% | 100% |
| Class D | 0.05% | 0.05% | 0.01% | 100% | 101% |
| Class F (optional) | 0.01% | -0.02% | -0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.13% | 0.23% | 101% | 103% |
| Class C | -0.01% | 0.12% | -0.06% | 100% | 100% |
| Class E | 0.02% | -0.11% | 0.04% | 100% | 100% |
| Overall | 0.00% | -0.04% | 0.08% | 100% | 101% |
| Class D | 0.04% | 0.07% | 0.56% | 99% | 102% |
| Class F (optional) | 0.05% | -0.15% | 0.06% | 101% | 100% |

# Conclusions

This contribution proposes to disable SbTMVP and affine merge modes for 8x8 CU to make real-time hardware designs less challenging. It is shown that the coding loss is 0.00% while the problem is alleviated.

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).