| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0166-v2 |
| --- | --- |

| Title: | CE2-related: Simplification of constructed affine merging candidate derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a simplification method for constructed affine merging candidate derivation. The reference index checks are removed, which reduces the worst case number of reference index checks from 60 to 48. The translational model checks are removed, which saves 40 motion vector (MV) similarity checks. It is reported that the proposed method results in 0.01% and 0.05% luma BD-rates with trivial runtime changes for VTM3.0-RA and VTM3.0-LB, respectively.

# Introduction

In VTM3.0, when constructing the subblock-based merging candidate list, if the subblock-based merging candidate list is not full after adding the inherited affine candidates, constructed affine candidates are derived. The derivation for constructed affine merging candidate includes two stages. The first stage is to derive the four CPMVs at the four corners of the current coding unit (CU) as shown in Figure 1. The MV(s) of CP0 is the first available MV in block B2, B3, or A2. The MV(s) of CP1 is the first available MV in block B1 or B0. The MV(s) of CP2 is the first available MV in block A1 or A0. The MV(s) of CP3 is the temporal MV in block RB in the collocated picture. It requires eight comparison in the worst case to check whether the MVs in each position is available. The second stage is to check the validity of the control point (CP) sets in a CP set list. The CP set list is { {CP0, CP1, CP2}, {CP0, CP1, CP3}, {CP0, CP2, CP3}, {CP1, CP2, CP3}, {CP0, CP1}, and {CP0, CP2} }. For each CP set, the reference index in each reference list is checked. For each reference list, if the reference indices of the CPMVs of the CP set are not the same or one of the CPMVs’ reference index is not larger than or equal to 0, the CPMVs in that reference list are set to be unavailable. The reference index check of the CPMVs of the CP set for list 0 (L0) and list 1 (L1) requires at most 52 reference index comparisons. To prevent translation motion for the entire CU, for each CP set, the similarity of the CPMVs in each reference list is also checked. For each reference list, if all of the CPMVs of the CP set are the same, all of the CPMVs of the CP set are set to be unavailable. This CPMV check of the CPMVs of the CP set for L0 and L1 requires at most 40 MV comparisons for both x and y directions. After the above two types of checks, for 6-parameter affine model, the MVs at CP0, CP1, and CP2 derived by a CPMV set are inserted to the subblock-based merging candidate list as one candidate without any pruning; for 4-parameter affine model, the MVs at CP0 and CP1 derived by a CPMV set are inserted to the subblock-based merging candidate list as one candidate without any pruning. Therefore, for CP sets {CP0, CP1, CP3}, {CP0, CP2, CP3}, and {CP1, CP2, CP3}, CP2, CP1, and CP0 need to be derived, respectively. The derivation includes eight add operations (list 0 and list 1). The example of CP set {CP0, CP1, CP3} is shown below:

CP2.hor = CP3.hor + CP0.hor - CP1.hor

CP2.ver = CP3.ver + CP0.ver - CP1.ver

For CP set {CP0, CP2}, CP1 need to be derived. The derivation includes 16 add operations, 8 shift operations and 4 rounding operations (list 0 and list 1). The derivation is shown below:

CP1.hor = (CP0.hor << shift) + ((CP2.ver - CP0.ver) << (shift + Log2[cuW] - Log2[cuH]))

CP1.ver = (CP0.ver << shift) - ((CP2hor - CP0.hor) << (shift + Log2[cuW] - Log2[cuH]))

roundAffineMv(CP1.hor, CP1.ver, shift ),

where shift is equal to the maximum CU depth, cuW is the current block’s width, and cuH is the current block’s height.

Figure 1.  The neighbouring blocks for constructed affine candidates’ CPMV derivation.

# Proposed method

The proposed method includes two parts. The first part is to change the CPMVs derivation process. Instead of selecting the first available MVs in each position, it is proposed to separate the selection of MV in list 0 and list 1, and only the MV with reference index equal to 0 is selected. For example, the MV of CP0 in the list 0 is the first MV of list 0 whose reference index is equal to 0 in block B2, B3 and A2; the MV of CP0 in the list 1 is the first available MV of list 1 whose reference index is equal to 0 in block B2, B3 and A2. The “same reference index” checks for all CP sets are removed. This simplification can reduce the worst case number of comparison for reference index or inter-coded flag from 60 to 48. The second part is to remove the translational motion model checks, which can reduce 40 times of MV checks.

# Experiment results

The proposed method has been implemented on top of VTM3.0 [1]. The common test conditions [2] are used for evaluation. The results are summarized in Table 1.

Table1.  Results of the simplified constructed affine candidate derivation.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.12% | -0.12% | 101% | 102% |
| Class A2 | 0.01% | 0.04% | 0.06% | 101% | 101% |
| Class B | 0.01% | 0.11% | -0.01% | 101% | 100% |
| Class C | 0.02% | 0.01% | -0.06% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | -0.03% | 101% | 101% |
| Class D | 0.02% | 0.06% | -0.01% | 101% | 104% |
| Class F | -0.03% | -0.03% | -0.03% | 101% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.03% | 0.03% | 100% | 101% |
| Class C | 0.06% | 0.14% | 0.27% | 101% | 101% |
| Class E | 0.02% | 0.30% | 0.21% | 101% | 102% |
| Overall | 0.05% | 0.11% | 0.16% | 101% | 101% |
| Class D | 0.12% | -0.23% | 0.16% | 101% | 103% |
| Class F | 0.16% | 0.02% | -0.36% | 101% | 100% |

# Conclusions

In this proposal, a simplified constructed affine merging candidate derivation is proposed. It is reported that the proposed method results in very minor BD-rates and runtime changes.

# References

[1] VTM3.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).